| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0529-v2 |
| --- | --- |

| Title: | CE3.3 related: Intra 67 modes coding with 3MPM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Narae Choi Yinji Piao Kiho Choi Chanyul Kim (Samsung) | Tel: Email: | [+82-10-4011-8397] [narae1.choi@samsung.com] |
| Source: |  |  |  |

_____________________________

# Abstract

This contribution presents the performance and text of 67 intra mode coding when using 3MPM and 6-bits fixed-length coding(FLC) for remaining modes which is similar to HEVC intra mode coding. 67 modes with 3MPM on BTM shows only 0.3% loss in RA configuration with 5% encoder complexity reduction.

# Introduction

In current VTM s/w, there are in total 35 intra modes which is coded by 3MPM with 5-bit FLC for remaining mode coding. To support 67 intra prediction modes, current intra mode coding is modified to the remaining mode coding only from 5-bit FLC to 6-bit FLC.

# Results

The proposed intra mode coding was implemented on top of BMS s/w with VTM/BMS configurations under test common condition [1].

Table I. 67mode_3MPM

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.45% | -0.39% | 102% | 102% |  |  |  |  |  |
| Class A2 | -0.51% | -0.84% | -0.62% | 102% | 105% |  |  |  |  |  |
| Class B | -0.76% | -1.02% | -1.17% | 101% | 99% |  |  |  |  |  |
| Class C | -0.45% | -0.48% | -0.61% | 101% | 96% | 0.72% | 0.46% | 0.50% | 93% | 98% |
| Class E | -1.16% | -1.73% | -1.53% | 102% | 95% | 0.74% | 0.52% | 0.24% | 94% | 100% |
| Overall | -0.61% | -0.89% | -0.88% | 102% | 99% |  |  |  |  |  |
| Class D | 0.01% | -0.21% | -0.09% | 102% | 99% | 0.46% | 0.18% | 0.37% | 94% | 98% |
| Class F (optional) | 0.25% | 0.16% | 0.23% | 101% | 99% | 1.28% | 1.13% | 0.75% | 94% | 99% |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.33% | -0.27% | 101% | 101% |  |  |  |  |  |
| Class A2 | -0.42% | -0.70% | -0.45% | 101% | 103% |  |  |  |  |  |
| Class B | -0.36% | -0.45% | -0.65% | 102% | 105% | 0.32% | 0.16% | -0.15% | 95% | 101% |
| Class C | -0.18% | -0.01% | -0.24% | 102% | 100% | 0.33% | 0.26% | -0.07% | 94% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.29% | -0.36% | -0.43% | 102% | 102% |  |  |  |  |  |
| Class D | 0.03% | -0.05% | -0.12% | 104% | 98% | 0.21% | 0.04% | -0.20% | 112% | 99% |
| Class F (optional) | 0.16% | 0.22% | 0.27% | 112% | 106% | 0.81% | 0.53% | 0.70% | 120% | 106% |

Table I. 67mode_3MPM_plus1FullRDEnc

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.30% | -0.43% | -0.41% | 105% | 100% |  |  |  |  |  |
| Class A2 | -0.65% | -0.92% | -0.68% | 105% | 104% |  |  |  |  |  |
| Class B | -0.88% | -1.05% | -1.24% | 107% | 97% |  |  |  |  |  |
| Class C | -0.58% | -0.56% | -0.67% | 109% | 102% | 0.49% | 0.30% | 0.30% | 100% | 98% |
| Class E | -1.34% | -1.70% | -1.55% | 106% | 99% | 0.53% | 0.46% | 0.22% | 98% | 101% |
| Overall | -0.76% | -0.92% | -0.93% | 107% | 100% |  |  |  |  |  |
| Class D | -0.22% | -0.14% | -0.05% | 111% | 101% | 0.20% | 0.11% | 0.30% | 100% | 97% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.38% | -0.33% | 99% | 105% |  |  |  |  |  |
| Class A2 | -0.48% | -0.73% | -0.43% | 97% | 106% |  |  |  |  |  |
| Class B | -0.43% | -0.46% | -0.67% | 99% | 106% | 0.23% | -0.07% | -0.17% | 98% | 101% |
| Class C | -0.29% | -0.20% | -0.27% | 103% | 106% | 0.22% | 0.10% | -0.23% | 99% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.37% | -0.43% | -0.45% | 100% | 106% |  |  |  |  |  |
| Class D | -0.07% | 0.21% | -0.04% | 108% | 108% | 0.08% | -0.18% | -0.32% | 103% | 106% |
| Class F (optional) |  |  |  |  |  |  |  |  |  |  |

# Specification Text

This section is modified on top of HEVC specification (Rec. ITU-T H.265 v4) clause 8.4.2. Modified texts are highlighted in yellow.

    - Derivation process for luma intra prediction mode

Input to this process is a luma location ( xPb, yPb ) specifying the top-left sample of the current luma prediction block relative to the top-left luma sample of the current picture.

In this process, the luma intra prediction mode IntraPredModeY[ xPb ][ yPb ] is derived.

Table 8-1 specifies the value for the intra prediction mode and the associated names.

Table 8-1 – Specification of intra prediction mode and associated names

| Intra prediction mode | Associated name |
| --- | --- |
| 0 | INTRA_PLANAR |
| 1 | INTRA_DC |
| 2..66 | INTRA_ANGULAR2..INTRA_ANGULAR66 |

IntraPredModeY[ xPb ][ yPb ] labelled 0..66 represents directions of predictions as illustrated in Figure 8-1.

Figure 8-1 – Intra prediction mode directions (informative)

IntraPredModeY[ xPb ][ yPb ] is derived by the following ordered steps:

- The neighbouring locations ( xNbA, yNbA ) and ( xNbB, yNbB ) are set equal to ( xPb − 1, yPb ) and ( xPb, yPb − 1 ), respectively.

- For X being replaced by either A or B, the variables candIntraPredModeX are derived as follows:

- The availability derivation process for a block in z-scan order as specified in clause 6.4.1 is invoked with the location ( xCurr, yCurr ) set equal to ( xPb, yPb ) and the neighbouring location ( xNbY, yNbY ) set equal to ( xNbX, yNbX ) as inputs, and the output is assigned to availableX.

- The candidate intra prediction mode candIntraPredModeX is derived as follows:

- If availableX is equal to FALSE, candIntraPredModeX is set equal to INTRA_DC.

- Otherwise, if CuPredMode[ xNbX ][ yNbX ] is not equal to MODE_INTRA or pcm_flag[ xNbX ][ yNbX ] is equal to 1, candIntraPredModeX is set equal to INTRA_DC,

- Otherwise, if X is equal to B and yPb − 1 is less than ( ( yPb  >>  CtbLog2SizeY )  <<  CtbLog2SizeY ), candIntraPredModeB is set equal to INTRA_DC.

- Otherwise, candIntraPredModeX is set equal to IntraPredModeY[ xNbX ][ yNbX ].

- The candModeList[ x ] with x = 0..2 is derived as follows:

- If candIntraPredModeB is equal to candIntraPredModeA, the following applies:

  - If candIntraPredModeA is less than 2 (i.e., equal to INTRA_PLANAR or INTRA_DC), candModeList[ x ] with x = 0..2 is derived as follows:

candModeList[ 0 ] = INTRA_PLANAR	(8-21)

candModeList[ 1 ] = INTRA_DC		(8-22)

candModeList[ 2 ] = INTRA_ANGULAR50	(8-23)

  - Otherwise, candModeList[ x ] with x = 0..2 is derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-24)

candModeList[ 1 ] = 2 + ( ( candIntraPredModeA + 61 ) % 64 )	(8-25)

candModeList[ 2 ] = 2 + ( ( candIntraPredModeA − 2 + 1 ) % 64 )	(8-26)

- Otherwise (candIntraPredModeB is not equal to candIntraPredModeA), the following applies:

  - candModeList[ 0 ] and candModeList[ 1 ] are derived as follows:

candModeList[ 0 ] = candIntraPredModeA	(8-27)

candModeList[ 1 ] = candIntraPredModeB	(8-28)

  - If neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_PLANAR, candModeList[ 2 ] is set equal to INTRA_PLANAR,

  - Otherwise, if neither of candModeList[ 0 ] and candModeList[ 1 ] is equal to INTRA_DC, candModeList[ 2 ] is set equal to INTRA_DC,

  - Otherwise, candModeList[ 2 ] is set equal to INTRA_ANGULAR50.

- IntraPredModeY[ xPb ][ yPb ] is derived by applying the following procedure:

- If prev_intra_luma_pred_flag[ xPb ][ yPb ] is equal to 1, the IntraPredModeY[ xPb ][ yPb ] is set equal to  candModeList[ mpm_idx[ xPb ][ yPb ] ].

- Otherwise, IntraPredModeY[ xPb ][ yPb ] is derived by applying the following ordered steps:

- The array candModeList[ x ], x = 0..2 is modified as the following ordered steps:

  - When candModeList[ 0 ] is greater than candModeList[ 1 ], both values are swapped as follows:

( candModeList[ 0 ], candModeList[ 1 ] ) = Swap( candModeList[ 0 ], candModeList[ 1 ] )	(8-29)

  - When candModeList[ 0 ] is greater than candModeList[ 2 ], both values are swapped as follows:

( candModeList[ 0 ], candModeList[ 2 ] ) = Swap( candModeList[ 0 ], candModeList[ 2 ] )	(8-30)

  - When candModeList[ 1 ] is greater than candModeList[ 2 ], both values are swapped as follows:

( candModeList[ 1 ], candModeList[ 2 ] ) = Swap( candModeList[ 1 ], candModeList[ 2 ] )	(8-31)

- IntraPredModeY[ xPb ][ yPb ] is derived by the following ordered steps:

  - IntraPredModeY[ xPb ][ yPb ] is set equal to rem_intra_luma_pred_mode[ xPb ][ yPb ].

  - For i equal to 0 to 2, inclusive, when IntraPredModeY[ xPb ][ yPb ] is greater than or equal to candModeList[ i ], the value of IntraPredModeY[ xPb ][ yPb ] is incremented by one.

| Table 9-43 – Syntax elements and associated binarizations |  |  |  |
| --- | --- | --- | --- |
| Syntax structure | Syntax element | Binarization |  |
|  |  | Process | Input parameters |
| slice_segment_data( ) | end_of_slice_segment_flag | FL | cMax = 1 |
|  | end_of_subset_one_bit | FL | cMax = 1 |
| sao( ) | sao_merge_left_flag | FL | cMax = 1 |
|  | sao_merge_up_flag | FL | cMax = 1 |
|  | sao_type_idx_luma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_type_idx_chroma | TR | cMax = 2, cRiceParam = 0 |
|  | sao_offset_abs[ ][ ][ ][ ] | TR | cMax = ( 1 << ( Min( bitDepth, 10 ) − 5 ) ) − 1, cRiceParam = 0 |
|  | sao_offset_sign[ ][ ][ ][ ] | FL | cMax = 1 |
|  | sao_band_position[ ][ ][ ] | FL | cMax = 31 |
|  | sao_eo_class_luma | FL | cMax = 3 |
|  | sao_eo_class_chroma | FL | cMax = 3 |
| coding_quadtree( ) | split_cu_flag[ ][ ] | FL | cMax = 1 |
| coding_unit( ) | cu_transquant_bypass_flag | FL | cMax = 1 |
|  | cu_skip_flag | FL | cMax = 1 |
|  | palette_mode_flag | FL | cMax = 1 |
|  | pred_mode_flag | FL | cMax = 1 |
|  | part_mode | 9.3.3.7 | ( xCb, yCb ) = ( x0, y0), log2CbSize |
|  | pcm_flag[ ][ ] | FL | cMax = 1 |
|  | prev_intra_luma_pred_flag[ ][ ] | FL | cMax = 1 |
|  | mpm_idx[ ][ ] | TR | cMax = 2, cRiceParam = 0 |
|  | rem_intra_luma_pred_mode[ ][ ] | FL | cMax = 63 |
|  | intra_chroma_pred_mode[ ][ ] | 9.3.3.8 | - |
|  | rqt_root_cbf | FL | cMax = 1 |
| prediction_unit( ) | merge_flag[ ][ ] | FL | cMax = 1 |
|  | merge_idx[ ][ ] | TR | cMax = MaxNumMergeCand − 1, cRiceParam = 0 |
|  | inter_pred_idc[ x0 ][ y0 ] | 9.3.3.9 | nPbW, nPbH |
|  | ref_idx_l0[ ][ ] | TR | cMax = num_ref_idx_l0_active_minus1, cRiceParam = 0 |
|  | mvp_l0_flag[ ][ ] | FL | cMax = 1 |
|  | ref_idx_l1[ ][ ] | TR | cMax = num_ref_idx_l1_active_minus1, cRiceParam = 0 |
|  | mvp_l1_flag[ ][ ] | FL | cMax = 1 |
| transform_tree( ) | split_transform_flag[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_luma[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_cb[ ][ ][ ] | FL | cMax = 1 |
|  | cbf_cr[ ][ ][ ] | FL | cMax = 1 |
| mvd_coding( ) | abs_mvd_greater0_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_greater1_flag[ ] | FL | cMax = 1 |
|  | abs_mvd_minus2[ ] | EG1 | - |
|  | mvd_sign_flag[ ] | FL | cMax = 1 |
| transform_unit( ) | tu_residual_act_flag | FL | cMax = 1 |

# Conclusions

This document reports the result of 67 intra mode coding when using 3MPM and 6-bit FLC for remaining modes. Since there is only 0.3% difference in RA configuration on BMS compared to 6MPM with truncated unary coding for remaining mode in other proposal, adoption to VTM s/w as default setting is strongly recommended.

# Reference

- JVET, “JVET common test conditions and software reference configurations”, JVET-J1010, 10th JVET Meeting, San Diego, US, 10-20 Apr., 2018

- JVET, “Versatile Video Coding (Draft 1)”, JVET-J1001, 10th JVET Meeting, San Diego, US, 10-20 Apr., 2018

# Patent rights declaration(s)

Samsung Electronics Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).