| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0531-v1 |
| --- | --- |

| Title: | CE10-related: Combined tests of CE10.1.4 and CE10.1.8 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Man-Shu Chiang, Chih-Wei Hsu Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {man-shu.chiang,cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, the combined test regarding multi-hypothesis prediction in JVET-J0018 and JVET-J0014 are proposed. In JVET-J0018, multi-hypothesis prediction is applied to advanced motion vector prediction (AMVP) mode, skip or merge mode, and intra mode, which are tested in CE10.1.1, CE10.1.2, CE10.1.3, respectively. CE10.1.4 is the combined test of CE10.1.1, CE10.1.2, and CE10.1.3. In JVET-J0014, multi-hypothesis prediction is applied to merge mode, which is tested in CE10.1.5 to CE10.1.8 with different parameter settings. In this contribution, combined results of CE10.1.4 and CE10.1.8 is proposed. It is reported that, compared to VTM-1.0, this proposal achieves -2.45% and -2.58% luma BD-rates for RA and LB, respectively, with 39% and 54% encoding time increases and 9% and 7% decoding time increases. Compared to BMS-1.0, this proposal achieves xxx% and xxx% luma BD-rates for RA and LB, respectively, with xx% and xx% encoding time increases and xx% and xx% decoding time increases.

# Introduction

In CE10.1, multi-hypothesis prediction applied to uni-prediction of AMVP mode, skip or merge mode, and intra mode in JVET-J0018 are tested in CE10.1.1, CE10.1.2, and CE10.1.3, respectively. CE10.1.4 is the combined test of CE10.1.1, CE10.1.2, and CE10.1.3. Compared against VTM-1.0, CE10.1.4 can achieve -1.29% and -0.76% luma BD-rates for RA and LB, respectively, with 19% and 26% encoding time increases and 6% and 5% decoding time increases.

Multi-hypothesis prediction in JVET-J0014 is also tested in CE10.1.5 to CE10.1.8 and is applied to merge mode. The signaling is unchanged, but it is possible to specify one or more additional inter prediction signals. The resulting overall prediction signal is accumulated iteratively by each additional inter prediction signal with a weight. Totally, the reference index, MVP index, MVDs, and weights need to be signaled. There are “1” and “1 or 2” hypotheses tested in CE10.1.5 and CE10.1.6, respectively. Moreover, coding block (CB) size constraint is applied to CE10.1.5 and CE10.1.6 for CE10.1.7 and CE10.1.8, respectively. Compared against VTM-1.0, CE10.1.8 can achieve -1.84% and -2.27% luma BD-rates for RA and LB, respectively, with 25% and 38% encoding time increases and 5% and 8% decoding time increases.

# Proposed methods

In this contribution, combined results of CE10.1.4 and CE10.1.8 is proposed. In the test, multi-hypothesis prediction is applied to AMVP, skip, merge, and intra mode. When multi-hypothesis prediction is applied to merge mode, syntax in JVET-J0018 and JVET-J0014 are combined, and up to 2 hypotheses can be added.

# Syntax and semantics

The proposed syntax changes are highlighted in blue as follows.

| coding_unit( x0, y0, cbWidth, cbHeight ) { | Descriptor |
| --- | --- |
| if( slice_type != I ) { |  |
| pred_mode_flag | ae(v) |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] = = MODE_INTRA ) { |  |
| [Ed. (BB): Intra prediction yet to be added, pending further specification development.] |  |
| } else { |  |
| if( skip_flag[ x0 ][ y0 ] ) { |  |
| [Ed. (BB): Inter prediction yet to be added, pending further specification development.] |  |
| } else { |  |
| merge_flag[ x0 ][ y0 ] | ae(v) |
| if( merge_flag[ x0 ][ y0 ] ) { |  |
| if( MaxNumMergeCand > 1 ) |  |
| merge_idx[ x0 ][ y0 ] | ae(v) |
| mh_merge_flag | ae(v) |
| if(!mh_merge_flag) { |  |
| intra_inter_mode_flag | ae(v) |
| if( intra_inter_mode_flag) |  |
| intra_pred_mode | ae(v) |
| } |  |
| if (!mh_merge_flag && ! intra_inter_mode_flag){ |  |
| mh_pred_data |  |
| } |  |
| } else { |  |
| if(inter_dir != 3) { |  |
| umb_flag | ae(v) |
| } |  |
| if(umb_flag) { |  |
| umb_merge_idx | ae(v) |
| } |  |
| mh_pred_data |  |
| } |  |
| } |  |
| } |  |
| if( CuPredMode[ x0 ][ y0 ] != MODE_INTRA ) |  |
| cu_cbf | ae(v) |
| if( cu_cbf ) { |  |
| transform_tree( x0, y0, cbWidth, cbHeight ) |  |
| } |  |

mh_merge_flag equals to TRUE specifies that multi-hypothesis prediction is applied to merge mode.

intra_inter_mode_flag equals to TRUE specifies that multi-hypothesis prediction is applied to intra mode.

intra_pred_mode is a function to signal the selected intra prediction mode with contexts as follows:

–	A flag is first signaled to indicate whether the selected intra prediction mode is one of three MPMs or not.

–	If the above flag is TRUE, the selected intra prediction mode can be acquired by an index, which indicates into the derived MPM list.

–	If the above flag is FALSE, the selected intra prediction mode is the only one intra prediction mode, which does not belong to the derived MPM list.

umb_flag equals to TRUE specifies that multi-hypothesis prediction is applied to uni-prediction for AMVP mode.

umb_merge_idx specifies the merge index to generate the merge-indexed prediction for multi-hypothesis prediction.

mh_pred_data is a function to signal the syntax for multi-hypothesis prediction in JVET-J0014

# Simulation results

The test has been implemented on BMS-1.0 [1], and results are summarized in Table 1. The simulations were performed following the JVET common test conditions [2].

Table 1.  Combined test of CE10.1.4 with speedup and CE10.1.8

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.67% | -2.22% | -1.86% | 128% | 108% |  |  |  |  |  |
| Class A2 | -1.78% | -1.40% | -1.22% | 139% | 110% |  |  |  |  |  |
| Class B | -3.48% | -2.35% | -2.11% | 141% | 111% | -2.48% | -1.95% | -1.76% | 114% |  |
| Class C | -2.25% | -1.36% | -1.53% | 146% | 105% | -1.40% | -0.99% | -1.33% | 116% | 103% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -2.45% | -1.87% | -1.72% | 139% | 109% |  |  |  |  |  |
| Class D | -2.39% | -0.82% | -0.84% | 152% | 109% | -1.15% | -0.71% | -0.79% | 118% | 106% |
| Class F | -2.02% | -1.21% | -1.13% | 145% | 105% | -1.77% | -1.33% | -1.19% | 112% | 104% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -3.01% | -2.05% | -2.07% | 149% | 111% |  |  |  |  |  |
| Class C | -2.41% | -1.06% | -1.56% | 151% | 102% | -1.36% | -0.51% | -0.63% | 121% | 103% |
| Class E | -2.10% | 1.34% | 0.18% | 168% | 108% | -0.62% | 0.49% | 0.67% | 127% | 105% |
| Overall | -2.58% | -0.87% | -1.34% | 154% | 107% |  |  |  |  |  |
| Class D | -1.63% | -1.21% | -1.37% | 153% | 108% | -1.09% | -0.61% | -1.14% | 124% | 106% |
| Class F | -4.27% | -3.28% | -3.35% | 152% | 107% | -2.96% | -2.32% | -2.10% | 116% | 105% |

# Conclusions

In this contribution, combined results of CE10.1.4 and CE10.1.8 is proposed. Compared against VTM-1.0, this mechanism can achieve -2.45% and -2.58% luma BD-rates for RA and LB, respectively, with 39% and 54% encoding time increases and 9% and 7% decoding time increases. Compared against BMS-1.0, this mechanism can achieve xxx% and xxx% luma BD-rates for RA and LB, respectively, with xx% and xx% encoding time increases and xx% and xx% decoding time increases.

# Reference

[1] BMS-1.0: https://jvet.hhi.fraunhofer.de/svn/svn_VVCSoftware_BMS/tags/BMS-1.0

[2] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).