| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0536_r1 |
| --- | --- |

| Title: | Non-CE3: Adaptive multiple cross-component linear model |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sheng-Po Wang1, Po-Han Lin, Chang-Hao Yau, Chun-Lung Lin and Ching-Chieh Lin2 2870 Zanker Road, Suite 140, San Jose, CA 95134, USA | Tel: Email: | +886-3-5915059 samperwang@itri.org.tw 1 JackLin@itri.org.tw 2 |
| Source: | ITRI International |  |  |

_____________________________

- Abstract

In this contribution, an adaptively grouping mechanism is utilized to classify neighboring samples into groups to derive a linear model for each group. Then, chroma samples are predicted based on the reconstructed luma samples by using a linear model in the nearest group. Test results show 1.54%, 9.65% and 10.06%BD-rate saving on Luma and chroma respectively under All Intra test condition over VTM1.0.

Introduction

There are single linear model (CCLM), multiple model LM (MMLM), and multiple down-sampling filter LM (MFLM) defined in BMS1.0 [1-2] for cross component prediction. However, both CCLM and MMLM are unable to adaptively deal with multiple luma-chroma relationships in a single block. This contribution utilizes an adaptive grouping algorithm to dynamically generate single or multiple groups and derive linear model for each group respectively. Then, chroma samples can be predicted by linear model of the nearest group.

Adaptive Multiple Linear Model

The proposed method can classify samples from four lines of top and left neighbouring reconstructed blocks into groups according to both luma and chroma values. Neighboring samples are scanned and determined if it can join an existing group according to the criteria (1) and (2).

Ygroup_min – Ymargin < Yp < Ygroup_max + Ymargin  (1)

Cgroup_min – Cmargin < Cp < Cgroup_max + Cmargin   (2)

Note that Yp and Cp denotes the luma and chroma value of a neighboring sample. Ygroup_max and Cgroup_max denotes the maximum luma and chroma value in a group. Ygroup_min and Cgroup_min denotes the minimum luma and chroma value in a group. Ymargin and Cmargin denotes the margin of the group for luma and chroma respectively.

If the neighboring sample cannot satisfy the criteria of any existing group, a new group will be created. As a result, all the neighboring samples will be classified into a number of groups as illustrated in Figure 1.

Figure 1. Group classification of predictor pixels

After the grouping procedure is completed, a pair of parameters α and β for each group will be derived by minimizing the regression error following the equation defined in CCLM.

When predicting a chroma samples, a nearest group g is firstly determined according to the averaged luma value of the group. Then the LM parameters αg and βg of the nearest group will be used to derive the chroma value with the following equation:

Simulation results

This contribution induces four lines of neighboring samples from top and left neighbouring reconstructed blocks. Groups with less than 4 samples will be ignored. The anchor is VTM1.0. Test condition is JVET CTC on SDR[3]. The test result shows 1.54% BD-rate saving for luma.

Table1 Result of Test1 - all Intra

Conclusion

This contribution proposes to classify neighboring samples into single or multiple groups and then apply cross component linear model to each group. The test results show 1.54%, 9.65% and 10.06%BD-rate saving on Luma and chroma respectively under All Intra test condition. It is recommended to further study in a CE.

Reference

- Jianle Chen, Elena Alshina, Gary J. Sullivan, Jens-Rainer Ohm, and Jill Boyce, “Algorithm Description of Joint Exploration Test Model 7 (JEM 7)”, JVET-G1001-v1, July 2017

- Geert Van der Auwera, Jin Heo, and Alexey Filippov, “Description of Core Experiment 3: Intra Prediction and Mode Coding”, JVET-J1023_r1, Apr. 2018

- Jill Boyce, K. Suehring, X. Li, and Vadim Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, April 2018.

Patent rights declaration(s)

ITRI International may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).