| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0540-v1 |
| --- | --- |

| Title: | CE2-related: Reduced filter shape size for ALF without classification |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin Nan Hu Marta Karczewicz | Tel: Email: | martak@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution presents results of CE2 test 4.1.4 where the largest ALF filter shape is reduced from 9x9 to 7x7 and classification process is disabled. Test results provide 1.58%, 3.08% and 2.79% luma gain in AI, RA, and LB configurations respectively over VTM-1.0 anchor for 7x7 max filter size, and provide 1.78%, 3.30% and 3.10% luma gain in AI, RA, and LB configurations respectively over VTM-1.0 anchor for 9x9 max filter size.

# Introduction

In the BMS software, the largest ALF filter shape is 9x9 for luma and 5x5 for chroma components, while classifier block size is 2x2.

In the CE2 test 4.1.4 reduced largest filter size to 7x7 and increased filter classifier block size to 4x4 for luma component are tested for the ALF in BMS.

This contribution provides test results when classification is disabled on top of CE tests in the CE2-4.1.4 software.

# Experimental Results

The discussed changes were implemented into CE2-4.1.4 software and BMS software configured in VTM configuration is used as anchor. Results are summarized in below tables and can be found in detail in the accompanied Excel tables.

Table 1. Summary of all tests results for VTM configuration (positive numbers is coding gain).

| Test | AI |  |  | RA |  |  | LB |  |  | RA Enc | RA Dec |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1. luma 7x7, classifier 4x4, chroma 5x5 (CE2-4.1.4) | 2.99 | 3.56 | 4.07 | 4.99 | 2.24 | 1.66 | 4.19 | 1.64 | 1.85 | 104% | 122% |
| 2. luma 7x7, classifier 4x4, chroma 7x7 (CE2-4.1.4) | 2.98 | 4.20 | 4.73 | 4.98 | 3.14 | 2.66 | 4.23 | 2.44 | 2.57 | 104% | 122% |
| 3. luma 7x7, chroma 5x5 | 1.59 | 3.66 | 4.17 | 3.11 | 2.31 | 1.70 |  |  |  | 102% | 116% |
| 4. luma 7x7, chroma 7x7 | 1.58 | 4.31 | 4.83 | 3.08 | 3.21 | 2.69 | 2.79 | 2.94 | 3.00 | 102% | 116% |
| 5. luma 9x9, classifier 4x4, chroma 5x5 (CE2-4.1.4) | 3.17 | 3.54 | 4.05 | 5.21 | 2.20 | 1.66 | 4.46 | 1.83 | 2.01 | 107% | 129% |
| 6. luma 9x9, classifier 4x4, chroma 7x7 (CE2-4.1.4) | 3.16 | 4.19 | 4.71 | 5.23 | 3.15 | 2.63 | 4.49 | 2.59 | 2.72 | 108% | 129% |
| 7. luma 9x9, chroma 5x5 | 1.79 | 3.65 | 4.16 | 3.30 | 2.29 | 1.67 |  |  |  | 104% | 122% |
| 8. luma 9x9, chroma 7x7 | 1.78 | 4.29 | 4.82 | 3.30 | 3.15 | 2.61 | 3.10 | 2.89 | 3.12 | 104% | 123% |

# Conclusion

Comparing to CE tests, which include classification, and the tests performed in this contribution with disabled classification, it can be concluded that:

- For 7x7 max filter size, classification luma gain is 1.88% in RA (comparing test#1 and test#3), runtime increase is 2% at encoder and 6% at decoder.

- For 9x9 max filter size, classification luma gain is 1.91% in RA (comparing test#5 and test#7), runtime increase is 4% at encoder and 7% at decoder.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).