| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0XXX-v1 |
| --- | --- |

| Title: | Cross-check of JVET-K0276: CE3-related: Mode-dependent multiple reference line intra prediction with bi-prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Sychev Maxim Huawei Technologies, Russia | Tel: Email: | Sychev.maxim@huawei.com |
| Source: | Huawei Technologies |  |  |

_____________________________

# Abstract

This document reports the cross-check results for JVET-K0276. The simulations were conducted following the common test conditions in JVET-J1010. The obtained simulation results match the results reported in JVET-K0276. Small deviation of encoder time in several tests are observed. Additionally LDP results are provided.

# Introduction

Proponent of K0276 proposes to add bi-prediction to mode-dependent multiple-reference line intra prediction as tested in sub-CE 3.5, test 5.4.4 and reported in JVET-K0051. Cross-check was conducted base on source code provided by proponent of K0276.

# Code Analysis

The provided source code was analyzed and corresponds to the description of the proposal JVET-K0276 [3].

# Test results

According to the JVET-K0276 [3] and the common test conditions in JVET-J1010 [4], simulations were performed for random-access (RA) and low-delay (LD) configuration with both VTM and BMS configuration.

Table 1 Results with anchor VTM & BMS

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -1.65% | -1.07% | -0.89% | 189% | 101% | -0.93% | -0.90% | -0.86% | 133% | 104% |
| Class A2 | -1.09% | -0.62% | -0.39% | 189% | 101% | -0.67% | -0.39% | -0.07% | 133% | 103% |
| Class B | -1.14% | -0.45% | -0.75% | 186% | 100% | -0.77% | -0.30% | -0.41% | 132% | 103% |
| Class C | -0.78% | -0.41% | -0.54% | 183% | 103% | -0.59% | -0.31% | -0.46% | 131% | 102% |
| Class E | -1.00% | -0.94% | -0.87% | 179% | 102% | -0.66% | -0.55% | -0.58% | 132% | 103% |
| Overall | -1.12% | -0.66% | -0.69% | 185% | 101% | -0.72% | -0.46% | -0.47% | 132% | 103% |
| Class D | -0.56% | -0.15% | -0.25% | 183% | 102% | -0.35% | -0.14% | -0.15% | 131% | 103% |
| Class F (optional) | -0.45% | -0.21% | -0.19% | 173% | 102% | -0.34% | -0.27% | -0.39% | 131% | 103% |

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.82% | -0.98% | -0.92% | 119% | 101% | -0.49% | -0.77% | -0.50% | 121% | 104% |
| Class A2 | -0.46% | -0.69% | -0.49% | 112% | 101% | -0.31% | -0.48% | -0.16% | 116% | 104% |
| Class B | -0.53% | -0.50% | -0.99% | 113% | 100% | -0.40% | -0.14% | -0.43% | 117% | 103% |
| Class C | -0.40% | -0.42% | -0.45% | 113% | 101% | -0.25% | 0.01% | -0.36% | 116% | 102% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.54% | -0.61% | -0.73% | 114% | 101% | -0.36% | -0.29% | -0.37% | 117% | 103% |
| Class D | -0.21% | -0.12% | -0.47% | 113% | 100% | -0.09% | -0.31% | -0.34% | 115% | 103% |
| Class F (optional) | -0.28% | -0.30% | -0.02% | 119% | 101% | -0.20% | -0.09% | 0.06% | 118% | 102% |

|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-1.0 |  |  |  |  | Over BMS-1.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.16% | -0.18% | -0.03% | 109% | 102% | -0.14% | -0.19% | 0.08% | 109% | 103% |
| Class C | -0.13% | 0.09% | 0.09% | 107% | 101% | -0.14% | 0.07% | -0.19% | 108% | 102% |
| Class E | -0.06% | -0.57% | 0.83% | 102% | 101% | -0.33% | -0.51% | -0.77% | 102% | 102% |
| Overall | -0.12% | -0.19% | 0.23% | 107% | 101% | -0.19% | -0.18% | -0.22% | 107% | 103% |
| Class D | -0.10% | -0.12% | -0.26% | 105% | 100% | -0.06% | 0.48% | -0.18% | 106% | 102% |
| Class F (optional) | -0.23% | -0.24% | -0.23% | 110% | 101% | -0.10% | -0.16% | -0.38% | 111% | 100% |

# Conclusion

Performance improvements of mode-dependent multiple reference line intra prediction with bi-prediction have been cross-checked, and the simulation results reported in JVET-K0276 are confirmed. Small differences in decoding time are noticed. Although the results in comparison with BMS-1.0 software in VTM configuration in decoder are a little bit slower by cross-check.

# References

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” JVET-J1023, San Diego, USA, April 2018.

- B. Bross, H. Schwarz, D. Marpe, T. Wiegand, “CE3: Multiple reference line intra prediction (Test 5.4.1, 5.4.2, 5.4.3 and 5.4.4),” JVET-K0051, Ljubljana, SI, July 2018.

- B. Bross, P. Keydel, H. Schwarz, D. Marpe, T. Wiegand , “CE3-related: Mode-dependent multiple reference line intra prediction with bi-prediction”, JVET-K0276, Ljubljana, SI, July 2018.

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.