| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0546-v1 |
| --- | --- |

| Title: | BoG report on CE4 related contributions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Haitao Yang | Email: | haitao.yang@huawei.com |
| Source: | BoG |  |  |

_____________________________

- Abstract

Affine motion compensation (12)

JVET-K0052 Non-CE4: A study on the affine merge mode [M. Zhou (Broadcom)]

This contribution studied line buffer usage of the affine merge mode in BMS1.0 and advocated the following three changes to simplify the design, namely, 1) directly using 4-parameter affine motion model to derive the seed vectors for the current PU of the affine merge mode; 2) disabling the affine merge mode for PUs whose width is less than 8 to enable motion data line buffer sharing, and 3) having the affine (merge) mode and the regular merge/skip and AMVP mode share the same motion data line buffer. For 4K video, the proposed changes reduce the line buffer size of the affine merge mode from roughly 18,688 bytes to 320 bytes, without comprising compression efficiency.  Relative to the BMS1.0 anchor, the overall BD-rate changes are 0.03% in RA, -0.07% in LD-B and -0.03% in LD-P, respectively.  Compared to the BMS1.0 VTM configurations but with “Affine” and “HighPrecMv” on, the overall BD-rate changes are 0.00% in RA, 0.02% in LD-B and 0.06% in LD-P, respectively. It is recommended to study the simplification of the affine (merge) mode to make it more implementation friendly.

This contribution proposes using 4-parameter affine motion model to derive the seed vectors for the current PU of the affine merge mode. BMS affine firstly derive the bottom-left MV and then use the pseudo 6-param model to derive two CPMVs of the current block, which is a redundant operation.

Recommendation: when inheriting the 4-param affine model from neighboring blocks, remove redundant operation of deriving the bottom-left CPMV of the neighboring block. Use 4-parameter model to compute seed vectors for the current CU. Adopt this in BMS affine.

It is proposed to reuse the motion data line buffer as regular inter mode. It is noted that the potential increase of line buffer size for 4K is about 18 KB if we store affine CPMVs separately.

Recommendation: further study in CE.

It is shown in this contribution there is no performance drop when storing 1/4 precision MV instead of 1/16 precision MV in line buffer. Suggest further study.

It is commented that storing high-precision MV in motion buffer causes higher memory bandwidth access and larger line buffer size as well.

JVET-K0508 Crosscheck of JVET-K0052 (Non-CE4: A study on the affine merge mode) [H. Chen, H. Yang, J. Chen (Huawei)] [late] [miss]

JVET-K0367 Non-CE4: BMS affine improvements [H. Chen, H. Yang, J. Chen (Huawei)]

In this contribution, affine prediction in BMS1.0 is improved by 4 aspects: 1) bit-exact SIMD implementation for affine ME; 2) optimize for adaptive sub-block in affine MC; 3) division free for affine merge candidate derivation; 4) modify the condition check for affine merge mode.

For affine SIMD optimization, experimental results reportedly show on average 9%/13% encoding time reduction in RA/LB configurations over VTM with affine on, and 3%/7% encoding time reduction in RA/LB configurations over BMS anchor. For other aspects, experimental results reportedly show on average 0.00%/0.01% luma BD-rate change with 1%/1% encoding time reduction and 2%/2% decoding time reduction in RA/LB configurations over VTM with affine on.

Recommendation: adopt the SIMD implementation into BMS affine, after code review from W.-J. Chien and Yuwen He.

The second point on adaptive sub-block in affine MC is not relevant since fixed 4x4 sub-block partition has been adopted into BMS affine.

The contribution further replace the division operation by shifting when inheriting the affine model from neighboring blocks. This could be applied to both affine merge candidate derivation and affine MVP candidate derivation. It is commented that the rounding of the motion vectors should be aligned with other rounding operations applied to motion vectors, e.g. to AMVR.

The contribution further proposes to enable affine merge to blocks w&h >= 8. Note that in current BMS, the block size restriction w*h >=64 pixels is applied to affine merge, w&h >= 16 to affine inter. It is commented that this fixes a bug of CPMV storage in the current BMS affine merge.

Recommendation: adopt this restriction to BMS affine. Note that the same restriction on affine merge mode is also proposed in JVET-K0103 and JVET-K0052.

It is mentioned that the JVET-K0103 propose applying the same block size restriction, w&h >= 8, to both affine inter and affine merge mode.

JVET-K0484 Cross-check of JVET-K0367: Non-CE4: BMS affine improvements [K. Zhang (Bytedance)] [late] [miss]

JVET-K0095 CE4-related: Harmonization of CE4.1.7 and CE4.1.3 [J. Lee, J. Nam, N. Park, H. Jang, J. Lim, S. Kim (LGE)]

It is about 1) slice level switching of 4/6-param affine model, 2) conditional signaling of 6-param model at block level, and 3) by-pass coding of the flag for 4/6-param model switching at block level.

Recommendation: further study in CE.

JVET-K0413 Crosscheck of JVET-K0095: Harmonization of CE4.1.7 and CE4.1.3 [Y. Han, Y. Zhang, W.-J. Chien (Qualcomm)] [late]

JVET-K0101 CE4-related: Affine MVD Coding [S. Paluri, M. Salehifar, S. Kim (LGE)]

In this proposal, a new motion vector difference coding (MVD) for Affine Motion Vector Difference (AMVD) is introduced. In affine mode, two control motion vectors are used to derive affine motion vectors for each sub-block. One is motion vector from left top and the other is motion vector from right top. Hence, two MVDs are coded. The proposed method jointly exploits the similarity of the MVD between the MVD from left and MVD from right. It is reported that 0.09% BD-rate saving is observed for Random Access configuration.

It is claimed that the joint coding of component from 2 CPMVs could also be applied to the case of 3 CPMVs.

It is claimed that this method could be combined with the MVD coding in JVET-K0337.

It is commented that affine MVD coding requires coding 2/3 MVs instead of 1 MV, which may need a coding method with higher efficiency.

Further study is encouraged.

JVET-K0432 Cross-check of JVET-K0101: CE4-related: Affine MVD Coding [T.-H. Li, H.-J. Jhu, Y.-J. Chang (Foxconn)] [late] [miss]

JVET-K0102 CE4-related: Interweaved Prediction for Affine Motion Compensation [K. Zhang, L. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

With the affine motion compensation (AMC) in the benchmark set (BMS), a coding-block is divided into sub-blocks as small as 4×4, each of which is assigned with an individual motion vector derived by the affine model. In this contribution, an interweaved prediction approach is proposed for AMC. With the interweaved prediction, a coding block is divided into sub-blocks with two different dividing patterns. Then two auxiliary predictions are generated by AMC with the two dividing patterns. The final prediction is calculated as a weighted-sum of the two auxiliary predictions. The interweaved prediction is only applied on the luma-component for affine-coded blocks with uni-prediction. Simulation results reportedly show 0.29% BD-rate savings under BMS Random Access (RA) configurations.

Multi-hypothesis prediction with two hypothesis is proposed for affine MC. Sub-block motion vectors at two sets of different sampling positions are derived and two prediction hypothesis is obtained by applying MC using the two sets of motion vectors. The two predictions are then averaged to get the final prediction.

It is applied to luma component in case of uni-prediction of affine coded blocks. So the worst case of memory access bandwidth is not changed.

It is noted that weighting matrices are designed to for combining the two prediction hypothesis.

It is noted that for boundary and corner positions in the auxiliary block partitioning patent, motion compensation is performed on 2x2 and 2x4/4x2 blocks.

-0.3% BD-Rate is observed for BMS tool test.

Recommendation: further study in CE.

JVET-K0455 Cross-check of JVET-K0102: CE4-related: Interweaved Prediction for Affine Motion Compensation [Y. He (InterDigital)] [late] [miss]

JVET-K0103 CE4-related: Simplified Affine Prediction [K. Zhang, L. Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

In this contribution, affine prediction in the benchmark set (BMS) is modified in three aspects. First, the sub-block size is fixed to be 4×4, instead of being calculated by an equation with division operations.

This was already decided in track B discussion earlier.

Second point in the contribution, the constraints of block size for affine merge mode and affine AMVP mode are unified. Both affine merge mode and the affine AMVP mode are applicable when the width and height of a block is greater than or equal to 8.

The coding performance is slightly degraded. And the encoding time for VTM tools test is increased by 2%. In case of 6-param affine inter prediction for 8x8 CU, 3 MVs instead of 4 MVs are signaled. It is commented that this does not reduce parsing efforts in BMS affine. It is claimed that the proposed change could reduce the number of lines in BMS SW.

Third, the sub-block size for chroma components is expanded from 2×2 to 4×4. Simulation results reportedly show 0.03% BD-rate increase on the Y component for Random Access (RA) configuration in average compared with BMS-1.1.

This implies using different motion vectors for the MC of luma and chroma. Additional sub-block MV derivation for chroma component is required.

Recommendation: test the 4x4 block chroma sub-block in CE.

JVET-K0507 Crosscheck of JVET-K0103 (CE4-related: Simplified Affine Prediction) [H. Chen, H. Yang, J. Chen (Huawei)] [late] [miss]

JVET-K0193 CE4-related: On performance improvements of Enhanced Interpolation Filter (EIF) [M. Sychev, G. Zhulikov, T. Solovyev, H. Chen, J. Chen (Huawei)] [late]

It is shown that the coding artifacts along a sharp edge in case of rotation could be alleviated by the per-pixel affine MC using EIF interpolation filter.

Combined with other techniques in affine MC, at maximum 0.7% coding gain could be obtained for VTM tool test in RA configuration.

It is commented that EIF may have interdependencies with OBMC and ALF in terms of coding gain.

Further study is encouraged.

JVET-K0448 Crosscheck of JVET-K0193: CE4-related: On performance improvements of Enhanced Interpolation Filter (EIF) [?? (Fraunhofer HHI)] [late] [miss]

JVET-K0194 CE4-related: On further complexity reduction of Enhanced Interpolation Filter (EIF) [M. Sychev, G. Zhulikov, T. Solovyev, J. Chen (Huawei)] [late]

Proposed two aspects of design improvement and complexity reduction of Enhanced Interpolation Filter (EIF) for inter prediction. EIF was initially proposed in JVET-J0024 and further studied in JVET-K020. Currently studied version of EIF use 3-tap high-pass filter (EIF3) and proposed as a solution for complexity reduction and worst case memory bandwidth reduction raised in JVET-J0081 for sub-block size 4x4. There are two aspects included in the document:

- Directional Bilinear Interpolation Filter (DBIF) with reduced number of operation by 1/3 in respect to bilinear in EIF.

- 16-bit friendly design of EIF.

Further study is encouraged.

JVET-K0449 Crosscheck of JVET-K0194: CE4-related: On further complexity reduction of Enhanced Interpolation Filter (EIF) [?? (Fraunhofer HHI)] [late] [miss]

JVET-K0267 CE4-related: Virtual Temporal Affine [F. Galpin, T. Poirier, F. LeLeannec, A. Robert (Technicolor)]

This contribution reports the results of a modified temporal affine merge candidate using virtual affine model from collocated candidates. We report the results of implementation in JVET VTM-1.0 and BMS-1.0. For tool ON/tool OFF test, simulation results reportedly show an average luma BD-rate gain of -0.03%, with gain up to -0.40% on CTC sequences, for Random Access (RA) compared with VTM. Moreover, additional tests on affine class sequences show an average luma BD-rate gain in RA of -0.23% for VTM, with gain up to -0.78%. In BMS configuration on the same affine class, simulation results reportedly show an average luma BD-rate gain of -0.28%, with gains up to -1.00%.

It is proposed to use the motion info in the top-left and top-right, or top-left and bottom-left corner, in the temporal collocated block, scale the two motion vectors, and then use them as merge candidates. At maximum two candidates will be derived.

Recommendation: study in CE.

JVET-K0268 CE4-related: Affine tools from J0022 [F. Galpin, A. Robert, T. Poirier, F. LeLeannec (Technicolor)]

This contribution reports the results of all affine mode enhancement described in JVET-J0022 [1], implemented within the JVET VTM-1.0 and BMS-1.0. Simulation results reportedly show an average luma BD-rate gain of -3.77% in Random Access (RA) configuration, compared with VTM. Moreover, additional tests on affine class sequences show an average luma BD-rate gain in RA of -12.21% for TM, resp. -11.53%, for BMS without affine.

JVET-K0335 CE4-related: Shape dependent control point selection for affine mode [Y. He, X. Xiu, Y. Ye (InterDigital)]

Affine coding mode is a tool in Benchmark Set (BMS). It is a four-parameter motion model which can describe translation, zooming, and rotation. The model parameters are signaled by motion vectors at two control points: top-left and top-right corners of an affine coding block. A shape dependent control point selection method is proposed in this document, where the positions of two control points are determined based on the coding block shape. Two sets of control points are defined for different coding block shape: top-left and top-right corners are selected for the coding block with horizontal and square shape; top-left and bottom-right corners are selected for the coding block with vertical shape. Compared to the VTM-1.0 anchor, the proposed method achieves average Y BD-rate reduction of 3.22% for Random Access and 2.26% for Low Delay B. Compared to the VTM-1.0 with affine mode enabled, it achieves average Y BD-rate reduction of 0.24% for Random Access and 0.20% for Low Delay B. Compared to the BMS-1.0 anchors, Y BD-rate reduction of 0.19% for Random Access cases and 0.24% for Low Delay B cases is reported.

The second part of the proposal is about affine MVP list construction. The sorting of constructed affine motion predictors are removed in the proposal. Not applicable to JVET-K0337 affine MVP list construction which had just been adopted in BMS.

Shape dependent control point selection could be applied to sorting inherited affine model from neighboring blocks in the list construction of affine merge and affine MVP.

Recommendation: further study shape dependent control point selection in CE, for both affine merge and affine MVP.

JVET-K0509 Crosscheck of JVET-K0335 (CE4-related: Shape dependent control point selection for affine mode) [H. Chen, H. Yang, J. Chen (Huawei)] [late]

JVET-K0481 Integrating affine-based motion model in HEVC encoder for future video coding [M. S. Sayed (Egypt-Japan Univ. Sci. & Tech.)] [late] [miss]

In this contribution, it is proposed to add deformable block matching as an additional tool to the HEVC encoder. It is proposed to use affine transformation in a simple way to be integrated in the HEVC encoder to represent more complicated motion types such as rotation, zooming and deformation. It is claimed that the proposed tool shows slight reduction in the BD-rate however it represents a start point for further improvements in the direction of deformable block matching. It is claimed that better performance is expected from the proposed tool with improvements in its motion estimation and compensation processes.

Triangle-shaped PU is proposed. The motion vector at three vertices are coded. Per-pixel motion compensation is performed. Scan line algorithm for 2-D affine transformations is employed for calculating motion vector of each pixel inside a triangle partition. Motion accuracy is 1/4. Motion vector coding is not changed.

The proposed method is implemented on top of HM. Results show only slight coding gain on affine sequences.

It is commented that 1/4 accuracy for affine motion compensation may be a reason why this is no coding gain.

Merge mode enhancement (14)

ATMVP modifications

JVET-K0080 CE4-2.6 related: Simplified ATMVP with fixed sub-block size [H. Jang, J. Lim, J. Nam, S. Kim (LGE)]

Contribution discussed together with JVET-K0346.

JVET-K0412 Crosscheck of JVET-K0080: Simplified ATMVP with fixed sub-block size [Y. Han, W.-J. Chien (Qualcomm)] [late]

JVET-K0189 Non-CE4: ATMVP simplification [H. Chen, H. Yang, J. Chen (Huawei)]

Contribution discussed together with JVET-K0346.

JVET-K0460 Cross-check of JVET-K0189: Non-CE4: ATMVP simplification [Y. He (InterDigital)] [late] [miss]

JVET-K0346 CE4-related: One simplified design of advanced temporal motion vector prediction (ATMVP) [X. Xiu, Y. He, Y. Ye (InterDigital)]

Three modifications to BMS ATMVP are suggested,

- One fixed collocated picture is used to derive temporal motion information.

- Slice level adaptive sub-block switching, 8x8 or 4x4.

- Constrain the region from where ATMVP motion is derived to the collocated CTU plus one 4x4 block column outside the collocated CTU at the right hand side, the same region for HEVC TMVP.

Note that the first one point is the same as CE4.2.6.a, and the second point is the same as CE4.2.5.b.

It is commented that the third point could reduce the worst case memory access bandwidth.

Recommendation: integrate the three modifications into BMS ATMVP.

It is noted that several experts support adopting the modified BMS ATMVP to VTM, some other experts request further discussion.

JVET-K0423 Cross-check of JVET-K0346: CE4-related: One simplified design of advanced temporal motion vector prediction (ATMVP) [F. Le Léannec] [late] [miss]

Modified merge candidates

JVET-K0056 Non-CE4: Merge mode modification [T. Solovyev, J. Chen, S. Ikonin (Huawei)] [late]

This contribution describes the enhanced merge mode based on VTM 1.0 software. The proposed method extends the merge candidates list by spatial and temporal candidates. Additional spatial candidates are located in current CTU line and bottom line of above CTU. The total amount of checking positions is limited by 22 and final number of candidates is limited to 10.

It is claimed that additional two TMVP candidates provide about 0.2% coding gain.

The order of candidates, regular spatial and temporal candidates, additionally 2 temporal candidates, at most three long distance spatial candidates.

Recommendation: study this in CE.

JVET-K0104 CE4-related: History-based Motion Vector Prediction [L.Zhang, K.Zhang, H. Liu, Y. Wang, P. Zhao, D. Hong (Bytedance)]

No presenter

JVET-K0456 Cross-check of JVET-K0104: CE4-related: History-based Motion Vector Prediction [Y. He (InterDigital)] [late] [miss]

JVET-K0301 CE4-related: Extension of merge and AMVP candidates for inter prediction [G. Li, X. Xu, X. Li, S. Liu (Tencent)]

In this contribution, it is proposed to add two spatial neighboring positions, which are from the middle position at the left and top edge of the current block, into motion vector predictor candidate list. The new candidates are put at the beginning of the merge candidate list and the AMVP candidate list, if available. The number of allowed merge candidates or AMVP candidates is kept unchanged. The proposed method was tested on BMS-1.0. It is reported that with VTM configuration, the result has 0.14%, 0.12%, and 0.16% BD-rate improvement for Random Access, Low Delay B, and Low Delay P, respectively.

Recommendation: study this in CE.

JVET-K0491 Cross-check of JVET-K0301: CE4-related: extension of merge and AMVP candidates for inter prediction [T. Zhou, T. Ikai (Sharp)] [late] [miss]

JVET-K0352 CE4-related: Encoder optimization on top of CE 4.2.13 [J. Ye, X. Li, S. Liu (Tencent)]

This contribution proposes an encoder optimization algorithm on top of CE 4.2.13. The proposed method reports 0.30% and 0.29% luma coding gain compare to CE 4.2.13 Test A in RA and LDB configuration on VTM with 1-2% encoding time increase.

It is claimed that about 0.15% coding gain if it is applied to VTM.

Recommendation: consider using this encoder optimization in CE on merge related tests, and in the anchors as well.

JVET-K0161 Spatial-temporal merge mode (non subblock STMVP) [T. Zhou, T. Ikai (Sharp)]

Note that this contribution number is invalid on JVET document website. And the contribution is registered again as JVET-K0532.

This contribution presents a spatial-temporal merge mode which is a simplified version of STMVP mode in JEM. It is asserted that this simplified merge mode produces more coding gain by courtesy of its optimized reference position while its’ no subblock feature is beneficial for hardware design. It is reported the bd-rate gain is 0.80 % and 0.28 % in RA and LB condition respectively.

Recommendation: study in CE.

JVET-K0511 Crosscheck of JVET-K0161: Spatial-temporal merge mode (non subblock STMVP) [G. Li (Tencent)] [late] [miss]

List construction

JVET-K0065 CE4 related: Candidate list reordering [L. Xu, F. Chen, S. Ye, L. Wang (Hikvision)]

This contribution proposes a candidate list reorder method for Merge and AMVP mode. Instead of using fixed candidate list construction order, the proposed method adopts different approaches of candidate list construction order for blocks with different shapes. Compared with BMS1.0, the results reportedly show that the proposed swapping method achieves 0.35%, 0.32% and 0.16% BD rate reduction for, LB, LP, and RA configurations, respectively. Compared with VTM1.0, it also reportedly shows that the proposed method achieves 0.32%, 0.32% and 0.13 % BD rate reduction for, LB, LP, and RA configurations respectively, while brings negligible increase of encoding and decoding complexity.

When constructing the list either for merge mode or for AMVP, the checking order of neighboring blocks is dependent on CU shape.

Recommendation: study this in CE.

JVET-K0417 Crosscheck of JVET-K0065: CE4 related: Candidate list reordering [J. Chen, K. Choi (Samsung)] [late]

JVET-K0304 CE4-related: Ranking based spatial merge candidate list for inter prediction [G. Li, X. Xu, X. Li, S. Liu (Tencent)]

In this contribution, it is proposed to replace the original spatial merge candidate list construction method with a ranking based method and the positions of merge candidates are extended to include more potential candidates. The proposed spatial candidates are derived and put at the beginning of the merge candidate list. The number of maximum merge candidates is kept unchanged. Other non-spatial merge candidates are added afterwards until the merge list is full. The proposed method was tested on BMS-1.0 reference software. It is reported that with VTM configuration, the result has 0.23%, 0.38%, and 0.51% BD-rate improvement for Random Access, Low Delay B, and Low Delay P, respectively.

It is proposed to sort spatial candidates in descending order of occurrence. The spatial candidates can be fetched from an L-shaped area around the current block. The existing spatial candidates are replaced by the candidates derived in this way.

For VTM test, 0.2%/0.4%/0.5% for RA/LDB/LDP configurations.

Recommendation: study this in CE.

JVET-K0493 Crosscheck of JVET-K0304 (CE4-related: ranking based spatial merge candidate list for inter prediction) [C.-M. Tsai (MediaTek)] [late] [miss]

JVET-K0364 Non-CE4: separate merge candidate list for sub-block modes [T. Fu, H. Chen, H. Yang, J. Chen (Huawei)]

Currently, several sub-block based motion vector prediction (MVP) methods has been proposed, e.g., ATMVP, STMVP, Affine Merge enhancement in JVET-J0024 and Planar MVP in JVET-J0061. All of these methods generate fine granularity motion fields for CUs. Because of the commonality among these methods, this contribution presents a separate merge candidate list for sub-block based MVPs. The candidate list is constructed by integrating two or more sub-block based MVPs into a separate merge candidate list.

Three tests in this contribution,

- Three merge list, one for VTM merge list with ATMVP, one for planar MV, one for affine candidates.

- Two merge list, one for VTM, one for affine candidates + planar MV + ATMVP.

- Two merge list, one for VTM merge list with ATMVP, one for affine candidates + planar MV.

It is commented that part of the coding gain could be obtained by more encoder RD checks.

It is commented that separate list can reduce the complexity of list construction at the decoder side.

Recommendation: study this in CE.

Buffer reduction

JVET-K0246 CE4.2-related: MV buffer reduction for non-adjacent spatial merge candidates [Y.-L. Hsiao, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes two motion vector (MV) buffer reduction methods for non-adjacent spatial merge candidates. The first method is to apply a coding tree unit (CTU) row constraint, where candidates belonging to above CTU rows and non-adjacent to the top boundary of the current CTU row are replaced with candidates belonging to above CTU rows and adjacent to the top boundary of the current CTU row. The second method is to apply spatial MV compression. Similar to the temporal MV compression in HEVC that only stores MV data of the top left 4x4 block in each 16x16 block of a reference picture, only the MV data of the bottom-right-biased center 4x4 block in each 16x16 block of the current picture are stored for future use. Results of adding non-adjacent spatial candidates same as those in CE4.2.15 with the proposed methods are summarized as follows.

Non-adjacent spatial merge candidates in CE4.2.15 & CTU row constraint & spatial MV compression
VTM-1.0-RA: {Y-BD-rate = -1.02%, EncT=106%, DecT=101%}
VTM-1.0-LB: {Y-BD-rate = -0.75%, EncT=107%, DecT=101%}
BMS-1.0-RA: {Y-BD-rate = -1.11%, EncT=104%, DecT=102%}
BMS-1.0-LB: {Y-BD-rate = -0.81%, EncT=102%, DecT=101%}

Two modifications are proposed, 1) using only the motion of the blocks to the top boundary of the current CTU row, 2) perform spatial MV compression in the coded area except for the adjacent 4x4 blocks of the current CU.

It is commented that the spatial MV compression may not reduce buffer in some architectures of chip design.

It is commented that the spatial MV compression may have interdependencies with affine MC in terms of coding efficiency.

It is suggested that for spatial MV compression, we may also test compressing only the coded area outside of the current CTU.

Recommendation: study in CE.

JVET-K0473 Crosscheck of JVET-K0246: MV buffer reduction for non-adjacent spatial merge candidates [J. Ye, X. Li (Tencent)] [late] [miss]

JVET-K0297 CE4-related: Reduce line buffer for additional merge candidates [J. Ye, X. Li, S. Liu (Tencent)]

This contribution is proposed to reduce line buffer for the additional spatial merge candidates. Two methods are proposed to reduce line buffer usage. These methods are implemented on top of CE 4.2.13 software. Method 1 is reported 0.01%, and 0.003% loss compared to the original CE 4.2.13 Test B in RA, and LDB configuration on VTM, and 0.00%, and 0.01% loss compared to the original CE 4.2.13 Test B in RA, and LDB configuration in BMS. Method 2 is reported 0.07%, and 0.08% loss compared to the original CE 4.2.13 Test B in RA, and LDB configuration on VTM and 0.06%, and 0.01% loss compared to the original CE 4.2.13 Test B in RA, and LDB configuration in BMS.

Two modifications are proposed, 1) using only the motion of the blocks to the top boundary of the current CTU row, 2) perform spatial MV compression in the above CTU row, but still keep the uncompressed motion of the blocks to the top boundary of the current CTU row.

Recommendation: study in CE.

JVET-K0427 Cross-check of JVET-K0297: CE4-related: Reduce line buffer for additional merge candidates [B. Choi (Sharp)] [late] [miss]

JVET-K0350 CE4-related: Improvement on Merge/Skip mode with line buffer restriction [Y. Han, Y.-H. Chao, W.-J. Chien, M. Karczewicz (Qualcomm)]

This contribution describes modification on Merge/Skip mode on top of the CE4 test 4.2.3 which adds additional non-adjacent spatial merge candidates and unified affine/normal merge candidate list. In order to reduce the line buffer size, the non-adjacent spatial merge candidates are restricted to be within the current CTU and its line buffer. For the “Tool on case 1 (VTM as reference)” tests in RA case, the test 1 “Non-adjacent spatial merge candidate candidates” achieved objective gain of 0.82% in terms of average luma BD-rate improvement. For the “Tool off case 1 (BMS as reference)” test in RA case, the test 2 “Test 1 + Unified merge candidate list” achieved 0.83% coding gain.

It is proposed to keep only the motion in the adjacent row or column of the current CTU in the coded area outside of the current CTU.

Recommendation: study in CE.

Motion vector coding (1)

JVET-K0302 CE4-related: Bilateral Motion Vector Prediction [B. Choi, F. Bossen, A. Segall (Sharp)]

This document proposes a modification of the AMVP mode. It is asserted that the proposed method exploits the linearity of motion vectors in high resolution sequences. In the proposal, the motion vector difference (MVD) and motion vector predictor (MVP) for list1 are derived from those of list0. This is asserted to have a benefit of saving the bits to explicitly represent MVD and MVP for list1. In addition, bits for ref_idx and ref_pred_idc are also saved by using the first reference picture and the bi-directional prediction only. The simulation results show the benefit of the approach, with a -1.19% coding gain in RA, relative to VTM1.0, with 111% encoder complexity.

In the symmetric bilateral MV prediction mode, the MV in list 1 is inferred by mirroring the MV in list 0. The ref_idx is set to 0.

In the asymmetric bilateral MV prediction mode, the MVD in list 1 is inferred by mirroring the MVD in list 0. Ref_idx in list 1 is set to 0. The AMVP index is signaled for list 1. In addition, adaptive motion vector resolution is applied for coding the MVD in list 0.

It is suggested to test each piece of the proposed mechanism separately in CE.

Recommendation: study in CE.

JVET-K0415 CE4-related Cross-check of JVET-K0302 [S. Jeong (??)] [late]

Reference picture boundary padding (1)

JVET-K0195 CE4-5 related: Inter/Intra Boundary Padding [J. Brandenburg, R. Skupin, H. Schwarz, D. Marpe, T. Schierl, T. Wiegand]

In HEVC, reference pictures are extended by a perpendicular padding of the picture boundary samples. CE4-5 investigates three new methods for boundary padding which rely either on inter-prediction based techniques (CE4-5.2 and CE4-5.3) or intra-prediction based techniques (CE4-5.1).

This document proposes a harmonized boundary padding approach using inter-prediction based per-picture techniques in combination with an intra-prediction based per-reference technique. When the amount of motion compensated samples in the padded area of a reference block falls below a threshold, intra-prediction based boundary padding is applied.

Reported experimental results show BD-rate gains in RA and LD configurations. For VTM RA overall (Class A1 to C), -0,24% YUV-BD-rate gain at 109% combined Enc/Dec runtime are reported. For small video resolutions such as Class D, YUV-BD-rate gains of -0.61% at 125% combined Enc/Dec runtime for RA are reported.

64 samples outside the picture boundary is padded using the proposed combined method.

It is commented that the coding performance may depend on the area padded.

Recommendation: study in CE.