| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0552-v1 |
| --- | --- |

| Title: | BoG on HDR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. François | Tel: Email: | edouard.francois@technicolor.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

The BoG met July 15 to review CE12 and other HDR video related contributions and topics identified in the AHG-7 report.

The BoG recommends the following:

- reconducting the CE on HDR mapping, to further investigate the out-of-loop and in-loop mapping and the luma/chroma rate allocation and balance

- creating a CE for investigating mapping for SDR content (based on JVET-K0309, JVET-K0468)

- updating the HDR CTCs as follows:

  - For tests not requiring visual assessment (fixed QP case), consider the first 300 frames of the following HLG content: DayStreet, PeopleInShoppingCenter, SunsetBeach, the initial version of FlyingBirds sequence provided by NHK

  - For tests requiring visual assessment (fixed bitrate case), consider the full 600 frames of the following HLG content: DayStreet, PeopleInShoppingCenter, SunsetBeach

  - Mandating usage of HDRTools to report HDR metrics (including wPSNR)

- reporting performance comparisons of VTM and BMS against the HM for HDR content, as done for the SDR case

# Contribution review

## CE12 report – JVET-K0032

This contribution provides a summary report of Core Experiment 12 on mapping for HDR content. CE12 aims at evaluating approaches for mapping of HDR content. The considered technologies are adaptive quantization, out-of-loop reshaping and in-loop reshaping. Test results against VTM anchor are provided for each performed test. Crosschecking reports are integrated in this contribution.

VTM was used as anchor, with luma-based QP adaptation activated (macro SHARP_LUMA_DELTA_QP set to 1), chroma QP adaptation (macro ER_CHROMA_QP_WCG_PPS set to 1), and wPSNR-based RDO (macro WCG_EXT set to 1).

The list of tools tested in the out-of-loop and in-loop mapping approaches is summarized in the table below.

| Tool out-of-loop mapping | Acronym | Description |
| --- | --- | --- |
| Dynamic range adaptation | DRA | consists in applying a luma mapping and luma-based chroma scaling to the input video luma and chroma components, as a pre-encoding process, and to apply to inverse process as a post-decoding process |
| Harmonization of DRA chroma scale and QP | qpHarmoniz | consists in adjusting the luma-based chroma scaling to preserve a luma/chroma bit allocation ensured by the luma-based chroma QP offset derivation in the core HEVC design (cf Table 8-10 of HEVC specification) |
| DRA refinement | Refint_Flt / Refint_Fix | consists in applying a correction to the inverse DRA tables applying at the decoder to compensate the distortions resulting from the compression. The process may apply per slice. A floating-point was first delivered. A fixed-point version was further delivered. |

| Tool in-loop mapping | Acronym | Description |
| --- | --- | --- |
| In-loop Luma reshaper | Reshap | maps an input luma code value to an altered value inside the coding loop. For intra slice, reshaping is explicitly performed to the input signal. Inverse reshaping applies after loop filtering and before DPB buffering. For an inter slice, only inverse reshaping applies to the prediction signal before loop filtering. |
| luma-based Chroma QP Offset | chrQpOffset | Adds a luma-dependent QP offset to chroma signal, to compensate the Luma signal interaction with the Chroma signal |
| luma-based Chroma scaling | chrScaling | luma-based Chroma QP Offset is alternatively implemented by scaling the chroma residue signal in each TU |
| Loop Filter application to original domain | LF intraOrig or LF interOrig | Intra: LF is applied in the original domain, after applying the inverse reshaping Inter: LF is applied in the original domain. |
| Loop Filter application to reshaped domain | LF intraResh or LF interResh | Intra: LF is applied in the reshaped domain, before applying the inverse reshaping Inter: LF is applied in the reshaped domain. A pair of forward and inverse reshaping is needed before and after LF. |
| Region of Interest | ROI | Adaptively turns on/off the reshaping of a CTU based on the average luma value in the CTU |

The summary of tests performance is provided in the table below.

|  |  | AI - Over VTM1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | DE100 | PSNRL | wPsnrY | wPsnrU | wPsnrV |
| out-of-loop |  |  |  |  |  |  |
| CE12-1 | DRA + RefintFlt | -18.8% | -0.7% | 1.7% | -11.6% | -41.1% |
| CE12-3.1 | DRA + RefintFlt + qpHarmoniz | -11.7% | -3.1% | -0.8% | -11.3% | -28.8% |
| CE12-3.2 | DRA + RefintFix + qpHarmoniz | -11.7% | -3.1% | -0.8% | -11.4% | -28.8% |
| CE12-3.3 | DRA + qpHarmoniz | -8.0% | -3.1% | -0.7% | -0.2% | -14.7% |
| in-loop |  |  |  |  |  |  |
| CE12-5 | Reshap | 13.7% | -2.8% | -2.1% | 12.3% | 13.0% |
| CE12-6.1 | Reshap + chrQpOffset | -1.5% | -2.8% | -2.0% | -1.9% | -4.8% |
| CE12-6.2 | Reshap + chrScaling | 0.5% | -2.8% | -2.1% | 2.0% | 2.6% |
| CE12-7.1 | Reshap + LF_intraOrig_interOrig | -1.4% | -2.1% | -1.5% | -1.9% | -4.7% |
| CE12-7.2 | Reshap + LF_intraResh_interOrig | -1.5% | -2.8% | -2.0% | -1.9% | -4.8% |
| CE12-7.3 | Reshap + LF_intraResh_interResh |  |  |  |  |  |
| CE12-8 | Reshap + ROI |  |  |  |  |  |

Note: CE12-6.1, CE12-7.2 and CE12-7.3 are identical in AI configuration.

|  |  | RA - Over VTM1.0 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | DE100 | PSNRL | wPsnrY | wPsnrU | wPsnrV |
| out-of-loop |  |  |  |  |  |  |
| CE12-1 | DRA + RefintFlt | 10.6% | -6.2% | -4.0% | 35.1% | 212.0% |
| CE12-3.1 | DRA + RefintFlt + qpHarmoniz | 10.3% | -6.6% | -4.4% | 34.5% | 68.5% |
| CE12-3.2 | DRA + RefintFix + qpHarmoniz | 10.4% | -6.6% | -4.4% | 34.5% | 68.4% |
| CE12-3.3 | DRA + qpHarmoniz | 15.6% | -6.5% | -4.3% | 70.5% | 118.1% |
| in-loop |  |  |  |  |  |  |
| CE12-5 | Reshap | 11.0% | -2.6% | -2.0% | 12.0% | 10.3% |
| CE12-6.1 | Reshap + chrQpOffset | -1.3% | -2.6% | -2.0% | 0.0% | -0.1% |
| CE12-6.2 | Reshap + chrScaling | 2.0% | -2.6% | -2.0% | 5.9% | 8.9% |
| CE12-7.1 | Reshap + LF_intraOrig_interOrig | -1.1% | -1.7% | -1.3% | 0.1% | 0.0% |
| CE12-7.2 | Reshap + LF_intraResh_interOrig | -1.4% | -2.4% | -1.8% | -0.1% | -0.1% |
| CE12-7.3 | Reshap + LF_intraResh_interResh | -1.3% | -2.6% | -2.0% | 0.0% | -0.1% |
| CE12-8 | Reshap + ROI | -1.2% | -2.2% | -1.7% | 0.1% | -0.1% |

Note: CE12-6.1 and CE12-7.3 are identical in RA configuration.

Discussion

Benefits from mapping were observed in both out-of-loop and in-loop approaches.

Recommendations from proponents of CE12 was to adopt a mapping solution.

It was observed that different luma/chroma allocation was used in the reported tests for these two approaches.

It was also commented that, as said in CE report recommendations, it is needed to further investigate in-loop or out-of-loop mapping before adoption.

The BoG recommends to reconduct the CE on HDR mapping, with the following topics to consider:

- Investigate luma/chroma rate allocation and balance

- Further investigate and differentiate pros and cons of out-of-loop and in-loop mapping

## CE contributions

JVET-K0298 CE12: report of dynamic range adaptation (DRA) and DRA refinement [E. Francois (Technicolor), D. Rusanovskyy (Qualcomm)]

Discussed as part of the CE review.

JVET-K0308 CE12: HDR In-loop Reshaping (CE12-5, 12-6, 12-7 and 12-8) [. Lu, F. Pu, P. Yin, W. Husak, S. McCarthy, T. Chen (Dolby)]

Discussed as part of the CE review.

JVET-K0392 Cross-check for CE12.6.1 and CE12.6.2 [J. Zhao, K. Misra (Sharp)]

Discussed as part of the CE review.

## CE-related contributions

JVET-K0309 In-loop Reshaping for SDR Video [F. Pu, T. Lu, P. Yin, W. Husak, S. McCarthy, T. Chen (Dolby)]

In-loop reshaping is proposed for HDR video in JVET-J0015 and is further tested in JVET-K00xx for CE12: Mapping for HDR content. This contribution proposes to use the same in-loop reshaping coding tool for SDR video. Compared to VTM SDR Anchor, the contribution shows the following BD-rate for UHD and HD clips under CTC:

- AI {Y, U, V}:

Class A1:	{-1.50%, 0.56%, 0.43%};

Class A2:	{-1.37%, 3.14%, 3.43%};

Class B:	{-0.76%, 0.83%, 0.04%}

Enc time: 105%, Dec time: 104%

- RA {Y, U, V}:

Class A1:	{-1.58%, 2.49%, 2.49%};

Class A2:	{-1.28%, 2.47%, 2.57%};

Class B:	{-1.09%, 0.89%, -0.27%}

Enc time: 110%, Dec time: 103%

Discussion

Reshaping function is computed for the first intra picture, based on the analysis of the pixel variance (computed within 5x5 window). The proposed syntax allows the encoder to select at CTU level the activation of the mapping.

It was commented that for HDR, mapping was so far only considered at slice level, while in this proposal the mapping is theoretically (considering the proposed syntax) applicable at CTU-level. But in actual reported tests, the mapping activation was done at the slice level. More investigations for justifying the need of CTU-level adaptation seem to be needed.

It was commented that testing the approach also for HLG content would be interesting. In that case, the proponent suggests to only use YUV-PSNR for HLG, and not consider the other HDR-related metrics.

The proponent suggests to also investigate mapping for SDR content in a CE.

The BoG recommends creating a CE for investigating mapping for SDR content.

JVET-K0468 CE12-related: In-loop chroma refinement [E. François, C. Chevance, F. Hiron (Technicolor)]

Out-of-loop mapping function refinement was proposed for HDR video in JVET-J0022 and JVET-K0298. In this contribution, the technology is evaluated for chroma components when applied in-loop. The reported BD-rate variations in VTM1.0 for RA configuration are as follows. For HLG content, -4.26% for DE100, 0.06% for PSNRL100, 0.06%, -7.49% and -9.90% for wPSNR-Y, U and V. For PQ content, -1.61% for DE100, 0.05% for PSNRL100, 0.05%, -6.94% and -9.89% for wPSNR-Y, U and V. Encoder and decoder runtimes are both evaluated at 101% of the VTM1.0 runtimes.

Discussion

It was suggested to also consider this technology in CE for HDR and SDR content.

The BoG recommends investigating the in-loop chroma refinement (JVET-K0468) in CE on HDR mapping, and in the (potential) CE related to mapping for SDR content.

# HDR CTC – HLG sequences length reduction

After AhG discussions, The group had the mandate to study and evaluate available HDR test content, with one goal being the reduction of the number of frames used for testing in the HLG sequences.

In AhG 7 discussions prior to the JVET meeting, the adhoc group had tentatively made the decision to reduce the length of HLG sequences to 300 frames. However, for the SDR case, the group tends to consider that 10s sequences are required for properly perform visual tests.

BoG suggests updating the HDR CTC by considering 2 cases:

- For tests not requiring visual assessment (fixed QP case), consider the first 300-frames of the following HLG content

  - DayStreet, PeopleInShoppingCenter, SunsetBeach, first 300 frames

  - The initial version of FlyingBirds sequence provided by NHK, which was 300-frames length, and had less blurring issues than the 600-frames one, is recommended to be used

- For tests requiring visual assessment (fixed bitrate case), consider 600-frames length HLG content

  - DayStreet, PeopleInShoppingCenter, SunsetBeach

BoG recommends reporting performance comparisons of VTM and BMS against the HM for HDR content, as done for the SDR case.

BoG recommends the clearly specify in the CTCs that the reported HDR metrics including wPSNR shall be derived from HDRMetrics.