| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0554-v1 |
| --- | --- |

| Title: | CE1-related: Joint proposal for picture boundary partitioning by Fraunhofer HHI and Huawei |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Adam Wieckowski, Jackie Ma, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Han Gao, Semih Esenlik, Jianle Chen | Tel: Email: | adam.wieckowski@hhi.fraunhofer.de han.gao@huwei.com |
| Source: | Fraunhofer HHI, Huawei |  |  |

_____________________________

# Abstract

The BoG on picture boundary partitioning agreed to recommend a picture boundary partitioning algorithm based on a solution proposed in CE1-2.0.5. A fix to this solution was provided by Huawei. The BoG agreed that the proposed combination is recommended to the JVET. This document provides a preliminary draft text for the combined solution.

The proposed fix deals with an issue that does not occur in the CTC. The results reported in the CE1 experiment remain valid.

# Method description

The picture boundary partitioning proposed in CE1-2.0.5 allows to terminate the QT tree before the picture boundary is exactly matched by partitioning boundaries, in only a single picture boundary is exceeded. In such a case, an implicit BT split parallel to the exceeded picture boundary is used to match the picture boundary.

The fix provided by Huawei ensures that no QT split is allowed if the block size is smaller or equal to minimal QT size. In such a case, implicit BT partitioning is used to match picture boundary. If the current block exceeds over the bottom picture boundary, an implicit horizontal split is used. Otherwise, if a block exceeds over the right picture boundary, a vertical split is used.

The attached draft text was agreed on as a BoG recommendation.

# Results

As minQtSize constraint does not need to be violated in the CTC for the boundary handling, the results are the same as reported in the CE1-2.0.5.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co.,Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).