| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0555-v1 |
| --- | --- |

| Title: | Cross-check of JVET-K0059: CE3-related: Reference sample processing for wide-angle intra-prediction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Jonathan Pfaff, Michael Schäfer Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-188, jonathan.pfaff@hhi.fraunhofer.de |
| Source: | Fraunhofer-HHI |  |  |

_____________________________

# Abstract

This contribution is a cross-check report of JVET-K0059 [1]. It is confirmed that the cross-check results are identical with the proponent’s results.

# Description of the method to be crosschecked

The tool described in [1] modifies the method of wide-angle intra prediction in two ways. First (Test1), more reference samples are used for the wide-angle intra prediction. Second, a smoothing of the reference samples for the wide-angle intra prediction is applied. In a second test (Test 2), also the availability of extended reference samples is checked in order to decide whether wide-angle intra prediction is applicable or not.

# Results

According to the common test conditions in JVET-J1010 [2], the method of [1], split into Test 1 and Test 2, was evaluated for the intra-only (AI) and random-access (RA) configurations with the BMS-1.0 software in VTM and BMS configuration. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and GCC 7.2.1 compiler. The results are reported in the tables below. The slight increase in the decoder runtime reported in the tables below in comparison to the decoder runtime reported in [1] is due to measurement inconsistencies on the cluster of the crosschecker. Some random local decoder runs by the testers confirmed the decoder runtimes reported in [1]

Table 1: Test 1, Results for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,28% | -0,19% | -0,32% | 112% | 109% |
| Class A2 | -0,30% | -0,34% | -0,35% | 110% | 107% |
| Class B | -0,33% | -0,56% | -0,51% | 110% | 108% |
| Class C | -0,34% | -0,54% | -0,63% | 110% | 106% |
| Class E | -0,53% | -0,59% | -0,49% | 111% | 104% |
| Overall | -0,35% | -0,46% | -0,47% | 111% | 107% |
| Class D | -0,35% | -0,55% | -0,62% | 111% | 108% |

Table 2: Test 1, Results for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,22% | -0,23% | -0,28% | 104% | 101% |
| Class A2 | -0,20% | -0,44% | -0,29% | 104% | 101% |
| Class B | -0,20% | -0,36% | -0,52% | 106% | 106% |
| Class C | -0,19% | -0,34% | -0,58% | 105% | 105% |
| Class E |  |  |  |  |  |
| Overall | -0,20% | -0,35% | -0,44% | 105% | 104% |
| Class D | -0,17% | -0,19% | -0,41% | 105% | 104% |

Table 3: Test 1, Results for BMS AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,17% | -0,01% | -0,20% | 104% | 109% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0,26% | -0,34% | -0,39% | 104% | 106% |
| Class C | -0,26% | -0,26% | -0,32% | 104% | 106% |
| Class E | -0,38% | -0,48% | -0,47% | 104% | 111% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0,26% | -0,31% | -0,31% | 104% | 109% |

Table 4: Test 1, Results for BMS RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | -0,18% | -0,33% | -0,27% | 104% | 107% |
| Class C | -0,12% | -0,13% | -0,41% | 105% | 109% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0,10% | -0,40% | -0,41% | 105% | 113% |

Table 5: Test 2, Results for VTM AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,28% | -0,26% | -0,36% | 110% | 104% |
| Class A2 | -0,30% | -0,37% | -0,34% | 108% | 102% |
| Class B | -0,33% | -0,52% | -0,50% | 108% | 101% |
| Class C | -0,36% | -0,51% | -0,71% | 107% | 103% |
| Class E | -0,52% | -0,58% | -0,43% | 107% | 102% |
| Overall | -0,36% | -0,46% | -0,48% | 108% | 102% |
| Class D | -0,36% | -0,51% | -0,73% | 109% | 102% |

Table 6: Test 2, Result for VTM RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,23% | -0,37% | -0,35% | 106% | 107% |
| Class A2 | -0,21% | -0,26% | -0,26% | 106% | 105% |
| Class B | -0,21% | -0,37% | -0,53% | 107% | 110% |
| Class C | -0,22% | -0,34% | -0,49% | 106% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0,22% | -0,34% | -0,43% | 107% | 106% |
| Class D | -0,19% | -0,48% | -0,73% | 115% | 107% |

Table 7: Test 2, Results for BMS AI configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,18% | -0,05% | -0,19% | 101% | 108% |
| Class A2 | -0,22% | -0,25% | -0,25% | 100% | 107% |
| Class B | -0,27% | -0,36% | -0,37% | 100% | 104% |
| Class C | -0,26% | -0,26% | -0,23% | 100% | 103% |
| Class E | -0,39% | -0,46% | -0,56% | 100% | 107% |
| Overall | -0,26% | -0,29% | -0,32% | 100% | 106% |
| Class D | -0,25% | -0,27% | -0,35% | 101% | 105% |

Table 8: Test 2, Results for BMS RA configuration

|  | Y | U | V | enc time | dec time |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,16% | -0,20% | -0,21% | 103% | 112% |
| Class A2 | -0,14% | -0,12% | -0,09% | 105% | 114% |
| Class B | -0,18% | -0,33% | -0,33% | 104% | 109% |
| Class C | -0,12% | -0,01% | -0,42% | 105% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0,15% | -0,17% | -0,28% | 104% | 110% |
| Class D | -0,11% | -0,18% | -0,15% | 104% | 112% |

# References

- A. Filippov et al., “CE3-related: Reference sample processing for wide-angle intra-prediction,” JVET-K0059, JVET 11th Meeting: Ljubljana, SI, 10–18 July 2018

- J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” JVET-J1010, San Diego, USA, April 2018.