| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0556-v2 |
| --- | --- |

| Title: | CE1-related: Constraint for binary and ternary partitions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {cw.hsu, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Virtual pipeline data units (VPDUs) are defined as non-overlapping MxM-luma(L)/NxN-chroma(C) units in a picture. In hardware decoders, successive VPDUs are processed by multiple pipeline stages at the same time; different stages process different VPDUs simultaneously. The VPDU size is roughly proportional to the buffer size in most pipeline stages, so it is very important to keep the VPDU size small. In HEVC hardware decoders, the VPDU size is set to maximum transform block (TB) size. The VPDU size is roughly proportional to the buffer size in most pipeline stages, so it is very important to keep the VPDU size small. Enlarging maximum TB size from 32x32-L/16x16-C (as in HEVC) to 64x64-L/32x32-C (as in the current VVC) can bring coding gains, which results in 4X of VPDU size (64x64-L/32x32-C) expectedly in comparison with HEVC. However, in addition to quadtree (QT) coding unit (CU) partitioning, ternary tree (TT) and binary tree (BT) are adopted in VVC for achieving additional coding gains, and TT and BT splits can be applied to 128x128-L/64x64-C coding tree blocks (CTUs) recursively, which leads to 16X of VPDU size (128x128-L/64x64-C) in comparison with HEVC. To reduce the VPDU size in VVC, one constraint for TT and BT is proposed, and the VPDU size is defined as 64x64-L/32x32-C for the following.

Cond. 1:  For each VPDU containing one or multiple CUs, the CUs are completely contained in the VPDU.
Cond. 2:  For each CU containing one or more VPDUs, the VPDUs are completely contained in the CU.
Proposed constraint:  For each CTU, the above two conditions shall not be violated, and the processing order of CUs shall not leave a VPDU and re-visit it later.

# Introduction

Figure 1 shows examples of TT and BT splits that cause problems for 64x64-L/32x32-C pipelining.

Figure 1.  Examples of undesirable TT and BT splits for 64x64-L/32x32-C pipelining (VPDUs indicated by dashed lines)

# Proposed method

We define VPDUs as non-overlapping units of 64x64-L/32x32-C samples in the picture. The following constraint is proposed for TT and BT.

Cond. 1:  For each VPDU containing one or multiple CUs, the CUs are completely contained in the VPDU.
Cond. 2:  For each CU containing one or more VPDUs, the VPDUs are completely contained in the CU.
Proposed constraint:  For each CTU, the above two conditions shall not be violated, and the processing order of CUs shall not leave a VPDU and re-visit it later.

Examples in Figure 1 are disallowed by the two constraints. Figure 2 shows some examples that are allowed.

Figure 2.  Examples of allowed TT and BT splits for 64x64-L/32x32-C pipelining (VPDUs indicated by dashed lines)

# Conclusions

One constraint is proposed for TT and BT to allow 64x64-L/32x32-C pipelining to be used in VVC hardware decoders.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).