| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0559-v1 |
| --- | --- |

| Title: | Report of BoG on Picture Boundary Partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Kiran Misra | Email: | misrak@sharplabs.com |
| Source: | BoG |  |  |

_____________________________

# Abstract

This is a draft report of the BoG on Picture Boundary Partitioning.

The BoG met on July 12, 2018, July 13, 2018 and July 15, 2018 to further study the different picture boundary partitioning proposals and suggest the most viable solution that has a reasonable tradeoff between compression and imposing very specific dependencies of the syntax decoding from the position in the picture.

Recommendations:

- Further study of JVET-K0366, JVET-K0320, CE1-2.0.10 in a CE

- CE1-2.0.5 with fix for minQTSize constraint being obeyed be included in VTM/BMS and VVC draft. Draft text is provided in JVET-K0554.

# General discussion:

A strawman categorization template was shared by the BoG chair.

A participant suggested following additional categorization criteria:

- Any additional syntax added w.r.t. VTM

- Modification to regular CU inside picture boundary

- Modification to decoding processing individual CU

- MaxBTSize, MinBTSize etc. is there any change

It was noted that point 2 is related to signaling split structure. The categorization template was refined based on this input to:

Categories (column headings in summary excel, note “LPD” means Luma Position Dependent):

| Category | Description |
| --- | --- |
| LPD: Implicit Splits | Does the proposal make use of implicit splits e.g. BT split direction? |
| LPD: Additional syntax | Are any additional syntax elements used for the proposal with respect to VTM? |
| LPD: Syntax constraints used for splits | Do split syntax element values have semantic/decoding changes w.r.t. VTM e.g. split_flag 0 implies a different split type instead of NO_SPLIT |
| LPD: Changes to context selection | Is the context selection process changed with respect to VTM? |
| LPD: Changes to decoding/parsing process of CU besides split flag | Changes to decoding/parsing process of CU besides split flag? e.g. inference of cbf_flag value |
| LPD: Distance computation to boundary | The decoding process is dependent on distance from current CU edge to picture boundary edge. |
| LPD: Change in MinQTSize | Is the MinQTSize parameter value (signaled in SPS header) based constraint violated? |
| LPD: Change in MaxBTDepth for CTU at picture boundary | Is the MaxBTDepth parameter value (signaled in SPS header) based constraint violated for CTU at picture boundary? |
| HLS | Are new high level syntax elements added? |

On July 13, 2018, the following additional categorization criteria were added:

- Luma gain

- Number of checks in boundary CTU (worst-case)

- Data (if any) that needs to be stored for boundary CTU (worst-case)

- Operation count in boundary CTU (worst-case)

# Discussion

In addition to the summary included in the spreadsheet table, some additional points were captured when discussing some of the contributions, which are noted below.

CE1-2.0.3, CE1-2.0.4, CE1-2.0.8 were withdrawn from consideration.

## JVET-K0280 CE1-2.0.5

Proponent asserted that BTdepth of splits used at picture boundary should not be counted. If counted then the BT depth may exceed MaxBTDepth signaled in SPS. It was further asserted that this is a necessary thing to support multiple of 4 number of samples inside the picture boundary for a CTU.

CE1-2.0.10 (JVET-K0224), CE1-2.0.12 (JVET-K0111) and CE1-2.0.11 (JVET-K0287) proponents asserted it is not needed to increase MaxBTDepth or reducing  MaxBTSize to support multiple of 4 number of samples inside the picture boundary for a CTU.

After further discussion it was noted that:

- In CE1-2.0.10 (JVET-K0224), the change may be needed for CU outside picture boundary

- In CE1-2.0.12 (JVET-K0111), the change is not needed

- In CE1-2.0.11 (JVET-K0287), the change may be needed for implicit splits.

It was further noted that CE1-2.0.5 based (i.e. CE1-2.0.1 to CE-2.0.9, JVET-K0366) keeps the same MaxBTDepth value based binarization by maintaining a count of implicit splits.

It was noted that CE1-2.0.10 does not count implicit splits and uses the same MaxBTDepth for binarization.

## JVET-K0376 CE1-2.0.9: Picture Boundary Handling

Same as CE1-2.0.5

## JVET-K0287 CE1-2.0.11: Picture Boundary Handling

The proponent commented that the semantic of NO_SPLIT is changed for boundary CU’s but the decision tree (i.e. partitions resulting) from QT/BT/TT is not changed.

Implicit splits after NO_SPLIT. It was noted that for the proposal the same partitioning may be achieved using two or more type of signaling. A participant noted that at this meeting it was preliminary agreed to allow such behavior in the VTM.

The crosschecker commented that the check of being inside picture boundary was implemented within the partitioning structure. The proponent noted that this check already exists in VTM but at the QT split level.

Supplemental information was provided with change to implicit splitting that does not cause splitting beyond MinQTSize. This aspect was crosschecked (JVET-K0418).

The implicit splitting part may not obey the constraint placed by MaxBTDepth.

## JVET-K0111 CE1: Picture Boundary Split in JVET-J0024 (Test 2.0.12)

It was commented that with implicit QT splits one is effectively reducing MaxBTSize. However this does not mean the proposal exceeds MaxBTSize. It was noted by the cross checker that the proposal had a loss of 0.4% for class D.

## JVET-K0317 CE1: Implicit QT and BT Partitions on Picture Boundary (Test 2.0.14)

It was commented that even not signaling syntax element implies a syntax constraint and was noted as such in the summary table.

## JVET-K0320 CE1-related: Zero-Unit for Picture Boundary Handling

Crosscheck ongoing. Crosschecker confirmed all aspects of summary table except the “MaxBTDepth” aspect. The proposal redefines QT, BT and TT split derivation for non-power of 2 blocks (e.g. at picture boundary).

## JVET-K0209 CE1: tests 2.0.1 and 2.0.2

CE1-test 2.0.1 was reviewed.

CE1-test 2.0.2 was withdrawn from consideration.

## JVET-K0224 CE1.2.0.10: CU partitioning along picture boundaries

The proponent noted that the change in minQTSize is only for bottom-right CU. A participant commented that to support multiple of 4 number of samples inside the picture boundary for a CTU; the proposal will generate QT splits whose value is always known (redundant syntax). It was further noted that an evil encoder can generate a bitstream that exceeds the MaxBTDepth.

It was agreed by the group to have a design that does not allow invalid bitstream.

It was further clarified by the proponent that the MaxBTDepth needs to be exceeded for CU outside picture boundary but this was not needed for CU’s inside the picture boundary.

It was commented by proponent of JVET-K0366 that by modifying their proposal one may be able to address the concern of exceeding MaxBTDepth.

## JVET-K0316 CE1: Implicit QT,BT and MTT Partitions on Picture Boundary (Test 2.0.15)

Same as Test 2.0.14 but makes use of TT if its partitioning aligns with picture boundary.

## JVET-K0366 CE1-related: Partial CU for picture boundary

For boundary CU’s at leaf of a partition tree has a valid and invalid part. No residue is sent for the valid part only when valid part has a non-power of 2 dimension.

Crosscheck still in progress. The proponent noted that the increase in MaxBTDepth can be addressed by further modifications to their proposal.

## JVET-K0280 CE1-2.0.7

Document reviewed. Withdrawn from consideration.

# Other discussion

It was asserted by a participant that adding new partition types to just handle picture boundary is not desirable. The group agreed with this assertion.

A participant suggested that JVET-K0320 introduces new partition type. The proponent disagreed with this assertion. A participant noted that the parsing process was changed (i.e. cbf_flag).

After a first pass the group considered if we need new categories. The following new categories were agreed with the data for these categories to be added offline by proponents:

- Luma gain

Analysis:

- Number of checks in boundary CTU (worst-case)

- Data (if any) that needs to be stored for boundary CTU (worst-case)

- Operation count in boundary CTU (worst-case)

It was agreed to compute the above data for VTM as well.

It was initially agreed that the analysis be provided for the following cases:

(a) 128x56 samples inside picture boundary for a 128x128 CTU

(b) 56x56 samples inside picture boundary for a 128x128 CTU.

After some further discussion it was agreed that just computing the number of checks for the worst-case parsing path should be sufficient.

A participant requested analysis for 124x124 samples inside picture boundary for a 128x128 CTU. It was agreed to provide the initial analysis for cases (a) and (b) listed above.

The BoG agreed that the technology described in JVET-K0320 would require further study.

The proponents noted that the proposals CE1-2.0.2 and CE1.2.0.7 were out-of-scope for the BoG and agreed that these proposals do not need further consideration in the BoG.

A participant commented that the proponents of: CE1-2.0.5, CE1-2.0.10, CE1-2.0.12, CE1-2.0.14, JVET-K0366, CE1-2.0.1, CE1-2.0.11, CE1-2.0.15 should provide rough syntax tables and semantics. The group agreed to this suggestion.

The VVC check count analysis was reviewed and agreed to by the participants.

A participant requested the check count analysis be shared. It was commented that it may be desirable that all participants share the data. It was agreed that participants would provide their analysis which would be uploaded as part of the BoG report data analysis.

For CE1-2.0.11 analysis multiple participants commented that the leaf node may need to counted for the worst-case.

The group agreed that CE1-2.0.14 and CE1-2.0.15 needs further study.

It was commented by a participant that number of checks may not reflect complexity sufficiently and one may need to also count the number of syntax elements parsed (since it would add context processing, entropy decoding of bins etc.).

It was noted by proponents of CE1-2.0.5 that the minQTSize constraint violation can be addressed by a modification. The modification was crosschecked by Tencent and Huawei. The proponents agreed to upload the modification as part of their contribution. It was noted that this is not an issue for CTC.

It was noted that CE1-2.0.12 makes use of implicit splits only at the picture boundary which leads to a smaller gain of 0.14%.

The group agreed that CE1-2.0.12 needs further study.

It was commented by a participant that changing the coding unit parsing for picture boundary may not be desirable which is done for the case of JVET-K0366. The proponent asserted that this proposal can handle complicated picture boundary cases with lesser constraints.

The BoG recommends further study of JVET-K0366, JVET-K0320, CE1-2.0.10 in a CE.

Recommendation: Further study of JVET-K0366, JVET-K0320, CE1-2.0.10 in a CE

The BoG recommends CE1-2.0.5 with fix for minQTSize constraint being obeyed be adopted. Draft text is provided in JVET-K0554.

Recommendation: CE1-2.0.5 with fix for minQTSize constraint being obeyed be included in VTM/BMS and VVC draft. Draft text is provided in JVET-K0554.

The BoG reviewed the fix for minQTSize constraint for CE1-2.0.5.

The proponent of CE1-2.0.10 (JVET-K0224) commented that the minQTSize fix was also proposed in their contribution. Multiple participants commented that the software in CE1-2.0.10 does not obey the constraint for minQTSize.

A participant commented that it may not be desirable to change the MaxBTDepth inside the picture boundary.