#### 7.3.4.3	Coding quadtree syntax

s

| coding_quadtree( x0, y0, log2CbSize, cqtDepth ) { | Descriptor | Chccks |
| --- | --- | --- |
| if( ( x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples ¦¦ && y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples ) && log2CbSize > MinQtLog2SizeY ) |  | 3 |
| qt_split_cu_flag[ x0 ][ y0 ] | ae(v) | 2 |
| } |  | . |
| if( qt_split_cu_flag[ x0 ][ y0 ] ) { |  | 1 |
| x1 = x0 + ( 1 << ( log2CbSize − 1 ) ) |  | . |
| y1 = y0 + ( 1 << ( log2CbSize − 1 ) ) |  | . |
| coding_quadtree( x0, y0, log2CbSize − 1, cqtDepth + 1 ) |  | . |
| if( x1 < pic_width_in_luma_samples ) |  | 1 |
| coding_quadtree( x1, y0, log2CbSize − 1, cqtDepth + 1 ) |  | . |
| if( y1 < pic_height_in_luma_samples ) |  | 1 |
| coding_quadtree( x0, y1, log2CbSize − 1, cqtDepth + 1 ) |  | . |
| if( x1 < pic_width_in_luma_samples && y1 < pic_height_in_luma_samples ) |  | 1 |
| coding_quadtree( x1, y1, log2CbSize − 1, cqtDepth + 1 ) |  | . |
| } else |  | . |
| multi_type_tree( x0, y0, 1 << log2CbSize, 1 << log2CbSize, 0 , 0 ) |  | 37 + 5 |
| } |  | . |
| CHECKS (WORST CASE) |  | 42+5 |

* See analysis from 7.3.4.4

#### Multi-type tree syntax

| multi_type_tree( x0, y0, cbWidth, cbHeight, mttDepth, partIdx ) { | Descriptor | Checks |
| --- | --- | --- |
| % Determine allowSplitBtVer The variable allowBtSplit is derived as follows: - If one or more of the following conditions are true, allowBtSplit is set equal to FALSE: - 1. cbSize is less than or equal to MinBtSizeY - 2. cbWidth is greater than MaxBtSizeY - 3. cbHeight is greater than MaxBtSizeY - 4. mttDepth is greater than or equal to MaxMttDepth Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE: - 5. mttDepth is greater than 0 - 6. partIdx is equal to 1 - 7. MttSplitMode[ x0 ][ y0 ][ mttDepth − 1 ] is equal to parallelTtSplit Otherwise if the following condition is true, allowBtSplit is set equal tot FALSE - P. x0 + ( 1 << log2CbSize ) > pic_width_in_luma_samples - Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE: - 8. slice_type is not equal to I - 9 mttDepth is equal to 1 - 10. cbSize is greater than 1 << MinQtLog2SizeY - 11. partIdx is equal to 1 - 12. MttSplitMode[ x0 ][ y0 ][ mttDepth − 1 ] is equal to perpendicularBtSplit - 13. MttSplitMode[ x0 − partOffsetX ][ y0 − partOffsetY ][ mttDepth ] is equal to btSplit Note: The above statements were removed in the Ljubljana meeting – Otherwise, allowBtSplit is set equal to TRUE. |  | 7 |
| % Determing allowSplitBtHor The variable allowBtSplit is derived as follows: - If one or more of the following conditions are true, allowBtSplit is set equal to FALSE: - 1. cbSize is less than or equal to MinBtSizeY - P. cbWidth is greater than MaxBtSizeY - P. cbHeight is greater than MaxBtSizeY - P. mttDepth is greater than or equal to MaxMttDepth Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE: - P. mttDepth is greater than 0 - P. partIdx is equal to 1 - 2. MttSplitMode[ x0 ][ y0 ][ mttDepth − 1 ] is equal to parallelTtSplit Otherwise if the following condition is true, allowBtSplit is set equal tot FALSE - P. y0 + ( 1 << log2CbSize ) > pic_height_in_luma_sample - Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE: - P. slice_type is not equal to I - P mttDepth is equal to 1 - 3. cbSize is greater than 1 << MinQtLog2SizeY - P. partIdx is equal to 1 - 4. MttSplitMode[ x0 ][ y0 ][ mttDepth − 1 ] is equal to perpendicularBtSplit - 5. MttSplitMode[ x0 − partOffsetX ][ y0 − partOffsetY ][ mttDepth ] is equal to btSplit Note: Believe that the above conditions were removed in the Ljubljana meeting – Otherwise, allowBtSplit is set equal to TRUE. |  | 2 |
| % Determine allowSplitTtVer The variable allowTtSplit is derived as follows: - If one or more of the following conditions are true, allowTtplit is set equal to FALSE: - 1. cbSize is less than or equal to 2 * MinTtSizeY - 2. cbWidth is greater than MaxTtSizeY - 3. cbHeight is greater than MaxTtSizeY - 4. mttDepth is greater than or equal to MaxMttDepth Otherwise if the following condition is false, allowTtSplit is set equal tot FALSE - P. x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples Otherwise if all of the following condition is true, allowTtSplit is set equal to TRUE - P. y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples 5. x0 + 3*( 1 << log2CbSize )/4 == pic_width_in_luma_samples - Otherwise if all of the following conditions are true, allowTtSplit is set equal to FALSE: - P. slice_type is not equal to I - P. mttDepth is greater than 0 - P. partIdx is equal to 1 - 5. MttSplitMode[ x0 ][ y0 ][ mttDepth − 1 ] is equal to perpendicularBtSplit - 6. MttSplitMode[ x0 − partOffsetX ][ y0 − partOffsetY ][ mttDepth ] is equal to parallelTtSplit - Note: Believe that the above conditions were removed in the Ljubljana meeting – Otherwise, allowTtSplit is set equal to TRUE. |  | 4+1 |
| % Determine allowSplitTtHor The variable allowTtSplit is derived as follows: - If one or more of the following conditions are true, allowSTtplit is set equal to FALSE: - 1. cbSize is less than or equal to 2 * MinTtSizeY - P. cbWidth is greater than MaxTtSizeY - P. cbHeight is greater than MaxTtSizeY - P. mttDepth is greater than or equal to MaxMttDepth Otherwise if the following condition is false, allowBtSplit is set equal tot FALSE - P. y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples Otherwise if all of the following condition is true, allowTtSplit is set equal to TRUE - P. w0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples 2. y0 + 3*( 1 << log2CbSize )/4 == pic_height_in_luma_samples - Otherwise if all of the following conditions are true, allowTtSplit is set equal to FALSE: - P. slice_type is not equal to I - P. mttDepth is greater than 0 - P. partIdx is equal to 1 - 2. MttSplitMode[ x0 ][ y0 ][ mttDepth − 1 ] is equal to perpendicularBtSplit - 3. MttSplitMode[ x0 − partOffsetX ][ y0 − partOffsetY ][ mttDepth ] is equal to parallelTtSplit - Note: The above statements were removed in the Ljubljana meeting – Otherwise, allowTtSplit is set equal to TRUE. |  | 1 + 1 |
| % Determine allowNoSplit The variable allowNoSplit is derived as follows: - If one or more of the following conditions are false, allowNosplit is equal to FALSE - P. y0 + ( 1 << log2CbSize ) <= pic_height_in_luma_samples - P. x0 + ( 1 << log2CbSize ) <= pic_width_in_luma_samples |  |  |
| if( ( allowSplitBtVer ¦ ¦ allowSplitBtHor ¦ ¦ allowSplitTtVer ¦ ¦ allowSplitTtHor ) |  | 3 |
| && allowNoSplit ) ) |  | 1 |
| mtt_split_cu_flag | ae(v) | 0 |
| if( mtt_split_cu_flag ) { |  | 1 |
| if( ( allowSplitBtHor ¦ ¦ allowSplitTtHor ) && ( allowSplitBtVer ¦ ¦ allowSplitTtVer ) ) |  | 3 |
| mtt_split_cu_vertical_flag | ae(v) | 2 |
| if( ( allowSplitBtVer && allowSplitTtVer && mtt_split_cu_vertical_flag ) ¦ ¦ ( allowSplitBtHor && allowSplitTtHor && !mtt_split_cu_vertical_flag ) ) |  | 5 |
| mtt_split_cu_binary_flag | ae(v) | 6 |
| if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_BT_VER ) { |  | 1 |
| multi_type_tree( x0, y0, cbWidth / 2, cbHeight, mttDepth + 1, 0 ) |  | . |
| multi_type_tree( x0 + ( cbWidth / 2 ), y0, cbWidth / 2, cbHeightY, mttDepth + 1, 1 ) |  | . |
| } else if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_BT_HOR ) { |  | 1 |
| multi_type_tree( x0, y0, cbWidth, cbHeight / 2, mttDepth + 1, 0 ) |  | . |
| multi_type_tree( x0, y0 + ( cbHeight / 2 ), cbWidth, cbHeight / 2, mttDepth + 1, 1 ) |  | . |
| } else if( MttSplitMode[ x0 ][ y0 ][ mttDepth ] = = SPLIT_TT_VER ) { |  | 1 |
| multi_type_tree( x0, y0, cbWidth / 4, cbHeight, mttDepth + 1, 0 ) |  | . |
| multi_type_tree( x0 + ( cbWidth / 4 ), y0, cbWidth / 2, cbHeight, mttDepth + 1, 1 ) |  | . |
| multi_type_tree( x0 + ( 3 * cbWidth / 4 ), y0, cbWidth / 4, cbHeight, mttDepth + 1, 2 ) |  | . |
| } else { /* SPLIT_TT_HOR */ |  | 0 |
| multi_type_tree( x0, y0, cbWidth, cbHeight / 4, mttDepth + 1, 0 ) |  | . |
| if(y0 + ( 3 * cbHeight / 4 ), <= pic_height_in_luma_samples) |  | 1 |
| multi_type_tree( x0, y0 + ( cbHeight / 4 ), cbWidth, cbHeight / 2, mttDepth + 1, 1 ) |  | . |
| if( y0 + cbHeight <= pic_height_in_luma_samples) |  | 1 |
| multi_type_tree( x0, y0 + ( 3 * cbHeight / 4 ), cbWidth, cbHeight / 4, mttDepth + 1, 2 ) |  | . |
| } |  | . |
| } else |  | 0 |
| coding_unit( x0, y0, cbWidth, cbHeight ) |  | . |
| } |  | . |
| CHECKS (WORST CASE) |  | 37 + 5 |

#### Coding quadtree semantics

qt_split_cu_flag[ x0 ][ y0 ] specifies whether a coding unit is split into coding units with half horizontal and vertical size. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

When qt_split_cu_flag[ x0 ][ y0 ] is not present, the following applies:

- If one or more of the two following conditions are true, the value of qt_split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 1:

- x0 + ( 1  <<  log2CbSize ) is greater than pic_width_in_luma_samples.

- y0 + ( 1  <<  log2CbSize )  is greater than pic_height_in_luma_samples.

–	Otherwise, the value of qt_split_cu_flag[ x0 ][ y0 ] is inferred to be equal to 0.

#### 7.4.5.4	Multi-type tree semantics

The variables allowSplitBtVer, allowSplitBtHor, allowSplitTtVer allowSplitTtHor, allowNoSplit are derived as follows:

- The allowed binary split process as specified in clause 6.4.1 is invoked with the binary split mode SPLIT_BT_VER, the coding block width cbWidth, the coding block height cbHeight, the location ( x0, y0 ), the current multi-type tree depth mttDepth, the current partition index partIdx, , pic_width_in_luma_samples, pic_height_in_luma_samples as input, and the output is assigned to allowSplitBtVer.

- The allowed binary split process as specified in clause 6.4.1 is invoked with the binary split mode SPLIT_BT_HOR, the coding block height cbHeight, the coding block width cbWidth, the location ( x0, y0 ), the current multi-type tree depth mttDepth, the current partition index partIdx, , pic_width_in_luma_samples, pic_height_in_luma_samples as input, and the output is assigned to allowSplitBtHor.

- The allowed ternary split process as specified in clause 6.4.2 is invoked with the ternary split mode SPLIT_TT_VER, the coding block width cbWidth, the coding block height cbHeight, the location ( x0, y0 ), the current multi-type tree depth mttDepth, the current partition index partIdx, , pic_width_in_luma_samples, pic_height_in_luma_samples as input, and the output is assigned to allowSplitTtVer.

- The allowed ternary split process as specified in clause 6.4.2 is invoked with the ternary split mode SPLIT_TT_HOR, the coding block height cbHeight, the coding block width cbWidth, the location ( x0, y0 ), the current multi-type tree depth mttDepth, the current partition index partIdx, , pic_width_in_luma_samples, pic_height_in_luma_samples as input, and the output is assigned to allowSplitTtHor.

- The allowed no split process as specified in clause 6.4.2 is invoked with the no split mode NO_SPLIT, the coding block height cbHeight, the coding block width cbWidth, the location ( x0, y0 ), pic_width_in_luma_samples, pic_height_in_luma_samples as input and the output is assigned to allowNoSplit.

mtt_split_cu_flag equal to 0 specifies that a coding unit is not split. mtt_split_cu_flag equal to 1 specifies that a coding unit is split into two coding units using a binary split or into three coding units using a ternary split as indicated by the syntax element mtt_split_cu_binary_flag. The binary or ternary split can be either vertical or horizontal as indicated by the syntax element mtt_split_cu_vertical_flag.

When mtt_split_cu_flag is not present, , it is inferreed as follows:

- If allowNoSplit is equal to TRUE, the value of mtt_split_cu_flag is inferred to be equal to 0.

- Otherwise, the value of mtt_split_cu_flag is inferred to be equal to 1.

mtt_split_cu_vertical_flag equal to 0 specifies that a coding unit is split horizontally. mtt_split_cu_vertical_flag equal to 1 specifies that a coding unit is split vertically

When mtt_split_cu_vertical_flag is not present, it is inferred as follows:

- If allowSplitBtHor is equal to TRUE or allowSplitTtHor is equal to TRUE, the value of mtt_split_cu_vertical_flag is inferred to be equal to 0.

- Otherwise, the value of mtt_split_cu_vertical_flag is inferred to be equal to 1

mtt_split_cu_binary_flag equal to 0 specifies that a coding unit is split into three coding units using a ternary split. mtt_split_cu_binary_flag equal to 1 specifies that a coding unit is split into two coding units using a binary split.

When mtt_split_cu_binary_flag is not present, it is inferred as follows:

- If allowSplitBtVer is equal to FALSE and allowSplitBtHor is equal to FALSE, the value of mtt_split_cu_binary_flag is inferred to be equal to 0.

- Otherwise if allowSplitTtVer is equal to FALSE and allowSplitTtHor is equal to FALSE, the value of mtt_split_cu_binary_flag is inferred as to be equal to 1.

- Otherwise if allowSplitBtHor is equal to TRUE and allowSplitTtVer is equal to TRUE, the value of mtt_split_cu_binary_flag is inferred to be equal to !mtt_split_cu_vertical_flag.

- Otherwise (allowSplitBtVer is equal to TRUE and allowSplitTtHor is equal to TRUE), the value of mtt_split_cu_binary_flag is inferred to be equal to mtt_split_cu_vertical_flag.

The variable MttSplitMode[ x ][ y ][ mttDepth ] is derived from the value of mtt_split_cu_vertical_flag and from the value of mtt_split_cu_binary_flag as defined in Table 7-4 for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1.

Figure 7-1 – Multi-type tree spliting modes indicated by MttSplitMode (informative)

MttSplitMode[ x0 ][ y0 ][ mttDepth ] represents horizontal and vertical binary and ternary splittings of a coding unit within the multi-type tree as illustrated in Figure 7-1. The array indices x0, y0 specify the location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture.

Table 7-4 – Specification of MttSplitMode[ x ][ y ][ mttDepth ] for x = x0..x0 + cbWidth − 1 and y = y0..y0 + cbHeight − 1

| MttSplitMode[ x0 ][ y0 ][ mttDepth ] | mtt_split_cu_vertical_flag | mtt_split_cu_binary_flag |
| --- | --- | --- |
| SPLIT_TT_HOR | 0 | 0 |
| SPLIT_BT_HOR | 0 | 1 |
| SPLIT_TT_VER | 1 | 0 |
| SPLIT_BT_VER | 1 | 1 |

### 6.4.1	Allowed binary split process

Input to this process is a binary split mode btSplit, a coding block width cbWidth, a coding block height cbHeight, a location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture, a multi-type tree depth mttDepth and a partition index partIdx.

Output of this process is the variable allowBtSplit.

Table 6-2 – Specification of parallelTtSplit, perpendicularBtSplit, cbSize, partOffsetX and partOffsetY based on btSplit.

| btSplit | parallelTtSplit | perpendicularBtSplit | cbSize | partOffsetX | partOffsetY |
| --- | --- | --- | --- | --- | --- |
| SPLIT_BT_VER | SPLIT_TT_VER | SPLIT_BT_HOR | cbWidth | 0 | cbHeight |
| SPLIT_BT_HOR | SPLIT_TT_HOR | SPLIT_BT_VER | cbHeight | cbWidth | 0 |

The variables parallelTtSplit, perpendicularBtSplit, cbSize, partOffsetX and partOffsetY are derived as specified in  Table 6-2.

The variable allowBtSplit is derived as follows:

- If one or more of the following conditions are true, allowBtSplit is set equal to FALSE:

- cbSize is less than or equal to MinBtSizeY

- cbWidth is greater than MaxBtSizeY

- cbHeight is greater than MaxBtSizeY

- mttDepth is greater than or equal to MaxMttDepth

Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE:

- mttDepth is greater than 0

- partIdx is equal to 1

- MttSplitMode[ x0 ][ y0 ][ mttDepth − 1 ] is equal to parallelTtSplit

- Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE:

- slice_type is not equal to I

- mttDepth is equal to 1

- cbSize is greater than 1  <<  MinQtLog2SizeY

- partIdx is equal to 1

- MttSplitMode[ x0 ][ y0 ][ mttDepth − 1 ] is equal to perpendicularBtSplit

- MttSplitMode[ x0 − partOffsetX ][ y0 − partOffsetY ][ mttDepth ] is equal to btSplit

Note: The above statements were removed in the Ljubljana meeting

–	Otherwise, allowBtSplit is set equal to TRUE.

### Allowed ternary split process

Input to this process is a ternary split mode ttSplit, a coding block width cbWidth, a coding block height cbHeight, a location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture, a multi-type tree depth mttDepth and a partition index partIdx.

Output of this process is the variable allowTtSplit.

Table 6-3 – Specification of parallelTtSplit, perpendicularBtSplit, cbSize, partOffsetX and partOffsetY based on btSplit.

| ttSplit | parallelTtSplit | perpendicularBtSplit | cbSize | partOffsetX | partOffsetY |
| --- | --- | --- | --- | --- | --- |
| SPLIT_TT_VER | SPLIT_TT_VER | SPLIT_BT_HOR | cbWidth | 0 | cbHeight |
| SPLIT_TT_HOR | SPLIT_TT_HOR | SPLIT_BT_VER | cbHeight | cbWidth | 0 |

The variables parallelTtSplit, perpendicularBtSplit, cbSize, partOffsetX and partOffsetY are derived as specified in  Table 6-2.

The variable allowTtSplit is derived as follows:

- If one or more of the following conditions are true, allowSTtplit is set equal to FALSE:

- cbSize is less than or equal to 2 * MinTtSizeY

- cbWidth is greater than MaxTtSizeY

- cbHeight is greater than MaxTtSizeY

- mttDepth is greater than or equal to MaxMttDepth

- Otherwise if all of the following conditions are true, allowTtSplit is set equal to FALSE:

- slice_type is not equal to I

- mttDepth is greater than 0

- partIdx is equal to 1

- MttSplitMode[ x0 ][ y0 ][ mttDepth − 1 ] is equal to perpendicularBtSplit

- MttSplitMode[ x0 − partOffsetX ][ y0 − partOffsetY ][ mttDepth ] is equal to parallelTtSplit

- Note: The above statements were removed in the Ljubljana meeting

–	Otherwise, allowTtSplit is set equal to TRUE.