| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K0562-v1 |
| --- | --- |

| Title: | Report of BoG on Software development and CTC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Frank Bossen | Email: | frank@bossentech.com |
| Source: | BoG |  |  |

_____________________________

# Summary

The BoG on Software development and CTC met on Monday July 16 2018 from 9:15 to 13:15 in room Povodni Moz 2. It reviewed input contributions related to software development and CTC, and encoder optimizations. Recommendations from the BoG include:

- JVET-K0054: adopt unified PSNR calculation with runtime configurability (default off). Usage should be described in CTC document

- JVET-K0149: enhance dtrace to be able to produce additional proposed statistics. Add reference to external viewer in SW manual (with proper disclaimer).

- JVET-K0154: enable FULL_NBIT macro by default (pending SIMD code review)

- JVET-K0261: adopt proposed SW cleanup changes (change macro values if needed and remove disabled code)

- JVET-K0312: add tool usage metrics to SW. SW coordinators to determine best implementation (e.g., with or without dtrace). Encourage CU-level tool proponents to add and report such metrics (proportion of samples affected by tool)

- JVET-K0349: migrate from svn to git (GitLab) (this was previously recommended in AHG3 report)

- JVET-K0410: add fields in Excel reporting template to capture information such as processor, SIMD settings, compiler, compiler options, decoder options

- JVET-K0461: adopt guidelines for VVC SW development, add paragraphs on “const” variables and preference for shorter functions

- JVET-K0390: adopt proposed rate control method and bug fixes (pending code review)

- JVET-K0206: adopt changes into VTM (remains disabled by default). SW AHG to decide whether to enable corresponding macro (command line parameter would still disable the feature by default)

# Discussion

JVET-K0054 Unification of PSNR calculation for JVET CTC [R. Chernyak, T. Solovyev, S. Ikonin, J. Chen (Huawei)]

This contribution proposes unification of PSNR calculation algorithm for parallel RA simulations, which allows to achieve absolute matching between parallel and sequential RA simulations.

Proposes to write out PSNR floating-point value as 64-bit hexadecimal number.

Comment: don’t like using log files for PSNR. Compute it on decoded output instead.

Using MSE instead of PSNR would require knowing picture sizes and involves more computations in script.

Recommendation: adopt with cfg-level configuration. Default is off. Mention it in CTC document.

JVET-K0149 Reference software extension for coding block statistics [J. Sauer, J. Schneider, M. Bläser, M. Wien (RWTH Aachen)]

An extension for the benchmark set software BMS for writing coding block statistic data is proposed. The proposed extension is based on the “dtrace” implementation available in the BMS/VTM. The implementation enables for switchable output of information on coding tools on a block basis, such as block partitioning, motion vectors, or prediction modes into a separate statistics file. This data may then be used for statistics analysis or visualization of the applied coding tools with the reconstructed YUV file. Visualization of the data is demonstrated using an open source YUV player.

Proponent willing to help maintain code.

Viewer available at https://github.com/IENT/YUView

Recommendation: enhance dtrace to be able to produce additional proposed statistics. Add reference to external viewer in SW manual (with proper disclaimer).

JVET-K0154 On encoding distortion evaluation of VTM/BMS software [T. Chujoh, T. Ikai (Sharp)]

The bit depth of previous test sequences is 8-bit, and it is effective to evaluate the 8-bit distortion when 10-bit coding using IBDI. However, the bit depth of most of test sequences is 10-bit, and it is considered that 10-bit distortion evaluation should be appropriate for those sequences. In this contribution, it is proposed to fix BMS software regarding distortion evaluation. All changes are only encoder where the major points in the software are lambda setting and distortion accumulation. The experimental results reportedly show there are some gains on average for most of sequences and there are significant chroma gains for HDR PQ sequences. It is recommended to fix the next VVC software and to use 10-bit distortion evaluation on CTC.

Proposes to increase precision of SAD/SSD computation (currently LSBs are discarded).

Up to ~25% BD difference (chroma PQ). Otherwise much smaller gains <0.1%.

Reported similar results in HM.

How does it affect SIMD?

Recommendation: Enable FULL_NBIT macro by default (pending review of SIMD code)

JVET-K0496 Crosscheck report of JVET-K0154 on encoding distortion evaluation [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)] [late]

JVET-K0433 Crosscheck of JVET-K0154 on On encoding distortion evaluation of VTM/BMS software [X. Li (Tencent)] [late]

JVET-K0261 AHG3: VVC Software Cleanup [A. Wieckowski, K. Sühring, H. Schwarz]

At the San Diego meeting, the NextSoftware was adopted as the code-base for the development of the VVC standard. The software was originally intended as alternative implementation of the JEM software. One of the goals for development was compatibility with both the JEM as well as the HM software. With the transition to VTM and BMS, this goal is now obsolete. Yet a new requirement arises: a clean code-base. This contribution proposes to remove a set of implementation options from the code that are no longer needed and would require extra effort to be maintained. Only tools that are no longer under investigation and small encoder-only changes are proposed for removal. No normative changes to either the VTM or BMS are proposed.

Non-normative changes provide RD gains of about 0.1%.

Many macros relate to RQT.

Recommendation: adopt proposed SW changes (change macro values if needed and remove disabled code).

JVET-K0312 AHG13: Reporting of adjusted decoder runtimes in tool on/off tests [J. Boyce, D. Gurulev, V. Aristarkhov (Intel), A. Tourapis (Apple)]

This contribution proposes additional reporting of adjusted decoder runtimes in future “tool on” and “tool off” tests, based upon the frequency of the tool usage in the coded bitstream. A proposed revision to the AHG13 tool on/off reporting template is provided, which reports the percentage of pixels directly impacted by the tool, as well as the adjusted decoder runtime. A BMS decoder software patch for two example BMS tools is also provided.

In the v3 version of the document, the reporting template contains data for the Affine tool for the VTM tool-on test from AHG13, and some errors in the reporting template were corrected.

Comment: unify with dtrace? Note: dtrace makes code run about 20% slower. Two decoder runs may be required: one for timing and one for stats.

Objection to have external parser that needs to aggregate CU-level information.

It was deemed useful to have information about per-sample frequency of tool usage.

Some concern expressed about scaling run time numbers, since baseline numbers may be noisy.

Recommendation: add tool usage metrics to SW. SW coordinators to determine best implementation (e.g., with or without dtrace). Encourage CU-level tool proponents to add and report such metrics.

JVET-K0389 AHG3: Proposed software management for VTM [E. Thomas, A. Gabriel (TNO)] [late]

Recently, the management of the VTM software was discussed on the JVET reflector. It appears that the complexity of the project stretches the current project organisation to its limits. This contributions proposes a new software management for the VTM project that tries to alleviate these problems. In particular, the goal is twofold:

- facilitate explorations of techniques diverging from the main code base

- leverage mainstream tooling in software development to implement best practices increasing performance, stability and code quality of the software

Proposes migration to git.

Proposes to add unit tests to improve bug detection. Would add burden to proponents. It was remarked that interaction between tools may be major sources of bugs. Unit tests wouldn’t detect these.

Proposes to add functional tests. It was remarked that is already done to some extent.

Recommendation: migrate from svn to git (GitLab).

JVET-K0410 Comments on timing measurement variations for JVET experiments [F. Glapin, T. Poirier, F. Le Léannec, E. François (Technicolor)] [late]

At the San Diego meeting (JVET 10th meeting), the NextSoftware was adopted as the code-base for the development of the VVC standard. The further experiments will be conducted using this codebase. Among various criteria, encoding and decoding runtime variations can be used to assess the performance of tested tools. This contribution aims at discussing about the reliability of these encoding/decoding runtime metrics, considering the high variability of possible testing conditions (level of code optimization, operating systems, compiler versions, type of processors …). In this contribution, this variability is illustrated by a set of experiments. Some recommendations related to the usage and interpretation of the encoding/decoding runtime metrics are proposed for the further JVET experimental work.

Performed 10 different assessments to determine what variables affect run time. These were performed using the decoder only and a single sequence. Ratio is between VTM and BMS.

|  | description | Timing ratio variation |
| --- | --- | --- |
| Assessment 1 | Different runs in exactly same test configuration | 0.5% |
| Assessment 2 | CPU generation | 16% |
| Assessment 3 | Computer grid workload | 19% |
| Assessment 4 | Turbo mode ON/OFF | 11% |
| Assessment 5 | Compiler version | 21% |
| Assessment 6 | Cmake settings/Compiler optimization | 9% |
| Assessment 7 | SIMD level | 13~18% |
| Assessment 8 | decoder software options | 27% |
| Assessment 9 | With/without Bitstream concatenation | 3% |
| Assessment 10 | BMS macro ON/off | 1% |

Should check offline what cause use/nonuse of –O3 in cmake (assessment 6).

Suggests to:

| Parameters | Decoder | Encoder |
| --- | --- | --- |
| SIMD settings | SCALAR. Build with auto-vectorization off. | SSE42 |
| Compiler | Inform in contribution the compiler used for testing. | free |
| Compiling option | -O3 | -O3 |
| Decoder options | No MD5, no output |  |
| Bitstream concatenation | With Bitstream concatenation |  |
| Grid turbo | Inform in the contribution | Inform in the contribution |
| BMS macro | Always ON (VTM emulated by cfg parameters) | Always ON (VTM emulated by cfg parameters) |

Recommendation: Add fields in Excel template to capture information such as processor, SIMD settings, compiler, compiler options, decoder options. Further study encouraged.

JVET-K0461 AHG3: Guidelines for VVC reference software development [F. Bossen, X. Li, K. Sühring (AHG chairs)] [late]

Proposes SW guidelines for VVC reference SW development based on HM version. Several changes including naming conventions and formatting. Note that paragraph on doxygen is removed.

Suggestion to encourage usage of const: agreed.

Suggestion to encourage keeping functions to a reasonable size (e.g. ~100 lines): agreed.

Recommendation: adopt guidelines for VVC SW development, add paragraphs on “const” and function size.

JVET-K0445 Compress performance report of VTM/BMS for 8K test sequences [S. Iwamura, S. Nemoto, A. Ichigaya (NHK)]

Proponent present but said contribution was moved to another category. Not discussed.

JVET-K0390 Rate Control for VVC [Y. Li, Z. Chen (Wuhan University), X. Li, S. Liu (Tencent)] [late]

This contribution presents some improvements based on the current R-lambda rate control algorithm. With the proposed algorithm, when targeting the VTM1.1 anchor bit rate, there is 2.94% (luma) and 9.03% (luma) coding efficiency improvement in low delay B and random access configuration when compared with the original rate control algorithm. The proposed algorithm includes three parts. One is splitting the skip and non-skip area when the parameters are updated in picture level. Another modification is changing the updating strategy of the rate-distortion parameters. Besides, in the current software, the GOP size is 16 in random access configuration, while the rate control method only support GOP size equals to 8. We fixed this problem as well in this contribution.

Bug fix related to rate computation. Another about assumption of GOP size.

Algorithmic changes. Treat skipped and non-skipped blocks differently.

VTM (LB) with RC is currently much worse than fixed RC (up to 20%).

Seems beneficial.

Recommendation: adopt (pending code review).

JVET-K0472 Crosscheck of JVET-K0390: Rate Control for VVC [Q. Yu, J. Zheng (HiSilicon)] [late] [miss]

JVET-K0206 AHG10: Improved perceptually optimized QP adaptation and associated distortion measure [C. Helmrich, H. Schwarz, D. Marpe, T. Wiegand (HHI)]

In JVET-H0047, the authors proposed a CTU-wise subjectively optimized QP adaptation (QPA) along with a correspondingly weighted PSNR (WPSNR) distortion measure. This QPA approach was submitted and evaluated for JVET’s 2018 Call for Proposals and is a part of the VTM/BMS 1 software.

This document proposes seven improvements to the QPA and, if applicable, the associated distortion metric:

- disable the CTU size and beating reduction for base QPs higher than 37 or GOP sizes lower than 5

- use a frame-wise instead of CTU-wise QPA for base QPs higher than 37 or input smaller than 480p

- use a more isotropic high-pass filter kernel with 9 instead of 5 taps in the visual activity computation

- adapt the lower activity limit amin to the modified high-pass filter and make it bit-depth dependent

- use β=12 instead of β=23 as activity weighting exponent for more conservative bit-rate distribution

- re-adjust the mean activity value apic for correct rate matching after changed filtering, amin, and β

- replace fixed chroma QP offsets with the transmission of picture-adaptive slice chroma QP offsets.

The subjective merit of these modifications is demonstrated via formal comparative subjective evaluation. It is suggested to adopt the proposed improvements to the QPA design in the next version of the VTM/BMS.

It was noted that slice chroma QP offsets should be available in VTM with the adoption of separate trees.

Note: package also includes a packed YUV format output mode.

Recommendation: adopt changes into VTM (remains disabled by default). SW AHG to decide whether to enable macro (command line parameter would still be disabled by default).