| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K1021-v1 |
| --- | --- |

| Title: | CE1 description: Partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Jackie Ma Fraunhofer HHI Fabrice Le Léannec Technicolor Min Woo Park Samsung Electronics | Email: | jackie.ma@hhi.fraunhofer.de fabrice.leleannec@technicolor.com m.w.park@samsung.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document describes the Core Experiment 1 on Partitioning presented at the 11th Meeting in Ljubljana in July 2018.

# Introduction

It was decided to have a CE on partitioning that follows up on some topics in [1, 2]. There are three categories that were requested to be studied in this CE:

- Picture boundary handling (Section 3)

- Split restrictions (Section 4)

- Separate trees (Section 5)

A list of participants (including proponents and cross-checkers) is given below in Section 2.

# Participant list

| # | Name | Affiliation | Email | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
|  | Yu-Chen Sun | Alibaba Inc. | yuchen.s@alibaba-inc.com |  |  |
|  | Alexis Tourapis | Apple | atourapis@apple.com |  |  |
|  | Minhua Zhou | Broadcom | minhua.zhou@broadcom.com |  |  |
|  | Wade Wan | Broadcom | wade.wan@broadcom.com |  |  |
|  | Peisong Chen | Broadcom | peisong.chen@broadcom.com |  |  |
|  | Kai Zhang | Bytedance | zhangkai.video@bytedance.com | X |  |
|  | Li Zhang | Bytedance | lizhang.idm@bytedance.com |  |  |
|  | Jonathan Taquet | Canon | jonathan.taquet@crf.canon.fr |  |  |
|  | Chris Rosewarne | Canon | chris.rosewarne@cisra.canon.com.au |  |  |
|  | Andrew Dorrell | Canon | andrew.dorrell@cisra.canon.com.au |  |  |
|  | Julien Le Tanou | Ericsson | julien.le.tanou@ericsson.com |  |  |
|  | Jung Won Kang | ETRI | jungwon@etri.re.kr |  |  |
|  | Jackie Ma | Fraunhofer HHI | jackie.ma@hhi.fraunhofer.de |  | X |
|  | Adam Wieckowski | Fraunhofer HHI | adam.wieckowski@hhi.fraunhofer.de |  | X |
|  | Jiangqing Zhu | Fujitsu | zhujianqing@cn.fujitsu.com |  |  |
|  | Jie Yao | Fujitsu | yaojie@cn.fujitsu.com |  |  |
|  | Quanhe Yu | HiSilicon | yuquanhe@hisilicon.com |  |  |
|  | Jianhua Zheng | HiSilicon | zhengjianhua@hisilicon.com |  |  |
|  | Haechul Choi | HNU | choihc@hanbat.ac.kr |  |  |
|  | Heeji Han | HNU | sarhakor@naver.com |  |  |
|  | Daehyeok Gwon | HNU | skyeye0530@naver.com |  |  |
|  | Jaeryun Choe | HNU | tjswo9395@naver.com |  |  |
|  | Aram Baek | HNU | aram98123@naver.com |  |  |
|  | Alexey Filippov | Huawei | alexey.filippov@huawei.com |  |  |
|  | Vasily Rufitskiy | Huawei | vasily.rufitskiy@huawei.com |  |  |
|  | Sergey Ikonin | Huawei | sergey.ikonin@huawei.com |  |  |
|  | Yin Zhao | Huawei | yin.zhao@huawei.com |  |  |
|  | Han Gao | Huawei | han.gao@huawei.com |  | X |
|  | Semih Eselink | Huawei | semih.esenlik@huawei.com |  |  |
|  | Jianle Chen | Huawei | jianle.chen@huawei.com |  |  |
|  | Rahul Vanam | InterDigital | rahul.vanam@interdigital.com |  |  |
|  | Sriram Sethuraman | Ittiam Systems Pvt. Ltd. | sriram.sethuraman@protonmail.com |  |  |
|  | Kai Kawamura | KDDI | ki-kawamura@kddi.com |  |  |
|  | Yi-Wen Chen | Kwai Inc. | yiwenchen@kwai.com |  |  |
|  | Xianglin Wang | Kwai Inc. | xianglinwang@kwai.com |  |  |
|  | Ling Li | LGE | l.li@lge.com |  |  |
|  | Junghak Nam | LGE | junghak.nam@lge.com |  |  |
|  | Shih-Ta Hsiang | MediaTek | shih-ta.hsiang@mediatek.com | X |  |
|  | Chih-Wei Hsu | MediaTek | cw.hsu@mediatek.com | X |  |
|  | Olena Chubach | MediaTek | olena.chubach@mediatek.com |  |  |
|  | Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com |  |  |
|  | Mohammed Sarwer | MediaTek | Mohammed.sarwer@mediatek.com |  |  |
|  | Chong Soon Lim | Panasonic | chongsoon.lim@sg.panasonic.com |  |  |
|  | Han Boon Teo | Panasonic | hanboon.teo@sg.panasonic.com |  |  |
|  | Tadamasa Toma | Panasonic | toma.tadamasa@jp.panasonic.com |  |  |
|  | Ted Hsieh | Qualcomm | thsieh@qti.qualcomm.com |  |  |
|  | Wei-Jung Chien | Qualcomm | wchien@qti.qualcomm.com |  |  |
|  | Han Huang | Qualcomm | hanhuang@qti.qualcomm.com |  |  |
|  | Yinji Piao | Samsung | yj1003.piao@samsung.com |  |  |
|  | Min Woo Park | Samsung | m.w.park@samsung.com |  |  |
|  | Minsoo Park | Samsung | mss.park@samsung.com |  |  |
|  | Kiho Choi | Samsung | kiho14.choi@samsung.com |  |  |
|  | Dongjae Won | Sejong University | bigdj2002@sju.ac.kr |  |  |
|  | Kiran Misra | Sharp | misrak@sharplabs.com | X |  |
|  | Tomohiro Ikai | Sharp | ikai.tomohiro@sharp.co.jp |  |  |
|  | Yukinobu Yasugi | Sharp | yasugi.yukinobu@sharp.co.jp |  |  |
|  | Andrew Segall | Sharp | asegall@sharplabs.com |  |  |
|  | Byeongdoo Choi | Sharp | choib@sharplabs.com |  |  |
|  | Kenji Kondo | Sony | kenji.kondo@sony.com |  |  |
|  | Fabrice Le Léannec | Technicolor | fabrice.leleannec@technicolor.com |  |  |
|  | Tangi Poirier | Technicolor | tangi.poirier@technicolor.com |  |  |
|  | Fabrice Urban | Technicolor | fabrice.urban@technicolor.com |  |  |
|  | Franck Galpin | Technicolor | franck.galpin@technicolor.com |  |  |
|  | Edouard François | Technicolor | edouard.francois@technicolor.com |  |  |
|  | Xiang Li | Tencent | xlxiangli@tencent.com |  |  |
|  | Meng Xu | Tencent | mengxxu@tencent.com | X |  |
|  | Tao Lin | Tongji University | lintao@tongji.edu.cn |  |  |
|  | John Son | WILUS Inc. | john.son@wilusgroup.com |  |  |
|  | Koohyar Minhoo |  | koohyar.minoo@gmail.com |  |  |
|  | Jae Hoon Kim |  | i.jaehoon.kim@icloud.com |  |  |
|  | Dan Grois |  | grois@ieee.org |  |  |
|  | Limin Wang |  | cuiwang@gmail.com |  |  |

# SubCE1: Picture boundary handling

## SubCE1.1.1 (SubCE2.0.10 in JVET-J1021, JVET-K0021, JVET-K0224)

In JVET-K0224, when the current CU is across both right and bottom boundaries of the current picture, QT split is inferred. Otherwise, when the current CU size is less than or equal to the minQTsize, BT split is inferred. The constraint on MaxBTDepth is obeyed in all the CUs inside the current picture boundaries. The MaxBTDepth value is fixed without being increased by boundary splitting process.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 1.1.1 | S-T. Hsiang (MediaTek) | Picture boundary handling using quad and binary splits | JVET-K0224 | T. Poirier (Technicolor) H. Gao (Huawei) |

## SubCE1.2.1 (JVET-K0320)

In JVET-K0320 the concept of a Zero-Unit (ZU) is proposed to handle the coding structure at picture boundaries. A ZU is defined as a special Coding Unit (CU) with width or height not in a form of 2N. A ZU is coded as a normal CU, except that it has no residues. cbf_flag is signaled to be 0 mandatorily by a conformance encoder in a ZU. A ZU can also be binary-split/ternary-split/quarter-split into sub-CUs or sub-ZUs as a normal CU. The parsing process is uniform for CTUs at boundaries and not at boundaries.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 1.2.1 | K. Zhang (Bytedance) | Picture boundary handling by Zero-Unit. ZU can be split or not. | JVET-K0320 | H. Gao (Huawei) |
| 1.2.2 | K. Zhang (Bytedance) | Picture boundary handling by Zero-Unit. ZU must be split (constrained at encoder) | JVET-K0320 |  |

## SubCE1.3.1 (JVET-K0366)

In JVET-K0366 the concept of a partial CU is proposed. A partial CU is defined as a CU which contains both areas inside and outside a picture. For partial CU, only the area inside the picture boundary is considered valid and coded, as shown in Figure 1.

Figure 1 a partial CU with both valid and invalid regions

A partial CU may be further split as in VTM, however, with partial CU, it is not necessary to force split until all smaller CUs fit the picture boundary. A bin is signaled in the boundary CUs to indicate whether quad tree split is used. If not used, another bin is signaled to indicate whether it is binary split or non-split. When it is non-split, it is inferred as partial CU. It may be further restricted that the valid CU area has either width or height equal to non-power-of-2. For partial CU, skip mode or CBF zero (no coefficient) is inferred for the valid CU area.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 1.3.1 | M. Xu (Tencent) | Picture boundary handling by partial CU | JVET-K0366 | H. Gao (Huawei) |

# SubCE2: Split constraints

## SubCE2.1.1 (JVET-K0556)

Virtual pipeline data units (VPDUs) are defined as non-overlapping MxM-luma(L)/NxN-chroma(C) units in a picture. In hardware decoders, successive VPDUs are processed by multiple pipeline stages at the same time. Further, different stages process different VPDUs simultaneously. The VPDU size is almost proportional to the buffer size in most pipeline stages, therefore it is desired to keep the VPDU size small. In HEVC hardware decoders, the VPDU size is set to the maximum transform block (TB) size. Enlarging the maximum TB size from 32x32-L/16x16-C (as in HEVC) to 64x64-L/32x32-C (as in the current VVC) can bring coding gains, which results in 4X of VPDU size (64x64-L/32x32-C) expectedly in comparison with HEVC. However, in addition to quadtree (QT) coding unit (CU) partitioning, ternary tree (TT) and binary tree (BT) are adopted in VVC for achieving additional coding gains, and TT and BT splits can be applied to 128x128-L/64x64-C coding tree blocks (CTUs) recursively, which leads to 16X of VPDU size (128x128-L/64x64-C) in comparison with HEVC.

To reduce the VPDU size in VVC, one constraint for TT and BT is proposed, and the VPDU size is defined as 64x64-L/32x32-C under the following constraint:

For each CTU, the processing order of CUs shall not leave a VPDU and re-visit it later. Further, the following two conditions shall not be violated:

- For each VPDU containing one or multiple CUs, the CUs are completely contained in the VPDU.

- For each CU containing one or more VPDUs, the VPDUs are completely contained in the CU.

Different syntax design to meet the constraints with different coding performance impact are studied and tested.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 2.1.1 | C.-W. Hsu | - For each VPDU containing one or multiple CUs, the CUs are completely contained in the VPDU - For each CU containing one or more VPDUs, the VPDUs are completely contained in the CU - For each CTU, the above two conditions shall not be violated, and the processing order of CUs shall not leave a VPDU and re-visit it later | JVET-K0566 | C. Rosewarne (Canon) |
| 2.1.2 | C.-W. Hsu | - Prohibit ternary split of edges longer than MAX_LUMA_TB_SIZE - Prohibit vertical binary split when width is MAX_LUMA_TB_SIZE and height is larger than MAX_LUMA_TB_SIZE - Prohibit horizontal binary split when width is larger than MAX_LUMA_TB_SIZE and height is MAX_LUMA_TB_SIZE | JVET-K0566 |  |
| 2.1.3 | C.-W. Hsu | - Set MAX_TT_SIZE = MAX_LUMA_TB_SIZE - Set MAX_BT_SIZE = MAX_LUMA_TB_SIZE | JVET-K0566 | Y. Zhao (Huawei) |
| 2.1.4 | C.-W. Hsu | - Force QT split when width is larger than MAX_LUMA_TB_SIZE and height is larger than MAX_LUMA_TB_SIZE - (i.e. Force 128x128 CTU to split into 4 64x64 CUs) | JVET-K0566 | J. Ma (Fraunhofer HHI) |

# SubCE3: Separate trees

## SubCE3.1.1, SubCE3.1.2 (SubCE5.3.2, SubCE5.4.2 in JVET-J1021, JVET-K0021, JVET-K0354)

This test evaluates support for separate tree structures in inter slices. Separate trees are signaled on a coding unit (CU) basis, and the separate partitions are only available for intra coded regions. This is realized by indicating the partition information for a CTU, and then, indicating the prediction type for the resulting CUs.

A first test evaluates the performance of the method without changing any signaling for intra slices.  The benefit of the approach is that it enables intra coding tools to operate in a similar manner in both intra and inter types.

A second test evaluates the performance when the method is applied to both intra and inter slices. The impact of this change is that an encoder could select to use shared trees in an intra slice if desired, and the result provides a harmonized solution for supporting both separate and shared trees consistently in all slice types.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 3.1.1 | K. Misra (Sharp) | CU level separate trees for intra-coded CUs in inter slices | JVET-K0354 | T. Poirier (Technicolor) A. Wieckowski (Fraunhofer HHI) |
| 3.1.2 | K. Misra (Sharp) | Separate trees for intra-coded CUs in inter slices combined with support for both separate and shared trees in intra-slices. | JVET-K0354 | A. Wieckowski (Fraunhofer HHI) |

## SubCE3.2.1, SubCE3.2.2

At the 11th Meeting it was encouraged to include the following two tests into this CE.

- There should be a flag added at CTU level for an inter slice. If the flag is one, the whole CTU is intra and is coded like an intra CTU in an intra slice.

- As an additional test, possible in combination with the first one, it should be evaluated to consider having a flag at the CTU level or CU level to select between separate and shared trees.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 3.2.1 | K. Misra (Sharp) | Separate tree support at CTU level for intra-coded CTUs in inter slices. | JVET-K_Notes |  |
| 3.2.2 | K. Misra (Sharp) | Combination of 3.1.2 and 3.2.1. For all slices, flag sent at CTU level. If flag is true, then CTU is coded as intra CTU using separate trees. If flag is false, then method of 3.1.2 is used. | JVET-K_Notes |  |

## SubCE3 Test configuration

The test in SubCE3 shall use test content constructed to create a larger amount of intra-coded regions in an inter-slice.  This is done as the sequences in the CTC do not exhibit this general behavior. Software for creating the sequences shall be provided within two weeks from the finalization of this CE. The sequences shall be coded using the same encoder configuration as described in the common test conditions with the exception that All Intra results need not be provided.

Each test sequences will be created by spatially mixing two sequences from the CTC.  For simplicity, the mixing will be performed on a checker board basis, and the location of the checkerboard pattern will be changed for every GOP.  A visual description appears in Figure 2

Figure 2: Synthetic content creation process.  Goal is to create sequences with a large number of intra coded regions within an inter slices to better understand the performance of an intra coding tools

To be clear, the purpose of the requested tests with synthetic content is to evaluate the performance of the approaches with sequences that have a large amount of intra coded regions within an inter-coded slice. In addition to this additional content, the test configuration in Section 6 will also be performed.  The goal of this test primarily is to evaluate the complexity impact (e.g. run-time) of the proposed approaches on CTC content.

# Test configurations

The SubCEs are to be conducted under the common test conditions (CTC) as established at the 11th Meeting in Ljubljana. Further, implementations are to be made into VTM-2 which is the outcome software of the same meeting.

# Timeline

The following timeline is agreed for this CE:

- Aug 8 (T1): Finalization of the CE document

- 2 weeks after VTM-2 SW release (T2): Finished implementation of the technologies into the code basis. Initial study by cross-checkers can begin. Proponents may continue to modify the software in this branch until T3.

- Sep 12 (T3): Changes to software branches is frozen and proponent report results to CE coordinators for inclusion in CE report.

# References

- J. Ma, F. Le Léannec, M. W. Park, “Description of Core Experiment 1 (CE1): Partitioning,” JVET-J1021, Apr. 2018, San Diego.

- J. Ma, F. Le Léannec, M. W. Park, “Summary report for CE1: 1 Partitioning,” JVET-K0021, July. 2018, Ljubljana.