| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K1021-v1 |
| --- | --- |

| Title: | CE1 description: Partitioning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Jackie Ma Fraunhofer HHI Fabrice Le Léannec Technicolor Min Woo Park Samsung Electronics | Email: | jackie.ma@hhi.fraunhofer.de fabrice.leleannec@technicolor.com m.w.park@samsung.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

This document describes the Core Experiment 1 on Partitioning presented at the 11th Meeting in Ljubljana in July 2018.

# Introduction

It was decided to have a CE on partitioning that follows up on some topics in [1, 2]. There are three categories that were requested to be studied in this CE:

- Picture boundary handling (Section 3)

- Split restrictions (Section 4)

- Separate trees (Section 5)

A list of participants (including proponents and cross-checkers) is given below in Section 2.

# Participant list

| # | Name | Affiliation | Email | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
|  | Yu-Chen Sun | Alibaba Inc. | yuchen.s@alibaba-inc.com |  |  |
|  | Alexis Tourapis | Apple | atourapis@apple.com |  |  |
|  | Minhua Zhou | Broadcom | minhua.zhou@broadcom.com |  |  |
|  | Wade Wan | Broadcom | wade.wan@broadcom.com |  |  |
|  | Peisong Chen | Broadcom | peisong.chen@broadcom.com |  |  |
|  | Kai Zhang | Bytedance | zhangkai.video@bytedance.com | X |  |
|  | Li Zhang | Bytedance | lizhang.idm@bytedance.com |  |  |
|  | Jonathan Taquet | Canon | jonathan.taquet@crf.canon.fr |  |  |
|  | Chris Rosewarne | Canon | chris.rosewarne@cisra.canon.com.au |  |  |
|  | Andrew Dorrell | Canon | andrew.dorrell@cisra.canon.com.au |  |  |
|  | Julien Le Tanou | Ericsson | julien.le.tanou@ericsson.com |  |  |
|  | Jung Won Kang | ETRI | jungwon@etri.re.kr |  |  |
|  | Adam Wieckowski | Fraunhofer HHI | adam.wieckowski@hhi.fraunhofer.de |  | X |
|  | Jiangqing Zhu | Fujitsu | zhujianqing@cn.fujitsu.com |  |  |
|  | Jie Yao | Fujitsu | yaojie@cn.fujitsu.com |  |  |
|  | Quanhe Yu | HiSilicon | yuquanhe@hisilicon.com |  |  |
|  | Jianhua Zheng | HiSilicon | zhengjianhua@hisilicon.com |  |  |
|  | Haechul Choi | HNU | choihc@hanbat.ac.kr |  |  |
|  | Heeji Han | HNU | sarhakor@naver.com |  |  |
|  | Daehyeok Gwon | HNU | skyeye0530@naver.com |  |  |
|  | Jaeryun Choe | HNU | tjswo9395@naver.com |  |  |
|  | Aram Baek | HNU | aram98123@naver.com |  |  |
|  | Alexey Filippov | Huawei | alexey.filippov@huawei.com |  |  |
|  | Vasily Rufitskiy | Huawei | vasily.rufitskiy@huawei.com |  |  |
|  | Sergey Ikonin | Huawei | sergey.ikonin@huawei.com |  |  |
|  | Yin Zhao | Huawei | yin.zhao@huawei.com |  |  |
|  | Han Gao | Huawei | han.gao@huawei.com |  | X |
|  | Semih Eselink | Huawei | semih.esenlik@huawei.com |  |  |
|  | Jianle Chen | Huawei | jianle.chen@huawei.com |  |  |
|  | Rahul Vanam | InterDigital | rahul.vanam@interdigital.com |  |  |
|  | Sriram Sethuraman | Ittiam Systems Pvt. Ltd. | sriram.sethuraman@protonmail.com |  |  |
|  | Kai Kawamura | KDDI | ki-kawamura@kddi.com |  |  |
|  | Yi-Wen Chen | Kwai Inc. | yiwenchen@kwai.com |  |  |
|  | Xianglin Wang | Kwai Inc. | xianglinwang@kwai.com |  |  |
|  | Ling Li | LGE | l.li@lge.com |  |  |
|  | Junghak Nam | LGE | junghak.nam@lge.com |  |  |
|  | Shih-Ta Hsiang | MediaTek | shih-ta.hsiang@mediatek.com | X |  |
|  | Chih-Wei Hsu | MediaTek | cw.hsu@mediatek.com | X |  |
|  | Olena Chubach | MediaTek | olena.chubach@mediatek.com |  |  |
|  | Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com |  |  |
|  | Mohammed Sarwer | MediaTek | Mohammed.sarwer@mediatek.com |  |  |
|  | Chong Soon Lim | Panasonic | chongsoon.lim@sg.panasonic.com |  |  |
|  | Han Boon Teo | Panasonic | hanboon.teo@sg.panasonic.com |  |  |
|  | Tadamasa Toma | Panasonic | toma.tadamasa@jp.panasonic.com |  |  |
|  | Ted Hsieh | Qualcomm | thsieh@qti.qualcomm.com |  |  |
|  | Wei-Jung Chien | Qualcomm | wchien@qti.qualcomm.com |  |  |
|  | Han Huang | Qualcomm | hanhuang@qti.qualcomm.com |  |  |
|  | Yinji Piao | Samsung | yj1003.piao@samsung.com |  |  |
|  | Dongjae Won | Sejong University | bigdj2002@sju.ac.kr |  |  |
|  | Kiran Misra | Sharp | misrak@sharplabs.com | X |  |
|  | Tomohiro Ikai | Sharp | ikai.tomohiro@sharp.co.jp |  |  |
|  | Yukinobu Yasugi | Sharp | yasugi.yukinobu@sharp.co.jp |  |  |
|  | Andrew Segall | Sharp | asegall@sharplabs.com |  |  |
|  | Byeongdoo Choi | Sharp | choib@sharplabs.com |  |  |
|  | Kenji Kondo | Sony | kenji.kondo@sony.com |  |  |
|  | Fabrice Le Léannec | Technicolor | fabrice.leleannec@technicolor.com |  |  |
|  | Tangi Poirier | Technicolor | tangi.poirier@technicolor.com |  |  |
|  | Fabrice Urban | Technicolor | fabrice.urban@technicolor.com |  |  |
|  | Franck Galpin | Technicolor | franck.galpin@technicolor.com |  |  |
|  | Edouard François | Technicolor | edouard.francois@technicolor.com |  |  |
|  | Xiang Li | Tencent | xlxiangli@tencent.com |  |  |
|  | Meng Xu | Tencent | mengxxu@tencent.com | X |  |
|  | John Son | WILUS Inc. | john.son@wilusgroup.com |  |  |
|  | Koohyar Minhoo |  | koohyar.minoo@gmail.com |  |  |
|  | Jae Hoon Kim |  | i.jaehoon.kim@icloud.com |  |  |
|  | Dan Grois |  | grois@ieee.org |  |  |
|  | Limin Wang |  | cuiwang@gmail.com |  |  |

# SubCE1: Picture boundary handling

## SubCE1.1.1 (SubCE2.0.10 in JVET-J1021, JVET-K0021, JVET-K0224)

In JVET-K0224, when the current CU is across both right and bottom boundaries of the current picture, QT split is inferred. Otherwise, when the current CU size is less than or equal to the minQTsize, BT split is inferred. The constraint on MaxBTDepth is obeyed in all the CUs inside the current picture boundaries. The MaxBTDepth value is fixed without being increased by boundary splitting process.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 1.1.1 | S-T. Hsiang (MediaTek) | Picture boundary handling using quad and binary splits | JVET-K0224 | T. Poirier (Technicolor) H. Gao (Huawei) |

## SubCE1.2.1 (JVET-K0320)

In JVET-K0320 the concept of a Zero-Unit (ZU) is proposed to handle the coding structure at picture boundaries. A ZU is defined as a special Coding Unit (CU) with width or height not in a form of 2N. A ZU is coded as a normal CU, except that it has no residues. cbf_flag is signaled to be 0 mandatorily by a conformance encoder in a ZU. A ZU can also be binary-split/ternary-split/quarter-split into sub-CUs or sub-ZUs as a normal CU.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 1.2.1 | K. Zhang (Bytedance) | Picture boundary handling by Zero-Unit | JVET-K0320 | H. Gao (Huawei) |

## SubCE1.3.1 (JVET-K0366)

In JVET-K0366 the concept of a partial CU is proposed. A partial CU is defined as a CU which contains both areas inside and outside a picture. For partial CU, only the area inside the picture boundary is considered valid and coded, as shown in Figure 1.

Figure 1 a partial CU with both valid and invalid regions

A partial CU may be further split as in VTM, however, with partial CU, it is not necessary to force split until all smaller CUs fit the picture boundary. A bin is signaled in the boundary CUs to indicate whether quad tree split is used. If not used, another bin is signaled to indicate whether it is binary split or non-split. When it is non-split, it is inferred as partial CU. It may be further restricted that the valid CU area has either width or height equal to non-power-of-2. For partial CU, skip mode or CBF zero (no coefficient) is inferred for the valid CU area.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 1.3.1 | M. Xu (Tencent) | Picture boundary handling by partial CU | JVET-K0366 | H. Gao (Huawei) |

# SubCE2: Split constraints

## SubCE2.1.1 (JVET-K0556)

Virtual pipeline data units (VPDUs) are defined as non-overlapping MxM-luma(L)/NxN-chroma(C) units in a picture. In hardware decoders, successive VPDUs are processed by multiple pipeline stages at the same time. Further, different stages process different VPDUs simultaneously. The VPDU size is almost proportional to the buffer size in most pipeline stages, therefore it is desired to keep the VPDU size small. In HEVC hardware decoders, the VPDU size is set to the maximum transform block (TB) size. Enlarging the maximum TB size from 32x32-L/16x16-C (as in HEVC) to 64x64-L/32x32-C (as in the current VVC) can bring coding gains, which results in 4X of VPDU size (64x64-L/32x32-C) expectedly in comparison with HEVC. However, in addition to quadtree (QT) coding unit (CU) partitioning, ternary tree (TT) and binary tree (BT) are adopted in VVC for achieving additional coding gains, and TT and BT splits can be applied to 128x128-L/64x64-C coding tree blocks (CTUs) recursively, which leads to 16X of VPDU size (128x128-L/64x64-C) in comparison with HEVC.

To reduce the VPDU size in VVC, one constraint for TT and BT is proposed, and the VPDU size is defined as 64x64-L/32x32-C under the following constraint:

For each CTU, the processing order of CUs shall not leave a VPDU and re-visit it later. Further, the following two conditions shall not be violated:

- For each VPDU containing one or multiple CUs, the CUs are completely contained in the VPDU.

- For each CU containing one or more VPDUs, the VPDUs are completely contained in the CU.

Different syntax design to meet the constraints with different coding performance impact are studied and tested.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 2.1.1 | C.-W. Hsu | - For each VPDU containing one or multiple CUs, the CUs are completely contained in the VPDU - For each CU containing one or more VPDUs, the VPDUs are completely contained in the CU - For each CTU, the above two conditions shall not be violated, and the processing order of CUs shall not leave a VPDU and re-visit it later | JVET-K0566 | C. Rosewarne (Canon) |
| 2.1.2 | C.-W. Hsu | - Prohibit ternary split of edges longer than MAX_LUMA_TB_SIZE - Prohibit vertical binary split when width is MAX_LUMA_TB_SIZE and height is larger than MAX_LUMA_TB_SIZE - Prohibit horizontal binary split when width is larger than MAX_LUMA_TB_SIZE and height is MAX_LUMA_TB_SIZE | JVET-K0566 |  |
| 2.1.3 | C.-W. Hsu | - Set MAX_TT_SIZE = MAX_LUMA_TB_SIZE - Set MAX_BT_SIZE = MAX_LUMA_TB_SIZE | JVET-K0566 | Y. Zhao (Huawei) |

# SubCE3: Separate trees

## SubCE3.1.1, SubCE3.1.2 (SubCE5.3.2, SubCE5.4.2 in JVET-J1021, JVET-K0021, JVET-K0354)

This test evaluates adaptively switching between a shared or separate tree structure for intra and inter slices. The switching is signaled on a coding unit (CU) basis, and it is only available for intra coded regions. The selection is received by first receiving the partition information for the CTU, and then, receiving the prediction type for each resulting CU.  In the case that the prediction type is intra, a flag is received indicating if the intra region in the CU will be further refined with a separate or shared tree structure. The test includes inferring shared or separate tree flag value based on block size.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 3.1.1 | K. Misra (Sharp) | CU level separate tree decision for inter slices | JVET-K0354 | T. Poirier (Technicolor) A. Wieckowski (Fraunhofer HHI) |
| 3.1.2 | K. Misra (Sharp) | CU level separate tree decision for intra and inter slices | JVET-K0354 | A. Wieckowski (Fraunhofer HHI) |

## SubCE3.2.1, SubCE3.2.2

At the 11th Meeting it was encouraged to include the following two tests into this CE.

- There should be a flag added at CTU level for an inter slice. If the flag is one, the whole CTU is intra and is coded like an intra CTU in an intra slice.

- As an additional test, possible in combination with the first one, it should be evaluated to consider having a flag at the CTU level or CU level to select between separate and shared trees.

| # | Tester | Test | Related Docs. | Cross-checkers |
| --- | --- | --- | --- | --- |
| 3.2.1 |  |  | JVET-K_Notes |  |
| 3.2.2 |  |  | JVET-K_Notes |  |

# Test configurations

The SubCEs are to be conducted under the common test conditions (CTC) as established at the 11th Meeting in Ljubljana. Further, implementations are to be made into VTM-2 which is the outcome software of the same meeting.

# Timeline

The following timeline is agreed for this CE:

- Aug 8 (T1): Finalization of the CE document

- 2 weeks after VTM-2 SW release (T2): Finished implementation of the technologies into the code basis. Initial study by cross-checkers can begin. Proponents may continue to modify the software in this branch until T3.

- Sep 12 (T3): Changes to software branches is frozen and proponent report results to CE coordinators for inclusion in CE report.

# References

- J. Ma, F. Le Léannec, M. W. Park, “Description of Core Experiment 1 (CE1): Partitioning,” JVET-J1021, Apr. 2018, San Diego.

- J. Ma, F. Le Léannec, M. W. Park, “Summary report for CE1: 1 Partitioning,” JVET-K0021, July. 2018, Ljubljana.