| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K1022-v1 |
| --- | --- |

| Title: | Description of Core Experiment 2 (CE2): Adaptive Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Output document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Ching-Yeh Chen | Tel Email: | vseregin@qti.qualcomm.com chingyeh.chen@mediatek.com |  |
| Source: | CE2 coordinators |  |  |

_____________________________

# Abstract

The goal of this Core Experiment is to improve and enhance Adaptive Loop Filtering technique in VVC. The software basis for this CE is VTM-2.0, CE tests are performed according to the CTC and compared with VTM and BMS(?) anchors.

It is also noted that combination of technologies listed in this CE are encouraged.

# Tests performed in CE

| Test# | Description | Document# |
| --- | --- | --- |
| CE2.1 | 7x7Cross+3x3Square filter shape and filter symmetry | JVET-K0239 |
| CE2.2 | Filter coefficients coding | JVET-K0239 |
| CE2.3 | Fixed filters, temporal filters and CU-level adaptivity | JVET-K0371 JVET-K0382 |
| CE2.4 | Low latency encoder for ALF | JVET-K0382 |
| CE2.5 | Filter coefficients restriction for bit shift operation | JVET-K0215 |
| CE2.6 | Subsampled Laplacian calculation | JVET-K0164 |
| CE2.7 | Subsampled Laplacian calculation for highest temporal layer | JVET-K0327 |

# Detailed description of tests

#### CE2.1 7x7Cross+3x3Square filter shape and filter symmetry in JVET-K0239

In JVET-K0239, the filter footprint is 9x7Cross+3x3Square and two kinds of symmetries are supported, as shown in below figures (a) and (b). According to the simulation results, the BD-rate increase is very minor. Therefore, in this CE test, the filter shape is further reduced to 7x7Cross+3x3Square for luma component. And the filter symmetries will also be tested. Chroma filters are 5x5 (the same as in VTM).

The filter shape in ALF with two coefficient symmetry characteristics. (a) 9x7Cross+3x3Square with half symmetry, 9 coefficients. (b) 9x7Cross+3x3Square with full symmetry, 4 coefficients.

#### CE2.2 Filter coefficients coding in JVET-K0239

In K0239, absolute value of filter coefficient is signaled using exp-Golomb binarization followed by a coefficient sign. In the contribution, filter coefficient range was limited to 10 bits.

- 2.2.1 Test this coefficient coding scheme applied for both luma and chroma filter coefficients

- 2.2.2 Test limiting filter coefficients range to 10 bits, center coefficient is unsinged and other coefficients have a sign bin.

#### CE2.3 Fixed, temporal filters and CU-level adaptivity in JVET-K0371/K0382

In K0371, the number of fixed filters was reduced, temporal filters can be used to filter the current picture, CU-level enable ALF flags are utilized. In K0382, CTB can have the following options of filters: (1) use the set of filters signalled for the current picture; (2) use temporal filter set derived from the previous pictures; (3) use a set of fixed filters from 16 available sets; (4) no filter applied

- 2.3.1 Test the performance of fixed filters.

- 2.3.2 Test the performance of the temporal filters.

- 2.3.3 Test the performance when CU-level ALF enable flags are applied.

#### CE2.4. Low latency encoder for ALF from JVET-K0382

In VTM, ALF filters are derived for a picture and decision of using the derived filters for the picture or not is done on CTB basis.

- 2.4.1 Test using filters derived from the previous pictures to be used to filter the current picture for VTM

- 2.4.2 Test using filters derived from previous pictures to be used to filter the current picture with combination of CE2.3 tests.

#### CE2.5 Filter coefficients restriction for bit shift operation from JVET-K0215

In J0215, the filter coefficients used in filtering of the luma/chroma samples are divided into 2 groups. The first group contains that coefficients {L0, L1, L2, L3, L4, L5, L7, L8, L9, L10. L14, L15, L16, L17}, and the second group contains the rest of the coefficients. The first group coefficients are normatively restricted to use only the following set of values: {–64, –32, –16, –8, –4, 0, 4, 8, 16, 32, 64}, hence multiplication with those coefficients can be implemented as a single bit shift operation. The allowed filter coefficient values are mapped to {–5, –4, –3, –2, –1, 0, 1, 2, 3, 4, 5} respectively before binarization in order to reduce the signalling overhead.

The number of coefficients that apply the coefficient restriction as well as the coefficient value range will be tested according to new filter shapes in VTM2.

#### CE2.6 Subsampled Laplacian calculation from JVET-K0164

4×4 block classification based on subsampled sum-modified-Laplacian (SSML) is used. On below figures (a), (b), (c), and (d), the positions of calculating 1-D Laplacian, for gradients, gv, gh, gd1 and gd2, respectively, for a 4×4 block are shown. All of them are calculated at the subsampled positions within 8×8 window. The rest of derivation process for class index with the calculated gradients, gv, gh, gd1 and gd2, is the same as in VTM.

(a) Calculation of vertical gradient, gv      (b) Calculation of horizontal gradient, gh

(c) Calculation of diagonal gradients, gd1       (d) Calculation of diagonal gradients, gd2

- 2.6.1 Test subsampled positions for 4 gradients (vertical, horizontal, diagonal1, and diagonal2 gradients) are different according to each direction of gradient.

- 2.6.2 Test the unified subsampled positions D1 shown on figure (c) is used for calculation of all 4 gradients.

- 2.6.3 Test the unified subsampled positions D2 shown on figure (d) is used for calculation of all 4 gradients

#### CE2.7 Subsampled Laplacian calculation for highest temporal layer from JVET-K0327

In 4×4 block classification, the derivative calculation of 2×2 block for those pictures at the highest temporal layer is simplified by calculating the derivative only at top-left sample position of each 2×2 block as follows

- G2×2dir=gdiri0,j0

where dir is the direction such as horizontal, vertical, diagonal and anti-diagonal, and gdiri0,j0 is the derivative at the top-left sample position of each 2×2 luma block.

Following two tests will be tested:

- 2.7.1 Test proposed modification applied for highest temporal layer.

- 2.7.2 Test proposed modification applied for highest temporal layer, and applying subsampled Laplacian calculation method from CE2.6 to the lower temporal layers.

# Test conditions

To be updated

CTC conditions as agreed at the 11th JVET meeting (Ljubljana, DI) for SDR will be used for the evaluation of objective performance (JVET-K1010). The first anchor is the ‘VVC Test Model’ or VTM. The second anchor is the ‘Benchmark Model Set’ or BMS. AI, RA, and LB test configurations shall be used and providing the results for each of these configurations is mandatory. The BMS version 2.0 software shall be used with ‘bms’ configuration files to produce the BMS anchor, while ‘vtm’ configuration files shall be used to produce the VTM anchor.

Proposals will be compared with respect to objective compression efficiency while complexity will also be assessed. An example of complexity assessment is provided in BoG JVET-K0521.

# Timeline

| T1 | End of 11th JVET meeting + 3 weeks: August 8, 2018 | CE test description finalized |
| --- | --- | --- |
| T2 | T1 + 2 weeks: August 22, 2018 | Initial software of CE tests released; initial study by cross-checkers begins |
| T3 | 3 weeks before 12th JVET meeting: September 12, 2018 | Software of CE tests is finalized, and full test results are provided; final cross-check begins |
| T4 |  | Document upload: test results, cross-check reports, CE summary report |

# Appendix of Participants

| # | Name | Company | E-mail | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
| 1 | Yu-Chen Sun | Alibaba | yuchen.s@alibaba-inc.com |  |  |
| 2 | Alexis Tourapis | Apple | atourapis@apple.com |  |  |
| 3 | Jae Hoon Kim | Apple | i.jaehoon.kim@icloud.com |  |  |
| 4 | Li Zhang | ByteDance | lizhang.idm@bytedance.com |  |  |
| 5 | Wade Wan | Broadcom | wade.wan@broadcom.com |  |  |
| 6 | Peisong Chen | Broadcom | peisong.chen@broadcom.com |  |  |
| 7 | Minhua Zhou | Broadcom | minhua.zhou@broadcom.com |  |  |
| 8 | Jonathan TAQUET | Canon | Jonathan.TAQUET@crf.canon.fr |  |  |
| 9 | Guillaume LAROCHE | Canon | guillaume.laroche@crf.canon.fr |  |  |
| 10 | Patrice Onno | Canon | patrice.onno@crf.canon.fr |  |  |
| 11 | Dan Grois | Comcast | groisz@yahoo.com |  |  |
| 12 | Peng Yin | Dolby | pyin@dolby.com |  |  |
| 13 | Jungwon Kang | ETRI | jungwon@etri.re.kr |  |  |
| 14 | Sung-Chang Lim | ETRI | sclim@etri.re.kr |  |  |
| 15 | Johannes Erfurt | HHI | johannes.erfurt@hhi.fraunhofer.de | X |  |
| 16 | Wang-Q Lim | HHI | wang.lim@hhi.fraunhofer.de | X |  |
| 17 | Li Wang | Hikvision | wangli7@hikvision.com |  |  |
| 18 | Jianhua Zheng | HiSilicon | zhengjianhua@hisilicon.com |  |  |
| 19 | Quanhe Yu | HiSilicon | yuquanhe@hisilicon.com |  |  |
| 20 | Haechul Choi | HNU | choihc@hanbat.ac.kr |  |  |
| 21 | Heeji Han | HNU | sarhakor@naver.com |  |  |
| 22 | Daehyeok Gwon | HNU | skyeye0530@naver.com |  |  |
| 23 | Jaeryun Choe | HNU | tjswo9395@naver.com |  |  |
| 24 | Alexey Filippov | Huawei | alexey.filippov@huawei.com | X |  |
| 25 | Vasily Rufitskiy | Huawei | vasily.rufitskiy@huawei.com | X |  |
| 26 | Sergey Ikonin | Huawei | sergey.ikonin@huawei.com | X |  |
| 27 | Victor Stepin | Huawei | stepin.victor@huawei.com | X |  |
| 28 | Roman Chernyak | Huawei | chernyak.roman@huawei.com | X |  |
| 29 | Semih Esenlik | Huawei | semih.esenlik@huawei.com | X |  |
| 30 | Zhipin Deng | Intel | zhipin.deng@intel.com |  |  |
| 31 | Xu Lidong | Intel | lidong.xu@intel.com |  |  |
| 32 | Rahul Vanam | InterDigital | Rahul.vanam@InterDigital.com | X |  |
| 33 | Yuwen He | InterDigital | Yuwen.He@InterDigital.com | X |  |
| 34 | Sriram Sethuraman | Ittiam | sriram.sethuraman@ittiam.com |  |  |
| 35 | Yoshitaka Kidani | KDDI | yo-kidani@kddi.com |  |  |
| 36 | Yi-Wen Chen | Kwai | yiwenchen@kwai.com |  |  |
| 37 | Xianglin Wang | Kwai | xianglinwang@kwai.com |  |  |
| 38 | Junghak Nam | LGE | junghak.nam@lge.com |  |  |
| 39 | Hyeongmun Jang | LGE | hm.jang@lge.com |  |  |
| 40 | Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com | X |  |
| 41 | Ching-Yeh Chen | MediaTek | chingyeh.chen@mediatek.com | X |  |
| 42 | Tzu-Der Chuang | MediaTek | peter.chuang@mediatek.com | X |  |
| 43 | Yu-Chi Su | MediaTek | yu-chi.su@mediatek.com | X |  |
| 44 | Chen-Yen Lai | MediaTek | jenny.lai@mediatek.com | X |  |
| 45 | Andrey Norkin | Netflix | anorkin@netflix.com |  |  |
| 46 | Alireza Aminlou | Nokia | alireza.aminlou@nokia.com |  |  |
| 47 | Tadamasa Toma | Panasonic | toma.tadamasa@jp.panasonic.com |  |  |
| 48 | Vadim Seregin | Qualcomm | vseregin@qti.qualcomm.com | X |  |
| 49 | Nan Hu | Qualcomm | nanh@qti.qualcomm.com | X |  |
| 50 | Dmytro Rusanovskyy | Qualcomm | dmytror@qti.qualcomm.com | X |  |
| 51 | Akshay Gadde | Qualcomm | gadde@qti.qualcomm.com | X |  |
| 52 | Marta Karczewicz | Qualcomm | martak@qti.qualcomm.com | X |  |
| 53 | Woongil Choi | Samsung | woongil.choi@samsung.com |  |  |
| 54 | Kiho Choi | Samsung | kiho14.choi@samsung.com |  |  |
| 55 | Jie Chen | Samsung | jie1222.chen@samsung.com |  |  |
| 56 | Eiichi Sasaki | Sharp | eiichi.sasaki@sharp.co.jp |  |  |
| 57 | Takeshi Chujoh | Sharp | chujoh.takeshi@sharp.co.jp |  |  |
| 58 | Yukinobu Yasugi | Sharp | yasugi.yukinobu@sharp.co.jp |  |  |
| 59 | Tomohiro Ikai | Sharp | ikai.tomohiro@sharp.co.jp |  |  |
| 60 | Byeongdoo Choi | Sharp | choib@sharplabs.com |  |  |
| 61 | Andrew Segall | Sharp | asegall@sharplabs.com |  |  |
| 62 | Kiran Misra | Sharp | misrak@sharplabs.com |  |  |
| 63 | Masaru Ikeda | Sony | Masaru.Ikeda@sony.com |  |  |
| 64 | Philippe Bordes | Technicolor | Philippe.bordes@technicolor.com |  |  |
| 65 | Philippe De Lagrange | Technicolor | Philippe.delagrange@technicolor.com |  |  |
| 66 | Edouard Francois | Technicolor | edouard.francois@technicolor.com |  |  |