| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K1022-v5 |
| --- | --- |

| Title: | Description of Core Experiment 2 (CE2): Adaptive Loop Filter |  |  |
| --- | --- | --- | --- |
| Status: | Output document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Vadim Seregin Ching-Yeh Chen | Tel Email: | vseregin@qti.qualcomm.com chingyeh.chen@mediatek.com |  |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

# This document describes Core Experiment 2 (CE2) on Adaptive Loop Filter. The experiments are conducted after the 11th JVET meeting and the results, including cross-checks, are reported as input contributions to the 12th JVET meeting.

# Introduction

# The following tests will be performed:

| Test# | Description | Document# |
| --- | --- | --- |
| CE2.1 | 7x7Cross+3x3Square filter shape and filter symmetry | JVET-K0239 |
| CE2.2 | Filter coefficients coding | JVET-K0239 |
| CE2.3 | Fixed filters, temporal filters and CU-level adaptivity | JVET-K0371 JVET-K0382 |
| CE2.4 | Low latency encoder for ALF | JVET-K0382 |
| CE2.5 | Filter coefficients restriction for bit shift operation | JVET-K0215 |
| CE2.6 | Subsampled Laplacian calculation | JVET-K0164 |
| CE2.7 | Subsampled Laplacian calculation for highest temporal layer | JVET-K0327 |
| CE2.8 | Ranking based classification | JVET-K0285 |

# Detailed description of tests

#### CE2.1 7x7Cross+3x3Square filter shape and filter symmetry (JVET-K0239)

In JVET-K0239, the filter footprint is 9x7Cross+3x3Square and two kinds of symmetries are supported, as shown in below figures (a) and (b). According to the simulation results, the BD-rate increase is very minor. Therefore, in this CE test, the filter shape is further reduced to 7x7Cross+3x3Square for luma component. And the filter symmetries will also be tested. Chroma filters are 5x5 (the same as in VTM).

The filter shape in ALF with two coefficient symmetry characteristics. (a) 9x7Cross+3x3Square with half symmetry, 9 coefficients. (b) 9x7Cross+3x3Square with full symmetry, 4 coefficients.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE2.1.1 | Add full symmetry in 7x7 diamond filter | Yu-Chi Su (MediaTek) | Sung-Chang Lim (ETRI) |
| CE2.1.2 | Use 7x7C+3x3S with two symmetries instead of 7x7 diamond filter | Yu-Chi Su (MediaTek) | Sung-Chang Lim (ETRI) |

#### CE2.2 Filter coefficients coding (JVET-K0239)

In K0239, the filter coefficients are mapped from signed values to unsigned values first and coded by using 0-th order exp-Golomb code. In the contribution, filter coefficient range was limited to 10 bits.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE2.2.1 | Test this coefficient coding scheme applied for both luma and chroma filter coefficients | Yu-Chi Su (MediaTek) | Nan Hu (Qualcomm) |
| CE2.2.2 | Test limiting filter coefficients range to 10 bits, center coefficient is unsigned and other coefficients have a sign bin | Yu-Chi Su (MediaTek) |  |

#### CE2.3 Fixed, temporal filters and CU-level adaptivity (JVET-K0371/K0382)

In K0371, the number of fixed filters was reduced, temporal filters can be used to filter the current picture, and CU-level enable ALF flags are utilized. In K0382, CTB can have the following options of filters: (1) use the set of filters signalled for the current picture; (2) use temporal filter set derived from the previous pictures; (3) use a set of fixed filters from 16 available sets; (4) no filter applied.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE2.3.1 | Test the performance of fixed filters | Nan Hu (Qualcomm) |  |
| CE2.3.2 | Test the performance of the temporal filters | Nan Hu (Qualcomm) |  |
| CE2.3.3 | Test the performance when CU-level ALF enable flags are applied | Nan Hu (Qualcomm) |  |

#### CE2.4. Low latency encoder for ALF (JVET-K0382)

In VTM, ALF filters are derived for a picture and decision of using the derived filters for the picture or not is done on CTB basis.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE2.4.1 | Test using filters derived from the previous pictures to be used to filter the current picture for VTM | Nan Hu (Qualcomm) |  |
| CE2.4.2 | Test using filters derived from previous pictures to be used to filter the current picture with combination of CE2.3 tests | Nan Hu (Qualcomm) |  |

#### CE2.5 Filter coefficients restriction for bit shift operation (JVET-K0215)

In J0215, the filter coefficients used in filtering of the luma/chroma samples are divided into 2 groups. The first group contains that coefficients {L0, L1, L2, L3, L4, L5, L7, L8, L9, L10. L14, L15, L16, L17}, and the second group contains the rest of the coefficients. The first group coefficients are normatively restricted to use only the following set of values: {–64, –32, –16, –8, –4, 0, 4, 8, 16, 32, 64}, hence multiplication with those coefficients can be implemented as a single bit shift operation. The allowed filter coefficient values are mapped to {–5, –4, –3, –2, –1, 0, 1, 2, 3, 4, 5} respectively before binarization in order to reduce the signalling overhead.

The number of coefficients that apply the coefficient restriction as well as the coefficient value range will be tested according to new filter shapes in VTM2.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE2.5.1 | Test using two groups of filter coefficients | Semih Esenlik (Huawei) | Rahul Vanam (Interdigital) |

#### CE2.6 Subsampled Laplacian calculation (JVET-K0164)

4×4 block classification based on subsampled sum-modified-Laplacian (SSML) is used. On below figures (a), (b), (c), and (d), the positions of calculating 1-D Laplacian, for gradients, gv, gh, gd1 and gd2, respectively, for a 4×4 block are shown. All of them are calculated at the subsampled positions within 8×8 window. The rest of derivation process for class index with the calculated gradients, gv, gh, gd1 and gd2, is the same as in VTM.

(a) Calculation of vertical gradient, gv      (b) Calculation of horizontal gradient, gh

(c) Calculation of diagonal gradients, gd1       (d) Calculation of diagonal gradients, gd2

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE2.6.1 | Test subsampled positions for 4 gradients (vertical, horizontal, diagonal1, and diagonal2 gradients) are different according to each direction of gradient | Sung-Chang Lim (ETRI) | Yu-Chi Su (MediaTek) |
| CE2.6.2 | Test the unified subsampled positions D1 shown on figure (c) is used for calculation of all 4 gradients | Sung-Chang Lim (ETRI) | Yu-Chi Su (MediaTek) |
| CE2.6.3 | Test the unified subsampled positions D2 shown on figure (d) is used for calculation of all 4 gradients | Sung-Chang Lim (ETRI) | Yu-Chi Su (MediaTek) |

#### CE2.7 Subsampled Laplacian calculation for highest temporal layer (JVET-K0327)

In 4×4 block classification, the derivative calculation of 2×2 block for those pictures at the highest temporal layer is simplified by calculating the derivative only at top-left sample positionof each 2×2 block as follows

- G2×2dir=gdiri0,j0

where dir is the direction such as horizontal, vertical, diagonal and anti-diagonal, and gdiri0,j0 is the derivative at the top-left sample position of each 2×2 luma block.

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE2.7.1 | Test proposed modification applied for highest temporal layer | Rahul Vanam (InterDigital) | Semih Esenlik (Huawei) |

#### CE2.8 Ranking based classification (JVET-K0285)

In K0285, ranking based classification is proposed, which can operate in a switching manner. In the contribution, the classification is sample based. In CE tests, the classification is derived on 4x4 basis for the following two classifications.

Ranking based classification method #1:

For some pre-defined nonnegative weights wkl (with w00=1), the first ranking feature descriptor r1Bij is defined as

r1Bij=k=-11l=-11mBij<wkl*m(Bi+k,j+l)  ?  1  : 0 ,

where Bij is a local 4 × 4 block at location (i,j) as illustrated in the below figure and m(Bij) is the average value of the reconstructed samples in Bij. The descriptor r1Bij can take values in the range of 0 to 8, inclusive for each local 4 × 4 block Bij. After that, the value of r1Bij is quantized to 5 distinct values, 0,1,..,4. Then the dynamic range for average sample values mBij for 4 × 4 blocks Bij is divided into 5 bands with about equal size and inside each band, the quantized rank of the block Bij in its local neighborhood is used as additional criterion, which gives 5×5 = 25 classes.

Ranking based classification method #2:

The same procedure as method #1 is applied except that the second ranking feature descriptor r2Bij and local variation vBij are used instead of r1Bij and mBij. The feature descriptors r2Bij and vBij for each local 4 × 4 block Bij at each location (i,j) are given as

r2Bij=klmBij<wkl*sij(k,l)  ?  1  : 0

and

vBij=max⁡(mBij-max⁡(Bij),mBij-min⁡(Bij))

where sij(k,l) and max⁡(Bij) (min(Bij)) are the reconstructed samples and the maximum (minimum) sample value in Bij. The dynamic range for local variations vBij is divided into 5 bands which are specified by some pre-defined thresholds T1,T2,T3,T4.

Bi+1j-1

Bi+1j

Bi+1j+1

Bi-1j+1

Bi-1j

Bij+1

Bij-1

Bij

Bi-1j-1

Neighboring 4×4 blocks: Bij and its 8 neighboring 4×4 blocks Bi-1j-1, …, Bi+1j+1

| Test | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE2.8.1 | Test classification method #1 | Wang-Q Lim (HHI) | Nan Hu (Qualcomm) |
| CE2.8.2 | Test classificaiotn method #2 | Wang-Q Lim (HHI) |  |
| CE2.8.3 | Test switching between classification method #1 and method #2 | Wang-Q Lim (HHI) |  |
| CE2.8.4 | Test switching between Laplcation classification from VTM and method #1 | Wang-Q Lim (HHI) |  |
| CE2.8.5 | Test switching between Laplcation classification from VTM and method #2 | Wang-Q Lim (HHI) |  |

# Test conditions

CTC conditions as agreed at the 11th JVET meeting (Ljubljana, Slovenia) for SDR will be used for the evaluation of objective performance (JVET-K1010). The software basis and anchor is VTM-2, while BMS based tests are optional.

Proposals will be compared with respect to objective compression efficiency while complexity will also be assessed, as discussed in BoG JVET-K0521. The following table is used to summarize the computation complexity of each test.

| Test | Operations of classification process per luma sample | Operations of filtering process per luma sample | Operations of filtering process per chroma sample | Frequency change of filters (Y, Cb, Cr) | Line buffer size (L, C) | memory for storing filters | memory access for luma | memory access for chroma |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Anchor |  |  |  |  |  |  |  |  |

# Timeline

| T1 | End of 11th JVET meeting + 3 weeks: August 8, 2018 | CE test description finalized |
| --- | --- | --- |
| T2 | VTM2.0 release + 2 weeks: August 29, 2018 | Initial software of CE tests released; initial study by cross-checkers begins |
| T3 | 3 weeks before 12th JVET meeting: September 12, 2018 | Software of CE tests is finalized, and full test results are provided; final cross-check begins |
| T4 |  | Document upload: test results, cross-check reports, CE summary report |

# Appendix of Participants

| # | Name | Company | E-mail | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
| 1 | Yu-Chen Sun | Alibaba | yuchen.s@alibaba-inc.com |  |  |
| 2 | Alexis Tourapis | Apple | atourapis@apple.com |  |  |
| 3 | Jae Hoon Kim | Apple | i.jaehoon.kim@icloud.com |  |  |
| 4 | Li Zhang | ByteDance | lizhang.idm@bytedance.com |  |  |
| 5 | Wade Wan | Broadcom | wade.wan@broadcom.com |  |  |
| 6 | Peisong Chen | Broadcom | peisong.chen@broadcom.com |  |  |
| 7 | Minhua Zhou | Broadcom | minhua.zhou@broadcom.com |  |  |
| 8 | Jonathan TAQUET | Canon | Jonathan.TAQUET@crf.canon.fr |  |  |
| 9 | Guillaume LAROCHE | Canon | guillaume.laroche@crf.canon.fr |  |  |
| 10 | Patrice Onno | Canon | patrice.onno@crf.canon.fr |  |  |
| 11 | Dan Grois | Comcast | groisz@yahoo.com |  |  |
| 12 | Yongjo Ahn | Digital Insights | yjahn@digitalinsights.co.kr |  |  |
| 13 | Peng Yin | Dolby | pyin@dolby.com |  |  |
| 14 | Jungwon Kang | ETRI | jungwon@etri.re.kr | X | X |
| 15 | Sung-Chang Lim | ETRI | sclim@etri.re.kr | X | X |
| 16 | Johannes Erfurt | HHI | johannes.erfurt@hhi.fraunhofer.de | X |  |
| 17 | Wang-Q Lim | HHI | wang.lim@hhi.fraunhofer.de | X |  |
| 18 | Li Wang | Hikvision | wangli7@hikvision.com |  |  |
| 19 | Jianhua Zheng | HiSilicon | zhengjianhua@hisilicon.com |  |  |
| 20 | Quanhe Yu | HiSilicon | yuquanhe@hisilicon.com |  |  |
| 21 | Haechul Choi | HNU | choihc@hanbat.ac.kr |  |  |
| 22 | Heeji Han | HNU | sarhakor@naver.com |  |  |
| 23 | Daehyeok Gwon | HNU | skyeye0530@naver.com |  |  |
| 24 | Jaeryun Choe | HNU | tjswo9395@naver.com |  |  |
| 25 | Alexey Filippov | Huawei | alexey.filippov@huawei.com | X | X |
| 26 | Vasily Rufitskiy | Huawei | vasily.rufitskiy@huawei.com | X | X |
| 27 | Sergey Ikonin | Huawei | sergey.ikonin@huawei.com | X | X |
| 28 | Victor Stepin | Huawei | stepin.victor@huawei.com | X | X |
| 29 | Roman Chernyak | Huawei | chernyak.roman@huawei.com | X | X |
| 30 | Semih Esenlik | Huawei | semih.esenlik@huawei.com | X | X |
| 31 | Zhipin Deng | Intel | zhipin.deng@intel.com |  |  |
| 32 | Xu Lidong | Intel | lidong.xu@intel.com |  |  |
| 33 | Rahul Vanam | InterDigital | Rahul.vanam@InterDigital.com | X | X |
| 34 | Yuwen He | InterDigital | Yuwen.He@InterDigital.com | X |  |
| 35 | Po-Han Lin | ITRI | pohan@itri.com |  |  |
| 36 | Ching-Chieh Lin | ITRI | JackLin@itri.com |  |  |
| 37 | Sriram Sethuraman | Ittiam | sriram.sethuraman@ittiam.com |  |  |
| 38 | Yoshitaka Kidani | KDDI | yo-kidani@kddi.com |  |  |
| 39 | Yi-Wen Chen | Kwai | yiwenchen@kwai.com |  |  |
| 40 | Xianglin Wang | Kwai | xianglinwang@kwai.com |  |  |
| 41 | Junghak Nam | LGE | junghak.nam@lge.com |  |  |
| 42 | Hyeongmun Jang | LGE | hm.jang@lge.com |  |  |
| 43 | Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com | X |  |
| 44 | Ching-Yeh Chen | MediaTek | chingyeh.chen@mediatek.com | X |  |
| 45 | Tzu-Der Chuang | MediaTek | peter.chuang@mediatek.com | X |  |
| 46 | Yu-Chi Su | MediaTek | yu-chi.su@mediatek.com | X |  |
| 47 | Chen-Yen Lai | MediaTek | jenny.lai@mediatek.com | X |  |
| 48 | Andrey Norkin | Netflix | anorkin@netflix.com |  |  |
| 49 | Alireza Aminlou | Nokia | alireza.aminlou@nokia.com |  |  |
| 50 | Tadamasa Toma | Panasonic | toma.tadamasa@jp.panasonic.com |  |  |
| 51 | Vadim Seregin | Qualcomm | vseregin@qti.qualcomm.com | X | X |
| 52 | Nan Hu | Qualcomm | nanh@qti.qualcomm.com | X | X |
| 53 | Dmytro Rusanovskyy | Qualcomm | dmytror@qti.qualcomm.com | X | X |
| 54 | Akshay Gadde | Qualcomm | gadde@qti.qualcomm.com | X | X |
| 55 | Marta Karczewicz | Qualcomm | martak@qti.qualcomm.com | X | X |
| 56 | Woongil Choi | Samsung | woongil.choi@samsung.com |  |  |
| 57 | Kiho Choi | Samsung | kiho14.choi@samsung.com |  |  |
| 58 | Jie Chen | Samsung | jie1222.chen@samsung.com |  |  |
| 59 | Byeungwoo Jeon | SKKU | bjeon@skku.edu |  |  |
| 60 | Jeeyoon Park | SKKU | jiyoonpark@skku.edu |  |  |
| 61 | Eiichi Sasaki | Sharp | eiichi.sasaki@sharp.co.jp |  |  |
| 62 | Takeshi Chujoh | Sharp | chujoh.takeshi@sharp.co.jp |  |  |
| 63 | Yukinobu Yasugi | Sharp | yasugi.yukinobu@sharp.co.jp |  |  |
| 64 | Tomohiro Ikai | Sharp | ikai.tomohiro@sharp.co.jp |  |  |
| 65 | Byeongdoo Choi | Sharp | choib@sharplabs.com |  |  |
| 66 | Andrew Segall | Sharp | asegall@sharplabs.com |  |  |
| 67 | Kiran Misra | Sharp | misrak@sharplabs.com |  |  |
| 68 | Masaru Ikeda | Sony | Masaru.Ikeda@sony.com |  |  |
| 69 | Philippe Bordes | Technicolor | Philippe.bordes@technicolor.com |  |  |
| 70 | Philippe De Lagrange | Technicolor | Philippe.delagrange@technicolor.com |  |  |
| 71 | Edouard Francois | Technicolor | edouard.francois@technicolor.com |  |  |
| 72 | Greg Ko | WILUS | greg.ko@wilusgroup.com |  |  |