| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K1024-v1 |
| --- | --- |

| Title: | Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Haitao Yang Shan Liu Kai Zhang | Email: | haitao.yang@huawei.com shanl@tencent.com zhangkai.video@bytedance.com |
| Source: | CE coordinators |  |  |

_____________________________

- Abstract

The goal of Core Experiment 4 (CE4) is to investigate various inter prediction and motion vector coding techniques on top of VTM and BMS. It comprises 4 categories, 1) affine motion compensation, 2) affine merge mode, 3) merge mode enhancement, 4) motion vector coding, 5) reference picture boundary padding, 6) other topics.

CE4.1: Affine motion compensation

Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 4.x.x | Cross-model inheritance for affine candidate derivation | K0185 | H. Chen (Huawei) |  |
| 4.x.x | Simplification of affine AMVP list construction | K0185 | H. Chen (Huawei) |  |
| 4.x.x | Simplified affine MVP list construction | K0094 | J. Lee (LGE) |  |
| 4.x.x | Spatial & temporal affine AMVP candidate derivation a) Additional spatial candidates {a2, b3} for predicting the affine motion model of current CU in AMVP affine only b) Additional virtual temporal candidate to predict the Affine motion model of current CU in AMVP affine only - 4x4 resolution for temporal buffer - 8x8 resolution for temporal buffer - 16x16 resolution for temporal buffer | K0267 | A. Robert (Technicolor) |  |
| 4.x.x | Shape dependent control points selection for affine MVP | K0335 | Y. He (InterDigital) |  |
| 4.x.x | a) by-pass coding of the flag for 4/6-param model switching at block level b) conditional signaling of 6-param model at block level c) slice level switching of 4/6-param affine model | K0095 | J. Lee (LGE) |  |
| 4.x.x | Slice-level 4-parameters/6-parameters switching | J1024 | A. Robert (Technicolor) |  |
| 4.x.x | Slice level 4/6 affine parameter model switch | K0337 | Y. Han (Qualcomm) |  |
| 4.x.x | 1) line buffer reduction for affine inherited candidates, location 1 2) line buffer reduction for affine inherited candidates, location 2 | K0052 | M. Zhou (Broadcom) |  |
| 4.x.x | Simplification of affine AMVP list construction combined with line buffer reduction | K0185 | H. Chen (Huawei) |  |
| 4.x.x | CTU restriction on affine inherited candidates for line buffer reduction | K0339 | Y. Han (Qualcomm) |  |
| 4.x.x | The sub-block size for chroma components is expanded from 2×2 to 4×4. | K0103 | K. Zhang (Bytedance) |  |
| 4.x.x | 3(scaling)/3(rotation)/4/6 model switching | K0124 | K. Kondo (Sony) |  |

Test condition and evaluation criteria

CTC applied here.

For tests modify affine MVP list construction, the complexity analysis of list construction should be provided and crosschecked. Generally speaking, the complexity should not be higher than that of VTM affine MVP list construction if no distinguishable coding gain is observed.

| Number of | Max number of |  |  | Operations |  |  |  | Buffer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| candidates in the final list | inherited affine candidate | constructed affine candidate | candidate comparison | add | sht. | cmp. | scal. | additional buffers |

Technical description

Cross-model inheritance for MERGE/AMVP candidate derivation (JVET-K0185)

In this contribution, six-parameter motion model represented by three control points is proposed. The 4-parameter and 6-parameter motion models are selected at the CU level by using an additional affine motion type flag.

4-parameter motion model is converted to 6-parameter affine model by using 4-parameter model formula to generate the motion vector of bottom-left control point. During MC and motion vector derivation for sub-blocks, unified 6-parameter affine model is used.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | Cross-model inheritance for affine candidate derivation | H. Chen (Huawei) |  |

Simplification of affine AMVP list construction (JVET-K0185)

The affine AMVP candidate list is constructed as follow:

- Inherited affine candidates

Up to two candidates derived from the affine motion models of neighboring affine coded blocks are inserted into the candidate list. The scan order for this category of candidates is same as AMVP in HEVC. For the left predictor, the scan order is: {A0, A1}. For the above scan predictor, the order is: {B0, B1, B2}.

- Constructed affine candidates

If the number of inherited affine candidates is less than two, different combinations of the motion vector of neighbor coded block of control points are inserted into the candidate list until the number of candidate is two.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | Simplification of affine AMVP list construction | H. Chen (Huawei) |  |

Motion vector prediction in Affine AMVP (JVET-K0218, JVET-K0219, JVET-K0267, JVET-K0268)

Affine candidate derivation

The addition of spatial candidates into the list of affine motion models used to predict the affine motion model of current CU has been tested in JVET-K0218, JVET-K0267 and JVET-K0268. It will be tested in this SubCE in the scope of Affine AMVP. As an example, candidate positions {a2,b3} will be tested.

Figure 1. Positions of spatial and temporal candidates of motion information.

The use of additional temporal candidates to predict the affine motion model of current CU has been proposed in JVET-K0219, JVETK0267 and JVET-K0268. This takes the form of either temporal inheritance of affine motion seeds (which involves the storage of affine seeds in the temporal picture buffer), or the construction of candidate affine seeds based on temporal non-affine motion vector predictors.

In the latter case, positions {a0, b0, c0} for temporal vector predictors will be tested, as in contribution JVET-K0267. Alternative positions may be tested.

Furthermore, with regards to the additional temporal affine candidates, various resolution levels for the temporal buffer (4x4, 8x8, and 16x16) will be tested as in JVET-K0267.

Figure 2: Position of virtual temporal candidates for affine model construction

Slice-level 4-parameters/6-parameters switching

The technique described in previous section will be tested with and without the slice-level switching between 4-parameters and 6-parameters, that was proposed in test 4.1.3.d of JVET-J01024.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | Spatial & temporal affine AMVP candidate derivation a) Additional spatial candidates {a2, b3} for predicting the affine motion model of current CU in AMVP affine only b) Additional virtual temporal candidate to predict the Affine motion model of current CU in AMVP affine only - 4x4 resolution for temporal buffer - 8x8 resolution for temporal buffer - 16x16 resolution for temporal buffer c) a)+b) | A. Robert (Technicolor) |  |
| 4.x.x | Slice-level 4-parameters/6-parameters switching | A. Robert (Technicolor) |  |

Line buffer reduction for affine model inheritance (JVET-K0052)

Figure 3: shared motion data line buffer for affine merge and regular merge/skip mode (e.g. 4-parameter affine motion model)

As shown in Figure 3, if the selected neighboring PU is located at the top CTU boundary, the motion vectors stored in the regular motion data line buffer rather than the control point motion vectors of the selected PU are used for derivation of the seed motion vectors of the current PU of the affine merge mode. For example in Figure 3, if the current PU uses the affine merge mode and inherits the affine motion data from the neighboring PU E located at top CTU boundary, then motion vectors in the regular motion data line buffer, i.e.  vLE0=vLE0x,vLE0y at sample position xLE0,yLE0 and   vLE1=(vLE1x,vLE1y) at the sample position xLE1,yLE1 with yLE1=yLE0, instead of affine seed motion vectors of the neighboring PU E, i.e. vE0=vE0x,vE0y at the top-left sample position xE0,yE0 and  vE1=(vE1x,vE1y) at the top-right sample position xE1,yE1, are used for the derivation of control point vectors v0 and v1 of the current PU.

Motion vectors vLE0 and vLE1 used for motion compensation of the bottom-left and bottom-right sub-blocks of PU E is calculated by using the 4-parameter or 6-parameter affine motion model, depending on the affine motion model used by the neighboring PU E.

The control point motion vectors v0 and v1 of the current PU coded in affine merge mode are derived by the 4-parameter affine motion mode regardless of whether the neighboring PU E uses the 4-parameter or 6-parameter affine motion model.

v0x=vLE1x-vLE0x(xLE1-xLE0)(x0-xLE0)-vLE1y-vLE0y(xLE1-xLE0)(y0-yLE0)+vLE0xv0y=vLE1y-vLE0y(xLE1-xLE0)(x0-xLE0)+vLE1x-vLE0x(xLE1-xLE0)(y0-yLE0)+vLE0y

v1x=vLE1x-vLE0x(xLE1-xLE0)(x1-xLE0)-vLE1y-vLE0y(xLE1-xLE0)(y1-yLE0)+vLE0xv1y=vLE1y-vLE0y(xLE1-xLE0)(x1-xLE0)+vLE1x-vLE0x(xLE1-xLE0)(y1-yLE0)+vLE0y

The same method is applied to the inherited affine motion candidate derivation if the current PU is located along the top CTU boundary.

No change is introduced if the current PU is not located along the top CTU boundary.

Refer to the example depicted in Figure 3, in the tests xLE0 is set to xE0 and xLE1 is set to xE1 to avoid non-power of 2 computation in equations above. Two tests will be conducted:

- yLE1=yLE0=y0 - 1

- yLE1=yLE0=y0 - 3

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | 1) line buffer reduction for affine inherited candidates, location 1 2) ine buffer reduction for affine inherited candidates, location 2 | M. Zhou (Broadcom) |  |

Alternative scaling and rotation model (JVET-K0124)

An encoder can choose four modes which are a conventional translate, scaling, rotation and Affine. A number of parameters are 2 (one motion vector), 3, 3 and 4 (two MVs) for translate mode, scaling mode, rotation mode and Affine mode.

As shown in the below figure, the motion model index is defined and it is sent to the decoder. The number in parenthesizes shows the number of parameters of differential motion vector that are sent to a decoder.

When the scaling mode is chosen, three parameters are used to predict as shown in the below figure. It is one motion vector (v0x, v0y) and one element (v1x). For the prediction process, the frame work is used the same as for Affine transform in JEM software. The parameter (v1y) is implicitly assumed as the same as (v0y).

A framework that switching between 3(scaling)/3(rotation)/4(simplified Affine)/6(Affine) is tested. In Ljubljana meeting, 4/6 parameter model was adopted. In this test A, to further study to combine with 3 parameter model, it’s  implemented on the 4/6 parameter model.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | 3(scaling)/3(rotation)/4/6 model switching | K. Kondo (Sony) |  |

4/6-param model switching (JVET-K0095)

The following three approaches are proposed and tested in CE4. The flag for 4/6-param model switching at block is by-pass coded to reduce the CABAC throughput. 6-param affine model is allowed at block-level only when a neighboring block is in affine mode. Slice-level 4/6-param model switching is proposed.

The following 3 methods are tested:

- by-pass coding of the flag for 4/6-param model switching at block level;

- conditional signaling of 6-param model at block level;

- slice level switching of 4/6-param affine model.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | a) by-pass coding of the flag for 4/6-param model switching at block level b) conditional signaling of 6-param model at block level c) slice level switching of 4/6-param affine model | J. Lee (LGE) |  |

Block size restriction on affine MC for chroma component (JVET-K0103)

In this contribution, the sub-block size for chroma components is expanded from 2×2 to 4×4.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | The sub-block size for chroma components is expanded from 2×2 to 4×4 | K. Zhang (Bytedance) |  |

Shape dependent control point selection for affine mode (JVET-K0335)

To solve the MV interpolation accuracy problem of affine mode, control points are selected based on coding block shape for the affine coding mode. If coding block width is greater than or equal to coding block height, the control points are defined by top-left corner (P0) and top-right corner (P1) as shown in the following figure (a); if coding block width is smaller than coding block height, the control points are defined by top-left corner (P0) and bottom-left (P2) corner as shown in following figure (b).

(a)                           (b)

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | Shape dependent control points selection for affine MVP | Y. He (InterDigital) |  |

Slice level adaptive affine model selection (JVET-K0337)

4-parameter affine model with two control points Figure 1 (a), and an additional 6-parameter affine model with three control point MVs Figure 1 (b), are used.

(a) 				         (b)

Figure 1: 4-paramenter affine model (a) and 6-parameter affine model (b).

When the 6-parameter affine model is applied, the current block is divided into sub-blocks in the same way as for the 4-parameter affine model in JEM. The motion vector mv=[mvx,mvy] at a location (x,y) in W×H block can be derived from the MVs of the three control points (mv0, mv1, and mv2) as follows

mv x=mv1x-mv0xwx+mv2x-mv0xhy+mv0x

mv y=mv1y-mv0ywx+mv2y-mv0yhy+mv0x.

The affine models are adaptively selected at slice level on top of the CU level 4/6 affine parameter model switch design from JVET-K0185 as following: The 4-parameter and 6-parameter motion models are selected at the CU level by using an additional affine motion type flag. 4-parameter motion model is converted to 6-parameter affine model by using 4-parameter model formula to generate the motion vector of bottom-left control point. During MC and motion vector derivation for sub-blocks, unified 6-parameter affine model is used.

Slice header flags affine4_flag and affine6_flag indicate whether the 4-parameter affine and 6-parameter affine model are applied. If one of them is equal to 1, affine_flag signaled at block level indicates whether the allowed affine model is used for this block. If both are equal to 1, there is an affine_type flag signaled at block level. If affine_type is equal to 1, the 6-parameter affine model is used, otherwise the 4-parameter affine model is applied. To determine the affine4_flag and affine6_flag for the current slice the statistics from the previous slice are used.

The following test will be conducted:

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | Slice level 4/6 affine parameter model switch | Y. Han (Qualcomm) |  |

Simplified affine MVP list construction (JVET-K0094)

The following affine MVP candidate list construction is simplified and tested in CE4.

- Inherited affine candidate with no-scaling and scanning order

- Constructed affine candidate with simplification

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | Simplified affine MVP list construction | J. Lee (LGE) |  |

CE4.2: Affine merge mode

Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |
| 4.x.x | Common base for affine merge mode | - | H. Chen (Huawei) |  |
| 4.x.x | Simplification of the common base a) simplify affine merge list construction by reducing pruning operation b) simplify affine merge list construction by reducing MV scaling operation c) simplify affine merge list construction by reducing maximum number of candidates d) reduce line buffer in affine merge mode d) a)+b)+c) | K0186 | H. Chen (Huawei) |  |
| 4.x.x | Modified affine merge candidates | K0245 | T.-D. Chuang (MediaTek) |  |
| 4.x.x | Spatial & temporal merge candidate derivation a) Additional spatial candidates {a2, b3} for predicting the affine motion model of current CU in merge affine only b) Additional virtual temporal candidate to predict the Affine motion model of current CU in merge affine only - 4x4 resolution for temporal buffer - 8x8 resolution for temporal buffer - 16x16 resolution for temporal buffer | K0267 | F. Galpin (Technicolor) |  |
| 4.x.x | a) simplified pruning process for affine merge list construction b) reduced maximum number of candidates for affine merge list construction c) a)+b) | K0339 | Y. Han (Qualcomm) |  |
| 4.x.x | a) Inherited affine candidate with simplification and unification b) a) + optimization and simplification of the common base | K0095 | J. Lee (LGE) |  |
| 4.x.x | Shape dependent control points selection for affine MVP & merge | K0335 | Y. He (InterDigital) |  |
| 4.x.x | Line buffer reduction for affine inherited candidates (both MVP and merge) | K0052 | M. Zhou (Broadcom) |  |

Test condition and evaluation criteria

CTC applied here.

Common base: separate list for affine merge candidates, 5 candidates, both inherited and constructed candidates.

Modifications of each single piece of the common base is suggested to be tested separately, including modified or additional inherited affine candidates, modified or additional constructed affine candidates, additional motion models, the number of candidates in the list, the level of encoder and RD optimizations.

For example, in case of testing modified or additional inherited affine candidate, the constructed affine candidates in the common base should be kept unchanged; in case of testing modified or additional constructed affine candidate, the inherited affine candidates in the common base should be kept unchanged.

Generally, RD optimization in the separate affine merge list should not be changed. And the candidate number should be aligned among all tests. Any change on RD optimization should be reported.

The complexity analysis of list construction should be provided and crosschecked.

| Number of | Max number of |  |  | Operations |  |  |  | Buffer |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| candidates in the final list | inherited affine candidate | constructed affine candidate | candidate comparison | add | sht. | cmp. | scal. | additional buffers |

Technical description

Common base for affine merge mode

The common base is based on JVET-K0186. In the common base for affine merge enhancement, the affine merge candidate list is constructed as following steps:

- Insert inherited affine candidates into candidate list

Figure 1 Candidates position for affine merge mode

Inherited affine candidate means that the candidate is derived from the valid neighbor reconstructed block coded with affine mode.

In the common base, as shown in Figure 1, the scan order for the candidate block is A1, B1, B0, A0 and B2.

- Insert constructed affine candidates

If the number of candidates in affine merge candidate list is less than MaxNumAffineCand, constructed affine candidates are insert into the candidate list.

Constructed affine candidate means the candidate is constructed by combining the neighbor motion information of each control point.

The motion information for the control points is derived firstly from the specified spatial neighbors and temporal neighbor shown in Figure 2. CPk (k=1, 2, 3, 4) represents the k-th control point. A0, A1, A2, B0, B1, B2 and B3 are spatial positions for predicting CPk (k=1, 2, 3); T is temporal position for predicting CP4.

The coordinates of CP1, CP2, CP3 and CP4 is (0, 0), (W, 0), (H, 0) and (W, H), respectively, where W and H are the width and height of current block.

The motion information of each control point is obtained according to the following priority order:

- For CP1, the checking priority is B2->B3->A2. B2 is used if it is available. Otherwise, if B2 is available, B3 is used. If both B2 and B3 are unavailable, A2 is used. If all the three candidates are unavailable, the motion information of CP1 cannot be obtained.

- For CP2, the checking priority is B1->B0;

- For CP3, the checking priority is A1->A0;

- For CP4, T is used.

Secondly, the combinations of controls points are used to construct the motion model.

Motion vectors of three control points are needed to compute the transform parameters in 6-parameter affine model. The three control points can be selected from one of the following four combinations ({CP1, CP2, CP4}, {CP1, CP2, CP3}, {CP2, CP3, CP4}, {CP1, CP3, CP4}). For example, use CP1, CP2 and CP3 control points to construct 6-parameter affine motion model, denoted as Affine (CP1, CP2, CP3).

Motion vectors of two control points are needed to compute the transform parameters in 4-parameter affine model. The two control points can be selected from one of the following six combinations ({CP1, CP4}, {CP2, CP3}, {CP1, CP2}, {CP2, CP4}, {CP1, CP3}, {CP3, CP4}). For example, use the CP1 and CP2 control points to construct 4-parameter affine motion model, denoted as Affine (CP1, CP2).

The combinations of constructed affine candidates are inserted into to candidate list as following order:

{CP1, CP2, CP3}, {CP1, CP2, CP4}, {CP1, CP3, CP4}, {CP2, CP3, CP4}, {CP1, CP2}, {CP1, CP3}, {CP2, CP3}, {CP1, CP4}, {CP2, CP4}, {CP3, CP4}

- Insert zero motion vectors

If the number of candidates in affine merge candidate list is less than MaxNumAffineCand, zero motion vectors are insert into the candidate list, until the list is full.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | Common base of affine merge enhancement | H. Chen (Huawei) |  |

Affine merge enhancement (JVET-K0186)

Various simplification of the common base are tested.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | Simplification of the common base a) simplify affine merge list construction by reducing pruning operation b) simplify affine merge list construction by reducing MV scaling operation c) simplify affine merge list construction by reducing maximum number of candidates d) a)+b)+c) | H. Chen (Huawei) |  |

Motion vector prediction in Merge Affine (JVET-K0245)

In this test, the combined candidate list in proposed. The affine candidates are added to merge candidate list. There are two kinds of affine candidates. The first one is spatial inherited affine candidate. The second one is corner derived affine candidates. Spatial inherited affine candidates use affine model from one of neighboring coding units (CUs) located at A1, B1, B0, A0, and B2 to derive current CU’s affine model as shown in Figure 1. In this proposal, spatial inherited affine candidates only use 6-parameter affine model. For each spatial location, if the spatial inherited affine candidate is available, then its original spatial inherited candidates will not be added to the merge candidate list. The spatial affine candidates searching order goes as A1  B1  B0 A0 B2.

Corner derived affine candidates simply set three spatial candidates as control points. For example, the motion vector (MV) sets of block {(B2, A1, B1), (B2, A0, B1), (B2, A1, B0), (B2, A0, B0)} as shown in Figure 1. Available motion triplets of spatial neighbors with the same reference picture are inserted. For each inserted affine candidate, no pruning and partial pruning will be tested. For example, each affine candidate only check the redundancy with the first available affine candidate. The complexity analysis of how many add/shift/comparison/mult are used will be provided.

Figure 1. Position for spatial inherited affine and corner derived affine candidates.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | a) Unified merge list with only inherited affine candidates, candidate number is 10 b) Unified merge list with only inherited affine candidates and corner derived affine candidates, candidate number is 10 | T.-D. Chuang (MediaTek) |  |

Motion vector prediction in Merge Affine (JVET-K0218, JVET-K0219, JVET-K0267, JVET-K0268)

The tests driven in this section will be performed on top of the common base for affine merge mode.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | Additional spatial & temporal affine merge candidate derivation of the common base a) Additional spatial candidates {a2, b3} for predicting the affine motion model of current CU in merge affine only b) Additional virtual temporal candidate to predict the Affine motion model of current CU in merge affine only - 4x4 resolution for temporal buffer - 8x8 resolution for temporal buffer - 16x16 resolution for temporal buffer c) a)+b) | F. Galpin (Technicolor) |  |

Affine merge candidate list construction (JVET-K0339)

Two categories of affine merge candidates are considered and pruned before inserted into the affine merge candidate list which is constructed separately from the normal merge candidate list.

The first category of affine merge candidate is inherited affine merge candidate and could be further divided into two sub-categories depending the position of the merge candidates. In Figure 2, index 1-5 indicates the adjacent spatial affine merge candidate positions while index 6-49 represent the non-adjacent affine merge candidate positions. Those affine merge candidates are checked in ascending index order. If the block is coded using affine mode and the motion information is different from other affine merge candidates in the affine merge candidate list, the motion information will be inserted into the affine merge candidate list.

The second category of affine merge candidate is called virtual affine merge candidates where affine merge candidate constructed as a combination of motion vectors from spatial neighboring blocks. The neighboring MVs are divided into three groups: S0=mvA,mvB,mvC , S1=mvD,mvE and S2=mvF,mvG. The motion vector in each group is scanned in a given order and the first available motion vectors is selected as the corresponding control point motion vector. A total number of 12 possible combinations could be produced from this category.

The target reference picture is determined as follows. If there is a reference picture that is simultaneously referred to by motion vectors in S0, S1 and S2, then such a reference picture with the smallest reference index is selected as the target one; if there is a reference picture that is simultaneously referred to by motion vectors in S0 and in S1, then such a reference picture with the smallest reference index is selected as the target one; if there is a reference picture that is simultaneously referred to by motion vectors in S0 and in S2, then such a reference picture with the smallest reference index is selected as the target one; otherwise, the virtual affine merge candidate is unavailable.

Figure 2: Spatial neighboring blocks used to derive the spatial merge candidates.

Figure 3: Virtual affine merge candidate derivation from motion vectors of neighboring blocks.

Restricted and simplified affine merge list construction methods will be tested.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | a) CTU restriction on affine merge candidates b) simplified pruning process for affine merge list construction c) reduced maximum number of candidates for affine merge list construction d) a)+b)+c) | Y. Han (Qualcomm) |  |

Affine merge improvement (JVET-K0094)

Up to two inherited affine merge candidate can be considered on the top of the common base. Simplification and unification approached are considered. And the common base is considered with the proposed inherited candidate in terms of the optimization and simplification.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | a) Inherited affine candidate with simplification and unification b) a) + optimization and simplification of the common base | J. Lee (LGE) |  |

CE4.3: Merge mode enhancement

Test specification

| Test # | Description | Source | Tester | Crosschecker |
| --- | --- | --- | --- | --- |

Test condition and evaluation criteria

CTC applied here.

For test on long distance merge mode, several constrains are provided:

- No additional line buffer

- Max number of potential additional candidates ???

- Max number of operations/comparisons/conditions in the pruning ???

- Max merge candidates in the list (list size shall be constant) (6, or 8 ???)

Technical description

| Proposal | Candidate derivation | Tester | Crosschecker |
| --- | --- | --- | --- |
| K0532/K0161 (Sharp) | Spatial-temporal merge mode (non subblock STMVP) | Sharp |  |
| K0286/K0297 (Tencent) | Long distance merge mode | Jing Ye jingye@tencent.com |  |
| K019(Hisilicon) | Merge mode enhancement | Hisilicon |  |
| K0301 | Extension of merge and AMVP candidates for inter prediction | Guichun Li guichunli@tencent.com |  |
| K0304 | Ranking based spatial merge candidate list for inter prediction | Guichun Li guichunli@tencent.com |  |
| K0244/K0245/K0246 | General Merge Mode | MediaTek |  |
| K0065 (Hikvision) | All spatial candidates in BMS/VTM | Liying Xu (Hikvision) |  |
| K0056 | Merge mode modification | Timofey Solovyev (Huawei) |  |

Spatial-temporal merge mode (non subblock STMVP) in K0532/K0161 (Sharp)

In this proposal, no subblock STMVP is proposed as a spatial-temporal merge mode. This proposed method refers motion vectors of a collocated block and two spatial positions and derive the spatial temporal merge candidate by an average of motion vectors of above block, left block and a temporal block.

As for spatial block, at most following two spatial positions are checked for above side. Same apply for the left side.

Max candidate number is 7 in this test.

Note: As suggested in BoG. the VVC/HEVC default special merge positions (A0, A1, B0, B1, B2) can be used as an alternative (replacement) as middle position (i.e. Amid, Lmid)..

Afar: (nPbW * 5 / 2, -1), Amid (nPbW / 2, -1)

Lfar: (-1, nPbH * 5 / 2), Lmid (-1, nPbH/2)

Note: VVC/HEVC default position of spatial candidate is shown below.

Long distance merge candidates in K0286 and K0297 (Tencent)

Figure 1 additional merge candidates

The contribution proposes to derive additional spatial merge candidates from positions in an outer reference area which has an offset of (-96, -96) to the current block without any line buffer increasing.

As shown in Figure 1, the positions are marked as Ai, Bi, Ci, Di and Ei. The candidates are checked from inside to the outside. And the order of the candidates is Ai, Bi, Ci, Di, and Ei. The maximum searching rounds (maximum i) will be tested. Different pruning method will be tested. The memory access to the left side of current CU with different constrain will be tested.

Merge mode enhancement JVET-K0198 (Hisilicon)

Part A: Non-adjacent spatial candidates

More spatial positions are checked as shown in Figure. The extended spatial positions from 6 to 27 are checked according to their numerical order after the temporal candidate. In order to save the MV line buffer, all the spatial candidates are restricted within two CTU lines. That is, the spatial candidates beyond the CTU line above the current CTU line are excluded.

Key points for Part A

Additional 6 merge candidates are added

New candidates are appended to the end of the current merge list

Redundancy checking is performed for the added merge candidates

Test Specifications

Test with appropriate additional merge candidates number.

Take line buffer restriction into consideration for additional merge candidates.

Check the performance based on VTM and BMS.

--------------------------------------------------------------------------

Part B: Merge offset extension

In Merge offset extension, more candidates are checked based on existed candidate. New candidates has mv offset to the mv of existed candidate. New extended mv offset candidates are constructed only based on the first candidate of merge candidate list.

Key points for Part B

- Additional 8 merge candidates are added.

- Cross direction offset is checked firstly, and then X direction offsets.

- New candidates are appended to the end of the current merge list.

Test Specifications

Test how much gain come from symmetric coding.

Test with fewer additional merge candidates.

Check the performance based on VTM and BMS.

--------------------------------------------------------------------------

Part C: Combined Average Merge Candidates

- The first 4 candidates in the merge list is used for deriving the new candidates

- The reference picture in L0 and L1 is decided by voting, separately

- Combination of the 4 candidates is performed to derive 6 pair candidates

- For each pair of candidates, L0 and L1 motion vectors are averaged separately

Extension of merge and AMVP candidates for inter prediction JVET-K0301 (Tencent)

It is proposed to add two spatial neighboring positions, which are from the middle position at the left and top edge of the current block, into motion vector predictor candidate list. The new candidates are put at the beginning of the merge candidate list and the AMVP candidate list, if available. The number of allowed merge candidates or AMVP candidates is kept unchanged.

Tests:

- Add merge candidates at middle position for inter merge;

- Add new AMVP predictor candidates from middle position;

- Combining middle candidates for merge and AMVP;

- Evaluate interaction with other merge/amvp tools in VTM/BMS

Ranking based spatial merge candidate list for inter prediction JVET-K0304 (Tencent)

it is proposed to replace the original spatial merge candidate list construction method with a ranking based method and the positions of merge candidates are extended to include more potential candidates. It is proposed to sort spatial candidates in descending order of occurrence. The spatial candidates can be fetched from an L-shaped area around the current block.  The proposed spatial candidates are derived and put at the beginning of the merge candidate list. The number of maximum merge candidates is kept unchanged. Other non-spatial merge candidates are added afterwards until the merge list is full.

Tests:

- Ranking based spatial merge list;

- Evaluate the impact of candidate search range;

- Evaluate interaction with other merge related tools in VTM/BMS, e.g., temporal candidates, ATMVP,

General Merge Mode JVET-K0244/JVET-K0245/JVET-K0246 MediaTek

Pairwise average candidates as replacement of HEVC combined candidates

Pairwise average candidates are generated by averaging predefined pairs of candidates in the current merge candidate list. The averaged motion vectors are calculated separately for each reference list; if both MVs are available in one list, the MV with the larger merge index is scaled to the reference picture of the merge candidate with the smaller merge index if the reference pictures are different, and the maximum scaling count constraint are applied, and then calculate averaged MV; if only one MV is available, use the MV directly; if no MV is available, keep this list invalid. The pairwise average candidates replace the combined candidates from HEVC. There are 6 potential additional candidates, and the constraint for pruning are applied to lower max number of operations/comparisons/conditions, and the max merge candidates is increased. The max number of 8/10 will be tested.

Test:

- Adding pairwise-average candidates and removing HEVC combined candidates with candidate number to 8

- Adding pairwise-average candidates and removing HEVC combined candidates with candidate number to 10

Middle spatial and multiple temporal candidates

In additional to the spatial candidates located at A1, B1, B0, A0 and B2 in HEVC, two middle spatial candidates located at ML and MT are added as shown in following Figure. The middle spatial candidates at the middle positions of left and top edges are added by following the order {ML, MT}. In contrast to temporal candidates in HEVC (RB or CT if RB is not available), four temporal candidates from colocated blocks are added by following the order {RB, CT, LB, RT} as shown in following Figure. There are 5 potential additional candidates (middle spatial candidates: 2, multiple temporal candidates: 3), and the constraint for pruning are applied to lower max number of operations/comparisons/conditions, and the max merge candidates is increased.

Figure. Positions for spatial (darkest blocks) and temporal (brightest blocks) candidates.

Tests:

1. Adding middle spatial candidates
2. Adding multiple temporal candidates
3. Adding middle spatial and multiple temporal candidates

Non-adjacent merge candidates with buffer size reduction

This test proposes two motion vector (MV) buffer reduction methods for non-adjacent spatial merge candidates. The first method is to apply a coding tree unit (CTU) row constraint, where candidates belonging to above CTU rows and non-adjacent to the top boundary of the current CTU row are replaced with candidates belonging to above CTU rows and adjacent to the top boundary of the current CTU row. The second method is to round position of the referenced the non-adjacent MV candidate to the center 4x4 of a 16x16 block. The proposed method will be implemented on top of K0297.

Tests:

1. CTU row constraint only
2. CTU row constraint + reference MV position rounding

Block shape based candidate list reordering JVET-K0065 (Hikvision)

The proposed method is motivated by that motion information of the neighboring blocks that have more adjacent area with current block would be more convinced to be utilized to predict current one. Therefore, aimed at Merge mode and AMVP mode, the proposed method reorders the candidate list of merge mode and AMVP mode according to the block shape rather than adopting an identical candidate list construction method for all blocks.

Table 1 the proposed candidate order for Merge mode

| Width> height | B1A1A0B0 |
| --- | --- |
| Width=height | B1A1B0A0 |
| Width<height | A1B1B0A0 |

Table 2 the proposed candidate order for AMVP mode

| Width>= height | B0B1B2 (scaleB0scaleB1scaleB2),A0A1 (scaleA0scaleA1) |
| --- | --- |
| Width<height | A0A1 (scaleA0scaleA1), B0B1B2 (scaleB0scaleB1scaleB2) |

Tests:

(1) BMS with block shape based candidate list reordering

(2) VTM with block shape based candidate list reordering

Separate merge candidate list for sub-block modes (JVET-K0364)

Currently, several sub-block based motion vector prediction (MVP) methods has been proposed, e.g., ATMVP, STMVP, Affine Merge enhancement in JVET-J0024 and Planar MVP in JVET-J0061. All of these methods generate fine granularity motion fields for CUs. Because of the commonality among these methods, this contribution presents a separate merge candidate list for sub-block based MVPs. The candidate list is constructed by integrating two or more sub-block based MVPs into a separate merge candidate list.

Three tests in this contribution,

- Three merge list, one for VTM merge list with ATMVP, one for planar MV, one for affine candidates.

- Two merge list, one for VTM, one for affine candidates + planar MV + ATMVP.

- Two merge list, one for VTM merge list with ATMVP, one for affine candidates + planar MV.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | a) additional flag for ATMVP b) separate list for ATMVP + affine merge in VTM c) separate list for ATMVP + affine merge in test 4.x.x d) separate list for ATMVP + planar merge in test 4.x.x e) separate list for ATMVP + affine merge in test 4.x.x + planar merge in test 4.x.x | H. Chen (Huawei) |  |

Merge mode modification (K0056)

Long-distance motion vector prediction (LDMVP)

In this method, more spatial positions are checked as shown in Figure 1. The candidates are checked from inside to the outside. And the order of the candidates is Ai, Bi, Ci, Di, Ei. These candidates are added after TMVP candidates and before the combined Bi-predictive candidates.

Figure 1. Additional spatial candidates

Additional temporal candidates

In this method more TMVP candidates are added to the merge list. Only the TMVP candidates of the positions that are located within the same CTU row in the collocated reference picture are used.

Restrictions

In this method LDMVP pattern modifications limiting access to motion information beyond the current CTU are used. More specially, following restrictions are considered:

- Using motion information from only the bottom CU line from one top CTU line is allowed

- Using motion information from only the bottom CU line from top CTU (plus one adjacent element from the top right and top left CTU) is allowed

- Using motion information from only the last CU column (adjacent column) from left CTU (plus one adjacent element from the top left CTU) is allowed.

Combination of restrictions 1 and 3 is depicted in Figure 2. Combination of restrictions 2 and 3 is depicted in Figure 3. These combinations are used in test specifications.

For the candidates that are not in the allowed positions adjustment is performed.

Figure 2. Combination of restrictions 1 and 3

Figure 3. Combination of restrictions 2 and 3

Test specification

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
|  | K0056 Merge mode modification - Non-adjacent spatial merge candidate candidates with restrictions 1 and 3 - Non-adjacent spatial merge candidate candidates with restrictions 2 and 3 - a + Enlarge merge candidate list size to 8 - a + Enlarge merge candidate list size to 11 - a + Enlarge merge candidate list size to 16 - c + TMVP candidates - d + TMVP candidates - e + TMVP candidates | Timofey Solovyev (Huawei) |  |

History-based Motion Vector Prediction (JVET-K0104)

This contribution presents a History-based Motion Vector Prediction (HMVP) method for inter coding. In HMVP, a table of HMVP candidates is maintained and updated on-the-fly. After decoding a non-affine inter-coded block, the table is updated by adding the associated motion information as a new HMVP candidate to the last entry of the table. A First-In-First-Out (FIFO) or constraint FIFO rule is applied to remove and add entries to the table. The HMVP candidates could be applied to either merge candidate list or AMVP candidate list.

Tests:

- Check the performance of HMVP applied to merge mode

- Check the performance of HMVP applied to AMVP mode

- Check the performance of HMVP applied to AMVP and merge mode

CE4.4: Motion vector coding

Symmetrical mode for bi-prediction (K0188)

In this contribution, a mode is proposed for motion information coding in bi-prediction. A symmetrical mode flag indicating whether symmetrical mode is used or not is explicitly signaled if the prediction direction is bi-prediction. When the flag is true, the reference index and MVD for list 0 are derived from motion information of list 1. Moreover, the reference index of list 1 is inferred as 0 for this mode.

As shown in Figure 1, the ref_idx_l0 is decided by searching a reference picture in list 0, which is mirrored with the reference picture for list 1. Since the distance between current picture and the backward reference picture is same as the distance between current picture and the forward reference picture, MVD0 is just set to –MVD1. The final motion vectors are shown in below formula.

mvx0, mvy0=(mvpx0-mvdx1,mvpy0-mvdy1) mvx1, mvy1=(mvpx1+mvdx1,mvpy1+mvdy1)

Figure 1 Illustration for symmetrical mode

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
| 4.x.x | a) symmetrical mode on top of VTM and BMS b) combine with the MVD coding methods in UMVE | Huanbang Chen (Huawei) |  |

Bilateral Motion Vector Prediction (K0302)

The proposed BiMVP mode is another type of AMVP. Keeping the existing AMVP mode, a flag (bilateral_mode_flag) is employed to activate the BiMVP. bilateral_mode_flag equal to 1 indicates the BiMVP is applied for the current CU, while bilateral_mode_flag equal to 0 indicates the existing AMVP mode is done. When activated, BiMVP mode does not signal ref_idx and ref_pred_idc. Instead, the first reference pictures whose ref_idx values are equal to 0 for both list0 and list are used for bi-prediction. Uni-prediction is not allowed. When bilateral_mode_flag is equal to 1, an additional flag (symmetric_flag) is signalled to indicate whether the BiMVP for the current CU is the Symmetric BiMVP mode or the Asymmetric BiMVP mode.

In the symmetric BiMVP mode, only mvp_flag and coded MVD for list0 are signaled. Then, both MV predictor (V1x, V1y) and MV difference (offset_x, offset_y) for list1 are derived from MVP = (V0x, V1y) and MVD = (offset_x, offset_y) for list0 by mirroring values. For example, when the forward MV is equal to (Vx + offset_x, Vy + offset_y), its paired backward MV is derived as equal to (-Vx - offset_x, -Vy - offset_y). Since the derived MV for list1 is exactly symmetric with MV for list1, it is called Symmetric BiMVP mode.

In the asymmetric BiMVP mode, mvp_flag, coded MVD for list0 and mvp_flag for list1 are signaled. It means that the MVP for list is not derived from MVP for list0 and explicitly indicated by signaling. For example, where the MV for list0 is equal to (V0x + offset_x, V0y + offset_y), its paired MV for list1 set equal to (V1x - offset_x, V1y - offset_y). Note that both (V0x, V0y) and (V1x, V1y) are determined by the values of mvp_flag. Even though it is called Asymmetric BiMVP mode in this contribution, its MVD values are symmetric.

BiMVP can have only a MVD value among a set of predefined MVD values in integer-pel resolution. It employs motion vector compression and reconstruction process, which are intended to reduce the MVD bits and the encoding complexity.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
|  | - Asymmetric BiMVP mode - Symmeetric BiMVP mode - a+b - MVD compression + reconstruction - a+d - c+d | Byeongdoo Choi (Sharp) |  |

Ultimate motion vector expression (K0115)

Ultimate motion vector expression (UMVE) in J0024 can be either skip or direct(or merge) modes, which use proposed motion vector expression method with neighboring motion information. As the skip and the merge modes in HEVC, UMVE also makes a candidate list from neighboring motion information. Among those candidates in the list, a MV candidate is selected, and is further expanded by new motion vector expression method.

UMVE provides a new motion vector expression with simplified signaling. The expression method includes starting point, motion magnitude, and motion direction.

UMVE search process

UMVE search point

Base candidate index defines the starting point. Base candidate index indicates the best candidate among candidates in the list as follows.

| Base candidate IDX | 0 | 1 | 2 | 3 |
| --- | --- | --- | --- | --- |
| Nth MVP | 1st MVP | 2nd MVP | 3rd MVP | 4th MVP |

Distance index is motion magnitude information. Distance index indicates the pre-defined distance from the starting point information. Pre-defined distance is as follows:

| Distance IDX | 0 | 1 | 2 | 3 | 4 | 5 | 6 | 7 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Pixel distance | 1/4-pel | 1/2-pel | 1-pel | 2-pel | 4-pel | 8-pel | 16-pel | 32-pel |

Direction index represents the direction of the MVD relative to the starting point. The direction index can represent of the four directions as shown below.

| Direction IDX | 00 | 01 | 10 | 11 |
| --- | --- | --- | --- | --- |
| x-axis | + | – | N/A | N/A |
| y-axis | N/A | N/A | + | – |

The UMVE method can be implemented on the test model or the Benchmark Set (BMS) as independent UMVE candidates with additional information signaled.

| Test # | Description | Tester |
| --- | --- | --- |
| 4.1 | Ultimate motion vector expression UMVE candidates as independent merge candidates with additional information signaled | Seungsoo Jeong (Samsung) |

Planar Motion Vector Prediction (K0135)

To generate a smooth fine granularity motion field, Figure 1 gives a brief description of the planar motion vector prediction process.

Planar motion vector prediction is achieved by averaging a horizontal and a vertical linear interpolation on 4x4 block basis as follows.

Figure 1. Planar motion vector prediction process

•	Extrapolate motion vectors of each sub-block inside a coding block similarly as intra prediction planar mode.

•	Motion in the L-shaped area is padded in the same manner as intra reference pixel padding, separately for L0 and L1.

•	Bi-directional or uni-directional: decided based on the prediction direction of the motion in the L-shaped area. Once there are two or more L0 or L1 MVs, L0 or L1 direction is activated.

•	Ref_idx in L0 and L1 is set to 0.

•	Use the bottom-right corner temporal motion vectors for planar prediction.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
|  | a) Planar Motion Vector Prediction on Merge b Planar Motion Vector Prediction on Inter c) a+b |  |  |

CE4.5: Reference picture boundary padding

Reference Picture Boundary Padding (K0117)

This contribution proposes a padding method for padding outside areas of a picture with motion compensation with motion information of edge pixel of the picture.

Key points

- If the area padded by MC is shorter than the size of the padding area, the rest of the area is filled with extrapolated edge pixels of the external picture.

- A conventional padding method is performed when an adjacent area is unavailable or there is no information about a motion in a boundary block.

- In case of bi-directional motion in a boundary 4x4 block, bi-directional MC is used for padding.

- The last pixel of each position is extrapolated to induce a left upper portion, a right upper portion, a left lower portion, and a right padding area.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
|  | xxx | xxx |  |

Proponents are encouraged to provide data on the impact on DPB memory, on possible complications of on-the-fly processing and on subjective quality of affected areas.

Multi-directional boundary padding (J0014)

Multi-directional boundary padding (MDBP) uses angular intra prediction to extend the reference frame border, whenever the referenced pixel area is partially or entirely outside the area of the reconstructed reference frame.

The best intra mode is estimated in both encoder and decoder by using template areas that are located inside the reconstructed reference frame as shown below. The template area is filled by using all available intra modes revered and deriving sample values of the template area by intra prediction from the outer-most pixel line. The best intra mode leads to minimal SAD with respect to the original reconstructed sample values.

Figure 8: Template area used for estimating the MDBP angular mode

Proponents are encouraged to provide data on the impact on DPB memory, on possible complications of on-the-fly processing and on subjective quality of affected areas.

Inter/Intra Boudnary Padding (K0195)

When the decoder performs motion compensation, if the motion vector points to a block outside the frame boundary, a part of the reference block is not available. For each region with size of 4xM or Mx4 along the boundary of the reference picture to be padded, M being the desired frame boundary extension, a motion vector is derived from the nearest 4x4 block inside the frame. If the nearest 4x4 block is intra coded, a zero motion vector is used.  If the nearest 4x4 block is coded with bi-directional inter prediction, only the motion vector, which points to the pixel farther away from the frame boundary, is used in motion compensation for padding. After the motion vector derivation, motion compensation is then performed to obtain the pixels in the padding region with the consideration of average pixel value offset between the nearest 4x4 block and its corresponding block in its reference picture. Boundary padding areas for which no motion compensation can be performed, are filled using perpendicular boundary padding as in HEVC. For each reference block that contains boundary padding pixel, when the amount of pixels stemming from perpendicular boundary padding exceeds 50%, intra-prediction based padding as in JVET-J0014 (MDBP) is carried out.

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
|  | a) DPB memory impact b) Subjective quality impact of affected areas | J. Brandenburg (HHI) |  |

Proponents are encouraged to provide data on the impact on DPB memory, on possible complications of on-the-fly processing and on subjective quality of affected areas.

CE4.6: Other topics

Non-Temporal IC (K0118)

At the end of processing of each CU, linear models (Ax + B = y) are computed by comparing the prediction pixels (x) and the reconstructed pixels (y). The models are computed using linear regression. The lower edge pixels are used for model computation. Two sets of models are computed, one for each colour component. Each set contains models for the two directions (if available). These parameters (sets of A and B) are then stored in the motion information buffer for later access.

The conditions under which non-temporal IC is applicable is kept same as that of JEM LIC. These conditions depend on the size of the current CU, type of prediction mode and whether the CU uses merge mode or not. If applicable, a flag is signaled to indicate whether non-temporal IC is to be applied to a given inter coded CU or not. If the tool is to be applied, the motion compensation pixels (x) are refined using the application of the linear model (Ax + B) to obtain the new set of prediction pixels. The model used is derived from the current CU’s neighbors. Top and top left and left neighbours are considered for borrowing the parameter. Since multiple neighbors may be available, pruning is performed to obtain a single model. Intra coded neighbours are discarded in the pruning process. Next, the neighbours which have trivial model [x = y] are discarded in the pruning process. If no model is remaining, a trivial model is used. If model(s) still remain, the neighbours with the same reference index as the current CU are given higher priority. If multiple neighbours have the same reference index, the top neighbour is given higher priority compared to the top-left neighbour.

Additionally, when performing RDO check for determining whether non-temporal IC will be used or not, mean-removed SAD (Sum of Absolute Difference) is used instead of the regular SAD for the ME (Motion Estimation) process.

The linear regression method used is same as that in JEM. Also, using two sides for parameter derivation improves the gain.

Key points

- LIC model pre-computation and storage after processing a CU

- LIC model pruning to get the only one model applied to a CU

| Test # | Description | Tester | Crosschecker |
| --- | --- | --- | --- |
|  | Non-temporal IC using left neighbor CU | Anish Tamse (Samsung) |  |
|  | Non-temporal IC using left and above neighbor CU | Anish Tamse (Samsung) |  |

Temporal motion data storage reduction (K0086)

The motion information is achieved from different spatial candidates compared with HEVC for motion vector compression.

Suggested motion data storage reduction could be refer alternative spatial candidate as below example figure:

And It is reported that BD-rate and various storage reduction ratio based on various motion vector storage unit granularity

| Test # | Description | Tester |
| --- | --- | --- |
| 4.X | Motion information storage reduction based on alternative spatial candidates with various granuarity | Hyeongmun Jang (LGE) |

Test condition and evaluation criteria

In general, each testing item is required to be compared against 1) VTM and 2) BMS. Each testing item will be evaluated using BD rates, encoding time, decoding time according to Common Test Conditions (CTC) in JVET-K1010. Simulation with RA and LDB configurations are mandatory.

Sub-CE specific test condition and evaluation criteria may be defined for a sub-CE if considered informational for tool evaluation. Please refer to the section of test condition and evaluation criteria in each sub-CE for information.

For getting encoding and decoding time ratio of each test, the same SIMD setting, either SSE or AVX2, must be used for generating both anchor and a test. For easy comparison of encoding and decoding time of different techniques, it is suggested to use SSE as the default SIMD setting by encoder configuration.

SW integration guideline

According to the information from SW coordinator,

- BMS with JEM_TOOLS macro disabled should produce the same bitstream as VTM.

- Bitstreams of BMS with VTM config file and BMS with JEM_TOOLS macro disabled should only differ by the tool indication flags in the SPS, which should have a very minor impact on bit rate.

Implementing a proposed technique on BMS SW only could serve the purpose of both tool on tests and tool off tests. Therefore, following guideline for SW integration is suggested,

- Proponent implement the technique in BMS SW, for both tool on tests and tool off tests

- For the two type of tool on tests, the results of VTM or VTM+ToolX are obtained by disabling BMS tools (except ToolX) via configuring encoder parameters of BMS SW (no change of macro setting)

Timeline

| T1 | May 11, 2018 | Finalize CE description |
| --- | --- | --- |
| T2 | June 1, 2018 | Deliver software packages and begin cross-checking (proponents can continue to modify the software until T3) |
| T3 | June 19, 2018 | Finish cross-checking and report results to CE group |

Participant list

| Company/ Organization | Participant Name | Email Address |
| --- | --- | --- |
| Technicolor | Antoine Robert Philippe Bordes Tangi Poirier Fabrice Le Léannec Franck Galpin Ya Chen | antoine.robert@technicolor.com philippe.bordes@technicolor.com tangi.poirier@technicolor.com fabrice.leleannec@technicolor.com franck.galpin@technicolor.com Ya.CHEN@technicolor.com |
| HHI | Martin Winken Robert Skupin Jens Brandenburg Anastasia Henkel | martin.winken@hhi.fraunhofer.de robert.skupin@hhi.fraunhofer.de jens.brandenburg@hhi.fraunhofer.de anastasia.henkel@hhi-extern.fraunhofer.de |
| Huawei | Huanbang Chen Haitao Yang Jianle Chen Sychev Maxim Solovyev Timofey | chenhuanbang@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com Sychev.Maxim@huawei.com solovyev.timofey@huawei.com |
| LGE | Seung Hwan Kim Jaehyun Lim Jaeho Lee Hyeongmun Jang | seunghwan3.kim@lge.com Jaehyun.lim@lge.com jhlee.lee@lge.com hm.jang@lge.com |
| Tencent | Xiaozhong Xu Jing Ye Guichun Li Shan Liu Meng Xu Xiang Li | xiaozhongxu@tencent.com jingye@tencent.com guichunli@tencent.com shanl@tencent.com mengxxu@tencent.com xlxiangli@tencent.com |
| USTC | Yue Li Li Li | lytt@mail.ustc.edu.cn lili90th@gmail.com |
| HiSilicon | Na Zhang Xu Chen Jianhua Zhang | zhangna25@huawei.com anderson.chen@hisilicon.com zhengjianhua@hisilicon.com |
| BBC | Saverio Blasi | saverio.blasi@bbc.co.uk |
| ETRI | Jungwon Kang Sung-Chang Lim Hahyun Lee | jungwon@etri.re.kr sclim@etri.re.kr hanilee@etri.re.kr |
| Panasonic | Kiyofumi Abe Jingya Li | abe.kiyo@jp.panasonic.com jingya.li@sg.panasonic.com |
| Fujitsu | Jianqing Zhu Kimihiko Kazui | zhujianqing@cn.fujitsu.com kazui.kimihiko@jp.fujitsu.com |
| Sony | Kenji Kondo | kenji.kondo@sony.com |
| Qualcomm | Yi-Wen Chen Chun-Chi Chen Yan Zhang Yu Han Wei-Jung Chen | yiwench@qti.qualcomm.com chunchic@qti.qualcomm.com yzh@qti.qualcomm.com yuhan@qti.qualcomm.com wchien@qti.qualcomm.com |
| Alibaba | Yu-Chen Sun | yuchen.s@alibaba-inc.com |
| Hikvision | Fangdong Chen | chenfangdong@hikvision.com |
| Sharp | Weijia Zhu Byeongdoo Choi Tomonori Hashimoto Takeshi Chujoh Tomohiro Ikai | zhuw@sharplabs.com choib@sharplabs.com tomonori.hashimoto@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |
| Ericsson | Ruoyang Yu | ruoyang.yu@ericsson.com |
| Samsung Electronics | Min Woo Park Seungsoo Jeong Anish Tamse Kiho Choi Jie Chen Minsoo Park | m.w.park@samsung.com ss00.jeong@samsung.com tamse.anish@samsung.com kiho14.choi@samsung.com jie1222.chen@samsung.com mss.park@samsung.com |
| Nokia | Jani Lainema | jani.lainema@nokia.com |
| InterDigital | Yuwen He Xiaoyu Xiu Daniel Luo Yan Ye | Yuwen.He@interdigital.com Xiaoyu.Xiu@interdigital.com Daniel.Luo@InterDigital.com Yan.Ye@interdigital.com |
| Arris | Krit Panusopone Yue Yu David Baylon Seungwook Hong Limin Wang | Krit.panusopone@arris.com Yue.yu@arris.com David.baylone@arris.com Seungwook.hong@arris.com Limin.wang@arris.com |
| MediaTek | Tzu-Der Chuang Ching-Yeh Chen Chih-Wei Hsu Yu-Wen Huang Shih-Ta Hsiang | Peter.Chuang@mediatek.com chingyeh.chen@mediatek.com cw.hsu@mediatek.com yuwen.huang@mediatek.com shih-ta.hsiang@mediatek.com |
| NHK | Shunsuke Iwamura | iwamura.s-gc@nhk.or.jp |
| Netflix | Andrey Norkin | anorkin@netflix.com |
| Orange | Gordon Clare | gordon.clare@orange.com |
| Broadcom | Minhua Zhou Pensong Chen Wade Wan | minhua.zhou@broadcom.com peisong.chen@broadcom.com wade.wan@broadcom.com |
| Canon | Jonathan Taquet | Jonathan.TAQUET@crf.canon.fr |
| PKU | Zhao Wang | zhaowang@pku.edu.cn |
| Chips&Media | Wook Je Jeong | nick.jeong@chipsnmedia.com |
| Aachen University | Johannes Sauer | sauer@ient.rwth-aachen.de |
| HIT | Yang Wang Xiaopeng Fan | wangyang.cs@hit.edu.cn fxp@hit.edu.cn |
| WILUS | Greg Ko | greg.ko@wilusgroup.com |
| ITRI | Ching-Chieh Lin | JackLin@itri.com |
| Bytedance | Kai Zhang Li Zhang | zhangkai.video@bytedance.com lizhang.idm@bytedance.com |