| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K1025-v1 |
| --- | --- |

| Title: | Description of Core Experiment 5 (CE5): Arithmetic Coding Engine |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | Core Experiment Description |  |  |
| Author(s) or Contact(s): | Heiner Kirchhoffer Fraunhofer HHI Amir Said Qualcomm | Email: | heiner.kirchhoffer@ hhi.fraunhofer.de asaid@qti.qualcomm.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Abstract

This Core Experiment (CE) investigates the proposed modifications to the binary arithmetic coding engine for the Versatile Video Coding (VVC) standard effort by the JVET. The experiments conducted in this CE address different aspects of the entropy coding engine design including the probability update procedure and the coding interval subdivision. Compression efficiency, memory requirement, and computational complexity are addressed in this CE with the latter being either by experimental results or by calculations.

# Introduction

The proposed modifications to the CABAC engine target different aspects of the design. This Core Experiment (CE) categorizes the techniques into three sub-categories. The first category, subtest 1, comprises of modifications to the probability estimation and its representation only. The second category, subtest 2, includes changes to the coding interval subdivision only, whereas the third category, subtest 3, addresses combinations of subtest 1 with changes to the coding interval subdivision. Note that the used categorization approach is in terms of best-effort.

# Participants List

| Affiliation | Participants | P | C |
| --- | --- | --- | --- |
| Apple | Alexis Tourapis (atourapis@apple.com) |  |  |
| Broadcom | Peisong Chen (peisong.chen@broadcom.com) Wade Wan (wade.wan@broadcom.com) Minhua Zhou (minhua.zhou@broadcom.com) |  |  |
| Bytedance | Li Zhang (lizhang.idm@bytedance.com) |  |  |
| Canon Australia | Andrew Dorrell (Andrew.Dorrell@cisra.canon.com.au) Chris Rosewarne (Chris.Rosewarne@cisra.canon.com.au) |  |  |
| Etri | Jungwon Kang (jungwon@etri.re.kr) |  |  |
| HHI | Heiner Kirchhoffer (heiner.kirchhoffer@hhi.fraunhofer.de) Detlev Marpe (detlev.marpe@hhi.fraunhofer.de) Tung Nguyen (tung.nguyen@hhi.fraunhofer.de) Jan Stegemann (jan.stegemann@hhi.fraunhofer.de) | x |  |
| Hikvision | Li Wang (wangli7@hikvision.com) |  |  |
| HiSilicon | Quanhe Yu (yuquanhe@hisilicon.com) Jianhua Zheng (zhengjianhua@hisilicon.com) |  |  |
| Huawei | Jianle Chen (jianle.chen@huawei.com) Roman Chernyak (chernyak.roman@huawei.com) Alexey Filippov (Alexey.Filippov@huawei.com) Sergey Ikonin (Sergey.Ikonin@huawei.com) Vasily Rufitskiy (vasily.rufitskiy@huawei.com) |  |  |
| InterDigital | Rahul Vanam (Rahul.vanam@interdigital.com) |  |  |
| Kwai | Yi-Wen Chen (yiwenchen@kwai.com) Xianglin Wang (xianglinwang@kwai.com) |  |  |
| MediaTek | Ching-Yeh Chen (chingyeh.chen@mediatek.com) Tzu-Der Chuang (peter.chuang@mediatek.com) Shih-Ta Hsiang (shih-ta.hsiang@mediatek.com) Chih-Wei Hsu (cw.hsu@mediatek.com) Yu-Wen Huang (yuwen.huang@mediatek.com) | x |  |
| Panasonic | Chong Soon Lim (chongsoon.lim@sg.panasonic.com) Han Boon Teo (hanboon.teo@sg.panasonic.com) |  |  |
| Qualcomm | Wei-Jung Chien (wchien@qti.qualcomm.com) H. E. Egilmez (hegilmez@qti.qualcomm.com) Ted Hsieh (thsieh@qti.qualcomm.com) Amir Said (asaid@qti.qualcomm.com) | x |  |
| Samsung | Kiho Choi (kiho14.choi@samsung.com) | x |  |
| Sejong University | Dongjae Won (bigdj2002@sju.ac.kr) (hajama0123@sju.ac.kr) |  |  |
| Sharp Labs | Frank Bossen (frank.bossen@me.com) Byeongdoo Choi (choib@sharplabs.com) Kiran Misra (misrak@sharplabs.com) Andrew Segall (asegall@sharplabs.com) | x |  |
| Sharp | Tomohiro Ikai (ikai.tomohiro@sharp.co.jp) Eiichi Sasaki (eiichi.sasaki@sharp.co.jp) |  |  |
| Sony | Karl Sharman (Karl.Sharman@sony.com) |  |  |
| Technicolor | Fabrice Leleannec (Fabrice.Leleannec@technicolor.com) Ya Chen (Ya.CHEN@technicolor.com) |  |  |
| Tsinghua University | Benben Niu (nbb16@mails.tsinghua.edu.cn) |  |  |
|  | Dan Grois (grois@ieee.org) |  |  |
|  | Jae Hoon Kim (i.jaehoon.kim@icloud.com) |  |  |
|  | Dohyeon Park (dhpark@kau.kr) |  |  |
|  | Yasser Syed (Yasser_Syed@comcast.com) |  |  |

# Description of Experiments and Coding Tools

This CE describes 7 experiments in subtest 1, 4 experiments in Subtest 2, and 1 experiments in subtest 3, resulting in a total number of experiments equal to 12.

## Subtest 1: Probability estimation

In this subtest, the proposed probability estimation techniques are tested using the 512x64 interval subdivision table of CABACEngine 1 (from BMS1.1 code as specified in Sec. 4.1).

### Experiment CE5.1.1 (Sharp Labs, JVET-K0273)

Description:

The probability estimation is expressed as:

if (symbol)

q = q + ((32767 − q) >> n)

else

q = q − (q >> n)

where q is a count or probability estimate and n represents an adaptation rate or window size. More generally the value 32767 can be expressed as 2b−1 where b is number of bits used to represent q.

This estimation function can be used for single or double window estimation, fixed or variable window size.

See JVET-K0273 for more details.

Cross-checkers:

### Experiment CE5.1.2 (Samsung, JVET-K0170)

Description:

In JEM, when double-window probability estimation model is on, the probability is updated with linear combination for probability estimation that consists of only two summands corresponding N0=16 (short window) and N1=128 (long window):

P0 = (Y >> 4) + P0 – (P0 >> 4)

P1 = (Y >> 7) + P1 – ( P1>> 7)

P = (P0 + P1 + 1) >> 1

where Y = 2^15.

To start probability estimation it is necessary to initialize probability. Different from JEM, only short window size model is updated until the counter reaches 31. When the counter reaches 31, long window model starts updating with initial value borrowed from short window model. This approach significantly reduces negative effect from wrong initialization and also reduces the complexity due to less update of long window size model.

The probability estimation process is as follows,

•       bFastProbUpdate = counter < 31

•       counter++

•       MPS pass:

•       P0mps = P0 – (P0 >> 4)

•       P1mps = bFastProbUpdate ? P0mps: P1 – (P1 >> 7)

•       LPS pass

•       P0lps = P0 + ((2^15 – P0) >> 4)

•       P1lps = bFastProbUpdate ? P0lps : P1 + ((2^15 – P1) >> 7)

•       If(P0 + P1 > 2^15)

•       mps = 1 – mps

•       P1lps = 2^15 – P1lps

•       P0lps = 2^15 – P0lps

•       P= (P0 + P1) >> 1

Cross-checkers:

### Experiment CE5.1.3 (HHI, JVET-J0014)

Description:

The entropy coding stage employs a counter-based probability estimator, which uses two custom window sizes na and nb for each context model, initialized at the beginning of each slice. After decoding or encoding a binary symbol xt the counts q=a,b of the probability estimator are updated as follows:

qt+1=qt+2BITSq-qt≫nq,  if xt=1qt-qt≫nq                      ,  if xt=0,

where BITSq defines the number of bits used to represent each counter. The binary logarithm of the allowed window sizes are in the interval [2, 9]. The number of bits, used to represent the individual counters, is chosen so the memory consumption of each context model does not exceed 32 bits including counters and window size pairs. Therefore, the resolution of each counter is set to BITSq≤15. Before addressing the interval subdivision table, the counters q'=a',b' are shifted to the same resolution: q't=qt≫BITSq-BITSmin,  where BITSmin= minBITSa, BITSb. For coding a binary symbol, the counters are weighted c't=a't+b't≫1 and the interval subdivision table is accessed with c't.

For this experiment, BITSa=10 and BITSb=14.

Two configurations are tested in this experiment:

- na=4 and nb=7

- Individual values for na and nb are used for each context model

Cross-checkers:

### Experiment CE5.1.4 (HHI, JVET-K0430)

Description:

The state-based probability estimator of JVET-K0430 is used. Instead of using the rangeTabLPS of JVET-K0430, the 512x64 interval subdivision table of CABACEngine 1 is used. The table elements need to be adapted to the state-representation of JVET-K0430. The combination with custom widow sizes is also tested in this experiment.

Cross-checkers:

### Experiment CE5.1.5 (HHI, JVET-K0430)

Description:

The same configuration as in CE5.1.4 is used except that only one state variable (S0) is used.

Cross-checkers:

### Experiment CE5.1.6 (Qualcomm, JVET-K0380)

Description:

This experiment tests the double-window probability estimation, modified to use different number of bits in the estimation finite-state machines. This version is defined by the recursive equations

bin = 1    →     U[k+1] = U[k] + ((2Bu – U[k]) >> a),     V[k+1] = V[k] + ((2Bv – V[k]) >> b),

bin = 0    →               U[k+1] = U[k] – (U[k] >> a),                V[k+1] = V[k]  – (V[k]) >> b),

P[k] = U[k] + (V[k]) >> (Bu – Bv)).

where P[k] represents the probability estimate used for binary arithmetic coding. The two positive integer window shift parameters a and b can be different for each context, are in the interval 2, 3, …, 9, and represented together using two-bit codes. Parameters Bu and Bv are the same for all contexts.

Cross-checkers:

### Experiment CE5.1.7 (Qualcomm, JVET-K0385)

Description:

Same type of experiment as in CE5.1.6, but with a single window.

Cross-checkers:

## Subtest 2: Coding interval subdivision

In this subtest, the proposed techniques for the coding interval subdivision (range-probability product computation) are tested using the double-window probability estimation of the CABACEngine 1 (from BMS1.1 code as specified in Sec. 4.1).

### Experiment CE5.2.1 (Sharp Labs, JVET-K0273)

Description:

Consider having probability estimates qa and qb resulting from two different estimators, each estimate being 15 bits wide, and a range value r which is 9 bits wide (same as AVC and HEVC).

Let us further define q as qa+qb, and qLPS as q ^ ((q & 32768) ? 65535 : 0). Additionally, let r3 = r >> 5 and q5 = qLPS >> 10.

The subinterval range rLPS is computed as:

rLPS = (r3 * q5 >> 1) + 4.

See JVET-K0273 for more details.

Cross-checkers:

### Experiment CE5.2.2 (MediaTek, JVET-K0249)

Description:

The interval subdivision is realized using a multiplier. The range interval is divided into the rangeOne and the rangeZero. The rangeOne and the rangeZero derivation is modified as follows.

probIdx   = ( P >= 16384 ) ? 63 – (P>>9) : (P>>9)

rangeLPS  = ((2*probIdx +1) * (2*( range >> 5 ) + 1)) >> 3

where the P is the probability of a context and the range is the current range in binary arithmetic coder (BAC). The hardware synthesis result will be provided.

Cross-checkers:

### Experiment CE5.2.3 (HHI, JVET-J0014)

Description:

The coding interval subdivision technique of JVET-J0014 is tested. The two counter variables ‘m_P0’ and ‘m_P1’ of class ‘BinProbModel_JMP’ are used to address the proposed table ‘rangeTabLPS’ containing the pre-calculated LPS range values:

c=m_P0+m_P1>>1

pStateIdx=c-214XORvalMPS-1≫15-BITSRTAB,

with valMPS=c>>14 and BITSRTAB defining the resolution of the table containing the range values.

Different dimensions of the table ‘rangeTabLPS’ are tested (like 32x8, 64x16, etc.).

Cross-checkers:

### Experiment CE5.2.4 (Qualcomm, JVET-J0383)

Description:

In this experiment, before the computation of the interval sub-division, the probabilities are scaled using the integer function

z(P) = floor(log2(P))

which can be computed efficiently with “bit-scan-reverse” instructions, simple hardware, or table look-up, using the same table employed for renormalization of the arithmetic coding interval.

When the probability estimate is scaled by 2R, and M bits are used for representing probability (after the most-significant 1), and N bits for the range (in the same manner), we obtain operands (or table entries)

m(P) = (P >> (z(P) – M)) & (2M – 1),      n(R) = (R >> (8 – N)) & (2N – 1),

use direct multiplication, or a 2M  × 2N  table with 8-bit elements according to

TN [m, n] = ( ((2M+1 + 1) + 2×m) × ((2N+1 + 1)+2×n) ) >> (M + N – 4),

and, for interval sub-division, use

RL = ( TN [m(P), n(R)] >> (R – 2 – z(P)) ) | 2.

Configurations corresponding to tables with sizes 16 × 16, 8 × 16, and 8 × 8 are tested.

Cross-checkers:

## Subtest 3: Combinations of subtest 1 with coding interval subdivision techniques

In this subtest, the probability update techniques of subtest 1 are combined with the proposed coding interval subdivision techniques.

### Experiment CE5.3.1 (HHI, JVET-K0340)

Description:

The state-based probability estimator of JVET-K0430 is configured as in subtest 1 and combined with the coding interval subdivision technique of JVET-K0430. Various dimensions of the proposed ‘rangeTabLPS’ table are tested (like 32x8, etc.).

Cross-checkers:

# Test conditions

The common test conditions as specified in JVET-K1010 are modified as described in this section. Unless stated differently, they shall be used for all experiments in this CE.

## Reference

A modified version of the BMS1.1 software shall be used for all tests in the CE. The following change applies: In file ‘Contexts.h’ change variable ‘m_Log2WindowSize1’ of class ‘BinProbModel_JMP’ from ‘8’ to ‘7’. This modified BMS software is referred to as ‘CEM1 software’ in this document.

The common test conditions according to JVET-K1010 shall be used for all simulations. Different from JVET-K1010, the CEM1 software is used for all simulations (VTM and BMS) and the configuration file parameter ‘CABACEngine’ shall be set to ‘1’.

## Test Configurations

The proponents shall implement their CE tests in class ‘BinProbModel_JMP’ of the CEM1 software.

Whenever it makes sense, proposed changes to the probability update, to the coding interval subdivision, and to the context probability initialization shall be tested separately.

In addition, relevant combinations shall also be tested.

For each test that uses modified context probability initialization tables, a second test shall be done with the original context probability initialization tables as the only difference.

Additional test configurations reflecting the relevant operation points are encouraged.

## Test sequences

The test sequences as described in JVET-K1010 shall be used for the experiments.

# Results Reporting

Proponents and cross-checkers shall report the compression efficiency together with the measured encoder and decoder run times relative to the CEM1 software as specified above. In addition, results are reported relative to the unmodified BMS1.1 software using the unmodified VTM and BMS configuration as specified in JVET-K1010.

Additional experimental results are encouraged.

## Compression Efficiency

To obtain objective rate-distortion measurements, four-point BD-rate and BD-PSNR metrics shall be used to evaluate coding efficiency. Rates and PSNR values obtained for all test points shall be provided as well. Compression efficiency, the encoder, and decoder run-time results shall also be reported with the common spreadsheet template that is being used for most JVET contributions.

## Computational complexity

Encoder and decoder run times for the tests in shall be provided as part of the compression efficiency report. In addition to that, proponents are encouraged to provide complexity analysis voluntarily using measures such as arithmetic operations, memory requirements, hardware considerations, and data throughput.

Proponents shall report amount of memory used:

- Per-context state (RAM): State is the information that may change after a bin is encoded/decoded.

- Per-context initializers (ROM): e.g., table-based probability and window initializations.

- Per-slice state initialization (RAM): e.g., temporal initialization updates.

# Timeline (update)

General timeline for core experiments (as agreed by JVET).

| T1 | May 15, 2018 | - Final CE description uploaded |
| --- | --- | --- |
| T2 | June 1, 2018 | - Proponents completed initial integration of tools (in individual branch) - Cross checkers can start initial study of tools / software (Note: Proponents can continue to modify the software until T3) |
| T3 | June 19, 2018 | - Proponents froze changes to software branches - Proponents tag software and announce on JVET reflector - The names of the cross checkers and the list of specific tests for each CE tool are announced on JVET reflector - Proponents provide full test results - Cross checkers start final cross checks |

Test results and cross check results are reported to CE coordinators (for inclusion into CE report).

Proponent

Cross-Checker