| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K1026-v2 |
| --- | --- |

| Title: | Description of core experiment CE6: transforms and transform signaling |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Amir Said Xin Zhao | Email: | asaid@qti.qualcomm.com xinzzhao@tencent.com |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction

The purpose of this Core Experiment (CE) is to investigate the coding tools related to transforms and transform signaling on top of the Test Model. In this CE, the tradeoff between compression efficiency and complexity of different transformation tools will be investigated. In addition to encoder and decoder run-times, other complexity metrics, e.g., arithmetic operation counts, storage, transform core bit-precision and internal bit-precision will be also evaluated. The list of tools and experiments to be performed are detailed in this document.

# Participant list

| Affiliation | Name | E-mail | P/C |
| --- | --- | --- | --- |
| Alibaba | Yu-Chen Sun | yuchen.s@alibaba-inc.com |  |
| Apple | Alexandros Tourapis | atourapis@apple.com |  |
| Arris | Yue Yu | Yue.Yu@arris.com |  |
|  | Seungwook Hong | Seungwook.Hong@arris.com |  |
|  | Limin Wang | Limin.Wang@arris.com |  |
| B<>com | Victorien Lorcy | victorien.lorcy@b-com.com |  |
| BBC | Andre Seixas Dias | andre.seixasdias@bbc.co.uk |  |
|  | Saverio Blasi | saverio.blasi@bbc.co.uk |  |
| BrightCove | Yuriy Reznik | yreznik@brightcove.com | C |
| Broadcom | Minhua Zhou | minhua.zhou@broadcom.com |  |
|  | Peisong Chen | peisong.chen@broadcom.com |  |
|  | Wade Wan | wade.wan@broadcom.com |  |
| ByteDance | Li Zhang | lizhang.idm@bytedance.com |  |
| Canon | Chris Rosewarne | Chris.Rosewarne@cisra.canon.com.au |  |
|  | Andrew Dorrell | Andrew.Dorrell@cisra.canon.com.au |  |
| Chips & Media | D.Y. Kim | brad.kim@chipsnmedia.com |  |
| Chosun Univ. | Bumshik Lee | bslee@chosun.ac.kr |  |
| Comcast | Dan Grois | groisz@yahoo.com |  |
|  | Yasser Syed | Yasser_Syed@comcast.com |  |
| Ericsson | Jacob Ström | jacob.strom@ericsson.com |  |
| ETRI | Jungwon Gang | jungwon@etri.re.kr |  |
|  | Seongchang Im | sclim@etri.re.kr |  |
| FastVDO | Pankaj Topiwala | pankajtva@gmail.com |  |
| Florida Atl. Univ. | Hari Kalva | hari.kalva@fau.edu |  |
| Foxconn | Yao-Jen Chang | yaojen.chang.foxconn@gmail.com |  |
| HHI | Heiko Schwarz | heiko.schwarz@hhi.fraunhofer.de | P/C |
|  | Mischa Siekmann | mischa.siekmann@hhi.fraunhofer.de |  |
|  | Santiago de Luxán Hernández | santiago.de.luxan@hhi.fraunhofer.de |  |
| Hikvision | Xuguang Zuo | zuoxuguang@hikvision.com |  |
| HiSilicon | Y. Lin | linyongbing@hisilicon.com |  |
|  | Q. Yu | yuquanhe@hisilicon.com |  |
|  | J. Zheng | zhengjianhua@hisilicon.com |  |
| Huawei | Jianle Chen | jianle.chen@huawei.com | P/C |
|  | Haitao Yang | haitao.yang@huawei.com |  |
|  | Yin Zhao | yin.zhao@huawei.com |  |
|  | Sergey Ikonin | Sergey.Ikonin@huawei.com |  |
|  | Alexey Filippov | Alexey.Filippov@huawei.com |  |
|  | Vasily Rufitskiy | Vasily.Rufitskiy@huawei.com |  |
|  | Alexander Karabutov | karabutov.alexander@huawei.com |  |
|  | Cheung Auyeung | cheung.auyeung@huawei.com |  |
| INSA | Wassim Hamidouche | Wassim.Hamidouche@insa-rennes.fr |  |
| Intel | Samuel Wong | samuel.wong@intel.com |  |
| InterDigital | Yan Ye | yan.ye@interdigital.com |  |
|  | Yuwen He | yuwen.he@interdigital.com |  |
|  | Xiaoyu Xiu | xiaoyu.xiu@interdigital.com |  |
|  | Rahul Vanam | rahul.vanam@interdigital.com |  |
|  | Saurav Bandyopadhyay | Saurav.Bandyopadhyay@InterDigital.com |  |
| ITRI | Po-Han Lin | pohan@itri.com |  |
|  | Ching-Chieh Lin | JackLin@itri.com |  |
| KDDI | Kei Kawamura | ki-kawamura@kddi.com |  |
|  | Yoshitaka Kidani | yo-kidani@kddi.com |  |
|  | Sei Naito | se-naitou@kddi.com |  |
| Kwai Inc. | Xianglin(Shawn) Wang | xianglinwang@kwai.com |  |
|  | Yi-Wen Chen | yiwenchen@kwai.com |  |
| LGE | Seung-Hwan Kim | seunghwan3.kim@lge.com | P/C |
|  | Mehdi Salehifar | mehdi.salehifar@lge.com |  |
|  | Moonmo Koo | moonmo.koo@lge.com |  |
|  | Jaehyun Lim | jaehyun.lim@lge.com |  |
| MediaTek | Chih-Wei Hsu | cw.hsu@mediatek.com |  |
|  | Yu-Wen Huang | yuwen.huang@mediatek.com |  |
|  | Ching-Yeh Chen | chingyeh.chen@mediatek.com |  |
|  | Tzu-Der Chuang | peter.chuang@mediatek.com |  |
|  | Man-Shu Chiang | man-shu.chiang@mediatek.com |  |
|  | Mohammed Sarwer | Mohammed.Sarwer@mediatek.com |  |
|  | Shih-Ta Hsiang | shih-ta.hsiang@mediatek.com |  |
| Netflix | Andrey Norkin | anorkin@netflix.com |  |
| NHK | Shunsuke Iwamura | iwamura.s-gc@nhk.or.jp |  |
| Nvidia | Ankur Saxena | ankur81saxena@gmail.com |  |
| Orange | Pierrick Philippe | pierrick.philippe@orange.com | P/C |
| Panasonic | Kiyofumi Abe | abe.kiyo@jp.panasonic.com | P/C |
|  | Toma Tadamasa | toma.tadamasa@jp.panasonic.com |  |
| Peking University | Kui Fan | kuifan@pku.edu.cn |  |
| Qualcomm | Amir Said | asaid@qti.qualcomm.com | P/C |
|  | Hilmi E. Egilmez | hegilmez@qti.qualcomm.com |  |
|  | Yung-Hsuan Chao | yunghsua@qti.qualcomm.com |  |
|  | Vadim Seregin | vseregin@qti.qualcomm.com |  |
|  | Ted Hsieh | thsieh@qti.qualcomm.com |  |
| Samsung | Kiho Choi | kiho14.choi@samsung.com | P/C |
| Sejong University | Yung-Lyul Lee | yllee@sejong.ac.kr |  |
|  | Nam-Uk Kim | nukim@sju.ac.kr |  |
| Sharp | Yukinobu Yasugi | yasugi.yukinobu@sharp.co.jp |  |
|  | Takeshi Chujoh | chujoh.takeshi@sharp.co.jp |  |
|  | Tomohiro Ikai | ikai.tomohiro@sharp.co.jp |  |
|  | Eiichi Sasaki | eiichi.sasaki@sharp.co.jp |  |
| Sharp Labs | Kiran Misra | misrak@sharplabs.com |  |
| SKKU | Byeungwoo Jeon | bjeon@skku.edu |  |
|  | Jeeyoon Park | jiyoonpark@skku.edu |  |
| Sony | Masaru Ikeda | Masaru.Ikeda@sony.com | P/C |
|  | Takeshi Tsukuba | takeshi.tsukuba@sony.com |  |
| Technicolor | Fabrice Le Léannec | Fabrice.Leleannec@technicolor.com | P/C |
|  | Eduard Francois | Edouard.Francois@technicolor.com |  |
|  | Karam Naser | Karam.NASER@technicolor.com |  |
| Tencent | Xin Zhao | xinzzhao@tencent.com | P/C |
|  | Xiang Li | xlxiangli@tencent.com |  |
|  | Liang Zhao | leolzhao@tencent.com |  |
|  | Sehoon Yea | sehoonyea@tencent.com |  |
| Tsinghua University | Benben Niu | nbb16@mails.tsinghua.edu.cn |  |
|  | Liqiang Wang | wlq15@mails.tsinghua.edu.cn |  |
| WILUS Inc. | John Son | john.son@wilusgroup.com |  |
|  | Zhu Li | zhu.li@ieee.org |  |
|  | Limin Wang | cuiwang@gmail.com |  |

# Core Experiment tasks

The following tasks shall be performed in this Core Experiment (CE):

- Evaluate compression efficiency, computational complexity and memory of transforms studied in this CE;

- Investigate interactions between primary and secondary transform schemes;

- Report the results and conclusions of these experiments.

Core experiments (CEs) are organized according to the following three categories:

- Primary transformations: CE6.1.1 to CE6.1.8 test proposals that introduce changes related to primary transforms only.

- Secondary transformation: CE6.2.1 to CE6.2.3 test proposals that introduce changes for secondary transforms only.

- Combined primary and secondary transformation: CE6.3.1 to CE6.3.3 test proposals that introduce changes for both primary and secondary transforms.

Additional requirements for core experiments in each category are listed in Section 5.3.

# Description of tools

The table below summarizes the tools to be considered in this CE and is followed by additional information for each case.

| CE6.1 – Primary Transforms |  |  |  |  |
| --- | --- | --- | --- | --- |
| CE # | Proponent | Related Docs. | Summary of the tool | Cross-checkers |
| 6.1.1 | LGE | JVET-K0096 | - AMT index coding optimization - Fast DST-7/DCT-8 based on DFT and reduced transform kernel | HHI Qualcomm |
| 6.1.2 | Huawei | JVET-K0139 | Spatially Varying Transform (SVT) for inter blocks | Qualcomm |
| 6.1.3 | Tencent | JVET-K0290 | Primary transform core with 8-bit precision | Samsung Huawei |
| 6.1.4 | Tencent | JVET-K0291 | Fast DST-7/DCT-8 with dual implementation support | Brightcove Samsung Qualcomm |
| 6.1.5 | Sony | JVET-K0121 | Chroma AMT: Extending AMT to chroma transform block (TB) with an additional flag which indicates whether to use default transform type or the transform type of the collocated luma TB as chroma transform type. | Huawei |
| 6.1.6 | Qualcomm, Orange | JVET-K0272, JVET-K0299 | Efficient Implementations of AMT with Transform Adjustment Stages | Tencent |
| 6.1.7 | Panasonic Technicolor Sony Tencent | JVET-K0130 JVET-K0265 JVET-K0394 JVET-K0292 | AMT simplification by reusing DCT-2 partial butterfly | Brightcove Huawei Panasonic Technicolor Sony Tencent |
| 6.1.8 | Technicolor | JVET-J0022 | AMT with DCT-II |  |
| CE6.2 – Secondary Transforms |  |  |  |  |
| CE # | Proponent | Related Docs. | Summary of the tool | Cross-checkers |
| 6.2.1 | LGE | JVET-K0099 | - RST 4x4 - RST4x4 and RST8x8 |  |
| 6.2.2 | LGE | JVET-K0100 | - Variant tests of 6.2.1 w.r.t complexity reduction (i.e. memory reduction) |  |
| 6.2.3 | Samsung | JVET-K0174 | Modified NSST sets and signaling |  |
| CE6.3 – Combinations and Signalling |  |  |  |  |
| CE # | Proponent | Related Docs. | Summary of the tool | Cross-checkers |
| 6.3.1 | HHI | JVET-K0305, JVET-K0306 | Set of transforms: A set of predefined transform candidates, and each candidate specifies a primary horizontal transform, a primary vertical transform, and a non-separable secondary transform. The transform candidates depend on the block size and the prediction mode. | Brightcove Tencent |
| 6.3.2 | Tencent | JVET-K0085 | Coupled primary and secondary transform | HHI |
| 6.3.3 | Qualcomm | JVET-K0405 | Secondary Transforms Coupled with Simplified Primary Transformation | HHI Tencent |

Detailed description on each coding tool tested in this CE are provided below.

## CE6.1: Core Experiments on Primary Transformation

### CE 6.1.1: Primary Transform [JVET-K0096]

For AMT index coding optimization, the following two aspects are jointly investigated on AMT index coding optimization:

1) AMT index depends on intra prediction mode.

2) Banalization and context modeling for AMT index coding.

In addition, efficient implementation of DST-7/DCT-8 is investigated by using DFT and skipping transform calculation for large transform block.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.1-a | AMT index coding based on intra prediction mode | M. Koo (LGE) |  |
| 6.1.1-b | Efficient implementation of DST-7/DCT-8 by partially skipped transform for large block | M. Koo (LGE) |  |
| 6.1.1-c | 6.1.1-b + DFT | M. Koo (LGE) | Qualcomm |

### CE 6.1.2: Spatially varying transform [JVET-K0139]

For an inter-predicted CU with root_cbf equal to 1, a svt_flag may be signaled to indicate whether the whole residual block or a sub-part of the residual block is decoded. The latter case is called a spatially varying transform (SVT), in which a part of the residual block is coded and the rest of the residual block is forced to be zero.

When SVT is used for a CU, SVT type and SVT position information are further decoded from the bitstream. There are two SVT types and three SVT positions, as indicated in Figure 1. For SVT-V (or SVT-H), the TU width (or height) equals to half of the CU width (or height), and three TU positions are equally spaced as (i/4) of the CU width (or height) to the top-left corner of the CU, where i=0, 1 and 2. The three positions are hence denoted as position 0, 1 and 2.

Both TU and CU boundaries are filtered in de-blocking filtering. SVT-V (or SVTH) is allowed when CU width (or height) is in the range of [8, 32]. To ensure the gap between two filtered boundaries are not smaller than 4 pixels, Position 1 of SVT-V (or SVT-H) is not allowed when CU width (or height) equals to 8.

Position-dependent core transform is applied in SVT. The three positions are associated with different core transforms. More specifically, the horizontal and vertical transforms for each SVT position is specified in Figure 1. Since only DCT-8 and DST-7 core transforms are adopted to VVC draft 2, in order not to introduce additional core transform, the DST-1 used in JVET-K0139 for Position 1 is replaced by DCT-2.

A fast algorithm is designed for SVT. For each SVT mode, a RD cost is estimated based on the SSD of the residual-skipped part. An SVT mode is skipped in RDO if the estimated RD cost of the SVT mode is larger than the actual RD cost of the best mode. In addition, only the best 3 SVT modes in terms of the estimated RD cost are tried in RDO.

Figure 1 Illustration of six SVT modes

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.2-a | Normatively enable SVT for inter blocks predict by Merge mode with merge idx 0/1 or by AMVP mode | Y. Zhao (Huawei) | Qualcomm |
| 6.1.2-b | Normatively enable SVT for all inter blocks | Y. Zhao (Huawei) |  |
| 6.1.2-c | Inter AMT | Y. Zhao (Huawei) |  |
| 6.1.2-d | 6.1.2-a + Inter AMT | Y. Zhao (Huawei) |  |
| 6.1.2-e | 6.1.2-b + Inter AMT | Y. Zhao (Huawei) |  |

### CE 6.1.3: Primary transform core with 8-bit precision [JVET-K0290]

A set of 8-bit transform cores for DCT-2, DST-7/DCT-8 are proposed.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.3-a | Primary transform core with 8-bit precision for both forward and backward transform | X. Zhao (Tencent) | K. Choi (Samsung) |
| 6.1.3-b | Primary transform core with 8-bit precision for only backward transform | X. Zhao (Tencent) | K. Choi (Samsung) Y. Zhao (Huawei) |

### CE 6.1.4: Fast DST-7/DCT-8 with dual implementation support [JVET-K0291]

A fast DST-7/DCT-8 method, which supports dual implementations (identical results between matrix multiplication and fast method) is proposed. Some integer coefficients of DST-7/DCT-8 cores are adjusted slightly, such that it is guaranteed identical results can be achieved between matrix multiplication or fast method.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.4-a | Fast method based DST-7/DCT-8 implementation using adjusted transform cores for intra AMT | X. Zhao (Tencent) | K. Choi (Samsung) Qualcomm |
| 6.1.4-b | Fast method based DST-7/DCT-8 implementation using adjusted transform cores for intra and inter AMT | X. Zhao (Tencent) | K. Choi (Samsung) |

### CE 6.1.5: Chroma AMT [JVET-K0121]

It is proposed to extend AMT to chroma transform block (TB) with an additional flag (chromaAmtFlag) which indicates whether to use default transform type or the transform type of the collocated luma TB as chroma transform.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.5-a | Chroma AMT | Sony | Y. Zhao (Huawei) |
| 6.1.5-b |  |  |  |

### CE 6.1.6: Efficient Implementations of AMT with Transform Adjustment Stages [JVET-K0272, JVET-K0299]

This is a technique proposed to reduce the complexity of AMT by providing an efficient alternative that approximates the AMT transforms using only a transform of the DCT-2 family and adjustment stages of low complexity. The proposed adjustment stages are defined using sparse block-band orthogonal matrices, which provide good approximation for the set of AMT transforms in VTM (DST-7/DCT-8)

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.6-a | Efficient Implementations of AMT with Transform Adjustment Stages | Qualcomm | X. Zhao (Tencent) |
| 6.1.6-b | Further simplification for AMT complexity reduction | Orange | Qualcomm |

### CE 6.1.7: AMT simplification by reusing DCT-2 partial butterfly [JVET-K0130, JVET-K0265, JVET-K0292, JVET-K0394]

## Test1: Change AMT transform to DST-4/DCT-4

Proposal contributions: JVET-K0130, JVET-K0265, JVET-K0394

CE test number: CE6.1.7-a

This test proposes to replace DST-7/DCT-8 to DST-4/DCT-4. Smaller than or equal to N-point DCT-4 logics are included in 2N-point DCT-2 partial butterfly logic. So DCT-4/DST-4 transforms can be enabled without additional transform core from DCT-2.

## Test2: Remove dependency of AMT index on intra/inter

Proposal contributions: JVET-K0130

CE test number: CE6.1.7-b, CE6.1.7-c

This test proposes to align the assignment of transforms to AMT index on intra and inter.

## Test3: Add DST-2/DCT-2 and switch AMT candidate depending on intra direction

Proposal contributions: JVET-K0265, JVET-K0394

CE test number: CE6.1.7-d, CE6.1.7-e

This test proposes to add DST-2/DCT-2 as AMT candidate. The AMT candidate (transform subset) is selected based on the intra prediction mode.

## Test4: Embed DST-7/DCT-8 into DCT-2

Proposal contributions: JVET-K0292

CE test number: CE6.1.7-f, CE6.1.7-g

This test proposes to embed DST-7/DCT-8 into DCT-2. The transform core of DCT-2 is replaced by a new transform core which is a mixture of DCT-2 bases and DST-7 bases, namely COT (Compound Orthonormal Transform). With the proposed method, 16-point and 32-point DST-7 transform cores can be extracted from the proposed 64-point DCT-2.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.7-a | AMT intra: DST-4 / DCT-4 AMT inter: DCT-4 / DST-4 Max AMT size: 32 | K. Naser (Technicolor) Panasonic Sony | X. Zhao (Tencent) |
| 6.1.7-b | AMT intra: DST-7 / DCT-8 AMT inter: DST-7 / DCT-8 (align amt_idx as intra) Max AMT size: 32 | K. Abe (Panasonic) | T. Tsukuba (Sony) |
| 6.1.7-c | AMT intra: DST-4 / DCT-4 AMT inter: DST-4 / DCT-4 (align amt_idx as intra) Max AMT size: 32 | K. Abe (Panasonic) | T. Tsukuba (Sony) |
| 6.1.7-d | AMT intra: DST-7 / DCT-8 / DST-2 / DCT-2 AMT inter: DCT-8 / DST-7 Max AMT size: 32 | T. Tsukuba (Sony) K. Naser (Technicolor) | K. Abe (Panasonic) |
| 6.1.7-e | AMT intra: DST-4 / DCT-4 / DST-2 / DCT-2 AMT inter: DCT-4 / DST-4 Max AMT size: 32 | T. Tsukuba (Sony) Technicolor | K. Abe (Panasonic) |
| 6.1.7-f | 32- and 64-point DCT-2 replaced by COT AMT intra: DST-7 / DCT-8 AMT inter: DCT-8 / DST-7 Max AMT size: 32 | X. Zhao (Tencent) | K. Naser (Technicolor ) Y. Zhao (Huawei) |
| 6.1.7-g | 32- and 64-point DCT-2 replaced by COT 16-point and 32-point: AMT intra: DST-7 / DCT-8 AMT inter: DCT-8 / DST-7 4-point and 8-point: AMT intra: DST-4 / DCT-4 AMT inter: DCT-4 / DST-4 Max AMT size: 32 | X. Zhao (Tencent) | K. Naser (Technicolor) |

### CE 6.1.8: AMT with DCT-II JVET-J0022

In this test, DCT-II is included in the AMT set, and a lookup table is defined to map the intra prediction mode and block size to the corresponding AMT subset. This is similar to what is defined in JVET-J0022, with a further restriction to use only two transforms additional to DCT-II in order to comply with VTM configuration.

## CE6.2: Core Experiments on Secondary Transformation

### CE6.2.1 Reduced Secondary Transform (RST) [JVET K0099, JVET-K0100]

Reduced Secondary Transform (RST) is investigated as follows. For 4x4 RST, Kx16 direct matrix multiplication is applied; 2) Rx64 direct matrix multiplication for 8x8 NSST is applied for 8x8 RST. In addition, 4x4 RST and 8x8 RST are conditionally applied based on block size.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.2.1-a | RST 4x4 only | M. Koo (LGE) |  |
| 6.2.1-b | RST4x4 and RST 8x8 | M. Koo (LGE) |  |
| 6.2.1-c | 6.2.1-b is applied conditionally based on block size | M. Koo (LGE) |  |

### Experiment title

Experiment description

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.2.x-a |  |  |  |
| 6.2.x-b |  |  |  |

### CE 6.2.3 Transform Signaling [JVET-K0174]

The non-separable secondary transform (NSST) with the following two modifications:

- Reduced angle sets based on grouping intra prediction mode

- Signaling with 1 on/off flag with 4 modes

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.2.3.-a | Modified NSST sets and signaling for 4x4 only | Samsung |  |
| 6.2.3.-b | Modified NSST sets and signaling for 4x4 and 8x8 | Samsung |  |

## CE6.3: Core Experiments on Combinations and Transform Signaling

### CE 6.3.1: Set of transforms [JVET-K0305, JVET-K0306]

The transform is selected among a set of predefined transform candidates for each CU. Each of the transform candidates specifies a primary horizontal transform, a primary secondary transform, and a non-separable secondary transform (which may also be equal to the identity transform). The ordered list of transform candidates depends on the block size, prediction mode and channel type. The transform chosen is signaled as follows: When cbf is equal to 1, a flag is transmitted which specifies whether the first transform in the candidate list is used. When this flag is equal to 0, the following applies: If the number of non-zero transform coefficient levels is greater than a threshold, another syntax element is transmitted which indicates the transform candidate used; otherwise, the second transform candidate of the list is used.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.3.1-a | Set of transforms comprising primary transforms only | HHI |  |
| 6.3.1-b | Set of transforms comprising primary and 4x4 secondary transforms | HHI | X. Zhao (Tencent) |
| 6.3.1-c | Set of transforms comprising primary and secondary transforms including dimensions higher than 4x4 | HHI |  |

### CE 6.3.2: Coupled primary and secondary transform [JVET-J0054, JVET-K0085, JVET-K0099, JVET-K0100]

It is proposed to couple the utilization of primary and secondary transforms, and the combination of primary and secondary transform is indicated by a single transform index. Instead of signaling the indices of primary and secondary transform independently, the primary and secondary transform is coupled and signaled by only one transform index. In addition, the proposed method can be applied for both luma and chroma components.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.3.2-a | Coupled primary and secondary transform for luma only, and secondary transform based on 4x4 NSST | X. Zhao (Tencent) |  |
| 6.3.2-b | Coupled primary and secondary transform for both luma and chroma, and secondary transform based on 4x4 NSST | X. Zhao (Tencent) |  |
| 6.3.2-c | Coupled primary and secondary transform for both luma and chroma, and secondary transform based on 4x4 NSST, with reduced transform sets based on grouping intra prediction modes | X. Zhao (Tencent) | Qualcomm |
| 6.3.2-d | Coupled primary and secondary transform for luma only, and secondary transform based on RST for both 4x4 and 8x8 | M. Koo (LGE) |  |
| 6.3.2-e | Coupled primary and secondary transform for both luma and chroma, and secondary transform based on RST for both 4x4 and 8x8 | M. Koo (LGE) |  |
| 6.3.2-f | Coupled primary and secondary transform for both luma and chroma, and secondary transform based on RST for both 4x4 and 8x8, with reduced transform sets based on grouping intra prediction modes | M. Koo (LGE) |  |

### CE 6.3.3: Secondary Transforms Coupled with a Simplified Primary Transformation [JVET-K0405]

This contribution proposes a simplification based on the idea of coupling primary and secondary transforms and signaling them together. A set of transforms Tset = {T0, T1, T2, T3, T4} is determined where each transforms Tx defines a combination of horizontal primary transform, vertical primary transform and a secondary transform. The signaling is the same as AMT in VTM with a 1-bit flag and 2-bit signaling of transform indexes.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.3.3-a | Secondary transforms based on 4x4 NSST coupled with simplified primary transform | Qualcomm | X. Zhao (Tencent) |
| 6.3.3-b | Secondary transforms based on both 4x4 NSST and 8x8 NSST coupled with a simplified primary transform | Qualcomm |  |

# Conditions

The following test conditions shall be used for this CE:

## Software

The Test Model [1] shall be used for these experiments, as specified in JVET-K1010 [2].

## Test configurations

This CE shall use the common test conditions as described in JVET-K1010. Experimental results shall be reported under two configurations: Test Mode (VTM) and Benchmark Set (BMS) configurations, as provided in JVET-K1010. The VTM and BMS configurations are obtained by using BMS 1.0 software with VTM configuration files (encoder_xxx_vtm1.cfg) and BMS configuration files (encoder_xxx_bms1.cfg), respectively.

The following requirements apply to all CEs:

- As in CTC, all experiments should report results with transform dimensions not larger than 64.

- If transform dimensions larger than 64 are applied, experiment results using anchors (VTM and BMS) with the same max transform dimensions, if applicable, should be reported.

- Any encoder or decoder optimizations (e.g., SIMD or similar implementations, RD optimization changes) that are not in the test model shall be described, and individual experiment results on the optimizations shall be reported.

- For experiments which have impact on the transform precision, i.e., modifications on shift operations in transform, modifications on bit-depth of intermediate data representation, results using low-QP configurations, i.e., QP 5, 10, 15 and 20 shall be provided as supplemental information.

- If new transforms are proposed for both intra and inter residuals, individual results of applying the proposed methods on intra and inter residuals should be reported separately.

## Additional requirements per CE

The following test requirements are additional required, according to the CE. The anchors are described below for each additional test, and the test method is the anchor plus the proposed method. In the following tests, primary (AMT) and secondary transform (NSST) should be enabled/disabled by the configuring the related parameter in encoder configuration files.

### Additional requirements for CE6.1 – primary transforms

- Experimental results with secondary transform disabled (BMS configuration, NSST disabled) shall be reported.

- Experimental results with secondary transform enabled (VTM configuration, NSST enabled) shall be reported as supplemental information.

### Additional requirements for CE6.2 – secondary transforms

- Experimental results using the primary transforms specified in the Test Model [1] with Benchmark Set (BMS) configuration (VTM configuration, AMT enabled) shall be reported.

### Additional requirements for CE6.3 – combinations and signaling

- If new primary transforms are also proposed, then experimental results with secondary transform disabled (BMS configuration, NSST disabled) and enabled (VTM configuration, NSST enabled) shall be both reported.

## Test sequences

The test sequences as described in JVET-J1010 shall be used for experiments.

# Evaluation Criteria

## Quantitative metrics

To obtain objective rate-distortion measurements, the Test Model (TM) [1] and the Benchmark Set (BMS) [1] will be used as two separate anchors. In the BMS anchor, all the coding tools in the BMS are enabled. Proposals will be evaluated using BD rates, encoder and decoder run-time, and experiments will be performed under Common Test Conditions (CTC) as described in JVET-K1010.

## Qualitative evaluation

If sequences coded using a proposed alternative transform exhibit subjective improvements or degradations, proponents shall describe these subjective differences.

## Complexity metrics

For the understanding of complexity, the following shall be provided:

- Operation counts (number of multiplications, additions, and bit-shift operations) for individual transforms. If encoder and decoder use different number of transform operations, operation counts shall be reported for both encoder and decoder separately.

- Memory requirements for storing transform cores

  - Total memory for sets of several transforms (minimum amount, in bits)

  - Amount of memory needed to be accessed to compute one transform

- Specify the minimum bit-precision required by the transforms

  - To represent transform coefficients

  - Precision of arithmetic operations needed during transform computation

  - For storing intermediate data representation (e.g., between vertical and horizontal transformations)

- Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel.

- Other operations and memory requirements relevant to the proposed tool, e.g., scaling matrix modification in quantization

- Encoder and decoder times obtained on the same computing platform for both reference and proposed tools should be provided.

- List of all combinations of transform types (e.g., DCT-2 and DST-7) and transforms block sizes (e.g., 4x4, 4x8, 8x4 and 8x8) used for primary transformation.

- List of all combinations of transforms block size and transform type used for secondary transformation.

- Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations).

- If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. (Especially, the number of multiplications required per pixels)

# Participant task checklist

Participants shall:

- Write source code or receive source code for their corresponding tool. Typically, proponents write the source code and cross-verifiers receive it.

- Examine the software to determine if it is implementing the proposed technique;

- Compile the software and run the tests described in this CE;

- Cross-verifiers shall compare their results to those provided by the proponent;

- Comment on the proposed technique and on the results.

Cross-checkers are asked to provide information about their test conditions and encouraged to comment on the proposed technology. The following table can be used to provide that information.

| Software used for crosschecking |  |
| --- | --- |
| Does crosschecking confirm whether the SW implementation is fully aligned with the algorithm description? |  |
| Does crosschecking confirm whether ALL data points coincide with the results provided by proponent? |  |
| Does crosschecking inspect other things that are not listed above? If yes, please describe. |  |
| Commentary on the potential benefits and disadvantages of the proposed technology (highly encouraged) |  |

# Timeline

General timeline for core experiments

| T1 | Aug. 12, 2018 | - Final CE description uploaded |
| --- | --- | --- |
| T2 | Aug .29, 2018 | - Proponents complete initial integration of tools (in individual branch) - Cross checkers can start initial study of tools / software (Note: Proponents can continue to modify the software until T3) |
| T3 | Sep. 12, 2018 | - Proponents freeze changes to software branches - Proponents tag software and announce on JVET reflector - The names of the cross checkers and the list of specific tests for each CE tool are announced on JVET reflector - Proponents provide full test results - Cross checkers start final cross checks |

All test results and cross check results are to be reported to CE coordinators, for inclusion into CE report.

# References

[1] Test Model software

[2] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET_K1010, Ljubljana, July 2018.