| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K1026-v3 |
| --- | --- |

| Title: | Description of core experiment CE6: transforms and transform signaling |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core experiment description |  |  |
| Author(s) or Contact(s): | Amir Said Xin Zhao | Email: | asaid@qti.qualcomm.com xinzzhao@tencent.com |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction

The purpose of this Core Experiment (CE) is to investigate the coding tools related to transforms and transform signaling on top of the Test Model. In this CE, the tradeoff between compression efficiency and complexity of different transformation tools will be investigated. In addition to encoder and decoder run-times, other complexity metrics, e.g., arithmetic operation counts, storage, transform core bit-precision and internal bit-precision will be also evaluated. The list of tools and experiments to be performed are detailed in this document.

# Participant list

| Affiliation | Name | E-mail | P/C |
| --- | --- | --- | --- |
| Alibaba | Yu-Chen Sun | yuchen.s@alibaba-inc.com |  |
| Apple | Alexandros Tourapis | atourapis@apple.com |  |
| Arris | Yue Yu | Yue.Yu@arris.com |  |
|  | Seungwook Hong | Seungwook.Hong@arris.com |  |
|  | Limin Wang | Limin.Wang@arris.com |  |
| B<>com | Victorien Lorcy | victorien.lorcy@b-com.com |  |
| BBC | Andre Seixas Dias | andre.seixasdias@bbc.co.uk |  |
|  | Saverio Blasi | saverio.blasi@bbc.co.uk |  |
| BrightCove | Yuriy Reznik | yreznik@brightcove.com | C |
| Broadcom | Minhua Zhou | minhua.zhou@broadcom.com |  |
|  | Peisong Chen | peisong.chen@broadcom.com |  |
|  | Wade Wan | wade.wan@broadcom.com |  |
| ByteDance | Li Zhang | lizhang.idm@bytedance.com |  |
| Canon | Chris Rosewarne | Chris.Rosewarne@cisra.canon.com.au |  |
|  | Andrew Dorrell | Andrew.Dorrell@cisra.canon.com.au |  |
| Chips & Media | D.Y. Kim | brad.kim@chipsnmedia.com |  |
| Chosun Univ. | Bumshik Lee | bslee@chosun.ac.kr |  |
| Comcast | Dan Grois | groisz@yahoo.com |  |
|  | Yasser Syed | Yasser_Syed@comcast.com |  |
| Ericsson | Jacob Ström | jacob.strom@ericsson.com |  |
| ETRI | Jungwon Gang | jungwon@etri.re.kr |  |
|  | Seongchang Im | sclim@etri.re.kr |  |
| FastVDO | Pankaj Topiwala | pankajtva@gmail.com |  |
| Florida Atl. Univ. | Hari Kalva | hari.kalva@fau.edu |  |
| Foxconn | Yao-Jen Chang | yaojen.chang.foxconn@gmail.com |  |
| HHI | Heiko Schwarz | heiko.schwarz@hhi.fraunhofer.de | P/C |
|  | Mischa Siekmann | mischa.siekmann@hhi.fraunhofer.de |  |
|  | Santiago de Luxán Hernández | santiago.de.luxan@hhi.fraunhofer.de |  |
| Hikvision | Xuguang Zuo | zuoxuguang@hikvision.com |  |
| HiSilicon | Y. Lin | linyongbing@hisilicon.com |  |
|  | Q. Yu | yuquanhe@hisilicon.com |  |
|  | J. Zheng | zhengjianhua@hisilicon.com |  |
| Huawei | Jianle Chen | jianle.chen@huawei.com | P/C |
|  | Haitao Yang | haitao.yang@huawei.com |  |
|  | Yin Zhao | yin.zhao@huawei.com |  |
|  | Sergey Ikonin | Sergey.Ikonin@huawei.com |  |
|  | Alexey Filippov | Alexey.Filippov@huawei.com |  |
|  | Vasily Rufitskiy | Vasily.Rufitskiy@huawei.com |  |
|  | Alexander Karabutov | karabutov.alexander@huawei.com |  |
|  | Cheung Auyeung | cheung.auyeung@huawei.com |  |
| INSA | Wassim Hamidouche | Wassim.Hamidouche@insa-rennes.fr |  |
| Intel | Samuel Wong | samuel.wong@intel.com |  |
| InterDigital | Yan Ye | yan.ye@interdigital.com |  |
|  | Yuwen He | yuwen.he@interdigital.com |  |
|  | Xiaoyu Xiu | xiaoyu.xiu@interdigital.com |  |
|  | Rahul Vanam | rahul.vanam@interdigital.com |  |
|  | Saurav Bandyopadhyay | Saurav.Bandyopadhyay@InterDigital.com |  |
| ITRI | Po-Han Lin | pohan@itri.com |  |
|  | Ching-Chieh Lin | JackLin@itri.com |  |
| KDDI | Kei Kawamura | ki-kawamura@kddi.com |  |
|  | Yoshitaka Kidani | yo-kidani@kddi.com |  |
|  | Sei Naito | se-naitou@kddi.com |  |
| Kwai Inc. | Xianglin(Shawn) Wang | xianglinwang@kwai.com |  |
|  | Yi-Wen Chen | yiwenchen@kwai.com |  |
| LGE | Seung-Hwan Kim | seunghwan3.kim@lge.com | P/C |
|  | Mehdi Salehifar | mehdi.salehifar@lge.com |  |
|  | Moonmo Koo | moonmo.koo@lge.com |  |
|  | Jaehyun Lim | jaehyun.lim@lge.com |  |
| MediaTek | Chih-Wei Hsu | cw.hsu@mediatek.com |  |
|  | Yu-Wen Huang | yuwen.huang@mediatek.com |  |
|  | Ching-Yeh Chen | chingyeh.chen@mediatek.com |  |
|  | Tzu-Der Chuang | peter.chuang@mediatek.com |  |
|  | Man-Shu Chiang | man-shu.chiang@mediatek.com |  |
|  | Mohammed Sarwer | Mohammed.Sarwer@mediatek.com |  |
|  | Shih-Ta Hsiang | shih-ta.hsiang@mediatek.com |  |
| Netflix | Andrey Norkin | anorkin@netflix.com |  |
| NHK | Shunsuke Iwamura | iwamura.s-gc@nhk.or.jp |  |
| Nvidia | Ankur Saxena | ankur81saxena@gmail.com |  |
| Orange | Pierrick Philippe | pierrick.philippe@orange.com | P/C |
| Panasonic | Kiyofumi Abe | abe.kiyo@jp.panasonic.com | P/C |
|  | Toma Tadamasa | toma.tadamasa@jp.panasonic.com |  |
| Peking University | Kui Fan | kuifan@pku.edu.cn |  |
| Qualcomm | Amir Said | asaid@qti.qualcomm.com | P/C |
|  | Hilmi E. Egilmez | hegilmez@qti.qualcomm.com |  |
|  | Yung-Hsuan Chao | yunghsua@qti.qualcomm.com |  |
|  | Vadim Seregin | vseregin@qti.qualcomm.com |  |
|  | Ted Hsieh | thsieh@qti.qualcomm.com |  |
| Samsung | Kiho Choi | kiho14.choi@samsung.com | P/C |
| Sejong University | Yung-Lyul Lee | yllee@sejong.ac.kr |  |
|  | Nam-Uk Kim | nukim@sju.ac.kr |  |
| Sharp | Yukinobu Yasugi | yasugi.yukinobu@sharp.co.jp |  |
|  | Takeshi Chujoh | chujoh.takeshi@sharp.co.jp |  |
|  | Tomohiro Ikai | ikai.tomohiro@sharp.co.jp |  |
|  | Eiichi Sasaki | eiichi.sasaki@sharp.co.jp |  |
| Sharp Labs | Kiran Misra | misrak@sharplabs.com |  |
| SKKU | Byeungwoo Jeon | bjeon@skku.edu |  |
|  | Jeeyoon Park | jiyoonpark@skku.edu |  |
| Sony | Masaru Ikeda | Masaru.Ikeda@sony.com | P/C |
|  | Takeshi Tsukuba | takeshi.tsukuba@sony.com |  |
| Technicolor | Fabrice Le Léannec | Fabrice.Leleannec@technicolor.com | P/C |
|  | Eduard Francois | Edouard.Francois@technicolor.com |  |
|  | Karam Naser | Karam.NASER@technicolor.com |  |
| Tencent | Xin Zhao | xinzzhao@tencent.com | P/C |
|  | Xiang Li | xlxiangli@tencent.com |  |
|  | Liang Zhao | leolzhao@tencent.com |  |
|  | Sehoon Yea | sehoonyea@tencent.com |  |
| Tsinghua University | Benben Niu | nbb16@mails.tsinghua.edu.cn |  |
|  | Liqiang Wang | wlq15@mails.tsinghua.edu.cn |  |
| Ubilinx | Eric Chai | eric.chai@ublnx.com |  |
|  | Weimin Zeng | weimin.zeng@ublnx.com |  |
| WILUS Inc. | John Son | john.son@wilusgroup.com |  |
|  | Zhu Li | zhu.li@ieee.org |  |
|  | Limin Wang | cuiwang@gmail.com |  |

# Core Experiment tasks

The following tasks shall be performed in this Core Experiment (CE):

- Evaluate compression efficiency, computational complexity and memory of transforms studied in this CE;

- Investigate interactions between primary and secondary transform schemes;

- Report the results and conclusions of these experiments.

Core experiments (CEs) are organized according to the following three categories:

- Primary transformations: CE6.1.1 to CE6.1.8 test proposals that introduce changes related to primary transforms only.

- Secondary transformation: CE6.2.1 to CE6.2.3 test proposals that introduce changes for secondary transforms only.

- Combined primary and secondary transformation: CE6.3.1 to CE6.3.3 test proposals that introduce changes for both primary and secondary transforms.

Additional requirements for core experiments in each category are listed in Section 5.3.

# Description of tools

The table below summarizes the tools to be considered in this CE and is followed by additional information for each case. In this document, MTS (Multiple Transform Selection) can be also referred as AMT.

| CE6.1 – Primary Transforms |  |  |  |  |
| --- | --- | --- | --- | --- |
| CE # | Proponent | Related Docs. | Summary of the tool | Cross-checkers |
| 6.1.1 | LGE | JVET-K0096 | - Selection of MTS candidates - Fast DST-7/DCT-8 based on DFT - 32 point MTS based on skipping high frequency coefficients | HHI B<>com Qualcomm |
| 6.1.2 | Huawei | JVET-K0139 | Spatially Varying Transform (SVT) for inter blocks | Qualcomm Sony |
| 6.1.3 | Tencent | JVET-K0290 | Primary transform core with 8-bit precision | Samsung Huawei |
| 6.1.4 | Tencent | JVET-K0291 | Fast DST-7/DCT-8 with dual implementation support | Brightcove Samsung Qualcomm B<>com |
| 6.1.5 | Sony | JVET-K0121 | Chroma AMT: Extending AMT to chroma transform block (TB) with an additional flag which indicates whether to use default transform type or the transform type of the collocated luma TB as chroma transform type. | Withdrawn |
| 6.1.6 | Qualcomm, Orange | JVET-K0272, JVET-K0299 | Efficient Implementations of AMT with Transform Adjustment Stages | Tencent B<>com |
| 6.1.7 | Panasonic Technicolor Sony Tencent | JVET-K0130 JVET-K0265 JVET-K0394 JVET-K0292 | AMT simplification by reusing DCT-2 partial butterfly | Brightcove Huawei Panasonic Technicolor Sony Tencent |
| 6.1.8 | Technicolor | JVET-J0022 | AMT with DCT-II | Sony |

| CE6.2 – Secondary Transforms |  |  |  |  |
| --- | --- | --- | --- | --- |
| CE # | Proponent | Related Docs. | Summary of the tool | Cross-checkers |
| 6.2.1 | LGE | JVET-K0099 JVET-K0100 | - reduced secondary transform with one kernel for 35 transform set - worst case handled reduced secondary transform with one kernel for 4 transform sets | Tencent B<>com |
| 6.2.2 | Samsung | JVET-K0174 | Modified NSST sets and signaling | LGE |

| CE6.3 – Combinations and Signalling |  |  |  |  |
| --- | --- | --- | --- | --- |
| CE # | Proponent | Related Docs. | Summary of the tool | Cross-checkers |
| 6.3.1 | HHI | JVET-K0305, JVET-K0306 | Set of transforms: A set of predefined transform candidates, and each candidate specifies a primary horizontal transform, a primary vertical transform, and a non-separable secondary transform. The transform candidates depend on the block size and the prediction mode. | Brightcove Tencent LGE |
| 6.3.2 | Tencent | JVET-K0085 | Coupled primary and secondary transform | LGE HHI B<>com |
| 6.3.3 | Qualcomm | JVET-K0405 | Secondary Transforms Coupled with Simplified Primary Transformation | HHI Tencent |

Detailed description on each coding tool tested in this CE are provided below.

## CE6.1: Core Experiments on Primary Transformation

### CE 6.1.1: Primary Transform [JVET-K0096]

For selection of MTS candidates, the following two tests are performed:

Test 1: Using 2 MTS candidates for Angular mode and 3 candidates for non-angular mode. Here, non-angular mode indicates DC and Planar modes.

Test 2: Using 1 MTS candidate for all modes. Here, 1 MTS candidate indicates DST7.

For the Fast DST-7/DCT-8 based on DFT, implementation of DST-7/DCT-8 using DFT.

For 32 point MTS based on skipping high frequency coefficients,  a method to zero-out high frequency component for larger transform (i.e., 32x32) is tested.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.1-a | Using 2 MTS candidates for Angular mode and 3 candidates for non-angular mode. Here, non-angular mode indicates DC and Planar modes | M. Koo (LGE) | HHI B<>com |
| 6.1.1-b | Using 1 MTS candidate for all modes. Here, 1 MTS candidate indicates DST7 | M. Koo (LGE) | HHI B<>com |
| 6.1.1-c | implementation of DST-7/DCT-8 using DFT | M. Koo (LGE) | Qualcomm HHI B<>com |
| 6.1.1-d | a method to zero-out high frequency component for larger transform (i.e., 32x32) | M. Koo (LGE) | HHI B<>com |

### CE 6.1.2: Spatially varying transform [JVET-K0139]

For an inter-predicted CU with root_cbf equal to 1, a svt_flag may be signaled to indicate whether the whole residual block or a sub-part of the residual block is decoded. The latter case is called a spatially varying transform (SVT), in which a part of the residual block is coded and the rest of the residual block is forced to be zero.

When SVT is used for a CU, SVT type and SVT position information are further decoded from the bitstream. There are two SVT types and two SVT positions, as indicated in Figure 1. For SVT-V (or SVT-H), the TU width (or height) equals to half of the CU width (or height) or 1/4 of the CU width (or height), signaled by another flag. If one side of CU is 8, 1/4 of the side is not allowed. Hence, there are at most 8 SVT modes for a CU.

Both TU and CU boundaries are filtered in de-blocking filtering. SVT-V (or SVTH) is allowed when CU width (or height) is in the range of [8, maxSize]. When one side of the TU is greater than 32, the corresponding transform is set as DCT-2, and thus the maximum non-DCT-2 transform size is still 32. The maxSize is signaled in slice header. For HD and 4K sequences, maxSize is set as 64 for all slices; for other smaller resolution sequences, maxSize is set as 32 for all slices.

Position-dependent core transform is applied in SVT. The two positions of two types are associated with different core transforms. More specifically, the horizontal and vertical transforms for each SVT position is specified in Figure 1.For example, the horizontal and vertical transforms for SVT-V position 0 is DCT-8 and DST-7, respectively.

A fast algorithm is designed for SVT. For each SVT mode, a RD cost is estimated based on the SSD of the residual-skipped part. An SVT mode is skipped in RDO if the estimated RD cost of the SVT mode is larger than the actual RD cost of the best mode. In addition, only the best 4 SVT modes in terms of the estimated RD cost are tried in RDO.

Figure 1 Illustration of SVT modes

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.2-a | Normatively enable SVT for inter blocks predict by Merge mode with merge idx 0/1 or by AMVP mode | Y. Zhao (Huawei) | Qualcomm |
| 6.1.2-b | Normatively enable SVT for all inter blocks | Y. Zhao (Huawei) | Sony |

### CE 6.1.3: Primary transform core with 8-bit precision [JVET-K0290]

A set of 8-bit transform cores for DCT-2, DST-7/DCT-8 are proposed.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.3-a | Primary transform core with 8-bit precision for both forward and backward transform | X. Zhao (Tencent) | K. Choi (Samsung) |
| 6.1.3-b | Primary transform core with 8-bit precision for only backward transform | X. Zhao (Tencent) | K. Choi (Samsung) Y. Zhao (Huawei) |

### CE 6.1.4: Fast DST-7/DCT-8 with dual implementation support [JVET-K0291]

A fast DST-7/DCT-8 method, which supports dual implementations (identical results between matrix multiplication and fast method) is proposed. Some integer coefficients of DST-7/DCT-8 cores are adjusted slightly, such that it is guaranteed identical results can be achieved between matrix multiplication or fast method.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.4-a | Fast method based DST-7/DCT-8 implementation using adjusted transform cores | X. Zhao (Tencent) | K. Choi (Samsung) Qualcomm B<>com |

### CE 6.1.5: Chroma AMT [JVET-K0121] (Withdrawn)

It is proposed to extend AMT to chroma transform block (TB) with an additional flag (chromaAmtFlag) which indicates whether to use default transform type or the transform type of the collocated luma TB as chroma transform.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.5-a | Chroma AMT | Sony | Withdrawn |

### CE 6.1.6: Efficient Implementations of AMT with Transform Adjustment Stages [JVET-K0272, JVET-K0299]

This is a technique proposed to reduce the complexity of AMT by providing an efficient alternative that approximates the AMT transforms using only a transform of the DCT-2 family and adjustment stages of low complexity. The proposed adjustment stages are defined using sparse block-band orthogonal matrices, which provide good approximation for the set of AMT transforms in VTM (DST-7/DCT-8)

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.6-a | Efficient Implementations of AMT with Transform Adjustment Stages (Max. AMT size = 32) | Qualcomm | X. Zhao (Tencent) B<>com |
| 6.1.6-b | Further simplification for AMT complexity reduction | Orange | Qualcomm X. Zhao (Tencent) |
| 6.1.6-c | Efficient Implementations of AMT with Transform Adjustment Stages (Max. AMT size = 64) | Qualcomm | B<>com |

### CE 6.1.7: AMT simplification by reusing DCT-2 partial butterfly [JVET-K0130, JVET-K0265, JVET-K0292, JVET-K0394]

## Test1: Change AMT transform to DST-4/DCT-4

Proposal contributions: JVET-K0130, JVET-K0265, JVET-K0394

CE test number: CE6.1.7-a

This test proposes to replace DST-7/DCT-8 to DST-4/DCT-4. Smaller than or equal to N-point DCT-4 logics are included in 2N-point DCT-2 partial butterfly logic. So DCT-4/DST-4 transforms can be enabled without additional transform core from DCT-2.

## Test2: Remove dependency of AMT index on intra/inter

Proposal contributions: JVET-K0130

CE test number: CE6.1.7-b, CE6.1.7-c

This test proposes to align the assignment of transforms to AMT index on intra and inter.

## Test3: Add DST-2/DCT-2 and switch AMT candidate depending on intra direction

Proposal contributions: JVET-K0265, JVET-K0394

CE test number: CE6.1.7-d, CE6.1.7-e

This test proposes to add DST-2/DCT-2 as AMT candidate. The AMT candidate (transform subset) is selected based on the intra prediction mode.

## Test4: Embed DST-7/DCT-8 into DCT-2

Proposal contributions: JVET-K0292

CE test number: CE6.1.7-f, CE6.1.7-g

This test proposes to embed DST-7/DCT-8 into DCT-2. The transform core of DCT-2 is replaced by a new transform core which is a mixture of DCT-2 bases and DST-7 bases, namely COT (Compound Orthonormal Transform). With the proposed method, 16-point and 32-point DST-7 transform cores can be extracted from the proposed 64-point DCT-2.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.7-a | AMT intra: DST-4 / DCT-4 AMT inter: DCT-4 / DST-4 Max AMT size: 32 | K. Naser (Technicolor) Panasonic Sony | X. Zhao (Tencent) |
| 6.1.7-b | AMT intra: DST-7 / DCT-8 AMT inter: DST-7 / DCT-8 (align amt_idx as intra) Max AMT size: 32 | K. Abe (Panasonic) | T. Tsukuba (Sony) |
| 6.1.7-c | AMT intra: DST-4 / DCT-4 AMT inter: DST-4 / DCT-4 (align amt_idx as intra) Max AMT size: 32 | K. Abe (Panasonic) | T. Tsukuba (Sony) |
| 6.1.7-d | AMT intra: DST-7 / DCT-8 / DST-2 / DCT-2 AMT inter: DCT-8 / DST-7 Max AMT size: 32 | T. Tsukuba (Sony) K. Naser (Technicolor) | K. Abe (Panasonic) |
| 6.1.7-e | AMT intra: DST-4 / DCT-4 / DST-2 / DCT-2 AMT inter: DCT-4 / DST-4 Max AMT size: 32 | T. Tsukuba (Sony) Technicolor | K. Abe (Panasonic) |
| 6.1.7-f | 32- and 64-point DCT-2 replaced by COT AMT intra: DST-7 / DCT-8 AMT inter: DCT-8 / DST-7 Max AMT size: 32 | X. Zhao (Tencent) | K. Naser (Technicolor ) Y. Zhao (Huawei) |
| 6.1.7-g | 32- and 64-point DCT-2 replaced by COT 16-point and 32-point: AMT intra: DST-7 / DCT-8 AMT inter: DCT-8 / DST-7 4-point and 8-point: AMT intra: DST-4 / DCT-4 AMT inter: DCT-4 / DST-4 Max AMT size: 32 | X. Zhao (Tencent) | K. Naser (Technicolor) |

### CE 6.1.8: AMT with DCT-II JVET-J0022

In this test, DCT-II is included in the AMT set, and a lookup table is defined to map the intra prediction mode and block size to the corresponding AMT subset. This is similar to what is defined in JVET-J0022, with a further restriction to use only two transforms additional to DCT-II in order to comply with VTM configuration.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.8-a | DCT-II is included in AMT together with a lookup table | Technicolor | Masaru Ikeda (Sony) |
| 6.1.8-b | 6-1.8-a using only DCT-II, DST-IV and DCT-VIII | Technicolor | Masaru Ikeda (Sony) |

## CE6.2: Core Experiments on Secondary Transformation

### CE6.2.1 Reduced Secondary Transform (RST) [JVET K0099, JVET-K0100]

Reduced Secondary Transform (RST) is investigated with the following aspects.

- Use one transform kernel per transform set

- Use only 4 transform set (instead of 35)

- Apply worst case handling method to reduce the number of multiplications (using this method the worst case number of multiplications becomes 8 per pixel. Note that 4x4 NSST requires 16 multiplications per pixel)

- Combination with 6.1.1.a (Using 2 MTS candidates for Angular mode and 3 candidates for non-angular mode)

- Combination with 6.1.1.b (Using 1 MTS candidate for all modes)

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.2.1-a | a+b+c+e | M. Koo (LGE) | X. Zhao (Tencent) B<>com |
| 6.2.1-b | a+b+c+d | M. Koo (LGE) |  |
| 6.2.1-c | a+e | M. Koo (LGE) | X. Zhao (Tencent) |
| 6.2.1-d | a+d | M. Koo (LGE) |  |

### CE 6.2.3 Transform Signaling [JVET-K0174]

The non-separable secondary transform (NSST) with the following two modifications:

- Reduced angle sets based on grouping intra prediction mode

- Signaling with 1 on/off flag with 4 modes

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.2.3.-a | Modified NSST sets and signaling for 4x4 only | Samsung | LGE |
| 6.2.3.-b | Modified NSST sets and signaling for 4x4 and 8x8 | Samsung | LGE |

## CE6.3: Core Experiments on Combinations and Transform Signaling

### CE 6.3.1: Set of transforms [JVET-K0305, JVET-K0306]

The transform is selected among a set of predefined transform candidates for each CU. Each of the transform candidates specifies a primary horizontal transform, a primary secondary transform, and a non-separable secondary transform (which may also be equal to the identity transform). The ordered list of transform candidates depends on the block size, prediction mode and channel type. The transform chosen is signaled as follows: When cbf is equal to 1, a flag is transmitted which specifies whether the first transform in the candidate list is used. When this flag is equal to 0, the following applies: If the number of non-zero transform coefficient levels is greater than a threshold, another syntax element is transmitted which indicates the transform candidate used; otherwise, the second transform candidate of the list is used.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.3.1-a | Set of transforms comprising primary transforms only | HHI | Withdrawn |
| 6.3.1-b | Set of transforms comprising primary and 4x4 secondary transforms | HHI | X. Zhao (Tencent) |
| 6.3.1-c | Set of transforms comprising primary and secondary transforms including dimensions higher than 4x4 | HHI | Moonmo Koo (LGE) |

### CE 6.3.2: Coupled primary and secondary transform [JVET-J0054, JVET-K0085, JVET-K0099, JVET-K0100]

It is proposed to couple the utilization of primary and secondary transforms, and the combination of primary and secondary transform is indicated by a single transform index. Instead of signaling the indices of primary and secondary transform independently, the primary and secondary transform is coupled and signaled by only one transform index. In addition, the proposed method can be applied for both luma and chroma components.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.3.2-a | Coupled primary and secondary transform for luma only, and secondary transform based on 4x4 NSST | X. Zhao (Tencent) | M. Salehifar (LGE) B<>com |
| 6.3.2-b | Coupled primary and secondary transform for both luma and chroma, and secondary transform based on 4x4 NSST | X. Zhao (Tencent) | M. Salehifar (LGE) |
| 6.3.2-c | Coupled primary and secondary transform for both luma and chroma, and secondary transform based on 4x4 NSST, with reduced transform sets based on grouping intra prediction modes | X. Zhao (Tencent) | Qualcomm |
| 6.3.2-d | Coupled primary and secondary transform for luma only, and secondary transform based on RST for both 4x4 and 8x8 | M. Koo (LGE) |  |
| 6.3.2-e | Coupled primary and secondary transform for both luma and chroma, and secondary transform based on RST for both 4x4 and 8x8 | M. Koo (LGE) |  |
| 6.3.2-f | Coupled primary and secondary transform for both luma and chroma, and secondary transform based on RST for both 4x4 and 8x8, with reduced transform sets based on grouping intra prediction modes | M. Koo (LGE) |  |

### CE 6.3.3: Secondary Transforms Coupled with a Simplified Primary Transformation [JVET-K0405]

This contribution proposes a simplification based on the idea of coupling primary and secondary transforms and signaling them together. A set of transforms Tset = {T0, T1, T2, T3, T4} is determined where each transforms Tx defines a combination of horizontal primary transform, vertical primary transform and a secondary transform. The signaling is the same as AMT in VTM with a 1-bit flag and 2-bit signaling of transform indexes.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.3.3-a | Secondary transforms based on 4x4 NSST coupled with simplified primary transform | Qualcomm | X. Zhao (Tencent) |
| 6.3.3-b | Secondary transforms based on both 4x4 NSST and 8x8 NSST coupled with a simplified primary transform | Qualcomm |  |

# Conditions

The following test conditions shall be used for this CE:

## Software

The Test Model [1] shall be used for these experiments, as specified in JVET-K1010 [2].

## Test configurations

This CE shall use the Common Test Conditions (CTC) as described in JVET-K1010. Experimental results shall be reported under CTC using VTM configuration, as described in JVET-K1010. The VTM configuration is obtained by using BMS 2.0.1 software with VTM configuration files (encoder_xxx_vtm1.cfg). Experimental results obtained under BMS (Benchmark Set) configuration are optional and regarded as supplemental information. The BMS configuration is obtained by using BMS 2.0.1 software with BMS configuration files (encoder_xxx_bms1.cfg).

The following requirements apply to all CEs:

- As in CTC, all experiments should report results with transform dimensions not larger than 64.

- If transform dimensions larger than 64 are applied, experiment results using anchors (VTM and BMS) with the same max transform dimensions, if applicable, should be reported.

- Any encoder or decoder optimizations (e.g., SIMD or similar implementations, RD optimization changes) that are not in the test model shall be described, and individual experiment results on the optimizations shall be reported.

- For experiments which have impact on the transform precision, i.e., modifications on shift operations in transform, modifications on bit-depth of intermediate data representation, results using low-QP configurations, i.e., QP 5, 10, 15 and 20 shall be provided as supplemental information.

- If new transforms are proposed for both intra and inter residuals, individual results of applying the proposed methods on intra and inter residuals should be reported separately.

## Additional requirements per CE

The following test requirements are additional required, according to the CE. The anchors are described below for each additional test, and the test method is the anchor plus the proposed method. In the following tests, primary (MTS, or as known as AMT) and secondary transform (NSST) should be enabled/disabled by the configuring the related parameter in encoder configuration files.

### Additional requirements for CE6.1 – primary transforms

- Experimental results with MTS enabled for both intra and inter coded blocks (VTM configuration).

### Additional requirements for CE6.3 – combinations and signaling

- If new primary transforms are also proposed, experimental results without the secondary transform shall be reported.

## Test sequences

The test sequences as described in JVET-J1010 shall be used for experiments.

# Evaluation Criteria

## Quantitative metrics

To obtain objective rate-distortion measurements, the Test Model (TM) [1] and the Benchmark Set (BMS) [1] will be used as two separate anchors. In the BMS anchor, all the coding tools in the BMS are enabled. Proposals will be evaluated using BD rates, encoder and decoder run-time, and experiments will be performed under Common Test Conditions (CTC) as described in JVET-K1010.

## Qualitative evaluation

If sequences coded using a proposed alternative transform exhibit subjective improvements or degradations, proponents shall describe these subjective differences.

## Complexity metrics

For the understanding of complexity, the following shall be provided:

- Operation counts (number of multiplications, additions, and bit-shift operations) for individual transforms. If encoder and decoder use different number of transform operations, operation counts shall be reported for both encoder and decoder separately.

- Memory requirements for storing transform cores

  - Total memory for sets of several transforms (minimum amount, in bits)

  - Amount of memory needed to be accessed to compute one transform

- Specify the minimum bit-precision required by the transforms

  - To represent transform coefficients

  - Precision of arithmetic operations needed during transform computation

  - For storing intermediate data representation (e.g., between vertical and horizontal transformations)

- Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel.

- Other operations and memory requirements relevant to the proposed tool, e.g., scaling matrix modification in quantization

- Encoder and decoder times obtained on the same computing platform for both reference and proposed tools should be provided.

- List of all combinations of transform types (e.g., DCT-2 and DST-7) and transforms block sizes (e.g., 4x4, 4x8, 8x4 and 8x8) used for primary transformation.

- List of all combinations of transforms block size and transform type used for secondary transformation.

- Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations).

- If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. (Especially, the number of multiplications required per pixels)

# Participant task checklist

Participants shall:

- Write source code or receive source code for their corresponding tool. Typically, proponents write the source code and cross-verifiers receive it.

- Examine the software to determine if it is implementing the proposed technique;

- Compile the software and run the tests described in this CE;

- Cross-verifiers shall compare their results to those provided by the proponent;

- Comment on the proposed technique and on the results.

Cross-checkers are asked to provide information about their test conditions and encouraged to comment on the proposed technology. The following table can be used to provide that information.

| Software used for crosschecking |  |
| --- | --- |
| Does crosschecking confirm whether the SW implementation is fully aligned with the algorithm description? |  |
| Does crosschecking confirm whether ALL data points coincide with the results provided by proponent? |  |
| Does crosschecking inspect other things that are not listed above? If yes, please describe. |  |
| Commentary on the potential benefits and disadvantages of the proposed technology (highly encouraged) |  |

# Timeline

General timeline for core experiments

| T1 | Aug. 12, 2018 | - Final CE description uploaded |
| --- | --- | --- |
| T2 | Sep. 6, 2018 | - Proponents complete initial integration of tools (in individual branch) - Cross checkers can start initial study of tools / software (Note: Proponents can continue to modify the software until T3) |
| T3 | Sep. 14, 2018 | - Proponents freeze changes to software branches - Proponents tag software and announce on JVET reflector - The names of the cross checkers and the list of specific tests for each CE tool are announced on JVET reflector - Proponents provide full test results - Cross checkers start final cross checks |

All test results and cross check results are to be reported to CE coordinators, for inclusion into CE report.

# References

[1] Test Model software

[2] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, JVET_K1010, Ljubljana, July 2018.