| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K1027-v3 |
| --- | --- |

| Title: | Description of Core Experiment 7 (CE 7): Quantization and coefficient coding |  |  |
| --- | --- | --- | --- |
| Status: | Output document approved by JVET |  |  |
| Purpose: | CE Description |  |  |
| Author(s) or Contact(s): | Heiko Schwarz Muhammed Coban Cheung Auyeung | Email: | heiko.schwarz@hhi.fraunhofer.de mcoban@qti.qualcomm.com cheung.auyeung@huawei.com |
| Source: | CE Coordinators |  |  |

_____________________________

# Introduction

The purpose of this core experiment is to explore the coding efficiency and complexity impact of proposed algorithms for quantization and transform coefficient coding. This core experiment evaluates the impact of:

- Quantization of transform coefficient

- Context modeling/selection/coding order for syntax elements related to transform coefficients

- Scanning of transform coefficient levels

The objectives of this CE include:

- Investigation of coding efficiency and complexity of quantization schemes.

- Investigation of the impact of context selection and signaling schemes for transform coefficient levels on the coding efficiency and complexity.

- Investigation of coding efficiency and complexity of transform coefficient scanning order methods.

# Participants List

| Affiliation | Person, Email address | P | C |
| --- | --- | --- | --- |
| Qualcomm | Muhammed Coban <mcoban@qti.qualcomm.com> Dmytro Rusanovskyy <dmytror@qti.qualcomm.com> Jie Dong <jiedong@qti.qualcomm.com> Ted Hsieh <thsieh@qti.qualcomm.com> Marta Karczewicz <martak@qti.qualcomm.com> | x | x |
| Fraunhofer HHI | Heiko Schwarz <heiko.schwarz@hhi.fraunhofer.de> Tung Nguyen <tung.nguyen@hhi.fraunhofer.de> Christian Helmrich <christian.helmrich@hhi.fraunhofer.de> | x | x |
| Samsung | Yinji Piao <yj1003.piao@samsung.com> Kiho Choi <kiho14.choi@samsung.com> |  | x |
| Huawei | Alexey Filippov <alexey.filippov@huawei.com> Jianle Chen <Jianle.chen@huawei.com> Cheung Auyeung <cheung.auyeung@huawei.com> Yang Haitao <haitao.yang@huawei.com> Yin Zhao <yin.zhao@huawei.com> Sergey Ikonin <Sergey.Ikonin@huawei.com> Biao Wang <biao.wang@huawei.com> Vasily Rufitskiy <vasily.rufitskiy@huawei.com> Alexander Karabutov <karabutov.alexander@huawei.com> Sergey Ikonin <Sergey.Ikonin@huawei.com> | x |  |
| Orange | Pierrick Philippe <pierrick.philippe@orange.com> Felix Henry <felix.henry@orange.com> Gordon Clare <gordon.clare@orange.com> | x |  |
| MediaTek | Mohammed Sarwer <Mohammed.Sarwer@mediatek.com> Olena Chubach <Olena.Chubach@mediatek.com> Yu-Wen Huang <yuwen.huang@mediatek.com> Chih-Wei Hsu <cw.hsu@mediatek.com> Ching-Yeh Chen <chingyeh.chen@mediatek.com> T.-D. Chuang <peter.chuang@mediatek.com> Shih-Ta Hsiang <shih-ta.hsiang@mediatek.com> |  |  |
| Sharp | Eiichi Sasaki <eiichi.sasaki@sharp.co.jp> Takeshi Chujoh <chujoh.takeshi@sharp.co.jp> Tomohiro Ikai <ikai.tomohiro@sharp.co.jp> Kiran Misra <misrak@sharplabs.com> Segall, Andrew <asegall@sharplabs.com> |  |  |
| Apple | Alexis Tourapis <atourapis@apple.com> Jae Hoon Kim <i.jaehoon.kim@icloud.com> |  |  |
| KDDI | Kei Kawamura <ki-kawamura@kddi.com> Kyohei Unno <ky-unno@kddi-research.jp> Yoshitaka Kidani <yo-kidani@kddi.com> |  |  |
| Kwai | Yi-Wen Chen <yiwenchen@kwai.com> Xianglin Wang <xianglinwang@kwai.com> |  |  |
| Bytedance | Li Zhang <lizhang.idm@bytedance.com> Hongbin Liu <liuhongbin.01@bytedance.com> |  |  |
| Nokia | Jani Lainema <jani.lainema@nokia.com> |  |  |
| Broadcom | Minhua Zhou <minhua.zhou@broadcom.com> Wade Wan <wade.wan@broadcom.com> Peisong Chen <peisong.chen@broadcom.com> |  |  |
| Ericsson | Julien Le Tanou <julien.le.tanou@ericsson.com> |  |  |
| Sony | Takeshi Tsukuba <Takeshi.Tsukuba@sony.com> Karl Sharman <karl.sharman@eu.sony.com> |  |  |
| InterDigital | Xiaoyu Xiu <xiaoyu.xiu@interdigital.com> Yuwen He <Yuwen.He@InterDigital.com> Rahul Vanam <Rahul.vanam@interdigital.com> |  |  |
| Canon | Chris Rosewarne <chris.rosewarne@cisra.canon.com.au> Andrew Dorrell <andrew.dorrell@cisra.canon.com.au> |  |  |
| BBC | Andre Seixas Dias <Andre.SeixasDias@bbc.co.uk> Saverio Blasi <saverio.blasi@bbc.co.uk> |  |  |
| Technicolor | Fabien Racape <fabien.racape@technicolor.com> Ya Chen <Ya.chen@technicolor.com> Edouard Francois <Edouard.francois@technicolor.com> Philippe de Lagrange <Philippe.delagrange@technicolor.com> Leleannec Fabrice <Fabrice.Leleannec@technicolor.com> |  |  |
| HiSilicon | Jianhua Zheng <zhengjianhua@hisilicon.com> Quanhe Yu <yuquanhe@hisilicon.com> |  |  |
| ETRI | Jungwon Kang <jungwon@etri.re.kr> Sung-Chang Lim <sclim@etri.re.kr> |  |  |
| Panasonic | Kiyofumi Abe <abe.kiyo@jp.panasonic.com> Han Boon Teo <hanboon.teo@sg.panasonic.com> Chong Soon Lim <chongsoon.lim@sg.panasonic.com> |  |  |
| LG | Jung-Ah Choi <jungah.choi@lge.com> Jin Heo <jin78.heo@lge.com> Sunmi Yoo <sunmi.yoo@lge.com> Jangwon Choi <jangwon84.choi@lge.com> Ling Li <l.li@lge.com> Seung Hwan Kim <seunghwan3.kim@lge.com> Mehdi Salehifar <mehdi.salehifar@lge.com> Moonmo Koo <moonmo.koo@lge.com> Jaehyun Lim <jaehyun.lim@lge.com> |  | x |
| Dolby | Sean McCarthy <smcca@dolby.com> Peng Yin <pyin@dolby.com> |  |  |
| Comcast | Dan Grois <grois@ieee.org> |  |  |
| Intel | Sabine Roessel <sabine.roessel@intel.com> |  |  |
| Tsinghua University | Benben Niu <nbb16@mails.tsinghua.edu.cn> |  |  |
| Tencent | Min Gao <maxwellgao@tencent.com> Xiang Li <xlxiangli@tencent.com> |  |  |
| WILUS | Dongcheol Kim <dongcheol.kim@wilusgroup.com> |  |  |
| Foxconn | Yao-Jen Chang <yaojen.chang.foxconn@gmail.com> |  |  |

P: proponent, C: cross-checker

# Tools under Test

The following tools will be considered in this CE. Details are described in sec. 6.

|  | Proponent | Document | Tool description |
| --- | --- | --- | --- |
| 1 | Huawei | K0321 | Context model selection for transform coefficient levels with complexity reduction |
| 2 | Qualcomm | K0070/K0319 | Entropy coding for dependent quantization |
| 3 | HHI | K0072/K0319 | Reduction of the number of context-coded bins |
| 4 | Orange/ Huawei | J0024 | Transform domain residual sign prediction |
| 4 | KDDI | K0398 | Block-size dependent coefficient scanning |
| 5 | Huawei | K0140 | Adaptive residual scaling |
| 6 | HHI | K0206 | Perceptual QP adaptation |

# Test Description

## Software, reference configurations, and test conditions

All tools have to be integrated into the VTM-2 software (outcome of the Ljubljana meeting).  For sub-CE 7.1, results have to be reported for the following two configurations:

- VTM-2 software and common test conditions (standard configuration files);

- VTM-2 software in low-complexity encoder configuration (common test conditions, but dependent quantization and RDOQ disabled and sign data hiding enabled).

For sub-CEs 7.2 and 7.3, results have to be reported for VTM-2 software and common test conditions only (additional test conditions are optional).

All tools are tested against the corresponding anchor (same set of tools, except the tool to be tested).

An additional integration of the investigated tools into the BMS software is not required for the core experiment. Nonetheless, participants are encouraged to additionally report results for the BMS software.

## Coding Performance Measurements

Measure impact on bitrate/PSNR using provided data (4 points, BD-rate). Proponents are encouraged to provide supplementary results at low and high QPs. In particular for coding tools that are mainly designed for improving perceptual quality, proponents are encouraged to provide additional quality metrics (such as MS-SSIM).

## Complexity Considerations

To provide an indicative assessment of complexity, the following numbers should be provided:

- Encoding time (compared to the reference configurations);

- Decoding time (compared to the reference configuration).

In addition, proponents are encouraged to provide the following (but not limited to) information for complexity and entropy throughput analysis:

- Number of context models

- Number of context-coded and bypass bins in average case and the worst case

- Table size of scanning paths

- Analysis of the frequency of context switches

- Analysis of the dependency between two consecutive bins

- Analysis of entropy parsing dependency (whether bits to bins conversion depends on complex reconstruction and/or pixel processing process)

- Analysis of entropy de-binarization dependency (whether bins to symbols conversion depends on complex reconstruction and/or pixel processing process)

## Sub-CEs

The core experiment is split into the following sub tests:

Sub-CE 7.1:  Transform coefficient coding

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| 7.1.1 | Cheung Auyeung (Huawei) | Context model selection for transform coefficient levels with complexity reduction | Yinji Piao (Samsung) |
| 7.1.2 | Muhammed Coban (Qualcomm) | Entropy Coding for Dependent Quantization | Heiko Schwarz (HHI) |
| 7.1.3 | Heiko Schwarz (HHI) | Reduction of the number of context-coded bins | Muhammed Coban (Qualcomm) |
| 7.1.4 | F. Henry (Orange), A. Filippov (Huawei) | Sign prediction in transform domain (without inverse transform seriality) | Tung Nguyen (HHI) Muhammed Coban (Qualcomm) |

Sub-CE 7.2:  Block adaptive quantization / residual scaling

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| 7.2.1 | Yin Zhao (Huawei) | Adaptive residual scaling | Y. Kidani (KDDI) |
| 7.2.2 | Yin Zhao (Huawei) | Adaptive residual scaling for large blocks only | Y. Kidani (KDDI) |
| 7.2.3 | Yin Zhao (Huawei) | Non-normative delta QP signaling with delta QP being mapped from ARS scaler - Use original pixels of a luma block to estimate a scaler (with the same method in 7.2.1 except for using the original pixels of the block) and map the scaler to an integer delta QP - LCU-level delta QP signaling | Y. Kidani (KDDI) |
| 7.2.4 | Yin Zhao (Huawei) | Adaptive residual scaling (ARS) on top of encoder adaptive QP signaling (method 1) - 7.2.1 + QPA (as proposed in JVET-K0206 or as the version in VTM2) | Christian Helmrich (HHI) |
| 7.2.5 | Yin Zhao (Huawei) | Adaptive residual scaling (ARS) on top of encoder adaptive QP signaling (method 2) - 7.2.1 + LCU-level delta QP signaling in 7.2.3 |  |
| 7.2.6 | Christian Helmrich (HHI) | JVET-K0206: Adaptive QP signaling (encoder only) | Y. Kidani (KDDI) |

Sub-CE 7.3:  Coefficient scanning

|  | Tester | Tool | Cross checker |
| --- | --- | --- | --- |
| 7.3.1 | Y. Kidani (KDDI) | Block size dependent coefficient scanning | Jung-Ah Choi (LG) |
| 7.3.2 | Y. Kidani (KDDI) | Block size dependent coefficient scanning with the NSST in BMS | Yin Zhao (Huawei) |
| 7.3.3 | Y. Kidani (KDDI) | Block size dependent coefficient scanning at low-QP values (QP = 12, 17, 22, 27) | Yin Zhao (Huawei) |

# Timeline

General timeline for core experiments (as agreed by JVET).

| T1 | Aug. 8, 2018 | - Final CE description uploaded |
| --- | --- | --- |
| T2 | 2 weeks after VTM-2 software release | - Proponents completed initial integration of tools (in individual branch) - Cross checkers can start initial study of tools / software (Note: Proponents can continue to modify the software until T3) |
| T3 | Sep. 12, 2018 | - Proponents froze changes to software branches - Proponents tag software and announce on JVET reflector - The names of the cross checkers and the list of specific tests for each CE tool are announced on JVET reflector - Proponents provide full test results - Cross checkers start final cross checks |

Test results and cross check results are reported to CE coordinators (for inclusion into CE report).

# Detailed Description of Coding Tools

## JVET-K0321: Context model selection for transform coefficient levels with complexity reduction

This proposal is intended to reduce the complexity for context model selection for transform coefficient levels. In this proposal, the context model selection method is the same as the description in JVET-K0072 [12], except that the transform coefficient positional based context switching is constrained to the first 4x4 coefficient group in order to reduce computation complexity of other coefficient groups.

The context modelling and binarization depends on the following parameters as defined in JVET-K0072:

•	state:	state of the dependent quantizer

•	numSig:     the number of non-zero levels in the local neighbourhood;

•	sumAbs1:  the sum of partially reconstructed absolute levels (absLevel1) after the first pass

in the local neighbourhood;

•	sumAbs:    the sum of reconstructed absolute levels in the local neighbourhood

Let x and y represent the coordinates of a current scan position inside the transform block. The selected probability models depend on the diagonal position d, which is set according to

d = x + y

Context modelling for the significance flag

The probability model used for coding the sig_coeff_flag for luma component is selected among a set of 54 probability models (3×18).  It is determined as follows:

•	If the sig_coff_flag is in the first coefficient group, the context index ctxIdSig  is determined according to

ctxIdSig = 18 * max(0, state−1) + min( sumAbs1, 5 ) + ( d < 2 ? 12 : ( d < 5 ? 6 : 0 ) )

•	Otherwise the sig_coeff_flag is not in the first coefficient group; the context index ctxIdSig is determined according to

ctxIdSig = 18 * max(0, state−1) + min( sumAbs1, 5 )

The probability model used for coding the sig_coeff_flag for chroma component is selected among a set of 36 probability models (3×12). It is determined as follows:

•	If the sig_coff_flag is in the first coefficient group,  The context index ctxIdSig is determined according to

ctxIdSig = 12 * max(0, state−1) + min( sumAbs1, 5 ) + ( d < 2 ? 6 : 0 ) )

•	Otherwise the sig_coeff_flag is not in the first coefficient group; the context index ctxIdSig is determined according to

ctxIdSig =  12 * max(0, state−1) + min( sumAbs1, 5 )

Context modelling for the parity flag

Similar as for the significance flag, different sets of probability models are used for luma and chroma. The same set of probability models is used for both chroma components. For the luma component, one of 21 probability models is selected; for the chroma component, one of 11 probability models is selected. Hence, in total, 32 context models (21 for luma and 11 for chroma) are used for coding the par_level_flag.

The context index ctxIdPar for luma component specifying the probability model used is determined as follows:

•	If the current scan position is equal to the position of the last non-zero level (as indicted by the transmitted x and y coordinates), ctxIdPar is set equal to 0;

•	Otherwise, if the parity flag is in the first coefficient group, the context index is set to

ctxIdPar = 1 + min( sumAbs1 – numSig, 4 ) + ( d == 0 ? 15 : ( d < 3 ? 10 : 5 ) ) )

•	Otherwise (the parity flag is not in the first coefficient group), the context index is set to

ctxIdPar = 1 + min( sumAbs1 – numSig, 4 ) + 5

The context index ctxIdPar for chroma component specifying the probability model used is determined as follows:

•	If the current scan position is equal to the position of the last non-zero level (as indicted by the transmitted x and y coordinates), ctxIdPar is set equal to 0;

- Othewise, if the parity flag is in the first coefficient group, the context index is set to

ctxIdPar = 1 + min( sumAbs1 – numSig, 4 ) + ( d == 0 ? 5 : 0 )

•	Otherwise (the parity flag is not in the first coefficient group), the context index is set to

ctxIdPar = 1 + min( sumAbs1 – numSig, 4 )

Context modelling for the greater flags

No changes from JVET-K0072 [12].

Rice parameter for binarization of abs_remainder

No changes from JVET-K0072 [12].

## JVET-K0070/K0319 Entropy Coding for Dependent Quantization

This proposal proposes a coefficient significance driven state-machine based entropy coding for dependent quantization described in JVET-K0070/JVET-K0319 [10][13].

In this algorithm a 4-state, two scalar quantizer Trellis Coded Quantization similar to the one described in JVET-J0014 is used [4].  Two scalar quantizers as shown below are used for performing the quantization.

Figure 1: Illustration of the two scalar quantizers used: The first quantizer Q0 maps the transform coefficient levels (black numbers below the points) to even integer multiples of the quantization step size Δ.  The second quantizer Q1 maps the transform coefficient levels to odd integer multiples of the quantization step size Δ or to zero [JVET-J0014].

Transform coefficients are quantized in coding order and the quantizer that is used to perform the quantization is selected based on a state machine that is driven by its previous state and the significance of the previously coded coefficient. The state machine is described by the following figure:

|  | current state; next state for …. sig == 0; sig==1. 0; 0; 2. 1; 2; 0. 2; 1; 3. 3; 3; 1. |
| --- | --- |

Figure 2: State machine diagram for coefficient significance-driven state machine.

On the encoder side, the trellis to decide the coefficient levels is illustrated below figure:

Figure 3: Encoder trellis diagram for coefficient significance driven state machine.

The states 0 and 1 use quantizer Q0, and states 2 and 3 use quantizer Q1. For significance map coding the context set used to code is selected based on the quantizer used, i.e. Qx where x=state>>1. Greater than 1flag (gt1) context set only uses one context set that is identical to the context set used for coding of gt1 flag used in coefficient coding in JVET-K0071.

JVET-K0070 codes gtX {X=0..4} in interleaved fashion and groups the bypass coded bins. Alternatively, gt0, gt1, gt2 and, if regular bin coded, gt3 and gt4 bins can be coded bit plane by bit plane similar to JVET-K0319 style for improved decoder throughput. Restrictions on number of regular coded bins per coefficient group are to be investigated.

## JVET-K0072/K0319: Reduction of the number of context-coded bins

In this experiment, the impact of reducing the number of context-coded bins is investigated. The experiment is based on the transform coefficient coding (and quantization) presented in JVET-K0072 (and adopted to the VTM). The intent of the experiment is to reduce the worst-case decoder complexity by limiting the maximum number of context-coded bins (similar to JVET-K0319).

The experiment includes the following:

- Limit the number of rem_abs_gt2_flags

- Limit the number of rem_abs_gt1_flags

- Simplified derivation of Rice parameter

- Reduce the total number of context models

Suitable maximum numbers are determined as part of the core experiment.

## JVET-J0024: Transform Domain Residual Sign Prediction

The technology is now well known as it was originally proposed in JVET-D0031 and later improved in JVET-J0024. The proposed technology comprises the following steps:

- Preparation prediction of boundary residuals

- Calculation spectrum of boundary line

- Calculation cost function for hypothesis

- Sign values decoding

Preparation prediction of boundary residuals

Figure 4: Pixels used to derive cost value

The first step to get prediction of boundary residuals is to predict boundary pixels of current block. To realize it, a linear extrapolation of lines (Xi,j)/columns (Zi,j) is used:

PXi=2Xi,-1-Xi,-2PZi=2Z-1,i-Z-2,i

where PXi and PZi are linear extrapolations. Pixels of a current block could be presented as a sum of residuals and an inter/intra-predictor:

Yi,j=Ri,j+Pi,j

where Ri,j and Pi,j denote residuals and a predictor, respectively. The second step is to remove an inter/intra predictor and to perform forward transform:

xn=T(PXi-Pi,0)

zm=T(PZi-P0,i)

Calculation spectrum of boundary line

Spectrum of neighboring boundary are calculated as follows:

lyn=m=0M-1ωmRn,mcosπm2M,

tym=n=0N-1ωnRn,mcosπn2N,

where ωi=1M, i=02M, i>0, M is block height.

Calculation cost function for hypothesis

Cost function values should be calculated for every combination of signs to be predicted. So, the number of hypotheses is 2max_num_pred_signs . SSE is used as cost function:

costk=n=0N-1xn-lyn2+m=0M-1zm-tym2

Sign values decoding

Similarly to RSP (as described in JVET-D0031), sign positions are consequently fetched from sign_pred_pos_list starting from the 1st one. At every step, hypothesis having minimal cost function value is chosen and coefficient signs are assigned in accordance with corresponding sign prediction errors. If a sign prediction error value is 1, a coefficient sign is assigned as specified by the hypothesis. Otherwise, the sign is inverted, i.e. “+” is changed to “–” and vice versa. Only those hypotheses are further checked that specify the same signs as previously restored using sign prediction error values as described above. This process is repeated for each element of sign_pred_pos_list. TD-RSP is applied to restore signs of luma quantized transform coefficients only. Chroma coding blocks are not processed by TD-RSP.

Suggested test

A typical decoder will perform the inverse transform of the residues in parallel for all CUs of a CTU prior to the serial intra reconstruction stage. This helps reducing the overall latency as the inverse transform is a computationally intensive stage.

The major drawback of the Residue Sign Prediction approach is that the determination of the residue signs is dependent on samples from previously decoded neighboring blocks, because, in the current design, these reference samples are used to derive a cost function that determines the most probable sign values. Since these signs are required to perform the inverse transforms, this means that the inverse transform can no longer be computed in parallel for all CUs of a CTU. The issue is shown on the diagrams below.

The test aims at replacing the existing cost function by one that does not rely on reference samples. Preliminary investigations indicate that a cost function relying purely on the energy of residue samples provides gains. This is what will be tested in the CE.

Figure 5: Illustration of processing

## JVET-K0398: Block size dependent coefficient scanning

Mode dependent coefficient scanning (MDCS) is introduced in HEVC and JEM for improving coding efficiency. That is applicable for relatively small block by 4x4, 8x4, 4x8, 8x8 and the scanning orders are selected among horizontal, vertical and diagonal scan depends on intra prediction mode. On the other hand, diagonal scan is used fixedly for relatively large blocks whose block size is larger than 8.

|  | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- |
| 4 | M | M | D | D | D |
| 8 | M | M | D | D | D |
| 16 | D | D | D | D | D |
| 32 | D | D | D | D | D |
| 64 | D | D | D | D | D |

Figure 6: Decision table of JEM-MDCS. The vertical line and horizontal line indicate block height and width respectively and M and D denote MDCS and diagonal scanning.

The proposed coefficient scanning is modified based on the ratio of block size. When the ratio of horizontal and vertical size is equal or larger than four, mode-dependent coefficient scanning (MDCS) is applied. The MDCS itself is the same as in JEM. It is noted that MDCS is not applied for both VTM and BMS but diagonal scan is applied for VTM and alternative coefficient coding is applied for BMS respectively instead of MDCS. It is also noted that transform block size 64x64 luma and 32x32 chroma are excluded in VTM and BMS, so these blocks are removed from shown following figure.

|  | 4 | 8 | 16 | 32 | 64 |
| --- | --- | --- | --- | --- | --- |
| 4 | M | M | D | D | D |
| 8 | M | M | D | M | M |
| 16 | D | D | D | D | M |
| 32 | D | M | D | D | D |
| 64 | D | M | M | D | D |

Figure 7: Decision table of proposed method. The vertical line and horizontal line indicate block height and width respectively and M and D indicate MDCS and diagonal scanning.

Regarding a test, competitive analyses of the NSST in BMS should be investigated because it is highly interacted with scanning order. Also, it should be required to study low-QP values (QPs = 12, 17, 22, 27).

## JVET-K0140: Adaptive Residual Scaling

Visual distortion of a block is influenced by the background of the block. Luminance masking and contrast masking effect of the background are incorporated to adjust residual of a block. The supporting background are two lines around the current CU.

A luminance factor fl(Ba) is derived based on the average luminance of the background Ba (normalized to 8-bit),

flBa=1+60-Ba150,       Ba∈0,60   1,                      Ba∈60,170       1+Ba-170425,    Ba∈170,255.

A contrast masking factor fcBc is derived based on the Mean Absolute Difference (MAD) between the background luminance and the average luminance (i.e., a way to measure the contrast Bc).

fcBc=1,            Bc≤5Bc+510,     Bc>5.

The scaler is based on the product of the two factors and is constrained in a range of [2/2,2]. For the flexibility of the scaler, a slice-header parameter s is signaled to further adjust the scaler. At encoder, the parameter s is derived from the geometric mean of masking factors calculated from original luma samples of 16x16 blocks inside the slice, and is written into the slice header.

scaler=clipflBa×fcBc×s2, 0.5, 2.0

To avoid floating-point calculation, the luminance and contrast masking factors and scaler are all integerized into a 9-bit integers, and the squared root operation is realized by a look-up table with 49 entries.

The scaler is derived based on the reconstructed luma samples around the current CU, and is then used to adjust the residual at decoder. The quantization at encoder can still be implemented with scaling of quantization step size using the scaler.

## JVET-K0206: Perceptual QP Adaptation

The perceptual QP adaptation (QPA) algorithm proposed in JVET-H0047 and further improved in JVET-K0206 is based on a subjectively motivated block-wise weighted distortion metric Dw derived from a local visual activity value, where B denotes the sub-blocks (here CTUs) of the picture, indexed via k, wk is the perceptual weight (also called visual sensitivity measure), ak is the block’s visual activity, apic is the picture’s mean visual activity, and s, s' are the original and reconstructed pel values, respectively, of the picture’s luma component.

During encoding, the perceptual QPA makes use of wk to adapt in each CTU at k the perceptually optimized QP and Lagrange parameter based on the default pre-assigned fixed QPslice and Lagrange parameter λslice. Using the adapted QPk and λk in each CTU block ensures that, in terms of visual quality, the coding distortion is optimally distributed within the given picture and within the pictures of the video signal.

JVET-K0206 added the option to, when the --PerceptQPA or --SliceChromaQPOffsetPeriodicity encoder configuration parameter is provided with a positive value, employ input-dependent adaptive luma-to-chroma QP offsets on a slice-by-slice basis. This chroma QPA effectively lowers the coding bit-rate of the chroma channel when its mean visual activity is relatively high compared to the luma channel’s visual activity. At the same time, it compensates for the picture-averaged fixed luma-to-chroma QP mapping by QpCQPY to prevent undesired re-shifting of coding bits from luma to chroma.

The slice-wise chroma QP offsets are transmitted as the slice_cb_qp_offset and slice_cr_qp_offset elements.

# References

- Y.-W Chen, et al., “Description of SDR, HDR and 360° video coding technology proposal by Qualcomm and Technicolor – low and high complexity versions”, JVET-J0021, Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, San Diego, USA, April 2018.

- J. Chen, et al., “Further Improvements to HMKTA-1.0”, VCEG-AZ07, June 2015.

- T. Nguyen, D. Marpe, T. Wiegand, “Non-CE11: Proposed Cleanup for Transform Coefficient Coding”, JCTVC-H0228, 8th Meeting: San José, CA, USA, 1–10 February, 2012.

- Bartnik, et al., “Description of SDR, HDR and 360° video coding technology proposal by Fraunhofer HHI”, JVET-J0014, Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, San Diego, USA, April 2018.

- Alshin, et al., “Description of SDR, HDR and 360° video coding technology proposal by Samsung, Huawei, GoPro, and HiSilicon – mobile application scenario”, JVET-J0024, Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, San Diego, USA, April 2018.

- Chen, et al., “Description of SDR, HDR and 360° video coding technology proposal by Huawei, GoPro, HiSilicon, and Samsung – general application scenario”, JVET-J0025, Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, San Diego, USA, April 2018.

- Hsu, et al., “Description of SDR video coding technology proposal by MediaTek”, JVET-J0018, Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, San Diego, USA, April 2018.

- Y. Kidani, K. Kawamura, S. Naito, “CE7: Block size dependent coefficient scanning (CE7.4.1)”, JVET-K0398, Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Ljubljana, SI, July 2018.

- Iwamura, et al., “Description of SDR and HDR video coding technology proposal by NHK and Sharp”, JVET-J0027, Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, San Diego, USA, April 2018.

- M. Coban, J. Dong, M. Karczewicz, “Entropy Coding for Dependent Quantization“, JVET-K0070, Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Ljubljana, SI, July 2018.

- H. Schwarz, T. Nguyen, D. Marpe, T. Wiegand, “CE7: Transform coefficient coding and dependent quantization (Tests 7.1.2, 7.2.1),” JVET-K0071, Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Ljubljana, SI, July 2018.

- H. Schwarz, T. Nguyen, D. Marpe, T. Wiegand, “Non-CE7: Alternative entropy coding for dependent quantization,” JVET-K0072, Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Ljubljana, SI, July 2018.

- Y. Zhao, H. Yang, J. Chen, “CE7: Adaptive quantization step size scaling (Test 7.3.1),” JVET-K0140, Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Ljubljana, SI, July 2018.

- C. Helmrich, H. Schwarz, D. Marpe, T. Wiegand, “AHG10: Improved perceptually optimized QP adaptation and associated distortion measure,” JVET-K0206, Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Ljubljana, SI, July 2018.

- J. Dong, M. Coban, M. Karczewicz, “TCQ with high throughput coefficient coding“, JVET-K0319, Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Ljubljana, SI, July 2018.

- C. Auyeung, J. Chen, “CE 7.1.4: JEM 7.0 coefficient coding with complexity reduction,” JVET-K0321, Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, Ljubljana, SI, July 2018.