| Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10 July – 18 July 2018 | Document: JVET-K1029-v4 |  | Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 3rd Meeting: Geneva, CH, 26–31 May 2016 | Document: JVET-C0010 |
| --- | --- | --- | --- | --- |

| Title: | Description of Core Experiment 9 (CE9): Decoder Side Motion Vector Derivation |  |  |  |
| --- | --- | --- | --- | --- |
| Status: |  | Output document of JVET |  |  |
| Purpose: |  | Core Experiment Description |  |  |
| Author(s) or Contact(s): |  | Semih Esenlik, Yi-Wen Chen, Fangdong Chen | Email: | semih.esenlik@huawei.com yiwenchen@kwai.com chenfandong@hikvision.com |
| Source: |  | CE coordinators |  |  |

____________________________

# Introduction

The goal of Core Experiment 9 (CE9) is to investigate decoder side motion vector derivation tools. The tools in the scope of this CE include decoder side motion vector refinement methods. The core experiment is organized into two sub-tests:

- CE9.1: Partial Usage of Refined MVs

- CE9.2: DMVR Design

- Document List

Following list includes the input contributions that are considered in the context of Core Experiment 9.

| JVET-K0105 | CE9-related: Simplification of Decoder Side Motion Vector Derivation |
| --- | --- |
| JVET-K0041 | Decoder Side MV Refinement/Derivation with CTB-level concurrency and other normative complexity reduction techniques |
| JVET-K0187 | CE9-related: Low latency template based motion vector refinement |
| JVET-K0275 | Non-CE9: DMVR without Intermediate Buffers and with Padding |
| JVET-K0288 | CE9-related: Memory bandwidth reduction for DMVR |
| JVET-K0295 | CE9-related: Constrained Decoder Side Motion Vector Derivation |
| JVET-K0347 | CE9-related: Addressing the decoding latency issue for decoder-side motion vector refinement (DMVR) |
| JVET-K0360 | CE9-related: Bilateral Matching with Constrained Motion Vector Storage |
| JVET-K0378 | CE9-Test 2.8: DMVR with bilateral matching and 2 half-pel points search |
| JVET-K0359 | CE9.2.5/9.2.6: DMVR with Template-free Bilateral Matching |
| JVET-K0342 | CE9.1.3: Complexity reduction on decoder-side motion vector refinement (DMVR) |
| JVET-K0217 | CE9: DMVR with Bilateral Matching (Test 2.9) |
| JVET-K0199 | CE9.1.1b: Simplification of DMVR |

# List of Participants

| Participants | Proponent | Cross-checker |
| --- | --- | --- |
| Yi-Wen Chen (yiwenchen@kwai.com) |  |  |
| Semih Esenlik (semih.esenlik@huawei.com) Jianle Chen (Jianle.Chen@huawei.com) Ivan Krasnov (ivan.krasnov@huawei.com) Haitao Yang (haitao.yang@huawei.com) Huanbang Chen (chenhuanbang@huawei.com) Xiang Ma (maxiang6@huawei.com) | X | X |
| Wei-Jung Chien (wchien@qti.qualcomm.com) Chun-Chi Chen (chunchic@qti.qualcomm.com) Chao-Hsiung Hung (chaohsiu@qti.qualcomm.com) | X | X |
| Xiaoyu Xiu (xiaoyu.xiu@interdigital.com) Yuwen He (yuwen.he@interdigital.com) Daniel Luo (daniel.luo@interdigital.com) | X | X |
| Meng Xu (mengxxu@tencent.com) Xiang Li (xlxiangli@tencent.com) Sehoon Yea (sehoonyea@tencent.com) | X | X |
| Sriram Sethuraman (Sriram.sethuraman@ittiam.com) | X | X |
| Xu Chen (anderson.chen@hisilicon.com) Jianhua Zheng (zhengjianhua@hisilicon.com) | X | X |
| Hongbin Liu (liuhongbin.01@bytedance.com) Li Zhang (lizhang.idm@bytedance.com) | X | X |
| Fangdong Chen (chenfangdong@hikvision.com) | X | X |
| Tzu-Der Chuang (peter.chuang@mediatek.com) Chih-Wei Hsu (cw.hsu@mediatek.com) Yu-Wen Huang (yuwen.huang@mediatek.com) Ching-Yeh Chen (chingyeh.chen@mediatek.com) Shih-Ta Hsiang (shih-ta.hsiang@mediatek.com) |  |  |
| Minhua Zhou (minhua.zhou@broadcom.com) |  |  |
| Yue Li (lytt@mail.ustc.edu.cn) |  |  |
| Ola Hugosson (Ola.Hugosson@arm.com) |  |  |
| Yin, Peng (pyin@dolby.com) |  |  |
| Kenji Kondo (kenji.kondo@sony.com) |  |  |
| Segall, Andrew (asegall@sharplabs.com) |  |  |
| Kyohei Unno (ky-unno@kddi-research.jp) Kei Kawamura (ki-kawamura@kddi.com) |  |  |
| Hahyun Lee (hanilee@etri.re.kr) Jungwon Kang (jungwon@etri.re.kr) Sung-Chang Lim (sclim@etri.re.kr) |  |  |
| Jing Ya LI (jingya.li@sg.panasonic.com) Chong Soon LIM (chongsoon.lim@sg.panasonic.com) Kiyofumi Abe (abe.kiyo@jp.panasonic.com) |  | X |

# Experimental Conditions and Evaluation

## Software

VTM and BMS software are going to be used. If there is only one software base, the proponents will integrate their code only on this software. In this case VTM and BMS encoder configuration files are going to be used for simulations.

## Anchors

- Proponents implement the tests in BMS SW, for both VTM and BMS tests.

- First anchor: BMS software is going to be used with all BMS tools switched on (default configuration) as anchor. If the proposed technique aims to replace an existing tool in the BMS, that tool is switched off in the test simulation but not in the anchor simulation (which must be same for all tests in all Subtests).

- Second anchor: BMS software is going to be used with all BMS tools switched off via configuring encoder parameters of BMS SW (no change of macro setting).

## Test Sequences, Bit Rates and Coding Conditions

CTC as described in [1].

## Reporting of the Results

All test results and crosscheck results shall be available at T4 and need to be uploaded to the SVN servers that are to be specified later. All crosscheck results will be included in the CE summary report.

## Evaluation of CE Results

## Requirements

- Reference block pre-fetches shall be guaranteed at least for a region of 128x128

- Refined motion vectors of left 128x128 CTU shall not be used for reference data pre-fetch by the DMVR of the current CTU

- Reconstructed samples of neighboring blocks (except for the top CTU boundary neighboring blocks) shall not be used for template matching of the DMVR

- Wavefront parallelism shall be possible. Refined motion vectors or reconstructed samples from the wavefront diagonal or from the right of the wavefront diagonal shall not be used (in particular the top-right CTU shall not be used).

## Worst case latency characteristics

- Description of  processing steps (parallel/sequential)

- Description of data dependency within a 128x128 region of DMVR

## Memory storage

- Intermediate buffers

- Description of the usage of refined motion vectors after consumed by MC

## Complexity in terms of operations

- Number of multiplies and additions

- Number of comparisons

- Number of on-chip memory accesses

## Memory BW increase

- Description of reference block sizes

## Coding efficiency measurements

- Measure impacts on bitrate/PSNR using provided data (4 QP points, BD-rate).

## Runtime measurement

- The encoding and decoding times must be provided using the SSE42 SIMD setting (both anchor and test).

# CE9.1: Partial Usage of Refined MVs

## Functionality Addressed and Expected Gains

- Partial usage of refined MVs in order to improve objective subjective gain.

## Description of the tests

## JVET-K0295 CE9-related: Constrained Decoder Side Motion Vector Derivation (JVET-K0295, Tencent)

## In JVET-K0295, when the previous block coded right before current CU where the spatial MV predictor comes from is regarded in DMVD mode, such MV predictor is marked as unavailable for parsing and motion refinement of current CU. The method is to be adjusted according to the new CE constraint and tested.

## Decoder Side MV Refinement/Derivation with CTB-level concurrency and other normative complexity reduction techniques (JVET-K0041, Ittiam)

A hypothetical CTU-level pipeline is assumed where

- all CUs in a CTB get a complete pipeline slot for reference data pre-fetch (before a subsequent pipeline slot in which MV refinement and motion compensation happen), and

- the refinement and motion compensation can be performed for all CUs in a CTB in parallel.

A configurable number CTUs by which current CTU-row lags the top CTU-row is introduced to allow selective availability of refined MVs from the top CTB-row as AMVP candidates for non-merge CUs and as starting point for refinement for merge CUs within the current CTB.

Also, the refined MVs are used for de-blocking and as TMVPs for future access units.

The experiments will also study additional variants possible with the specified CE requirements.

## CE9: DMVR with Bilateral Matching (Test 2.9) (JVET-K0217, Huawei)

The refined MV is used for de-blocking and temporal MV prediction. However it is not used for any spatial MV prediction.

## CE9-related: Addressing the decoding latency issue for decoder-side motion vector refinement (JVET-K0347, Interdigital)

In this CE test, three methods which uses either unrefined or refined MVs of one bilateral-matching CU for different performance/parallelism tradeoff for the bilateral-matching.

1) The first method uses the unrefined MVs of one bilateral-matching CU to predict the MVs of it neighboring CUs that are in the same CTU. But, for the neighboring CUs that are not in the same CTU, the refined MVs of the current CU are used as the predictor. Additionally, the refined MVs and the unrefined MVs are used for the TMVP/ATMVP and deblocking filter.

2) In the second method, the unrefined MVs of one DMVR CU are used to predict the MVs of its neighboring CU that is in the same CTU row. For the neighboring CUs located in top CTU rows (excluding the top-right CTU), the refined MVs are used for spatial MV prediction. The refined MVs and the unrefined MVs are used for the TMVP/ATMVP and deblocking filter, respectively.

## CE9.1.1: Simplification of DMVR (JVET-K0199, HiSilicon)

In JVET-K0199, refined mv is not using for spatial/de-blocking and temporal mv prediction. In this test, DMVR will be test under different test situation to avoid latency problem.

## CE9-related: Bilateral Matching with Constrained Motion Vector Storage (JVET-K0360, Qualcomm)

## This CE test applies a template-free bilateral matching technique instead of template matching to DMVR. It computes a matching cost between two uni-prediction reference blocks pointed to by merge-block MVs. A search process based on the testing bilateral matching around original MVs is performed within a local search range. The matching cost applied to this test is either a Mean Removed Sum of Absolute Difference (MRSAD) or a SAD. Resulting MVs can be further refined within a half-pel interval horizontally and vertically. The MV refinement process is switchable adaptively based on the MV difference between merge candidates. At the end, refined MVs are taken for motion compensation and un-refined ones are for MV prediction and deblocking filter.

## Test Specifications

| # | description | Tester | Crosschecker |
| --- | --- | --- | --- |
| CE9.1.1 | The refined MV is used for de-blocking and temporal MV prediction, and not for spatial prediction. | S. Esenlik (Huawei) | D. Luo (InterDigital) |
| CE9.1.2 | Unrefined MVs and refined MVs are used for spatial MV prediction with the same CTU and across CTU boundaries. Refined MVs are used for temporal MV prediction and unrefined MV are used for de-blocking. | D. Luo (InterDigital) | M. Xu (Tencent) |
| CE9.1.3 | Unrefined MVs and refined MVs are used for spatial MV prediction with the same CTU row and across CTU top boundaries. Refined MVs are used for temporal MV prediction and unrefined MV are used for de-blocking. | D. Luo (InterDigital) | C.-C. Chen (Qualcomm) |
| CE9.1.4 | Top CTB-row refined MV availability rules. - Lag=0 - use no refined MV for AMVP or as refinement center - Lag=2 – use top-left and top CTB refined MVs for AMVP or as refinement center Study variants given CE requirements | S. Sethuraman (Ittiam) | X. Chen (HiSilicon) |
| CE9.1.5 | The refined MV is used for de-blocking and temporal MV prediction, and not for spatial prediction. Or using refined mv with some limitation for spatial or de-blocking prediction | X. Chen (HiSilicon) | C.-C. Chen (Qualcomm) |
| CE9.1.6 | Refined MV is used for MC and original MV is used for MV prediction and deblocking. | C.-C. Chen (Qualcomm) | D. Luo (InterDigital) |
| CE9.1.7 | The CE requirements will be fulfilled. In addition, the refined MV in previous CU is not used as predictor. - The MV in previous CU is marked as unavailable - The MV in previous CU is marked as unavailable, with unrefined MV compressed - The MV in previous CU is not marked as unavailable but only unrefined MV can be used and not compressed - The MV in previous CU is not marked as unavailable but only unrefined MV can be used and compressed | M. Xu (Tencent) | S. Esenlik (Huawei) |

# CE9.2: DMVR Design

## Functionality Addressed and Expected Gains

- Complexity vs. coding gain tradeoff

- Investigated aspects: interpolation filters, padding, search range, early termination and matching method.

- Complexity reduction in terms of decoding time, number of operations and memory access.

## Description of the tests

## Non-CE9: DMVR without Intermediate Buffers and with Padding (JVET-K0275, Huawei)

According to the proposal all of the candidate motion vector pairs (including the initial central motion vector pair) are first rounded to nearest integer sample resolution. In the figure below an integer search space is shown, where all of the candidate search points are first rounded to the closest integer pixel point.

After the construction of the rounded search space MRSAD search operation is applied on the uni-lateral predictions from refPic0 and refPic1. Note that since the candidate motion vectors are all rounded to integer samples, no interpolation filtering is necessary.

After the best candidate is selected, the rounding operation is reverted by application of inverse rounding.

According to the proposal, motion compensation is applied after DMVR using the final refined MV, and padding is applied based on the difference between the refined and the initial motion vectors.

## Decoder Side MV Refinement/Derivation with CTB-level concurrency and other normative complexity reduction techniques (JVET-K0041, Ittiam)

## The proposal presented the following two complexity reduction aspects:

- A new search pattern for MV refinement which does not exceed 5 new evaluations per stage of refinement and ensures that cost at eight positions around the current center position are considered from the 2nd stage of refinement onwards. Both DCTIF and bilinear interpolation based results were presented.

- A 5-parameter 2-D parametric error surface based sub-pixel estimation technique was proposed that uses only integer distance evaluated SADs to obtain the lowest cost position at up to 1/16th pel accuracy.

Let E(0,0),E(-1,0),E(0,-1),E(1,0) and E(0,1) be SAD values obtained using the Integer-distance refinement at the center and its left, top, right, bottom 1-pixel distance positions

The following 2D separable parabolic function is used for the error surface:

E(x,y) = A*(x-x0)2 + B*(y-y0)2 + C

The optimum sub-pixel offset with respect to the center (x0, y0) is computed as follows:

x0 = (E(-1,0) – E(1,0)) / (2*N*(E(-1,0) + E(1,0) – 2* E(0,0)))

y0 = (E(0,-1) – E(0,1)) / (2*N*(E(0,-1) + E(0,1) – 2* E(0,0)))

where N=1,2,4 and 8 for 1/2, 1/4, 1/8 and 1/16 of a pixel distance accuracy respectively.

In one variant, bilateral matching with symmetric mirroring is used for both integer distance and error surface evaluation. In another variant, bilateral matching is used for integer distance refinement and averaged bi-lateral template based independent error surfaces for L0 and L1 references. Bilinear interpolation is used for lower complexity in this case.

Initial results were also presented with mean-removed SAD. So, combinations with MRSAD and other proposed memory bandwidth reduction methods will be studied in experiments.

## CE9.1.3: Complexity reduction on decoder-side motion vector refinement (DMVR) (JVET-K0342, Interdigital)

In this test, an early termination method is applied to the bilateral matching. Specifically, the sum of absolute difference (SAD) between the two prediction signals (L0 and L1 prediction) using the initial motion vectors of the current CU is calculated. If the SAD is no larger than a predefined threshold, the DMVR is skipped; otherwise, the DMVR is still applied to refine the two motion vectors of the current block.

## CE9.2.5/9.2.6: DMVR with Template-free Bilateral Matching (JVET-K0359, Qualcomm)

### The MV refinement process in this CE test is switchable based adaptively on the MV difference between the selected merge candidate and all the previous ones in the merge list. Shorter interpolation filters (2-, 4-tap) for motion compensation will be tested.

## CE9-Test 2.8: DMVR with bilateral matching and 2 half-pel points search (JVET-K0378, USTC)

A two-point half pel search method is proposed to reduce the complexity of the half pel search operation. The half-pel points to be searched are selected according to the point with minimum matching cost and the point with second best matching cost.

## CE9-related: Simplification of Decoder Side Motion Vector Derivation (JVET-K0105, Bytedance)

Two aspects are tested:

- Disallow DMVR for 4x4 blocks

- Perform SAD calculation on every second row

It was recommended that restriction to even larger block sizes should be considered.

## CE9.1.1: Simplification of DMVR (JVET-K0199, HiSilicon)

In this test, DMVR with bilateral template matching will be test under the DMVR configuration that was adopted in this meeting (SR=2, half-pel on, not using refined mv for spatial and deblocking).

## CE9-related: Low latency template based motion vector refinement (JVET-K0187, Hikvision)

According to the proposal，a motion vector refinement (MVR) based on neighboring motion information. With simple motion information of neighboring blocks, a template is generated to refine an initial MV of the Merge mode candidate. When this method is enabled, the initial motion vector of list0 or MV of list1 in the merge candidates will be further refined by a template matching process. In order to obtain a refined MV, template matching performs a SAD-based search between a template and a block with the same size as the template in a reference picture. One flag is transmitted to signal whether the MVR technique is enabled for a block using Merge mode.

## CE9-related: Memory bandwidth reduction for DMVR (JVET-K0288, Tencent)

## In JVET-K0288, it is proposed to use bilinear interpolation filter for MV refinement and set search range SR to 2 pixels. Moreover, it is proposed to use 6-tap interpolation filter for final MC process for DMVR blocks. Other filters with taps less than 6 will also be tested.

## Test Specifications

| # | description | Tester | Crosschecker |
| --- | --- | --- | --- |
| CE9.2.1 | SAD/MRSAD computation on integer samples without interpolation | S. Esenlik (Huawei) | F. Chen (Hikvision) |
| CE9.2.2 | Padding applied to eliminate memory extension | S. Esenlik (Huawei) | K. Abe (Panasonic) |
| CE9.2.3 | CEX.1.1 and CEX.1.2 combined. | S. Esenlik (Huawei) | C.-C. Chen (Qualcomm) |
| CE9.2.4 | A new search pattern for refinement - DCTIF - Bilinear interpolation | S. Sethuraman (Ittiam) | F. Chen (Hikvision) |
| CE9.2.5 | Use of parametric error surface for estimation of sub-pixel distance offsets using bilinear interpolation. - Bilateral matching with symmetric mirroring - Averaged bilateral template based independent sub-pixel error surface for L0 and L1 | S. Sethuraman (Ittiam) | D. Luo (InterDigital) |
| CE9.2.6 | Study combinations of CEX.2.4 and CEX.2.5 with other proposed complexity reduction methods | S. Sethuraman (Ittiam) | C.-C. Chen (Qualcomm) |
| CE9.2.7 | Early termination for the bilateral matching based on the initial similarity between L-0 and L1 predictions | D. Luo (InterDigital) | S. Esenlik (Huawei) |
| CE9.2.8 | Bilateral template matching is applied on the current DMVR configuration in BMS. | X. Chen (HiSilicon) | S. Sethuraman (Ittiam) |
| CE9.2.9 | 1. Restrict DMVR to different block sizes. 2. Calculate SAD for every two rows. | H. Liu (Bytedance) | S. Esenlik (Huawei) |
| CE9.2.10 | Generate the template with the neighboring MVs under different test conditions | F. Chen (Hikvision) | S. Sethuraman (Ittiam) |
| CE9.2.11 | Combine proposed low latency template matching with low latency DMVR | F. Chen (Hikvision) | Y. Li (USTC) |
| CE9.2.12 | 2 point half-pel search. | Y. Li (USTC) | M. Xu (Tencent) |
| CE9.2.13 | - Test early-termination method based on MV difference between merge candidates. - Test shorter interpolation filters (2-, 4-tap) for motion compensation | C.-C. Chen (Qualcomm) | S. Esenlik (Huawei) |
| CE9.2.14 | - Use 4-tap filter for final motion compensation - Use 6-tap filter for final motion compensation - Use adaptive 4-tap/6-tap for final motion compensation | M. Xu (Tencent) | Y. Li (USTC) |
| CE9.2.15 | - Combination of 9.2.2, 9.2.7 and 9.2.13. - Bilinear interpolation applied for search. - Padding is applied to eliminate memory extension. - Test early-termination method based on MV difference between merge candidates. - Early termination for the bilateral matching based on the initial similarity between L-0 and L1 predictions | S. Esenlik (Huawei) | H. Liu (Bytedance) |
| CE9.2.16 | - Combination of 9.2.2, 9.2.7, 9.2.9 and 9.2.13. - Bilinear interpolation applied for search. - Padding is applied to eliminate memory extension. - Test early-termination method based on MV difference between merge candidates. - Early termination for the bilateral matching based on the initial similarity between L-0 and L1 predictions - Restrict DMVR to different block sizes. - Calculate SAD for every two rows. | S. Esenlik (Huawei) | M. Xu (Tencent) |

# Time-line and Responsibilities

| T1 | 2018-August-08 | Final CE description |
| --- | --- | --- |
| T2 | 2018-August-31 | BMS2.1 is available (tentative) |
| T3 | 2 Weeks after T2 | Completion of final version of the tool integration. Test results and answers to the questions in Section 4.5 are shared with participants. Crosscheck starts. |
| T4 | 2 weeks after T3 | Completion of the crosschecking. The crosscheck results are shared to coordinators. |
| T4 | 2018-September-24 | CE summary report uploaded. |

# References

- “JVET common test conditions and software reference configurations”, JVET-K1010, F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring.