| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K1030-v3 |
| --- | --- |

| Title: | Description of Core Experiment 10: Combined and multi-hypothesis prediction |  |  |
| --- | --- | --- | --- |
| Status: | Output document |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Chih-Wei Hsu Martin Winken | Email: | cw.hsu@mediatek.com martin.winken@hhi.fraunhofer.de |
| Source: | CE coordinators |  |  |

_____________________________

- Abstract

The main goal of this Core Experiment (CE) is to improve the inter prediction of the Test Model by the use of combined predictions. CE10.1 is to test prediction to be combined coming from multiple hypothesis, where one hypothesis refers to prediction from inter mode or from intra mode. CE10.2 is to test prediction to be combined from using motions of neighboring coding units (CUs). CE10.3 is to test prediction to be combined from non-rectangular prediction partitions within one CU. CE10.4 is to test prediction to be combined using diffusion filters. CE10.5 is to test prediction to be combined using multiple affine compensated blocks.

Participants

| Nr. | Name | Company | Email | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
| 1 | Chih-Wei Hsu Man-Shu Chiang Zhi-Yi Lin | MediaTek | cw.hsu@mediatek.com Man-Shu.Chiang@mediatek.com Zhi-Yi.Lin@mediatek.com | X | X |
| 2 | Martin Winken Jennifer Rasch | Fraunhofer HHI | martin.winken@hhi.fraunhofer.de | X | X |
| 3 | Xiu, Xiaoyu | InterDigital | Xiaoyu.Xiu@InterDigital.com | X | X |
| 4 | Ru-Ling Liao | Panasonic | ruling.liao@sg.panasonic.com | X | X |
| 5 | Max Bläser | RWTH Aachen University | blaeser@ient.rwth-aachen.de | X | X |
| 6 | Yongjo Ahn | Digital Insights | yjahn@digitalinsights.co.kr | X |  |
| 7 | Kai Zhang | Bytedance | zhangkai.video@bytedance.com | X |  |

CE10.1: Multi-hypothesis prediction

JVET-K0257 (MediaTek)

In JVET-K0257, multi-hypothesis prediction mode is described to improve uni-prediction of advanced motion vector prediction (AMVP) mode, skip/merge mode, and intra mode. The goal is to combine an existing prediction mode with an extra merge indexed prediction. When applied to skip or merge mode, the merge index for the extra merge indexed prediction is implicitly derived as the succeeding merge candidate (i.e., the already signaled merge index plus one).

Within this CE, the performance of the method described in JVET-K0257 is to be studied for different requirements in terms of the bandwidth for motion compensation, interaction with other tools and computational complexity. Settings affecting these requirements are e.g., the length of the sub-pel interpolation filter.

JVET-K0269 (HHI)

In JVET-K0269, a multi-hypothesis inter prediction mode is described.  In this multi-hypothesis inter prediction mode, one or more additional prediction signals are signaled, in addition to the conventional uni/bi prediction signal.  The resulting overall prediction signal is obtained by sample-wise weighted superposition.

Within this CE, the performance of the method described in JVET-K0269 is to be studied for different settings resulting in different requirements in terms of memory access bandwidth and computational complexity.  Settings affecting these requirements are e.g., the number of additional hypotheses and the length of the sub-pel interpolation filter.

Combination of JVET-K0257 (MediaTek) and JVET-K0269 (HHI)

Within this test, the interaction of the proposed methods of JVET-K0257 with the multi-hypothesis inter prediction of JVET-K0269 is to be tested.  For this purpose, the 3 tools described in JVET-K0257 will be tested individually and jointly with the tool described in JVET-K0269.  The 3 tools of JVET-K0257 are

- Multi-hypothesis prediction for improving AMVP mode

- Multi-hypothesis prediction for improving skip or merge mode

- Multi-hypothesis prediction for improving intra mode

The tool described in JVET-K0269 is:

- Multi-hypothesis inter prediction

Test scenarios

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE10.1.1.a | Multi-hypothesis prediction for uni-prediction as described in 2.1 (JVET-K0257) - Explicit signaling with merge index - Additional 1 hypothesis for uni-prediction - Total 2 hypothesis - Disabled for small block sizes | Chih-Wei Hsu (MediaTek) | Martin Winken (HHI) |
| CE10.1.1.b | Multi-hypothesis prediction for skip/merge mode as described in 2.1 (JVET-K0257) - Implicit derivation of hypothesis - Additional 1 or 2 hypothesis - Total at most 4 hypothesis - 2-tap (bilinear) interpolation filter for multi-hypotheses | Chih-Wei Hsu (MediaTek) | Martin Winken (HHI) |
| CE10.1.1.c | Multi-hypothesis prediction for intra and inter mode as described in 2.1 (JVET- K0257) - Explicit signaling - Additional 1 hypothesis for merge mode - Additional 1 hypothesis for intra mode - Total 2 hypothesis | Chih-Wei Hsu (MediaTek) | Martin Winken (HHI) |
| CE10.1.1.d | Multi-hypothesis prediction as described in 2.1 (JVET- K0257) - Combined test of CE10.1.1.a and CE10.1.1.b and CE10.1.1.c - Disabled for small block sizes - 2-tap (bilinear) interpolation filter for multi-hypotheses | Chih-Wei Hsu (MediaTek) | Martin Winken (HHI) |
| CE10.1.2.a | Multi-hypothesis prediction as described in JVET-K0269 - Additional 2 hypothesis - Total at most 4 hypothesis - Disabled for small block sizes | Martin Winken (HHI) | Chih-Wei Hsu (MediaTek) |
| CE10.1.2.b | Multi-hypothesis prediction as described in JVET-K0269 - Additional 2 hypothesis - Total at most 4 hypothesis - Disabled for small block sizes 4-tap interpolation filter for multi-hypotheses | Martin Winken (HHI) | Chih-Wei Hsu (MediaTek) |
| CE10.1.2.c | Multi-hypothesis prediction as described in JVET-K0269 - Additional 2 hypothesis - Total at most 4 hypothesis - Disabled for small block sizes full-pel only (no sub-pel interpolation filtering) for multi-hypotheses | Martin Winken (HHI) | Chih-Wei Hsu (MediaTek) |
| CE10.1.3 | Combined test of JVET-K0257 and JVET-K0269 | Chih-Wei Hsu (MediaTek) Martin Winken (HHI) | Choi, Byeongdoo (SHARP) |

CE10.2: Overlapped block motion compensation

JVET-K0258 + JVET-K0259 (MediaTek)

In JVET-K0258, OBMC complexity can be reduced by applying OBMC to CU boundary only. Parallel blending method is also proposed.

In JVET-K0259, methods to reduce OBMC memory bandwidth and line buffer are proposed. Samples outside original reference region are not fetched and are replaced with generated samples by replicating the right-most column and the bottom-most row of the original fetched reference picture samples for fractional sample accuracy motion compensation (MC), where w’ and h’ are the width and height of OBMC region, respectively. For blocks coded with sub-block mode, the same padding method can be applied to reduce the required bandwidth. As for line buffer usage, the number of blended lines for above block boundary is reduced from 4 to 2, if current block is at coding tree unit (CTU) row boundary.

In this CE, the methods described in JVET-K0258 and JVET-K259 are jointly considered for different settings resulting in different requirements in terms of memory access bandwidth and computational complexity. The tests include number of padding lines, number of line buffer, constraints for block size, sub CU and boundary CU, and parallel blending.

JVET-K0345 (InterDigital)

In this test, to reduce the complexity, some OBMC processes are skipped when the following conditions are met: 1) both of the neighboring motion and the current motion are either uni-predicted or bi-predicted; 2) the same reference picture(s) are used by the neighboring motion and the current motion; 3) the MV difference between the neighboring motion and the current motion is no larger than 1/2-pel.

JVET-K0258, JVET-K0259 (MediaTek) + JVET-K0345 (InterDigital)

The OBMC early termination method in JVET-K0345 will be tested jointly with the OBMC blending method in JVET-K0258 and bandwidth/line-buffer reduction method in JVET-K0259.

Test scenarios

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE10.2.1 | - Pad the right-most column and the bottom-most row of the reference picture samples - Reduce blended lines to 2 if across CTU boundary - Apply to CU boundary only - Parallel blending | Zhi-Yi Lin (MediaTek) | Xiu, Xiaoyu (InterDigital) |
| CE10.2.2 | - Skip OBMC process to reduce complexity | Xiu, Xiaoyu (InterDigital) | Zhi-Yi Lin (MediaTek) |
| CE10.2.3 | Combined test of CE10.2.1 and CE10.2.2 | Xiu, Xiaoyu (InterDigital) | Hongbin Liu (Bytedance) |

CE10.3: Non-rectangular partitions

JVET-K0144 (Panasonic)

In JVET-K0144, a triangular partition is introduced for motion compensated prediction. A CU can be split using two triangular prediction units, either in diagonal or inverse diagonal direction. Each triangular prediction unit in the CU is inter-predicted using its own motion vectors and reference frame index, which are derived from a merge candidate list. The triangular prediction unit mode is applied to CUs in skip or merge mode.

Within this CE, the possible solutions to the memory access bandwidth and computational complexity of the triangular prediction units are studied. The performance of restricting the triangular prediction units to uni prediction is also studied.

JVET-K0146 (RWTH Aachen University)

In JVET-K0146, a geometric partitioning mode (GEO) is introduced. In allows the splitting of a block by a straight line into two partitions. Each partition can be inter-predicted (MERGE / AMVP) or intra-predicted (modified planar prediction). In continuation of JVET-K0146 it is suggested to further study the impact of non-rectangular partitioning on coding efficiency. In order to address and reduce the memory bandwidth impact, in contrast to JVET-K0146, only uni-prediction shall be considered for inter-predicted segments in all test cases. Also, as diagonal partitions were proposed in this CE it is suggested to investigate how geometric partitioning performs when configured to use only diagonal partitions, an extended set of geometric partitions or the full geometric partitioning.

JVET-K0270 (Digital Insights)

In JVET-K0270, diagonal motion partitions (DMPs) for inter prediction on top of MTT block structure are proposed. In this contribution, DMPs can be applied to AMVP and MERGE mode. In this meeting, many experts concerned about memory bandwidth of worst case. In worst case, the proposed method needs double memory bandwidth compared to bi-prediction.

In this CE, the performance of the proposed method described in JVET-K0270 is to be studied for different settings resulting in different requirements in terms of memory bandwidth and complexity. Settings affecting these requirements are e.g., the inter-prediction directions and modes.

Combination of JVET-K0144 (Panasonic) and JVET-K0270 (Digital Insights)

In this test, the coding performance of combining the triangular prediction units (JVET-K0144) for skip/merge mode and the diagonal motion partitions (JVET-K0270) for AMVP mode is evaluated.

Test scenario

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| 10.3.1.a | - Two triangular prediction units - Applied to skip or merge mode - Disabled for CUs smaller than 8x8 - Restricted to uni prediction (only for small CU size) | Ru-Ling Liao (Panasonic) | Max Bläser (RWTH Aachen University) |
| 10.3.1.b | - Two triangular prediction units - Applied to skip or merge mode - Disabled for CUs smaller than 8x8 - Restricted to uni prediction (for all CU size) | Ru-Ling Liao (Panasonic) | Max Bläser (RWTH Aachen University) |
| 10.3.2.a | Geometric partitioning (inter and intra) with uni-prediction constraint as described in 4.1 - GEO restricted to diagonals - Applied to MERGE / AMVP - Disabled for smaller than 8x8 | Max Bläser (RWTH Aachen University) | Ru-Ling Liao (Panasonic) |
| 10.3.2.b | Same constraints as 10.3.2.a, except - GEO restricted to extended set of partitions | Max Bläser (RWTH Aachen University) | Ru-Ling Liao (Panasonic) |
| 10.3.2.c | Same constraints as 10.3.2.a, except - No restrictions on GEO partitioning | Max Bläser (RWTH Aachen University) | Han Gao (Huawei) |
| 10.3.3.a | Diagonal motion partitions with uni-prediction constraint as described in 4.1 - MERGE mode (Non-SKIP) - AMVP mode | Yongjo Ahn (Digital Insights) | Taeyoung Na (SK telecom) |
| 10.3.3.b | Diagonal motion partitions with uni-prediction constraint as described in 4.1 - Only AMVP mode | Yongjo Ahn (Digital Insights) | Taeyoung Na (SK telecom) |
| 10.3.4 | Combined test of 10.3.1.b and 10.3.3.b | Yongjo Ahn (Digital Insights) Ru-Ling Liao (Panasonic) | Taeyoung Na (SK telecom) |

CE10.4: Diffusion filtering of inter- and intra-prediction signals

JVET-K0323 (HHI)

Diffusion filters may be applied to the prediction signal generated by a hybrid video codec. Two types of diffusion filters were proposed: Uniform and signal dependent diffusion filters. The uniform diffusion filters correlate the extended prediction signal n times using a symmetric filter mask. The signal dependent diffusion filters use the input prediction signal to identify structures of the underlying signal and diffuse along edges rather than perpendicular to them.

An analysis of the worst case number of computations will be provided as was suggested during the meeting in Ljubljana 2018. Further, some variants will be suggested that have a reduced complexity concerning the number of iterations and in terms of filter size.

Test scenarios

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE10.4.1 | • Fast Encoder Decisions and default restrictions • Additionally sending diffusion parameters in merge case • Low complexity variant | Jennifer Rasch (HHI) | Jacob Ström (Ericsson) |

CE10.5: Multiple affine compensated blocks

JVET- K0102

In this contribution, an interweaved prediction approach is proposed for Affine Motion Compensation (AMC). With the interweaved prediction, a coding block is divided into sub-blocks with two different dividing patterns. Then two auxiliary predictions are generated by AMC with the two dividing patterns. The final prediction is calculated as a weighted-sum of the two auxiliary predictions.

Test scenarios

| # | Test | Tester | Cross-checker |
| --- | --- | --- | --- |
| CE10.5.1 | - Interweaved prediction for Affine Motion Compensation (AMC) - The second dividing pattern covers the whole current block | K. Zhang (Bytedance) | Yuwen He (InterDigital) |
| CE10.5.2 | - Interweaved prediction for Affine Motion Compensation (AMC) - The second dividing pattern does not cover the whole current block | K. Zhang (Bytedance) | Yuwen He (InterDigital) |

Test conditions and evaluation criteria

The VVC Test Model (VTM) [1] will be used as one anchor and the Benchmark Set (BMS) will be used as another anchor, where VTM and BMS are configured as follow

- VTM Configuration:

VVC software with BMS branch as provided in the repository are used with JEM tools disabled by SPS flags (using VTM cfg files).

- BMS Configuration:

Same software as in VTM Configuration but using BTM cfg files.

Proposals will be evaluated using BD rates, encoder run time, decoder time with Common Test Conditions (CTC) [2] as agreed at the Lubljana meeting, where VTM results are mandatory and BMS results are optional. Each proponent is suggested to provide additional information related to memory bandwidth and computational complexity.

Expected interdependency with other CE

This CE is related to CE3: Intra prediction and mode coding, and CE4: Inter prediction and MV coding. It is recommended to do combined tests with selected outcomes of CE3 and CE4 to provide information for the group.

Time-line and Responsibilities

General timeline for core experiments (as agreed by JVET).

| T1 | Aug 8, 2018 | - Final CE description uploaded |
| --- | --- | --- |
| T2 | Sep 3, 2018 | - Proponents completed initial integration of tools (in individual branch) - Cross checkers can start initial study of tools / software (Note: Proponents can continue to modify the software until T3) |
| T3 | Sep 17, 2018 | - Proponents froze changes to software branches - Proponents tag software and announce on JVET reflector - The names of the cross checkers and the list of specific tests for each CE tool are announced on JVET reflector - Proponents provide full test results - Cross checkers start final cross checks |

Test results and cross check results are reported to CE coordinators (for inclusion into CE report).

Reference

[1] Description of Test Model

[2] Common test conditions and software reference configurations

Appedix: Full participant list

| Company/ Organization | Participant Name | Email Address |
| --- | --- | --- |
| HHI | Martin Winken Jennifer Rasch Jonathan Pfaff Anastasia Henkel Ivan Zupancic | martin.winken@hhi.fraunhofer.de Jennifer.Rasch@hhi.fraunhofer.de jonathan.pfaff@hhi.fraunhofer.de anastasia.henkel@hhi-extern.fraunhofer.de ivan.zupancic@hhi.fraunhofer.de |
| MediaTek | Chih-Wei Hsu ChunChia Chen Man-Shu Chiang Yu-Wen Huang ChingYeh Chen Peter Chuang Shih-Ta Hsiang Zhi-Yi Lin | cw.hsu@mediatek.com chunchia.chen@mediatek.com Man-Shu.Chiang@mediatek.com yuwen.huang@mediatek.com chingyeh.chen@mediatek.com Peter.Chuang@mediatek.com Shih-ta.Hsiang@mediatek.com Zhi-Yi.Lin@mediatek.com |
| RWTH Aachen University | Max Blaeser Mathias Wien Johannes Sauer | blaeser@ient.rwth-aachen.de wien@ient.rwth-aachen.de sauer@ient.rwth-aachen.de |
| Tencent | Jing Ye Xiaozhong Xu Guichun Li Meng Xu Xiang Li | jingye@tencent.com xiaozhongxu@tencent.com guichunli@tencent.com mengxxu@tencent.com xlxiangli@tencent.com |
| ETRI | Sung-Chang Lim Jung Won Kang | sclim@etri.re.kr jungwon@etri.re.kr |
| SHARP | Zhu Weijia Choi, Byeongdoo Misra, Kiran Tomonori Hashimoto Zhou Tianyang Chujoh Takeshi Ikai Tomohiro Yukinobu Yasugi | zhuw@sharplabs.com choib@sharplabs.com misrak@sharplabs.com tomonori.hashimoto@sharp.co.jp zhou.tianyang@sharp.co.jp chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp yasugi.yukinobu@sharp.co.jp |
| Panasonic | Ru-Ling Liao Chong Soon Lim Li Jingya | ruling.liao@sg.panasonic.com chongsoon.lim@sg.panasonic.com jingya.li@sg.panasonic.com |
| HiSilicon | Zhengjianhua Yongbing Lin | zhengjianhua@hisilicon.com linyongbing@hisilicon.com |
| Broadcom | Minhua Zhou Wade Wan Peisong Chen | minhua.zhou@broadcom.com wade.wan@broadcom.com peisong.chen@broadcom.com |
| Qualcomm | Yu Han Chun-Chi Chen Nan Hu Wei-Jung Chien | yuhan@qti.qualcomm.com chunchic@qti.qualcomm.com nanh@qti.qualcomm.com wchien@qti.qualcomm.com |
| Samsung | Yinji Piao Kiho Choi Jie Chen Seungsoo Jeong Anish Tamse | yj1003.piao@samsung.com kiho14.choi@samsung.com jie1222.chen@samsung.com ss00.jeong@samsung.com tamse.anish@samsung.com |
| LGE | Seung Hwan Kim Ling Li Sunmi Yoo | seunghwan3.kim@lge.com l.li@lge.com sunmi.yoo@lge.com |
| Huawei | Xu Wei Wei Hai Tao Yang Jianle Chen Zhang Na Alexey Filippov Vasily Rufitskiy | xuweiwei3@huawei.com haitao.yang@huawei.com jianle.chen@huawei.com zhangna25@huawei.com alexey.filippov@huawei.com vasily.rufitskiy@huawei.com |
| Canon | Chris Rosewarne Christophe Gisquet | Chris.Rosewarne@cisra.canon.com.au christophe.gisquet@gmail.com |
| Yonsei University | Sang-hyo Park | spark@barunict.kr |
| Netflix | Andrey Norkin | anorkin@netflix.com |
| Peking University | Zhao Wang, | zhaowang@pku.edu.cn |
| BBC | Saverio Blasi | Saverio.Blasi@bbc.co.uk Andre.SeixasDias@bbc.co.uk |
| Alibaba | YuChen Sun | yuchen.s@alibaba-inc.com |
| Apple | Alexis Tourapis | atourapis@apple.com |
| PIXTREE | Jaeseob Shin | jsshin@pixtree.com |
| Sony | Masaru Ikeda | Masaru.Ikeda@sony.com |
| Technicolor | Leleannec Fabrice Antoine Robert Tangi Poirier Edouard Francois Angi Poirier | Fabrice.Leleannec@technicolor.com antoine.robert@technicolor.com tangi.poirier@technicolor.com Edouard.francois@technicolor.com angi.poirier@technicolor.com |
| Nvidia | Ankur | ankur81saxena@gmail.com |
| Arris | Panusopone, Krit Hong, Seungwook Wang, Limin | Krit.Panusopone@arris.com Seungwook.Hong@arris.com Limin.Wang@arris.com |
| ITRI | Po-Han Lin Ching-Chieh Lin | pohan@itri.com JackLin@itri.com |
| Hikvision | Xuguang Zuo | zuoxuguang@hikvision.com |
| Florida Atlantic University | Hari Kalva | hari.kalva@fau.edu |
| Ericsson | Jacob Ström Ruoyang Yu | jacob.strom@ericsson.com ruoyang.yu@ericsson.com |
| Digital Insights | Yongjo Ahn | yjahn@digitalinsights.co.kr |
| Chips&Media | Wook Jeong | nick.jeong@chipsnmedia.com |
|  |  | yaojen.chang.foxconn@gmail.com |
| Bytedance | Kai Zhang Hongbin Liu Li Zhang | zhangkai.video@bytedance.com liuhongbin.01@bytedance.com lizhang.idm@bytedance.com |
| Dolby Laboratories | Peng Yin | pyin@dolby.com |
| Ittiam Systems Pvt. Ltd | S. Sethuraman | sriram.sethuraman@ittiam.com |
| Kwai | Yi-Wen Chen Xianglin Wang | yiwenchen@kwai.com xianglinwang@kwai.com |
|  |  | i.jaehoon.kim@icloud.com |
| Sejong University |  | bigdj2002@sju.ac.kr hajama0123@sju.ac.kr |
| InterDigital | Xiu, Xiaoyu Yuwen He Daniel Luo | Xiaoyu.Xiu@InterDigital.com Yuwen.He@InterDigital.com Daniel.Luo@InterDigital.com |
|  | Limin Wang | cuiwang@gmail.com |
| HNU | Haechul Choi Heeji Han Daehyeok Gwon jaeryun Choe | choihc@hanbat.ac.kr sarhakor@naver.com skyeye0530@naver.com tjswo9395@naver.com |