| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K1031-v1 |
| --- | --- |

| Title: | Description of Core Experiment 11 (CE11): Deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JVET |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Andrey Norkin Anand Meher Kotra | Tel: Email: | anorkin@netflix.com anand.meher.kotra@huawei.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The goal of this Core Experiment (CE) is to improve and/or enhance deblocking filtering techniques in the test model (VTM) and Benchmark Set (BMS). Techniques covered by this CE are related long deblocking filters, filtering on the 4x4 boundaries, and other modifications to deblocking.

The deblocking filter techniques are evaluated according to subjective picture quality as well as BD-rate gains. These are organized in CE11.1, CE11.2 and CE11.3 which address aspects of the long deblocking filters, general deblocking filtering aspects and filtering at 4x4 block boundaries. Participants in this activity are Canon, Ericsson, Huawei, KDDI, MediaTek, NHK, Panasonic, Samsung, Qualcomm, SHARP, and Sony.

The software basis for this CE is BMS-2.0, for VTM based comparisons the BMS software is configured to produce VTM-2.0 [3]. For the test sequences, configurations and test conditions, in JVET-K1010 [1] for SDR and JVET-K1011 [2] for HDR (if required) are used. For subjective viewing, additional results with sequences encoded with higher QPs may be required.

# Participants

| Nr. | Name | Company | Email | Propo-nents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
| 1 | Alexis Tourapis | Apple | atourapis@apple.com |  |  |
| 2 | Patrice Onno | Canon | patrice.onno@crf.canon.fr |  |  |
| 3 | Jonathan Taquet | Canon | jonathan.taquet@crf.canon.fr |  |  |
| 4 | Christophe Gisquet | Canon | christophe.gisquet@crf.canon.fr |  |  |
| 5 | Dan Grois | Comcast | grois@ieee.org |  |  |
| 6 | Peng Yin | Dolby | pyin@dolby.com |  |  |
| 7 | Kenneth Andersson | Ericsson | kenneth.r.andersson@ericsson.com |  |  |
| 8 | Jungwon Kang | ETRI | jungwon@etri.re.kr |  |  |
| 9 | Hahyun Lee | ETRI | hanilee@etri.re.kr |  |  |
| 10 | Christian Helmrich | HHI | christian.helmrich@hhi.fraunhofer.de |  |  |
| 11 | Haechul Choi, | HNU | choihc@hanbat.ac.kr |  |  |
| 12 | Heeji Han, HNU | HNU | sarhakor@naver.com |  |  |
| 13 | Daehyeok Gwon | HNU | skyeye0530@naver.com |  |  |
| 14 | jaeryun Choe | HNU | tjswo9395@naver.com |  |  |
| 15 | Sergey Ikonin | Huawei | sergey.ikonin@huawei.com |  |  |
| 16 | Filippov Alexey | Huawei | alexey.filippov@huawei.com |  |  |
| 17 | Vasily Rufitskiy | Huawei | vasily.rufitskiy@huawei.com |  |  |
| 18 | Biao Wang | Huawei | biao.wang@huawei.com |  |  |
| 19 | Rahul Vanam | InterDigital | Rahul.vanam@interdigital.com |  |  |
| 20 | Kyohei Unno | KDDI | ky-unno@kddi-research.jp |  |  |
| 21 | Kei Kawamura | KDDI | ki-kawamura@kddi.com |  |  |
| 22 | Yi-Wen Chen | Kwai | yiwenchen@kwai.com |  |  |
| 23 | Xianglin Wang | Kwai | xianglinwang@kwai.com |  |  |
| 24 | Seung Hwan Kim | LG | seunghwan3.kim@lge.com |  |  |
| 25 | Junghak Nam | LG | junghak.nam@lge.com |  |  |
| 26 | Hyeongmun Jang | LG | hm.jang@lge.com |  |  |
| 27 | Chih-Wei Hsu | MediaTek | cw.hsu@mediatek.com |  |  |
| 28 | Chia-ming Tsai | MediaTek | Chia-ming.Tsai@mediatek.com |  |  |
| 29 | Ching-Yeh Chen | MediaTek | chingyeh.chen@mediatek.com |  |  |
| 30 | Tzu-Der Chuang | MediaTek | Peter.Chuang@mediatek.com |  |  |
| 31 | Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com |  |  |
| 32 | Atsuro Ichigaya | NHK | ichigaya.a-go@nhk.or.jp |  |  |
| 33 | S. Iwamura | NHK | iwamura.s-gc@nhk.or.jp |  |  |
| 34 | S. Nemoto | NHK | nemoto.s-fy@nhk.or.jp |  |  |
| 35 | Tadamasa Toma | Panasonic | toma.tadamasa@jp.panasonic.com |  |  |
| 36 | Dmytro Rusanovskyy | Qualcomm | dmytror@qti.qualcomm.com |  |  |
| 37 | Wei-Jung Chien | Qualcomm | wchien@qti.qualcomm.com |  |  |
| 38 | Kiho Choi | Samsung | kiho14.choi@samsung.com |  |  |
| 39 | Woongil Choi | Samsung | woongil.choi@samsung.com |  |  |
| 40 | Weijia Zhu | Sharp | zhuw@sharplabs.com |  |  |
| 41 | Andrew Segall | Sharp | asegall@sharplabs.com |  |  |
| 42 | Kiran Misra | Sharp | misrak@sharplabs.com |  |  |
| 43 | Takeshi Chujoh | Sharp | chujoh.takeshi@sharp.co.jp |  |  |
| 44 | Tomohiro Ikai | Sharp | ikai.tomohiro@sharp.co.jp |  |  |
| 45 | Dongjae Won | SJU | bigdj2002@sju.ac.kr |  |  |
| 46 |  | SJU | hajama0123@sju.ac.kr |  |  |
| 47 | Teruhiko Suzuki | Sony | teruhiko.s@sony.com |  |  |
| 48 | Masaru Ikeda | Sony | masaru.ikeda@sony.com |  |  |

# Test conditions and evaluation criteria

Two anchors which include directly using VTM with and without ALF will be used in this CE and a test on the BMS. Proposals will be compared with respect to subjective, objective quality, and complexity.

# Tool description

## Introduction

The main goal with this Core Experiment is to enhance the subjective quality of the Test Model and to ensure that CUs can be filtered in parallel.

Following aspects will be studied:

- Long deblocking filters and decisions

- General deblocking aspects

- Deblocking 4x4 block boundaries

Different technologies will be compared at a subjective viewing at the next meeting.

## Tests specification

| Test# | Description | Document# |
| --- | --- | --- |
| 11.1.1 | Long deblocking filters and fixes (only luma) | JVET-K0307 |
| 11.1.2 | Extended Deblocking Filter (only luma) | JVET-K0393 |
| 11.1.3 | Long deblocking filters (only luma) | JVET-K0232 |
| 11.1.4 | Tests on long deblocking (only long for luma also filtering chroma when long filters are used for luma) | JVET-K0334 |
| 11.1.5 | Long-tap deblocking filter (only luma) | JVET-K0112 |
| 11.1.6 | Long-tap deblocking filter for luma and chroma | JVET-K0152 |
| 11.1.7 | Deblocking Improvements for Large CUs both luma and chroma | JVET-K0315 |
| 11.1.8 | CE2-related: Longer Tap Deblocking Filter | JVET-K0369 |
| 11.1.9 | CE2-related: Long-tap luma filter + long tap chroma filter | JVET-K0318 |
| 11.2.1 | Non-CE2: Higher-precision modifications to VVC deblocking filters | JVET-K0203 |
| 11.2.2 | Luma-adaptive deblocking filter (QP offset change based on luma level) | JVET-K0386 |
| 11.3.1 | CU deblocking on 4x4 grid for luma | JVET-K0318 |
| 11.3.2 | CE2-2.1.1-related: Filtering of sub-block boundaries for luma | JVET-K0318 |
| 11.3.3 | CE2-related: Deblocking for 4 x N and N x 4 CUs | JVET-K0369 |
| 11.3.4 | Parallel deblocking for 4 x N and N x 4 block boundaries | JVET-J0018 |

### CE 11.1: long-tap deblocking filter

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE11.1.1 | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-K0307 | Masaru Ikeda Masaru.Ikeda@sony.com; Jie Dong jiedong@qti.qualcomm.com |
| CE11.1.2 | Kei Kawamura ki-kawamura@kddi.com JVET-K0393 | Kenneth Andersson kenneth.r.andersson@ericsson.com |
| CE11.1.3 | Chia-Ming Tsai chia-ming.tsai@mediatek.com JVET-K0232 | Kenneth Andersson kenneth.r.andersson@ericsson.com |
| CE11.1.4 | Dmytro Rusanovskyy dmytror@qti.qualcomm.com JVET-K0334 | Kenneth Andersson kenneth.r.andersson@ericsson.com |
| CE11.1.5 | Woong IL Choi woongil.choi@samsung.com JVET-K0112 | Kei Kawamura ki-kawamura@kddi.com Kenneth Andersson kenneth.r.andersson@ericsson.com |
| CE11.1.6 | Masaru Ikeda masaru.ikeda@sony.com JVET-K0152 | Kenneth Andersson kenneth.r.andersson@ericsson.com |
| CE11.1.7 | Kiran Misra misrak@sharplabs.com JVET-K0315 | Kenneth Andersson kenneth.r.andersson@ericsson.com Anand Meher Kotra anand.meher.kotra@huawei.com |
| CE11.1.8 | Anand Meher Kotra Anand.meher.kotra@huawei.com JVET-K0369 | Patrice Onno patrice.onno@crf.canon.fr |
| CE11.1.9 | Kenneth Andersson kenneth.r.andersson@ericsson.com | TBD |

#### CE11.1.1 JVET-K0307

- Technology to be investigated

HEVC deblocking filters are extended with longer deblocking filters for luma. One of 7, 5 and 3 sample filtering on respective side of the block boundary can be applied for a 16-sample segment of the block boundary if current and neighbouring block size >= 32, >=16, >=8 respectively and corresponding conditions for 8 lines of the boundary are fulfilled as shown below.

| Size | Conditions for each of the 8 lines of a 16 sample segment (line 0, 3, 4, 7, 8, 11, 12, 15) |
| --- | --- |
| 7 | HEVCstrong_weak&& p0-2*p2+p4+q0-2*q2+q4<3*β32 && p0-p2-p3+p5+q0-q2-q3+q5<3*β32&& p0-2*p3+p6+q0-2*q3+q6<3*β32 && p0-p3-p4+p7+q0-q3-q4+q7<3*β32 |
| 5 | HEVCstrong_weak&& p0-p4+q0-q4<β8 && p0-p5+q0-q5<β8 |
| 3 | HEVCstrong_weak&& p0-p1-p2+p3+q0-q1-q2+q3<3*β32 |
| 0-3 | HEVC filter and decisions apply but reducing maximum filter length to half the minimum distance between current and neighboring CU boundaries |

Block boundary samples pi and qi for i=0 to S-1 are then replaced by linear interpolation as follows:

- pi´=(fi*Middles+64-fi*Ps)+32)≫6, clipped to pi±tc

- qi´=(fi*Middles+64-fi*Qs+32)≫6), clipped to qi±tc

where f, Middle, P and Q are given below:

| S | Filter kernels |
| --- | --- |
| 7 | fi=59-i*9, can also be described as f=59,50,41,32,23,14,5 Middle7=(2*po+qo+p1+q1+p2+q2+p3+q3+p4+q4+p5+q5+p6+q6+8)≫4 P7=(p6+p7+1)≫1, Q7=(q6+q7+1)≫1 |
| 5 | fi=58-i*13, can also be described as f=58,45,32,19,6 Middle5=(2*po+qo+p1+q1+p2+q2+p3+q3+p4+q4+8)≫4 P5=(p4+p5+1)≫1, Q5=(q4+q5+1)≫1 |
| 3 | fi=53-i*21, can also be described as f=53,32,11 Middle3=(3*po+qo+p1+q1+2*(p2+q2)+8)≫4 P3=(p2+p3+1)≫1, Q3=(q2+q3+1)≫1 |

- Questions to be answered

Is it sufficient to filter 7 pixels on both sides of a large block boundary? Expects improvements in subjective quality from reducing discontinuities across large blocks, especially for low bitrates.

A: Visual tests are expected to be done during the meeting

#### CE11.1.2 JVET-K0393

- Technology to be investigated

HEVC deblocking filters are extended with longer deblocking filters for luma.

The proposed method consisted of two parts; edge boundary detection of large intra block and extension of the filtered samples on edge filtering process.

When the following conditions are true, the proposed decision process is continued:

- Edge boundary is located between two blocks of which cross edge sizes are 16 or longer than 16.

- The filter type is strong.

- Both blocks are not intra coded

It is noted that a block size is replaced by an edge size. Direction of edge size depends on that of edge boundary like EDGE_VER or EDGE_HOR.

When the above criteria is not true, the default short deblocking filter may be applied.

The longer filter is applied on both sides if above conditions are true and gradient checks for samples p0 to p7 and q0 to q7 of smoothness are passed. Following filter kernels are used.

p6 = 3,5,2,1,1,1,1,1,1,0,0,0,0,0,0,0
p5 = 1,2,5,2,1,1,1,1,1,1,0,0,0,0,0,0
p4 = 0,1,3,3,3,1,1,1,1,1,1,0,0,0,0,0
p3 = 0,1,2,2,2,2,2,1,1,1,1,1,0,0,0,0
p2 = 0,0,1,2,2,2,2,2,2,1,1,1,0,0,0,0
p1 = 0,0,1,1,2,2,2,2,2,2,1,1,0,0,0,0
p0 = 0,0,0,1,2,2,2,2,2,2,2,1,0,0,0,0

There are no changes for chroma deblocking filter process.

#### CE11.1.3 JVET-K0232

- Technology to be investigated

HEVC deblocking filters are extended with longer strong filters for luma. Three sets of long strong filter sets are added in addition to the HEVC strong filter set. The proposed long strong filtering is applied when the both side length condition, inter-side and intra side difference conditions which represents the smoothness of both sides, and long strong filtering decision of adjacent block edges condition are satisfied. The proposed long strong filter sets are applied according to the selected filter set and the length of each side. 7 samples or 4 samples on respective side of the boundary may be filtered. In addition, the strong filtering for chroma on large block boundary, and parallel deblocking solutions for 4 x N and N x 4 block boundaries proposed in JVET-J0018 are also to be studied.

#### CE11.1.4 JVET-K0334

- Technology to be investigated

HEVC deblocking filters are extended with longer deblocking filters for luma. A longer filter that filters 4 samples on each side of the boundary when block size on both sizes are >= 16, HEVC strong/weak filter is indicating strong filter and sum of absolute value of Laplacian for every second line for the whole block boundary and compare that with a threshold. The threshold is specific for intra and non intra slices and also depends on a PPS flag indicate smooth threshold flag. This decision process was further extended in JVET-J0056 with multi-factor selection of deblocking filter.

Following filter kernels are used:

h0=0, 2, 9, 12, 1, 3, 13, 16, 5, 3, 0,

h1=-1, 6, 7, 8, -2, 13, 15, 10, 8, 0, 0,

h2=-1, 4, 12, -3, 6, 14, 14, 16, 2, 0, 0,

h3=-3, 16, -3, 1, 5, 16, 18, 14, 0, 0, 0

This above filter kernels are further extended in JVET-J0056, e.g. Table 1 presents 15 pre-selected filters which are adaptively selected.

Clipping is used to limit the amount of filtering. A boundary strength of 3 is used to determine tc for both luma and chroma. Increase tc offset if QP > 42 and decrease if QP<= 35, used for both luma and chroma. Tc is then scaled by 3 for first two samples near the boundary and scaled by 2 for the other samples.

Long filter can be turned on for respective 4 sample segment. Deblocking applied for 4x4 grid for luma.

#### CE11.1.5 JVET-K0112

- Technology to be investigated

HEVC deblocking filters are extended with longer deblocking filters for luma. 7 samples on a side of the boundary is filtered when length orthogonal to block boundary is >= 16, strong/weak filter determines use strong filter, sum of absolute pixel difference for pixel p8 and p0 and q8 and q0 compared to a threshold. Only 3 samples are filtered for the upper block when filtering horizontal boundaries. Decision on 4 sample segments of an 8x8 grid.

Following filters are used for left block in vertical filtering:

p6	{3, 5, 2, 1, 1, 1, 1, 1, 1}	      p7 ~ q0

p5	{1, 2, 5, 2, 1, 1, 1, 1, 1, 1}	   p7 ~ q1

p4	{1, 3, 3, 3, 1, 1, 1, 1, 1, 1}	   p6 ~ q2

p3	{1, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1}	p6 ~ q3

p2	{1, 2, 2, 2, 2, 2, 2, 1, 1, 1}	   p5 ~ q3

p1	{1, 1, 2, 2, 2, 2, 2, 2, 1, 1}	   p5 ~ q3

p0	{1, 2, 2, 2, 2, 2, 2, 2, 1}	      p4 ~ q3

For the upper block in horizontal filtering only p0 to p2.

p2    {3, 5, 2, 2, 1, 1, 1, 1}  p3 ~ q3

p1    {2, 2, 4, 2, 2, 2, 1, 1}  p3 ~ q3

P0    {2, 2, 2, 2, 2, 2, 2, 2}  p3 ~ q3

#### CE11.1.6 JVET-K0152

- Technology to be investigated

HEVC deblocking filters are extended with longer deblocking filters for luma and chroma. 7 samples on each side of the boundary is filtered when the minimal length orthogonal to block boundary is >= 32, sum of absolute pixel difference and also Laplacians, and comparison with absolute value of pixel difference across boundary in comparison with tc. Decision is taken on two lines of 4 sample segments on a 4x4 grid.

Following filtering apply to P side and similar for Q side:

p6′= (7*p7+2*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (6*p7+p6+2*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (5*p7+p6+p5+2*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (4*p7+p6+p5+p4+2*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (3*p7+p6+p5+p4+p3+2*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (2*p7+p6+p5+p4+p3+p2+2*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

The decision which has two additional conditions is also modified, accordingly. The first one is for large block decision since the proposed filter can be applied when the block width or height which orthogonally crosses the block edge is equal to or larger than 32. The second one is for wide strong filter decision. It is extended based on strong filter decision.

Also propose a longer filter for chroma for filtering of p0 to p2 and similarly for q0 to q2 as follows:

p2′= (3*p3+2*p2+p1+p0+q0+4) >> 3

p1′= (2*p3+p2+2*p1+p0+q0+q1+4) >> 3

p0′= (p3+p2+p1+2*p0+q0+q1+q2+4) >> 3

The decision which has three additional conditions is also modified, accordingly. The first one is for decision of boundary strength as well as large block. The proposed filter can be applied when the block width or height which orthogonally crosses the block edge is larger than 16. The second and third one are basically similar to luma’s decision, which are on/off decision and strong filter decision, respectively.

- Questions to be answered

  - Subjective and objective impact of modified luma filter

  - Subjective and objective impact of modified chroma filter

  - Subjective and objective impact of the whole proposal

A: Visual tests are expected to be done to evaluate the subjective quality during the meeting.

#### CE11.1.7 JVET-K0315

[1]	Technology to be investigated

JVET-J0077 incorporates stronger filters for both luma and chroma, compared to JEM deblocking. Additionally, the proposed modification includes a control process that considers the block sizes on both sides of the boundary being deblocked. The stronger filters are used for luma samples that correspond to larger block sizes, while the JEM deblocking filters are still used for luma samples corresponding to the smaller block sizes.  For chroma, the filter selection uses a different approach, and a stronger filter is applied when chroma samples on either sides of the deblocking boundary belong to a large block.

Luma stronger filters (same as JVET-J0038):

p6′= (7*p7+2*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (6*p7+p6+2*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (5*p7+p6+p5+2*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (4*p7+p6+p5+p4+2*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (3*p7+p6+p5+p4+p3+2*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (2*p7+p6+p5+p4+p3+p2+2*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

q6′= (7*q7+2*q6+q5+q4+q3+q2+ q1+q0+p0+ 8) >> 4

q5′= (6*q7+q6+2*q5+q4+q3+q2+q1+q0+p0+p1+ 8) >> 4

q4′= (5*q7+q6+q5+2*q4+q3+q2+q1+q0+p0+p1+p2+ 8) >> 4

q3′= (4*q7+q6+q5+q4+2*q3+q2+q1+q0+p0+p1+p2+p3+ 8) >> 4

q2′= (3*q7+q6+q5+q4+q3+2*q2+q1+q0+p0+p1+p2+p3+p4+8) >> 4

q1′= (2*q7+q6+q5+q4+q3+q2+2*q1+q0+p0+p1+p2+p3+p4+p5+8) >> 4

q0′= (q7+q6+q5+q4+q3+q2+q1+2*q0+p0+p1+p2+p3+p4+p5+p6+8) >> 4

Luma stronger filters are used for large blocks where large block corresponds to width >= 32 for a vertical edge, and height>=32 for a horizontal edge. The control process is further based on gradients computed for two lines of 4 sample segments; comparison of absolute pixel value difference with tc; and comparison of other absolute pixel value differences with β. More gradients are computed for large block side.

Chroma stronger filters:

p0′ = ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 )  >>  3

p1′ = ( p2 + p1 + p0 + q0 + 2 )  >>  2

p2′ = ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 )  >>  3

q0′ = ( p1 + 2 * p0 + 2 * q0 + 2 * q1 + q2 + 4 )  >>  3

q1′ = ( p0 + q0 + q1 + q2 + 2 )  >>  2

q2′= ( p0 + q0 + q1 + 3 * q2 + 2 * q3 + 4 )  >>  3

Chroma stronger filters are used on both sides of the block boundary.  Specifically, the strong filter is used when any of the following conditions is true:

- The edge type is vertical an p0 belongs to CU with width >= 32

- The edge type is horizontal and p0 belongs to CU with height >= 32

- The edge type is vertical and q0 belongs to CU with width >= 32

- The edge type is horizontal and q0 belongs to CU with height >= 32

[2] Questions to be answered

- Subjective impact on quality for CfP sequences, CTC sequences, and sequences with real strong edges (e.g. screen content) at low bit rates

- Objective impact of the changes for the same set of sequences content under CTC.

A: Visual tests are expected to be done to evaluate the subjective quality during the meeting.

#### CE11.1.8 JVET-K0369

Two technologies are to be investigated

- New luma longer tap filters for large blocks are proposed.

- Deblocking for 4 x 4 CUs

New luma longer tap filters:

Filter condition:                       HEVCstrongweak

&&

p0-p3- p4+p7+q0-q3- q4+q7<3*β32 &&

p0-p2-p3+p5+q0-q2-q3+q5<3*β32

Proposed filter coefficients for longer tap filter:

p6′= (6*p7+3*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (5*p7+p6+3*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (4*p7+p6+p5+3*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (3*p7+p6+p5+p4+3*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (2*p7+p6+p5+p4+p3+3*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (p7+p6+p5+p4+p3+p2+3*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

q6′= (6*q7+3*q6+q5+q4+q3+q2+ q1+q0+p0+ 8) >> 4

q5′= (5*q7+q6+3*q5+q4+q3+q2+q1+q0+p0+p1+ 8) >> 4

q4′= (4*q7+q6+q5+3*q4+q3+q2+q1+q0+p0+p1+p2+ 8) >> 4

q3′= (3*q7+q6+q5+q4+3*q3+q2+q1+q0+p0+p1+p2+p3+ 8) >> 4

q2′= (2*q7+q6+q5+q4+q3+3*q2+q1+q0+p0+p1+p2+p3+p4+8) >> 4

q1′= (q7+q6+q5+q4+q3+q2+3*q1+q0+p0+p1+p2+p3+p4+p5+8) >> 4

q0′= (q7+q6+q5+q4+q3+q2+q1+2*q0+p0+p1+p2+p3+p4+p5+p6+8) >> 4

Proposed longer tap filter condition for horizontal boundaries overlapping with CTU boundaries:

HEVCstrongweak &&

p3-p2-p1+p0+q0-q3- q4+q7<3*β32 &&

p3-p2-p1+p0+q0-q2-q3+q5<3*β32

Proposed longer tap filter coefficients for horizontal boundaries overlapping with CTU boundaries:

p2′= (6*p3+3*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (5*p3+p2+3*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (5*p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

q6′= (6*q7+3*q6+q5+q4+q3+q2+ q1+q0+p0+ 8) >> 4

q5′= (5*q7+q6+3*q5+q4+q3+q2+q1+q0+p0+p1+ 8) >> 4

q4′= (4*q7+q6+q5+3*q4+q3+q2+q1+q0+p0+p1+p2+ 8) >> 4

q3′= (3*q7+q6+q5+q4+3*q3+q2+q1+q0+p0+p1+p2+p3+ 8) >> 4

q2′= (2*q7+q6+q5+q4+q3+3*q2+q1+q0+p0+p1+p2+2*p3+8) >> 4

q1′= (q7+q6+q5+q4+q3+q2+3*q1+q0+p0+p1+p2+3*p3+8) >> 4

q0′= (q7+q6+q5+q4+q3+q2+q1+2*q0+p0+p1+p2+4*p3+8) >> 4

- Questions to be answered

  - Is the “Longer tap” reducing the blocking artifacts at large block boundaries?

  - Is the Longer tap variant proposed for “line buffer” reduction, good enough to reduce blocking artifacts appearing on horizontal block boundaries overlapping with CTU boundaries?

#### CE11.1.9 JVET-K0318

[1] Technology to be investigated:

On top of long-tap luma filter in test CE11.1.1:

Use HEVC luma filter and decisions with following modifications.

Determine the number of samples to modify in block P and Q as follows:

lengthP = edge==EDGE_HOR ? (CUP_height_chroma >> 2) : (CUP_width_chroma >>2)

lengthQ = edge==EDGE_HOR ? (CUQ_height_chroma >> 2) : (CUQ_width_chroma >>2)

Only filter boundary if both lengthQ and lengthP is larger than 0.

The HEVC strong filter (reading 4 samples and modifying 3 samples on respective side) is only applied if both sides have a maximum deblocking filter length larger than 2.

Strong/weak filter and number of samples to filter is determined for each line. Inter blocks are only filtered if Laplacian gradients near the boundary on respective side is below the structure threshold “beta” but for intra blocks that check is by passed.

“beta” parameter is divided by 8 compared to what is used for luma.

Question to be answered: Is there a benefit from deblocking with long chroma deblocking filterlong in addition to long luma filter?

### CE 11.2: General aspects of deblocking filter

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE11.2.1 | Christophe Gisquet christophe.gisquet@crf.canon.fr JVET-K0203 | TBD |
| CE11.2.2 | Atsuro Ichigaya ichigaya.a-go@nhk.or.jp JVET-K0386 | Anand Meher Kotra anand.meher.kotra@huawei.com |

#### CE11.2.1: Higher-precision modifications to VVC deblocking filters JVET-K0203

- Technology to be investigated [TBD: should the proposed sub-tests be separate tests?]

- chroma deblocking decision from luma. Chroma filtering ON/OFF decision switches one the same pixel granularity as luma one does.

- tc scaling test. Possible sub-tests:

- Further scaling on clipping range (but keeping the same formulas)

- Combining with TcOffset

- Comparing HEVC bitdepth-scaling to proposal’s, e.g.: encode 8 bits content using a tc scale divided by 4 compared to the 10 bits, and use a more appropriate one

- Combination of most meaningful Tests b.1 & b.2

- Additional chroma samples being filtered with one of the above

#### CE11.2.2 Luma-adaptive deblocking filter JVET-K0386

- Technology to be investigated

The variable LL of the reconstructed luma level is derived as follow:

LL= ( ( p0,0 + p0,3 + q0,0 + q0,3 ) >> 2 ) / ( 1 << iBitDepth ),               (1)

where, the sample values pi,k and qi,k with i = 0..3 and k = 0 and 3 are derived the same as HEVC as shown in Figure 2.

Figure 2: Sample position of pi,k and qi,k

The variables QpQ and QpP are set equal to the QpY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

The variable qPL is derived as follows:

qPL = ( ( QPQ + QPP +1 ) >> 1 ) + qpOffset,                    (2)

where qpOffset is specified in Table 1.

The subsequent derivation processes of the variables β and tC are same as HEVC.

Table 1. Specification of qpOffset as a function of LL

| transfer_characteristics == 16 (PQ format) |  | transfer_characteristics == 18 (HLG format) |  | Otherwise (conventional SDR format) |  |
| --- | --- | --- | --- | --- | --- |
| Conditions | qpOffset | Conditions | qpOffset | Conditions | qpOffset |
| LL <= 0.84 | 0 | LL <= 0.8 | 0 | LL <= 0.875 | 0 |
| 0.84 < LL <= 0.91 | 1 | 0.8 < LL <= 0.9 | 1 | 0.875 < LL | 1 |
| 0.91 < LL <= 0.99 | 2 | 0.9 < LL | 2 |  |  |
| 0.99 < LL | 3 |  |  |  |  |

### CE 11.3: Deblocking at 4x4 block boundaries

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE11.3.1 | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-K0318 | TBD |
| CE11.3.2 | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-K0318 | TBD |
| CE11.3.3 | Anand Meher Kotra Anand.meher.kotra@huawei.com JVET-K0369 | TBD |
| CE11.3.4 | Chia-Ming Tsai chia-ming.tsai@mediatek.com JVET-J0018 | TBD |

#### CE11.3.1 CU deblocking on 4x4 grid for luma JVET-K0318

Technology to be investigated:

Determine the number of samples to modify in block P and Q as follows:

lengthP = edge==EDGE_HOR ? (CUP_height >> 1) – 1 : (CUP_width>>1) - 1

lengthQ = edge==EDGE_HOR ? (CUQ_height >> 1) – 1 : (CUQ_width>>1) - 1

This means that only one sample will be filtered when the distance between boundaries are 4.

The weak filter can be used with two sample filtering on at least one side if lengthP or lengthQ is larger than 1. The strong filter can be used if both lengthP and lengthQ are larger than 2.

Question to be answered: Is there a subjective benefit of deblocking of 4x4 grid compared to 8x8 grid?

#### CE11.3.2 Filtering of sub-block boundaries for luma JVET-K0318

Technology to be investigated:

Deblock on sub-block boundaries on 8x8 grid for CUs that use ATMVP.

Question to be answered: Is there a subjective benefit of deblocking of sub-block boundaries from ATMVP?

#### CE11.3.3 CE2-related: Deblocking for 4 x N and N x 4 CUs JVET-K0369

Two solutions are proposed to be studied here

- Solution 1: Always enforce HEVC normal filter with maximum one sample modification when HEVC filter is turned on for the respective edge

- Solution 2: When block width/height is equal to 4 for vertical edge/horizontal edge respectively, then a maximum of three samples are used in filter decision and only one sample is modified by the filter.

Question to be answered:

Is deblocking necessary for edges belonging to 4 x N and N x 4 CU’s (or) is an 8 x 8 grid filtering good enough.

#### CE11.3.4 Parallel deblocking for 4 x N and N x 4 block boundaries JVET-J0018

Technology to be investigated:

In JVET-J0018, to support parallel deblocking for 4xN and Nx4 blocks, p1i and q1i are used to replace the pixels outside p1i and q1i when doing filtering decision and operations. Within this CE, the performance of the parallel deblocking method described in JVET-J0018 is to be studied in terms of subjective quality and computational complexity.

Question to be answered:

## Expected Gains

- Subjective picture quality improvement while preferably keeping rate-distortion performance.

- Coding efficiency improvement.

## Evaluation Criteria

1. Measure impact on Bitrate/PSNR relative to the anchor(s).

2. Visual impact on output pictures of representative sequences (including SDR, HDR test sequences; such as FoodMarket, Campfire, Park Running, Kimono for RA and Class E sequences for LDB) encoded at QP equal to 37 and 42. The subjective test is expected to be held to determine the visual impact.

TBD: Any additional sequences to be added?

3. Complexity (e.g., decoding time measure, line buffer, filter taps, parallelization).

## Subjective test

Variant a: Subjective test is held in Macao. The proponents should provide the encoded/decoded sequences to the test chair/organizer.

Variant b: Subjective test is conducted by the test chair before the meeting. In this case the test plan needs to be developed.

# Interdependency with other CEs

There may be interaction with CE2 on Adaptive Loop Filter.

# Test conditions and data to be provided

The test conditions are according to CTC on VTM with ALF turned on and VTM with ALF turned off. [The results based on the BMS are needed as well]. In addition, the proponents need to provide the results for QP42 and possibly for the additional test sequences chosen by the CE.

# Time-line and Responsibilities

2018-August - XX:	Final CE description uploaded

2018-August - XX:	Cross-checking begins, proponents provide software

2018-Sept - XX: Final version of CE software, full test results must be provided at this time.

# References

- “JVET common test conditions and software reference configurations”, JVET-K1010.

- “JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-K1011.

- “Algorithm description for Versatile Video Coding and Test Model 2 (VTM2)”, JVET-K1002.