| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K1031-v2 |
| --- | --- |

| Title: | Description of Core Experiment 11 (CE11): Deblocking |  |  |
| --- | --- | --- | --- |
| Status: | Output Document to JVET |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Andrey Norkin Anand Meher Kotra | Tel: Email: | anorkin@netflix.com anand.meher.kotra@huawei.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The goal of this Core Experiment (CE) is to improve and/or enhance deblocking filtering techniques in the test model (VTM). Techniques covered by this CE are related long deblocking filters, filtering on the 4x4 boundaries, and other modifications to deblocking.

The deblocking filter techniques are evaluated according to subjective picture quality as well as BD-rate gains. These are organized in CE11.1, CE11.2 and CE11.3 which address aspects of the long deblocking filters, general deblocking filtering aspects and filtering at 4x4 block boundaries. Participants in this activity are Canon, Ericsson, Huawei, KDDI, MediaTek, NHK, Panasonic, Samsung, Qualcomm, SHARP, and Sony.

The software basis for this CE is VTM-2.0 [2]. For the test sequences, configurations and test conditions, in JVET-K1010 [1] for SDR sequences are used. For subjective viewing, additional results with sequences encoded with higher QPs may be required.

# Participants

| Nr. | Name | Company | Email |
| --- | --- | --- | --- |
| 1 | Alexis Tourapis | Apple | atourapis@apple.com |
| 2 | Patrice Onno | Canon | patrice.onno@crf.canon.fr |
| 3 | Jonathan Taquet | Canon | jonathan.taquet@crf.canon.fr |
| 4 | Christophe Gisquet | Canon | christophe.gisquet@crf.canon.fr |
| 5 | Dan Grois | Comcast | grois@ieee.org |
| 6 | Yongjo Ahn | Digital Insights | yjahn@digitalinsights.co.kr |
| 7 | Peng Yin | Dolby | pyin@dolby.com |
| 8 | Taoran Lu | Dolby | tlu@dolby.com |
| 9 | Kenneth Andersson | Ericsson | kenneth.r.andersson@ericsson.com |
| 10 | Jungwon Kang | ETRI | jungwon@etri.re.kr |
| 11 | Hahyun Lee | ETRI | hanilee@etri.re.kr |
| 12 | Christian Helmrich | HHI | christian.helmrich@hhi.fraunhofer.de |
| 13 | Haechul Choi | HNU | choihc@hanbat.ac.kr |
| 14 | Heeji Han | HNU | sarhakor@naver.com |
| 15 | Daehyeok Gwon | HNU | skyeye0530@naver.com |
| 16 | Jaeryun Choe | HNU | tjswo9395@naver.com |
| 17 | Sergey Ikonin | Huawei | sergey.ikonin@huawei.com |
| 18 | Filippov Alexey | Huawei | alexey.filippov@huawei.com |
| 19 | Vasily Rufitskiy | Huawei | vasily.rufitskiy@huawei.com |
| 20 | Biao Wang | Huawei | biao.wang@huawei.com |
| 21 | Jianle Chen | Huawei | Jianle.chen@huawei.com |
| 22 | Rahul Vanam | InterDigital | Rahul.vanam@interdigital.com |
| 23 | Kyohei Unno | KDDI | ky-unno@kddi-research.jp |
| 24 | Kei Kawamura | KDDI | ki-kawamura@kddi.com |
| 25 | Yi-Wen Chen | Kwai | yiwenchen@kwai.com |
| 26 | Xianglin Wang | Kwai | xianglinwang@kwai.com |
| 27 | Seung Hwan Kim | LG | seunghwan3.kim@lge.com |
| 28 | Junghak Nam | LG | junghak.nam@lge.com |
| 29 | Hyeongmun Jang | LG | hm.jang@lge.com |
| 30 | Jie Zhao | LG | jie.zhao@lge.com |
| 31 | Chih-Wei Hsu | MediaTek | cw.hsu@mediatek.com |
| 32 | Chia-ming Tsai | MediaTek | Chia-ming.Tsai@mediatek.com |
| 33 | Ching-Yeh Chen | MediaTek | chingyeh.chen@mediatek.com |
| 34 | Tzu-Der Chuang | MediaTek | Peter.Chuang@mediatek.com |
| 35 | Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com |
| 36 | Atsuro Ichigaya | NHK | ichigaya.a-go@nhk.or.jp |
| 37 | S. Iwamura | NHK | iwamura.s-gc@nhk.or.jp |
| 38 | S. Nemoto | NHK | nemoto.s-fy@nhk.or.jp |
| 39 | Tadamasa Toma | Panasonic | toma.tadamasa@jp.panasonic.com |
| 40 | Dmytro Rusanovskyy | Qualcomm | dmytror@qti.qualcomm.com |
| 41 | Wei-Jung Chien | Qualcomm | wchien@qti.qualcomm.com |
| 42 | Kiho Choi | Samsung | kiho14.choi@samsung.com |
| 43 | Woongil Choi | Samsung | woongil.choi@samsung.com |
| 44 | Weijia Zhu | Sharp | zhuw@sharplabs.com |
| 45 | Andrew Segall | Sharp | asegall@sharplabs.com |
| 46 | Kiran Misra | Sharp | misrak@sharplabs.com |
| 47 | Takeshi Chujoh | Sharp | chujoh.takeshi@sharp.co.jp |
| 48 | Tomohiro Ikai | Sharp | ikai.tomohiro@sharp.co.jp |
| 49 | Dongjae Won | SJU | bigdj2002@sju.ac.kr |
| 50 | ? | SJU | hajama0123@sju.ac.kr |
| 51 | Masaru Ikeda | Sony | masaru.ikeda@sony.com |
| 52 | Teruhiko Suzuki | Sony | teruhiko.s@sony.com |

# Test conditions and evaluation criteria

The proposals should be encoded according to two test conditions, which correspond to two anchors.

Anchor-1 is VTM-2 according to the CTC [1].

Anchor-2 is VTM-2 with ALF switched off (other conditions are the same as in Anchor 2).

The proposals in the CE shall be implemented based on VTM-2. The results for two test conditions shall be provided: Test Conditions 1 use configuration of Anchor 1, Test Conditions 2 use configuration of Anchor 2. The results for LDP configuration are mandatory for this CE.

Proposals will be compared with respect to subjective, objective quality, and complexity.

# Tool description

## Introduction

The main goal with this Core Experiment is to enhance the subjective quality of the Test Model and to ensure that CUs can be filtered in parallel.

Following aspects will be studied:

- Long deblocking filters and decisions

- General deblocking aspects

- Deblocking 4x4 block boundaries

Different technologies will be compared at a subjective viewing at the next meeting.

## Tests specification

| Test# | Description | Document# |
| --- | --- | --- |
| 11.1.1 | Long deblocking filters and fixes (only luma) | JVET-K0307 |
| 11.1.2 | Extended Deblocking Filter (only luma) | JVET-K0393 |
| 11.1.3 | Long deblocking filters (only luma) | JVET-K0232 |
| 11.1.4 | - Tests on long deblocking (only luma ) | JVET-K0334 |
| 11.1.5 | Long-tap deblocking filter (only luma) | JVET-K0112 |
| 11.1.6 | Long-tap deblocking filter for luma and chroma | JVET-K0152 |
| 11.1.7 | Deblocking Improvements for Large CUs both luma and chroma | JVET-K0315 |
| 11.1.8 | CE2-related: Longer Tap Deblocking Filter | JVET-K0369 |
| 11.1.9 | CE2-related: Long-tap luma filter + long tap chroma filter | JVET-K0318 |
| 11.1.10 | Combination of CE11.1.6 and CE11.1.7 | JVET-K0152 JVET-K0315 |
| 11.1.11 | Combination of CE11.1.1 and CE11.1.7 | JVET-K0307 JVET-K0315 |
| 11.2.1 | Non-CE2: Higher-precision modifications to VVC deblocking filters | JVET-K0203 |
| 11.2.2 | Luma-adaptive deblocking filter (QP offset change based on luma level) | JVET-K0386 |
| 11.3.1 | CU deblocking on 4x4 grid for luma | JVET-K0318 |
| 11.3.2 | CE2-2.1.1-related: Filtering of sub-block boundaries for luma | JVET-K0318 |
| 11.3.3 | CE2-related: Deblocking for 4 x N and N x 4 CUs | JVET-K0369 |
| 11.3.4 | Parallel deblocking for 4 x N and N x 4 block boundaries | JVET-J0018 |
| 11.3.5 | Parallel deblocking filter | JVET-D0044 |

### CE 11.1: long-tap deblocking filter

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE11.1.1 | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-K0307 | Masaru Ikeda Masaru.Ikeda@sony.com; - D.Rusanovskyy dmytror@qti.qualcomm.com |
| CE11.1.2 | Kei Kawamura ki-kawamura@kddi.com JVET-K0393 | Woong Il Choi woongil.choi@samsung.com |
| CE11.1.3 | Chia-Ming Tsai chia-ming.tsai@mediatek.com JVET-K0232 | Hyeongmun Jang hm.jang@lge.com |
| CE11.1.4 | Dmytro Rusanovskyy dmytror@qti.qualcomm.com JVET-K0334 | Kenneth Andersson kenneth.r.andersson@ericsson.com |
| CE11.1.5 | Woong IL Choi woongil.choi@samsung.com JVET-K0112 | Kei Kawamura ki-kawamura@kddi.com |
| CE11.1.6 | Masaru Ikeda masaru.ikeda@sony.com JVET-K0152 | Kenneth Andersson kenneth.r.andersson@ericsson.com |
| CE11.1.7 | Kiran Misra misrak@sharplabs.com JVET-K0315 | Masaru Ikeda Masaru.Ikeda@sony.com Taoran Lu tlu@dolby.com |
| CE11.1.8 | Anand Meher Kotra Anand.meher.kotra@huawei.com JVET-K0369 | Patrice Onno patrice.onno@crf.canon.fr Woong Il Choi woongil.choi@samsung.com |
| CE11.1.9 | Kenneth Andersson kenneth.r.andersson@ericsson.com - JVET-K0318 | Christian Helmrich christian.helmrich@hhi.fraunhofer.de |
| CE11.1.10 | Masaru Ikeda masaru.ikeda@sony.com JVET-K0152 Kiran Misra misrak@sharplabs.com JVET-K0315 | Anand Meher Kotra anand.meher.kotra@huawei.com |
| CE11.1.11 | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-K0307 Kiran Misra misrak@sharplabs.com JVET-K0315 | Jie Zhao jie.zhao@lge.com |

#### CE11.1.1 JVET-K0307

- Technology to be investigated

On top of deblocking on 4x4 grid and deblocking of sub-block boundaries in CE11.3.2. HEVC deblocking filters are extended with longer deblocking filters for luma. One of 7 and 5 sample filtering on respective side of the block boundary can be applied for a 16-sample segment of the block boundary if current and neighbouring block size >= 32 and >=16 respectively and corresponding conditions for 8 lines of the boundary are fulfilled as shown below.

| Size | Conditions for each of the 8 lines of a 16 sample segment (line 0, 3, 4, 7, 8, 11, 12, 15) |
| --- | --- |
| 7 | HEVCstrong_weak&& p0-2*p2+p4+q0-2*q2+q4<3*β32 && p0-p2-p3+p5+q0-q2-q3+q5<3*β32&& p0-2*p3+p6+q0-2*q3+q6<3*β32 && p0-p3-p4+p7+q0-q3-q4+q7<3*β32 |
| 5 | HEVCstrong_weak&& p0-p4+q0-q4<β8 && p0-p5+q0-q5<β8 |
|  | HEV&& |
| 0-3 | HEVC filter and decisions apply but reducing maximum filter length to half the minimum distance between current and neighboring CU boundaries |

Block boundary samples pi and qi for i=0 to S-1 are then replaced by linear interpolation as follows:

- pi´=(fi*Middles+64-fi*Ps)+32)≫6, clipped to pi±tc

- qi´=(fi*Middles+64-fi*Qs+32)≫6), clipped to qi±tc

where f, Middle, P and Q are given below:

| S | Filter kernels |
| --- | --- |
| 7 | fi=59-i*9, can also be described as f=59,50,41,32,23,14,5 Middle7=(2*po+qo+p1+q1+p2+q2+p3+q3+p4+q4+p5+q5+p6+q6+8)≫4 P7=(p6+p7+1)≫1, Q7=(q6+q7+1)≫1 |
| 5 | fi=58-i*13, can also be described as f=58,45,32,19,6 Middle5=(2*po+qo+p1+q1+p2+q2+p3+q3+p4+q4+8)≫4 P5=(p4+p5+1)≫1, Q5=(q4+q5+1)≫1 |
|  | =53-i*21, can also be described as f= =(3*+2*(+)+8)≫4 =(++1)≫1, =(++1)≫1 |

- Questions to be answered

Is it sufficient to filter 7 pixels on both sides of a large block boundary? Expects improvements in subjective quality from reducing discontinuities across large blocks, especially for low bitrates.

A: Visual tests are expected to be done during the meeting

#### CE11.1.2 JVET-K0393

- Technology to be investigated

HEVC deblocking filters are extended with longer deblocking filters for luma.

The proposed method consisted of two parts; edge boundary detection of large intra block and extension of the filtered samples on edge filtering process.

When the following conditions are true, the proposed decision process is continued:

- Edge boundary is located between two blocks of which cross edge sizes are 16 or longer than 16.

- The filter type is strong.

- At least one of both blocks is intra coded

It is noted that a block size is replaced by an edge size. Direction of edge size depends on that of edge boundary like EDGE_VER or EDGE_HOR.

When the above criteria is not true, the default short deblocking filter may be applied.

If the above criteria is true, at block P and block Q, following conditions are checked respectively. If (11.1.2-1) is true, extended filter is applied to block P. In the same manner, if (11.1.2-2) is true, extended filter is applied to block Q.

p60-2*p50+p40+p63-2*p53+p43<β2# SEQ Equation \* ARABIC 11.1.2-1

q60-2*q40+q40+q63-2*q53+q43<β2#11.1.2-2

Following filter kernels are used in the extended filtering process.

p6 = 3,5,2,1,1,1,1,1,1,0,0,0,0,0,0,0
p5 = 1,2,5,2,1,1,1,1,1,1,0,0,0,0,0,0
p4 = 0,1,3,3,3,1,1,1,1,1,1,0,0,0,0,0
p3 = 0,1,2,2,2,2,2,1,1,1,1,1,0,0,0,0
p2 = 0,0,1,2,2,2,2,2,2,1,1,1,0,0,0,0
p1 = 0,0,1,1,2,2,2,2,2,2,1,1,0,0,0,0
p0 = 0,0,0,1,2,2,2,2,2,2,2,1,0,0,0,0

There are no changes for chroma deblocking filter process.

#### CE11.1.3 JVET-K0232

- Technology to be investigated.

HEVC deblocking filters are extended with longer strong filters for luma. When all the following conditions are true, the proposed long strong filtering is applied. Otherwise, the HEVC strong filtering is applied for p-side and q-side

- The slice type is inter.

- The CB length at p-side and the CB length at q-side are either larger than or equal to eight.

- Both of the two inter-side difference values representing difference between p-side and q-side are larger than a threshold, inter_side_diff_th. The two inter-side difference values are defined as

InterSideDiffi =p0i-q1i+p1i-q0ia??1p0i-q1i+p1i-q0ia??1, where i = 0 or 3.

- Both of the two intra-side difference values representing variation within p-side plus variation within q-side are smaller than a threshold, intra_side_diff_th. The two intra-side difference values are defined as

IntraSideDiffi =, where i = 0 or 3.

- Any one of adjacent block edges also satisfies all the above four conditions.

When the long strong filtering decision output is yes, the long strong filter set selection is activated to select one of the three long strong filter sets for p-side and one of the three long strong filter sets for q-side. The selection based on the p-side and q-side length of the current block edge is described as follows, where lenp and lengq denote the CB length at p-side and the CB length at q-side, respectively.

- If “lenp ≥ 32 and lenq ≥ 32”, R8W7-ultra or R8W7-strong filtering is selected as follows:

  - If any one of adjacent block edges of the current block edge is also satisfied “lenp ≥ 32 and lenq ≥ 32”, R8W7-ultra filtering is applied to both p-side and q-side of the current block edge.

  - Otherwise (i.e., “lenp ≥ 32 and lenq ≥ 32” is only true for the current block edge), R8W7-strong filtering is applied to both p-side and q-side of the current block edge.

- If “32 > lenp ≥ 16”, R8W7-strong filtering is applied to p-side of the current block edge.

- If “16 > lenp ≥ 8”, R4W4-strong filtering is applied to p-side of the current block edge.

- If “32 > lenq ≥ 16”, R8W7-strong filtering is applied to q-side of the current block edge.

- If “16 > lenq ≥ 8”, R4W4-strong filtering is applied to q-side of the current block edge.

If R8W7-ultra filtering is applied to p-side, filtered sample values from  to  are as follows:

If R8W7-ultra filtering is applied to q-side, filtered sample values from  to  are as follows:

If R8W7-strong filtering is applied to p-side, filtered sample values from  to  are as follows:

If R8W7-strong filtering is applied to q-side, filtered sample values from  to  are as follows:

If R4W4-strong filtering is applied to p-side, filtered sample values from  to  are as follows:

If R4W4-strong filtering is selected at q-side, filtered sample values from  to  are as follows:

Within this CE, the performance of the deblocking method described in JVET-K0232, the filter coefficients, and the samples used in long strong filtering are to be studied.

#### CE11.1.4 JVET-K0334

- Technology to be investigated.

HEVC deblocking filters are extended with longer deblocking filters for luma. The deblocking process being tested in CE2.2.1.4 is similar to that in JEM with a long-tap filter (instead of strong HEVC filter) being conditionally applied to the boundaries of the block with size (orthogonal to the filtered boundary) being larger or equal to 16. A long deblocking filter is applied to large blocks boundary samples if smoothness criteria are met on both sides of the boundary:

- HEVC strong filter is selected

- Long block activity estimates PA and QA are being less than TLDB threshold

The activity estimates are computed independently for P and Q samples as a sum of absolute value of second derivatives:

PA=i=1N-1pi,j'' and QA=i=1M-1qi,j''

With N and M being the total number of samples involved in local activity estimate and variables pi,j'' and qi,j'' are absolute value of second derivatives:

pi,j''=pi-1,j-2pi,j+pi+1,j and qi,j''=qi-1,j-2qi,j+qi+1,j

The threshold TLDB being set equal to:

TLDB=(((perCountTh×count+1)≫1)+1)≫1) ×(BitDepth-8).

where count is a number of derivates in locale activity estimate and perCountTh is a parameter dependent on a slice type and on a PPS flag indicating smooth threshold, adaptively signaled by encoder.

When long filter is selected, the following long filters are used to modify pixels q0,q1, q2,q3 and  p0,p1, p2,p3, with filter hi is centered at pixel pi, where i=0,1,2,3:

h0=0, 2, 9, 12, 1, 3, 13, 16, 5, 3, 0,

h1=-1, 6, 7, 8, -2, 13, 15, 10, 8, 0, 0,

h2=-1, 4, 12, -3, 6, 14, 14, 16, 2, 0, 0,

h3=-3, 16, -3, 1, 5, 16, 18, 14, 0, 0, 0

Clipping is used to limit the amount of filtering. A boundary strength of 3 is used to determine tc for both luma and chroma. Increase tc offset if QP > 42 and decrease if QP<= 35, used for both luma and chroma. Tc is then scaled by 3 for first two samples near the boundary and scaled by 2 for the other samples.

Long filter can be turned on for respective 4 sample segment. Deblocking applied for 4x4 grid for luma.

#### CE11.1.5 JVET-K0112

- Technology to be investigated

HEVC deblocking filters are extended with longer deblocking filters for luma. 7 samples on a side of the boundary is filtered when length orthogonal to block boundary is >= 16, strong/weak filter determines use strong filter, sum of absolute pixel difference for pixel p8 and p0 and q8 and q0 compared to a threshold. Only 3 samples are filtered for the upper block when filtering horizontal boundaries. Decision on 4 sample segments of an 8x8 grid.

Following filters are used for left block in vertical filtering:

p6	{3, 5, 2, 1, 1, 1, 1, 1, 1}	      p7 ~ q0

p5	{1, 2, 5, 2, 1, 1, 1, 1, 1, 1}	   p7 ~ q1

p4	{1, 3, 3, 3, 1, 1, 1, 1, 1, 1}	   p6 ~ q2

p3	{1, 2, 2, 2, 2, 2, 1, 1, 1, 1, 1}	p6 ~ q3

p2	{1, 2, 2, 2, 2, 2, 2, 1, 1, 1}	   p5 ~ q3

p1	{1, 1, 2, 2, 2, 2, 2, 2, 1, 1}	   p5 ~ q3

p0	{1, 2, 2, 2, 2, 2, 2, 2, 1}	      p4 ~ q3

For the upper block in horizontal filtering only p0 to p2.

p2    {3, 5, 2, 2, 1, 1, 1, 1}  p3 ~ q3

p1    {2, 2, 4, 2, 2, 2, 1, 1}  p3 ~ q3

p0    {2, 2, 2, 2, 2, 2, 2, 2}  p3 ~ q3

#### CE11.1.6 JVET-K0152

- Technology to be investigated

HEVC deblocking filters are extended with longer deblocking filters for luma and chroma. 7 samples on each side of the boundary is filtered when the minimal length orthogonal to block boundary is >= 32, sum of absolute pixel difference and also Laplacians, and comparison with absolute value of pixel difference across boundary in comparison with tc. Decision is taken on two lines of 4 sample segments on a 4x4 grid.

The decision which has two additional conditions is also modified, accordingly. The first one is for large block decision since the proposed filter can be applied when the block width or height which orthogonally crosses the block edge is equal to or larger than 32. The second one is for proposed wide strong filter. The decision is extended based on HEVC strong filter decision (xUseStrongFiltering) as follows.

dp0 = xCalcDP( R[0] ); dp1 = xCalcDP( R[-2] ); dp2 = xCalcDP( R[-4] );

dq0 = xCalcDQ( R[0] ); dq1 = xCalcDQ( R[2] ); dq2 = xCalcDQ( R[4] );

dd0 = 2 * (dp0+dq0); dd2 = 2 * (dp1+dq1); dd4 = 2 * (dp2+dq2);

bdd = ( dd0 < (beta>>2) && dd2 < (beta>>2) && dd4 < (beta>>2) );

dd_strong0 = abs(R[-8] – R[-1]) + abs(R[0] – R[7]);

dd_strong1 = abs(R[-8] – R[-3]) + abs(R[2] – R[7]);

dd_strong2 = abs(R[-8] – R[-5]) + abs(R[4] – R[7]);

dd_strong_flag =

( (dd_strong0<(beta>>3)) && (dd_strong0<(beta>>3)) && (dd_strong0<(beta>>3)) );

If ( dd_strong_flag && bdd && (abs(R[-1]-R[0]) < ((tc*5+1)>>1)) ) is TRUE, and then the following filter is applied to P side in the same with Q side.

p6′= (7*p7+2*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (6*p7+p6+2*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (5*p7+p6+p5+2*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (4*p7+p6+p5+p4+2*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (3*p7+p6+p5+p4+p3+2*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (2*p7+p6+p5+p4+p3+p2+2*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

In this conribution, long-tap filter is also proposed for chroma. The decision which has three additional conditions is also modified, accordingly. The first one is for decision of boundary strength as well as large block. The proposed filter can be applied when the block width or height which orthogonally crosses the block edge is equal to or larger than 8 in chorma sample domain. The second and third one are basically the same as for HEVC luma decison, which are on/off decision and strong filter decision, respectively.

In the first decision, boundary strength (bS) is modified for chorma filtering as follows.

| Conditions | Y | U | V |
| --- | --- | --- | --- |
| At least one of the adjacent blocks is intra | 2 | 2 | 2 |
| At least one of the adjacent blocks has non-zero transform coefficients | 1 | 1 | 1 |
| Absolute difference between the motion vectors that belong to the adjacent blocks is greater than or equal to one integer luma sample | 1 | N/A | N/A |
| Motion prediction in the adjacent blocks refers to vectors is different | 1 | N/A | N/A |
| Otherwise | 0 | 0 | 0 |

The second and third decision is basically the same as HEVC luma strong filter decision as follows.

The second decision:

d < beta

d = Abs(p2,0−2*p1,0+ p0,0) + Abs( q2,0−2*q1,0+q0,0) + Abs(p2,1−2*p1,1+p0,1) + Abs(q2,1−2*q1,1+q0,1);

The third decision:

xUseStrongFiltering( );    // same as luma’s decision

If those decisions are satisfied, and then the following filter is applied to P side in the same with Q side.

p2′= (3*p3+2*p2+p1+p0+q0+4) >> 3

p1′= (2*p3+p2+2*p1+p0+q0+q1+4) >> 3

p0′= (p3+p2+p1+2*p0+q0+q1+q2+4) >> 3

The decision which has three additional conditions is also modified, accordingly. The first one is for decision of boundary strength as well as large block. The proposed filter can be applied when the block width or height which orthogonally crosses the block edge is larger than 16. The second and third one are basically similar to luma’s decision, which are on/off decision and strong filter decision, respectively.

- Questions to be answered

  - Subjective and objective impact of modified luma and chroma filter

A: Visual tests are expected to be done to evaluate the subjective quality during the meeting.

#### CE11.1.7 JVET-K0315

[1]	Technology to be investigated

JVET-J0077 incorporates stronger filters for both luma and chroma, compared to JEM deblocking. Additionally, the proposed modification includes a control process that considers the block sizes on both sides of the boundary being deblocked. The stronger filters are used for luma samples that correspond to larger block sizes, while the JEM deblocking filters are still used for luma samples corresponding to the smaller block sizes.  For chroma, the filter selection uses a different approach, and a stronger filter is applied when chroma samples on either sides of the deblocking boundary belong to a large block.

Luma is deblocked on 8x8 luma sample grid.

Luma stronger filters (same as JVET-J0038):

p6′= (7*p7+2*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (6*p7+p6+2*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (5*p7+p6+p5+2*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (4*p7+p6+p5+p4+2*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (3*p7+p6+p5+p4+p3+2*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (2*p7+p6+p5+p4+p3+p2+2*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

q6′= (7*q7+2*q6+q5+q4+q3+q2+ q1+q0+p0+ 8) >> 4

q5′= (6*q7+q6+2*q5+q4+q3+q2+q1+q0+p0+p1+ 8) >> 4

q4′= (5*q7+q6+q5+2*q4+q3+q2+q1+q0+p0+p1+p2+ 8) >> 4

q3′= (4*q7+q6+q5+q4+2*q3+q2+q1+q0+p0+p1+p2+p3+ 8) >> 4

q2′= (3*q7+q6+q5+q4+q3+2*q2+q1+q0+p0+p1+p2+p3+p4+8) >> 4

q1′= (2*q7+q6+q5+q4+q3+q2+2*q1+q0+p0+p1+p2+p3+p4+p5+8) >> 4

q0′= (q7+q6+q5+q4+q3+q2+q1+2*q0+p0+p1+p2+p3+p4+p5+p6+8) >> 4

Luma stronger filters are used for large blocks where large block corresponds to width >= 32 for a vertical edge, and height>=32 for a horizontal edge. The control process is further based on gradients computed for two lines of 4 sample segments; comparison of absolute pixel value difference with tc; and comparison of other absolute pixel value differences with β. More gradients are computed for large block side.

- The variables dpq0, dpq3, dp, dq, and d are derived as follows:

dp0, dp3, dq0, dq3 are first derived as in HEVC

if (p side is greater than or equal to 32)

dp0 = ( dp0 + Abs( p5,0 − 2 * p4,0 + p3,0 ) + 1 ) >> 1

dp3 = ( dp3 + Abs( p5,3 − 2 * p4,3 + p3,3 ) + 1 ) >> 1

if (q side is greater than or equal to 32)

dq0 = ( dq0 + Abs( q5,0 − 2 * q4,0 + q3,0 ) + 1 ) >> 1

dq3 = ( dq3 + Abs( q5,3 − 2 * q4,3 + q3,3 ) + 1 ) >> 1

dpq0, dpq3, dp, dq, d are then derived as in HEVC

- As in HEVC, When d is less than β, the following ordered steps apply:

- dpq is derived as in HEVC.

- sp3 = Abs( p3 − p0 ), derived as in HEVC

if (p side is greater than or equal to 32)

sp3 = ( sp3 + Abs( p7 − p3 ) + 1) >> 1

- sq3 = Abs( q0 − q3 ), derived as in HEVC

if (q side is greater than or equal to 32)

sp3 = ( sp3 + Abs( q7 − q3 ) + 1) >> 1

- As in HEVC derive, StrongFilterCondition = (dpq is less than ( β  >>  2 ), sp3 + sq3 is less than ( β  >>  3 ), and Abs( p0 − q0 ) is less than ( 5 * tC + 1 )  >>  1) ? TRUE : FALSE

- When StrongFilterCondition is TRUE, use luma stronger filter on the side with length perpendicular to boundary edge greater than or equal to 32.

Chroma deblocks on a 4x4 chroma sample grid

Chroma stronger filters (i.e. HEVC luma strong filter):

p0′ = ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 )  >>  3

p1′ = ( p2 + p1 + p0 + q0 + 2 )  >>  2

p2′ = ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 )  >>  3

q0′ = ( p1 + 2 * p0 + 2 * q0 + 2 * q1 + q2 + 4 )  >>  3

q1′ = ( p0 + q0 + q1 + q2 + 2 )  >>  2

q2′= ( p0 + q0 + q1 + 3 * q2 + 2 * q3 + 4 )  >>  3

Chroma stronger filters are used on both sides of the block boundary.  Specifically, the strong filter is used when any of the following conditions is true:

- The edge type is vertical an p0 belongs to CU with width >= 16 (chroma samples)

- The edge type is horizontal and p0 belongs to CU with height >= 16 (chroma samples)

- The edge type is vertical and q0 belongs to CU with width >= 16 (chroma samples)

- The edge type is horizontal and q0 belongs to CU with height >= 16 (chroma samples)

In another test (b), is same as above, except chroma stronger filters are used on both sides of the block boundary. Specifically, the strong filter is used when HEVC luma strong filter condition when computed for chroma is TRUE and any of the following conditions is true:

- The edge type is vertical an p0 belongs to CU with width >= 16 (chroma samples) and q0 belongs to CU with width >= 16 (chroma samples)

- The edge type is horizontal and p0 belongs to CU with height >= 16 (chroma samples) and q0 belongs to CU with height >= 16 (chroma samples)

[2] Questions to be answered

- Subjective impact on quality for CfP sequences, CTC sequences, and sequences with real strong edges (e.g. screen content) at low bit rates

- Objective impact of the changes for the same set of sequences content under CTC.

A: Visual tests are expected to be done to evaluate the subjective quality during the meeting.

#### CE11.1.8 JVET-K0369

Two technologies are to be investigated

- New luma longer tap filters for large blocks are proposed.

New luma longer tap filters:

Filter condition:                       HEVCstrongweak

&&

p0-p3- p4+p7+q0-q3- q4+q7<3*β32 &&

p0-p2-p3+p5+q0-q2-q3+q5<3*β32

Proposed filter coefficients for longer tap filter:

p6′= (6*p7+3*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (5*p7+p6+3*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (4*p7+p6+p5+3*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (3*p7+p6+p5+p4+3*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (2*p7+p6+p5+p4+p3+3*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (p7+p6+p5+p4+p3+p2+3*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

q6′= (6*q7+3*q6+q5+q4+q3+q2+ q1+q0+p0+ 8) >> 4

q5′= (5*q7+q6+3*q5+q4+q3+q2+q1+q0+p0+p1+ 8) >> 4

q4′= (4*q7+q6+q5+3*q4+q3+q2+q1+q0+p0+p1+p2+ 8) >> 4

q3′= (3*q7+q6+q5+q4+3*q3+q2+q1+q0+p0+p1+p2+p3+ 8) >> 4

q2′= (2*q7+q6+q5+q4+q3+3*q2+q1+q0+p0+p1+p2+p3+p4+8) >> 4

q1′= (q7+q6+q5+q4+q3+q2+3*q1+q0+p0+p1+p2+p3+p4+p5+8) >> 4

q0′= (q7+q6+q5+q4+q3+q2+q1+2*q0+p0+p1+p2+p3+p4+p5+p6+8) >> 4

Proposed longer tap filter condition for horizontal boundaries overlapping with CTU boundaries:

HEVCstrongweak &&

p3-p2-p1+p0+q0-q3- q4+q7<3*β32 &&

p3-p2-p1+p0+q0-q2-q3+q5<3*β32

Proposed longer tap filter coefficients for horizontal boundaries overlapping with CTU boundaries:

p2′= (6*p3+3*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (5*p3+p2+3*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (5*p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

q6′= (6*q7+3*q6+q5+q4+q3+q2+ q1+q0+p0+ 8) >> 4

q5′= (5*q7+q6+3*q5+q4+q3+q2+q1+q0+p0+p1+ 8) >> 4

q4′= (4*q7+q6+q5+3*q4+q3+q2+q1+q0+p0+p1+p2+ 8) >> 4

q3′= (3*q7+q6+q5+q4+3*q3+q2+q1+q0+p0+p1+p2+p3+ 8) >> 4

q2′= (2*q7+q6+q5+q4+q3+3*q2+q1+q0+p0+p1+p2+2*p3+8) >> 4

q1′= (q7+q6+q5+q4+q3+q2+3*q1+q0+p0+p1+p2+3*p3+8) >> 4

q0′= (q7+q6+q5+q4+q3+q2+q1+2*q0+p0+p1+p2+4*p3+8) >> 4

- Questions to be answered

  - Is the “Longer tap” reducing the blocking artifacts at large block boundaries?

  - Is the Longer tap variant proposed for “line buffer” reduction, good enough to reduce blocking artifacts appearing on horizontal block boundaries overlapping with CTU boundaries?

#### CE11.1.9 JVET-K0318

[1] Technology to be investigated:

On top of long-tap luma filter in test CE11.1.1:

Use HEVC luma filter and decisions with following modifications for chroma deblocking filtering.

Determine the number of samples to modify in block P and Q as follows:

lengthP = edge==EDGE_HOR ? (CUP_height_chroma >> 2) : (CUP_width_chroma >>2)

lengthQ = edge==EDGE_HOR ? (CUQ_height_chroma >> 2) : (CUQ_width_chroma >>2)

Only filter boundary if both lengthQ and lengthP is larger than 0.

The HEVC strong filter (reading 4 samples and modifying 3 samples on respective side) is only applied if both sides have a maximum deblocking filter length larger than 2.

Strong/weak filter and number of samples to filter is determined for each line. Inter blocks are only filtered if Laplacian gradients near the boundary on respective side is below the structure threshold “beta” but for intra blocks that check is by passed.

“beta” parameter is divided by 8 compared to what is used for luma.

Question to be answered: Is there a benefit from deblocking with long chroma deblocking filter in addition to long luma filter?

#### CE11.1.10 Combination of CE11.1.6 and CE11.1.7

The stronger filters are used for luma samples that correspond to larger block sizes, while the JEM deblocking filters are still used for luma samples corresponding to the smaller block sizes. Luma deblocks on 8x8 luma sample grid

Luma Stronger Filter :

p6′= (7*p7+2*p6+p5+p4+p3+p2+ p1+p0+q0+ 8) >> 4

p5′= (6*p7+p6+2*p5+p4+p3+p2+p1+p0+q0+q1+ 8) >> 4

p4′= (5*p7+p6+p5+2*p4+p3+p2+p1+p0+q0+q1+q2+ 8) >> 4

p3′= (4*p7+p6+p5+p4+2*p3+p2+p1+p0+q0+q1+q2+q3+ 8) >> 4

p2′= (3*p7+p6+p5+p4+p3+2*p2+p1+p0+q0+q1+q2+q3+q4+8) >> 4

p1′= (2*p7+p6+p5+p4+p3+p2+2*p1+p0+q0+q1+q2+q3+q4+q5+8) >> 4

p0′= (p7+p6+p5+p4+p3+p2+p1+2*p0+q0+q1+q2+q3+q4+q5+q6+8) >> 4

Luma stronger filters are used for large blocks where large block corresponds to width >= 32 for a vertical edge, and height>=32 for a horizontal edge. The adjacent block must be greater than or equal to 16. The control process is further based on gradients computed for two lines of 4 sample segments; comparison of absolute pixel value difference with tc; and comparison of other absolute pixel value differences with β. More gradients are computed for large block side.

- The variables dpq0, dpq3, dp, dq, and d are derived as follows:

dp0, dp3, dq0, dq3 are first derived as in HEVC

if (p side is greater than or equal to 32)

dp0 = ( dp0 + Abs( p5,0 − 2 * p4,0 + p3,0 ) + 1 ) >> 1

dp3 = ( dp3 + Abs( p5,3 − 2 * p4,3 + p3,3 ) + 1 ) >> 1

if (q side is greater than or equal to 32)

dq0 = ( dq0 + Abs( q5,0 − 2 * q4,0 + q3,0 ) + 1 ) >> 1

dq3 = ( dq3 + Abs( q5,3 − 2 * q4,3 + q3,3 ) + 1 ) >> 1

dpq0, dpq3, dp, dq, d are then derived as in HEVC

- As in HEVC, When d is less than β, the following ordered steps apply:

- dpq is derived as in HEVC.

- sp3 = Abs( p3 − p0 ), derived as in HEVC

if (p side is greater than or equal to 32)

sp3 = ( sp3 + Abs( p7 − p3 ) + 1) >> 1

- sq3 = Abs( q0 − q3 ), derived as in HEVC

if (q side is greater than or equal to 32)

sp3 = ( sp3 + Abs( q7 − q3 ) + 1) >> 1

- As in HEVC derive, StrongFilterCondition = (dpq is less than ( β  >>  2 ), sp3 + sq3 is less than ( β  >>  3 ), and Abs( p0 − q0 ) is less than ( 5 * tC + 1 )  >>  1) ? TRUE : FALSE

- When StrongFilterCondition is TRUE, use luma stronger filter on the side with length perpendicular to boundary edge greater than or equal to 32.

Chroma deblocks on a 4x4 chroma sample grid

Use of HEVC luma strong filter is also proposed for chroma when both sides of the chroma edge are greater than or equal to 16 (other threshold, such as 8 may be tested) and the following decision with three conditions are satisfied. The first one is for decision of boundary strength as well as large block. The proposed filter can be applied when the block width or height which orthogonally crosses the block edge is equal to or larger than 8 in chroma sample domain. The second and third one are basically the same as for HEVC luma decison, which are on/off decision and strong filter decision, respectively.

In the first decision, boundary strength (bS) is modified for chorma filtering as follows.

| Conditions | Y | U | V |
| --- | --- | --- | --- |
| At least one of the adjacent blocks is intra | 2 | 2 | 2 |
| At least one of the adjacent blocks has non-zero transform coefficients | 1 | 1 | 1 |
| Absolute difference between the motion vectors that belong to the adjacent blocks is greater than or equal to one integer luma sample | 1 | N/A | N/A |
| Motion prediction in the adjacent blocks refers to vectors is different | 1 | N/A | N/A |
| Otherwise | 0 | 0 | 0 |

The second and third decision is basically the same as HEVC luma strong filter decision as follows.

The second decision:

d < beta

d = Abs(p2,0−2*p1,0+ p0,0) + Abs( q2,0−2*q1,0+q0,0) + Abs(p2,1−2*p1,1+p0,1) + Abs(q2,1−2*q1,1+q0,1);

The third decision:

xUseStrongFiltering( );    // same as luma’s decision

If those decisions are satisfied, and then the following filter is applied to P side in the same with Q side.

p0′ = ( p2 + 2 * p1 + 2 * p0 + 2 * q0 + q1 + 4 )  >>  3

p1′ = ( p2 + p1 + p0 + q0 + 2 )  >>  2

p2′ = ( 2 * p3 + 3 * p2 + p1 + p0 + q0 + 4 )  >>  3

#### CE11.1.11 Combination of CE11.1.1 and CE11.1.7

The filtering method in CE11.1.1 (JVET-K0307) is modified to enable use of long asymmetric filters. For long asymmetric filters the number of samples deblocked on the larger block side is greater than that on the smaller block side. The deblocking decision processes described in CE11.1.1 (JVET-K0307) and CE11.1.7 (JVET-K0315) are combined to select from this extended filter set [TBD: How exactly are they combined?]. The extended filter set may be used for strong deblocking throughout the deblocking process [TBD: what are the details of the proposed combination?].

### CE 11.2: General aspects of deblocking filter

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE11.2.1 | Christophe Gisquet christophe.gisquet@crf.canon.fr JVET-K0203 | Anand Meher Kotra anand.meher.kotra@huawei.com |
| CE11.2.2 | Atsuro Ichigaya ichigaya.a-go@nhk.or.jp JVET-K0386 | Anand Meher Kotra anand.meher.kotra@huawei.com |

#### CE11.2.1: Higher-precision modifications to VVC deblocking filters JVET-K0203

- Technology to be investigated

TEST 1: chroma conditional filtering

Chroma deblocking decision from luma. Chroma filtering ON/OFF decision switches on the same pixel granularity as luma one does, following CTCs (10 bits). This is related to section 1.2.3.1 of contribution JVET-K0203.

TEST 2: tc parameter link to bit depth

This corresponds to section 1.1 of contribution JVET-K0203. This test requires 3 types of encoding. The goal is to show that the rule from HEVC equation 8-353 is sub-optimal. As a consequence, encoding at 2 bit depths is required, and showcasing “traditional (as in HEVC)” vs “scaling” is required. The following encodings are thus required:

- Per 10 bits CTC, using an “optimal” scaling from a high-precision tc curve

- Encoding with an internal bit depth of 8 bits:

  - Using the 10-bits scaling, and dividing it by 4 to mimic HEVC rule in equation  8-353

  - Proposing an “optimal” 8-bit scaling instead

TEST 3: combination of TEST1 and TEST2, following CTCs (10 bits)

TEST 4: TEST3 with longer chroma filter, following CTCs (10 bits)

The addition to TEST3 corresponds to what is described in section 1.3.2.2 of contribution JVET-K0203. Current filtering process for chroma samples (section 8.7.2.5.8 of H.265 v5) are as follows:

Δ = Clip3( −tC, tC, ( ( ( ( q0 − p0 )  <<  2 ) + p1 − q1 + 4 )  >>  3 ) )				(8-405)

p0′ = Clip1C( p0 + Δ )													(8-406)

q0′ = Clip1C( q0 − Δ ) 													(8-407)

The added filtering is:

Δ1 = Δ >> 1

p1′ = Clip1C( p1 + Δ1 )

q1′ = Clip1C( q1 – Δ1 )

#### CE11.2.2 Luma-adaptive deblocking filter JVET-K0386

- Technology to be investigated

The variable LL of the reconstructed luma level is derived as follow:

LL= ( ( p0,0 + p0,3 + q0,0 + q0,3 ) >> 2 ) / ( 1 << iBitDepth ),               (1)

where, the sample values pi,k and qi,k with i = 0..3 and k = 0 and 3 are derived the same as HEVC as shown in Figure 1.

Figure 1: Sample position of pi,k and qi,k

The variables QpQ and QpP are set equal to the QpY values of the coding units which include the coding blocks containing the sample q0,0 and p0,0, respectively.

The variable qPL is derived as follows:

qPL = ( ( QPQ + QPP +1 ) >> 1 ) + qpOffset,                    (2)

where qpOffset is specified in Table 1.

The subsequent derivation processes of the variables β and tC are same as HEVC.

Table 1. Specification of qpOffset as a function of LL

| transfer_characteristics == 16 (PQ format) |  | transfer_characteristics == 18 (HLG format) |  | Otherwise (conventional SDR format) |  |
| --- | --- | --- | --- | --- | --- |
| Conditions | qpOffset | Conditions | qpOffset | Conditions | qpOffset |
| LL <= 0.84 | 0 | LL <= 0.8 | 0 | LL <= 0.875 | 0 |
| 0.84 < LL <= 0.91 | 1 | 0.8 < LL <= 0.9 | 1 | 0.875 < LL | 1 |
| 0.91 < LL <= 0.99 | 2 | 0.9 < LL | 2 |  |  |
| 0.99 < LL | 3 |  |  |  |  |

### CE 11.3: Deblocking at 4x4 block boundaries

| Test | Proponent(s) | Cross-checker(s) |
| --- | --- | --- |
| CE11.3.1 | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-K0318 | Dmytro Rusanovskyy dmytror@qti.qualcomm.com |
| CE11.3.2 | Kenneth Andersson kenneth.r.andersson@ericsson.com JVET-K0318 | Chia-Ming Tsai chia-ming.tsai@mediatek.com |
| CE11.3.3 | Anand Meher Kotra Anand.meher.kotra@huawei.com JVET-K0369 | Kiran Misra misrak@sharplabs.com |
| CE11.3.4 | Chia-Ming Tsai chia-ming.tsai@mediatek.com JVET-J0018 | Kenneth Andersson kenneth.r.andersson@ericsson.com |
| CE11.3.5 | Hyeongmun Jang hm.jang@lge.com JVET-D0044 | Chia-Ming Tsai chia-ming.tsai@mediatek.com |

#### CE11.3.1 CU deblocking on 4x4 grid for luma JVET-K0318

Technology to be investigated:

Determine the number of samples to modify in block P and Q as follows:

lengthP = edge==EDGE_HOR ? (CUP_height >> 1) – 1 : (CUP_width>>1) - 1

lengthQ = edge==EDGE_HOR ? (CUQ_height >> 1) – 1 : (CUQ_width>>1) - 1

This means that only one sample will be filtered with the weak VVC filter on side P when the distance between boundaries on P side is 4 and only one sample will be filtered with the weak VVC filter on side Q when the distance between the boundaries on Q side is 4 .

The weak VVC filter can be used with two sample filtering on at least one side if lengthP or lengthQ is larger than 1. The strong VVC filter can be used if both lengthP and lengthQ are larger than 2.

Question to be answered: Is there a subjective benefit of deblocking of 4x4 grid compared to 8x8 grid?

#### CE11.3.2 Filtering of sub-block boundaries for luma JVET-K0318

Technology to be investigated:

On top of filtering on 4x4 grid for luma as in CE11.3.1, deblocking is done on sub-block boundaries on 8x8 grid for CUs that use ATMVP on an 8x8 grid and on sub-block boundaries on 4x4 grid for CUs that use ATMVP on an 4x4 grid.

Question to be answered: Is there a subjective benefit of deblocking of sub-block boundaries from ATMVP?

#### CE11.3.3 CE2-related: Deblocking for 4 x N and N x 4 CUs JVET-K0369

Sub-tests: a and b

Two solutions are proposed to be studied here:

- Sub-test a: When block width/height is equal to 4 for vertical edge/horizontal edge respectively and if the HEVC filter on/off condition (i.e. d < Beta, where d = d0 + d3 and d0 = dp0 + dq0 and d3= dp3 + dq3) is evaluated to true for the respective edge, then this method would enforce HEVC normal/weak filter with maximum one sample modification. Therefore the following HEVC condition is checked δ<10tC where δ=(9*q0-p0- (3*q1-p1+8≫4, and if the condition is evaluated to true, then samples p0 and q0 are modified, otherwise no filtering is applied.

- Sub-test b: When block width/height is equal to 4 for vertical edge/horizontal edge respectively, then a maximum of three samples are used in filter decision and only one sample is modified by the filter. i.e. p2i is used to replace p3i in strong/weak filter condition checks and both strong and weak filter are only allowed to modify p0 and q0

Question to be answered:

Is deblocking necessary for edges belonging to 4 x N and N x 4 CU’s (or) is an 8 x 8 grid filtering good enough.

#### CE11.3.4 Parallel deblocking for 4 x N and N x 4 block boundaries JVET-J0018

Technology to be investigated:

In JVET-J0018, to support parallel deblocking for 4xN and Nx4 blocks, p1i and q1i are used to replace the pixels outside p1i and q1i when doing filtering decision and filtering operations. For example, the below equation is the condition for deciding whether the HEVC deblocking filter is applied or not.

p20-2×p10+p00+p23-2×p13+p03+q20-2×q10+q00+q23-2×q13+q03<β

In the proposed method, if the lengths of two sides are both 4, then the filtering decision is modified to

p10-2×p10+p00+p13-2×p13+p03+q10-2×q10+q00+q13-2×q13+q03<β

If only the length of p side is 4, then the filtering decision is modified to

p10-2×p10+p00+p13-2×p13+p03+q20-2×q10+q00+q23-2×q13+q03<β

If only the length of q side is 4, then the filtering decision is modified to

p20-2×p10+p00+p23-2×p13+p03+q10-2×q10+q00+q13-2×q13+q03<β

Finally, 2 samples or 1 samples on respective side of the 4 x N and N x 4 block boundary may be filtered.

Within this CE, the performance of the parallel deblocking method described in JVET-J0018 is to be studied in terms of subjective quality and computational complexity.

Question to be answered:

#### CE11.3.5 Parallel deblocking filter JVET-D0044

Technology to be investigated:

In JVET-D0044, parallel processing for deblocking filter was already issued. Deblocking filter can be processed in parallel because it is not applied to 4x4 TU and PU boundaries. Within this CE, the performance of the parallel deblocking method described in JVET-D0044 is to be studied in terms of subjective quality and computational complexity [TBD: please provide the details of the method].

Question to be answered:

## Expected Gains

- Subjective picture quality improvement while preferably keeping rate-distortion performance.

- Coding efficiency improvement.

## Evaluation Criteria

1. Measure impact on Bitrate/PSNR. The BD-rate is calculated for CTC and for CTC with ALF switched off relative to Anchor-1 and Anchor-2 respectively. The results for LDP configurations are mandatory for both CTC and CTC with ALF switched off.

2. Visual impact on output pictures of representative sequences (including SDR, possibly HDR test sequences). The subjective test is expected to be held to determine the visual impact. The subjective test will use 10 second versions of the sequences. The proponents shall provide the bitstreams and decoders for the sequences that are chosen for evaluation in the subjective test.

3. Complexity of the proposals evaluated in the CE shall be compared. In particular, the following aspects of the proposals shall be summarized:

- decoding/encoding time vs the anchor

- number of samples from the block boundary that can be modified by deblocking (may be per block size)

- number of samples from the block boundary that are needed for deblocking decisions (may be per block size)

- number of filtering operations (such as additions, multiplications, comparisons)

- number of line buffers

- parallelization capabilities, such as size of a unit of a picture that can be processed in parallel to other units.

- The data should be provided for both luma and/or chroma if they are modified.

## Subjective test(s)

a) A

subjective test is conducted by the test chair before the Macao meeting.

The following test sequences and conditions will be used in the subjective tests. The sequence length is 10 seconds. This means that the versions of some sequences encoded for the subjective test are longer than those used in the CTC [1]. The following encoded sequences shall be provided at QP34 and QP39 [Choice of QP may be refined further].

The following sequences shall be provided to the MPEG by September 12 (uploaded to the JVET ftp, exact URL will be communicated later), with ALF off:

FoodMarket, 3840x2160p, 60fps, 600 frames, RA, QP 39

Campfire, 3840x2160p, 30fps, 300 frames, RA, QP 39

Kimono, 1920x1080p, 24fps, 240 frames, LDB, QP 34

KristenAndSara, 1280x720p, 60 fps, 600 frames, LDP, QP 34

Park Running, 3840x2160p, 50fps, 500 frames, RA, QP 39

The anchors for the subjective viewing will be provided by Huawei (Anand).

b) There is a possibility that an additional subjective test is performed at the Macao meeting if needed. The following sequences shall be prepared by the proponents for potential viewing at the Macao meeting:

FoodMarket, 3840x2160p, 60fps, 600 frames, RA, QP 34, 39, ALF on and ALF off

Campfire, 3840x2160p, 30fps, 300 frames, RA, QP, 34, 39, ALF on and ALF off

Kimono, 1920x1080p, 24fps, 240 frames, LDB, QP, 34, 39, ALF on and ALF off

KristenAndSara, 1280x720p, 60 fps, 600 frames, LDP, QP 34, 39, ALF on and ALF off

Park Running, 3840x2160p, 50fps, 500 frames, RA, QP 34, 39, ALF on and ALF off

## Subjective test description:

[TBD: To be defined with MPEG test chair]

# Interdependency with other CEs

There may be interaction with CE2 on Adaptive Loop Filter.

# Test conditions and data to be provided

The test conditions are according to CTC on VTM with ALF turned on and VTM with ALF turned off. In addition, the proponents need to provide results for additional QP and additional test sequences chosen by the CE (see above). The sequences length for the subjective viewing is 10 seconds.

# Time-line and Responsibilities

2018-August - 08:		Final CE description uploaded

2018-August - 29:		Cross-checking begins, proponents provide software

2018-September - 12: 	Final version of CE software, full test results must be provided at this time.

Sequences for the subjective test are uploaded to the ftp.

# References

- “JVET common test conditions and software reference configurations”, JVET-K1010.

- “Algorithm description for Versatile Video Coding and Test Model 2 (VTM2)”, JVET-K1002.