| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K1032-v5 |
| --- | --- |

| Title: | Description of Core Experiment 12 (CE12): mapping functions |  |  |
| --- | --- | --- | --- |
| Status: | Output document to JVET |  |  |
| Purpose: | CE description |  |  |
| Author(s) or Contact(s): | E. François, D. Rusanovskyy, P. Yin | Tel: Email: | edouard.francois@technicolor.com dmytror@qti.qualcomm.com pyin@dolby.com |
| Source: | CE coordinators |  |  |

_____________________________

# Introduction

The purpose of this CE is to investigate the performance of mapping for HDR and SDR content coding. The considered technologies are out-of-loop reshaping and in-loop reshaping, and related variants. The considered technologies will be compared using objective metrics and using subjective assessments to be reported by the proponents and the cross-checkers.

# Tools description

This CE will study the following tools:

- [JVET-K0298]: out-of-loop dynamic range adaptation (DRA)

- [JVET-K308]: in-loop reshaping for HDR

- [JVET-K309]: in-loop reshaping for SDR

- [JVET-K0468]: In-loop chroma refinement

## [JVET-K0298]: out-of-loop dynamic range adaptation (DRA)

Dynamic range adaptation (DRA) is applied on input video data, Y’CbCr 4:2:0 picture (10 bits per sample) prior to coding (pre-processing). Inverse DRA is implemented after decoding loop as a post-processing (output process). The DRA design includes three parts: DRA mapping, chroma sample refinement, harmonization of DRA scaling with coding QP.

Dynamic Range Adjustment (DRA) is range-adaptive mapping of luma samples and luma-dependent scaling of chroma samples. Mapping/scaling is applied as a pre-processing and its inverse is applied at the decoder side as post-processing. In practice, in decoder, inverse DRA can be implemented using 1D look-up-tables LUTinvDraY , LUTinvDraCb and LUTinvDraCr :

Ymap = LUTinvDraY[ Yrec ]

Cxmap = ( Cxdec - 2BDchr-1 ) * LUTinvDraCx[ Yrec_coloc ] + 2BDchr-1      x=b or r

DRA scales and scalar quantizer harmonization consists in adjusting the CC-DRA scale functions to preserve a luma/chroma bit allocation assumed by the core coding design for a given QP settings introduced by chroma QP offset table (cf Table 8-10 of HEVC specification).

DRA refinement process consists in a correction of the DRA inverse mapping functions to compensate the distortion resulting from the compression that may impact the inverse DRA process. The inverse DRA LUTs are corrected by the refinement LUTs, modeled by piece-wise linear functions.

LUTinvDraY[ k ] = Min( 2Blum – 1, Max( 0, LUTrefineY[ k ] * LUTinvDraY[ k ] )

LUTinvDraCx[ k ] = LUTrefineCx[ k ] * LUTinvDraCx[ k ]

LUTrefineY, LUTrefineCb, LUTrefineCr are built from piece-wise linear (PWL) tables, signaled per slice. The PWL pivot points are equi-distant, and size is signaled in the slice header. When a table is not signaled for a component, the table previously decoded is used.

## [JVET-K0468]: in-loop cross-component chroma refinement

The in-loop cross-component chroma refinement process directly operates to the reconstructed signal after loop filtering (UR, VR) as follows:

- Urefine = Max(0, Min( 2Bchr – 1, 2Bchr – 1 + LUTrefineU[ YRcoloc ] * ( UR – 2Bchr – 1 ))

- Vrefine = Max(0, Min( 2Bchr – 1, 2Bchr – 1 + LUTrefineV[ YRcoloc ] * ( VR – 2Bchr – 1 ))

where YRcoloc is the co-located reconstructed luma sample

LUTrefineU, LUTrefineV are built from piece-wise linear (PWL) tables, signaled per slice. When tables are not signaled, no refinement applies to the current slice.

## [JVET-K0308, JVET-K0309, Dolby]: in-loop reshaping

The in-loop reshaper proposed in JVET-J0015 maps the luma code values based on the slice type received by the decoder. The luma reshaping process is defined as following.

For an intra slice, coding process is performed in the reshaped domain, followed by inverse reshaping before storing in DPB. The simplified HDR intra slice decoding workflow is shown in Figure 1.

Figure 1. HDR Intra slice reconstruction with in-loop Luma Reshaper.

For an inter slice, reshaping is performed within the prediction loop. At encoder, the residue is generated by. The decoding sample is obtained as . All the other inter decoding tools are kept unchanged. A simplified HDR inter slice decoding workflow is shown in Figure 2.

Figure 2. HDR Inter slice reconstruction with in-loop Luma Reshaper

Reshaping parameters are transmitted once per IRAP picture.

The in-loop Luma reshaper with the following coding features are also going to be tested:

- Luma-based Chroma QP Offset: to compensate for the luma signal interaction with the Chroma signal, an additional chroma QP Offset is used to adjust chroma quantization parameters for chroma component decoding. The chroma QP Offset is derived as follows:

where  is the slope (first order derivative) of the  and  is the average luma value of the CU. This can be implemented with chroma residue scaling to avoid pipeline issues.

- Slice/CTU adaptation: in-loop reshaping activation can be done at slice and CTU level.

In JVET-K0308, variants of each aspect of in-loop reshaping for both luma and chroma for HDR-PQ has been tested. The following recommended parameters setting serves as a basis for in-loop reshaping improvement for HDR:

- sps_reshaper_chromaAdj set to 1 (chroma QP adjustement activated)

- sps_reshaper_ILF_opt set to 1 for AI and 3 for RA; this corresponds to LF activation in the reshaped domain for inter slice when TemporalId <= TemporalId_Thr, and in the reshaped domain for Intra slice before applying the inverse reshaping.

In JVET-K0309, the in-loop reshaping is applied to improve SDR video coding results using the same in-loop reshaping architecture for HDR, but different derivation of luma curve.

# Tests to be performed

The following tests aim at evaluating the pixel-level mapping, at comparing the out-of-loop and in-loop implementations performance for HDR, and test in-loop reshaping and chroma refinement for SDR.

## HDR Category

In this sub-CE, out-of-loop and in-loop mapping is explored for HDR.

Two luma mapping curves are considered for this sub-CE:

- Luma DRA: luma mapping curve used in JVET-K0298, built from a piece-wise linear (PWL) model based on anchor dQP table, normalized to [0,1023], and mapping the signal to full range values.

- Luma Reshaping: luma mapping curve used in JVET-K0308, signalled using anchor dQP model and built from anchor dQP table, normalized to [0,1023], and mapping the signal to full range values.

### CE12-1: out-of-loop dynamic range adaptation (JVET-K0298)

In this sub-CE, out-of-loop DRA will be tested using different settings for the luma mapping. Also, DRA settings will be refined to get similar luma/chroma balance as the anchor.

| Test | Proponent | Description | xchecker |
| --- | --- | --- | --- |
| CE12-1.1 | Qualcomm Technicolor (JVET-K0298) | out-of-loop mapping with Luma DRA and with aligned chroma/luma bit allocation with HDR anchor | T. Lu (Dolby) |
| CE12-1.2 |  | out-of-loop mapping with Luma Reshaping and with aligned chroma/luma bit allocation with HDR anchor | T. Lu (Dolby) |

### CE12-2: in-loop reshaping for HDR (JVET-K0308)

In this sub-CE, the following tests aim at further evaluating the variants of the in-loop approach.

| Test | Proponent | Description | xchecker |
| --- | --- | --- | --- |
| CE12-2.1 | Dolby (JVET-K0308) | Assess in-loop reshaping with Luma Reshaping versus Luma DRA | E. Francois (Tch) |
| CE12-2.2 |  | Test in-loop reshaping using Luma Reshaping, with chroma residue scaling with qp harmonization of JVET-K0298 | E. Francois (Tch), D. Rusanovskyy (QCM), K. Misra (Sharp) |

### CE12-3: in-loop cross-component chroma refinement (JVET-K0298)

| Test | Proponent | Description | xchecker |
| --- | --- | --- | --- |
| CE12-3.1 | Technicolor (JVET-K0298) | Test in-loop chroma refinement for HDR (JVET-K0468) versus HDR anchor | A. Dsouza (SMG) |

## SDR Category

### CE12-4: In-loop Reshaping for SDR (JVET-K0309)

| Test | Proponent | Description | Xchecker |
| --- | --- | --- | --- |
| CE12-4 | Dolby (JVET-K0309) | Test in-loop reshaping for SDR based on JVET-K0309 | L. Zhang (Bytedance), D. Rusanovskyy (QCM) |

Note that, if possible, the proponent will provide results of modified anchor, using dQP table based on the sequence-adaptive reshaping function used in CE12-4.

### CE12-5: in-loop cross-component chroma refinement (JVET-K0298)

In this test, it is not planned to use mapping. The tool will be tested into the HDR anchor, without mapping the content.

| Test | Proponent | Description | xchecker |
| --- | --- | --- | --- |
| CE12-5.1 | Technicolor (JVET-K0298) | Test in-loop chroma refinement for SDR (JVET-K0468) versus SDR anchor | D. Rusanovskyy (QCM) |

# Experimental Test Conditions

## Software

VTM software shall be used for these experiments.

## Test Sequences, Bit Rates, Coding Conditions

Tests sequences, bit rates, and coding conditions defined in the common tests conditions described in document JVET-K1011 will be used.

Proposed tools will be implemented in the VTM and performance will be compared to the VTM HDR anchor. The tools integration will be made in the BMS. For the simulations, the VTM results will be produced by running BMS with VTM configuration, using configuration parameters.

## Evaluation of Results

The following data shall be reported for each test:

- objective metrics, encoding and decoding runtime ratios, as specified in the HDR CTCs (JVET-K1011) and SDR CTCs (JVET-K1010).

  - Decoding time shall be computed with Yuv output activated.

- Luma / chroma bit allocation per sequence.

- Average number of operations per sample.

SDR and HDR results are to be reported in two distinct parts.

In addition, cross-checkers are mandated to report results from objective metrics evaluation and from visual assessments. Expert viewing assessments are planned to be made during the 12th JVET meeting, if appropriate SDR and HDR viewing equipment is available.

# Time-line

| T1 | Aug. 01 | Final CE description |
| --- | --- | --- |
| T2 | Sept. 06 | Completion of tool integration, test configuration announcement, made software available |
| T3 | Sept. 14 | Changes to software branches is frozen and proponent starts reporting results to CE coordinators for inclusion in CE report. Cross-checks start. |
| T4 | Sept. 19 | Completion of cross-check. Reporting of complete results to CE coordinators. |

# Participants

| Nr. | Name | Company | Email | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
| 1 | Taoran Lu Fangjun Pu Peng Yin Sean McCarthy | Dolby | tlu@dolby.com fpu@dolby.com pyin@dolby.com smcca@dolby.com | X | X |
| 2 | Dmytro Rusanovskyy Adarsh K. Ramasubramonian | Qualcomm | dmytror@qti.qualcomm.com | X | X |
| 3 | Edouard François | Technicolor | edouard.francois@technicolor.com | X | X |
| 4 | Andrew Segall Kiran Misra Takeshi Chujoh Tomohiro Ikai | Sharp | asegall@sharplabs.com misrak@sharplabs.com chujoh.takeshi@sharp.co.jp ikai.tomohiro@sharp.co.jp |  |  |
| 5 | Alexei Filippov Vasily Rufitskiy Sergei Ikonin | Huawei | alexey.filippov@huawei.com vasily.rufitskiy@huawei.com Sergey.Ikonin@huawei.com |  |  |
| 6 | Yi-Wen Chen Xianglin Wang | Kwai | yiwenchen@kwai.com xianglinwang@kwai.com |  |  |
| 7 | Peisong Chen Minhua Zhou Wade Wan | Broadcom | peisong.chen@broadcom.com minhua.zhou@broadcom.com wade.wan@broadcom.com |  |  |
| 8 | Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com |  |  |
| 9 | Pankaj Topiwala | FastVDO | pankajtva@gmail.com |  |  |
| 10 | Alexis Tourapis | Apple | atourapis@apple.com |  |  |
| 11 | Chirag Pujara Amith DSouza | Samsung | chirag.p@samsung.com amith.ds@samsung.com |  |  |
| 12 | Dan Grois | Comcast | grois@ieee.org |  |  |
| 13 | Koohyar Minoo |  | koohyar.minoo@gmail.com |  |  |
| 14 | Limin Wang |  | cuiwang@gmail.com |  |  |
| 15 | Li Zhang | Bytedance | lizhang.idm@btyedance.com |  |  |
| 16 | Jangwon Choi | LGE | jangwon84.choi@lge.com |  |  |
| 17 | Ching-Chieh Lin Po-Han Lin | ITRI | jacklin@itri.com pohan@itri.com |  |  |
| 18 | Jacob Ström | Ericsson | jacob.strom@ericsson.com |  |  |