| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K1033-v1 |
| --- | --- |

| Title: | Description of Core Experiment 13 (CE13): Coding tools for 360° omnidirectional video |  |  |
| --- | --- | --- | --- |
| Status: | Output document to JVET |  |  |
| Purpose: | Core Experiment description |  |  |
| Author(s) or Contact(s): | Philippe Hanhart Jian-Liang Lin | Tel: Email: | Philippe.Hanhart@InterDigital.com jl.lin@mediatek.com |
| Source: | CE coordinators |  |  |

_____________________________

# Abstract

The main goal of this Core Experiment (CE) is to study removing subjective artifacts at face boundaries, comparing normative decoding tools with non-normative pre- and post-processing. The tested coding tools include modifications to intra and inter prediction, as well as in-loop filtering and projection format adaptation. For pre- and post-processing, different amount of padding, blending, and post-filtering of seam artifacts will be tested. The tested solutions will be implemented for the hybrid equi-angular cubemap (HEC) projection and compared to the HEC with padding of 4 samples around face row with blending (PHEC) anchor.

# Participants

| Company | Name | Email | Proponent | Cross-checker |
| --- | --- | --- | --- | --- |
| Arris | Yue Yu Seungwook Hong Limin Wang | Yue.Yu@arris.com Seungwook.Hong@arris.com Limin.Wang@arris.com |  |  |
| Broadcom | Minhua Zhou Wade Wan Peisong Chen | minhua.zhou@broadcom.com wade.wan@broadcom.com peisong.chen@broadcom.com |  |  |
| Canon | Jonathan Taquet Patrice Onno | Jonathan.TAQUET@crf.canon.fr Patrice.Onno@crf.canon.fr |  |  |
| Ericsson | Julien Le Tanou | julien.le.tanou@ericsson.com |  |  |
| ETRI | Gun Bang Woowoen Gwun | gbang@etri.re.kr woogwun@gmail.com |  |  |
| GoPro | David Newman | dnewman@gopro.com |  |  |
| HHI | Robert Skupin | robert.skupin@hhi.fraunhofer.de |  |  |
| Huawei | Sergey Ikonin | sergey.ikonin@huawei.com |  |  |
| Intel | Jill Boyce Zhipin Deng | jill.boyce@intel.com zhipin.deng@intel.com |  |  |
| InterDigital | Philippe Hanhart Yuwen He | Philippe.Hanhart@interdigital.com Yuwen.He@interdigital.com |  |  |
| Korea Aerospace University | Yong-Uk Yoon | yuyoon@kau.kr |  |  |
| LG | Ling Li | l.li@lge.com |  |  |
| MediaTek | Jian-Liang Lin Ya-Hsuan Lee Yu-Wen Huang Cheng-Hsuan Shih Sheng-Yen Lin | jl.lin@mediatek.com Louise.Lee@mediatek.com yuwen.huang@mediatek.com ch.shih@mediatek.com Sheng-Yen.Lin@mediatek.com |  |  |
| Nokia | Alireza Aminlou | alireza.aminlou@nokia.com |  |  |
| Qualcomm | Muhammed Coban | mcoban@qti.qualcomm.com |  |  |
| RWTH Aachen Univesity | Johannes Sauer Max Bläser | sauer@ient.rwth-aachen.de blaeser@ient.rwth-aachen.de |  |  |
| Samsung | Chirag Pujara Kiho Choi Woongil Choi Amith Dsouza Anubhav Singh | chirag.p@samsung.com kiho14.choi@samsung.com woongil.choi@samsung.com amith.ds@samsung.com anubhav.23@samsung.com |  |  |
| Sharp | Sachin Deshpande Kiran Misra Tomohiro Ikai Kiran Misra | sdeshpande@sharplabs.com misrak@sharplabs.com ikai.tomohiro@sharp.co.jp misrak@sharplabs.com |  |  |
| Technicolor | Fabien Racape | Fabien.Racape@technicolor.com |  |  |
| TNO | Emmanuel Thomas | emmanuel.thomas@tno.nl |  |  |
| USTC | Dong Liu Yefei Wang | dongeliu@ustc.edu.cn wyfei@mail.ustc.edu.cn |  |  |
| Zhejiang University | Yule Sun Lu Yu | sunyule@zju.edu.cn yul@zju.edu.cn |  |  |
|  | Koohyar Minoo | koohyar.minoo@gmail.com |  |  |

# CE13.1: Padding and blending

## Summary

| Proposal | Description |
| --- | --- |
| 360Lib | Padding around face row with blending |

## Technical description

### HEC with padding around face row with blending

Please refer to JVET-K1004 [1] for details.

## Test specification

| Test # | Description | Proponent | Cross-checker |
| --- | --- | --- | --- |
| 1.1 | HEC with padding and blending (PHEC) - 4 samples padding around face row - 8 samples padding around face row | Anchor InterDigital | Samsung |

# CE13.2: Intra prediction

## Summary

| Proposal | Description |
| --- | --- |
| J0015 (InterDigital) | Intra prediction disabled across face discontinuities |
| J0019/K0141 (MediaTek) | Intra prediction using spherical neighbors |

## Technical description

### Intra prediction disabled across face discontinuities in J0015 (InterDigital)

For intra prediction (DC, planar, and angular modes, as well as cross-component linear model), the use of reference samples is modified as follows. If a CU does not cross a face discontinuity, then it is determined whether each of the available reference samples is located on the same side of the face discontinuity to which the current CU belongs, and only those available reference samples that are located on the same side of the face discontinuity are used in intra prediction.

For most probable modes (MPM) in intra prediction, the construction of the MPM list is modified as follows. If a CU does not cross a face discontinuity, then it is determined if each available spatial neighbor in the frame packed picture is also a spherical neighbor, and only the available frame packed neighbors that are also spherical neighbors are used to build the MPM list.

### Intra prediction using spherical neighbors in J0019/K0141 (MediaTek)

The intra prediction constructs efficient predictors by using the neighboring reconstructed pixels of the current CU. Similar to the geometry padding in inter prediction, the reference pixels of intra prediction can be derived from spherical neighboring pixels. In the proposed method, when a CU is located at the frame boundary (layout boundary) or the discontinuous edge(s), its neighboring pixels are replaced by the spherical neighboring reconstructed pixels, which are located at other faces.

## Test specification

| Test # | Description | Proponent | Cross-checker |
| --- | --- | --- | --- |
| 2.1 | HEC with intra prediction disabled across face discontinuities | InterDigital MediaTek |  |
| 2.2 | HEC with intra prediction using spherical neighbors | MediaTek | Samsung |

# CE13.3: Inter prediction

## Summary

| Proposal | Description |
| --- | --- |
| J0019/K0141 (MediaTek) | Geometry padding of reference pictures |
| J0023 (RWTH Aachen University) | Face extension of reference pictures |
| K0333 (InterDigital) | Horizontal geometry padding of reference pictures for PERP |

## Technical description

### Geometry padding of reference pictures in J0019/K0141 (MediaTek)

The inter prediction replaces the replication padding with the geometry padding when the reference pixels are outside the frame boundary (layout boundary) or across the discontinuous edge(s). The geometry padding in Fig. 2 demonstrates a pixel position qb qboutside the Bottom face is represented by its spherical corresponding pixel qa, which is located at the Front face. This spherical corresponding pixel is obtained by the projection mapping functions. When the derived coordinate qa is fractional, the 6-tap and 4-tap Lanczos interpolation filters could be applied for the luminance and chrominance components, respectively.

Fig. 2 Geometry padding process

### Face extension of reference pictures in J0023 (RWTH Aachen University)

Reference pictures are padded with projections of their neighboring faces. The padded reference pictures are then used for inter prediction instead of the original ones.

### Horizontal geometry padding of reference pictures for PERP in K0333 (InterDigital)

The horizontal geometry padding for PERP is the same as for ERP, with the exception that the offset should be based on the unpadded ERP width instead of the picture width to account for the size of the padded regions. Figure 1 depicts the horizontal geometry padding for PERP. If a reference block is outside the left (right) reference picture boundary, it is replaced with the “wrapped-around” reference block shifted to the right (left) by the ERP width. The traditional repetitive padding is still employed in the vertical direction. The blending of the left and right padded regions is kept out of loop, as a post-processing operation.

Figure 1. Horizontal geometry padding for PERP.

## Test specification

| Test # | Description | Proponent | Cross-checker |
| --- | --- | --- | --- |
| 3.1 | HEC with geometry padding of reference pictures - Using Lanczos 3/2 interpolation - Using bilinear interpolation - Using face-based padding | MediaTek InterDigital MediaTek InterDigital |  |
| 3.2 | HEC with face extension of reference pictures | RWTH Aachen University |  |
| 3.3 | PERP with horizontal geometry padding of reference pictures | InterDigital |  |

# CE13.4: In-loop filters

## Summary

| Proposal | Description |
| --- | --- |
| J0015 (InterDigital) K0142 (MediaTek) J0024/J0025/K0404 (Samsung) | In-loop filters disabled across face discontinuities |
| J0019/K0142 (MediaTek) | In-loop filters using spherical neighbors |
| J0023 (RWTH Aachen University) | Geometry corrected in-loop filters |

## Technical description

### In-loop filters disabled across face discontinuities in J0015 (InterDigital), K0142 (MediaTek), and J0024/J0025/K0404(Samsung)

To reduce face seam artifacts, deblocking is only applied where, considering the largest filter, all samples used in the filtering process are located on the same side of a face discontinuity. In SAO edge offset mode, only the samples that are located on the same side of the face discontinuity to which the current sample belongs are used for filtering the current sample. ALF is only applied where, considering the largest filter, all samples used in the filtering process are located on the same side of a face discontinuity.

### In-loop filters using spherical neighbors in J0019/K0142 (MediaTek)

### For further improvement, the second method is implemented based on the geometric continuity. The in-loop filtering process makes use of the pixel data located at the spherical corresponding positions for filtering so that the discontinuous edge, as well as the frame boundary (layout boundary), can be appropriately processed. Because the accessed pixels outside the layout boundary and discontinuous edge are obtained from other faces, extra memory buffers are needed.

#### De-blocking Filter

Based on the concept of geometric continuity, the de-blocking filter fetches the spherical corresponding pixels located at another face for edge filtering when the filtered pixels cross the frame boundary (layout boundary) or the discontinuous edge as illustrated in Fig. 1. The pixel data in the spherical corresponding positions are copied directly from the corresponding face. After the pixel data are collected for edge filtering, the motion vector located at the geometric pixel positions is also adjusted by the motion projection, which is proposed in JVET-0019 [1] for appropriate boundary strength derivation.

Fig. 1 Spherical corresponding pixels from another face for 360°-based DB filter based on geometric continuity

#### Sample Adaptive Offset filter

As illustrated in Fig. 2, when the referenced neighboring pixels cross the discontinuous edge, the proposed implementation uses the spherical corresponding pixels located at another face for SAO filtering. Moreover, the pixel data in the spherical corresponding positions are copied directly from the corresponding face.

Fig. 2 Spherical corresponding pixels from another face for 360°-based SAO filter based on geometric continuity

### Geometry corrected in-loop filters in J0023 (RWTH Aachen University)

For loop filters operating across face boundaries the samples of the neighboring CU are replaced with the spherical neighbors.

## Test specification

| Test # | Description | Proponent | Cross-checker |
| --- | --- | --- | --- |
| 4.1 | HEC with in-loop filters disabled across face discontinuities - Deblocking - SAO - ALF - All filters | MediaTek Samsung MediaTek MediaTek InterDigital MediaTek |  |
| 4.2 | HEC with in-loop filters using spherical neighbors - Deblocking - SAO - ALF - All filters | MediaTek | Samsung |
| 4.3 | HEC with geometry corrected in-loop filters | RWTH Aachen University | Samsung |

# CE13.5: Post-filtering

## Summary

| Proposal | Description |
| --- | --- |
| K0183 (Zhejiang University) | Post-filtering of seam artifacts using HEVC deblocking strength=1 |

## Technical description

### Post-filtering of seam artifacts in K0183 (Zhejiang University)

To reduce the seam artifacts of reconstructed viewports including face boundary, a face boundary post-filtering method is proposed. The seam artifacts are caused by coded blocks which are adjacent on the sphere but nonadjacent in the frame-packed image. The proposed filter imitates deblocking filter in HEVC. The difference is that BS is set a fixed strength of 1 and the QP in slice level is used to determine the parameters tc and β.

## Test specification

| Test # | Description | Proponent | Cross-checker |
| --- | --- | --- | --- |
| 5.1 | Post-filtering of seam artifacts using HEVC deblocking - PHEC using deblocking with boundary strength=1 - HEC with padding of 3*2 samples between cube face rows without blending using deblocking with boundary strength=1 | Zhejiang University | Samsung |

# CE13.6: Projection format adaptation

## Summary

| Proposal | Description |
| --- | --- |
| J0015/K0331/K0332 (InterDigital) | Adaptive frame packing |
| J0024/J0025/K0387 (Samsung) | Pre-rotation |

## Technical description

### Adaptive frame packing in J0015/K0331/K0332 (InterDigital)

#### Adaptive frame packing

To reduce the visibility of face seam artifacts, adaptive frame packing is proposed to periodically modify the location of discontinuous face boundaries in the frame packed pictures. The advantage of changing frame packing parameters at regular time intervals is that, for a discontinuous face boundary in the current picture, if there is a reference picture which uses a different frame packing where that same face boundary is continuous, then better prediction may be achieved for the areas located near the discontinuous face boundary in the current picture. For example, if the reference and current pictures are packed using different frame packing configurations as depicted in Figure 2(a) and Figure 2(b), respectively, then the face boundary between face #1 and face #4 in 3D geometry, marked by the red dashed lines, can be better predicted from the corresponding areas in the reference picture. This is because the areas near that face boundary in the reference picture, marked by the green line in Figure 2(a), are continuous and therefore generally easier to be coded at a higher quality; thus, better prediction may be provided for the current picture around these areas, reducing the visibility of the seam artifact in the current picture. Additionally, by changing frame packing parameters at regular intervals, locations of the discontinuous face boundaries in the frame packed pictures will change periodically. As a result, though face seams may still appear at some point in time, the duration of a face seam at the same location will be reduced. The result is visually more pleasant compared to having a face seam artifact always at the same location, which could catch the viewer’s attention and degrade perceived video quality when watching the viewport video.

Figure 2. Inter frame prediction in the case of different frame packing configurations between the reference (a) and current (b) pictures. The converted reference picture used to predict the current picture is shown in (c).

As depicted in Figure 2, when the frame packing configuration of the reference picture is different from that of the current picture, the reference picture is first converted into the frame packing configuration of the current picture. This conversion is straightforward and only involves “reshuffling” of the faces within the frame packed pictures. An example of the converted reference picture is shown in Figure 2(c).

The adaptive frame packing parameters are determined at the encoder side at every IRAP and signaled in the PPS.

#### Adaptive frame packing determination at the encoder

Considering the HEC 3×2 layout, there are six different combinations to select two face groups of three faces each. Using the face definitions from the 360Lib, these combinations are listed in Table 1. Considering that the top and bottom parts of the spherical video are typically less interesting and/or easier to encode, only the combinations where three faces along the equators are grouped together. This reduces the problem to four different combinations (combinations 1-4 in Table 1).

Table 1. Combinations to select two face groups of three faces

| Combination | Group #1 | Group #2 |
| --- | --- | --- |
| 1 | PX, PZ, NX | PY, NZ, NY |
| 2 | PX, NZ, NX | PY, PZ, NY |
| 3 | PZ, PX, NZ | PY, NX, NY |
| 4 | PZ, NX, NZ | PY, PX, NY |
| 5 | PX, PY, NX | PZ, NY, NZ |
| 6 | PZ, PY, NZ | PX, NY, NX |

For any of these four combinations, the three equatorial faces in group #1 are placed horizontally, whereas the remaining three face in group #2 may be rotated in the frame packed picture. However, rotations of ±90° may impact coding efficiency, especially for intra coding. Indeed, a better prediction can be achieved for a pattern composed of diagonal lines, i.e., from upper left to lower right, than for a pattern composed of anti-diagonal lines, i.e., from upper right to lower left, because some intra prediction directions (e.g., those close to the intra mode 2) require reference samples located below left and above right of the current block, respectively, which are more likely to be unavailable.

To determine which rotation is optimal for a face, the horizontal, vertical, diagonal, and anti-diagonal gradients are computed on the luma component using 1-D Laplacian. Then, gradient classification is applied on an 8×8 block basis to determine the dominant gradient in each block. The gradient values are accumulated over all samples within a block. The accumulated horizontal, vertical, diagonal, and anti-diagonal gradient values, denoted gh, gv, gd, and ga, respectively, are compared to each other’s to determine the block’s dominant gradient following the flowchart depicted in Figure 3, where α is a parameter used to classify between active and inactive gradients and is set to 2.

Figure 3. Flowchart for determining the dominant gradient for an 8×8 block.

For each face, a cost function is built as

| ci = wh∙gh,i+wv∙gv,i+wd∙gd,i+wa∙ga,i | (1) |
| --- | --- |

Where i is the face index, wh, wv, wd, and wa are the weights assigned to the horizontal, vertical, diagonal, and anti-diagonal dominant gradient counts gh, gv, gd, and ga, respectively. Considering that intra prediction for patterns composed of anti-diagonal lines may be less efficient than for other directions, the following set of weights are used: wh=wv=wa=-1, wd=1. Then, the overall cost for each combination (see Table 1) is computed as the sum of the individual cost of each face, considering the orientation of each face.

A subset of the first four adaptive frame packing combinations shown in Table 1 is used in a rotating manner at the beginning of each IRAP. Among the four combinations, the cost of each combination is calculated using the first picture of the input video sequence. If the cost of any combination is significantly higher than that of the best combination, then that combination is not used in coding the input video sequence to avoid reducing coding efficiency. Since only the first picture in the sequence is used in the determination, this solution can be used together with parallel encoding. Without loss of generality, after analyzing the first picture, assume that all four combinations are determined to be viable, and they are ranked based on their cost as C1, C2, C3, C4. For the first picture, the best combination, i.e., C1, is used. For the subsequent IRAP pictures, two sets of combinations are formed, i.e., {C1,C2} and {C3,C4}, and the optimal combination is selected as

| Copt=arg min{C1,C2}cif t % 2T=0arg min{C3,C4}cotherwise | (2) |
| --- | --- |

where T is the IRAP period, % is the modulo operator, and c is the cost function computed for the given IRAP picture. Adaptive frame packing parameters are selected by the encoder using the first picture of each IRAP and signaled once per IRAP.

### Pre-rotation in J0024/J0025/K0387 (Samsung)

It has been observed that rotation helps in improving the coding gain for 360° video. Rotation of the content before projection can help in reducing the discontinuities in the complex region (hard to encode) and align the edges in the content more suitable for the conventional video compression.

Input image

36 RSP image

Generation

(Low Resolution)

Compute

Gradient angles

Find the image

with max no. of

vertical and

horizontal gradients

Figure 4: Block diagram of optimal rotation parameter estimation algorithm

To arrive at the optimal rotation angle without encoding the video we devised the following algorithm steps,

- Downsize the image to 2K size and generate 36 RSP images at angles multiple of 10 degrees starting from 10 to 360 degrees. (only for X axis rotation)

- Compute gradient magnitudes and angles for each of these 36 images

- Find the image with maximum number of vertical and horizontal gradients from these images. Here we consider gradients between -5 to 5 degrees as horizontal and -85 to 95 degrees as vertical.

- After arriving at the best angle at the granularity of 10 degree these steps can be further repeated to reach to 1 degree resolution.

- This algorithm can also be applied at RAP level.

Though this algorithm was tested on RSP but it’s applicable for CMP and its variants

## Test specification

| Test # | Description | Proponent | Cross-checker |
| --- | --- | --- | --- |
| 6.1 | HEC with adaptive frame packing | InterDigital | Samsung |
| 6.2 | HEC with pre-rotation | Samsung |  |

# CE13.7: Combinations

## Test specification

| Test # | Description | Proponent | Cross-checker |
| --- | --- | --- | --- |
| 7.1 | Padding and in-loop filters disabled across face discontinuities 1.1.a + 4.1.d | MediaTek |  |
| 7.2 | In-loop filters disabled across face discontinuities and post-filtering of seam artifacts - 4.1.d + post-filtering using HEVC deblocking with boundary strength=1 - 4.1.d + post-filtering using HEVC deblocking with boundary strength=2 | Zhejiang University Samsung Samsung |  |
| 7.3 | Intra prediction and all in-loop filters disabled across face discontinuities 2.1 + 4.1.d | MediaTek |  |
| 7.4 | Intra prediction disabled across face discontinuities, face-based padding of reference pictures, and all in-loop filters disabled across face discontinuities 2.1 + 3.1.c + 4.1.d | MediaTek |  |
| 7.5 | Intra prediction disabled across face discontinuities, face-based padding of reference pictures, all in-loop filters disabled across face discontinuities, and adaptive frame packing 2.1 + 3.1.c + 4.1.d + 6.1 | InterDigital |  |
| 7.6 | Intra prediction disabled across face discontinuities, face-based padding of reference pictures, all in-loop filters disabled across face discontinuities, adaptive frame packing, and post-filtering of seam artifacts 2.1 + 3.1.c + 4.1.d + 6.1 + post-filtering using HEVC deblocking with boundary strength=2 | InterDigital | Samsung |
| 7.7 | Face extension of reference pictures and geometry corrected in-loop filters 3.2+ 4.3 | RWTH Aachen University | Samsung |
| 7.8 | Intra prediction using spherical neighbors, geometry padding of reference pictures, and all in-loop filters using spherical neighbors - 2.2 + 3.1.a + 4.2.d - 2.2 + 3.1.b + 4.2.d | MediaTek | Samsung |

# Test conditions and evaluation criteria

The normative decoding tools and non-normative pre- and post-processing solutions will be tested for both the Test Model (VTM) and the Benchmark Set (BMS). Proposals will be evaluated using BD rates, encoder run time, and decoder run time under the common test conditions and evaluation procedures for 360° video [2] with the exception that the face size for HEC will be set to 1280×1280 luma samples. Additionally, viewports will be extracted near the HEC vertexes for informal subjective viewing at the next meting to assess the visibility of seam artifacts.

# SW integration guideline

According to the information from SW coordinator,

- BMS with JEM_TOOLS macro disabled should produce the same bitstream as VTM

- Bitstreams of BMS with VTM config file and BMS with JEM_TOOLS macro disabled should only differ by the tool indication flags in the SPS, which should have a very minor impact on bit rate

Implementing a proposed coding tool on BMS SW only could serve the purpose of VTM and BMS tests. Therefore, the following guideline for SW integration is suggested

- Proponent implement the coding tool in BMS SW

- The results of VTM are obtained by disabling BMS tools via configuring encoder parameters of BMS SW (no change of macro setting)

# Timeline

| T1 | 2018-Aug-XX | - Final CE description |
| --- | --- | --- |
| T2 | 2018-XX-XX | - Proponents completed initial integration of tools (in individual branch) - Cross checkers can start initial study of tools / software (Note: Proponents can continue to modify the software until T3) |
| T3 | 2018-XX-XX | - Proponents froze changes to software branches - Proponents tag software and announce on JVET reflector - The names of the cross checkers and the list of specific tests for each CE tool are announced on JVET reflector - Proponents provide full test results - Cross checkers start final cross checks |

# References

- Y. Ye, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 7)”, JVET-K1004, July 2018, Ljubljana, SI.

- P. Hanhart, J. Boyce, K. Choi, “JVET common test conditions and evaluation procedures for 360° video”, JVET-K1012, July 2018, Ljubljana, SI.