| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 11th Meeting: Ljubljana, SI, 10–18 July 2018 | Document: JVET-K1034-v1 |
| --- | --- |

| Title: | Description of Core Experiment 14 (CE14): Post-reconstruction filtering |  |  |
| --- | --- | --- | --- |
| Status: | Output document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Li Zhang Sergey Ikonin | Tel Email: | lizhang.idm@bytedance.com sergey.ikonin@huawei.com |  |
| Source: | CE14 coordinators |  |  |

_____________________________

# Abstract

The goal of this Core Experiment (CE) is to study technolgoies of post-reconstruction filtering methods. The techniques are evaluated according to BD-rate gain, complexity (in both encoder and decoder). Participants in this activity are Alibaba, Apple, Bytedance, Broadcom, Comcast, Ericsson, ETRI, InterDigital, HHI, HiSilicon, KDDI, Kwai, MediaTek, Qualcomm, Samsung, Sejong University, SHARP, Sony, Technicolor and Tencent.

The software basis for this CE is BMS-2.0, for VTM based comparisons the BMS software is configured to produce VTM-2.0 [3]. For the test sequences, configurations and test conditions, in JVET-K1010 [1] for SDR.

It is also noted that combination of technologies listed in this CE are encouraged.

# Test conditions and evaluation criteria

Two anchors which include directly using VTM and BMS will be used in this CE. Proposals will primarily be compared with respect to objective quality, but subjective quality and complexity will also be measured according to the subtest requirements.

# Tool descriptions

## Test conditions

| CE | Tool on (VTM_tool_test) | Tool off (BMS_tool_test) |
| --- | --- | --- |
| 14 | VTM + sub-CE test vs VTM | BMS + sub-CE test vs BMS |

## Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 14.1 | CE2 related: Reduced complexity bilateral filter | JVET-K0274 |
| 14.2 | In-loop bilateral filter | JVET-K0384 |
| 14.3 | Hadamard Transform Domain Filter | JVET-K0068 |

## Detailed description of tests

### CE14.1 Reduced complexity bilateral filter JVET-K0274

[1] Technology to be investigated:

The basis of this experiment will be the bilateral filter presented in JVET-K0274. Three experiments will be carried out:

CE14.1.a Turning off the filtering for 4x4 intra blocks.

CE14.1.b Turning off the filtering for 4x4, 4x8 and 8x4 intra blocks

CE14.1.c Bilateral filter with LUT rows consisting of 16 8-bit values.

Information about latency and throughput of filtering in terms of number of clock cycles per sample is also solicited.

### CE14.2 Bilateral filter JVET-K0384

[1] Technology to be investigated:

Bilateral filter is always applied to luma blocks with non-zero transform coefficients and slice quantization parameter larger than 17. Therefore, there is no need to signal the usage of the bilateral filter. Bilateral filter, if applied, is performed on decoded samples right after the inverse transform. In addition, the filter parameters, i.e., weights are explicitly derived from the coded information.

The filtering process is defined as:

P0,  0'=P0, 0+k=1KWkPk, 0-P0, 0×Pk, 0-P0, 0, 			 (1)

where P0,  0 is the intensity of the current sample and P0, 0' is the modified intensity of the current sample, Pk,  0 and Wk(∙) are the intensity and weighting parameter for the k-th neighboring sample, respectively. An example of one current sample and its four neighboring samples (i.e., K=4), in the shape of a plus sign, is depicted in Figure 1.

Figure 1: Neighboring samples utilized in bilateral filter.

More specifically, the weight Wk(x) associated with the k-th neighboring sample is defined as follows:

Wk(x)=Distancek× Rangek(x) 					(2)

wherein

Distancek=e- 100002σd21+4*e- 100002σd2,

Rangekx=e– x28*QP – 17*(QP – 17)

and σd is dependent on the coded mode and coding block sizes. To further improve the coding performance, for inter-coded blocks, the intensity difference between current sample and one of its neighboring samples is replaced by a representative intensity difference among two windows covering current sample and the neighboring sample. Therefore, the equation of filtering process is revised to:

P0,  0'=P0, 0+k=1NWk1Mm=-M/2M/2abs(Pk, m-P0, m)×(Pk, 0-P0, 0)              (3)

wherein Pk, m and P0, m represent the m-th sample value within the windows centered at  Pk,  0 and P0,  0, respectively. In this proposal, the window size is set to 3×3. An example of two windows covering P2,  0 and P0,  0 are depicted in Figure 2.

Figure 2: Windows covering two samples utilized in weight calculation.

### CE14.3 Hadamard Transform Domain Filter JVET-K0068

Hadamard transform domain filter is always applied to luma reconstructed blocks with non-zero transform coefficients, excluding 4x4 blocks and if slice quantization parameter is larger than 17. The filter parameters are explicitly derived from the coded information. Proposed filter, if applied, is performed on decoded samples right after inverse transform.

For each pixel from reconstructed block pixel processing comprises the following steps:

- Scan for 4 neighboring pixels around processing pixel including current one according to scan pattern

- 4 point Hadamard transform of read pixels

Fi,σ=Ri2Ri2+σ2*R(i)

Spectrum filtering based on the following formula:

,wherein (i) is index of spectrum component in Hadamard spectrum, R(i)  is spectrum component of reconstructed pixels corresponding to index, σ is filtering parameter deriving from codec quantization parameter QP using following equation:

σ=2(1+0.126*QP-27)

- After filtering step the filtered pixels are placed to its original positions into accumulation buffer.

- After completing filtering of all pixels in CU the accumulated values are normalized by number of processing groups used for each pixel filtering.

## Evaluation and complexity analysis

- Measure impact on Bitrate/PSNR relative to the anchor(s).

- Complexity analysis (e.g., encoding and decoding time measure, complexity analysis by filling the table below)

.

| Test# | filter shape | Computational complexity (# of multiplications/ additions/shifts/checks*) | Parallel friendly (each sample can be filtered independently from other samples) | Cycles | memory requirement | How to derive filter coefficients | Minimal CU Size for applying filters |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 14.1 |  |  |  |  |  |  |  |
| 14.2 |  |  |  |  |  |  |  |
| 14.3 |  |  |  |  |  |  |  |

# * max/min/abs are countered as checks

# Interdependency with other CEs

There may be interaction with CE10 on Combined and multi-hypothesis prediction which includes the diffusion filter from JVET-K0323.

# Test conditions and data to be provided

The common test conditions are used to evaluate the objective coding gains of different methods.

It is noted that the generation of VTM results could be done by disabling tools in configuration files (i.e., disabling tools in SPS) from BMS.

# Time-line and Responsibilities

2018-Aug - 1:	Final CE description uploaded

2018-Aug - 25:	Cross-checking begins, proponents provide software

2018-Sep - 19:	Final version of CE software, full test results must be provided at this time (at least for proposals targeting to be promoted to VTM at the next meeting)

# Appendix of Participants

| Nr. | Name | Company | Email | Proponents | Cross-checker |
| --- | --- | --- | --- | --- | --- |
| 1 | Alexis Tourapis | Apple | atourapis@apple.com |  |  |
| 2 | Li Zhang | Bytedance | lizhang.idm@bytedance.com |  |  |
| 3 | Minhua Zhou | Broadcom | minhua.zhou@broadcom.com |  |  |
| 4 | Peisong Chen | Broadcom | peisong.chen@broadcom.com |  |  |
| 5 | Wade Wan | Broadcom | wade.wan@broadcom.com |  |  |
| 6 | Jacob Ström | Ericsson | jacob.strom@ericsson.com | X |  |
| 7 | Jianhua Zheng | HiSilicon | zhengjianhua@hisilicon.com |  |  |
| 8 | Quanhe Yu | HiSilicon | yuquanhe@hisilicon.com |  |  |
| 9 | Sergey Ikonin | Huawei | sergey.ikonin@huawei.com | X |  |
| 10 | Alexey Filippov | Huawei | alexey.filippov@huawei.com |  |  |
| 11 | Vasily Rufitskiy | Huawei | vasily.rufitskiy@huawei.com |  |  |
| 12 | Taquet Jonathan | Canon | Jonathan.TAQUET@crf.canon.fr |  |  |
| 13 | Yu-Wen Huang | MediaTek | yuwen.huang@mediatek.com |  |  |
| 14 | Chih-Wei Hsu | MediaTek | cw.hsu@mediatek.com |  |  |
| 15 | Ching-Yeh Chen | MediaTek | chingyeh.chen@mediatek.com |  |  |
| 16 | Tzu-Der Chuang | MediaTek | peter.chuang@mediatek.com |  |  |
| 17 | Chia-Ming Tsai | MediaTek | chia-ming.tsai@mediatek.com |  |  |
| 18 | Yu-Chi Su | MediaTek | yu-chi.su@mediatek.com |  |  |
| 19 | Chen-Yen Lai | MediaTek | jenny.lai@mediatek.com |  |  |
| 20 | Jae Hoon Kim | Icloud | i.jaehoon.kim@icloud.com |  |  |
| 21 | Rahul Vanam | InterDigital | Rahul.Vanam@InterDigital.com |  |  |
| 22 | Xiaoyu Xiu | InterDigital | xiaoyu.xiu@interdigital.com |  |  |
| 23 | Daniel Luo | InterDigital | Daniel.Luo@InterDigital.com |  |  |
| 24 | Yuwen He | InterDigital | yuwen.he@interdigital.com |  |  |
| 25 | Yi-Wen Chen | Kwai | yiwenchen@kwai.com |  |  |
| 26 | Xianglin Wang | Kwai | xianglinwang@kwai.com |  |  |
| 27 | Yoshitaka Kidani | KDDI | yo-kidani@kddi.com |  |  |
| 28 | Kei Kawamura | KDDI | ki-kawamura@kddi.com |  |  |
| 29 | Kyohei Unno | KDDI | ky-unno@kddi-research.jp |  |  |
| 30 | Kiho Choi | Samsung | kiho14.choi@samaung.com |  |  |
| 31 | Woongil Choi | Samsung | woongil.choi@samsung.com |  |  |
| 32 | Jie Chen | Samsung | jie1222.chen@samsung.com |  |  |
| 33 | Dongjae Won | Sejong university | bigdj2002@sju.ac.kr |  |  |
| 34 | Jaemin Ha | Sejong university | hajama0123@sju.ac.kr |  |  |
| 35 | Takeshi Chujoh | Sharp | chujoh.takeshi@sharp.co.jp |  |  |
| 36 | Tomohiro Ikai | Sharp | ikai.tomohiro@sharp.co.jp |  |  |
| 37 | Masaru Ikeda | Sony | Masaru.Ikeda@sony.com |  |  |
| 38 | Bordes Philippe | Technicolor | hilippe.bordes@technicolor.com |  |  |
| 39 | De Lagrange Philippe | Technicolor | Philippe.DeLagrange@technicolor.com |  |  |
| 40 | Jing Ye | Tencent | jingye@tencent.com |  |  |
| 41 | Dmytro Rusanovskyy | Qualcomm | dmytror@qti.qualcomm.com | X |  |
| 42 | Dan Grois | Comcast | grois@ieee.org |  |  |
| 43 | Yu-Chen Sun | Alibaba | yuchen.s@alibaba-inc.com |  |  |
| 44 | Shiori Sugimoto | NTT | sugimoto.shiori@lab.ntt.co.jp |  |  |
| 45 | Johannes Erfurt | HHI | johannes.erfurt@hhi.fraunhofer.de |  |  |
| 46 | Victor Stepin | Huawei | Stepin.Victor@huawei.com |  |  |
| 47 | Roman Chernyak | Huawei | chernyak.roman@huawei.com |  |  |
| 48 | Jungwon Kang | ETRI | jungwon@etri.re.kr |  |  |
| 49 | Yongbing Lin | HiSilicon | linyongbing@hisilicon.com |  |  |
| 50 | Peng Yin | Dolby | pyin@dolby.com |  |  |
| 51 | LAROCHE Guillaume | Canon | guillaume.laroche@crf.canon.fr |  |  |
| 52 | Byeongdoo Choi | Sharp | choib@sharplabs.com |  |  |