| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L0023-v2 |
| --- | --- |

| Title: | CE3: Summary Report on Intra Prediction and Mode Coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Geert Van der Auwera Jin Heo Alexey Filippov | Email: | geertv@qti.qualcomm.com jin78.heo@lge.com Alexey.Filippov@huawei.com |
| Source: | CE3 coordinators |  |  |

_____________________________

# Introduction

This is the summary report of the third Core Experiment (CE3) [1]. The goal of CE3 is to study intra prediction tools including mode coding for the VVC standard.

The following is the list of defined sub-tests in CE3:

- CE3.1: Multiple reference line prediction (9 tests)

- CE3.2: Intra prediction modes (9 tests)

- CE3.3: Intra reference sample interpolation (7 tests)

- CE3.4: Bidirectional prediction (3 tests)

- CE3.5: Cross-component prediction and separate chroma tree (18 tests)

- CE3.6: Intra mode coding (7 tests)

The CE3 description [1] originally defined 70 tests which were reduced to 53 after tests were withdrawn. This document summarizes the objective results (BD-rates, runtimes), cross-check reports and related input contributions.

# Test conditions and guidelines

The CTC conditions as agreed at the Ljubljana meeting for SDR are followed for the evaluation of objective performance (JVET-K1010). In CE3, it was agreed that it is mandatory to report results with both the ‘All Intra’ and ‘Random Access’ test configurations. One mandatory and one optional anchor are used to study the performance of the proposed technologies. The first anchor is the ‘VVC Test Model’ or VTM, which is mandatory. The second anchor is the ‘Benchmark Model Set’ or BMS, which is optional. The BMS version 2.0.1 software is used with ‘vtm’ configuration files to produce the VTM anchor. In this case, tools are implemented into the BMS version 2.0.1 software. The optional BMS anchor is generated using BMS version 2.1 software and tools are optionally implemented in this software as well. Proposals are compared with respect to objective compression efficiency while complexity is also assessed.

The source codes of the tests were uploaded by proponents into the following CE3 GitLab repository:

https://vcgit.hhi.fraunhofer.de/JVET-K-CE3/VVCSoftware_BMS.git

# Cross-check reporting

Each test is covered by at least one cross-checker and the cross-check reports are included within the respective sub-test sections. Within CE3 it was agreed that cross-checkers answer the following questions:

- Did the cross-checker study the software? If so, did the cross-checker find any significant inconsistencies between the description and the provided software?

- Do the cross-checked results closely match the proponent results for AI and RA conditions (VTM and optional BMS)?

- Were additional results cross-checked?

- Additional comments that may be helpful to the group?

This report summarizes the answers in tables reporting whether the objective test results match in one table column and all other comments in a second table column.

# CE3.1 on ‘Multiple reference line prediction’

## Defined tests [1]

### Explicit signalling of reference line index

| Test | Ref. Lines | Intra Prediction Modes | Prediction Averaging | Ref. Line Index Signalling | Block Size Restr. | Top CTU Restr. | MTS Restr. | Doc. # |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.1.1 | 0,1,3 | Angular modes if index > 0 | NA | Intra mode after line index | y |  | y | JVET-L0283 (HHI, Tencent, Foxconn, ITRI) |
| 1.1.2 | 0,1,3 | Even angular modes if index > 0 | NA | Intra mode after line index | y |  | y |  |
| 1.1.3 | 0,1,3 | Angular modes if index > 0 | NA | Intra mode after line index (MPM only) |  |  |  |  |
| 1.1.4 | 0,1,3 | Angular modes if index > 0 | NA | Intra mode after line index (MPM only) |  | y |  |  |

Table clarifications (JVET-K1023):

- Reference line index signaling:

  - Mode after reference line index - for reference line idx > 0, only include additional reference line modes in MPM list

  - Mode after reference line index (MPM only) - for reference line idx > 0, only include additional reference line modes in MPM list and only signal mpm index without remaining mode

- Restrictions

  - Block sizes, i.e., disable MRL for narrow WxH blocks with abs(log2W-log2H) > 1 and blocks smaller than 8x8

  - Top line of CTU restriction, i.e., disable MRL for the first line of blocks inside a CTU to prevent using extended reference samples outside the current CTU line

### Prediction averaging

| Test | Ref. Lines | Intra Prediction Modes | Prediction Averaging | Ref. Line Index Signaling | Block Size Restr. | Top CTU Restr. | MDIS Restr. | Doc. # |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1.2.1 | 0,1 | Angular modes | 50/50 extended/nearest | NA | y |  |  | JVET-L0431 (Sony) |
| 1.2.2 | 0,1 | Angular modes | 50/50 extended/nearest | NA | y | y |  |  |
| 1.2.3 idem 1.2.5 | 0,1 | Angular modes | 25/75 extended/nearest | NA | y | y |  | JVET-L0150 (ETRI) |
| 1.2.4 | 0,1 | Angular modes | 25/75 extended/nearest | NA | y | y | y |  |
| 1.2.5 idem 1.2.3 | 0,1 | Angular modes | 25/75 extended/nearest | NA | y | y |  | JVET-L0412 (Technicolor) |

## CE3.1: Test results

### CE3.1: VTM-2.0.1 anchor

|  | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 1.1.1 | -0.61% | -0.33% | -0.36% | 125% | 99% | -0.33% | -0.17% | -0.13% | 108% | 100% |
| 1.1.2 | -0.47% | -0.22% | -0.23% | 130% | 99% | -0.26% | -0.12% | -0.09% | 108% | 100% |
| 1.1.3 | -0.46% | -0.29% | -0.24% | 102% | 98% | -0.20% | -0.01% | -0.01% | 100% | 100% |
| 1.1.4 | -0.40% | -0.23% | -0.20% | 102% | 98% | -0.18% | -0.03% | 0.02% | 100% | 99% |
| 1.2.1 | -0.29% | -0.24% | -0.19% | 107% | 104% | -0.16% | -0.12% | -0.01% | 102% | 101% |
| 1.2.2 | -0.25% | -0.21% | -0.18% | 106% | 103% | -0.14% | -0.09% | -0.01% | 103% | 101% |
| 1.2.3 | -0.18% | -0.13% | -0.10% | 105% | 103% | -0.10% | -0.05% | -0.06% | 102% | 101% |
| 1.2.4 | -0.19% | -0.16% | -0.11% | 105% | 104% | -0.11% | -0.08% | 0.00% | 102% | 101% |
| 1.2.5 | -0.18% | -0.13% | -0.10% | 105% | 103% | -0.10% | -0.05% | -0.06% | 100% | 100% |

### CE3.1: Additional test results

When 4-tap intra interpolation filter (as being studied in CE3.3) is enabled, it is noticed that MRL provides additive or even extra gain by applying 4-tap Cubic interpolation filter (IF) on nonzero reference lines (i.e., mrl_idx > 0). The following additional tests have been performed combining CE3.1 test 1.1.3 with the 4-tap interpolation filters in BMS and CE3.3 tests 3.1.2 and 3.2.1. For each combination test, the 4-tap Cubic IF is applied to nonzero reference lines.

| Test | Method description |
| --- | --- |
| C1 | VTM-2.0.1 + CE3-1.1.3 + BMS-2.0.1 4-Tap IF (--Intra4Tap=1) |
| C2 | VTM-2.0.1 + CE3-1.1.3 + CE3-3.1.2 |
| C3 | VTM-2.0.1 + CE3-1.1.3 + CE3-3.2.1 |

|  | All Intra Main10 |  |  |  |  | Random Access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE3-1.1.3 | -0.46% | -0.29% | -0.24% | 102% | 98% | -0.20% | -0.01% | -0.01% | 100% | 100% |
| BMS 4-tap | -0.36% | -0.39% | -0.41% | 101% | 99% | -0.16% | -0.31% | -0.30% |  |  |
| C1 | -0.90% | -0.76% | -0.78% | 104% | 98% | -0.40% | -0.31% | -0.32% | 100% | 98% |
| CE3-3.1.2 | -0.46% | -0.58% | -0.61% | 103% | 102% | -0.19% | -0.14% | -0.13% | 103% | 103% |
| C2 | -0.95% | -0.88% | -0.87% | 103% | 98% | -0.41% | -0.18% | -0.15% | 100% | 98% |
| CE3-3.2.1 | -0.59% | -0.68% | -0.69% | 102% | 101% | -0.26% | -0.43% | -0.45% | 100% | 100% |
| C3 | -1.08% | -0.99% | -0.97% | 105% | 102% | -0.49% | -0.46% | -0.45% | 101% | 100% |

Detailed results can be found in JVET-L0283.

### CE3.1: Cross-check reports

| Test# | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 1.1.1 | Elie Mora (Ateme) | y | As I mention in the crosscheck report, our results show that there is no encoder runtime gain coming from excluding testing of odd modes on additional reference lines. However, this exclusion does bring losses as the MRL gains reduce from 0.61 to 0.47 in all intra. It seems straightforward to not exclude odd modes from encoder intra search on additional reference lines |
| 1.1.2 | Elie Mora (Ateme) | y | As I mention in the crosscheck report, our results show that there is no encoder runtime gain coming from excluding testing of odd modes on additional reference lines. However, this exclusion does bring losses as the MRL gains reduce from 0.61 to 0.47 in all intra. It seems straightforward to not exclude odd modes from encoder intra search on additional reference lines |
| 1.1.3 | Alexey Filippov (Huawei) | y | There is minor PSNR difference caused by rounding error. |
| 1.1.4 | Kiho Choi (Samsung) | y |  |
| 1.2.1 | Mohammed G. Sarwer (MediaTek) | y | There is very minor PSNR differences after 2 decimals. Class F not cross-checked. |
| 1.2.2 | Mohammed G. Sarwer (MediaTek) | y | There is very minor PSNR differences after 2 decimals. Class F not cross-checked. |
| 1.2.3 | Fabien Racapé (Technicolor) | y | The bdrates results are exact; the execution times are within the measurement error range. |
| 1.2.4 | Fabien Racapé (Technicolor) | y | The bdrates results are exact; the execution times are within the measurement error range. |
| 1.2.5 | Jinho Lee (ETRI) | y | Since I mentioned that the meaning of ‘Angular Modes’ was except H/V modes, results of CE3-1.2.3 are same as CE3-1.2.5. I think CE3-1.2.3 needs to be withdrawn, and ‘Selected’ would be removed if H/V modes are excepted in CE3-1.2.1 and CE3-1.2.2. |

## CE3.1: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-L0515 (ITRI) | 1.1 | CE3-related: Non-zero reference lines padding method on the top-line of CTU |

# CE3.2 on ‘Intra prediction modes’

## Defined tests [1]

| Test # | Description | Doc. # |
| --- | --- | --- |
| 2.1.1 | ‘Line-based’ intra coding mode with a restricted number of partitions per block size (with at least 16 samples per partition; ISP: intra sub-partitions tool) | JVET-L0076 (HHI) |
| 2.1.2 | Test 2.1.1 with a restriction: the resulting partitions must have a width of at least 4 samples |  |
| 2.2.1 | Non-linear weighted intra prediction with inverse DCT after prediction (residual added in spatial domain) + adapted MPM list | JVET-L0199 (HHI) |
| 2.2.2 | Non-linear weighted intra prediction with modified structure of the predictors (affine intra predictions, simplifications) |  |
| 2.3.1 | Only use DM and LM modes for 2xN or Nx2 chroma blocks | JVET-L0277 (Tencent) |
| 2.3.2 | Only use DM and LM modes for all chroma blocks |  |
| 2.4.1 | Enable chroma multiple direct mode signalling (MDMS) | JVET-L0420 (ITRI) |
| 2.4.2 | MDMS + fast encoder search |  |
| 2.5.1 | Proposed right-column and bottom-row prediction method for planar mode | JVET-L0084 (MediaTek) |
| 2.6.1 | Withdrawn |  |

## CE3.2: Test results

### CE3.2: VTM-2.0.1 anchor

|  |  | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.1.1 | ‘Line-based’ intra coding mode with a restricted number of partitions per block size (with at least 16 samples per partition) | -1.01% | -0.71% | -0.69% | 148% | 103% | -0.49% | -0.49% | -0.35% | 113% | 100% |
| 2.1.2 | Test 2.1.1 with a restriction: the resulting partitions must have a width of at least 4 samples | -0.82% | -0.58% | -0.56% | 143% | 103% | -0.46% | -0.42% | -0.29% | 112% | 101% |
| 2.2.1 | Non-linear weighted intra prediction with inverse DCT after prediction (residual added in spatial domain) | -2.46% | -1.96% | -1.94% | 264% | 127% | -1.25% | -1.16% | -1.21% | 130% | 106% |
| 2.2.2 | Non-linear weighted intra prediction with modified structure of the predictors | -1.63% | -0.96% | -0.95% | 275% | 113% | -0.90% | -0.80% | -0.85% | 137% | 105% |
| 2.3.1 | Only use DM and LM modes for 2xN or Nx2 chroma blocks | 0.00% | 0.23% | 0.32% | 95% | 95% | 0.02% | 0.18% | 0.32% | 94% | 94% |
| 2.3.2 | Only use DM and LM modes for all chroma blocks | 0.08% | 1.25% | 1.46% | 91% | 95% | 0.03% | 0.81% | 0.95% | 92% | 94% |
| 2.4.1 | Enable MDMS | -0.20% | -1.09% | -1.08% | 99% | 100% | -0.02% | -0.77% | -0.84% | 100% | 100% |
| 2.4.2 | MDMS + fast encoder search | -0.20% | -0.84% | -0.84% | 97% | 100% | 0.06% | -0.69% | -0.76% | 98% | 100% |
| 2.5.1 | Proposed right-column and bottom-row prediction method for planar mode | -0.07% | -0.04% | 0.02% | 100% | 98% | -0.01% | -0.01% | 0.08% | 100% | 98% |

### CE3.2: Additional test results

|  |  | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 2.1.1.1 | Test 2.1.1 with a single context to code the split flag | -0.99% | -0.68% | -0.67% | 148% | 104% | -0.49% | -0.46% | -0.28% | 113% | 101% |
| 2.1.2.1 | Test 2.1.2 with a single context to code the split flag | -0.80% | -0.57% | -0.54% | 143% | 103% | -0.43% | -0.42% | -0.37% | 112% | 100% |
| 2.1.1.2 | Test 2.1.1 combined with Test 3.2.1 (interpolation, bilateral) and Test 1.1.3 (MRL) | -1.93% | -1.55% | -1.53% | 152% | 102% | -0.92% | -0.86% | -0.88% | 114% | 99% |
| 2.1.1.3 | Test 2.1.1 with a different trade-off between encoder (only) run-time and gain | -0.82% | -0.55% | -0.61% | 128% | 101% | -0.39% | -0.28% | -0.19% | 108% | 97% |
| 2.1.1.4 | Test 2.1.1 with clipping only after the whole block has been reconstructed | -1.01% | -0.72% | -0.70% | 148% | 104% | -0.49% | -0.50% | -0.33% | 113% | 100% |

### CE3.2: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 2.1.1 | Liang Zhao (Tencent) | y |  |
| 2.1.2 | Liang Zhao (Tencent) | y |  |
| 2.2.1 | Fabien Racape (Technicolor) | y | Bd-rate results are exact; the execution times are within the measurement error range. |
|  | Sergey Ikonin (Huawei) | y | BD-rate results exactly match, the running times have slightly variations within the measurement error range |
| 2.2.2 | Fabien Racape (Technicolor) | y | Bd-rate results are exact; the execution times are within the measurement error range. |
|  | Sergey Ikonin (Huawei) | y | BD-rate results exactly match, the running times have slightly variations within the measurement error range |
| 2.3.1 | Santiago de Luxán Hernández (HHI) | y | The BD-Rate results match. However, there is approximately a 5% increase in both encoder and decoder run-times, probably due to different hardware configurations used to generate the results. It seems that the cross-check run-times are more reliable, given that they present a decoding time run-time not below 100% and the proposed should not reduce this run-time, since it only decreases the number of tests performed at the encoder side. |
| 2.3.2 | Santiago de Luxán Hernández (HHI) | y | The BD-Rate results match. However, there is approximately a 5% increase in both encoder and decoder run-times, probably due to different hardware configurations used to generate the results. It seems that the cross-check run-times are more reliable, given that they present a decoding time run-time not below 100% and the proposed should not reduce this run-time, since it only decreases the number of tests performed at the encoder side. |
| 2.4.1 | Xiang Ma (Huawei) | y | Only 0.0001 difference in PSNR in some points at RA config. |
| 2.4.2 | Xiang Ma (Huawei) | y | Only 0.0001 difference in PSNR in some points at RA config. |
| 2.5.1 | Steve Keating (Sony) | y | The results match those in JVET-L0084-v1 within 2 decimal places, run times are also a very close match. |

## CE3.2: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-L0053 |  | CE3-related: Chroma DM modification |
| JVET-L0067 |  | CE3-related: Modified chroma DM |
| JVET-L0087 |  | CE3-related: Boundary PDPC |
| JVET-L0139 | 2.4.x | CE3-related: Simplified MDMS |
| JVET-L0152 |  | CE3-related: Simplification of PDPC |
| JVET-L0164 |  | CE3-related: Decoder-side Intra Mode Derivation |
| JVET-L0204 |  | CE3-related: Disabling PDPC based on availability of reference samples |
| JVET-L0272 |  | CE3-related: Modified chroma derived mode |
| JVET-L0279 |  | CE3-related: Unification of angular intra prediction for square and non-square blocks |
| JVET-L0381 |  | CE3-related: 4-tap interpolation filter selection with quantization parameter |

# CE3.3 on ‘Intra reference sample interpolation’

## Defined tests [1]

| Test # | Description | Doc. # |
| --- | --- | --- |
| 3.1.1 | Interpolation filter selection between 4-tap cubic and 4-tap Gaussian filter based on intra prediction mode and block size (JVET-J0017) | JVET-L0130 (LGE) |
| 3.1.2 | Interpolation filter selection between 4-tap cubic and 4-tap Gaussian filter with MDIS conditions (JVET-K0064) | JVET-L0324 (Qualcomm) |
| 3.1.3 | Interpolation filter selection between 4-tap cubic and 6-tap Gaussian (convolution of [1 2 1]/4 smoothing filter and 4-tap Gaussian) with MDIS conditions (JVET-K0165) | JVET-L0151 (ETRI) |
| 3.1.4 | Harmonization of shape-, size- and mode-dependent selection of 4-tap interpolation filters (JVET-K0518) with simplified PDPC and wide-angle intra-prediction | JVET-L0275 (Huawei) |
| 3.2.1 | Bilateral reference sample filter + 4-tap cubic interpolation filter | JVET-L0179 (HHI) |
| 3.2.2 | Bilateral reference sample filter + 4-tap cubic interpolation filter + 4-tap Gaussian interpolation filter |  |
| 3.3.1 | Multiple 4-tap filter | JVET-L0052 (Samsung) |

## CE3.3: Test results

### CE3.3: VTM-2.0.1 anchor

| Test # | Description | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3.1.1 | Interpolation filter selection between 4-tap cubic and 4-tap Gaussian filter based on intra prediction mode and block size (JVET-J0017) | -0.41% | -0.44% | -0.42% | 101% | 101% | -0.18% | -0.11% | -0.07% | 100% | 100% |
| 3.1.2 | Interpolation filter selection between 4-tap cubic and 4-tap Gaussian filter with MDIS conditions (JVET-K0064) | -0.46% | -0.58% | -0.61% | 103% | 102% | -0.19% | -0.14% | -0.13% | 103% | 103% |
| 3.1.3 | Interpolation filter selection between 4-tap cubic and 6-tap Gaussian (convolution of [1 2 1]/4 smoothing filter and 4-tap Gaussian) with MDIS conditions (JVET-K0165) | -0.44% | -0.61% | -0.70% | 103% | 101% | -0.18% | -0.47% | -0.47% | 101% | 100% |
| 3.1.4 | Harmonization of shape-, size- and mode-dependent selection of 4-tap interpolation filters (JVET-K0518) with simplified PDPC and wide-angle intra-prediction | -0.45% | -0.57% | -0.57% | 102% | 103% | -0.17% | -0.13% | -0.07% | 100% | 101% |
| 3.2.1 | Bilateral reference sample filter + 4-tap cubic interpolation filter | -0.59% | -0.68% | -0.69% | 102% | 101% | -0.26% | -0.43% | -0.45% | 100% | 100% |
| 3.2.2 | Bilateral reference sample filter + 4-tap cubic interpolation filter + 4-tap Gaussian interpolation filter | -0.60% | -0.58% | -0.61% | 104% | 102% | -0.31% | -0.51% | -0.45% | 101% | 100% |
| 3.3.1 | Multiple 4-tap filter | -0.39% | -0.59% | -0.59% | 105% | 101% | -0.16% | -0.39% | -0.28% | 101% | 100% |

### CE3.3: Additional test results

| Test # | Description | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3.1.2.1 | 3.1.2 + reduced bits/filter tap | -0.45% | -0.60% | -0.62% | 102% | 102% | -0.17% | -0.09% | -0.11% | 102% | 102% |
| 3.1.2.2 | 3.1.2.1 versus anchor with VTM2 intra smoothing tool disabled | -0.66% | -0.73% | -0.75% | 104% | 100% | -0.35% | -0.39% | -0.39% | 101% | 99% |
| 3.1.2.3 | 3.1.2.1 + reduced bits/filter tap, 16-phase filters | -0.41% | -0.57% | -0.57% | 102% | 102% | -0.14% | -0.16% | -0.06% | 102% | 103% |
| 3.1.2.4 | 3.1.2.3 versus anchor with VTM2 intra smoothing tool disabled | -0.62% | -0.71% | -0.70% | 104% | 99% | -0.32% | -0.46% | -0.35% | 101% | 99% |
| 3.1.4.1 | 3.1.4 with unmodified MDIS ref. sample filtering conditions | -0.45% | -0.54% | -0.57% | 102% | 103% | -0.18% | -0.09% | -0.08% | 101% | 103% |
| 3.1.4.2 | Combined test 3.1.4.1 + 3.1.2.1 | -0.45% | -0.56% | -0.55% | 102% | 103% | -0.18% | -0.09% | -0.08% | 101% | 103% |
| 3.2.1.1 | 3.2.1 with cubic interpolation from test 3.1.2 (conditions, padding) | -0.66% | -0.64% | -0.64% | 101% | 101% | -0.28% | -0.18% | -0.16% | 100% | 99% |

| Test # | Description | All Intra Main10 - Over BMS-2.1 |  |  |  |  | Random Access Main10 - Over BMS-2.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 3.1.2.5 | 3.1.2 + reduced bits/filter tap (BMS2.1) | -0.42% | -0.54% | -0.55% | 101% | 101% | -0.16% | -0.20% | -0.13% | 100% | 100% |

### Analysis of test results

| Test # | Luma ref. sample interpolation (angular modes excluding 2, VDIA, DIA) | Chroma ref. sample interpolation chroma (angular modes excluding 2, VDIA, DIA) | Intra luma ref. sample filtering (smoothing) | intraHorVerDistThres table (draft spec) | Compression performance (Y/U/V BD-rates) for AI configuration |
| --- | --- | --- | --- | --- | --- |
| VTM2 | Linear (2-tap, 32-phase) | Linear (2-tap, 32-phase) | [1 2 1] / 4 Condition: VTM2 ref. sample filtering | {20, 14, 2, 0, 20, 0} | anchor |
| 3.1.1 | Cubic (4-tap, 32-phase, 9bit) Gaussian (4-tap, 32-phase, 7bit) Switching conditions: (W ≤ 8 ¦¦ (absAng ≤ 11 && W*H ≤ 64) if vertical angular mode (H ≤ 8 ¦¦ (absAng ≤ 11 && W*H ≤ 64) if horizontal angular mode | Linear (2-tap, 32-phase) | [1 2 1] / 4 Condition: VTM2 ref. sample filtering | {20, 14, 2, 0, 20, 0} | -0.41%/-0.44%/-0.42% |
| 3.1.2 | Cubic (4-tap, 32-phase, 9bit) Gaussian (4-tap, 32-phase, 7bit) Switching conditions: idem VTM2 ref. sample filtering | Linear (2-tap, 32-phase) | [1 2 1] / 4 for PL only (VTM2 ref. sample filtering condition) | {20, 14, 2, 0, 0, 0} | -0.46%/-0.58%/-0.61% |
| 3.1.2.1 | Cubic (4-tap, 32-phase, 7bit) Gaussian (4-tap, 32-phase, 5bit) Switching conditions: idem VTM2 ref. sample filtering | Linear (2-tap, 32-phase) | [1 2 1] / 4 for PL only (VTM2 ref. sample filtering condition) | {20, 14, 2, 0, 0, 0} | -0.45%/-0.60%/-0.62% |
| 3.1.2.3 | Cubic (4-tap, 16-phase, 7bit) Gaussian (4-tap, 16-phase, 5bit) Switching conditions: idem VTM2 ref. sample filtering | Linear (2-tap, 32-phase) | [1 2 1] / 4 for PL only (VTM2 ref. sample filtering condition) | {20, 14, 2, 0, 0, 0} | -0.41%/-0.57%/-0.57% |
| 3.1.3 | Cubic (4-tap, 32-phase, 9bit) Gaussian (6-tap, 32-phase, 9bit) Switching conditions: similar VTM2 ref. sample filtering excluding wide-angle and PL conditions | Idem luma | No [1 2 1] / 4 Filtered ref. sample array removed from code | {30, 14, 2, 0, 0, 0} | -0.44%/-0.61%/-0.70% |
| 3.1.4 | DCT-IF MC chroma filter (4-tap, 32-phase, 7bit, idem inter) Gaussian (4-tap, 32-phase, 5bit) Switching conditions: VTM2 ref. sample filtering + rectangular block conditions (incl. table with 4 elements) | Linear (2-tap, 32-phase) | [1 2 1] / 4 for PL, modes 2, VDIA, DIA (require no ref. sample interpolation) | {20,14, 2, 0, 0, 0} | -0.45%/-0.57%/-0.57% |
| 3.1.4.1 | DCT-IF MC chroma filter (4-tap, 32-phase, 7bit, idem inter) Gaussian (4-tap, 32-phase, 5bit) Switching conditions: VTM2 ref. sample filtering) | Linear (2-tap, 32-phase) | [1 2 1] / 4 for PL, modes 2, VDIA, DIA (require no ref. sample interpolation) | {20,14, 2, 0, 0, 0} | -0.45%/-0.54%/-0.57% |
| 3.1.4.2 | DCT-IF MC chroma filter (4-tap, 32-phase, 7bit, idem inter) Gaussian 3.1.2.1 (4-tap, 32-phase, 5bit) Switching conditions: VTM2 ref. sample filtering) | Linear (2-tap, 32-phase) | [1 2 1] / 4 for PL, modes 2, VDIA, DIA (require no ref. sample interpolation) | {20,14, 2, 0, 0, 0} | -0.45%/-0.56%/-0.55% |
| 3.2.1 | Cubic (4-tap, 32-phase, 9bit) Linear (2-tap, 32-phase) Switching condition: W ≤ 8 if vertical angular mode H ≤ 8 if horizontal angular mode | Idem luma | Bilateral filter Condition: WxH ≥16x16 [1 2 1] / 4 Condition: VTM2 ref. sample filtering | {20, 14, 2, 0, 20, 0} | -0.59%/-0.68%/-0.69% |
| 3.2.2 | Cubic (4-tap, 32-phase, 9bit) Gaussian (4-tap, 32-phase, 7bit) Switching conditions: W ≤ 8 if vertical angular mode H ≤ 8 if horizontal angular mode | Idem luma | Bilateral filter Condition: WxH ≥16x16 [1 2 1] / 4 Condition: VTM2 ref. sample filtering | {20, 14, 2, 0, 20, 0} | -0.60%/-0.58%/-0.61% |
| 3.2.1.1 | Cubic (4-tap, 32-phase, 9bit) Linear (2-tap, 32-phase) Switching condition: Unfiltered ref. samples (bilateral + VTM2 ref. filtering) | Linear (2-tap, 32-phase) | Bilateral filter Condition: WxH ≥16x16 [1 2 1] / 4 Condition: VTM2 ref. sample filtering | {20, 14, 2, 0, 0, 0} | -0.66%/-0.64%/-0.64% |
| 3.3 | Cubic pair: Cubic and Cubic-wise smoothing filter (4-tap, 32-phase, 9bit) Gaussian pair: Gaussian and Gaussian-wise smoothing filter (4-tap, 32-phase, 9bit) Switching conditions: Filter decision inside the pair: (x<8 ¦¦ y<8) Pair decision: W ≤ 16 && H ≤ 32 if vertical angular mode H ≤ 16 && W ≤ 32 if horizontal angular mode | Idem luma | [1 2 1] / 4 Condition: VTM2 ref. sample filtering | {20, 14, 2, 0, 20, 0} | -0.39%/-0.59%/-0.59% |

### CE3.3: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 3.1.1 | Geert Van der Auwera (Qualcomm) | y | The cross-checker found slightly higher (~2%) encoding and decoding times for the RA condition. The proposal utilizes the JEM7 cubic and Gaussian filters (Intra4Tap tool), but changes the JEM7 condition to switch between both filters by checking (width <= 8 ¦¦ (absAng <= 11 && width*height <= 64)). The VTM2 intra smoothing filtering ([1 2 1] + 2) / 4 is enabled, so it may happen that VTM2 filtering is followed by the Gaussian filtering. |
|  | Vasily Rufitskiy (Huawei) | y | Results match, except for RA has minor mismatches caused by difference in PSNR rounding methods used by cross-checker and proponent. |
| 3.1.2 | Virginie Drugeon (Panasonic) | y | The results exactly match the proponent's results for AI conditions and for RA conditions classes B, C and D. Yes, the software was studied and the provided software mostly matches with the description, with the exception of one small bug fix for harmonisation of WAIP and PDPC implementation in IntraPrediction::predIntraAng. The proposed implementation solves bugs that existed in the JEM implementation for 4tap interpolation (specifically issues related to interpolation at the borders of the array of reference samples, and application of 4tap interpolation even when the reference position falls on an integer pixel). Smoothing of reference samples in advance is disabled in IntraPrediction::initIntraPatternChType, but it seems that the buffer for filtered samples is still allocated in IntraPrediction::init, although it is not used in practice. |
|  | Philipp Merkle (HHI) | y | Yes, the software was studied and the provided software matches with the description. It also contains bug fixes for the MDIS condition threshold and for WAIP+PDPC harmonization. The simplification of padding the borders of the reference sample buffer prior to the prediction loop might also be beneficial for other proposals, as it avoids two conditional checks in the prediction loop for every sample. |
|  | Alexey Filippov (Huawei) | y | Results match, except for RA has minor mismatches caused by difference in PSNR rounding methods used by cross-checker and proponent. |
| 3.1.2.1 | Philipp Merkle (HHI) | y |  |
| 3.1.3 | Geert Van der Auwera (Qualcomm) | y | The cross-checker found slightly higher (~2%) decoding times for the AI and RA conditions. The proposed interpolation filtering utilizes the 4-tap, 32-phase cubic filter from JEM7 and a 6-tap, 32-phase Gaussian filter obtained by convolving the 3-tap ([1 2 1] + 2)/4 VTM2 intra smoothing filter with the 4-tap, 32-phase Gaussian interpolation filter from JEM7. The MDIS condition (diff) with modified threshold values is utilized for selecting between both filters. The VTM2 3-tap smoothing filtering is disabled. The proposed interpolation is applied to luma and chroma. |
|  | Vasily Rufitskiy (Huawei) | y | Results match, except for RA has minor mismatches caused by difference in PSNR rounding methods used by cross-checker and proponent. |
| 3.1.4 | Geert Van der Auwera (Qualcomm) | y |  |
|  | Sunmi Yoo (LGE) | y | Match closely for VTM (The difference comes from HexPSNR on/off in RA). They provided us another source code, which is the modification in the MDIS part disabled. The result of the additional one has similar coding efficiency with the original one. |
| 3.1.4.1 | Jonathan Pfaff (HHI) | y |  |
|  | Sunmi Yoo (LGE) | y |  |
| 3.2.1 | Vasily Rufitskiy (Huawei) | y | Results match, except for RA has minor mismatches caused by difference in PSNR rounding methods used by cross-checker and proponent. |
| 3.2.1.1 | Geert Van der Auwera (Qualcomm) | y |  |
| 3.2.2 | Vasily Rufitskiy (Huawei) | y | Results match, except for RA has minor mismatches caused by difference in PSNR rounding methods used by cross-checker and proponent. |
| 3.3.1 | Geert Van der Auwera (Qualcomm) | y | The cross-checker found slightly higher (~2%) decoding times for the AI and RA conditions. The 4-tap, 32-phase cubic and Gaussian filters from the ‘Intra4Tap’ tool (JEM7) are used together with two additional 4-tap, 32-phase filters. Filters are switched depending on the pixel position, intra mode and block size. The proposed interpolation is applied to luma and chroma. |

## CE3.3: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-L0381 (KDDI) | 3 | 4-tap interpolation filter selection with quantization parameter |

# CE3.4 on ‘Bidirectional prediction’

## Defined tests [1]

| Test # | Description | Doc. # |
| --- | --- | --- |
| 4.1.1 | Linear interpolation intra prediction (LIP) | JVET-L0131 (LGE) |
| 4.2.1 | Harmonization of distance-weighted directional intra prediction (DWDIP) with simplified PDPC | JVET-L0284 (Huawei) |
| 4.2.2 | Idem 4.2.1 |  |
| 4.3.1 | Withdrawn |  |

## CE3.4: Test results

### CE3.4: VTM-2.0.1 anchor

| Test # | Description | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 4.1.1 | Linear interpolation intra prediction (LIP) | -0.21% | -0.16% | -0.13% | 105% | 101% | -0.14% | -0.27% | -0.24% | 102% | 101% |
| 4.2.1 | Harmonization of distance-weighted directional intra prediction (DWDIP) with simplified PDPC | -0.04% | -0.04% | -0.03% | 103% | 100% | -0.08% | -0.06% | -0.06% | 101% | 101% |

### CE3.4: Additional test results

NA

### CE3.4: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 4.1.1 | Vasily Rufitskiy (Huawei) | y | Results match, except for RA has minor mismatches caused by difference in PSNR rounding methods used by cross-checker and proponent. |
| 4.2.1, 4.2.2 | Jonathan Pfaff (HHI) | y | I want to report that the two tests 4.2.1 and 4.2.2 are in fact one test, since PDPC is adopted to VTM. This has been confirmed by the testers. There were slight inconsistencies in the Encoder/Decoder runtime, but they could be resolved by checking the runtimes locally. |

## CE3.4: Related contributions

NA

# CE3.5 on ‘Cross-component prediction and separate chroma tree’

## Defined tests [1]

### CCLM-related

| Test# | Short description | Doc. # |
| --- | --- | --- |
| 5.1.1 | Replace the LMS algorithm by a straight-line equation for CCLM mode Luma to Chroma | JVET-L0191 (Canon) |
| 5.2.7 | CCLM + line buffer restriction at top CTU boundary (1 line) | JVET-L0136 (LGE) |
| 5.5.1 | CCLM; using 1 luma line (CU) | JVET-L0339 (Huawei) |
| 5.6.2 | CCLM; using 1 luma line (CU); using simplified method from test 5.1.1 | JVET-L0340 (Huawei) |
| 5.8.1 | If above side of the current CU cross CTU boundary, then only one line of above neighboring luma reconstructed samples is used in LM parameters derivation. | JVET-L0085 (MediaTek) |
| 5.8.2 | If above side of the current CU cross CTU boundary, then above neighboring luma reconstructed samples are not used in LM parameters derivation. |  |

### Cross-component prediction tools

| Test# | Short description | Doc. # |
| --- | --- | --- |
| 5.2.1 | CCLM + CCLM Cb-to-Cr + MMLM + MFLM + LM-Angular | JVET-L0251 (Qualcomm) |
| 5.2.2 | CCLM + CCLM Cb-to-Cr |  |
| 5.2.3 | CCLM + CCLM Cb-to-Cr + MMLM |  |
| 5.2.4 | CCLM + CCLM Cb-to-Cr + MMLM + MFLM |  |
| 5.3.1 | Multiple neighbor-based LM (MNLM): CCLM + CCLM Cb-to-Cr + MMLM (above+left) + MMLM (above) + MMLM (left) + Test 5.8.1 (1 line buffer at CTU boundary) | JVET-L0388 (Foxconn) |
| 5.4.1 | CCLM + MDLM | JVET-L0338 (Huawei) |
| 5.4.2 | CCLM + MDLM with line buffer constraint at CTU boundary |  |
| 5.5.2 | CCLM + MDLM, both using 1 luma line (CU) |  |
| 5.6.1 | CCLM + MDLM; using simplified method from test 5.1.1 | JVET-L0340 (Huawei) |
| 5.6.3 | CCLM + MDLM; both using 1 luma line (CU); both using simplified method from test 5.1.1 |  |
| 5.7.2 | Adaptive inter-residual prediction with fast RDO | JVET-L0378 (KDDI) |
| 5.9.1 | Adaptive Grouping LM | JVET-L0419 (ITRI) |

## CE3.5: Test results

### CE3.5: VTM-2.0.1 anchor

#### CCLM-related

|  |  | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 5.1.1 | Replace the LMS algorithm by a straight-line equation for CCLM mode Luma to Chroma | 0.11% | 0.48% | -0.02% | 100% | 100% | 0.08% | 0.78% | 0.19% | 101% | 97% |
| 5.2.7 | CCLM + line buffer restriction at top CTU boundary (1 line) | 0.01% | 0.01% | 0.04% | 100% | 100% | 0.01% | 0.08% | 0.02% | 100% | 100% |
| 5.5.1 | CCLM; using 1 luma line (CU) | 0.02% | 0.24% | 0.23% | 100% | 98% | 0.02% | 0.32% | 0.35% | 100% | 100% |
| 5.6.2 | CCLM; using 1 luma line (CU); using simplified method from test 5.1.1 | 0.13% | 0.77% | 0.25% | 100% | 98% | 0.08% | 1.13% | 0.44% | 99% | 99% |
| 5.8.1 | If above side of the current CU cross CTU boundary, then only one line of above neighboring luma reconstructed samples is used in LM parameters derivation. | 0.01% | 0.01% | 0.04% | 100% | 101% | 0.01% | 0.08% | 0.02% | 100% | 102% |
| 5.8.2 | If above side of the current CU cross CTU boundary, then above neighboring luma reconstructed samples are not used in LM parameters derivation. | 0.13% | 0.65% | 0.67% | 100% | 101% | 0.08% | 0.64% | 0.68% | 100% | 101% |

#### Cross-component prediction tools

|  |  | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 5.2.1 | CCLM + CCLM Cb-to-Cr + MMLM + MFLM + LM-Angular | -0.59% | -6.66% | -6.02% | 111% | 106% | -0.25% | -5.49% | -5.01% | 107% | 103% |
| 5.2.2 | CCLM + CCLM Cb-to-Cr | -0.17% | -2.26% | -1.32% | 100% | 102% | -0.04% | -1.75% | -1.16% | 101% | 102% |
| 5.2.3 | CCLM + CCLM Cb-to-Cr + MMLM | -0.49% | -4.52% | -4.29% | 102% | 102% | -0.22% | -3.74% | -3.74% | 103% | 103% |
| 5.2.4 | CCLM + CCLM Cb-to-Cr + MMLM + MFLM | -0.52% | -5.50% | -4.98% | 105% | 106% | -0.23% | -4.69% | -4.29% | 104% | 103% |
| 5.3.1 | MNLM: CCLM + CCLM Cb-to-Cr + MMLM + Above-MMLM + Left-MMLM + Test 5.8.1 (1 line buffer at CTU boundary) | -0.59% | -6.30% | -6.48% | 103% | 102% | -0.30% | -5.49% | -5.82% | 105% | 103% |
| 5.4.1 | CCLM + MDLM | -0.06% | -2.71% | -3.13% | 101% | 99% | -0.03% | -2.65% | -2.84% | 100% | 99% |
| 5.4.2 | CCLM + MDLM with line buffer constraint at CTU boundary | 0.07% | -1.52% | -1.81% | 101% | 99% | 0.04% | -1.55% | -1.77% | 100% | 99% |
| 5.5.2 | CCLM + MDLM, both using 1 luma line (CU) | -0.04% | -2.62% | -2.98% | 101% | 99% | -0.02% | -2.45% | -2.69% | 101% | 101% |
| 5.6.1 | CCLM + MDLM; using simplified method from test 5.1.1 | 0.03% | -2.12% | -2.87% | 101% | 99% | 0.02% | -1.88% | -2.66% | 100% | 98% |
| 5.6.3 | CCLM + MDLM; both using 1 luma line (CU); both using simplified method from test 5.1.1 | 0.05% | -1.88% | -2.67% | 101% | 99% | 0.03% | -1.59% | -2.38% | 101% | 100% |
| 5.7.2 | Adaptive inter-residual prediction with fast RDO | -0.14% | -2.41% | -2.67% | 101% | 100% | -0.02% | -2.25% | -2.11% | 101% | 100% |
| 5.9.1 | Adaptive Grouping LM | -0.14% | -1.48% | -1.76% | 102% | 102% | -0.07% | -1.21% | -1.42% | 103% | 101% |

### CE3.5: Additional test results

|  |  | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 5.2.3.1 | CCLM + MMLM (4 lines) | -0.33% | -2.87% | -2.72% | 101% | 100% | -0.17% | -2.55% | -2.20% | 99% | 97% |
| 5.2.3.2 | CCLM + MMLM (2 lines only) | -0.29% | -2.55% | -2.35% | 102% | 99% | -0.13% | -2.24% | -1.77% | 100% | 98% |
| 5.2.3.3 | CCLM + MMLM (2 lines only, disabled at CTU top) | -0.24% | -2.45% | -2.28% | 102% | 102% | -0.11% | -2.18% | -1.70% | 100% | 98% |
| 5.2.3.4 | CCLM + MMLM (2 lines only, 1line at CTU top) | -0.29% | -2.52% | -2.36% | 101% | 99% | -0.12% | -2.25% | -1.79% | 100% | 99% |
| 5.3.1.1 | MNLM w/o CTU boundary line buffer restriction: CCLM + CCLM Cb-to-Cr + MMLM + Above-MMLM + Left-MMLM | -0.60% | -6.33% | -6.54% | 103% | 102% | -0.30% | -5.56% | -5.90% | 103% | 102% |

### CE3.5: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 5.1.1 | Adarsh Ramasubramonian (Qualcomm) | y | No |
| 5.2.1 | Jangwon Choi (LGE) | y |  |
| 5.2.2 | D. Y. Kim (Chips&Media) | y | In terms of encoding and decoding time, only Class C and D sequences in AI configuration are reliable in the attached excel sheet. |
| 5.2.3 | Guillaume Laroche (Canon) | y | No |
| 5.2.4 | Guillaume Laroche (Canon) | y | No |
| 5.2.7 | Xiang Ma (Huawei) |  |  |
| 5.3.1 | Adarsh Ramasubramonian (Qualcomm) | y |  |
| 5.4.1 | Yoshitaka Kidani (KDDI) | y |  |
|  | Jonathan Pfaff (HHI) |  |  |
| 5.4.2 | Yoshitaka Kidani (KDDI) | y |  |
| 5.5.1 | Po-Han Lin (ITRI) |  |  |
| 5.5.2 | Po-Han Lin (ITRI) |  |  |
| 5.6.1 | Po-Han Lin (ITRI) |  |  |
| 5.6.2 | Chia-Ming Tsai (MediaTek) | y |  |
| 5.6.3 | Jangwon Choi (LGE) | y |  |
| 5.7.2 | Xiang Ma (Huawei) |  |  |
| 5.8.1 | Jangwon Choi (LGE) | y | No |
| 5.8.2 | Jangwon Choi (LGE) | y | No |
| 5.9.1 | Xiang Ma (Huawei) |  |  |

## CE3.5: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-L0065 (KWU) | JVET-L0136, JVET-L0339, JVET-L0340, JVET-L0085 | CE3-related: One-line CCLM for reduction of reference sample lines |
| JVET-L0066 (KWU) |  | CE3-related: One-line MMLM for reduction of reference sample lines |
| JVET-L0138 (LGE) |  | CE3-related: Reduced number of reference samples for CCLM parameter calculation |
| JVET-L0239 (InterDigital) |  | CE3-related: Enabling different chroma sample location types in CCLM |
| JVET-L0329 (Bytedance) | JVET-L0136, JVET-L0339, JVET-L0340, JVET-L0085 | CE3-related: CCLM prediction with single-line neighbouring luma samples |
| JVET-L0341 (Huawei) |  | CE3-related: CCLM coefficients derivation method without down-sampling operation |
| JVET-L0342 (Huawei) |  | CE3-related: Classification-based mean value for CCLM coefficients derivation |

# CE3.6 on ‘Intra mode coding’

## Defined tests [1]

| Test # | Description | Doc. # |
| --- | --- | --- |
| 6.1.1 | 6 MPM (5 neighbors; order of insertion is the same as in BMS 1.0) with intra mode dependent contexts for coding MPM index; truncated binary code for non-MPM; CTU-row constraint | JVET-L0250 (Qualcomm) |
| 6.2.1 | Extended number of MPM rather than 3 | JVET-L0165 (LGE) |
| 6.3.1 | Add additional intra modes in the MPM list and use truncated binarization (TB) code for signaling non-MPM modes (“reduced computational complexity” version) | JVET-L0219 (Huawei) |
| 6.3.2 | Add additional intra modes in the MPM list and use truncated binarization (TB) code for signaling non-MPM modes. (“Improved BD-Rate gain” version) | JVET-L0220 (Huawei) |
| 6.4.1 | More than 3 MPMs with bypass coded bin, non-MPM FLC | JVET-L0086 (MediaTek) |
| 6.4.2 | More than 3 MPMs with bypass coded bin, CTU-row constraint, non-MPM FLC coding |  |
| 6.5.1 | 6 MPM (5 neighbors; order of insertion is the same as in BMS 1.0) with intra mode independent contexts for coding MPM index; truncated binarization to code the non-MPM modes; CTU-row constraint | JVET-L0221 (Huawei, MediaTek, LGE, Qualcomm) |

## CE3.6: Test results

### CE3.6: VTM-2.0.1 anchor

|  |  | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 6.1.1 | 6 MPM (5 neighbors; order of insertion is the same as in BMS 1.0) with intra mode dependent contexts for coding MPM index; truncated binary code for non-MPM; CTU-row constraint | -0.42% | -0.40% | -0.35% | 102% | 102% | -0.17% | -0.13% | -0.16% | 100% | 99% |
| 6.2.1 | Extended number of MPM rather than 3 | -0.29% | -0.24% | -0.21% | 100% | 100% | -0.11% | -0.05% | 0.01% | 100% | 100% |
| 6.3.1 | Add additional intra modes in the MPM list and use truncated binarization (TB) code for signaling non-MPM modes (“reduced computational complexity” version) | -0.31% | -0.25% | -0.24% | 100% | 99% | -0.12% | -0.11% | -0.07% | 101% | 101% |
| 6.3.2 | Add additional intra modes in the MPM list and use truncated binarization (TB) code for signaling non-MPM modes. (“Improved BD-Rate gain” version) | -0.34% | -0.28% | -0.25% | 100% | 99% | -0.13% | -0.04% | -0.03% | 101% | 101% |
| 6.4.1 | More than 3 MPMs with bypass coded bin, non-MPM FLC | -0.33% | -0.30% | -0.27% | 100% | 98% | -0.12% | -0.05% | 0.01% | 100% | 98% |
| 6.4.2 | More than 3 MPMs with bypass coded bin, CTU-row constraint, non-MPM FLC coding | -0.29% | -0.24% | -0.21% | 100% | 99% | -0.10% | -0.04% | -0.06% | 100% | 98% |
| 6.5.1 | 6 MPM (5 neighbors; order of insertion is the same as in BMS 1.0) with intra mode independent contexts for coding MPM index; truncated binarization to code the non-MPM modes; CTU-row constraint | -0.35% | -0.34% | -0.29% | 103% | 99% | -0.13% | -0.17% | -0.07% | 101% | 99% |

### CE3.6: Additional test results

| Combined test of CE3.6 |  | JVET-L0222 (Huawei, MediaTek, LGE, Qualcomm) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 6.1, 6.2, 6.3, 6.4, 6.5 | Combined proposal of CE3.6 | -0.32% | -0.26% | -0.24% | 101% | 99% | -0.13% | -0.09% | -0.09% | 101% | 101% |

|  |  | All Intra Main10 - Over VTM-2.0.1 |  |  |  |  | Random Access Main10 - Over VTM-2.0.1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test# | Description | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| 6.4.1.1 | More than 3 MPMs with bypass coded bin, non-MPM Truncated Binary | -0.36% | -0.31% | -0.30% | 100% | 101% | -0.14% | -0.11% | -0.08% | 100% | 101% |
| 6.4.2.1 | More than 3 MPMs with bypass coded bin, CTU-row constraint, non-MPM Truncated Binary | -0.32% | -0.26% | -0.22% | 100% | 101% | -0.13% | 0.00% | -0.01% | 100% | 101% |

### Analysis of test results

|  | Max number of neighbors to access | Line buffer required? | Max layers of if conditions | Max number of comparison operator | Max number of logical operators | Max number of assignment operators | Max number of increments | Max number of bit operation | Parsing dependency? | Number of Context modeling for MPM coding | number of full RDO checks | Has LUT? | LUT size | Number of condition check for remaining modes | Non-MPM coding |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| VTM2 3MPM | 2 | N | 2 | 5 | 4 | 15 | 0 | 0 | N | 1 | 1 or 2 | N | - | 0 | 6-bit FLC |
| 6.1.1 | 5 | N | 4 | 33 | 11 | 35 | 6 | 0 | Y | 4 | 1 or 2 | Y | 68 | 1 | TB |
| 6.2.1a | 2 | N | 3 | 6 | 8 | 26 | 0 | 0 | N | 1 | 1 or 2 | N | - | 1 | TB |
| 6.3.1 | 2 | N | 4 | 9 | 5 | 33 | 4 | 2 | N | 0 | 1 or 2 | N | - | 1 | TB |
| 6.3.2 | 2 | N | 3 | 23 | 4 | 46 | 18 | 2 | N | 0 | 1 or 2 | Y | 68 | 1 | TB |
| 6.4.1 | 2 | Y | 3 | 8 | 6 | 27 | 4 | 3 | N | 1 | 1 or 2 | N | - | 0 | 6-bit FLC |
| 6.4.2 | 2 | N | 3 | 8 | 5 | 27 | 4 | 3 | N | 1 | 1 or 2 | N | - | 0 | 6-bit FLC |
| 6.5.1 | 5 | N | 4 | 33 | 11 | 35 | 6 | 0 | N | 4 | 1 or 2 | Y | 68 | 1 | TB |
| Combo | 2 | N | 3 | 8 | 5 | 27 | 4 | 0 | N | 1 | 1 or 2 | N | - | 1 | TB |

### CE3.6: Cross-check reports

| Test # | Cross-checker | Results match (y/n) | Cross checker comment |
| --- | --- | --- | --- |
| 6.1.1 | Chris Rosewarne (Canon) | y | Results were an exact match for AI and RA conditions - BMS with VTM configs tested only. Runtimes not reliable but seem ok. |
| 6.2.1 | Adarsh Ramasubramonian (Qualcomm) | y | Although only one test is defined, the proponents have tested with and without CTU-row constraints enabled and have therefore two sets of results. Both have been cross-checked and identified as v1 (with CTU-row constraint) and v2 (without CTU-row constraint) in the attached excel sheets. With CTU-row constraints enabled, gain is slightly lower than otherwise. There is small difference in the RD checks compared to anchor. In the anchor, two variables leftDir and aboveDir are initialized to DC, whereas it is initialized to PLANAR in this test. Initializing to PLANAR may have a small benefit for performance. |
| 6.3.1 | Ling Li (LGE) | y | 6.3.1a,b: Found minor mismatch in all the RA results which is fourth digit after decimal point. It happens in both anchor and tested result. It is probably due to the parallel encoding/decoding in RA. LGE uses parallel encoding/decoding with hex decimal as described in K1010. |
| 6.3.2 | Ling Li (LGE) | y | 6.3.2a,b: Found minor mismatch in all the RA results which is fourth digit after decimal point. It happens in both anchor and tested result. It is probably due to the parallel encoding/decoding in RA. LGE uses parallel encoding/decoding with hex decimal as described in K1010. |
| 6.4.1 | Steve Keating (Sony) | y | The results match within 2 decimal places, run times are also a very close match. |
| 6.4.2 | Ikeda Masaru (Sony) | y |  |
| 6.5.1 | Chris Rosewarne (Canon) | y | Results were an exact match for AI and RA conditions - BMS with VTM configs tested only. Runtimes not reliable but seem ok. |
| 6.6.1 | Liang Zhao (Tencent) | y |  |

## CE3.6: Related contributions

| Doc. # | Related test # | Title |
| --- | --- | --- |
| JVET-L0154 (KAU, ETRI) |  | MPM Modifications for Intra Mode Coding |
| JVET-L0155 (KAU, ETRI) |  | Most Frequent Mode (MFM) for Intra Mode Coding |
| JVET-L0280 (Tencent) |  | Intra mode coding |

# References

- G. Van der Auwera, J. Heo, A. Filippov, “Description of Core Experiment 3 (CE3): Intra Prediction and Mode Coding,” 11th JVET Meeting, Ljubljana, SI, July 2018, JVET-K1023