| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0026-v2 |
| --- | --- |

| Title: | CE6: Summary Report on Transforms and Transform Signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Amir Said Xin Zhao | Email: | asaid@qti.qualcomm.com xinzzhao@tencent.com |
| Source: | CE6 coordinators |  |  |

_____________________________

# Abstract

This contribution summarizes the activities of Core Experiment (CE) on Transforms and Transform Signalling. The goal of this CE is to study transform design and signaling for the VVC standard. The CE studies were divided into three categories, including:

(1) CE6-1: Primary transforms (21 tests, 11 proposals)

(2) CE6-2: Secondary transform (6 tests, 2 proposals)

(3) CE6-3: and transform combinations and signalling (7 tests, 3 proposals)

In this CE all experiments were done using based on the BMS-2.0.1 SW using VTM configuration. This document summarizes the test results, brief experiment definition, cross-check reports and complexity measurements.

# Introduction

The purpose of this Core Experiment (CE) is to investigate the coding tools related to transforms and transform signaling on top of the VTM-2.0.1. In this CE, the tradeoff between coding efficiency and complexity of different transformation tools will be investigated.

# Experiment List

Core experiments (CEs) are organized according to the following three categories, and the following table lists all the experiments in each category, and the corresponding input document to the Macau meeting.

| CE6.1 – Primary Transforms |  |  |  |  |
| --- | --- | --- | --- | --- |
| CE # | Proponent | Related Docs. | Summary of the tool | Cross-checkers |
| 6.1.1 | LGE | JVET-L0292, JVET-L0132 | Selection of MTS candidates. Fast DST-7/DCT-8 based on DFT. 32-point MTS based on skipping high frequency coefficients | HHI B<>com Qualcomm |
| 6.1.2 | Huawei | JVET-L0358 | Spatially Varying Transform (SVT) for inter blocks | Qualcomm Sony |
| 6.1.3 | Tencent | JVET-L0285 | Primary transform core with 8-bit precision | Samsung Huawei Dolby |
| 6.1.4 | Tencent | JVET-L0285 | Fast DST-7/DCT-8 with dual implementation support | Brightcove Samsung Qualcomm B<>com |
| 6.1.6 | Qualcomm, Orange | JVET-L0386 JVET-L0135 | Efficient Implementations of MTS with Transform Adjustment Stages | Tencent B<>com |
| 6.1.7 | Panasonic Technicolor Sony Tencent | JVET-L0118 JVET-L0262 JVET-L0486 JVET-L0287 | MTS simplification by reusing DCT-2 partial butterfly | Brightcove Huawei Panasonic Technicolor Sony Tencent |
| 6.1.8 | Technicolor | JVET-L0263 | MTS with DCT-II | Sony |

| CE6.2 – Secondary Transforms |  |  |  |  |
| --- | --- | --- | --- | --- |
| CE # | Proponent | Related Docs. | Summary of the tool | Cross-checkers |
| 6.2.1 | LGE | JVET-L0133 | Reduced secondary transform with one kernel for 35 transform sets. Worst case handled reduced secondary transform with one kernel for 4 transform sets | Tencent B<>com |
| 6.2.2 | Samsung | JVET-L0058 | Modified NSST sets and signaling | LGE |

| CE6.3 – Combinations and Signalling |  |  |  |  |
| --- | --- | --- | --- | --- |
| CE # | Proponent | Related Docs. | Summary of the tool | Cross-checkers |
| 6.3.1 | HHI | JVET-L0261 | Set of transforms: A set of predefined transform candidates, and each candidate specifies a primary horizontal transform, a primary vertical transform, and a non-separable secondary transform. The transform candidates depend on the block size and the prediction mode. | Brightcove Tencent LGE |
| 6.3.2 | Tencent | JVET-L0288 | Coupled primary and secondary transform | LGE HHI B<>com |
| 6.3.3 | Qualcomm | JVET-L0387 | Secondary Transforms Coupled with Simplified Primary Transformation | HHI Tencent |

# CE6-1: Primary Transforms

## Summary of results

The following table summarizes the results for CE6-1 using VTM-2.0.1 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-1.1a | JVET-L0292 | 0.11% | 0.21% | 0.22% | 78% | 99% | 0.06% | 0.09% | 0.12% | 96% | 100% | -0.05% | 0.01% | 0.10% | 97% | 101% |
| CE6-1.1b | JVET-L0292 | 0.30% | -0.17% | -0.17% | 61% | 97% | 0.13% | -0.05% | -0.06% | 91% | 101% | 0.05% | -0.17% | -0.21% | 95% | 100% |
| CE6-1.1c | JVET-L0132 | 0.01% | 0.00% | 0.03% | 97% | 90% | 0.01% | 0.03% | 0.04% | 99% | 99% | -0.01% | -0.06% | 0.01% | 99% | 100% |
| CE6-1.1d | JVET-L0132 | 0.11% | 0.01% | 0.01% | 98% | 95% | 0.03% | 0.02% | 0.02% | 99% | 100% | 0.02% | 0.04% | -0.14% | 100% | 101% |
| CE6-1.2a | JVET-L0358 |  |  |  |  |  | -0.48% | 0.21% | 0.03% | 109% | 102% | -0.94% | -0.06% | -0.10% | 114% | 104% |
| CE6-1.2b | JVET-L0358 |  |  |  |  |  | -0.61% | 0.20% | 0.00% | 114% | 102% | -1.02% | -0.04% | 0.03% | 122% | 104% |
| CE6-1.3a | JVET-L0285 | 0.00% | 0.00% | 0.05% | 99% | 98% | 0.01% | 0.09% | 0.07% | 98% | 98% | -0.01% | 0.24% | -0.09% | 98% | 99% |
| CE6-1.3b | JVET-L0285 | 0.01% | -0.08% | -0.01% | 99% | 97% | 0.01% | 0.03% | 0.04% | 99% | 99% | -0.01% | 0.16% | -0.20% | 98% | 99% |
| CE6-1.4a | JVET-L0286 | -0.01% | 0.01% | 0.04% | 95% | 89% | 0.00% | 0.03% | 0.04% | 98% | 98% | -0.04% | -0.18% | -0.31% | 98% | 98% |
| CE6-1.6a | JVET-L0386 | 0.03% | 0.01% | 0.06% | 98% | 90% | 0.02% | 0.02% | 0.09% | 99% | 98% | -0.01% | -0.21% | 0.46% | 97% | 95% |
| CE6-1.6b | JVET-L0135 | 0.06% | 0.04% | 0.07% | 97% | 88% | 0.05% | 0.05% | 0.07% | 99% | 98% | -0.02% | -0.07% | -0.08% | 99% | 100% |
| CE6-1.6c | JVET-L0386 | -0.02% | -0.06% | -0.06% | 98% | 91% | -0.07% | -0.26% | -0.23% | 101% | 98% | -0.05% | -0.23% | -0.47% | 101% | 101% |
| CE6-1.7a | JVET-L0262 | 0.23% | 0.08% | 0.08% | 101% | 95% | 0.19% | 0.22% | 0.22% | 99% | 98% | 0.05% | 0.05% | -0.11% | 97% | 97% |
| CE6-1.7b | JVET-L0118 |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| CE6-1.7c | JVET-L0118 | 0.23% | 0.08% | 0.08% | 101% | 95% | 0.19% | 0.22% | 0.22% | 99% | 98% | 0.05% | 0.05% | -0.11% | 97% | 97% |
| CE6-1.7d | JVET-L0486 | -0.21% | -0.18% | -0.15% | 97% | 95% | -0.07% | 0.00% | 0.02% | 97% | 97% | -0.10% | 0.13% | -0.29% | 96% | 95% |
| CE6-1.7e | JVET-L0486 | 0.08% | 0.02% | 0.06% | 98% | 93% | 0.11% | 0.17% | 0.14% | 97% | 97% | -0.01% | -0.24% | -0.15% | 96% | 95% |
| CE6-1.7f | JVET-L0287 | 0.06% | 0.07% | 0.14% | 99% | 97% | 0.08% | 0.13% | 0.15% | 99% | 98% | 0.15% | 0.12% | -0.18% | 97% | 96% |
| CE6-1.7g | JVET-L0287 | -0.05% | -0.07% | -0.06% | 100% | 99% | 0.09% | 0.09% | 0.12% | 99% | 99% | 0.08% | 0.10% | 0.06% | 98% | 98% |
| CE6-1.8a | JVET-L0263 | -0.05% | 0.09% | 0.11% | 101% | 100% | -0.02% | 0.09% | 0.06% | 100% | 100% | -0.04% | 0.05% | -0.27% | 101% | 101% |
| CE6-1.8b | JVET-L0263 | 0.03% | 0.14% | 0.14% | 101% | 98% | 0.08% | 0.16% | 0.22% | 100% | 101% | -0.03% | -0.03% | -0.01% | 100% | 99% |
| CTC + Inter MTS |  |  |  |  |  |  | -0.36% | 0.43% | 0.37% | 125% | 103% | -0.59% | 0.76% | 0.67% | 132% | 106% |

The following table summarizes the results for CE6-1 using VTM-2.0.1 w/ Inter MTS as anchor.

|  |  | All Intra w/ Inter MTS |  |  |  |  | Random Access w/ Inter MTS |  |  |  |  | Low Delay B w/ Inter MTS |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V |  | EncT | DecT |
| CE6-1.1a | JVET-L0292 | 0.11% | 0.21% | 0.22% | 78% | 99% | 0.04% | 0.08% | 0.15% | 96% | 100% | -0.02% | -0.06% | 0.21% |  | 99% | 100% |
| CE6-1.1b | JVET-L0292 | 0.30% | -0.17% | -0.17% | 61% | 97% | 0.12% | -0.09% | -0.03% | 93% | 100% | 0.03% | 0.00% | -0.12% |  | 97% | 99% |
| CE6-1.1c | JVET-L0132 | 0.01% | 0.00% | 0.03% | 97% | 90% | 0.00% | -0.06% | 0.05% | 97% | 97% | 0.00% | 0.04% | 0.25% |  | 97% | 96% |
| CE6-1.1d | JVET-L0132 | 0.11% | 0.01% | 0.01% | 98% | 95% | 0.04% | -0.02% | -0.01% | 96% | 98% | 0.03% | 0.07% | -0.05% |  | 96% | 98% |
| CE6-1.2a | JVET-L0358 |  |  |  |  |  | -0.37% | 0.03% | -0.04% | 107% | 101% | -0.60% | -0.30% | -0.12% |  | 110% | 100% |
| CE6-1.2b | JVET-L0358 |  |  |  |  |  | -0.45% | 0.07% | -0.03% | 110% | 100% | -0.65% | -0.32% | -0.03% |  | 114% | 100% |
| CE6-1.3a | JVET-L0285 | 0.00% | 0.00% | 0.05% | 99% | 98% | 0.02% | 0.04% | 0.11% | 100% | 99% | -0.02% | 0.26% | -0.01% |  | 100% | 100% |
| CE6-1.3b | JVET-L0285 | 0.01% | -0.08% | -0.01% | 99% | 97% | 0.02% | -0.08% | 0.00% | 100% | 99% | -0.03% | 0.08% | 0.05% |  | 101% | 101% |
| CE6-1.4a | JVET-L0286 | -0.01% | 0.01% | 0.04% | 95% | 89% | 0.00% | -0.03% | 0.07% | 96% | 97% | -0.01% | -0.12% | 0.02% |  | 97% | 97% |
| CE6-1.6a | JVET-L0386 | 0.03% | 0.01% | 0.06% | 98% | 90% | 0.01% | -0.05% | 0.03% | 97% | 96% | 0.01% | -0.05% | 0.03% |  | 97% | 96% |
| CE6-1.6b | JVET-L0135 | 0.06% | 0.04% | 0.07% | 97% | 88% | 0.03% | 0.03% | 0.06% | 96% | 96% | 0.00% | 0.00% | 0.05% |  | 96% | 96% |
| CE6-1.6c | JVET-L0386 | -0.02% | -0.06% | -0.06% | 98% | 90% | -0.14% | -0.19% | -0.19% | 104% | 96% | -0.18% | -0.03% | -0.35% |  | 104% | 95% |
| CE6-1.7a | JVET-L0262 | 0.23% | 0.08% | 0.08% | 101% | 95% | 0.14% | 0.06% | 0.16% | 100% | 97% | -0.17% | -0.12% | -0.01% |  | 101% | 99% |
| CE6-1.7b | JVET-L0118 |  |  |  |  |  | 0.01% | 0.04% | 0.03% | 98% | 97% | 0.01% | 0.10% | 0.53% |  | 99% | 98% |
| CE6-1.7c | JVET-L0118 | 0.23% | 0.08% | 0.08% | 101% | 95% | 0.14% | 0.01% | 0.14% | 100% | 97% | -0.16% | -0.03% | 0.14% |  | 101% | 99% |
| CE6-1.7d | JVET-L0486 | -0.21% | -0.18% | -0.15% | 97% | 95% | -0.08% | -0.04% | 0.01% | 98% | 97% | -0.08% | 0.20% | 0.09% |  | 99% | 98% |
| CE6-1.7e | JVET-L0486 | 0.08% | 0.02% | 0.06% | 98% | 93% | 0.07% | 0.01% | 0.06% | 100% | 96% | -0.22% | -0.22% | -0.14% |  | 100% | 98% |
| CE6-1.7f | JVET-L0287 | 0.06% | 0.07% | 0.14% | 99% | 97% | 0.06% | 0.07% | 0.16% | 99% | 97% | 0.07% | 0.21% | 0.13% |  | 100% | 100% |
| CE6-1.7g | JVET-L0287 | -0.05% | -0.07% | -0.06% | 100% | 99% | 0.03% | 0.00% | 0.11% | 101% | 99% | -0.07% | 0.05% | 0.16% |  | 101% | 101% |
| CE6-1.8a | JVET-L0263 |  |  |  |  |  | -0.01% | 0.01% | 0.05% |  |  |  |  |  |  |  |  |
| CE6-1.8b | JVET-L0263 |  |  |  |  |  | 0.05% | 0.02% | 0.09% |  |  |  |  |  |  |  |  |  |

## Test definition

### CE 6.1.1: Primary Transform

For selection of MTS candidates, the following two tests are performed:

Test 1: Using 2 MTS candidates for Angular mode and 3 candidates for non-angular mode.

Test 2: Using 1 MTS candidate for all modes.

32-point MTS is based on zero-out.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.1-a | Using 2 MTS candidates for Angular mode and 3 candidates for non-angular mode. Here, non-angular mode indicates DC and Planar modes | M. Koo (LGE) | HHI B<>com |
| 6.1.1-b | Using 1 MTS candidate for all modes. Here, 1 MTS candidate indicates DST7 | M. Koo (LGE) | HHI B<>com |
| 6.1.1-c | implementation of DST-7/DCT-8 using DFT | M. Koo (LGE) | Qualcomm HHI B<>com |
| 6.1.1-d | a method to zero-out high frequency component for larger transform (i.e., 32x32) | M. Koo (LGE) | HHI B<>com |

### CE 6.1.2: Spatially varying transform

For an inter-predicted CU with root_cbf equal to 1, a svt_flag may be signaled to indicate whether the whole residual block or a sub-part of the residual block is decoded. The latter case is called a spatially varying transform (SVT), in which a part of the residual block is coded and the rest of the residual block is forced to be zero.

When SVT is used for a CU, SVT type and SVT position information are further decoded from the bitstream. There are two SVT types and two SVT positions, as indicated in Figure 1. For SVT-V (or SVT-H), the TU width (or height) equals to half of the CU width (or height) or 1/4 of the CU width (or height), signaled by another flag. If one side of CU is 8, 1/4 of the side is not allowed. Hence, there are at most 8 SVT modes for a CU.

Both TU and CU boundaries are filtered in de-blocking filtering. SVT-V (or SVTH) is allowed when CU width (or height) is in the range of [8, maxSize]. When one side of the TU is greater than 32, the corresponding transform is set as DCT-2, and thus the maximum non-DCT-2 transform size is still 32. The maxSize is signaled in slice header. For HD and 4K sequences, maxSize is set as 64 for all slices; for other smaller resolution sequences, maxSize is set as 32 for all slices.

Position-dependent core transform is applied in SVT. The two positions of two types are associated with different core transforms. More specifically, the horizontal and vertical transforms for each SVT position is specified in Figure 1.For example, the horizontal and vertical transforms for SVT-V position 0 is DCT-8 and DST-7, respectively.

A fast algorithm is designed for SVT. For each SVT mode, a RD cost is estimated based on the SSD of the residual-skipped part. An SVT mode is skipped in RDO if the estimated RD cost of the SVT mode is larger than the actual RD cost of the best mode. In addition, only the best 4 SVT modes in terms of the estimated RD cost are tried in RDO.

Figure 1 Illustration of SVT modes

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.2-a | Normatively enable SVT for inter blocks predict by Merge mode with merge idx 0/1 or by AMVP mode with 1/4-pel MVD precision (i.e., imv=0) or by AFFINE mode | Y. Zhao (Huawei) | Qualcomm |
| 6.1.2-b | Normatively enable SVT for all inter blocks | Y. Zhao (Huawei) | Sony |

### CE 6.1.3: Primary transform core with 8-bit precision

A set of 8-bit transform cores for DCT-2, DST-7/DCT-8 are proposed.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.3-a | Primary transform core with 8-bit precision for both forward and backward transform | X. Zhao (Tencent) | K. Choi (Samsung) |
| 6.1.3-b | Primary transform core with 8-bit precision for only backward transform | X. Zhao (Tencent) | K. Choi (Samsung) Y. Zhao (Huawei) P. Yin (Dolby) |

### CE 6.1.4: Fast DST-7/DCT-8 with dual implementation support

A fast DST-7/DCT-8 method, which supports dual implementations (identical results between matrix multiplication and fast method) is proposed.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.4-a | Fast method based DST-7/DCT-8 implementation using adjusted transform cores | X. Zhao (Tencent) | K. Choi (Samsung) B<>com |

### CE 6.1.6: MTS with transform adjustment stages

MTS transforms are approximated using only a transform of the DCT-2 family and adjustment stages of low complexity.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.6-a | Efficient Implementations of MTS with Transform Adjustment Stages (Max. MTS size = 32) | Qualcomm | X. Zhao (Tencent) B<>com |
| 6.1.6-b | Further simplification for MTS complexity reduction | Orange | Qualcomm X. Zhao (Tencent) |
| 6.1.6-c | Efficient Implementations of MTS with Transform Adjustment Stages (Max. MTS size = 64) | Qualcomm | B<>com |

### CE 6.1.7: MTS simplification by reusing DCT-2 partial butterfly

Test1: MTS using DST-4/DCT-4

Test2: Unify MTS index for intra and inter

Test3: Add DST-2/DCT-2 and switch MTS candidate depending on intra direction

Test4: Embed DST-7/DCT-8 into DCT-2

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.7-a | MTS intra: DST-4 / DCT-4 MTS inter: DCT-4 / DST-4 Max MTS size: 32 | K. Naser (Technicolor) Panasonic Sony | X. Zhao (Tencent) |
| 6.1.7-b | MTS intra: DST-7 / DCT-8 MTS inter: DST-7 / DCT-8 (align mts_idx as intra) Max MTS size: 32 | K. Abe (Panasonic) | T. Tsukuba (Sony) |
| 6.1.7-c | MTS intra: DST-4 / DCT-4 MTS inter: DST-4 / DCT-4 (align mts_idx as intra) Max MTS size: 32 | K. Abe (Panasonic) | T. Tsukuba (Sony) |
| 6.1.7-d | MTS intra: DST-7 / DCT-8 / DST-2 / DCT-2 MTS inter: DCT-8 / DST-7 Max MTS size: 32 | T. Tsukuba (Sony) K. Naser (Technicolor) | K. Abe (Panasonic) |
| 6.1.7-e | MTS intra: DST-4 / DCT-4 / DST-2 / DCT-2 MTS inter: DCT-4 / DST-4 Max MTS size: 32 | T. Tsukuba (Sony) Technicolor | K. Abe (Panasonic) |
| 6.1.7-f | 32- and 64-point DCT-2 replaced by COT MTS intra: DST-7 / DCT-8 MTS inter: DCT-8 / DST-7 Max MTS size: 32 | X. Zhao (Tencent) | K. Naser (Technicolor ) Y. Zhao (Huawei) |
| 6.1.7-g | 32- and 64-point DCT-2 replaced by COT 16-point and 32-point: MTS intra: DST-7 / DCT-8 MTS inter: DCT-8 / DST-7 4-point and 8-point: MTS intra: DST-4 / DCT-4 MTS inter: DCT-4 / DST-4 Max MTS size: 32 | X. Zhao (Tencent) | K. Naser (Technicolor) |

### CE 6.1.8: MTS with DCT-II JVET-J0022

DCT-II is included in the MTS set, and a lookup table is defined to map the intra prediction mode and block size to the corresponding MTS subset.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.1.8-a | DCT-II is included in MTS together with a lookup table | Technicolor | Masaru Ikeda (Sony) |
| 6.1.8-b | 6-1.8-a using only DCT-II, DST-IV and DCT-IV | Technicolor | Masaru Ikeda (Sony) |

## Crosscheck reports

The following table summarizes the results for CE6-1 using VTM-2.0.1 as anchor.

| Test # | Crosschecker |  | Results match (Y/N) | Cross checker comment |
| --- | --- | --- | --- | --- |
| CE6-1.1a | HHI |  | Y | Coding results match those of the proponents. It was verified that the software matches the test description. |
|  | b<>com |  | Y | CE6-1.1a was verified by b<>com, it corresponds to the SIG3 option in the provided software. The signalization options provided are implemented in a straightforward manner and are consistent with the description provided as input to this meeting (JVET-L0292). TrQuant and IntraSearch and CabacReader/Writer are the main affected sources. Coding results match those of the proponent. As they were exhaustively verified under the CTC in AI and randomly verified for other conditions. |
| CE6-1.1b | HHI |  | Y | Coding results match those of the proponents. It was verified that the software matches the test description. |
|  | b<>com |  | Y | CE6-1.1b was verified by b<>com, it corresponds to the SIG4 option in the provided software. See above for the modifications and consistence of the |
| CE6-1.1c | HHI |  | Y | Coding results match those of the proponents. It was verified that the software matches the test description. |
|  | b<>com |  | Y | This option was checked by b<>com, the DST7/DCT8 implementations are based on FFT. Size 16 and 32 can be switched on/off independently (through the DST7_IMPL_WITH_FFT_xx macros). TrQuant_EMT.cpp is the main file affected by the changes. As the approach is based on FFT algorithms, wired unrolled FFT implementations with size 3,5, 11 and 13 are required. FFT 11 and 13 are coded with 4 functions each. The implementation corresponds to the description provided in JVET-K0096. The coding results match those of the proponent. |
| CE6-1.1d | HHI |  | Y | Coding results match those of the proponents. It was verified that the software matches the test description. |
|  | b<>com |  | Y | This experiment was verified by b<>com, skipping high frequency is enabled with the macro LG_RAMT. It was verified that the implementation conforms the description. Particularly, the effect of this high frequency skipping, was analyzed wrt its impact on residual coding elements and transform computation. The coding results reported by the proponent were verified and partially reproduced (the EMT=3 testing condition was not verified totally). No deviation were noticed wrt the results announced by the proponent. |
| CE6-1.2a | Qualcomm |  | Y | The coding gain results exactly match the results provided by the proponents. Similar encoding and decoding runtimes (less than 2% difference) are observed. |
| CE6-1.2b | Sony |  | Y | It was confirmed that the results were identical |
| CE6-1.3a | Samsung |  | Y | No significant inconsistencies between the description and the provided software. |
|  | Dolby |  | Y (metrics wPSNR and PSNR match) | Proponents only provided metrics: wPSNR and PSNR for HDR PQ content. No results are provided for DE100 and PSNR-L100. |
| CE6-1.3b | N/A |  | Y (all finished sequences) | code match with CE description |
|  | Samsung |  | Y | No significant inconsistencies between the description and the provided software. |
|  | Dolby |  | Y (metrics wPSNR and PSNR match) | Proponents only provided metrics: wPSNR and PSNR for HDR PQ content. No results are provided for DE100 and PSNR-L100 metrics. |
| CE6-1.4a | Samsung |  | Y | No significant inconsistencies between the description and the provided software. |
|  | b<>com |  | Y | The experiment was verified, through the provided macros MTS_CORE_ADJUSTMENT and FAST_DST7 both activated. Most of the modifications affect TrQuant_EMT.cpp where is implemented the fast DST7/DCT8 algorithm. Rom.cpp include the change for a few transform coefficients (some amplitudes are changed as described in JVET-L0286). The fast DCT8/DST7 algorithms are implemented in an unrolled fashion, exploiting the coefficient redundancies as described in the proposal. The DCT8 and DST7 codes are based on the same principle and are separate. The coding results of the proponents were identically reproduced in the cross-check. |
| CE6-1.6a | Tencent |  | Y | The results match with the ones in the proposal. |
|  | b<>com |  | Y | The experiment is activated with BMS2_0_1_CE6_1_6 macro, 6 taps per adjustment matrix is also selected. The code was verified and it is consistent with the description of the technology (JVET-L0386). The adjustment method is applied for the DST7/DCT8 for size 16 and 32. Two adjustment matrices are stored in m_Adj_dst7_xx in Rom.cpp. Those matrices are adjusted prior to (forward) or after (inverse) the transform. The adjustment coefficients are applied in reverse order (row wise) for DCT8/DST7 to exploit their duality. Partial DCT2/DCT3 butterfly algorithms are replicated for each transform with additional sign exchange and vector reversal to reproduce the DST2 and 3. The cross-check coding results match those of the proponents. |
| CE6-1.6b | Qualcomm |  | Y | The coding gain results exactly match the results provided by the proponents. Similar encoding and decoding runtimes (less than 2% difference) are observed. |
|  | Tencent |  | Y | The results match with the ones in the proposal. |
| CE6-1.6c | b<>com |  | Y | In addition to CE6-1.6a the 64-sized transforms are activated with an additional macro (ENABLE_TAF_B64). The code activates the MTS for size 64 and the implementation is similar to CE6-1.4a with this additional transform size. |
| CE6-1.7a | Tencent |  | Y | The results match with the ones in the proposal. SW has also been confirmed. |
| CE6-1.7b | Sony |  | Y | The results match with the ones in the proposal. SW has also been confirmed. |
| CE6-1.7c | Sony |  | Y | The results match with the ones in the proposal. SW has also been confirmed. |
| CE6-1.7d | Panasonic |  | Y | The results match with the ones in the proposal. SW has also been confirmed. |
| CE6-1.7e | Panasonic |  | Y | The results match with the ones in the proposal. SW has also been confirmed. |
| CE6-1.7f | Technicolor |  | Y (RA & LDB) | Code matches with CE description |
|  | N/A |  | Y (all sequences) | code match with CE description |
| CE6-1.7g | Technicolor |  | Y (RA & LDB) | Code matches with CE description |
| CE6-1.8a | Sony |  | Y (VTM, interMTS RA) | The results match with the ones in the proposal. SW has also been confirmed. |
| CE6-1.8b | Sony |  | Y (VTM, interMTS RA) | The results match with the ones in the proposal. SW has also been confirmed. |

## Complexity metrics

| Test CE6- | 1.1a | 1.1b | 1.1c | 1.1d |
| --- | --- | --- | --- | --- |
| Operation counts | same as VTM2.0.1 | same as VTM2.0.1 | 16-pt DST-7: 83 (Mult), 137 (Add/Sub), 47 (Shift) - 68% reduction of mult. count from matrix mult. 32-pt DST-7: 236 (Mult), 378 (Add/Sub), 88 (Shift) - 77% reduction of mult. count from matrix mult. | 16-pt DST-7: 83 (Mult), 137 (Add/Sub), 47 (Shift) 32-pt DST-7: half of VTM 2.0.1 |
| Memory requirements | same as VTM2.0.1 | Half of VTM2.0.1 (Just DST7 no DCT8) | 16-pt: 440 bits 32-pt: 938 bits | 16-pt: 440 bits 32-pt: half of VTM 2.0.1 |
| Bit-precision to represent transform coefficients | same as VTM2.0.1 | same as VTM2.0.1 | 10 bits (same as VTM 2.0.1) | 10 bits (same as VTM 2.0.1) |
| Precision of arithmetic operations during transform computation | same as VTM2.0.1 | same as VTM2.0.1 | 16-pt DST-7: - Mult: 11bits x 10bits (1st DFT), 12bits x 10bits (2nd DFT) - Add/Sub: 21bits operand (1st DFT), 22bits operand (2nd DFT) 32-pt DST-7: - Mult: 11bits x 10bits (1st DFT), 13bits x 10bits (2nd DFT) - Add/Sub: 21bits operand (1st DFT), 23bits operand (2nd DFT) | 16-pt DST-7: - Mult: 11bits x 10bits (1st DFT), 12bits x 10bits (2nd DFT) - Add/Sub: 21bits operand (1st DFT), 22bits operand (2nd DFT) 32-pt DST-7: - same as VTM 2.0.1 |
| Bit-precision for storing intermediate data representation | same as VTM2.0.1 | same as VTM2.0.1 | During fwd. hor. tr., - 16-pt DST-7: 22 (1st DFT interm.), 12 (btw. DFT stages), 22 (2nd DFT interm.), and 16 (Output) - 32-pt DST-7: 23 (1st DFT interm.), 13 (btw. DFT stages), 23 (2nd DFT interm.), and 16 (Output) | During fwd. hor. tr., - 16-pt DST-7: 22 (1st DFT interm.), 12 (btw. DFT stages), 22 (2nd DFT interm.), and 16 (Output) - 32-pt DST-7: same as VTM 2.0.1 |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. | No | No | Two stages (iterations), Each stage can be computed in parallel. 16-pt DST7: - 1st stage: 6 (Mult), 41 (Add/Sub), 31 (Shift) - 2nd stage: 77 (Mult), 96 (Add/Sub), 16 (Shift) 32-pt DST7: - 1st stage: 52 (Mult), 148 (Add/Sub), 56 (Shift) - 2nd stage: 184 (Mult), 230 (Add/Sub), 32 (Shift) | 16-pt DST-7: - same as 1.1c 32-pt DST-7: - No |
| Other operations and memory requirements | N/A | N/A | N/A | N/A |
| List of all combinations of transform types | A) Non-Angular mode: 0) DST7 for both horizontal and vertical transform 1) DST7 for Vertical and DCT8 for horizontal transform 2) DCT8 for Vertical and DST7 for horizontal transform B) Horizontal Modes 0) DST7 for both horizontal and vertical transform 1) DCT8 for Vertical and DST7 for horizontal transform C) Vertical Modes 0) DST7 for both horizontal and vertical transform 1) DST7 for Vertical and DCT8 for horizontal transform | DST7 for both horizontal and vertical transform | same as VTM2.0.1 | same as VTM2.0.1 |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). | same as VTM2.0.1 | same as VTM2.0.1 | The proposed DST-7 is reused in DCT-8 implementation as computation core along with reverse arrangement of input elements and alternating sign changes. Forward DST-7 and inverse DST-7 use the same DFT computation except for reordering and partial sign changes of inputs and outputs. | 16-pt DST-7: same as 1.1c 32-pt DST-7: same as VTM2.0.1 |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. | No | No | No | No |

| Test CE6- | 1.2a | 1.2b | 1.3a | 1.3b |
| --- | --- | --- | --- | --- |
| Operation counts | same as VTM2.0.1 | same as VTM2.0.1 | Same as VTM | Same as VTM |
| Memory requirements | same as VTM2.0.1 | same as VTM2.0.1 | Reduce 1.7KB memory (20% reduction) | Reduce 1.7KB memory (20% reduction) |
| Bit-precision to represent transform coefficients | same as VTM2.0.1 | same as VTM2.0.1 | Same as VTM | Same as VTM |
| Precision of arithmetic operations during transform computation | same as VTM2.0.1 | same as VTM2.0.1 | Same as VTM | Same as VTM |
| Bit-precision for storing intermediate data representation | same as VTM2.0.1 | same as VTM2.0.1 | Same as VTM | Same as VTM |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. | No | No | No | No |
| Other operations and memory requirements | N/A | N/A | N/A | N/A |
| List of all combinations of transform types | same as VTM2.0.1 (DCT-2, DST-7, DCT-8) | same as VTM2.0.1 (DCT-2, DST-7, DCT-8) | Same as VTM | Same as VTM |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). | same as VTM2.0.1 | same as VTM2.0.1 | Same as VTM | Same as VTM |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. | No | No | No | No |

| Test CE6- | 1.4a | 1.6a | 1.6b | 1.6c |
| --- | --- | --- | --- | --- |
| Operation counts | 16-pt DST-7/DCT-8: Mult: 127 Add:155 Shift: 16 32-pt DST-7/DCT-8: Mult: 620 Add:718 Shift: 32 | Size 16/32 adjustment: Mult 96/192 Add 97/193 Shift 17/33 Size 16/32 DCT2: Multi 86/342 Add 113/401 Shift 17/33 | Size 16/32 adjustment: Mult 74/128 Add 90/186 Size 16/32 DCT2: Mult 86/342 Add 113/401 Shift 17/33 | Size 16/32/64 adjustment: Mult 96/192/384 Add 97/193/385 Shift 17/33/65 Size 16/32 DCT2: Multi 86/342/1366 Add 113/401/1457 Shift 17/33/33 |
| Memory requirements | Same as VTM | Adjustment: 288 coefficients Overall memory: 7.144 KB (3.076 KB than VTM) | Adjustment: 167 coefficients Overall memory: 7.008 KB (3.212 KB less than VTM) | Adjustment: 672 coefficients Overall memory: 7.576 KB (2.644 KB less than VTM) |
| Bit-precision to represent transform coefficients | Same as VTM | 9 | 9 | 9 |
| Precision of arithmetic operations during transform computation | Same as VTM | Same as VTM | same as VTM | Same as VTM |
| Bit-precision for storing intermediate data representation | Same as VTM | Same as VTM | same as VTM | Same as VTM |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. | Same as VTM | Same as VTM | No additional iteration is required from BMS | Same as VTM |
| Other operations and memory requirements | none | none | none | none |
| List of all combinations of transform types | Same as VTM | Same as VTM | same as VTM | Same as VTM |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). | Support dual implementations, full matrix mult provides same results with fast method based on partial butterfly. | Forward/inverse DCT2 use the partial butterfly algorithm, the adjustments are applied as 1-D filtering operations. | DCT3/DCT3 use the partial butterfly algorithm, the adjustment stages are applied as a sparse matrix multiplication. | Forward/inverse DCT2 use the partial butterfly algorithm, the adjustments are applied as 1-D filtering operations. |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. | None | None | none | none |

| Test CE6- | 1.7a | 1.7b | 1.7c | 1.7d |
| --- | --- | --- | --- | --- |
| Operation counts | same as VTM | same as VTM | same as VTM | Same as VTM |
| Memory requirements | Reduced 3.4KB memory (40%) | same as VTM | Reduced 3.4KB memory (40%) | Same as VTM |
| Bit-precision to represent transform coefficients | same as VTM | same as VTM | same as VTM | Same as VTM |
| Precision of arithmetic operations during transform computation | same as VTM | same as VTM | same as VTM | Same as VTM |
| Bit-precision for storing intermediate data representation | same as VTM | same as VTM | same as VTM | Same as Vttm |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. | None | None | None | None |
| Other operations and memory requirements | None | None | None | None |
| List of all combinations of transform types | DST-7 and DCT-8 are replaced with DST4 and DCT-4. | same as VTM | DST-7 and DCT-8 are replaced with DST4 and DCT-4. | Same as VTM |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). | same as VTM | same as VTM | same as VTM | Same as VTM |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. | None | None | None | None |

| Test CE6- | 1.7e | 1.7f | 1.7g | 1.8a |
| --- | --- | --- | --- | --- |
| Operation counts | Same as VTM | Same as VTM | Same as VTM | Same as VTM |
| Memory requirements | Reduced 3.4KB memory (40%) | Reduced 3.2KB memory (37.6%) | Reduced 3.4KB memory (40%) | Same as VTM |
| Bit-precision to represent transform coefficients | Same as VTM | Same as VTM | Same as VTM | Same as VTM |
| Precision of arithmetic operations during transform computation | Same as VTM | Same as VTM | Same as VTM | Same as VTM |
| Bit-precision for storing intermediate data representation | Same as VTM | Same as VTM | Same as VTM | Same as VTM |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. | None | None | None | None |
| Other operations and memory requirements | None | None | None | None |
| List of all combinations of transform types | DST-7 and DCT-8 are replaced with DST4 and DCT-4. | Same as VTM | Same as VTM | Same as VTM |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). | Same as VTM | Same as VTM | Same as VTM | Same as VTM |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. | None | None | None | None |

| Test CE6- | 1.8b |
| --- | --- |
| Operation counts | Same as VTM |
| Memory requirements | Same as VTM |
| Bit-precision to represent transform coefficients | Same as VTM |
| Precision of arithmetic operations during transform computation | Same as VTM |
| Bit-precision for storing intermediate data representation | Same as VTM |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. | None |
| Other operations and memory requirements | None |
| List of all combinations of transform types | DST-7 and DCT-8 are replaced with DST4 and DCT-4. |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). | Same as VTM |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. | None |

## Related contributions

| Document | Related CE Test | Title |
| --- | --- | --- |
| JVET-L0059 (Samsung) |  | CE6-related: Simplification on MTS kernel derivation |
| JVET-L0060 (Samsung) | CE6-1.3, CE6-1.7g/f | CE6-related: Unified matrix for transform |
| JVET-L0111 (SKKU) |  | CE6-related: Transform Skip Condition on Transform Block size |
| JVET-L0134 (Nokia) |  | CE6-related: Shape adaptive transform selection |
| JVET-L0149 (LGE) |  | CE6-related: Complexity reduction method based on skipping high frequency coefficients for inter MTS |
| JVET-L0190 (Alibaba) |  | CE6-related: Simplification of Intra 4-Point Multiple Transforms Selection |
| JVET-L0195 (Wilus, Humax) |  | CE6-related: MTS for non-square CUs |
| JVET-L0264 (Technicolor) |  | CE6-related: Removed MTS CU-Flag and Reduced MTS Pairs |
| JVET-L0289 (Tencent) |  | CE6-related: Unification of Transform Skip mode and MTS |
| JVET-L0304 (LGE) |  | CE6-Related: Multiplication Free Transform |
| JVET-L0331 (Bytedance) |  | CE6 related: On Index Signalling of Multiple Transform Selection |
| JVET-L0353 (UESTC) | CE6-1.7 | CE6-related: MTS using DST-4 and transposed DCT-2 |
| JVET-L0395 (Qualcomm) | CE6-1.7 | CE6-related: MTS with 4-point DST/DCT-4 and large block support |
| JVET-L0407 (Qualcomm) |  | CE6-related: Transform skip for 2x2 chroma blocks and disable 2x2 chroma blocks in intra slices |
| JVET-L0421 (Technicolor) | CE6-1.1 | CE6-related: fast implementation of MTS transforms using matrix multiplication |
| JVET-L0489 (Erricson) |  | CE6-related: Transform Simplification |

# CE6-2: Secondary Transforms

## Summary of results

The following table summarizes the results for CE6-2 using VTM-2.0.1 as anchor.

|  |  |  |  |  | All Intra |  |  |  |  |  | Random Access |  |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-2.1a | JVET-L0133 | -1.02% | -1.98% | -2.34% | 94% | 96% | -0.58% | -1.37% | -1.73% | 101% | 99% | -0.28% | -0.84% | -1.13% | 100% | 99% |
| CE6-2.1b | JVET-L0133 | -1.16% | -1.86% | -2.21% | 124% | 98% | -0.61% | -1.38% | -1.71% | 109% | 100% | -0.30% | -0.69% | -1.25% | 105% | 100% |
| CE6-2.1c | JVET-L0133 | -1.60% | -2.44% | -2.79% | 92% | 95% | -0.91% | -1.60% | -2.09% | 101% | 100% | -0.40% | -0.79% | -1.22% | 101% | 99% |
| CE6-2.1d | JVET-L0133 | -1.75% | -2.40% | -2.77% | 121% | 96% | -0.93% | -1.65% | -2.11% | 109% | 99% | -0.43% | -0.89% | -1.28% | 104% | 100% |
| CE6-2.2a | JVET-L0058 | -0.60% | -1.70% | -2.40% | 371% | 100% | -0.40% | -1.60% | -2.20% | 177% | 101% | -0.20% | -0.70% | -1.30% | 137% | 100% |
| CE6-2.2b | JVET-L0058 | -1.80% | -2.60% | -3.20% | 383% | 103% | -1.00% | -2.20% | -2.60% | 185% | 101% | -0.40% | -1.30% | -1.70% | 143% | 100% |

## Test definition

### CE6.2.1 Reduced Secondary Transform (RST)

Reduced Secondary Transform (RST) is investigated with the following aspects.

- Use one transform kernel per transform set

- Use only 4 transform set

- Apply worst case handling method to reduce the number of multiplications

- Using 2 MTS candidates for Angular mode and 3 candidates for non-angular mode

- Using 1 MTS candidate for all modes

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.2.1-a | a+b+c+e | M. Koo (LGE) | X. Zhao (Tencent) B<>com |
| 6.2.1-b | a+b+c+d | M. Koo (LGE) | Kiho Choi (Samsung) |
| 6.2.1-c | a+e | M. Koo (LGE) | X. Zhao (Tencent) |
| 6.2.1-d | a+d | M. Koo (LGE) | Kiho Choi (Samsung) |

### CE 6.2.3 Transform Signaling

The non-separable secondary transform (NSST) with the following two modifications:

- Reduced angle sets based on grouping intra prediction mode

- Signaling with 1 on/off flag with 4 modes

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.2.3.-a | Modified NSST sets and signaling for 4x4 only | Samsung | LGE |
| 6.2.3.-b | Modified NSST sets and signaling for 4x4 and 8x8 | Samsung | LGE |

## Crosscheck reports

The following table summarizes the results for CE6-1 using VTM-2.0.1 as anchor.

| Test # | Crosscheck Doc. # |  | Results match (Y/N) | Cross checker comment |
| --- | --- | --- | --- | --- |
| CE6-2.1a | Tencent |  | Y | Results and sw implementation are aligned with the CE description. |
|  | b<>com |  | Y | The implementation corresponds to what is described by the proponents. In this test, there are both 8x8 and 4x4 RST. A 4x4 RST requires a transform of 16x16 coefficients, producing 16 transform coefficients. An 8x8 RST requires a transform of 16x64 coefficients, also producing 16 transform coefficients. In this test, only 4 transform sets are used, each comprising 2 transforms for the 2 MTS cases. Therefore, this proposal requires the storage of 4*2*16*16 + 4*2*16*64 = 10240 coefficients. These coefficients can be stored on 8 bits, thus the storage requirement is 10.24kB. Since only 1 MTS candidate is used, the number of RD checks for transform selection goes down from 4 to 2 compared to VTM. The results given by the proponents are confirmed. |
| CE6-2.1b |  |  |  |  |
| CE6-2.1c | Samsung |  | Y | No significant inconsistencies between the description and the provided software. |
|  | Tencent |  | Y | Results and sw implementation are aligned with the CE description. |
| CE6-2.1d | Samsung |  | Y | No significant inconsistencies between the description and the provided software. |
| CE6-2.2a | LGE |  |  |  |
| CE6-2.2b | LGE |  |  |  |

## Complexity metrics

| Test CE6- | 2.1a | 2.1b | 2.1c | 2.1d |
| --- | --- | --- | --- | --- |
| Operation counts | Primary transform: Same as 6.1.1b Secondary transform: NSST4x4: 256 (M), 240(A), 16(S) NSST8x8: 1024 (M), 960(A), 64(S) Worst Case multiplication per pixel: 8 | Primary transform: Same as 6.1.1a Secondary transform: NSST4x4: 256 (M), 240(A), 16(S) NSST8x8: 1024 (M), 960(A), 64(S) Worst Case multiplication per pixel: 8 | Primary transform: Same as 6.1.1b Secondary transform: NSST4x4: 256 (M), 240(A), 16(S) NSST8x8: 1024 (M), 960(A), 64(S) Worst Case multiplication per pixel: 16 | Primary transform: Same as 6.1.1a Secondary transform: NSST4x4: 256 (M), 240(A), 16(S) NSST8x8: 1024 (M), 960(A), 64(S) Worst Case multiplication per pixel: 16 |
| Memory requirements | 8x8 NSST: 16*64*8 /(8*1024)= 1 KB per kernel 4x4 NSST: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 2*4*(1+ 0.25) = 10 KB | 8x8 NSST: 16*64*8 /(8*1024)= 1 KB per kernel 4x4 NSST: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 4*(1+ 0.25) = 5 KB | 8x8 NSST: 16*64*8 /(8*1024)= 1 KB per kernel 4x4 NSST: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 103*(1+ 0.25) = 128.75 KB | 8x8 NSST: 16*64*8 /(8*1024)= 1 KB per kernel 4x4 NSST: 16*16*8/(8*1024)= 0.25 KB per Kernel Total = 103*(1+ 0.25) = 128.75 KB |
| Bit-precision to represent transform coefficients | 8 bits | 8 bits | 8 bits | 8 bits |
| Precision of arithmetic operations during transform computation | Same as VTM | Same as VTM | Same as VTM | Same as VTM |
| Bit-precision for storing intermediate data representation | Same as VTM | Same as VTM | Same as VTM | Same as VTM |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. | No | No | No | No |
| Other operations and memory requirements | N/A | N/A | N/A | N/A |
| List of all combinations of transform types | One NSST Kernel combined with primary 6.1.1b | One NSST Kernel combined with primary 6.1.1a | One NSST Kernel combined with primary 6.1.1b | One NSST Kernel combined with primary 6.1.1a |
| List of all combinations of transforms block size and transform type used for secondary transformation | For blocks with min(W,H)<8 : NSST4x4 For blocks with min(W,H)>=8 : NSST8x8 For worst case handling: for 4x4 block: Half NSST4x4 for 8x8 block: Half NSST8x8 | For blocks with min(W,H)<8 : NSST4x4 For blocks with min(W,H)>=8 : NSST8x8 For worst case handling: for 4x4 block: Half NSST4x4 for 8x8 block: Half NSST8x8 | For blocks with min(W,H)<8 : NSST4x4 For blocks with min(W,H)>=8 : NSST8x8 | For blocks with min(W,H)<8 : NSST4x4 For blocks with min(W,H)>=8 : NSST8x8 |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). | Primary transform: Same as 6.1.1b Secondary transform: Reduced matrix multiplications | Primary transform: Same as 6.1.1a Secondary transform: Reduced matrix multiplications | Primary transform: Same as 6.1.1b Secondary transform: Reduced matrix multiplications | Primary transform: Same as 6.1.1a Secondary transform: Reduced matrix multiplications |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. | N/A | N/A | N/A | N/A |

| Test CE6- | 2.2a | 2.2b |
| --- | --- | --- |
| Operation counts |  |  |
| Memory requirements |  |  |
| Bit-precision to represent transform coefficients |  |  |
| Precision of arithmetic operations during transform computation |  |  |
| Bit-precision for storing intermediate data representation |  |  |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. |  |  |
| Other operations and memory requirements |  |  |
| List of all combinations of transform types |  |  |
| List of all combinations of transforms block size and transform type used for secondary transformation |  |  |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). |  |  |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. |  |  |

## Related contributions

| Document | Related CE Test | Title |
| --- | --- | --- |
| JVET-L0153 (LGE) | CE6-2.1 | CE6-related: NSST modification for wide angle intra prediction |

# CE6-3: Combinations and Signalling

## Summary of results

The following table summarizes the results for CE6-2 using VTM-2.0.1 as anchor.

|  |  | All Intra |  |  |  |  | Random Access |  |  |  |  | Low Delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Test # | Doc. # | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| CE6-3.1b | JVET-L0261 | -0.75% | -0.54% | -0.55% | 97% | 101% | -0.36% | -0.15% | -0.26% | 99% | 101% | -0.23% | -0.01% | -0.27% | 101% | 103% |
| CE6-3.1c | JVET-L0261 | -1.67% | -1.03% | -1.04% | 97% | 100% | -0.95% | -0.27% | -0.29% | 99% | 100% | -0.45% | -0.40% | -0.45% | 100% | 102% |
| CE6-3.2a | JVET-L0288 | -0.46% | -0.39% | -0.37% | 100% | 101% | -0.23% | -0.10% | -0.10% | 100% | 100% | -0.13% | -0.09% | -0.57% | 99% | 100% |
| CE6-3.2b | JVET-L0288 | -0.50% | -0.65% | -0.68% | 101% | 100% | -0.24% | -0.45% | -0.53% | 100% | 100% | -0.17% | -0.52% | -0.86% | 100% | 101% |
| CE6-3.2c | JVET-L0288 | -0.43% | -0.59% | -0.62% | 98% | 99% | -0.22% | -0.37% | -0.45% | 98% | 98% | -0.15% | -0.24% | -0.30% | 98% | 98% |
| CE6-3.3a | JVET-L0387 | -0.36% | -0.52% | -0.51% | 96% | 97% | -0.19% | -0.15% | -0.15% | 99% | 100% | -0.10% | -0.09% | -0.10% | 100% | 101% |
| CE6-3.3b | JVET-L0387 | -1.30% | -0.96% | -1.00% | 97% | 99% | -0.73% | -0.11% | -0.22% | 99% | 100% | -0.34% | -0.05% | -0.18% | 100% | 100% |

## Test definition

### CE 6.3.1: Set of transforms

The transform is selected among a set of predefined transform candidates for each CU. Each of the transform candidates specifies a primary horizontal transform, a primary vertical transform, and a non-separable secondary transform. The ordered list of transform candidates depends on the block size, prediction mode and channel type.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.3.1-a | Set of transforms comprising primary transforms only | HHI | Withdrawn |
| 6.3.1-b | Set of transforms comprising primary and 4x4 secondary transforms | HHI | X. Zhao (Tencent) |
| 6.3.1-c | Set of transforms comprising primary and secondary transforms including dimensions higher than 4x4 | HHI | Moonmo Koo (LGE) |

### CE 6.3.2: Coupled primary and secondary transform

The utilization of primary and secondary transforms is coupled, and the combination is indicated by a single transform index. In addition, the proposed method can be applied for both luma and chroma components.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.3.2-a | Coupled primary and secondary transform for luma only, and secondary transform based on 4x4 NSST | X. Zhao (Tencent) | M. Salehifar (LGE) B<>com |
| 6.3.2-b | Coupled primary and secondary transform for both luma and chroma, and secondary transform based on 4x4 NSST | X. Zhao (Tencent) | M. Salehifar (LGE) |
| 6.3.2-c | Coupled primary and secondary transform for both luma and chroma, and secondary transform based on 4x4 NSST, with reduced transform sets based on grouping intra prediction modes | X. Zhao (Tencent) | Qualcomm |

### CE 6.3.3: Secondary transforms coupled with a simplified primary transformation

Simplification based on coupling primary and secondary transforms and signaling them together. A set of transforms Tset = {T0, T1, T2, T3, T4} is determined where each transforms Tx defines a combination of horizontal primary transform, vertical primary transform and a secondary transform.

| Test # | Description | Tester | Cross-checker |
| --- | --- | --- | --- |
| 6.3.3-a | Secondary transforms based on 4x4 NSST coupled with simplified primary transform | Qualcomm | X. Zhao (Tencent) |
| 6.3.3-b | Secondary transforms based on both 4x4 NSST and 8x8 NSST coupled with a simplified primary transform | Qualcomm |  |

## Crosscheck reports

The following table summarizes the results for CE6-1 using VTM-2.0.1 as anchor.

| Test # | Crosscheck Doc. # |  | Results match (Y/N) | Cross checker comment |
| --- | --- | --- | --- | --- |
| CE6-3.1b | Tencent |  | Y | The results match with the ones in the proposal. |
| CE6-3.1c | LGE |  | Y | Coding results match those of the proponents. |
| CE6-3.2a | LGE |  | Y | Coding results match those of the proponents. |
|  | b<>com |  | Y | The implementation matches what is described by the proponents. The NSST is applied using a straightforward matrix multiplication. The code includes a table for the NSST transform coefficients of size 35*3*16*16. These coefficients can be stored on 8 bits, thus the storage requirements for this table is 26.88kB. In this test, the look-up table that matches the intra prediction mode to a NSST transform set is the same as for the BMS NSST. Unlike the BMS NSST, there is no extra RD check; the only added complexity is the application of the NSST. The results given by the proponents are confirmed. |
|  | HHI |  | Y | Coding results match those of the proponents. It was verified that the software matches the test description. |
| CE6-3.2b | LGE |  | Y | Coding results match those of the proponents. |
|  | HHI |  | Y | Coding results match those of the proponents. It was verified that the software matches the test description. |
| CE6-3.2c | Qualcomm |  | Y | The coding gain results exactly match the results provided by the proponents. Similar encoding and decoding runtimes (around 2% difference) are observed. |
| CE6-3.3a | Tencent |  | Y | Coding results and sw implementation match with proposal. |
| CE6-3.3b |  |  |  |  |

## Complexity metrics

| Test CE6- | 3.1b | 3.1c | 3.2a | 3.2b |
| --- | --- | --- | --- | --- |
| Operation counts | Same as BMS (i.e. NSST4x4: 256(M), 256(A), 145(S) | 4x4 NSST: same as BMS 8x8 NSST: 1536(M), 1536(A), 192(S) | Primary transform: Same as VTM Secondary transform: NSST4x4: 256 (M), 240(A), 16(S) | Primary transform: Same as VTM Secondary transform: NSST4x4: 256 (M), 240(A), 16(S) |
| Memory requirements | 7.75 KB (Trafos + LUT) | 71.92 KB (Trafos+LUT) | 26.88 KB | 26.88 KB |
| Bit-precision to represent transform coefficients | Same as BMS | Same as BMS | Primary transform: Same with BMS Secondary transform: 8-bit | Primary transform: Same with BMS Secondary transform: 8-bit |
| Precision of arithmetic operations during transform computation | Same as BMS | Same as BMS | Same as VTM | Same as VTM |
| Bit-precision for storing intermediate data representation | Same as BMS | Same as BMS | Same as VTM | Same as VTM |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. | Same as BMS (i.e. 8 pairwise rotation layers + 1 permutation layer) Rotations in one layer can be done in parallel. | 4x4 NSST: same as BMS. 8X8 NSST: 4 iterations, 2 with 512 parallelizable operations, and 2 with 256 parallelizable operations | No | No |
| Other operations and memory requirements | N/A | N/A | N/A | N/A |
| List of all combinations of transform types | Combinations defined by LUT Primary transforms (V/H): (DCT-2,DCT-2), (DST-7,DST-7), (DCT-8,DST-7), (DST-7,DCT-8) | Combinations defined by LUT Primary transforms (V/H): (DCT-2,DCT-2), (DST-7,DST-7), (DCT-8,DST-7), (DST-7,DCT-8) | NSST1/2/3 coupled with AMT index 1/2/3 | NSST1/2/3 coupled with AMT index 1/2/3 |
| List of all combinations of transforms block size and transform type used for secondary transformation | NSST is applied for the same block sizes where MTS is applied | NSST is applied for the same block sizes where MTS is applied. W<8 or H<8: NSST 4x4 W>=8 or H>= 8: NSST 8x8 | NSST is applied for primary transform {DST7, DCT8}, {DCT8, DST7} and {DCT8, DCT8} NSST is applied for the same block sizes where MTS is applied | NSST is applied for primary transform {DST7, DCT8}, {DCT8, DST7} and {DCT8, DCT8} NSST is applied for the same block sizes where MTS is applied |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). | Primary transform: Same as VTM Secondary transform: Same as BMS (i.e. 4x4 HyGT) | Primary transform: Same as VTM Secondary transform: 4x4 HyGT & 8x8 HSMT | Primary transform: Same as VTM Secondary transform: Matrix multiplications | Primary transform: Same as VTM Secondary transform: Matrix multiplications |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. | No | No | No | No |

| Test CE6- | 3.2c | 3.3a | 3.3b |
| --- | --- | --- | --- |
| Operation counts | Primary transform: Same as VTM Secondary transform: NSST4x4: 256 (M), 240(A), 16(S) | Same as BMS | NSST4x4: 256 (M), 157(A), 145(S) NSST8x8: 1664 (M), 1665 (A), 81(S) |
| Memory requirements | 9.98 KB | Same as BMS | NSST4x4: 66112 bits NSST8x8: 1653120 bits |
| Bit-precision to represent transform coefficients | Primary transform: Same with BMS Secondary transform: 8-bit | Same as BMS | Same as BMS |
| Precision of arithmetic operations during transform computation | Same as VTM | Same as BMS | Same as BMS |
| Bit-precision for storing intermediate data representation | Same as VTM | Same as BMS | Same as BMS |
| Specify if a transform requires multiple iterations where transform output is fed back as input to the transform logic, and multiple iterations are required to produce final transform output. If yes, report the number of arithmetic operation required in each iteration, and if they can be computed in parallel. | No | Same as BMS | 4 iterations, 2 with 512 parallelizable operations, and 2 with 256 parallelizable operations |
| Other operations and memory requirements | N/A | None | None |
| List of all combinations of transform types | NSST1/2/3 coupled with AMT index 1/2/3 | DST-7, DCT-2 and 3 NSST candidates | DST-7, DCT-2 and 3 NSST candidates |
| List of all combinations of transforms block size and transform type used for secondary transformation | NSST is applied for primary transform {DST7, DCT8}, {DCT8, DST7} and {DCT8, DCT8} NSST is applied for the same block sizes where MTS is applied | Same as BMS | W<8 or H<8: NSST 4x4 W>=8 or H>= 8: NSST 8x8 |
| Provide analysis of implementation and arithmetic commonalities of proposed transforms. (e.g., stating which transforms are implemented using matrix multiplications, addition-only butterflies, or Givens rotations). | Primary transform: Same as VTM Secondary transform: Matrix multiplications | The 4x4 NSST is implemented with HyGT, same as in BMS. | The 4x4 NSST is implemented with HyGT, same as for BMS, while 8x8 NSST is implemented with hierarchical structure proposed in this CE |
| If the proposal requires additional computations at the encoder or decoder (e.g., additional transforms, or partial transforms along block boundaries), then the additional operations should be reported together with the other complexity metrics. | No | None | None |

## Related contributions

| Document | Related CE Test | Title |
| --- | --- | --- |

# References

- A. Said and X. Zhao, “Core Experiment 6: Arithmetic Coding Engine,” JVET-K1026, 9th meeting, Ljubljana, SI, 10–18 July 2018.