| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macau, CN, 3–12 October 2018 | Document: JVET-L0033-v1 |
| --- | --- |

| Title: | CE13: Summary report on coding tools for 360° omnidirectional video |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Philippe Hanhart Jian-Liang Lin Chirag Pujara | Email: | Philippe.Hanhart@InterDigital.com jl.lin@mediatek.com chirag.p@samsung.com |
| Source: | CE13 coordinators |  |  |

_____________________________

# Abstract

This contribution provides a summary report of Core Experiment 13 on coding tools for 360° omnidirectional video. Different methods to remove subjective artifacts at face boundaries are investigated in CE13, 1) padding and blending, 2) modified intra prediction, 3) inter prediction using geometry padding, 4) modified in-loop filters, 5) post-filtering, and 6) projection format adaptation. Combinations of different methods is also investigated. Most of these methods are implemented for HEC, with or without padding. Test results against PHEC (and HEC in some cases) are provided to show the coding efficiency of each normative decoding tool and non-normative pre- and post-processing. Test results are provided for the VTM configuration. BD-rates are computed based on the end-to-end WS-PSNR metric. Crosschecking results are integrated in this contribution.

# CE13.1: Padding and blending

## Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 13.1.1 | HEC with padding and blending (PHEC) - 4 samples padding around face row - 8 samples padding around face row | - JVET-L0228 (InterDigital) |

## Test results

### Compared to PHEC

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.1.1.a | - | - | - | - | - |
| 13.1.1.b | 0.70% | 0.58% | 0.68% | 100% | 98% |

### Compared to HEC

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.1.1.a | 0.32% | -0.45% | -0.60% | 103% | 100% |
| 13.1.1.b | 1.01% | 0.12% | 0.08% | 103% | 98% |

## Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 13.1.1.a | Samsung | 2 | F | Y |  | CTC anchor |
| 13.1.1.b | Samsung | 2 | F | Y |  |  |

Level: 1 simulation only, 2 code review + simulation, 3 independent implementation + simulation

Results: P partial results by spot-checking, F full results

Match: Y / N

EncT/DecT: M match in general, G gap observed

Comment: any information that might be helpful to the group

## Brief description of tests

### CE13.1.1

Please refer to JVET-K1004 [1] for details.

# CE13.2: Intra prediction

## Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 13.2.1 | HEC with intra prediction disabled across face discontinuities | JVET-L0229 (InterDigital/MediaTek) |
| 13.2.2 | HEC with intra prediction using spherical neighbors | JVET-L0345 (MediaTek) |

## Test results

### Compared to PHEC

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.2.1 | -0.33% | 0.30% | 0.49% | 100% | 98% |
| 13.2.2 | -0.38% | 0.41% | 0.48% | 100% | 101% |

### Compared to HEC

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.2.1 | -0.01% | -0.16% | -0.12% | - | 99% |
| 13.2.2 | -0.07% | -0.05% | -0.13% | 100% | 101% |

## Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 13.2.1 |  |  |  |  |  |  |
| 13.2.2 | Samsung | 2 | F | Y |  |  |

Level: 1 simulation only, 2 code review + simulation, 3 independent implementation + simulation

Results: P partial results by spot-checking, F full results

Match: Y / N

EncT/DecT: M match in general, G gap observed

Comment: any information that might be helpful to the group

## Brief description of tests

### CE13.2.1 in JVET-L0229 (InterDigital/MediaTek)

For intra prediction (DC, planar, and angular modes, as well as cross-component linear model), the use of reference samples is modified as follows. If a CU does not cross a face discontinuity, then it is determined whether each of the available reference samples is located on the same side of the face discontinuity to which the current CU belongs, and only those available reference samples that are located on the same side of the face discontinuity are used in intra prediction.

For most probable modes (MPM) in intra prediction, the construction of the MPM list is modified as follows. If a CU does not cross a face discontinuity, then it is determined if each available spatial neighbor in the frame packed picture is also a spherical neighbor, and only the available frame packed neighbors that are also spherical neighbors are used to build the MPM list.

### CE13.2.2 in JVET-L0345 (Mediatek)

Based on the spherical continuity in 360° videos, the reference pixels of intra prediction can be derived from spherical neighboring pixels. When a CU is located at the frame boundary (layout boundary) or the discontinuous edge(s), its neighboring pixels are replaced by the spherical neighboring reconstructed pixels, which are located at other faces. As the example shown in Figure 1, the neighboring pixels of the current CU are replaced by the top and left spherical neighboring pixels which are located at the Left face and Front face, respectively.

Figure 1. Spherical neighboring pixels

# CE13.3: Inter prediction

## Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 13.3.1 | HEC with face row based geometry padding of reference pictures - Using projection with Lanczos 3/2 interpolation - Using projection with bilinear interpolation - Using faceunfolding-based padding without interpolation | JVET-L0346 (MediaTek) JVET-L0230 (InterDigital) JVET-L0346 (MediaTek) |
| 13.3.2 | HEC with face based geometry padding extension of reference pictures | JVET-L0211 (RWTH Aachen University) |
| 13.3.3 | PERP with horizontal geometry padding of reference pictures | JVET-L0231 (InterDigital) |

## Test results

### Compared to PHEC

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.3.1.a | -1.61% | -1.10% | -1.06% | 103% | 120% |
| 13.3.1.b | -1.60% | -1.11% | -1.16% | 102% | 110% |
| 13.3.1.c | -0.97% | -0.52% | -0.46% | 102% | 105% |
| 13.3.1.NN | -1.37% | -0.92% | -0.88% | 102% | 107% |
| 13.3.2 | 0.29% | 4.59% | 3.46% | 99% | 322% |
| L0212 | 0.98% | 5.12% | 3.83% | 100% | 276% |

### Compared to HEC

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.3.1.a | -1.30% | -1.55% | -1.66% | 104% | 120% |
| 13.3.1.b | -1.29% | -1.56% | -1.76% | - | 110% |
| 13.3.1.c | -0.66% | -0.97% | -1.06% | 103% | 105% |
| 13.3.1.NN | -1.05% | -1.37% | -1.47% | 102% | 107% |

### Compared to PERP

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.3.3 | -0.22% | -0.29% | -0.38% | 101% | 98% |

## Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 13.3.1.a | P. Hanhart | 2 | F | Y | 102%/120% |  |
| 13.3.1.b | C.-H. Shih | 2 | F | Y | 102%/109% |  |
| 13.3.1.c | P. Hanhart | 2 | F | Y | 101%/106% |  |
| 13.3.1.NN | P. Hanhart | 2 | F | Y | 99%/109% |  |
| 13.3.2 | C.-H. Shih | 2 | F | Y | 102%/365% |  |
| 13.3.3 | C.-H. Shih | 2 | F | Y |  |  |
| L0212 |  |  |  |  |  |  |

Level: 1 simulation only, 2 code review + simulation, 3 independent implementation + simulation

Results: P partial results by spot-checking, F full results

Match: Y / N

EncT/DecT: M match in general, G gap observed

Comment: any information that might be helpful to the group

## Brief description of tests

### CE13.3.1 in JVET-L0346 (MediaTek) / JVET-L0230 (InterDigital)

The geometry padding in Figure 2 demonstrates a pixel position qb qbqboutside the Bottom face is represented by its spherical corresponding pixel qa, which is located at the Front face. This spherical corresponding pixel is obtained by using the HEC mapping functions. In Test 13.3.1.a, when the derived coordinate qa is fractional, 6-tap and 4-tap Lanczos interpolation filters are applied for the luminance and chrominance components, respectively. In Test 13.3.1.b, bilinear interpolation is applied for the luminance and chrominance components. In Test 13.3.1.c, the padding areas of sub-frame buffers are filled by duplicating spherical adjacent faces without any interpolation. In Test 13.3.1.NN, the interpolation method is replaced by nearest neighbor (NN) rounding.

Figure 2. Geometry padding process

To achieve the geometry padding in inter prediction, the proposed implementation maximally utilizes face continuity by alternatively constructing the top sub-frame and the bottom sub-frame for each reference frame data, as illustrated in Figure 3. Each sub-frame buffer has one 3x1 face region with an extended padding area. The padding size is the LCU size with additional 16 pixels, which yields the width of 144 pixels.

Figure 3. Top and bottom sub-frame buffers: 3x1 face region with geometry padding area

### CE13.3.2 in JVET-L0211 (RWTH Aachen University)

For a region of the size of a CTU around each face, pixels are projected from the neighboring faces. The corresponding pixel is found by projecting the pixel to the sphere and then to the location in the corresponding neighboring face using 360Lib. Lanczos 6-tap and 4-tap interpolation filters are used at the found location pB to generate the target pixel pA, for the luminance and chrominance components, respectively.

The extended faces are used as reference pictures instead of the original reference pictures. Thus, no modification of the en/decoder is required at block level.

To avoid a bug leading to a mismatch, ALF was disabled for this test.

### CE13.3.3 in JVET-L0231 (InterDigital)

The horizontal geometry padding for PERP is the same as for ERP, with the exception that the offset should be based on the unpadded ERP width instead of the picture width to account for the size of the padded regions. Figure 4 depicts the horizontal geometry padding for PERP. If a reference block is outside the left (right) reference picture boundary, it is replaced with the “wrapped-around” reference block shifted to the right (left) by the ERP width. The traditional repetitive padding is still employed in the vertical direction. The blending of the left and right padded regions is kept out of loop, as a post-processing operation.

Figure 4. Horizontal geometry padding for PERP.

# CE13.4: In-loop filters

## Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 13.4.1 | HEC with in-loop filters disabled across face discontinuities - Deblocking - Deblocking + SAO - Deblocking + ALF - All filters | JVET-L0347 (MediaTek) JVET-L0348 (MediaTek) JVET-L0232 (InterDigital) JVET-L0348 (MediaTek) |
| 13.4.2 | HEC with in-loop filters using spherical neighbors - Deblocking - Deblocking + SAO - Deblocking + ALF - All filters | JVET-L0349 (MediaTek) |
| 13.4.3 | HEC with in-loop filters using spherical neighbors | JVET-L0211 (RWTH Aachen University) |

## Test results

### Compared to PHEC

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.4.1.a | -0.57% | -0.01% | 0.11% | 101% | 99% |
| 13.4.1.b | -0.56% | -0.01% | 0.13% | 101% | 99% |
| 13.4.1.c | -0.57% | -0.55% | -0.13% | 99% | 103% |
| 13.4.1.d | -0.56% | -0.55% | -0.14% | 100% | 102% |
| 13.4.2.a | -0.62% | -0.31% | -0.20% | 99% | 100% |
| 13.4.2.b | -0.63% | -0.33% | -0.16% | 100% | 99% |
| 13.4.2.c | -0.64% | -0.90% | -0.47% | 100% | 101% |
| 13.4.2.d | -0.64% | -0.90% | -0.49% | 100% | 101% |
| 13.4.3 | 1.25% | 5.57% | 4.46% | 101% | 106% |
| L0212 | 2.13% | 6.36% | 5.21% | 102% | 92% |
| L0423.a | -0.59% | -0.51% | -0.06% | - | - |
| L0423.b |  |  |  |  |  |
| L0423.c | -0.58% | -0.60% | -0.17% | - | - |

### Compared to HEC

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.4.1.a | -0.26% | -0.46% | -0.49% | 101% | 99% |
| 13.4.1.b | -0.24% | -0.46% | -0.47% | 101% | 100% |
| 13.4.1.c | -0.25% | -1.00% | -0.73% | - | 103% |
| 13.4.1.d | -0.25% | -1.00% | -0.74% | 100% | 102% |
| 13.4.2.a | -0.31% | -0.76% | -0.80% | 100% | 100% |
| 13.4.2.b | -0.31% | -0.78% | -0.76% | 101% | 99% |
| 13.4.2.c | -0.33% | -1.34% | -1.07% | 101% | 101% |
| 13.4.2.d | -0.32% | -1.35% | -1.09% | 101% | 102% |
| L0423.a | -0.27% | -0.96% | -0.67% | - | - |
| L0423.b |  |  |  |  |  |
| L0423.c | -0.27% | -1.05% | -0.77% | - | - |

## Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 13.4.1.a | P. Hanhart | 2 | F | Y | 99%/99% |  |
| 13.4.1.b | P. Hanhart | 2 | F | Y | 99%/98% |  |
| 13.4.1.c | S.-Y. Lin | 2 | F | Y | 101%/102% |  |
| 13.4.1.d | P. Hanhart | 2 | F | Y | 99%/101% |  |
| 13.4.2.a |  |  |  |  |  |  |
| 13.4.2.b |  |  |  |  |  |  |
| 13.4.2.c |  |  |  |  |  |  |
| 13.4.2.d | Samsung | 2 | F | Y |  |  |
| 13.4.3 |  |  |  |  |  |  |
| L0212 |  |  |  |  |  |  |
| L0423.a |  |  |  |  |  |  |
| L0423.b |  |  |  |  |  |  |
| L0423.c |  |  |  |  |  |  |

Level: 1 simulation only, 2 code review + simulation, 3 independent implementation + simulation

Results: P partial results by spot-checking, F full results

Match: Y / N

EncT/DecT: M match in general, G gap observed

Comment: any information that might be helpful to the group

## Brief description of tests

### CE13.4.1 in JVET-L0347 (MediaTek) / JVET-L0348 (MediaTek) / JVET-L0232 (InterDigital)

### For the de-blocking filter, the filtering process is disabled when the filtered pixels cross the discontinuous edge.

For SAO, when the referenced pixels of current pixel in the SAO filtering process cross the discontinuous edge, the proposed modification disables the SAO filtering process of the current pixel to avoid inadequate filtering process.

For ALF, when filtering a CU, reconstructed sample positions used in filtering are clipped to be within the same side of the face discontinuity to which the current CU belongs.

### CE13.4.2 in JVET-L0349 (MediaTek)

#### De-blocking filter

Based on the concept of spherical continuity, the de-blocking filter fetches the spherical pixel positions located at another face for filtering when the filtered pixels cross the frame boundary (layout boundary) or the discontinuous edge as illustrated in Figure 5. The pixel data in the spherical positions are derived directly from the corresponding face and the boundary strength of the corresponding block edge is set to 2 for de-blocking filtering process.

Figure 5. Spherical pixel positions from another face for 360°-based de-blocking filter based on spherical continuity

#### SAO filter

When the referenced neighboring pixels of the current pixel in SAO filtering process cross the discontinuous edge, the proposed implementation uses the spherical pixel positions located at another face for SAO filtering as illustrated in Figure 6. Moreover, the pixel data in the spherical positions are copied directly from the corresponding face without any interpolation.

Figure 6. Spherical pixel positions from another face for 360°-based SAO filter based on spherical continuity

#### ALF

In ALF, during the determination of the filter coefficients or the filtering process, the accessed pixels crossing the frame boundary (layout boundary) or the discontinuous edge are changed to use the pixels data located at the spherical positions as illustrated in Figure 7.

Figure 7. Spherical pixel positions from another face for 360°-based ALF filter based on spherical continuity

### CE13.4.3 in JVET-L0211 (RWTH Aachen University)

For loop filters (DBF, SAO, ALF), the pixels accessed by the loop filter are changed if it operates across a face boundary. The correctly neighbors according to the 3D geometry are used instead of the neighbours in the coding format (Figure 8), although both can be the same in some cases. No reprojection of the pixels is performed.

Figure 8: Pixels accessed by loop filter across face boundaries (block in the middle) are changed such that the actual 3D neighbors (block at the right picture border) are used.

# CE13.5: Post-filtering

Dropped.

# CE13.6: Projection format adaptation

## Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 13.6.1 | HEC with adaptive frame packing | JVET-L0233 (InterDigital) |
| 13.6.2 | HEC with pre-rotation | JVET-L0075 (Samsung) |

## Test results

### Compared to PHEC

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.6.1 | -0.62% | -0.37% | -0.19% | 100% | 100% |
| 13.6.2 | -0.22% | -0.93% | -1.43% |  |  |

### Compared to HEC

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.6.1 | -0.31% | -0.82% | -0.79% | - | 100% |
| L0237 type-0 | -0.30% | 0.34% | 0.07% | 100% | 99% |
| L0237 type-1 | -0.32% | -0.55% | -0.74% | 99% | 99% |

## Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 13.6.1 | Samsung | 2 | F | Y |  |  |
| L0237 type-0 | C.-H Shih | 2 | F | Y |  |  |
| L0237 type-1 | C.-H. Shih | 2 | F | Y |  |  |
| 13.6.2 | P. Hanhart | 2 | F | Y | 101%/100% |  |

Level: 1 simulation only, 2 code review + simulation, 3 independent implementation + simulation

Results: P partial results by spot-checking, F full results

Match: Y / N

EncT/DecT: M match in general, G gap observed

Comment: any information that might be helpful to the group

## Brief description of tests

### CE13.6.1 in JVET-L0233 (InterDigital)

AFP periodically modifies the location of discontinuous face boundaries in the frame packed pictures to improve coding efficiency and reduce the visibility of face seam artifacts. AFP considers four frame packing configurations corresponding to yaw rotations of 0°, 90°, 180°, and 270°. Among the four combinations, the cost of each configuration is calculated based on gradient classification. For the first picture, the best combination is used. For the subsequent IRAP pictures, AFP alternates between the different configurations based on the cost of each combination computed for the given IRAP picture.

The frame packing parameters are determined at the encoder side at every IRAP and signaled in the PPS.

### CE13.6.2 in JVET-L0075 (Samsung)

# CE13.7: Combinations

## Test specification

| Test# | Description | Document# |
| --- | --- | --- |
| 13.7.1 | Padding and in-loop filters disabled across face discontinuities 1.1.a + 4.1.d | JVET-L0350 (MediaTek) |
| 13.7.2 | In-loop filters disabled across face discontinuities and post-filtering of seam artifacts - 4.1.d + post-filtering using HEVC deblocking with boundary strength=1 - 4.1.d + post-filtering using HEVC deblocking with boundary strength=2 | JVET-L-0422 (Zhejiang Univ.) Dropped |
| 13.7.3 | Intra prediction and all in-loop filters disabled across face discontinuities 2.1 + 4.1.d | JVET-L0351 (MediaTek) |
| 13.7.4 | Intra prediction disabled across face discontinuities, face-based padding of reference pictures, and all in-loop filters disabled across face discontinuities 2.1 + 3.1.c + 4.1.d | JVET-L0351 (MediaTek) |
| 13.7.5 | Intra prediction disabled across face discontinuities, face-based padding of reference pictures, all in-loop filters disabled across face discontinuities, and adaptive frame packing 2.1 + 3.1.c + 4.1.d + 6.1 | JVET-L0234 (InterDigital) |
| 13.7.6 | Intra prediction disabled across face discontinuities, face-based padding of reference pictures, all in-loop filters disabled across face discontinuities, adaptive frame packing, and post-filtering of seam artifacts 2.1 + 3.1.c + 4.1.d + 6.1 + post-filtering using HEVC deblocking with boundary strength=2 | JVET-L0235 (InterDigital) |
| 13.7.7 | Face extension of reference pictures and geometry corrected in-loop filters 3.2+ 4.3 | JVET-L0211 (RWTH Aachen University) |
| 13.7.8 | Intra prediction using spherical neighbors, geometry padding of reference pictures, and all in-loop filters using spherical neighbors - 2.2 + 3.1.a + 4.2.d - 2.2 + 3.1.b + 4.2.d | JVET-L0352 (MediaTek) JVET-L0236 (InterDigital) |

## Test results

### Compared to PHEC

|  | VTM (end-to-end WS-PSNR) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.7.1 | -0.02% | -0.26% | 0.01% | 99% | 102% |
| 13.7.2.a | -0.59% | -0.53% | -0.06% | - | - |
| 13.7.3 | -0.58% | -0.75% | -0.24% | 101% | 102% |
| 13.7.4 | -1.29% | -1.85% | -1.40% | 103% | 108% |
| 13.7.5 | -1.54% | -2.70% | -2.15% | 103% | 113% |
| 13.7.6 | -1.56% | -2.79% | -2.24% | 103% | 113% |
| 13.7.7 | 0.26% | 4.43% | 3.25% | 100% | 319% |
| L0212 | 0.99% | 5.02% | 3.85% | 101% | 298% |
| 13.7.8.a | -1.98% | -2.51% | -2.19% | 103% | 122% |
| 13.7.8.b | -1.98% | -2.47% | -2.17% | 103% | 114% |

|  | VTM (codec PSNR) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.7.1 | -0.31% | -1.50% | -1.16% | 99% | 102% |

### Compared to HEC

|  | VTM |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Test# | Y | U | V | EncT | DecT |
| 13.7.1 | 0.30% | -0.70% | -0.59% | 99% | 103% |
| 13.7.2.a | -0.28% | -0.98% | -0.67% | - | - |
| 13.7.3 | -0.27% | -1.20% | -0.84% | 101% | 102% |
| 13.7.4 | -0.98% | -2.29% | -2.00% | 103% | 108% |
| 13.7.5 | -1.23% | -3.14% | -2.74% | - | 113% |
| 13.7.6 | -1.24% | -3.23% | -2.83% | - | 113% |
| 13.7.8.a | -1.67% | -2.95% | -2.78% | 103% | 122% |
| 13.7.8.b | -1.67% | -2.91% | -2.76% | - | 114% |

## Crosscheck report

| Test# | Crosschecker | Level (1/2/3) | Results (P/F) | Match (Y/N) | EncT/DecT (M/G) | Comment |
| --- | --- | --- | --- | --- | --- | --- |
| 13.7.1 | P. Hanhart | 2 | F | Y | 100%/102% |  |
| 13.7.2.a | L. Liu | 2 | F | Y |  |  |
| 13.7.3 | P. Hanhart | 2 | F | Y | 100%/102% |  |
| 13.7.4 | P. Hanhart | 2 | F | Y | 103%/108% |  |
| 13.7.5 | S.-Y. Lin | 2 | F | Y | /109% |  |
| 13.7.6 | Samsung | 2 | F | Y |  |  |
| 13.7.7 |  |  |  |  |  |  |
| L0212 |  |  |  |  |  |  |
| 13.7.8.a | Samsung | 2 | F | Y |  |  |
| 13.7.8.b | Samsung | 2 | F | Y |  |  |

Level: 1 simulation only, 2 code review + simulation, 3 independent implementation + simulation

Results: P partial results by spot-checking, F full results

Match: Y / N

EncT/DecT: M match in general, G gap observed

Comment: any information that might be helpful to the group

## Brief description of tests

### CE13.7.1 in JVET-L0350 (MediaTek)

Padded HEC (PHEC) with in-loop filters disabled across face discontinuities (see Section 4.4.1).

### CE13.7.2 in JVET-L0422 (Zhejiang Univ.)

Post-fileting of discontinuous face boundaries using HEVC deblocking with boundary strength=1. In-loop filters disabled across face row discontinuous boundaries.

### CE13.7.3 in JVET-L0351 (MediaTek)

### Combination of tests 2.1 (see Section 2.4.1), and 4.1.d (see Section 4.4.1).CE13.7.4 in JVET-L0351 (MediaTek)

Combination of tests 2.1 (see Section 2.4.1), 3.1.c (see Section 3.4.1), and 4.1.d (see Section 4.4.1).

### CE13.7.5 in JVET-L0234 (InterDigital)

Combination of tests 2.1 (see Section 2.4.1), 3.1.c (see Section 3.4.1), 4.1.d (see Section 4.4.1), and 6.1 (see Section 6.4.1).

### CE13.7.6 in JVET-L0235 (InterDigital)

Combination of tests 2.1 (see Section 2.4.1), 3.1.c (see Section 3.4.1), 4.1.d (see Section 4.4.1), and 6.1 (see Section 6.4.1).

Post-filtering of face row boundaries using HEVC deblocking with boundary strength=2.

### CE13.7.7 in JVET-L0211 (RWTH Aachen University)

Combinations of tests 3.2 (see Section 3.4.2), and 4.3 (see Section 4.4.3).

### CE13.7.8 in JVET-L0352 (MediaTek) / JVET-L0236 (InterDigital)

CE13.7.8.a is a combination of tests 2.2 (see Section 2.4.1), 3.1.a (see Section 3.4.1), and 4.2.d (see Section 4.4.1).

CE13.7.8.b is a combination of tests 2.2 (see Section 2.4.1), 3.1.b (see Section 3.4.1), and 4.2.d (see Section 4.4.1).

# Related contributions

## CE13-related: Results for experiments as CE13.3.2, CE13.4.3 and CE13.7.7 with PHEC and impact of rotation on the coding performance of PHEC (JVET-L0212)

This document reports two experiment groups. 
In the first group 3 experiments which have been made similar to the CE13 sub-experiments experiments CE13.3.2, CE13.4.3 and CE13.7.7 are reported. Here, the same tools have been used on top of PHEC, while in the CE13 sub-experiments they were applied on top of HEC. It reports the results of geometry padding and adapted in-loop filters for 360° video as proposed initially in document JVET-J0023, implemented into the BMS-2.0.1 software. The VTM configuration was used for the anchor and experiments. 
CE13.3.2 investigates HEC with face based geometry padding of reference pictures. Using PHEC instead of HEC and compared to the BMS-2.0.1 reference, an average Y E2E WS-PSNR BD-rate loss of 0.95% for Random Access is obtained.
CE13.4.3 investigates HEC with in-loop filters using spherical neighbors. Using PHEC instead of HEC and compared to the BMS-2.0.1 reference, an average Y E2E WS-PSNR BD-rate loss of 2.10% for Random Access is obtained.
CE13.7.7 is the combination of CE13.3.2 and CE13.4.3. Using PHEC instead of HEC and compared to the BMS-2.0.1 reference, an average Y E2E WS-PSNR BD-rate loss of 0.95% for Random Access is obtained.
The second group of experiments investigates the impact of rotation on the coding performance of PHEC and padded EAC. Padding was added in a manner identical to PHEC to EAC. Both projection formats have been evaluated for “Roll” rotation ranging from 0 degrees to 315 degrees in steps of 45 degrees. Yaw and Pitch were kept constant to 0 degrees. PHEC outperforms padded EAC in most cases, but it should be noted that the performance difference does depend on the rotation. 
Further, the impact of rotation on the coding performance is evaluated, by comparing rotated PHEC against not rotated PHEC. There are losses of up to 4.78 % Y E2E WS-PSNR BD-rate, depending on the applied rotation. Of the investigated orientations, the original orientation is the best performing.

## CE13-related: Adaptive frame packing using chroma sample location type 1 (JVET-L0237)

HEC with adaptive frame packing (AFP) is studied in CE13 test 6.1 and can achieve {Y, U, V} BD-rate savings of {-0.31%, -0.82%, -0.79%} over HEC (JVET-L0233) for VTM configuration. The AFP periodically modifies the location of discontinuous face boundaries in the frame packed pictures to improve coding efficiency and reduce the visibility of face seam artifacts. When the frame packing configuration of the reference picture is different from that of the current picture, the reference picture needs to be first converted into the frame packing configuration of the current picture. This conversion involves the rotation of some of the HEC faces, which may create a misalignment between the luma and chroma sample locations, depending on the chroma sample location type. However, as reported in JVET-L0238, there are some problems in the way the 360Lib is handling chroma samples, especially when a face is rotated. This contribution reports the simulation results of AFP with the modifications proposed in JVET-L0238 for the 360Lib and the modifications proposed in JVET-L0239 for the cross-component linear model (CCLM) downsampling filter. For chroma sample location type 0, the BD-rate performance of AFP over HEC is {-0.30%, 0.34%, 0.07%}. For chroma sample location type 1 and modified CCLM downsampling filter, the BD-rate performance of AFP over HEC is {-0.32%, -0.55%, -0.74%}.

## CE13-related: HEC with in-loop filters using spherical neighbors (JVET-L0423)

HEC with in-loop filters using spherical neighbors is studied in this document. Blocks located at discontinuous face boundaries are filtered using HEVC deblocking filter. SAO and ALF are disabled in face row discontinuous boundaries. Three cases of difference boundary strength values are tested. There is no significant difference in coding performance between the three cases. In test a, the boundary strength is set to 1 and in test b the boundary strength is determined by prediction mode of blocks.

# References

- Y. Ye, J. Boyce, “Algorithm descriptions of projection format conversion and video quality metrics in 360Lib (Version 7)”, JVET-K1004, July 2018, Ljubljana, SI.