| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0043-v2 |
| --- | --- |

| Title: | AHG15: Hierarchical decoding property indications |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela | Email: | miska.hannuksela@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The contribution proposes profile_level( ) syntax structure that has the following parts:

- Profile indicator.

- Size of the extension that follows the constraint and extension flags. The extension contains the syntax elements whose presence are conditioned by the constraint and extension flags.

- Level indicator.

- A fixed number of flags for indicating constraints. These flags do not have an impact on the decoding process.

- A fixed number of flags for indicating extensions or otherwise affecting the decoding process.

- An extension part containing syntax elements are conditionally present depending on values of constraint and extensions flags.

The profile_level( ) syntax structure is proposed to be included at the beginning of the sequence parameter set syntax. Additionally, it is proposed to indicate profile_level( ) for an entire bitstream.

In addition, the contribution proposes:

- Including flags for bit depth and chroma format as flags affecting the decoding process in the profile_level( ) syntax structure.

- Including bit depth, chroma format, and separate colour plane syntax elements in the extension part of the profile_level( ) syntax structure.

- Excluding bit depth, chroma format, and separate colour plane syntax elements from the "base" syntax of the sequence parameter set.

# Motivation for the bit depth and chroma format related proposal parts

## Observations on HEVC constraint flags

Conformance of a bitstream to HEVC format range extensions profiles is indicated by general_profile_idc being equal to 4. Conformance of a bitstream to a particular HEVC format range extensions profile is indicated by a combination of constraint flag values that are present in the profile_tier_level( ) syntax structure (i.e., general_max_12bit_‌constraint_flag, general_max_10bit_‌constraint_flag, general_max_8bit_‌constraint_flag, general_max_422chroma_‌constraint_flag, general_max_420chroma_‌constraint_flag, general_max_monochrome_‌constraint_flag, general_intra_‌constraint_flag, general_one_picture_only_‌constraint_flag, and general_lower_bit_rate_‌constraint_flag). Our observations from the HEVC design are:

- Bit depth and chroma format indications within the profile_tier_level( ) syntax structure helped in avoiding a large number of defined general_profile_idc values for format range extensions profiles. Moreover, additional profiles for other combinations bit depth and chroma format could be specified in the future just by allowing other combinations of constraint flag values. Such flexibility seems desirable to be introduced in VVC already in version 1.

- Including the bit depth and chroma format indications in the profile-level syntax structure would avoid the need to repeat equivalent information elsewhere in the sequence parameter set.

- It seems unnecessary to be limited to flags only as opposed to multi-bit syntax elements in indicating bit depth and chroma format.

## Examples of bit depth constraints by external organizations

The specification for Video profiles for Television over 3GPP services (3GPP TS 26.116, available ftp://ftp.3gpp.org/tsg_sa/WG4_CODEC/Specs_update_after_SA81/26116-g00.zip) uses HEVC Main 10 profile but further constrains bit depths as follows:

Bitstreams conforming to the H.265/HEVC Full HD Operation Point shall be encoded with either 8 or 10 bit precision.

-	bit_depth_luma_minus8 = 0 or 2 (8 or 10 bits respectively)

-	bit_depth_chroma_minus8 = bit_depth_luma_minus8

Receivers conforming to the H.265/HEVC Full HD Operation Point shall support 8 bit and10 bit precision.

The above-specified limitations on bit_depth_luma_minus8 and bit_depth_chroma_minus8 are also present in the HEVC HDTV 10-bit IRD specified in the DVB Specification for the use of Video and Audio Coding in Broadcast and Broadband Applications (ETSI TS 101 154 v2.4.1, available http://www.etsi.org/deliver/etsi_ts/101100_101199/101154/02.04.01_60/ts_101154v020401p.pdf).

These examples give a reason to believe that external organizations might specify bit-depth and chroma format related sub-profiling also for VVC profiles.

# Proposal: hierarchical profile_level( ) syntax structure

## Overview

The profile_level( ) syntax structure is proposed to contain the following parts:

- Profile indicator.

5-bit fixed length code proposed as a starting point, subject to further study and pending further specification development.

- Size of the extension that follows the constraint and extension flags. The extension contains the syntax elements whose presence are conditioned by the constraint and extension flags.

It is proposed to indicate the size in units of 24 bits to provide both byte alignment and alignment for base64 encoding, which could be used when representing the syntax structure as a value of a MIME parameter.

- Level indicator.

8-bit fixed length code proposed as a starting point, subject to further study and pending further specification development.

- A fixed number of flags for indicating constraints. These flags do not have an impact on the decoding process.

48 flags are proposed as a starting point, subject to further study and pending further specification development.

Examples of constraint flags that could be specified are general_progressive_source_flag, general_interlaced_source_flag, general_non_packed_constraint_flag, and general_frame_only_constraint_flag, as specified in HEVC.

- A fixed number of flags for indicating extensions or otherwise affecting the decoding process.

48 flags are proposed as a starting point, subject to further study and pending further specification development.

- An extension part containing syntax elements are conditionally present depending on values of constraint and extensions flags.

A single extension part containing both syntax elements gated by extension flags and syntax elements gated by constraint flags is proposed to reduce the number of stuffing bits when compared to two separate extensions parts, one controlled by extension flags and another controlled by constraint flags.

Syntax elements for extensions precede syntax elements for constraints. Consequently, decoders may omit decoding of the remaining of the extension part when all syntax elements conditioned by the extension flags have been decoded. In other words, decoders are neither required to interpret the constraint flags, nor the extension syntax elements whose presence is conditioned by the constraint flags.

Bit depth, chroma format, and separate colour plane indications are proposed to be included and coded as fixed-length code words. The bit depth and chroma format syntax elements are proposed to affect normative decoder operation, i.e., they are not repeated elsewhere in the sequence parameter set.

Motivation:

- This design enables grouping of switchable elements (see JVET-L0042 as an example) and including syntax elements of any length (i.e. not just flags).

- This design is asserted be forward compatible: If a later version of VVC adds constraint flags and syntax elements whose presence is conditioned by those constraint flags, VVC v1 decoder will be able to omit those syntax elements in the profile_level( ) structure.

## Proposed syntax of profile_level( )

| profile_level( ) { | Descriptor |
| --- | --- |
| profile_idc | u(5) |
| reserved_zero_3bits | u(3) |
| num_24bit_extensions | u(4) |
| reserved_zero_4bits | u(4) |
| level_idc | u(8) |
| reserved_zero_constraint_48bits | u(48) |
| reserved_zero_dec_48bits | u(48) |
| if( num24bit_extensions > 0 ) { |  |
| startBitPos = bitstream_pointer_pos( ) |  |
| endBitPos = bitstream_pointer_pos( ) |  |
| numStuffingBits = num_24bit_extensions * 24 − ( endBitPos − startBitPos ) |  |
| for( i = 0; i < numStuffingBits; i++ ) |  |
| reserved_zero_bit | u(1) |
| } |  |
| } |  |

## Proposed semantics of profile_level( )

The profile_level( ) syntax structure is proposed to be included at the beginning of the sequence parameter set syntax and indicate the following:

- profile_idc and level_idc specify the profile and level, respectively, that the CVS conforms to.

- Constraint flags and the syntax elements gated by the constraint flags specify the constraints obeyed in the CVS.

- Extension flags and the syntax elements gated by the extension flags may affect the decoding of the CVS.

It is proposed to enable the use of the profile_level( ) syntax structure for indicating bitstream properties (e.g. by including it in a decoding parameter set that is active for the entire bitstream). The semantics of the profile_level( ) are then as follows:

- profile_idc and level_idc specify the profile and level, respectively, of a decoder that is capable of decoding the bitstream.

- Constraint flags and the syntax elements gated by the constraint flags specify the constraints obeyed in the bitstream.

- The semantics for extension flags and the syntax elements gated by the extension flags is specified separately for each syntax element. For example, a bit depth syntax element may indicate that the maximum bit depth applied in the entire bitstream. A restricting syntax element, such as deblock_restrictions of JVET-L0042, equal to 0 may indicate that restrictions are not applied in any CVS of the bitstream. When equal to 1 they may indicate that restrictions may be applied in one or more CVSs of the bitstream.

num_24bit_extensions * 24 + 120 specifies the number of bits in the profile_level( ) syntax structure.

The syntax function bitstream_pointer_pos( ) returns the bit position of the bitstream pointer after the removal of start code emulation prevention bytes. The bitstream pointer of the first bit of the bitstream, in decoding order, is equal to 0.

# Proposal: bit depth and chroma format in the profile_level( ) syntax structure

## General

Bit depth, chroma format, and separate colour plane indications are proposed to be included and coded as fixed-length code words in profile_level( ).

Two alternatives are outlined as follows:

- The bit depth and chroma format syntax elements affecting decoding process. Thi

- s option has the following ingredients:

- Including flags for bit depth and chroma format as flags affecting the decoding process in the profile_level( ) syntax structure.

- Including bit depth, chroma format, and separate colour plane syntax elements in the extension part of the profile_level( ) syntax structure. When indicated for the entire bitstream, the bit depth and chroma format indicate maximums. When specified in SPS, the bit depth and chroma format specify the values in use in the CVS.

- Excluding bit depth, chroma format, and separate colour plane syntax elements from the "base" syntax of the sequence parameter set.

- The bit depth and chroma format syntax elements indicated as constraints. This option has the following ingredients:

- Including flags for bit depth and chroma format as constraint flags in the profile_level( ) syntax structure.

- Including bit depth, chroma format, and separate colour plane syntax elements in the extension part of the profile_level( ) syntax structure.

- Keeping bit depth, chroma format, and separate colour plane syntax elements in the "base" syntax of the sequence parameter set.

The specification text for alternative 1 is included below. The proponent prefers alternative 1.

## Proposed syntax of profile_level( )

| profile_level( ) { | Descriptor |
| --- | --- |
| profile_idc | u(5) |
| reserved_zero_3bits | u(3) |
| num_24bit_extensions | u(4) |
| reserved_zero_4bits | u(4) |
| level_idc | u(8) |
| reserved_zero_constraint_48bits | u(48) |
| bit_depth_flag | u(1) |
| chroma_format_flag | u(1) |
| reserved_zero_dec_46bits | u(46) |
| if( num24bit_extensions > 0 ) { |  |
| startBitPos = bitstream_pointer_pos( ) |  |
| if( bit_depth_flag ) { |  |
| luma_bit_depth_minus8 | u(4) |
| chroma_bit_depth_minus8 | u(4) |
| } |  |
| if( chroma_format_flag ) |  |
| pl_chroma_format_idc | u(2) |
| endBitPos = bitstream_pointer_pos( ) |  |
| numStuffingBits = num_24bit_extensions * 24 − ( endBitPos − startBitPos ) |  |
| for( i = 0; i < numStuffingBits; i++ ) |  |
| reserved_zero_bit | u(1) |
| } |  |
| } |  |

## Proposed semantics of profile_level( )

bit_depth_flag equal to 0 specifies that luma_bit_depth_minus8 and chroma_bit_depth_minus8 are inferred to be equal to 0. bit_depth_flag equal to 1 specifies that luma_bit_depth_minus8 and chroma_bit_depth_minus8 are present.

chroma_format_flag equal to 0 specifies that ChromaFormatIdc is equal to 1 and SeparateColourPlaneFlag is equal to 0. chroma_format_flag equal to 1 specifies that pl_chroma_format_idc is present.

When bit_depth_flag or chroma_format_flag is equal to 1, num_24bit_extensions shall be greater than 0.

pl_chroma_format_idc specifies the chroma sampling relative to the luma sampling and whether the three colour components of the 4:4:4 chroma format are coded separately, as specified below. The value of chroma_format_idc shall be in the range of 0 to 3, inclusive.

| pl_chroma_format_idc | ChromaFormatIdc | SeparateColourPlaneFlag | Chroma format | SubWidthC | SubHeightC |
| --- | --- | --- | --- | --- | --- |
| 0 | 0 | 0 | Monochrome | 1 | 1 |
| not present | 1 | 0 | 4:2:0 | 2 | 2 |
| 1 | 2 | 0 | 4:2:2 | 2 | 1 |
| 2 | 3 | 0 | 4:4:4 | 1 | 1 |
| 3 | 3 | 1 | 4:4:4 | 1 | 1 |

It is a requirement for bitstream conformance that numStuffingBits is greater than or equal to 0.

[Ed. (MH): In the current VVC draft text occurrences of separate_colour_plane_flag are to be replaced with SeparateColourPlaneFlag and occurrences of chroma_format_idc are to be replaced with ChromaFormatIdc.]

The semantics of other syntax elements is unchanged compared to the VVC draft.

## Sketch of further constraints or extensions specified in profile_level( )

The syntax structure below illustrates an imaginary example how the profile_level( ) syntax structure would change if one additional constraint flag (constraint_example_flag) and one additional extension flag (dec_param_example_flag) were specified either in a later phase of VVC v1 development or in a later version of VVC and if both of these imaginary flags also controlled the presence of additional syntax elements in the extension part of the profile_level( ) syntax structure.

| profile_level( ) { | Descriptor |
| --- | --- |
| profile_idc | u(5) |
| reserved_zero_3bits | u(3) |
| num_24bit_extensions | u(4) |
| reserved_zero_4bits | u(4) |
| level_idc | u(8) |
| constraint_example_flag | u(1) |
| reserved_zero_constraint_47bits | u(47) |
| bit_depth_flag | u(1) |
| chroma_format_flag | u(1) |
| dec_param_example_flag | u(1) |
| reserved_zero_dec_45bits | u(45) |
| if( num24bit_extensions > 0 ) { |  |
| startBitPos = bitstream_pointer_pos( ) |  |
| if( bit_depth_flag ) { |  |
| luma_bit_depth_minus8 | u(4) |
| chroma_bit_depth_minus8 | u(4) |
| } |  |
| if( chroma_format_flag ) |  |
| pl_chroma_format_idc | u(2) |
| if( dec_param_example_flag ) { |  |
| // any extension-specific syntax elements |  |
| } |  |
| /* similar if clause for all specified extension flags with additional syntax elements would be added here */ |  |
| if( constraint_example_flag ) { |  |
| // any syntax elements giving properties for the constraint |  |
| } |  |
| /* similar if clause for all specified constraint flags with additional syntax elements would be added here */ |  |
| endBitPos = bitstream_pointer_pos( ) |  |
| numStuffingBits = 144 − ( endBitPos − startBitPos ) |  |
| for( i = 0; i < numStuffingBits; i++ ) |  |
| reserved_zero_bit | u(1) |
| } |  |
| } |  |

## Proposed changes in sequence parameter set syntax

The sequence parameter set syntax structure is proposed to:

- Start with a profile_level( ) syntax structure. The same design choice was also made in HEVC. This provides access to the interoperability indications prior to variable-length coded syntax elements or syntax elements not necessary for interoperability considerations.

- Exclude bit depth, chroma format, and separate colour plane syntax elements.

The proposed changes are indicated by red font colour below.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| profile_level( ) |  |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| qtbtt_dual_tree_intra_flag | ue(v) |
| log2_ctu_size_minus2 | ue(v) |
| log2_min_qt_size_intra_slices_minus2 | ue(v) |
| log2_min_qt_size_inter_slices_minus2 | ue(v) |
| max_mtt_hierarchy_depth_inter_slices | ue(v) |
| max_mtt_hierarchy_depth_intra_slices | ue(v) |
| sps_cclm_enabled_flag | ue(1) |
| sps_mts_intra_enabled_flag | ue(1) |
| sps_mts_inter_enabled_flag | ue(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).