| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0055-v1 |
| --- | --- |

| Title: | CE4-related: Redundant Removal for ATMVP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Anish Tamse Min Woo Park Seungsoo Jeong Kiho Choi Seoul R&D Campus 34, Seongchon-gil, Seocho-gu Seoul, 06765, Korea | Email: | tamse.anish@samsung.com m.w.park@samsung.com ss00.jeong@samsung.com kiho14.choi@samsung.com |
| Source: | Samsung Electronics Co., Ltd. |  |  |

_____________________________

# Abstract

In this contribution, it is proposed to remove a redundant candidate in merge candidate list construction process. A sub-block size for ATMVP candidate can be either 4x4 or 8x8, but although ATMVP candidate is based on sub-block prediction, CU size can be smaller than or equal to the sub-block size (i.e. ATMVP candidate is not a sub-block based prediction anymore). And also ATMVP candidate can be duplicated with TMVP candidate. Moreover, since the derivation process for ATMVP candidate is relatively more complex than derivation processes for other merge candidates, it can increase complexity of smaller CUs such as 4x4 and 8x8. Therefore, this contribution suggests several options to remove the redundancy: 1) Disallow ATMVP when CU size is equal to 4x4, 2) Disallow ATMVP when CU size is less than or equal to the sub-block size, 3) Disallow ATMVP when CU size is less than 8x8, and 4) Disallow ATMVP when width or height of CU is less than the sub-block size. The proposed methods reportedly show no performance change.

# Introduction

In VVC [1], ATMVP candidate is a temporal merge candidate based on sub-block prediction. A sub-block size for ATMVP can be either 4x4 or 8x8, which is decided by slice level signalling. But, sometimes ATMVP is allowed for CUs smaller than or equal to the sub-block size. In this case, ATMVP candidate is not a sub-block based prediction anymore and may be duplicated with TMVP candidate. Moreover, the derivation process for ATMVP candidate is relatively more complex than derivation processes for other merge candidates. It basically consists of 3 steps: 1) temporal motion vector search process from four neighboring blocks, 2) corresponding motion vector derivation process, and 3) sub-block wise motion vector derivation process. This can cause complexity increase of smaller CUs such as 4x4, 4x8, 8x4, and 8x8. Therefore, in order to remove the redundancy and to reduce complexity for smaller CUs, this contribution suggests several methods.

# Proposed Methods

Proposed methods are basically to disallow ATMVP candidates for smaller CUs. So, this proposal suggests following options:

- Method 1) Disallow ATMVP when CU size is equal to 4x4

- Method 2) Disallow ATMVP when CU size is less than or equal to the sub-block size

- Method 3) Disallow ATMVP when CU size is less than or equal to 8x8

- Method 4) Disallow ATMVP when width or height of CU is less than sub-block size

In case of Method 4, modifications can be represented as following working draft changes. Changes are highlighted in green and strikethrough.

- Derivation process for subblock-based temporal merging candidates ... The availability flag availableFlagSbCol is derived as follows. - If either slice_temporal_mvp_enable_flag or sps_sbtmvp_flag is equal to 0, availableFlagSbCol is set equal to 0. - Otherwise, if cbWidth <= (1 << Log2SbtmvpSize) or cbHeight <= (1 << Log2SbtmvpSize), availableFlagSbCol is set equal to 0. - Otherwise, the following ordered steps apply. - The location ( xCtb, yCtb ) of the top-left sample of the luma coding tree block that contains the current coding block and the location ( xCtr, yCtr ) of the below-right center sample of the current luma coding block are derived as follows: xCtb = ( xCb >> CtuLog2Size ) << CtuLog2Size (8-144) yCtb = ( yCb >> CtuLog2Size ) << CtuLog2Size (8-145) xCtr = xCb + ( cbWidth / 2 ) (8-146) yCtr = yCb + ( cbHeight / 2 ) (8-147) - The luma location ( xColCtrCb, yColCtrCb ) is set equal to the top-left sample of the collocated luma coding block covering the location given by ( xCtr, yCtr ) inside ColPic relative to the top-left luma sample of the collocated picture specified by ColPic. - The derivation process for subblock-based temporal merging base motion data as specified in clause 8.3.2.5 is invoked with the location ( xCtb, yCtb ), the location ( xColCtrCb, yColCtrCb ), the availability flags availableFlagA0, availableFlagA1, availableFlagB0 and availableFlagB1, and the prediction list utilization flags predFlagLXA0, predFlagLXA1, predFlagLXB0 and predFlagLXB1, and the reference indices refIdxLXA0, refIdxLXA1, refIdxLXB0 and refIdxLXB1, and the motion vectors mvLXA0, mvLXA1, mvLXB0 and mvLXB1, with X being 0 and 1 as inputs and the motion vectors ctrMvLX, the prediction list utilization flags ctrPredFlagLX and the reference indices ctrRefIdxLX of the collocated block, with X being 0 and 1, and the temporal motion vector tempMV as outputs. - The variable availableFlagSbCol is derived as follows: - If both ctrPredFlagL0 and ctrPredFlagL1 are equal to 0, availableFlagSbCol is set equal to 0. - Otherwise, availableFlagSbCol is set equal to 1. When availableFlagSbCol is equal to 1, the following applies: - The variables numSbX, numSbY, sbWidth, sbHeight and refIdxLXSbCol are derived as follows: numSbX = max( cbWidth >> Log2SbtmvpSize, 1 ) (8-148) numSbY = max( cbHeight >> Log2SbtmvpSize, 1 ) (8-149) sbWidth = cbWidth / numSbX (8-150) sbHeight = cbHeight / numSbY (8-151) refIdxLXSbCol = 0 (8-152) …

The WD changes for methods 1-3 at the same locations, which are summarized below.

Method 1)

- Derivation process for subblock-based temporal merging candidates ... - Otherwise, if cbWidth == 4 and cbHeight == 4, availableFlagSbCol is set equal to 0. ... - The variables numSbX, numSbY, sbWidth, sbHeight and refIdxLXSbCol are derived as follows: numSbX = max( cbWidth >> Log2SbtmvpSize, 1 ) (8-148) numSbY = max( cbHeight >> Log2SbtmvpSize, 1 ) (8-149)

Method 2)

- Derivation process for subblock-based temporal merging candidates ... - Otherwise, if cbWidth <= (1 << Log2SbtmvpSize) and cbHeight <= (1 << Log2SbtmvpSize), availableFlagSbCol is set equal to 0. ... - The variables numSbX, numSbY, sbWidth, sbHeight and refIdxLXSbCol are derived as follows: numSbX = max( cbWidth >> Log2SbtmvpSize, 1 ) (8-148) numSbY = max( cbHeight >> Log2SbtmvpSize, 1 ) (8-149)

Method 3)

- Derivation process for subblock-based temporal merging candidates ... - Otherwise, if cbWidth <= 8 and cbHeight <= 8, availableFlagSbCol is set equal to 0. ... - The variables numSbX, numSbY, sbWidth, sbHeight and refIdxLXSbCol are derived as follows: numSbX = max( cbWidth >> Log2SbtmvpSize, 1 ) (8-148) numSbY = max( cbHeight >> Log2SbtmvpSize, 1 ) (8-149)

# Experimental Results

The simulation was carried out under the JVET common test condition [2]. The software was compiled with GCC version 7.3 and the AVX SIMD instructions. Table 1 shows summary of all results.

Table 1. Summary of all results

|  | Test | BD-Rate (Y) | BD-Rate (U) | BD-Rate (V) | EncT | DecT |
| --- | --- | --- | --- | --- | --- | --- |
| RA | Method 1 | 0.00% | 0.01% | -0.03% | 101% | 99% |
|  | Method 2 | 0.00% | 0.01% | -0.03% | 100% | 99% |
|  | Method 3 | 0.01% | 0.00% | -0.04% | 100% | 100% |
|  | Method 4 | 0.01% | 0.01% | 0.00% | 99% | 100% |
| LDB | Method 1 | -0.02% | -0.09% | -0.15% | 100% | 99% |
|  | Method 2 | -0.03% | -0.17% | -0.20% | 100% | 100% |
|  | Method 3 | -0.03% | -0.21% | -0.23% | 100% | 100% |
|  | Method 4 |  |  |  |  |  |

Table 2. Results for Method 1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | -0.01% | 100% | 99% |
| Class A2 | -0.01% | 0.04% | -0.04% | 101% | 100% |
| Class B | 0.01% | -0.03% | -0.05% | 101% | 99% |
| Class C | -0.01% | 0.04% | -0.02% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.03% | 101% | 99% |
| Class D | 0.01% | -0.13% | -0.02% | 101% | 100% |
| Class F (optional) | -0.01% | 0.01% | 0.00% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.16% | -0.29% | 100% | 98% |
| Class C | 0.00% | -0.17% | -0.03% | 101% | 99% |
| Class E | -0.11% | 0.15% | -0.07% | 100% | 100% |
| Overall | -0.02% | -0.09% | -0.15% | 100% | 99% |
| Class D | -0.06% | 0.71% | -0.43% | 100% | 99% |
| Class F (optional) | 0.02% | 0.58% | 0.37% | 100% | 99% |

Table 3. Results for Method 2

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | -0.02% | -0.03% | 100% | 98% |
| Class A2 | 0.00% | 0.02% | -0.02% | 100% | 99% |
| Class B | 0.00% | 0.02% | -0.01% | 100% | 100% |
| Class C | -0.01% | 0.02% | -0.05% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.01% | -0.03% | 100% | 99% |
| Class D | 0.02% | -0.13% | -0.03% | 101% | 100% |
| Class F (optional) | 0.00% | 0.01% | 0.00% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | -0.11% | -0.27% | 101% | 99% |
| Class C | 0.02% | -0.19% | -0.13% | 100% | 99% |
| Class E | -0.14% | -0.23% | -0.19% | 100% | 101% |
| Overall | -0.03% | -0.17% | -0.20% | 100% | 100% |
| Class D | -0.06% | 0.71% | -0.43% | 100% | 99% |
| Class F (optional) | -0.07% | 0.38% | 0.24% | 101% | 102% |

Table 4. Results for Method 3

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.01% | 0.01% | 100% | 100% |
| Class A2 | 0.00% | -0.04% | -0.12% | 100% | 100% |
| Class B | 0.01% | -0.01% | -0.02% | 100% | 100% |
| Class C | 0.00% | 0.04% | -0.03% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.00% | -0.04% | 100% | 100% |
| Class D | -0.01% | 0.02% | 0.03% | 100% | 100% |
| Class F (optional) | -0.02% | -0.04% | -0.03% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.02% | -0.10% | -0.24% | 100% | 100% |
| Class C | 0.01% | -0.26% | -0.13% | 100% | 98% |
| Class E | -0.16% | -0.34% | -0.35% | 100% | 101% |
| Overall | -0.03% | -0.21% | -0.23% | 100% | 100% |
| Class D | 0.00% | 0.14% | -0.26% | 101% | 101% |
| Class F (optional) | -0.08% | 0.39% | 0.01% | 100% | 100% |

Table 5. Results for Method 4

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | 0.01% | 0.00% | 0.01% | 99% | 100% |
| Class C | 0.03% | 0.04% | 0.01% | 99% | 98% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.00% | 0.03% | 0.03% | 100% | 98% |
| Class F (optional) | 0.01% | 0.00% | 0.01% | 100% | 99% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | 0.02% | 0.03% | 0.04% | 100% | 97% |
| Class E | -0.10% | -0.29% | 0.28% | 100% | 100% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | 0.01% | 0.26% | -0.03% | 100% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

In this contribution, it is proposed to disallow ATMVP for smaller CUs to remove the redundancy and to reduce the complexity. All the proposed methods show no coding performance change. Therefore, it is suggested to adopt proposed methods into the next VVC working draft text.

# References

[1] “Versatile Video Coding (Draft 2),” JVET-K1001, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

[2] “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11, Ljubljana, SI, July 2018.

# Patent rights declaration(s)

Samsung Electronics Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).