| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, MO, 03–12 Oct. 2018 | Document: JVET-L0066-v1 |
| --- | --- |

| Title: | CE3-related: One-line MMLM for reduction of reference sample lines |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Name(s) Jongseok Lee Joohyung Byeon Seane Park Donggyu Sim | Email: | suk2080@kw.ac.kr, quswngud3@kw.ac.kr, psea1118@kw.ac.kr dgsim@kw.ac.kr |
| Source: | Kwangwoon University (KWU) |  |  |

_____________________________

- Abstract

This contribution proposes a modified Multi Model Linear Model (MMLM) which uses one-line buffer for luminance in down-sampling process. The MMLM mode in BMS-2.0.1 with a macro as ‘--LMChroma=2’ is required four neighboring reconstructed luminance sample lines in the down-sampling process. The proposed method uses one reconstructed luminance samples line for above side in order to reduce line buffer. Experimental results show an average BD-rate gain 0.27%, 2.38% and 2.23% for three components on average over BMS-2.0.1 with vtm_config and ‘--LMChroma=2’, respectively.

Introduction

In BMS-2.0.1 with the macro which is ‘--LMChroma=2’, MMLM (Multi Model Linear Model) mode downsample four neighboring reconstructed luminance sample lines to two lines in the down-sampling process. This contribution proposes one-line MMLM that uses one reconstructed luminance sample line at the upper adjacent to the current PU in order to reduce a luminance line buffer.

One-line Multi Model Cross Components Linear Model

The down-sampled luminance sample value (RecL') for neighboring and current reconstructed luminance samples is computed as

RecL'x,y=(RecL2x-1, -2 + 2∙RecL2x, -2 + RecL2x+1,-2  
                                                + RecL2x-1, -1 + 2∙RecL2x,-1 + RecL2x+1, -1+4)≫3,

0≤x<W,  0<y≤-2.

If the top-left PU of the current PU is not available, the down-sampled luminance sample value at the point
[0, y] is computed as

RecL'0,y=RecL0, 2y         +    RecL0, 2y-1 + 1  ≫1, 0<y≤-2.

Where, RecLx,y denotes reconstructed luminance sample value. x and y are the coordinate of chrominance samples in 4:2:0 sub-sampling condition and a coordinate [0, 0] is the left top point of a current chroma prediction unit. W and H are width and height of the chroma prediction unit, respectively.

In the proposed MMLM method, the down-sampled luminance samples for above reconstructed sample line are computed by

RecL'x,-1=RecL2x-1, -1 + 2∙RecL2x, -1  + RecL2x+1, -1+2≫2,  0<x<W.

If the left PU of the current PU is not available, the down-sampled luminance sample value computed as

RecL'0,-1=3∙RecL0, -1+RecL1,  -1+2≫2.

Four left reconstructed lines are also used in computing the CCLM and MMLM parameters, as the CCLM of BMS 2.0.1. For chroma sample prediction, the same algorithm to CCLM of BMS 2.0.1 is used, as shown in Fig. 1.

Figure 1. Diagram of the proposed One-line MMLM method

Results

The proposed One-line MMLM is implemented within BMS-2.0.1 software with VTM config. The following tables show BD-rate coding performance compare to BMS-2.0.1 with VTM config. in All Intra configuration. Table 1 shows RD performance and computation time of VTM-2.0.1 with MMLM and Cb to Cr residual prediction. Table 2 shows RD performance and computation time of VTM-2.0.1 with one-line MMLM and Cb to Cr residual prediction. 0.44%, 4.07% and 3.83% BD gains are found with one luminance line buffer at the upper adjacent to the current PU. Table 3 shows RD performance and computation time of VTM-2.0.1 with MMLM. Table 4 shows RD performance and computation time of VTM-2.0.1 with one-line MMLM. 0.27%, 2.38% and 2.23% BD gains are found with one luminance line buffer. It is suggested to consider the proposed MMLM mode for the potential adoption into BMS.

- Table 1. RD performance of VTM-2.0.1+MMLM+CbtoCr

|  | All Intra Main10 |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Reference: BMS-2.01_vtm | Over VTM-2.0.1 |  |  |  |  |  |  |  |
| Tested: One-line-CCLM | Y |  | U |  | V | EncT |  | DecT |
| Class A1 | -0.91% | -4.41% |  | -5.61% |  | 103% | 101% |  |
| Class A2 | -0.50% | -4.00% |  | -1.62% |  | 106% | 101% |  |
| Class B | -0.34% | -4.35% |  | -5.12% |  | 102% | 100% |  |
| Class C | -0.49% | -5.18% |  | -4.55% |  | 104% | 100% |  |
| Class E | -0.31% | -4.52% |  | -3.89% |  | 114% | 100% |  |
| Overall | -0.49% | -4.52% |  | -4.29% |  | 105% | 100% |  |
| Class D | -0.32% | -4.76% |  | -3.89% |  | 106% | 99% |  |
| Class F (optional) | -0.62% | -4.83% |  | -4.39% |  | 102% | 100% |  |

- Table 2. RD performance of VTM-2.0.1+One-line MMLM+CbtoCr

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference: VTM-2.01 | Over VTM-2.0.1 |  |  |  |  |
| Tested: One-line MMLM | Y | U | V | EncT | DecT |
| Class A1 | -0.81% | -3.40% | -4.91% | 97% | 101% |
| Class A2 | -0.45% | -3.66% | -1.32% | 102% | 101% |
| Class B | -0.32% | -3.99% | -4.55% | 102% | 100% |
| Class C | -0.42% | -4.71% | -3.98% | 105% | 100% |
| Class E | -0.30% | -4.42% | -3.83% | 114% | 100% |
| Overall | -0.44% | -4.07% | -3.83% | 104% | 100% |
| Class D | -0.29% | -4.17% | -3.58% | 106% | 98% |
| Class F (optional) | -0.50% | -3.86% | -3.41% | 102% | 100% |

- Table 3. RD performance of VTM-2.0.1+ MMLM

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference: BMS-2.01_vtm | Over VTM-2.0.1 |  |  |  |  |
| Tested: One-line MMLM | Y | U | V | EncT | DecT |
| Class A1 | -0.86% | -3.76% | -5.15% | 105% | 102% |
| Class A2 | -0.20% | -2.46% | -1.01% | 107% | 101% |
| Class B | -0.24% | -3.14% | -3.34% | 103% | 102% |
| Class C | -0.33% | -3.20% | -2.89% | 107% | 100% |
| Class E | -0.07% | -1.53% | -0.72% | 116% | 101% |
| Overall | -0.33% | -2.87% | -2.72% | 107% | 101% |
| Class D | -0.14% | -2.23% | -2.16% | 107% | 106% |
| Class F (optional) | -0.45% | -3.07% | -2.93% | 103% | 101% |

- Table 4. RD performance of VTM-2.0.1+One-line MMLM

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference: BMS-2.01_vtm | Over VTM-2.0.1 |  |  |  |  |
| Tested: One-line MMLM | Y | U | V | EncT | DecT |
| Class A1 | -0.75% | -2.67% | -4.49% | 98% | 100% |
| Class A2 | -0.13% | -2.04% | -0.73% | 103% | 102% |
| Class B | -0.21% | -2.73% | -2.75% | 101% | 102% |
| Class C | -0.24% | -2.58% | -2.22% | 108% | 101% |
| Class E | -0.06% | -1.56% | -0.66% | 101% | 100% |
| Overall | -0.27% | -2.38% | -2.23% | 102% | 101% |
| Class D | -0.13% | -1.81% | -1.77% | 113% | 104% |
| Class F (optional) | -0.32% | -1.88% | -1.89% | 108% | 102% |

Patent rights declaration(s)

Kwangwoon university may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).