| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0068 |
| --- | --- |

| Title: | DC prediction based Local Illuminance Compensation (LIC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jongseok Lee, Juntaek Park, Hansol Choi, Seanae Park, Donggyu Sim, Gun Bang, Huiyong Kim | Tel: Email: | +821054957422 juntaek@kw.ac.kr |
| Source: | Kwangwoon University and ETRI |  |  |

_____________________________

# Abstract

This contribution proposes an DC-based LIC (Local Illuminance Compensation) for lower complexity and alleviation of pipeline issue of the LIC in BMS 2.0.1. The LIC in BMS 2.0.1 employs a linear model by using the neighboring reconstructed samples of the current and corresponded blocks. With the linear model, the prediction signal for the current block is computed. To compute the linear model parameters, all the previous neighboring CUs have been reconstructed, thus, the prediction for the current block cannot start. In this contribution, we provide a method to alleviate the pipeline issue by computing the LIC parameter using only the residual signal not the reconstructed neighboring samples. Note that only the offset parameter not scale parameter is employed in the proposed method. With the proposed method, coding gains of -0.03% in the random-access configuration at Class C is observed.

# Introduction

LIC is performed to produce a predicted signal by compensating the illumination difference between a current CU and its reference CU. As shown in Fig. 1, L-shape reconstructed samples adjacent to the current CU and its reference CU are used to compute the linear model parameters for illumination compensation. After reconstruction of the L-shape adjacent to the current block, the parameter estimation of the linear model can start. The inter block coding/decoding with the LIC cannot be implemented in pipeline platforms.

Figure 1. Current block for local illumination compensation and its reconstructed reference samples

To solve this, K0118 [1] proposes to save the scale and offset parameters of the LIC after CU reconstruction, as shown in Fig. 2. The stored parameters can be used for LIC of adjacent blocks.

Figure 2. Parameter computation process of K0118

However, this method still has a problem that it cannot proceed before the processing of the neighbouring CU is all finished. Figure 3 shows example of a pipeline problem in binary tree case even with the stored parameters. The #2 CU cannot start to decode before computation and decoding of #1 CU. Additionally, an additional memory is required to store α (scale parameter) and β (offset parameter).

Fig. 3. A case of LIC pipeline issue for adjacent CUs

This contribution proposes an DC-based LIC method to alleviates LIC pipeline issue by calculate and save the LIC parameter before a previous CU process is completed. In addition, the proposed method uses only the offset parameter without using the scale parameter, it reduces memory size to save LIC parameter and computes the offset parameter with lower complexity.

# DC prediction based LIC

In this contribution, only offset parameter is used to compensate illumination change. The offset parameter can be computed from residual data not reconstructed sample values. As a result, the pipeline issue can be alleviated before full reconstruction of a CU. The offset β is represented by

β= Rn-α·CnN                                  (1)

where R(n) is neighboring reconstructed sample of the reference CU and C(n) is neighboring reconstructed sample of current CU. In the merge case, R(n) is the predictor signal of C(n) and C(n) is the reconstructed signal, as shown in Fig 4. Since α is assumed to be 1 and the difference between R(n) and C(n) is residual, the numerator of Eq. (1) can be denoted by Eq. (2).

Rn-Cn=Resin                                          (2)

Figure 4. Relation of neighboring reconstructed sample between merged CU and its reference CU

2D IDCT is implemented by row-column decomposition method. The DC of bottom boundary pixels for a 2D residual signal is the DC coefficient of the vertical inverse transform for the bottom boundary coefficients. Thus, the DC of the bottom boundary pixels can be derived in between 2D IDCT, as shown in Fig. 5.

Figure 5. Derivation of the offset parameter during 2D inverse transform

After the CU process is finished, the offset parameter computed at given CU is stored. If the given CU is applied LIC, the value of the stored offset parameter plus the value of the offset parameter which used for the LIC is stored. Figure 6 shows an example of storing offset parameter.

Figure 6. Example of storing offset parameter

With this method, it is possible to alleviate the pipeline issue of the LIC of BMS 2.0.1. In BMS 2.0.1 and K0018 method, LIC application can start after reconstruction of reference CU. But in proposed method, LIC application can start before reconstruction of reference CU, as shown in Fig 6.

Figure 6. Pipeline stages of the LIC in BMS 2.0.1 and the proposed LIC

In BMS 2.0.1, LIC flag can be signaled or not, depending on the size of the CU, type of prediction modes and merge on/off mode. When the LIC flag is signaled, the flag indicate whether LIC is applied to a given CU or not. In the proposed design, any LIC flag is not signaled. If the current CU is coded by the merge mode and its motion vector is the same to that of the left side CU, LIC is applied to the current CU. In the current design, the flag is not used. However, RD optimization with a flag can be used for the proposed method. Further study should be required to improve RD performance with the LIC on/off flag.

# Experiment results

The proposed DC prediction based LIC is implemented within VTM 2.0.1 software. The following tables show BD-rate compared to VTM 2.0.1 in Random access configuration.

## VTM 2.0.1 with proposed LIC

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| Reference: VTM 2.0.1 | Over VTM 2.0.1 |  |  |  |  |
| Tested: VTM 2.0.1 with proposed LIC | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.03% | 0.10% | -0.04% | 99% |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.01% | 0.07% | 0.05% | 104% |  |
| Class F (optional) |  |  |  |  |  |

# Patent rights declaration(s)

Kwangwoon university and ETRI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).