| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0070_r2 |
| --- | --- |

| Title: | CE4: Planar Motion Vector Prediction (Test 4.3.1 and 4.3.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Na Zhang Jianhua Zheng Yongbing Lin Huanbang Chen Haitao Yang Jianle Chen | Email: | zhangna25@hisilicon.com zhengjianhua@hisilicon.com |
| Source: | HiSilicon Technologies Co., Ltd, Huawei Technologies. Co., Ltd |  |  |

_____________________________

- Abstract

This contribution reports the results of Core Experiment (CE) 4.3.1 Test (a) – (e) and combined test CE4.3.2 Test (a) – (b) for the planar motion vector prediction (PMVP) method. In 4.3.1Test (a) (which is also 4.2.14 in the 11th JVET meeting), fixed 4x4 PMVP sub-block size is used, and PMVP is available when the width or the height of a block is larger than or equal to 16. It provides 0.63% BD bit-rate saving. In 4.3.1Test (b), fixed 8x8 PMVP sub-block size is used, and PMVP is available when the width or the height of a block is larger than or equal to 16. It provides 0.53% BD bit-rate saving. In 4.3.1Test (c), fixed 8x8 PMVP sub-block size is used, and PMVP is available when the width and the height of a block is larger than or equal to 16. It provides 0.47% BD bit-rate saving. In 4.3.1Test (d), adaptive PMVP sub-block size is used, and PMVP is available when the width or the height of a block is larger than or equal to 16. It provides 0.53% BD bit-rate saving. In 4.3.1Test (e), adaptive PMVP sub-block size is used, and PMVP is available when the width and the height of a block is larger than or equal to 16. It provides 0.48% BD bit-rate saving. In 4.3.2Test (a), 0.99% BD bit-rate saving is provided. In 4.3.2Test (b), 0.92% BD bit-rate saving is provided.

Introduction

To generate a smooth fine granularity motion field, the planar motion vector prediction method is proposed in [1].  Planar motion vector prediction is achieved by averaging a horizontal and vertical linear interpolation on 4x4 block basis as follows.

PMVP is available when the width or the height of a block is larger than or equal to 16 and the block is not on the left or above boundary of a picture (4.3.1Test (a)).

However, the complexity of the planar motion vector prediction method is a little high.

Proposed Method

To reduce the complexity of the PMVP method, four schemes are proposed:

- Fixed 8x8 PMVP sub-block size is used (4.3.1Test (b)).

- Fixed 8x8 PMVP sub-block size is used, and PMVP is available when the width and the height of a block is large than or equal to 16(4.3.1Test (c)).

- Adaptive PMVP sub-block size is used, and PMVP is available when the width or the height of a block is large than or equal to 16 (4.3.1Test (d)).

- Adaptive PMVP sub-block size is used, and PMVP is available when the width and the height of a block is large than or equal to 16 (4.3.1Test (e)).

For 3) and 4), one flag is signaled at slice header to indicate whether the PMVP sub-block size is 8x8 or 4x4. The sub-block size for the PMVP of the slices in the current picture is decided based on the average size of the PMVP CUs from the last coded picture in the same temporal layer, which is similar as the method in [2]. Assume the picture just coded is the i-th picture in the k-th temporal layer and it contains N CUs that are coded by the PMVP mode. Moreover, assume the sizes of those CUs are S0, S1, …, SN-1 . The average size of the PMVP CUs is calculated as . Then, when coding the (i+1)-th picture in the same k-th temporal layer, the corresponding PMVP sub-block size  is determined based on the following:

where thres is the threshold which is set to 40 for low-delay (LD) pictures and 5 for non-LD pictures. Additionally, for the first picture at each temporal layer, the PMVP sub-block size is always set to 4x4.

In this contribution, two combined tests with CE4.2.8 in JVET-L0369 [5], donated as CE4.3.2, are conducted. For CE4.3.2.a, CE4.3.1.a and CE4.2.8.b are naturally combined. For CE4.3.2.b, planar mode is moved into sub-block merge list to generate a unified sub-block list, and the length of sub-block merge list is enlarged to 6. The sub-block merge candidate list is constructed as following step:

1)	Insert ATMVP candidate

2)	Insert inherited affine candidates

3)	Insert planar candidate

4)	Insert constructed affine candidates

5)	Padding with zero motion vectors

Experimental Results

The proposed method has been implemented on VTM-2.0.1[3], and all the tests were conducted under the common test conditions [4]. The experimental results of the 4.3.1Test (a) – (c) compared to VTM-2.0.1 and VTM-2.0.1-AFFINE-ATMVP are shown in Table1. The experimental results of the 4.3.1Test (a), 4.3.1Test (b), 4.3.1Test (c), 4.3.1Test (d), and 4.3.1Test (e) compared to VTM-2.0.1 are shown in Table2, Table3, Table4, Table5, and Table6, separately. The experimental results of the test 4.3.2.a, and test 4.3.2.b compared to VTM-2.0.1 are shown in Table7, and Table8, separately. In Table 5, the LB decoding time is not reliable.

Table 1. Experimental results of the 4.3.1Test (a) – (c) compared to VTM-2.0.1 and VTM-2.0.1-AFFINE-ATMVP

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1-AFFINE-ATMVP |  |  |  |  |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Test (a) | -0.63% | -0.57% | -0.77% | 104% | 114% | -1.22% | -1.01% | -1.17% | 105% | 121% |
| Test (b) | -0.53% | -0.48% | -0.62% | 103% | 103% | -1.02% | -0.88% | -1.01% | 103% | 106% |
| Test (c) | -0.47% | -0.42% | -0.54% | 102% | 103% | -0.94% | -0.77% | -0.91% | 102% | 106% |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1-AFFINE-ATMVP |  |  |  |  |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Test (a) | -0.48% | -0.47% | -0.61% | 105% | 108% | -1.28% | -1.10% | -0.98% | 106% | 115% |
| Test (b) | -0.28% | -0.28% | -0.22% | 104% | 100% | -0.92% | -0.56% | -0.56% | 105% | 105% |
| Test (c) | -0.27% | -0.26% | -0.25% | 102% | 101% | -0.91% | -0.63% | -0.62% | 103% | 106% |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1-AFFINE-ATMVP |  |  |  |  |
| Test# | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Test (a) | -0.66% | -0.49% | -0.50% | 106% | 105% | -1.97% | -1.52% | -1.44% | 108% | 109% |
| Test (b) | -0.43% | -0.26% | -0.24% | 105% | 100% | -1.52% | -1.10% | -0.85% | 106% | 104% |
| Test (c) | -0.40% | -0.23% | -0.19% | 102% | 101% | -1.43% | -0.82% | -1.06% | 103% | 103% |

Table 2. Experimental results of the 4.3.1Test (a) compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.87% | -0.51% | -0.90% | 103% | 120% |
| Class A2 | -0.87% | -0.93% | -1.05% | 105% | 117% |
| Class B | -0.50% | -0.49% | -0.65% | 104% | 112% |
| Class C | -0.44% | -0.44% | -0.61% | 104% | 110% |
| Class E |  |  |  |  |  |
| Overall | -0.63% | -0.57% | -0.77% | 104% | 114% |
| Class D | -0.63% | -0.64% | -0.77% | 105% | 113% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.28% | -0.35% | -0.33% | 105% | 106% |
| Class C | -0.21% | -0.20% | -0.55% | 105% | 106% |
| Class E | -1.19% | -1.02% | -1.17% | 105% | 112% |
| Overall | -0.48% | -0.47% | -0.61% | 105% | 108% |
| Class D | -0.26% | 0.10% | -0.24% | 105% | 109% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.45% | -0.03% | -0.21% | 106% | 103% |
| Class C | -0.37% | -0.40% | -0.43% | 106% | 106% |
| Class E | -1.39% | -1.40% | -1.05% | 107% | 108% |
| Overall | -0.66% | -0.49% | -0.50% | 106% | 105% |
| Class D | -0.38% | -0.07% | -0.73% | 106% | 107% |

Table 3. Experimental results of the 4.3.1 Test (b) compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.79% | -0.54% | -0.79% | 102% | 104% |
| Class A2 | -0.72% | -0.76% | -0.87% | 103% | 102% |
| Class B | -0.42% | -0.41% | -0.55% | 103% | 103% |
| Class C | -0.33% | -0.32% | -0.40% | 104% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.53% | -0.48% | -0.62% | 103% | 103% |
| Class D | -0.41% | -0.51% | -0.42% | 104% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | -0.31% | -0.20% | 104% | 99% |
| Class C | -0.08% | -0.13% | -0.14% | 104% | 101% |
| Class E | -0.70% | -0.44% | -0.37% | 105% | 102% |
| Overall | -0.28% | -0.28% | -0.22% | 104% | 100% |
| Class D | -0.14% | -0.06% | -0.68% | 105% | 101% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.30% | -0.07% | 0.01% | 105% | 99% |
| Class C | -0.17% | -0.10% | -0.17% | 105% | 101% |
| Class E | -1.00% | -0.76% | -0.75% | 106% | 99% |
| Overall | -0.43% | -0.26% | -0.24% | 105% | 100% |
| Class D | -0.18% | 0.19% | -0.46% | 106% | 103% |

Table 4. Experimental results of the 4.3.1Test (c) compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.78% | -0.48% | -0.76% | 101% | 105% |
| Class A2 | -0.69% | -0.75% | -0.84% | 102% | 102% |
| Class B | -0.36% | -0.35% | -0.44% | 102% | 103% |
| Class C | -0.24% | -0.22% | -0.29% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.47% | -0.42% | -0.54% | 102% | 103% |
| Class D | -0.29% | -0.38% | -0.36% | 101% | 103% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.15% | -0.24% | -0.07% | 102% | 99% |
| Class C | -0.04% | -0.20% | -0.23% | 102% | 102% |
| Class E | -0.78% | -0.38% | -0.58% | 103% | 104% |
| Overall | -0.27% | -0.26% | -0.25% | 102% | 101% |
| Class D | -0.08% | 0.22% | 0.24% | 102% | 102% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.28% | 0.11% | 0.19% | 103% | 100% |
| Class C | -0.19% | -0.13% | -0.44% | 102% | 102% |
| Class E | -0.86% | -0.94% | -0.48% | 103% | 100% |
| Overall | -0.40% | -0.23% | -0.19% | 102% | 101% |
| Class D | -0.12% | 0.27% | -0.24% | 102% | 101% |

Table 5. Experimental results of the 4.3.1Test (d) compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.79% | -0.50% | -0.84% | 102% | 104% |
| Class A2 | -0.75% | -0.82% | -0.89% | 104% | 101% |
| Class B | -0.41% | -0.41% | -0.56% | 104% | 101% |
| Class C | -0.33% | -0.28% | -0.46% | 104% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.53% | -0.48% | -0.66% | 104% | 102% |
| Class D | -0.45% | -0.51% | -0.62% | 104% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.27% | -0.15% | -0.50% | 105% | 104% |
| Class C | -0.21% | -0.24% | -0.48% | 105% | 98% |
| Class E | -0.98% | -0.75% | -0.52% | 105% | 98% |
| Overall | -0.43% | -0.33% | -0.50% | 105% | 100% |
| Class D | -0.28% | 0.18% | -0.13% | 106% | 106% |

Table 6. Experimental results of the 4.3.1Test (e) compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.77% | -0.56% | -0.81% | 102% | 105% |
| Class A2 | -0.70% | -0.65% | -0.83% | 102% | 103% |
| Class B | -0.37% | -0.29% | -0.45% | 102% | 103% |
| Class C | -0.24% | -0.25% | -0.29% | 102% | 103% |
| Class E |  |  |  |  |  |
| Overall | -0.48% | -0.40% | -0.56% | 102% | 103% |
| Class D | -0.30% | -0.47% | -0.32% | 102% | 104% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.20% | -0.04% | -0.39% | 103% | 104% |
| Class C | -0.14% | -0.08% | -0.17% | 102% | 105% |
| Class E | -0.82% | -0.91% | -0.18% | 103% | 104% |
| Overall | -0.33% | -0.27% | -0.26% | 103% | 104% |
| Class D | -0.19% | 0.08% | -0.56% | 102% | 105% |
|  | Low delay P Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.33% | 0.18% | 0.24% | 103% | 104% |
| Class C | -0.23% | -0.32% | -0.16% | 103% | 103% |
| Class E | -0.98% | -1.31% | -0.73% | 104% | 102% |
| Overall | -0.46% | -0.36% | -0.14% | 103% | 103% |
| Class D | -0.16% | 0.21% | -0.63% | 102% | 106% |

Table 7. Experimental results of the Test 4.3.2.a compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.38% | -1.01% | -1.30% | 107% | 110% |
| Class A2 | -1.33% | -1.32% | -1.48% | 110% | 108% |
| Class B | -0.85% | -0.75% | -0.92% | 108% | 108% |
| Class C | -0.63% | -0.62% | -0.71% | 108% | 108% |
| Class E |  |  |  |  |  |
| Overall | -0.99% | -0.88% | -1.05% | 108% | 108% |
| Class D | -0.89% | -0.93% | -0.93% | 108% | 110% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.33% | -0.40% | -0.28% | 111% | 103% |
| Class C | -0.21% | -0.13% | -0.06% | 109% | 104% |
| Class E | -1.29% | -1.15% | -1.12% | 112% | 115% |
| Overall | -0.53% | -0.50% | -0.42% | 111% | 106% |
| Class D | -0.28% | 0.48% | 0.15% | 108% | 107% |

Table 8. Experimental results of the Test 4.3.2.b compared to VTM-2.0.1

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.20% | -0.85% | -1.09% | 105% | 109% |
| Class A2 | -1.22% | -1.21% | -1.38% | 106% | 109% |
| Class B | -0.79% | -0.71% | -0.82% | 105% | 108% |
| Class C | -0.65% | -0.54% | -0.64% | 104% | 107% |
| Class E |  |  |  |  |  |
| Overall | -0.92% | -0.79% | -0.94% | 105% | 108% |
| Class D | -0.80% | -0.79% | -0.79% | 104% | 110% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over [vtm-2.0.1] |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.34% | -0.28% | -0.16% | 106% | 104% |
| Class C | -0.22% | -0.22% | -0.24% | 104% | 104% |
| Class E | -1.39% | -1.22% | -1.37% | 107% | 115% |
| Overall | -0.56% | -0.50% | -0.49% | 106% | 106% |
| Class D | -0.26% | -0.38% | -0.24% | 103% | 106% |

Conclusion

This contribution proposes a planar motion vector prediction method. Simulation results show that compared to VTM-2.0.1, in RA condition, Test (a) provides 0.63% BD bit-rate saving; Test (b) provides 0.53% BD bit-rate saving; Test (c) provides 0.47% BD bit-rate saving; Test (d) provides 0.53% BD bit-rate saving; Test (e) provides 0.48% BD bit-rate saving. It is recommended to adopt Test (c) to the next version of VTM/BMS due to better tradeoff between performance and complexity.

Reference

- N. Zhang, Y. Lin, J. Zheng, “CE4.2.14:Planar Motion Vector Prediction,” JVET-K0135, Ljubljana, SI, 10–18 Jul. 2018.

- X. Xiu, Y. He, Y. Ye, “CE4.2.5: Simplifications on advanced temporal motion vector prediction (ATMVP),” JVET-K0341, Ljubljana, SI, 10–18 Jul. 2018.

- VTM-2.0.1, https://vcgit.hhi.fraunhofer.de/JVET-K-CE4/VVCSoftware_VTM/tree/VTM-2.0.1

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, Ljubljana, SI, 10–18 Jul. 2018.

- H. Chen, H. Yang, J. Chen, “CE4: Separate list for sub-block merge candidates (Test 4.2.8)”, JVET-L0369, the 12th JVET meeting, Oct. 2018, Macro, CN.

Patent rights declaration(s)

HiSilicon Technologies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Huawei Technologies Co., Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).