| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0073_r1 |
| --- | --- |

| Title: | CE11: Non-recursive deblocking of luma on 4x4 grid (CE11.3.1) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson, Z. Zhang, R. Sjöberg | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes non-recursive luma deblocking on 4x4 grid since CU’s are aligned on a 4x4 grid. The approach make sure that 4xN and 8xN blocks where N can be up to CTU size are deblocked. The approach also reduce worst case of deblocking filtering of adjacent 4x4 blocks by only filtering at most 50% of the block area compared to the anchor which filters at most 75% of the block area.

The proposed solution is claimed to improve subjective quality in cases of blocks that are large in one dimension. It also provides a luma BD rate impact of 0.08%/-0.12%/-0.21%/-0.20% for AI/RA/LD/LDP compared to VTM-2.0.1 and 0.01%/-0.25%/-0.18%/-0.40% for AI/RA/LD/LDP compared to VTM-2.0.1 when ALF is turned off.  Decoding time 97%/100%/99%/100% for AI/RA/LD/LDP compared to VTM-2.0.1 and 98%/99%/100%/100% for AI/RA/LD compared to VTM-2.0.1 when ALF is turned off.

# Introduction

The deblocking in the current test model is based on HEVC deblocking and is applied on an 8x8 grid. The proposed solution applies deblocking on a 4x4 grid since CU’s are aligned on a 4x4 grid as described in section 2. In section 3 objective results in comparison to VTM-2.0.1 and VTM-2.0.1 without ALF are shown. Finally some conclusions in section 4.

# Deblocking of luma on 4x4 grid

Determine the number of samples to modify in block P and Q as follows:

lengthP = edge==EDGE_HOR ? (CUP_height >> 1) – 1 : (CUP_width>>1) - 1

lengthQ = edge==EDGE_HOR ? (CUQ_height >> 1) – 1 : (CUQ_width>>1) - 1

This means that only one sample will be filtered at most with the weak VVC filter on side P when the distance between boundaries on P side is 4 and only at most one sample will be filtered with the weak VVC filter on side Q when the distance between the boundaries on Q side is 4.

The weak VVC filter can filter at most two samples on at least one side if lengthP or lengthQ is larger than 1. The strong VVC filter can be used if both lengthP and lengthQ are larger than 2, e.g. block size need to be 8xN for filtering of 3 samples on respective side of the boundary, where N is the length of the blocks parallel to the block boundary.

## Complexity analysis

Filtering is performed on an 4x4 grid. All filtering can be performed in parallel.

### Filtering

Worst case is reduced since adjacent 4x4 blocks will at most be filtered with 1 sample where the anchor can filter 3 samples on each side. So in worst case the anchor will filter 75% of the area of adjacent 4x4 blocks where the proposal only would filter 50% of the area of the blocks.

Proposal, 1 sample on each side in worst case:  26 = 2*13 operations per line (2 multiplications, 6 additions, 1 shifts, 4 cmp)

Anchor,  3 samples on each side in worst case 52 operations (2 multiplications, 28 additions, 12 cmp, 10 shifts)

Totally 52-26=26 operations less than the anchor.

### Decision

Need to determine the length of filtering for both sides of a boundary and compare lengths with thresholds: 6 operations per line (2 shifts, 4 cmp).

“d” calculation is performed twice as much in worst case as for the anchor: 4.5 operations per line (9 additions, 4 shifts, 5 cmp)/4.

Weak decisions: 1 operations per line (1 cmp)

Strong weak filter decision only performed for 8xN blocks which is comparable with filtering on 8x8 grid.

Totally 13.5-5=8.5 operations more than the anchor, since the anchor needs to perform strong/weak filter decision (12 cmp, 8 additions)/4= 5 operations per line.

# Results

## Visual example KristenAndSara

## Objective results

The simulations were performed using the CTC with the inclusion of LDP. We also tested the performance with ALF disabled. The decoding times for the anchor and test configurations were measured using the same machines, but encoding times were measured on different machines which make the encoding times unreliable. However, since the decoding times are magnitudes shorter and the deblocking filter is done once per picture for both encoder and decoder, we conclude the encoding time difference is negligible.

The results indicate that that the objective performance of the proposed filtering on 4x4 grid for luma is slightly better than the anchor except for all intra where it is slightly worse. It has slightly better objective performance when ALF is disabled than when it is enabled.  Encoding time is about 100% and decoding time is about 100% compared to the anchor with the exception for all intra where decoding time is reduced to about 98%.

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.07% | -0.01% | 0.00% | 100% | 96% | 0.04% | -0.01% | 0.00% | 104% | 95% |
| Class A2 | 0.09% | 0.00% | 0.00% | 102% | 96% | 0.03% | 0.00% | 0.00% | 102% | 97% |
| Class B | 0.11% | -0.01% | -0.01% | 105% | 97% | 0.05% | 0.00% | 0.00% | 98% | 98% |
| Class C | 0.09% | 0.00% | -0.01% | 102% | 99% | 0.04% | 0.00% | 0.00% | 102% | 100% |
| Class E | 0.03% | -0.01% | -0.01% | 105% | 98% | -0.12% | -0.01% | -0.01% | 101% | 98% |
| Overall | 0.08% | -0.01% | -0.01% | 103% | 97% | 0.01% | 0.00% | 0.00% | 101% | 98% |
| Class D | 0.08% | -0.02% | -0.01% | 100% | 100% | 0.03% | -0.01% | -0.01% | 97% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | -0.09% | 0.01% | -0.02% | 97% | 98% | -0.17% | -0.06% | 0.00% | 98% | 99% |
| Class A2 | -0.17% | -0.03% | -0.11% | 98% | 100% | -0.44% | -0.06% | -0.05% | 102% | 99% |
| Class B | -0.08% | 0.03% | 0.00% | 98% | 101% | -0.19% | -0.06% | -0.06% | 99% | 99% |
| Class C | -0.18% | 0.05% | -0.14% | 100% | 102% | -0.23% | -0.02% | -0.15% | 98% | 100% |
| Class E |  |  |  |  |  |  |  |  |  |  |
| Overall | -0.12% | 0.02% | -0.06% | 98% | 100% | -0.25% | -0.05% | -0.07% | 99% | 99% |
| Class D | -0.17% | -0.08% | -0.09% | 99% | 101% | -0.24% | -0.02% | 0.03% | 96% | 101% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.10% | -0.17% | -0.16% | 100% | 100% | -0.18% | -0.07% | -0.14% | 101% | 101% |
| Class C | -0.25% | 0.02% | -0.11% | 101% | 98% | -0.27% | -0.15% | -0.05% | 103% | 100% |
| Class E | -0.34% | -0.40% | -0.46% | 94% | 100% | -0.07% | -0.18% | 0.05% | 96% | 98% |
| Overall | -0.21% | -0.17% | -0.22% | 99% | 99% | -0.18% | -0.13% | -0.06% | 100% | 100% |
| Class D | -0.11% | 0.13% | -0.39% | 101% | 101% | -0.18% | 0.07% | 0.13% | 101% | 99% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #DIV/0! | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! |
|  | Low delay P Main10 |  |  |  |  |  |  |  |  |  |
|  | VTM-2.0.1 |  |  |  |  | VTM-2.0.1 ALF=0 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.13% | 0.12% | 0.11% | 100% | 100% | -0.35% | -0.26% | -0.05% | 102% | 100% |
| Class C | -0.20% | -0.17% | -0.16% | 97% | 100% | -0.34% | 0.02% | -0.17% | 101% | 100% |
| Class E | -0.31% | 0.06% | 0.27% | 104% | 99% | -0.56% | -0.15% | -0.29% | 100% | 101% |
| Overall | -0.20% | 0.01% | 0.06% | 100% | 100% | -0.40% | -0.14% | -0.15% | 101% | 100% |
| Class D | -0.04% | -0.18% | 0.39% | 106% | 100% | -0.29% | -0.04% | -0.18% | 100% | 100% |
| Class F (optional) | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusions

The proposal secures that large blocks in one dimension are deblocked, both 8xN blocks that are aligned on a 4x4 grid but also 4xN blocks. The approach also reduce worst case of filtering for the case of adjacent 4x4 blocks. It is proposed to adopt deblocking on 4x4 grid for luma.

# References

[1] A. Norkin, A. Meher Kotra, “Description of Core Experiment 11 (CE11): Deblocking”, JVET-K1031, Ljubljana, July 2018

[2] R. Sjöberg, K. Andersson, R. Yu, Z. Zhang, P. Wennersten, J. Lainema, A. Hallapuro, A. Aminlou, M. Hannuksela, R. Ghaznavi-Youvalari, J. Ridge, “Description of SDR and HDR video coding technology proposal by Ericsson and Nokia”, JVET-J0012, San Diego, April 2018

[3] K. Andersson, Z. Zhang, R. Sjöberg, “CE2-2.1.1: Long deblocking filters and fixes”, JVET-K0307, Ljubljana, July 2018

[4] K. Andersson, Z. Zhang, R. Sjöberg, “CE2-2.1.1 related: HEVC luma filters and decisions for chroma deblocking”, JVET-K0318, Ljubljana, July 2018

[5] J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations”, JVET-K1010, Ljubljana, July 2018

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).