| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0076-v2 |
| --- | --- |

| Title: | CE3: Line-based intra coding mode (Tests 2.1.1 and 2.1.2) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Santiago De Luxán Hernández, Heiko Schwarz, Detlev Marpe, Thomas Wiegand Fraunhofer HHI Einsteinufer 37 10587 Berlin, Germany | Tel: Email: | +49 30 31002-511 santiago.de.luxan@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This contribution presents the simulation results of the Line-based intra coding mode (CE3, JVET-K1023, Tests 2.1.1 and 2.1.2) on top of VTM. Various modifications have been carried out on the algorithm to solve the hardware related concerns expressed during the Ljubljana Meeting. Test 2.1.1 obtains a gain of 1,01% with encoding and decoding times of 148% and 103% respectively for the AI configuration and a gain of 0,49% with encoding and decoding times of 113% and 100% respectively for the RA configuration. Regarding Test 2.1.2, the results are a gain of 0,82% with encoding and decoding times of 143% and 103% respectively for the AI configuration and a gain of 0,46% with encoding and decoding times of 112% and 101% respectively for the RA configuration.

# Introduction

The Line-Based Intra Prediction Coding Mode was proposed in [1] and later updated in [2]. As shown in Figure 1, the algorithm divided luma intra-predicted blocks into 1-D partitions or lines. Therefore, a W×H dimensional block (where H is the height and W is the width) could be partitioned into H W×1 (horizontal split) or W 1×H (vertical split) lines. 

For each of these lines, a residual signal was generated by entropy decoding the coefficients sent by the encoder and then inverse quantizing and inverse transforming them. Then, the line was intra predicted and finally the corresponding reconstructed samples were obtained by adding the residual signal to the prediction signal. Therefore, the reconstructed values of each line were available to generate the prediction of the next line, which repeated the process and so on.

All resulting lines shared the same intra mode, so that it had to be signaled only once. On the other hand, the processing order of the sub-partitions introduced in [1] indicated whether the sub-partitions were coded downwards or rightwards (Normal Processing Order, NPO) or upwards or leftwards (Reversed Processing Order, RPO) according to the intra mode.

Besides, a flag was transmitted to the decoder to indicate whether the coding mode had to be used and if that was the case a second flag was sent to explicitly set the split type (horizontal or vertical).

Figure 1: Example of the division of a block in vertical or horizontal lines as presented in [1] [2]

Further details of the algorithm can be found in [1] [2].

Nevertheless, the following concerns were raised during the Ljubljana Meeting regarding its hardware implementation complexity:

- The resulting sub-partitions of the division of a block could have less than 16 samples, which is the number of samples of the smallest partition for luma in VTM-2.0 [3]. For example, a 4×4 block could be divided into 4 (4×1) lines, which could generate a throughput issue, given that it could produce a worst-case scenario bitstream where all the blocks are 1×4 (or 4×1) instead of  4×4 (the current case of VTM).

- If the number of resulting lines was large (for example, 64), there could be a problem for the encoder in terms of number of operations and memory accesses to check the necessary RD tests.

- Column sub-partitions (1×N) can potentially be more difficult to implement, since memory access are more expensive given that samples are allocated using a raster scan.

To address these issues, two new tests were proposed in [4]:

- Test 2.1.1 : Line-based intra coding mode with a restricted number of partitions per block size  (with at least 16 samples per partition)

- Test 2.1.2 : Test 2.1.1 with a restriction: the resulting partitions must have a width of at least 4 samples

The modifications of the algorithm are presented in the next sections.

# Calculation of the new sizes of the sub-partitions

## For Test 2.1.1

Given a W×H block (with W∙H >16), each sub-partition can be divided horizontally or vertically in k sub-partitions with w×h dimensions, whose values are displayed in Table I.

Table I: Values of w, h and k with respect to W and H (with W∙H >16) for Test 2.1.1

|  | Horizontal Split | Vertical Split |
| --- | --- | --- |
| w | W | W4 , if W4∙H≥16 16H, if W4∙H<16 |
| h | H4 , if H4∙W≥16 16W, if H4∙W<16 | H |
| k | Hh | Ww |

For instance, a 16×8 block can be divided into 4 (16×2) sub-partitions (horizontal split) or 4 (4×8) sub-blocks (vertical split). However, a 4×8 block can be divided into 2 4×4 sub-partitions (horizontal split) or 2 (2×8) sub-blocks (vertical split). Note that the number of sub-partitions can be only 2 (in the case of 4×8 and 8×4 blocks) or 4 (in all other valid cases). Figure 2 and Figure 3 show an example of these two possibilities.

Since the splits can only be applied if the resulting sub-partitions have at least 16 samples, 4×4 blocks are excluded. Therefore, the signalling required to indicate whether the mode is applied and which type of split shall be used is eliminated in this particular case.

With this new form of sub-partitioning, lines can only be obtained by dividing a 4×N block vertically or a N×4 block horizontally, where N≥16.

Since the sub-partitions are no longer limited to be lines, in the following the algorithm will be referred to as Intra Sub-Partitions (ISP).

Figure 2: Example of the horizontal and vertical division of 4×8 and 8×4 blocks into 2 sub-partitions

Figure 3: Example of the horizontal and vertical division of all blocks except 4×8, 8×4 and 4×4 into 4 sub-partitions

## For Test 2.1.2

Test 2.1.2 inherits all the characteristics from Test 2.1.1 and imposes a new restriction: when a block is divided, the resulting sub-partitions must have a width of at least 4 samples. This constraint affects only 4×H and 8×H blocks.

In the first case (4×H), the vertical split will no longer be performed, given that it would produce sub-partitions with a width of 1 or 2. Hence, the split flag will also be eliminated, given that only the horizontal split will be possible.

Regarding the 8×H blocks, the vertical split can still be used, but the number of sub-partitions will be fixed to 2 (instead of 4 in most cases). This ensures a sub-partition width of 4 for this type of blocks.

# Coefficients coding

Quantized transform coefficients from 1-D sub-partitions will be coded the same way as in [1] [2]. 2-D sub-partitions will use a 2-D transform and the resulting quantized coefficients will be coded as usual in VTM, with the following differences:

- If one of the dimensions of the sub-partition is 2, then dimensions of the coefficient groups used for the significance map will be 2×2 instead of 4×4.

- The context of each CBF will the value of the CBF of the previously coded sub-partition (the same method used for the lines in [1] [2]). For the first sub-partition one default context is selected.

# Interaction with other tools

## Wide Angles

The ISP algorithm can be used in combination with the wide angles according to the following parameters. In this case, the aspect ratio utilized to decide whether to use a wide angle or not is the one of the original block rather than the one of the sub-partitions.

On the other hand, if the block uses a wide angle, then the processing order will be selected according to the new angle instead of the replaced original intra mode. Let i be the intra mode of a block and let j be corresponding wide angle that replaces i. Then, the explicit mapping between j and the Processing Order is shown in Table II. Non-angular modes continue using always the NPO.

Table II: Processing Order according to j

|  | Horizontal Split | Vertical Split |
| --- | --- | --- |
| j <HOR_IDX | RPO | NPO |
| HOR_IDX≤j≤VER_IDX | NPO | NPO |
| j>VER_IDX | NPO | RPO |

## PDPC

Whenever a CU uses the ISP coding mode, the PDPC filters will not be applied to the resulting sub-partitions.

## MTS

If a CU uses the ISP coding mode, the MTS CU flag will be set to 0 and it will not be sent to the decoder. Therefore, the encoder will not perform RD tests for the different available transforms for each resulting sub-partition. The transform choice for the ISP mode will instead be fixed and selected according the intra mode, the processing order and the block size utilized [5]. Hence, no signalling is required.

Let tH and tV be the horizontal and the vertical transforms selected respectively for the w×h sub-partition, where w is the width and h is the height. Then the transform is selected according to the following rules:

- If w=1 or h=1, then there is no horizontal or vertical transform respectively.

- If w=2 or w>32, tH = DCT-II

- If h =2 or h >32, tV = DCT-II

- Otherwise:

- For the NPO (and the first sub-partition of a block using the RPO) the transforms are selecting according to

  - For the NPO (and the first sub-partition of a block using the RPO) the transforms are selecting according to Table III, which is the same transform table proposed in [5].

  - For the RPO (excluding the first sub-partition) the transforms are selected according to

  - For the RPO (excluding the first sub-partition) the transforms are selected according to Table IV.

Table III: Transform Selection for the NPO (and the first sub-partition of a block using the RPO) [5]

| Intra mode | tH | tV |
| --- | --- | --- |
| Planar Ang. 31,32,34,36,37 | DST-VII | DST-VII |
| DC Ang. 33, 35 | DCT-II | DCT-II |
| Ang. 2, 4, 6…28,30 Ang. 39,41,43…63,65 | DST-VII | DCT-II |
| Ang. 3,5,7…27,29 Ang. 38,40,42…64,66 | DCT-II | DST-VII |

Table IV: Transform Selection for the RPO (for all sub-partitions except the first one)

| Split Type | Intra Mode Type | tH | tV |
| --- | --- | --- | --- |
| Horizontal | Even | DCT-VIII | DCT-II |
| Horizontal | Odd | DCT-II | DST-VII |
| Vertical | Even | DCT-II | DCT-VIII |
| Vertical | Odd | DST-VII | DCT-II |

# Experimental results

According to the CE3 description [4] and the common test conditions [6], the proposed method is evaluated for intra-only (AI) and random-access (RA) configuration with BMS-2.0.1 software in VTM configuration. Class F has also been included in all the experiments carried out. The corresponding simulations were conducted on an Intel Xeon cluster (E5-2697A v4, AVX2 on, turbo boost off) with Linux OS and a GCC 7.2.1 compiler.

## Test 2.1.1

Table V: Results for Test 2.1.1 with an AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,32% | -0,08% | -0,18% | 146% | 103% |
| Class A2 | -0,50% | -0,29% | -0,34% | 144% | 104% |
| Class B | -0,88% | -0,75% | -0,64% | 148% | 103% |
| Class C | -1,57% | -1,26% | -1,16% | 151% | 105% |
| Class E | -1,67% | -0,97% | -1,00% | 149% | 101% |
| Overall | -1,01% | -0,71% | -0,69% | 148% | 103% |
| Class D | -1,49% | -1,13% | -1,26% | 141% | 104% |
| Class F (optional) | -3,41% | -2,70% | -2,62% | 156% | 104% |

Table VI: Results for Test 2.1.1 with a RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,34% | -0,33% | -0,35% | 113% | 101% |
| Class A2 | -0,40% | -0,41% | -0,26% | 111% | 100% |
| Class B | -0,49% | -0,53% | -0,27% | 112% | 101% |
| Class C | -0,69% | -0,62% | -0,50% | 115% | 98% |
| Class E |  |  |  |  |  |
| Overall | -0,49% | -0,49% | -0,35% | 113% | 100% |
| Class D | -0,63% | -0,55% | -0,44% | 104% | 100% |
| Class F (optional) | -2,63% | -1,90% | -2,03% | 118% | 101% |

## Test 2.1.2

Table VII: Results for Test 2.1.2 with an AI configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,28% | -0,12% | -0,17% | 142% | 101% |
| Class A2 | -0,47% | -0,23% | -0,29% | 139% | 103% |
| Class B | -0,81% | -0,65% | -0,61% | 143% | 103% |
| Class C | -1,28% | -1,09% | -0,88% | 145% | 104% |
| Class E | -1,14% | -0,60% | -0,72% | 144% | 101% |
| Overall | -0,82% | -0,58% | -0,56% | 143% | 103% |
| Class D | -1,06% | -0,78% | -1,15% | 135% | 103% |
| Class F (optional) | -2,80% | -2,20% | -2,08% | 148% | 104% |

Table VIII: Results for Test 2.1.2 with a RA configuration

|  | Y | U | V | EncT | DecT |
| --- | --- | --- | --- | --- | --- |
| Class A1 | -0,37% | -0,46% | -0,32% | 113% | 101% |
| Class A2 | -0,37% | -0,27% | -0,17% | 110% | 102% |
| Class B | -0,45% | -0,44% | -0,28% | 111% | 102% |
| Class C | -0,59% | -0,47% | -0,39% | 113% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0,46% | -0,42% | -0,29% | 112% | 101% |
| Class D | -0,50% | -0,53% | -0,07% | 103% | 99% |
| Class F (optional) | -2,23% | -1,42% | -1,66% | 116% | 100% |

## Additional results

### Clip only after the whole block has been reconstructed

In the VTM software, the result of adding the residual and the prediction samples to generate the reconstructed samples is clipped, in order to avoid having reconstructed values smaller than 0 or greater than 2B-1, where B is the bit depth. In this test, for a block utilizing the ISP algorithm, the clipping will not be performed after each sub-partition is reconstructed. Instead, it will be applied on all the reconstructed samples of the original block after all sub-partitions have been reconstructed. This test aims to reduce the implementation complexity by eliminating the non-linearity of the clipping after each sub-partition is decoded.

Table IX: Results for the tests using only clipping after all sub-partitions have been reconstructed

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | EncT | DecT | Y | EncT | DecT |
| Test 2.1.1 | -1,01% | 148% | 104% | -0,49% | 113% | 100% |
| Test 2.1.2 | -0,82% | 143% | 104% | -0,45% | 112% | 101% |

It can be observed that the difference between clipping after reconstructed each sub-partition and clipping only after all of them have been reconstructed is negligible in terms of BD-Rate performance for both Test 2.1.1 and Test 2.1.2.

### Encoder only speed-ups

By enabling different encoder only speed-ups, three different types of trade-offs between gain and encoder run-time are presented for Test 2.1.1.

Table X: Summary of trade-offs between gain and encoder run-time for Test 2.1.1

|  | AI |  |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Y | EncT | DecT |  | Y | EncT | DecT |
| Original |  | -1,01% | 148% | 103% |  | -0,49% | 113% | 100% |
| Alt. Trade-off 1 |  | -0,91% | 136% | 105% |  | -0,44% | 110% | 102% |
| Alt. Trade-off 2 |  | -0,88% | 132% | 100% |  | -0,41% | 109% | 96% |
| Alt. Trade-off 3 |  | -0,82% | 128% | 101% |  | -0,39% | 108% | 97% |

### Performance on top of the combination of CE3-1.1 and CE3-3.2

Additional test results have been provided on top of a combination of the Multiple Reference Line and Bilateral Filter tools respectively proposed in CE3-1.1 and CE3-3.2. It can be observed that the gain of the ISP algorithm and these two other methods is approximately 85% additive for AI and 89% for RA.

Table XI: Results for Test 2.1.1 on top of the combination of the CE3-1.1 and CE3-3.2

| AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- |
| Y | EncT | DecT | Y | EncT | DecT |
| -0,85% | 143% | 101% | -0,44% | 112% | 99% |

### Removal of the intra mode dependency of the context mode of the split flag

### In [1] [2], the context used to code the split flag depends on the intra mode. In this case, new results are provided where this dependency has been removed and one single context is used to code this syntax element. It can be observed that the loss is negligible.

Table XII: Results with a single context for the split flag

|  | AI |  |  | RA |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Y | EncT | DecT | Y | EncT | DecT |
| Test 2.1.1 | -0,99% | 148% | 104% | -0,49% | 113% | 101% |
| Test 2.1.2 | -0,80% | 143% | 103% | -0,43% | 112% | 100% |

# Cross-Check

Cross-checks for the tests have been performed by Tencent.

# Conclusion

The reported results show gains in all the tests and configurations. Class F yields clearly the highest BD-Rates. This is probably due to the fact this screen content is typically richer in sharp edges, which can be better modelled with smaller partitions. On the other hand, for this type of sequences the difference in the gains reported in the AI and RA configurations is much smaller than typically.

Regarding the individual tests behaviors, Test 2.1.1 obtains the highest gains, as expected. Test 2.1.2 has lower gains than Test 2.1.1 in terms of BD-Rate, given that the ISP algorithm is applied on a smaller number of all the blocks available. However, it reduces the hardware implementation complexity with respect to Test 2.1.1.

# References

| [1] | M. Albrecht, C. Bartnik, S. Bosse, J. Brandenburg, B. Bross, J. Erfurt, V. George, P. Haase, C. Herlmrich, A. Henkel, T. Hinz, S. de Luxán Hernández, S. Kaltenstadler, P. Keydel, H. Kirchhoffer, C. Lehmann, W.-Q. Lim, J. Ma, D. Maniry, D. Marpe, P. Merkle, T. Nguyen, J. Pfaff, J. Rasch, R. Rischke, C. Rudat, M. Schaefer, T. Schierl, H. Schwarz, M. Siekmann, R. Skupin, B. Stallenberger, J. Stegemann, K. Suehring, G. Tech, G. Venugopal, S. Walter, A. Wieckowski, T. Wiegand and M. Winken, "Description of SDR, HDR and 360º video coding technology proposal by Fraunhofer HHI," Document JVET-J0014, San Diego, USA, 2018. |
| --- | --- |
| [2] | S. De Luxán Hernández, H. M. D. Schwarz and T. Wiegand, "CE3: Line-based intra coding mode (Tests 1.4.1, 1.4.2 and 1.4.3)," Document JVET-K0049, Ljubljana, SI, 2018. |
| [3] | B. Bross, J. Chen and S. Liu, "Versatile Video Coding (Draft 2)," Document JVET-K1001, Ljubljana, SI, 2018. |
| [4] | G. Van der Auwera, J. Heo and A. Filippov, "Description of Core Experiment 3: Intra Prediction and Mode Coding," Document JVET-K1023, Ljubljana, SI, 2018. |
| [5] | Y. Lin, M. Mao, S. Song, J. Zheng, J. An and C. Zhu, "Prediction dependent transform for intra and inter frame coding," Document JVET-J0064, San Diego, US, 2018. |
| [6] | F. Bossen, J. Boyce, K. Suehring, X. Li and V. Seregin, "JVET common test conditions and software reference configurations for SDR video," Document JVET-K1010, Ljubljana, SI, 2018. |

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).