| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0080-v1 |
| --- | --- |

| Title: | CE1.1.1.1: CU partitioning along picture boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Shawmin Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results of Core Experiment 1.1.1.1 (CE1.1.1.1). The proposed method reportedly obeys the constraint on the maximum allowed binary-tree depth specified in the SPS for partitioning all the regular coding units (CUs) that are entirely contained within the current input picture boundaries. The extra processing for adaptively adjusting the allowed binary-tree depth for each CU is removed. This contribution further proposes to always adopt the inferred quad-tree split for partitioning the partial CUs crossing both right and bottom boundaries of the current picture, disregarding the constraint on the minimum allowed quad-tree CU size. When the current CU crosses both right and bottom picture boundaries and has the CU size less than or equal to the minimum quad-tree CU size, it is further partitioned into four smaller CUs with reduced size by half in both spatial dimension using one inferred quad-tree split, instead of using one inferred horizontal binary split further followed by one or two inferred vertical binary splits. The proposed method reportedly leads to 0.00%, 0.02%, and 0.03% average luma BD-rates for the AI, RA, and LB, respectively, under the VTM2.0.1 common test conditions (CTCs). It is asserted that the proposed CU partitioning at picture boundaries is simpler than that in VTM2.0.1.

# Introduction

The current VVC [1] allows to utilize quad-tree split and binary split for partitioning those partial CUs not entirely contained within the current input picture boundaries. However, the constraint on the maximum allowed binary-tree depth specified in the SPS is violated for partitioning the regular CUs that are entirely contained within the input picture boundaries. In [1], an additional counter, denoted by depthOffset, is maintained in each CU for tracking the number of the inferred binary splits performed for partitioning into the current CU along picture boundaries. The maximum allowed binary-tree depth is further increased by depthOffset for partitioning a current CU. The current method in VVC [1] enforces the constraint on the minimum allowed quad-tree CU size, which is equal to 8x8 under the VTM2.0.1 CTCs. When the width or the height of the current CU is less than or equal to the minimum allowed quad-tree CU size, quad-tree split is disallowed for partitioning the current CU.

# Proposed method

In the proposed method [2], the constraint on the maximum allowed binary-tree depth is obeyed for partitioning the regular CUs that are entirely contained within the current input picture boundaries. The additional counter for tracking the number of the inferred binary splits and other extra processing for adjusting the maximum allowed binary-tree depth is removed.

In the proposed method [2], when the current CU is across both right and bottom boundaries of the current picture, quad-tree split is inferred. Otherwise, when the current CU size is less than or equal to the minimum allowed quad-tree CU size, binary split is inferred. The constraint on the minimum allowed quad-tree CU size is not obeyed for partitioning the partial CUs across both right and bottom boundaries of the current picture. In this way, those partial CUs can be further partitioned into four smaller CUs with reduced size by half in both spatial dimension using one inferred quad-tree split, instead of using one stage inferred horizontal binary split further followed by one or two inferred vertical binary splits.

# Experimental results

The proposed method was integrated into the VTM2.0.1 reference software [3]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified VTM2.0.1 reference software. All the experimental experiments were conducted according to the CE1 test conditions specified in [4, 5]. Table 1 summarizes the simulation results of CE1.1.1.1 under the VTM2.0.1 configurations. It is shown that the proposed method with the simplified design for CU partitioning along picture boundaries does not exhibit significant difference in BD-rate and run time compared with the anchor results. The detailed simulation results can be found in the attached spreadsheets.

# Conclusions

This contribution reports the results of CE1.1.1.1 for partitioning the partial CUs that are not entirely contained within the current input picture boundaries. It is reportedly shown that the proposed method simplifies the process for partitioning the CUs along the picture boundaries while not incurring significant BD-rate changes in experimental results.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 2),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1001, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- S.-T. Hsiang and S.-M. Lei, “CE1.2.0.10: CU partitioning along picture boundaries,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K0224, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- The software VTM2.0.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-2.0.1

- F. Bossen, J. Boyce, X. Li, and V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1010, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- J. Ma, F. Le Léannec, and M. W. Park, “Description of Core Experiment 1 (CE1): Partitioning,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1021, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

Table 1: Summary of the average BD-rate and run time results for CE1.1.1.1

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 99% |
| Class A2 | 0.00% | 0.00% | 0.00% | 99% | 100% |
| Class B | 0.00% | 0.02% | 0.05% | 99% | 98% |
| Class C | 0.00% | 0.02% | 0.05% | 99% | 97% |
| Class E | 0.00% | -0.01% | 0.00% | 99% | 97% |
| Overall | 0.00% | 0.01% | 0.02% | 99% | 98% |
| Class D | 0.02% | 0.14% | -0.02% | 97% | 100% |
| Class F (optional) | 0.01% | 0.06% | 0.07% | 99% | 98% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.03% | 0.03% | 99% | 98% |
| Class A2 | 0.00% | 0.00% | -0.06% | 100% | 100% |
| Class B | 0.04% | 0.11% | -0.01% | 98% | 99% |
| Class C | 0.02% | 0.15% | 0.06% | 97% | 96% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 0.08% | 0.01% | 99% | 98% |
| Class D | 0.04% | 0.29% | 0.21% | 94% | 97% |
| Class F (optional) | 0.02% | 0.10% | 0.04% | 98% | 96% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.12% | 0.06% | 99% | 101% |
| Class C | 0.04% | -0.08% | -0.16% | 98% | 96% |
| Class E | -0.03% | -0.50% | -0.84% | 98% | 95% |
| Overall | 0.03% | -0.10% | -0.24% | 98% | 98% |
| Class D | 0.05% | 0.25% | -0.18% | 97% | 96% |
| Class F (optional) | 0.04% | 0.10% | -0.14% | 98% | 95% |