| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0081-v2 |
| --- | --- |

| Title: | CE1.2.1: Constraint for binary and ternary partitions |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chia-Ming Tsai, Chih-Wei Hsu, Tzu-Der Chuang, Ching-Yeh Chen, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chia-ming.tsai, cw.hsu, peter.chuang, chingyeh.chen, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Virtual pipeline data units (VPDUs) are defined as non-overlapping MxM-luma(L)/NxN-chroma(C) units in a picture. In hardware decoders, successive VPDUs are processed by multiple pipeline stages at the same time; different stages process different VPDUs simultaneously. The VPDU size is roughly proportional to the buffer size in most pipeline stages, so it is very important to keep the VPDU size small. In HEVC hardware decoders, the VPDU size is set to maximum transform block (TB) size. Enlarging maximum TB size from 32x32-L/16x16-C (as in HEVC) to 64x64-L/32x32-C (as in the current VVC) can bring coding gains, which results in 4X of VPDU size (64x64-L/32x32-C) expectedly in comparison with HEVC. However, in addition to quadtree (QT) coding unit (CU) partitioning, ternary tree (TT) and binary tree (BT) are adopted in VVC for achieving additional coding gains, and TT and BT splits can be applied to 128x128-L/64x64-C coding tree blocks (CTUs) recursively, which leads to 16X of VPDU size (128x128-L/64x64-C) in comparison with HEVC. To reduce the VPDU size in VVC, one constraint for BT and TT is proposed, and the VPDU size is defined as 64x64-L/32x32-C for the following.

Cond. 1:  For each VPDU containing one or multiple CUs, the CUs are completely contained in the VPDU.
Cond. 2:  For each CU containing one or more VPDUs, the VPDUs are completely contained in the CU.
Proposed constraint:  For each CTU, the above two conditions shall not be violated, and the processing order of CUs shall not leave a VPDU and re-visit it later.

To satisfy the above BT & TT constraint, four methods are tested, and BD-rates, encoding time, and decoding time of the four methods in comparison with VTM2.0.1 under the Random Access (RA) configuration are reported as follows.

[CE1.2.1.1] Encoder-only BT & TT constraint:
BD-rates = 0.26% (Y), 0.35% (U), 0.25% (V); EncT=95%; DecT=101%

[CE1.2.1.2] Normative BT & TT constraint:
BD-rates = 0.15% (Y), 0.20% (U), 0.14% (V); EncT=94%; DecT=101%

[CE1.2.1.3] Set maximum BT and TT size to 64:
BD-rates = 0.35% (Y), 0.43% (U), 0.27% (V); EncT=87%; DecT=101%

[CE1.2.1.4] Force 128x128 CTU to be split into four 64x64 CUs:
BD-rates: 1.55% (Y), 1.28% (U), 1.15% (V); EncT=84%; DecT=102%

# Introduction

Figure 1 shows examples of TT and BT splits that cause problem for 64x64-L/32x32-C pipelining.

Figure 1.  Examples of undesirable TT and BT splits for 64x64-L/32x32-C pipelining (VPDUs indicated by dashed lines)

# Proposed method

We define VPDUs as non-overlapping units of 64x64-L/32x32-C samples in the picture. The following constraint is proposed for BT and TT.

Cond. 1:  For each VPDU containing one or multiple CUs, the CUs are completely contained in the VPDU.
Cond. 2:  For each CU containing one or more VPDUs, the VPDUs are completely contained in the CU.
Proposed constraint:  For each CTU, the above two conditions shall not be violated, and the processing order of CUs shall not leave a VPDU and re-visit it later.

Examples in Figure 1 are disallowed by the constraint. Figure 2 shows examples that are allowed.

Figure 2.  Examples of allowed BT and TT splits for 64x64-L/32x32-C pipelining (VPDUs indicated by dashed lines)

# Implementation

To satisfy the proposed constraint, the following four methods are proposed, and the VPDU size is defined as 64x64-L/32x32-C for the following. Note that, in VTM2.0.1, a fast algorithm uses the BT split depth of the current best CU to early skip quad-tree split. In CE1.2.1.1 and CE1.2.1.2, the fast algorithm is not applied when current CU width or height is larger than 64.

## CE1.2.1.1

Before encoding one CU, a check process is performed. If the check process returns TRUE, normal encoding processes are applied to the current CU; otherwise, the current CU is forced to further split (i.e., skip the inter/intra mode decision of the current CU). This method is encoder-only and does not change the decoder.

[Check Process]
If (the above-left position of the current CU is the above-left position of a VPDU)
{
  If the current CU width is larger than 64 and the current CU width is not multiples of 64
    Return FALSE
  If the current CU height is larger than 64 and the current CU height is not multiples of 64
    Return FALSE
  If the VPDU to the left and the VPDU to the top of the current CU are not yet completely encoded
    Return FALSE

  Return TRUE
}
Else
{
  If the current CU is completely contained by a VPDU
    Return TRUE
  Else
    Return FALSE
}

## CE1.2.1.2

Apply the following normative restrictions to prohibit invalid BT and TT splits.

- For a 128x128 CTU, it shall not be partitioned by a TT split.

- For a 128xN or Nx128 CU, it shall not be partitioned by a TT split, where N ≤ 64.

- For a 128xN CU, the horizontal BT shall not be applied, where N ≤ 64.

- For an Nx128 CU, the vertical BT shall not be applied, where N ≤ 64.

Note that, the CU size equal to 128x128, 128x64, or 64x128 is allowed. The proposed text changes of VVC working draft [1] are highlighted in blue as follows.

- Allowed binary split process Input to this process is a binary split mode btSplit, a coding block width cbWidth, a coding block height cbHeight, a location ( x0, y0 ) of the top-left luma sample of the considered coding block relative to the top-left luma sample of the picture, a multi-type tree depth mttDepth, a MaxMttDepth offset depthOffset and a partition index partIdx. Output of this process is the variable allowBtSplit. Table 3-1 – Specification of parallelTtSplit, perpCbPos, perpCbSize, perpMaxPicSize and cbSize based on btSplit. btSplit = = SPLIT_BT_VER; btSplit = = SPLIT_BT_HOR. parallelTtSplit; SPLIT_TT_VER; SPLIT_TT_HOR. cbSize; cbWidth; cbHeight. The variables parallelTtSplit and cbSize are derived as specified in Table 3-1. The variable allowBtSplit is derived as follows: - If one or more of the following conditions are true, allowBtSplit is set equal to FALSE: - cbSize is less than or equal to MinBtSizeY - cbWidth is greater than MaxBtSizeY - cbHeight is greater than MaxBtSizeY - mttDepth is greater than or equal to MaxMttDepth + depthOffset - Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE - btSplit is equal to SPLIT_BT_VER - y0 + cbHeight is greater than pic_height_in_luma_samples - Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE - btSplit is equal to SPLIT_BT_HOR - x0 + cbWidth is greater than pic_width_in_luma_samples - y0 + cbHeight is smaller than or equal to pic_height_in_luma_samples - Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE: - mttDepth is greater than 0 - partIdx is equal to 1 - MttSplitMode[ x0 ][ y0 ][ mttDepth − 1 ] is equal to parallelTtSplit - Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE - btSplit is equal to SPLIT_BT_VER - cbWidth is smaller than or equal to 64 - cbHeight is larger than 64 - Otherwise if all of the following conditions are true, allowBtSplit is set equal to FALSE - btSplit is equal to SPLIT_BT_HOR - cbWidth is larger than 64 - cbHeight is smaller than or equal to 64 – Otherwise, allowBtSplit is set equal to TRUE.

## CE1.2.1.3

Set maximum BT and TT size to 64.

## CE1.2.1.4

Force QT split when CU width or height is larger than 64 (equivalent to “force 128x128 CTU to be split into four 64x64 CUs” under CTC)

# Experimental results

The proposed methods have been implemented on VTM2.0.1 [2]. The tests were performed following the JVET common test conditions (CTC) [3]. Table 1 and Table 2 are the results.

Table 1. Results of CE1.2.1.1: Encoder-only BT & TT constraint (left side) and CE1.2.1.2: Normative BT & TT constraint (right side)

|  | All Intra Main10 |  |  |  |  |  |  |  |  |  | All Intra Main10 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |  |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  |  | DecT |  | Y |  | U |  | V |  | EncT |  | DecT |
| Class A1 | 0.00% |  | 0.00% |  | 0.00% |  | 101% | 99% |  | 0.00% |  | 0.00% |  | 0.00% |  | 100% |  | 100% |  |
| Class A2 | 0.00% |  | 0.00% |  | 0.00% |  | 100% | 100% |  | 0.00% |  | 0.00% |  | 0.00% |  | 100% |  | 101% |  |
| Class B | 0.00% |  | 0.00% |  | 0.00% |  | 100% | 100% |  | 0.00% |  | 0.00% |  | 0.00% |  | 100% |  | 101% |  |
| Class C | 0.00% |  | 0.00% |  | 0.00% |  | 100% | 100% |  | 0.00% |  | 0.00% |  | 0.00% |  | 99% |  | 101% |  |
| Class E | 0.00% |  | 0.00% |  | 0.00% |  | 100% | 99% |  | 0.00% |  | 0.00% |  | 0.00% |  | 100% |  | 100% |  |
| Overall | 0.00% |  | 0.00% |  | 0.00% |  | 100% | 100% |  | 0.00% |  | 0.00% |  | 0.00% |  | 100% |  | 101% |  |
| Class D | 0.00% |  | 0.00% |  | 0.00% |  | 100% | 99% |  | 0.00% |  | 0.00% |  | 0.00% |  | 100% |  | 102% |  |
|  | Random Access Main 10 |  |  |  |  |  |  |  |  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT | DecT |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class A1 | 0.41% |  | 0.60% |  | 0.48% |  | 90% | 102% |  | 0.22% |  | 0.32% |  | 0.22% |  | 90% |  | 100% |  |
| Class A2 | 0.52% |  | 0.48% |  | 0.38% |  | 94% | 101% |  | 0.38% |  | 0.34% |  | 0.26% |  | 94% |  | 101% |  |
| Class B | 0.21% |  | 0.30% |  | 0.22% |  | 95% | 102% |  | 0.08% |  | 0.16% |  | 0.11% |  | 95% |  | 100% |  |
| Class C | 0.03% |  | 0.14% |  | 0.03% |  | 98% | 99% |  | 0.02% |  | 0.07% |  | 0.04% |  | 98% |  | 101% |  |
| Overall | 0.26% |  | 0.35% |  | 0.25% |  | 95% | 101% |  | 0.15% |  | 0.20% |  | 0.14% |  | 94% |  | 101% |  |
| Class D | 0.08% |  | 0.15% |  | 0.09% |  | 98% | 100% |  | 0.00% |  | -0.01% |  | 0.03% |  | 98% |  | 100% |  |
|  | Low delay B Main10 |  |  |  |  |  |  |  |  | Low delay B Main10 |  |  |  |  |  |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |  |  |  |  |  |
|  | Y |  | U |  | V |  | EncT | DecT |  | Y |  | U |  | V |  | EncT |  | DecT |  |
| Class B | 0.15% |  | 0.21% |  | 0.02% |  | 97% | 100% |  | 0.06% |  | 0.07% |  | -0.16% |  | 98% |  | 101% |  |
| Class C | 0.06% |  | 0.08% |  | 0.03% |  | 99% | 99% |  | 0.02% |  | 0.01% |  | -0.17% |  | 99% |  | 101% |  |
| Class E | 0.85% |  | 0.45% |  | 0.74% |  | 96% | 101% |  | 0.10% |  | 0.22% |  | -0.12% |  | 95% |  | 97% |  |
| Overall | 0.30% |  | 0.23% |  | 0.20% |  | 98% | 100% |  | 0.06% |  | 0.09% |  | -0.15% |  | 98% |  | 100% |  |
| Class D | 0.10% |  | 0.49% |  | -0.55% |  | 100% | 99% |  | 0.06% |  | 0.28% |  | 0.01% |  | 99% |  | 100% |  |

Table 2. Results of CE1.2.1.3: Set maximum BT and TT size to 64 (left side) and CE1.2.1.4: Force 128x128 CTU to be split into four 64x64 CUs (right side)

|  | All Intra Main10 |  |  |  |  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Overall | 0.00% | 0.00% | 0.00% | 100% | 100% | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class D | 0.00% | 0.00% | 0.00% | 100% | 102% | 0.00% | 0.00% | 0.00% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class A1 | 0.69% | 0.88% | 0.63% | 78% | 101% | 2.66% | 2.32% | 2.21% | 73% | 104% |
| Class A2 | 0.67% | 0.64% | 0.45% | 86% | 102% | 2.87% | 2.27% | 2.04% | 82% | 102% |
| Class B | 0.24% | 0.28% | 0.16% | 88% | 101% | 1.25% | 0.96% | 0.88% | 84% | 101% |
| Class C | 0.01% | 0.11% | 0.01% | 95% | 100% | 0.10% | 0.14% | 0.04% | 94% | 100% |
| Overall | 0.35% | 0.43% | 0.27% | 87% | 101% | 1.55% | 1.28% | 1.15% | 84% | 102% |
| Class D | 0.06% | 0.10% | 0.01% | 94% | 99% | 0.06% | 0.13% | -0.04% | 93% | 98% |
|  | Low delay B Main10 |  |  |  |  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT | Y | U | V | EncT | DecT |
| Class B | 0.17% | 0.08% | 0.14% | 92% | 102% | 0.85% | 0.49% | 0.30% | 89% | 99% |
| Class C | 0.05% | 0.07% | -0.21% | 98% | 101% | 0.10% | -0.13% | 0.11% | 97% | 99% |
| Class E | 0.51% | 0.60% | 0.55% | 81% | 100% | 2.26% | 2.35% | 1.78% | 73% | 100% |
| Overall | 0.22% | 0.21% | 0.13% | 91% | 101% | 0.95% | 0.75% | 0.60% | 87% | 100% |
| Class D | 0.05% | 0.15% | 0.13% | 98% | 99% | 0.05% | 0.46% | -0.10% | 97% | 99% |

# Conclusions

One constraint is proposed for BT and TT to allow 64x64-L/32x32-C pipelining to be used in VVC hardware decoders. Four methods are implemented to satisfy the constraint. From the results, the method of normative BT & TT constraint shows the lowest coding efficiency degradation among the four methods.

# Reference

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 2),” Document of Joint Video Experts Team, JVET-K1001, July 2018.

- VTM-2.0.1: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-2.0.1

- F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-K1010, July 2018.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).