| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0084-v1 |
| --- | --- |

| Title: | CE3.2.5: Generation of right-column and bottom-row predictors for planar mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {mohammed.sarwer, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution, a method to compute bottom-row and right-column prediction samples for intra planar mode is presented. At first, the bottom-right prediction sample is generated by averaging two different predictors. Then bottom-row prediction samples are computed based on weighted average of bottom-left and bottom-right samples, and right-column prediction samples are generated based on weighted average of top-right and bottom-right samples. Compared to VTM2.0.1, it is reported that the proposed planar mode achieves -0.07% (Y), -0.04% (U), and 0.02% (V) BD-rates for AI.

# Introduction

The planar prediction in VTM2.0.1 is defined as average of two linear predictions as shown in Figure 1, where W is the width of the coding block (CB) and H is the height of the CB.

Figure 1.  Planar prediction in VTM-2.0

At first the predictors of right-column and bottom-row are generated by copying top-right and bottom-left reference samples by using Equation 1 and Equation 2, respectively.

PrW, y=R(W, -1)                                                    (1)

Pbx, H=R(-1, H)                                                     (2)

Then the horizontal and vertical predictors of (x,y) position of the block is computed by Equation 3 and Equation 4.

Phx,y=W-1-x×R-1, y+x+1×Pr(W, y)                       (3)

Pvx,y=H-1-y×Rx, -1+y+1×Pb(x, H)                        (4)

The final predicted sample is obtained by Equation 5.

Px,y= (Phx, y≪log2H+ Pvx, y≪log2W)≫shift1                (5)

In Equation 5, shift1 is defined as follows.

shift1=1+ log2W+ log2H                                             (6)

# Proposed method

The first step to generate right-column and bottom-row predictors is to predict the bottom-right (BR) sample of a CB. The bottom-right sample (Pbr(W, H)) of a CB with width = W and height = H are shown in red square in Figure 2. The bottom-right sample is defined as the average value of two bottom-right predictors as follows.

PbrW, H=(Pbr1W, H+ Pbr2W, H+1)≫1                      (7)

(x, y)

Pbr(W, H)

R(W, -1)

R(-1, H)

R(-1, -1)

Figure 2.  Proposed planar prediction

Here, the first BR predictor Pbr1W, H is defined as follows.

Pbr1W, H= (w1×R-1, H+w2×RW, -1+scale2)≫shift2                     (8)

In Equation (8), shift2, w1, w2 and scale2 are defined as follows.

shift2=ceil(log2W+H)                                                       (9)

w1=                   W,  if W≤H1≪shift2-H,  else                                                 (10)

w2=                   1≪shift2-W,  if W≤HH,  else                                        (11)

scale2=1≪shift2≫1                                                        (12)

It should be noted that the multiplication operation of Equation 8 can be implemented by shift and addition operations.

The second bottom-right predictor Pbr2W, H is selected from a pre-defined candidate list. The candidate list of second predictor consists of 9 predictors (8 predictors from reference sample list and Pbr1), which are tabulated in Table 1.

Table 1.  Candidates list Cbr2(i) for Pbr2W, H computation

| Index (i) | Predictor sample Cbr2 (i) |
| --- | --- |
| 0 | Pbr1W, H |
| 1 | RW/2, -1 |
| 2 | RW, -1 |
| 3 | RW+W/2, -1 |
| 4 | R2W-1, -1 |
| 5 | R-1, H/2 |
| 6 | R-1, H |
| 7 | R-1, H+H/2 |
| 8 | R-1, 2H-1 |

The second predictor Pbr2W, H is defined as follows.

Pbr2W, H=Cbr2(ibest)                                                          (13)

The best index, ibest, is found by minimizing the following cost.

Costi= abs(X-Cbr2i)                                                        (14)

X=clip(0, maxPixelValue, RW, -1+R-1, H-R -1, -1)                         (15)

maxPixelValue=1≪bitDepth-1                                               (16)

In Equation 14, abs() is the absolute operation and the value of i is 0 to 8 as shown in Table 1.

After computing the Pbr1W, H and Pbr2W, H, the final bottom-right predictor PbrW, H is obtained by Equation 7. Finally, right-column and bottom-left reference samples are obtained by following equations.

PrW, y=H-1-y×RW, -1+ y+1×PbrW, H≫(log2H)           (17)

Pbx, H=(W-1-x×R-1, H+ x+1×PbrW, H)≫(log2W)           (18)

Once the right-column and bottom-left predicted samples are available, Equations 3, 4, and 5 are used to generate final prediction signal.

# Simulation results

The proposed methods are implemented in BMS2.0.1 with VTM configuration. All of the simulations are performed based on common test conditions [1]. Table 2 shows the results of the proposed method.

Table 2.  Simulation results of proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.07% | -0.07% | -0.04% | 100% | 97% |
| Class A2 | -0.07% | -0.03% | -0.05% | 101% | 98% |
| Class B | -0.04% | 0.03% | 0.10% | 100% | 99% |
| Class C | -0.03% | -0.03% | 0.12% | 100% | 99% |
| Class E | -0.17% | -0.14% | -0.13% | 101% | 98% |
| Overall | -0.07% | -0.04% | 0.02% | 100% | 98% |
| Class D | -0.03% | 0.17% | -0.03% | 100% | 100% |
| Class F (optional) | -0.01% | -0.12% | 0.10% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | 0.15% | 0.05% | 100% | 101% |
| Class A2 | -0.02% | -0.13% | 0.04% | 100% | 93% |
| Class B | 0.00% | -0.02% | 0.05% | 100% | 98% |
| Class C | 0.02% | -0.03% | 0.17% | 100% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.01% | -0.01% | 0.08% | 100% | 98% |
| Class D | 0.02% | -0.11% | 0.30% | 100% | 100% |
| Class F (optional) | -0.02% | -0.13% | 0.11% | 100% | 101% |

# Conclusions

As compared to VTM2.0.1, in All Intra configuration, the proposed method achieves BD-rates of -0.07% / -0.04% / 0.02% for Y/U/V, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-K1010.