| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0086-v2 |
| --- | --- |

| Title: | CE3.6.4: Intra mode coding with 6 MPMs with bypass coding bins and non-MPMs with fixed length coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {mohammed.sarwer, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents a luma intra mode coding method for VVC. In the proposed method, 6 most probable modes (MPMs) are computed based on the above and left neighboring modes. Truncated unary binarization with bypass coding bins is used to signal the MPM index. The 61 remaining modes are signaled using 6-bit fixed-length coding (FLC). In order to limit the line buffer requirement, MPM generation with a coding tree unit (CTU) row constraint is also tested. It is reported that under All Intra configuration this proposal without the CTU row constraint can achieve -0.33%/-0.30%/-0.27% Y/U/V BD-rate for VTM2.0. With the CTU row constraint, the BD-rate is -0.29%/-0.24%/-0.21%. Encoding time and decoding time of this proposal are similar to those of VTM2.0.1.

# Introduction

Intra prediction is a significant coding technique to reduce the spatial redundancy between the neighboring samples of the image. In VTM2.0.1, for luma intra mode coding, at first a list of 3 MPMs is generated. Truncated unary coding is used to signal MPM index. For 64 remaining modes, 6-bit FLC is used. Figure 1 shows the neighboring modes to generate the MPM list. Above mode (A) and left mode (L) are used to derive the list of 3 MPMs.

Figure 1: Neighboring modes for MPM generation in VTM2.0.1

In VTM2.0.1, after extracting the left neighboring mode (leftIntraDir) and above neighboring mode (aboveIntraDir ), the MPM list (mpm[]) is constructed as follows.

offset = 62;

mod = offset + 3;

if (leftIntraDir == aboveIntraDir)

{

if (leftIntraDir > DC_IDX)   // angular modes

{

mpm[0] = leftIntraDir;

mpm[1] = ((leftIntraDir + offset) % mod) + 2; // -1 mode of left mode

mpm[2] = ((leftIntraDir - 1) % mod) + 2;      // +1 mode of left mode

}

else   // non-angular

{

mpm[0] = PLANAR_IDX;

mpm[1] = DC_IDX;

mpm[2] = VER_IDX;

}

}

else

{

mpm[0] = leftIntraDir;

mpm[1] = aboveIntraDir;

if (leftIntraDir && aboveIntraDir)   // both modes are non-planar

{

mpm[2] = PLANAR_IDX;

}

else

{

mpm[2] = (leftIntraDir + aboveIntraDir) < 2 ? VER_IDX : DC_IDX;

}

}

# Proposed method

## MPM list generation

As shown in Figure 2, the proposed method uses two neighboring modes to generate the MPM list of 6 MPMs. The MPM generation part is categorized into 3 cases. Cases are defined based on the number of available angular modes in left (L) and above (A) neighbors. Assume, number of available angular modes is Nang. The value of Nang can be varied from 0 to 2.

A

L

Figure 2: Neighboring modes for MPM generation

### Case 0: Number of available angular modes (Nang) == 0

In this case, 6 MPM list is generated as follows.

mpm[0] = PLANAR_IDX;

mpm[1] = DC_IDX;

mpm[2] = VER_IDX;

mpm[3] = HOR_IDX;

mpm[4] = 2;

mpm[5] = DIA_IDX;

### Case 1: Number of available angular modes (Nang) == 1

According to definition, the number of angular modes in this case is 1. Assume,

- leftIntraDir = left mode

- aboveIntraDir = above mode

- AngMode is the available angular mode after checking left and above mode.

- availNonAngMode is the available non-angular mode after checking left and above mode. If any non-angular mode is not available, availNonAngMode = -1.

offset = 62;

mod = offset + 3;

if (both top and left modes are available and they are different)

{

mpm[0] = leftIntraDir;

mpm[1] = aboveIntraDir;

mpm[2] = (!availNonAngMode); // if availNonAngMode is PLANAR, mpm[2] is DC

// Otherwise mpm[2] is PLANAR

}

else

{

mpm[0] = AngMode;

mpm[1] = PLANAR_IDX;

mpm[2] = DC_IDX;

}

mpm[3] = ((AngMode + offset) % mod) + 2; // -1 mode of angular mode

mpm[4] = ((AngMode - 1) % mod) + 2; // +1 mode of the angular mode

mpm[5] = VER_IDX - (((AngMode > (VER_IDX - 2) && AngMode < (VER_IDX + 2))) << 5);

### Case 2: Number of available angular modes (Nang) == 2

According to definition, both the top and left modes of this category are angular modes,

Assume,

- leftIntraDir = left mode

- aboveIntraDir = above mode

offset = 62;

mod = offset + 3;

uiAngModeList[2] = { leftIntraDir, aboveIntraDir};

diff = abs(uiAngModeList[0] - uiAngModeList[1]);

isadjacentMode = !(diff % 63 - 1);

isleftLargeXORisMaxDiff = (uiAngModeList[0] > uiAngModeList[1]) ^ (!(diff - 64));

uiMinIdx = isadjacentMode & isleftLargeXORisMaxDiff;

uiMaxIdx = isadjacentMode & (!isleftLargeXORisMaxDiff);

mpm[0] = leftIntraDir;

mpm[1] = aboveIntraDir;

mpm[2] = PLANAR_IDX;

mpm[3] = DC_IDX;

mpm[4] = ((uiAngModeList[uiMinIdx] + offset) % mod) + 2; // -1

mpm[5] = ((uiAngModeList[uiMaxIdx] - 1) % mod) + 2;  // +1

## Bypass coding to signal MPM indices

In the proposed method, truncated unary coding is used to signal MPM indices. All of the bins are coded with bypass coding, and there is no context coding used for signaling MPM indices.

Table 1.  Binarization of MPM index

| MPM Index | Binarization |
| --- | --- |
| 0 | 1 |
| 1 | 01 |
| 2 | 001 |
| 3 | 0001 |
| 4 | 00001 |
| 5 | 00000 |

## FLC of non-MPM indices

In the proposed method, the number of non-MPMs is 61. Although the number of non-MPMs is not power of 2, still, for simplicity, the 6-bit FLC is used to signal non-MPM indices.

Since number of non-MPMs is not power of 2 and FLC is used to signal non-MPM indices, a special constraint needs to be applied in encoder and/or decoder.

- Encoder-side constraint: It is a requirement of bitstream conformance that the value of non-MPM-index shall not be larger than 60. If the non-MPM index is larger than 60, the value is clipped to 60.

- Decoder-side constraint: In the decoder side, if the decoded non-MPM index is larger than 60, it is truncated to 60.

## CTU-row constraint for removing line buffer requirement

In order to avoid the line buffer requirement during intra mode coding, the access to neighboring coding units (CUs) is allowed only if that CU belongs to the same CTU row. If the neighboring CU is outside of current CTU row, that neighboring mode is skipped for MPM generation.

# Simulation results

The proposed methods are implemented in BMS2.0.1 with VTM configuration. The number of RD checks of this proposal is aligned with that of VTM2.0.1. Table 2 shows the simulation results of the proposed methods without the CTU-row constraint described in Section 2.4. Table 3 shows the simulation results of the proposed methods when the CTU-row constraint is enabled. All of the tests are performed under common test conditions described in [2].

# Additional results

Table 4 and Table 5 show the simulation results of the proposed methods if truncated binary is used for non-MPM coding. Table 6 shows the comparison of this proposal and VTM2.0.1.

# Conclusions

Compared to VTM2.0.1, this proposal reportedly achieves BD-rates of -0.33%/-0.30%/-0.27% for Y/U/V, respectively, in All Intra configuration. If the CTU-row constraint is applied as simplification of the proposed method, the BD-rates of All Intra condition are -0.29%/-0.24%/-0.21% for Y/U/V, respectively.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 2),” Document of Joint Video Experts Team, JVET-K1001-v4, Ljubljana, SI, 10–18 July 2018.

[2] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-K1010.

Table 2.  Simulation results of the proposed methods without CTU-row constraint

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.22% | -0.18% | -0.20% | 100% | 97% |
| Class A2 | -0.31% | -0.31% | -0.37% | 100% | 97% |
| Class B | -0.33% | -0.31% | -0.20% | 100% | 98% |
| Class C | -0.34% | -0.34% | -0.18% | 100% | 99% |
| Class E | -0.44% | -0.37% | -0.50% | 100% | 99% |
| Overall | -0.33% | -0.30% | -0.27% | 100% | 98% |
| Class D | -0.16% | -0.03% | -0.30% | 100% | 101% |
| Class F (optional) | -0.49% | -0.51% | -0.52% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.14% | -0.06% | -0.07% | 101% | 99% |
| Class A2 | -0.15% | -0.09% | -0.09% | 100% | 93% |
| Class B | -0.13% | -0.15% | -0.07% | 100% | 99% |
| Class C | -0.09% | 0.10% | 0.26% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.12% | -0.05% | 0.01% | 100% | 98% |
| Class D | -0.04% | -0.40% | -0.13% | 100% | 99% |
| Class F (optional) | -0.33% | -0.23% | -0.01% | 100% | 100% |

Table 3.  Simulation results of proposed methods with CTU-row constraint

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | -0.19% | -0.16% | 100% | 97% |
| Class A2 | -0.26% | -0.22% | -0.27% | 100% | 99% |
| Class B | -0.30% | -0.23% | -0.18% | 100% | 99% |
| Class C | -0.31% | -0.27% | -0.14% | 100% | 99% |
| Class E | -0.35% | -0.29% | -0.35% | 100% | 99% |
| Overall | -0.29% | -0.24% | -0.21% | 100% | 99% |
| Class D | -0.16% | -0.05% | -0.20% | 101% | 101% |
| Class F (optional) | -0.44% | -0.40% | -0.45% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.13% | -0.07% | -0.04% | 100% | 101% |
| Class A2 | -0.14% | 0.00% | -0.10% | 100% | 92% |
| Class B | -0.09% | -0.13% | -0.03% | 100% | 100% |
| Class C | -0.06% | 0.08% | -0.08% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | -0.10% | -0.04% | -0.06% | 100% | 98% |
| Class D | -0.06% | -0.35% | -0.23% | 99% | 99% |
| Class F (optional) | -0.28% | -0.31% | -0.07% | 100% | 99% |

Table 4.  Simulation results of proposed methods without CTU-row constraint and using truncated binary coding for non-MPMs

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.24% | -0.26% | -0.26% | 100% | 101% |
| Class A2 | -0.32% | -0.32% | -0.40% | 100% | 103% |
| Class B | -0.35% | -0.27% | -0.25% | 100% | 101% |
| Class C | -0.38% | -0.29% | -0.20% | 99% | 100% |
| Class E | -0.49% | -0.43% | -0.50% | 100% | 100% |
| Overall | -0.36% | -0.31% | -0.30% | 100% | 101% |
| Class D | -0.21% | 0.00% | -0.25% | 100% | 101% |
| Class F (optional) | -0.54% | -0.50% | -0.48% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.05% | -0.04% | 99% | 103% |
| Class A2 | -0.18% | -0.18% | -0.14% | 100% | 102% |
| Class B | -0.12% | -0.11% | -0.05% | 100% | 100% |
| Class C | -0.11% | -0.10% | -0.09% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.14% | -0.11% | -0.08% | 100% | 101% |
| Class D | 0.00% | -0.16% | 0.25% | 100% | 99% |
| Class F (optional) | -0.35% | -0.36% | -0.08% | 100% | 101% |

Table 5.  Simulation results of proposed methods with CTU-row constraint and using truncated binary coding for non-MPMs

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.27% | -0.21% | -0.18% | 100% | 101% |
| Class A2 | -0.28% | -0.25% | -0.31% | 101% | 101% |
| Class B | -0.31% | -0.24% | -0.17% | 100% | 101% |
| Class C | -0.33% | -0.26% | -0.16% | 100% | 101% |
| Class E | -0.40% | -0.36% | -0.34% | 101% | 100% |
| Overall | -0.32% | -0.26% | -0.22% | 100% | 101% |
| Class D | -0.21% | 0.12% | -0.07% | 100% | 101% |
| Class F (optional) | -0.52% | -0.54% | -0.47% | 101% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.15% | -0.08% | -0.11% | 100% | 102% |
| Class A2 | -0.15% | 0.20% | -0.04% | 100% | 102% |
| Class B | -0.13% | -0.02% | 0.06% | 100% | 100% |
| Class C | -0.10% | -0.06% | 0.00% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.13% | 0.00% | -0.01% | 100% | 101% |
| Class D | -0.07% | -0.23% | -0.04% | 100% | 99% |
| Class F (optional) | -0.32% | -0.24% | 0.04% | 100% | 101% |

Table 6.  Comparison between VTM2.0.1 and this proposal

|  | VTM2.0.1 | Proposed |
| --- | --- | --- |
| Number of MPM | 3 | 6 |
| Number of neighboring modes | 2 | 2 |
| Line buffer | N | N |
| Maximum layer of if conditions | 2 | 3 |
| Max number of comparison operators | 2 | 8 |
| Max number of logical operators | 4 | 5 |
| Max number of assignment operators | 15 | 27 |
| Max number of increments | 0 | 4 |
| Max number of bit operations | 0 | 3 |
| Parsing dependency | N | N |
| Number of context model | 1 | 1 |
| Number of full RDO check | 1 or 2 | 1 or 2 |
| Has LUT? | N | N |
| Number of condition check for remaining modes | 0 | 0 |
| Non-MPM coding | 6-bit FLC | 6-bit FLC |