| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0087-v2 |
| --- | --- |

| Title: | CE3-related: Boundary PDPC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd, Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {mohammed.sarwer, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc |  |  |

_____________________________

# Abstract

In this contribution, position dependent intra prediction combination (PDPC) is applied only to top-left boundary samples. Compared to VTM2.0.1, it is reported that the proposed boundary PDPC can simplify the PDPC operation with 0.03% (Y), 0.05% (U), and 0.06% (V) BD-rates for AI.

# Introduction

In VTM2.0.1, PDPC is applied to planar, DC, horizontal, vertical, bottom-left angular mode and its 8 adjacent angular modes, and top-right angular mode and its 8 adjacent angular modes [1] [2]. The prediction sample pred(x,y) located at position (x, y) is predicted using an intra prediction mode (DC, planar, angular) and a linear combination of reference samples according to the PDPC expression [2]:

pred(x,y) = ( wL × R-1,y + wT × Rx,-1 – wTL × R-1,-1 +  (64 – wL – wT + wTL) × pred(x,y) + 32 )  >>  6,     (1)

where Rx,-1, R-1,y represent the reference samples located at the top and left of current sample (x, y), respectively, and R-1,-1 represents the reference sample located at the top-left corner of the current block. The weights wL, wT, and wTL depend on the prediction mode, width, height, and position. The detail of weight computations can be found in [2].

In VTM2.0.1, PDPC is applied to the all samples of a coding block (CB). It is observed that the effect of PDPC is reduced with increasing the (x, y) position and in some cases the effect of PDPC is completely vanished at the bottom-right corner of a CB. Figure 1 shows an example of variation of weight wL with x position of a 64x64 CB with planar mode. It is shown that the value of wL is 0 when x > = 12. Regardless of the value of the weight, in VTM2.0.1, PDPC is applied to all samples.

Figure 1. Variation of PDPC weight wL with x position of a 64x64 CB with planar mode (horizontal axis: x position, vertical axis: wL)

# Proposed method

In this proposal, PDPC is applied only to the top-left boundary positions. The number of vertical and horizontal lines for PDPC depends on the width and height of a CB. Assume, PDPC is applied to Nhor  horizontal lines and Nver vertical lines. Then,

if x≥ Nver &&y≥ Nhor,

PDPC is not applied

otherwise,

PDPC is applied as per Eq (1)

The set of thresholds values to determine Nhor and Nver are defined as follows.

T7={1, 2, 4, 4, 8, 8, 8}                                                           (2)

Nver=T[log2width-1]                                                        (3)

Nhor=T[log2height-1]                                                       (4)

# Simulation results

The proposed method is implemented in BMS2.0.1 with VTM configuration. All of the simulations are performed based on common test conditions [3]. Table 1 shows the results of the proposed method.

Table 2 shows the number of operations of the conventional PDPC and the proposed boundary PDPC in case of planar mode of a 8x8 and a 64x64 CB. For each horizontal line, weight wT is computed which requires 3 shift and 1 comparison operations. For each (x, y) position, wL is updated which also requires 3 shift and 1 comparison operations. Finally, 4 shifts, 3 additions, and 2 comparators are required to compute pred(x,y) of a (x, y) position.

# Conclusions

As compared to PDPC in VTM2.0.1, in All Intra configuration, the proposed method can reduce 24% - 76% operations with 0.03%/0.05%/0.06% Y/U/V BD-rates.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# Reference

[1] B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 2),” Document of Joint Video Experts Team, JVET-K1001.

[2] G. V. Auwere, V. Seregin, A. Said, A. K. Ramasubramonian, M. Ramasubramonian, “Simplified PDPC,” Document of Joint Video Experts Team, JVET-K0063.

[3] F. Bossen, J. Boyce, K. Suehring, X. Li, V. Seregin, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team, JVET-K1010.

Table 1.  Results of the proposed method

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.05% | 0.06% | 0.01% | 99% | 99% |
| Class A2 | 0.01% | 0.03% | -0.01% | 100% | 100% |
| Class B | 0.03% | 0.07% | 0.11% | 100% | 99% |
| Class C | 0.00% | 0.06% | 0.06% | 100% | 99% |
| Class E | 0.05% | 0.04% | 0.06% | 100% | 97% |
| Overall | 0.03% | 0.05% | 0.06% | 100% | 99% |
| Class D | -0.01% | 0.22% | -0.03% | 100% | 100% |
| Class F (optional) | -0.01% | -0.04% | 0.13% | 100% | 99% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | 0.01% | 0.05% | 0.17% | 100% | 102% |
| Class A2 | 0.03% | -0.07% | 0.07% | 100% | 101% |
| Class B | 0.01% | 0.06% | 0.03% | 100% | 100% |
| Class C | 0.00% | 0.12% | 0.04% | 100% | 99% |
| Class E |  |  |  |  |  |
| Overall | 0.01% | 0.05% | 0.07% | 100% | 100% |
| Class D | -0.04% | -0.18% | 0.17% | 100% | 100% |
| Class F (optional) | -0.04% | -0.02% | -0.06% | 100% | 100% |

Table 2. Number of operations

| Operations | 8x8 planar mode |  | 64x64 planar mode |  |
| --- | --- | --- | --- | --- |
|  | Conventional PDPC | Boundary PDPC | Conventional PDPC | Boundary PDPC |
| shift | 472 | 360 | 28864 | 6912 |
| comparator | 200 | 152 | 12352 | 2944 |
| additions | 192 | 144 | 12288 | 2880 |
| Total | 864 | 656 | 53504 | 12736 |
| Saving (in %) |  | 24.07% |  | 76.2% |