| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0089-v1 |
| --- | --- |

| Title: | CE4.4.4: Non-adjacent merge candidates with buffer size reduction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Ling Hsiao, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yuling.hsiao, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes two motion vector (MV) buffer reduction methods for non-adjacent spatial merge candidates. The first method is to apply a coding tree unit (CTU) boundary constraint. The second method is to apply reference MV position rounding and a 2-CTU constraint. The two methods are applied to the non-adjacent spatial merge candidates in CE4.4.2. Results are summarized as follows.

Non-adjacent spatial merge candidates in CE4.4.2 & CTU boundary constraint
VTM2.0.1-RA: {Y-BD-rate = -0.56%, EncT=100%, DecT=101%}
VTM2.0.1-LB: {Y-BD-rate = -0.41%, EncT=100%, DecT=102%}

Non-adjacent spatial merge candidates in CE4.4.2 & reference MV position rounding & 2-CTU constraint
VTM2.0.1-RA: {Y-BD-rate = -0.47%, EncT=100%, DecT=101%}
VTM2.0.1-LB: {Y-BD-rate = -0.36%, EncT=100%, DecT=101%}

# Introduction

Non-adjacent spatial merge candidates were planned in CE4.4.2, CE4.4.3, CE4.4.5, CE4.4.6, and 4.4.10, as described in JVET-K1024 [1]. Reduction of buffer size for non-adjacent merge candidates was presented in JVET-K0246 [2] and planned as CE4.4.4 in JVET-K1024. In this document, two methods are proposed to reduce the MV buffer requirement for non-adjacent spatial merge candidates from CE4.4.2. Adjacent spatial merge candidates are kept totally the same as those in VTM2.0.1. More details are introduced in the next sections.

# Proposed methods

## CTU boundary constraint

A CTU boundary constraint is applied to non-adjacent spatial merge candidates. MVs belonging to the above/left/above-left CTU and non-adjacent to the current CTU are replaced with MVs belonging to the above/left/above-left CTU and adjacent to the current CTU. As shown in Figure 1, the MV at block A is fetched as an alternative to MV at block B. This CTU boundary constraint also used in CE4.4.2.

Figure 1. Example of CTU boundary constraint; current CU size = 16x16

## Reference MV position rounding and 2-CTU constraint

Similar to the temporal MV compression in HEVC that only stores MV data of the top left 4x4 block in each 16x16 block of a reference picture, only the MV data of the top left 4x4 block in each 8x8 block of the current picture are stored for future use of MV coding. In addition, non-adjacent spatial MVs outside the current CTU and left CTU are not used. Figure 2 shows an example.

Figure 2. Example of reference MV position rounding and 2-CTU constraint; current CU size = 16x16

# Experiment results

The proposed modifications have been implemented on top of VTM2.0.1 with non-adjacent spatial merge candidates same as those in CE4.4.2. The configurations of CE4.4.2 are MaxNumMergeCand=8, MaxSearchRound=2, and MaxMergeCompNum=30, and they are used for the following two tests. The common test conditions [3] are used for evaluation. Table 1 summarizes the results of the CTU boundary constraint. Table 2 summarizes the results of the reference MV position rounding and 2-CTU constraint.

Table 1. CE4.4.2 with the CTU boundary constraint

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.36% | -0.44% | -0.61% | 100% | 102% |
| Class A2 | -0.57% | -0.91% | -1.06% | 99% | 102% |
| Class B | -0.75% | -0.80% | -0.87% | 99% | 101% |
| Class C | -0.46% | -0.42% | -0.55% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.56% | -0.65% | -0.77% | 100% | 101% |
| Class D | -0.32% | -0.26% | -0.36% | 101% | 100% |
| Class F (optional) | -0.38% | -0.38% | -0.40% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.60% | -0.66% | -0.66% | 100% | 105% |
| Class C | -0.31% | -0.48% | -0.46% | 100% | 101% |
| Class E | -0.23% | -0.51% | -0.48% | 100% | 98% |
| Overall | -0.41% | -0.56% | -0.55% | 100% | 102% |
| Class D | -0.21% | 0.22% | -0.32% | 101% | 101% |
| Class F (optional) | -0.58% | -0.16% | -0.62% | 101% | 101% |

Table 2. CE4.4.2 with the reference MV position rounding and 2-CTU constraint

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.26% | -0.27% | -0.41% | 100% | 101% |
| Class A2 | -0.50% | -0.75% | -0.83% | 100% | 102% |
| Class B | -0.64% | -0.61% | -0.75% | 100% | 101% |
| Class C | -0.41% | -0.48% | -0.44% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.47% | -0.53% | -0.62% | 100% | 101% |
| Class D | -0.31% | -0.25% | -0.48% | 101% | 101% |
| Class F (optional) | -0.35% | -0.38% | -0.33% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.54% | -0.68% | -0.66% | 100% | 103% |
| Class C | -0.24% | -0.39% | -0.46% | 101% | 102% |
| Class E | -0.23% | 0.08% | -0.02% | 100% | 98% |
| Overall | -0.36% | -0.39% | -0.43% | 100% | 101% |
| Class D | -0.16% | 0.42% | -0.50% | 101% | 101% |
| Class F (optional) | -0.52% | -0.46% | -0.15% | 101% | 101% |

# Conclusion

Two methods are proposed to reduce the MV buffer requirement for non-adjacent spatial merge candidates. Adding CE4.4.2 non-adjacent spatial merge candidates with a CTU boundary constraint achieved -0.56% and -0.41% luma BD-rates for VTM2.0.1-RA and VTM2.0.1-LB, respectively, with 100% and 100% encoding time and 101% and 102% decoding time. Adding CE4.4.2 non-adjacent spatial merge candidates with reference MV position rounding and a 2-CTU constraint achieved -0.47% and -0.36% luma BD-rates for VTM2.0.1-RA and VTM2.0.1-LB, respectively, with 100% and 100% encoding time and 101% and 101% decoding time.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] H. Yang, S. Liu, K. Zhangi, "Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding," Document of Joint Video Experts Team, JVET-K1024.

[2] Y.-L. Hsiao, T.-D. Chuang, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, "CE4.2-related: MV buffer reduction for non-adjacent spatial merge candidates," Document of Joint Video Experts Team, JVET-K0246.

[3] J. Boyce, K. Suehring, X. Li, V. Seregin, "JVET common test conditions and software reference configurations," Document of Joint Video Experts Team, JVET-K1010.