| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0090-v1 |
| --- | --- |

| Title: | CE4.4.12: Pairwise average candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yu-Ling Hsiao, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {yuling.hsiao, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes pairwise average candidates. Pairwise average candidates are generated by averaging predefined pairs of candidates in the current merge candidate list. Results are summarized as follows.

Pairwise average candidates with merge list size equals to 6, coexist with HEVC combined candidates
VTM2.0.1-RA: {Y-BD-rate = -0.40%, EncT=100%, DecT=101%}
VTM2.0.1-LB: {Y-BD-rate = 0.05%, EncT=101%, DecT=102%}

Pairwise average candidates with merge list size equals to 8, coexist with HEVC combined candidates
VTM2.0.1-RA: {Y-BD-rate = -0.61%, EncT=101%, DecT=101%}
VTM2.0.1-LB: {Y-BD-rate = -0.11%, EncT=102%, DecT=102%}

Pairwise average candidates with merge list size equals to 10, coexist with HEVC combined candidates
VTM2.0.1-RA: {Y-BD-rate = -0.69%, EncT=101%, DecT=102%}
VTM2.0.1-LB: {Y-BD-rate = -0.14%, EncT=102%, DecT=101%}

Pairwise average candidates with merge list size equals to 6, replace HEVC combined candidates
VTM2.0.1-RA: {Y-BD-rate = -0.38%, EncT=100%, DecT=101%}
VTM2.0.1-LB: {Y-BD-rate = 0.01%, EncT=101%, DecT=102%}

Pairwise average candidates with merge list size equals to 8, replace HEVC combined candidates
VTM2.0.1-RA: {Y-BD-rate = -0.54%, EncT=101%, DecT=100%}
VTM2.0.1-LB: {Y-BD-rate = -0.07%, EncT=101%, DecT=102%}

Pairwise average candidates with merge list size equals to 10, replace HEVC combined candidates
VTM2.0.1-RA: {Y-BD-rate = -0.61%, EncT=101%, DecT=101%}
VTM2.0.1-LB: {Y-BD-rate = -0.16%, EncT=102%, DecT=102%}

# Introduction

Pairwise average candidates as replacement of HEVC combined candidates were presented in JVET-J0018 [1] and JVET-K0245 [2] and planned as CE4.4.12 in JVET-K1024 [3]. More details are introduced in the next sections.

# Proposed methods

Pairwise average candidates are generated by averaging predefined pairs of candidates in the current merge candidate list, and the predefined pairs are defined as {(0, 1), (0, 2), (1, 2), (0, 3), (1, 3), (2, 3)}, where the numbers denote the merge indices to the merge candidate list. The averaged motion vectors are calculated separately for each reference list. If both motion vectors are available in one list, these two motion vectors are averaged even when they point to different reference pictures; if only one motion vector is available, use the one directly; if no motion vector is available, keep this list invalid. The pairwise average candidates replaces the combined candidates in HEVC standard.

The complexity analysis of pairwise average candidates is summarized in the Table 1. For the worst case of additional calculations for averaging (the last column in Table 1), 4 additions and 4 shifts are needed for each pair (MVx and MVy in L0 and L1), and 4 reference index comparisons are needed for each pair (refIdx0 is valid and refIdx1 is valid in L0 and L1). There are 6 pairs, leading to 24 additions, 24 shifts, and 24 reference index comparisons in total. The combined candidates in HEVC standard use 2 reference index comparisons for each pair (refIdx0 is valid in L0 and refIdx1 is valid in L1), and there are 12 pairs, leading to 24 reference index comparisons in total.

Table 1. Operation analysis for the pairwise average candidates

| Merge list size | Max number of potential candidates | Max number of candidate comparisons | Max number of MV scalings | Max number of temporal candidates | Additional local buffer | Max number of memory access | Others |
| --- | --- | --- | --- | --- | --- | --- | --- |
| 6, 8, 10 | 6 | 0 | 0 | 0 | 0 | 0 | Replace HEVC combined candidates, need additional calculations for averaging |

# Experiment results

The proposed pairwise average candidates have been implemented on top of VTM2.0.1. The common test conditions [4] are used for evaluation. Table 2, Table 3, and Table 4 summarize the results of adding pairwise average candidates with merge list sizes equal to 6, 8, and 10, respectively. Table 5, Table 6, and Table 7 summarize the results of replacing HEVC combined candidates with pairwise average candidates with merge list sizes equal to 6, 8, and 10, respectively.

Table 2. Results of adding pairwise average candidates with merge list size equal to 6

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.50% | -0.28% | -0.45% | 100% | 101% |
| Class A2 | -0.42% | -0.34% | -0.48% | 100% | 101% |
| Class B | -0.42% | -0.19% | -0.32% | 100% | 100% |
| Class C | -0.31% | -0.17% | -0.18% | 100% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.40% | -0.23% | -0.34% | 100% | 101% |
| Class D | -0.39% | -0.44% | -0.49% | 100% | 99% |
| Class F (optional) | -0.08% | 0.00% | -0.03% | 100% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.01% | 0.21% | 0.11% | 101% | 105% |
| Class C | 0.10% | 0.15% | 0.44% | 101% | 101% |
| Class E | 0.05% | 0.28% | 0.40% | 100% | 99% |
| Overall | 0.05% | 0.21% | 0.29% | 101% | 102% |
| Class D | 0.12% | 0.98% | 0.32% | 101% | 100% |
| Class F (optional) | -0.12% | 0.10% | 0.28% | 101% | 101% |

Table 3. Results of adding pairwise average candidates with merge list size equal to 8

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.70% | -0.45% | -0.60% | 100% | 101% |
| Class A2 | -0.69% | -0.60% | -0.76% | 101% | 102% |
| Class B | -0.58% | -0.34% | -0.57% | 100% | 100% |
| Class C | -0.51% | -0.41% | -0.38% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.61% | -0.43% | -0.56% | 101% | 101% |
| Class D | -0.60% | -0.60% | -0.75% | 101% | 100% |
| Class F (optional) | -0.17% | -0.14% | -0.16% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.09% | 0.14% | -0.20% | 102% | 105% |
| Class C | -0.07% | 0.01% | 0.07% | 102% | 101% |
| Class E | -0.20% | 0.20% | 0.26% | 101% | 99% |
| Overall | -0.11% | 0.11% | 0.01% | 102% | 102% |
| Class D | 0.01% | 0.35% | -0.41% | 102% | 101% |
| Class F (optional) | -0.03% | 0.53% | 0.18% | 102% | 101% |

Table 4. Results of adding pairwise average candidates with merge list size equal to 10

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.78% | -0.44% | -0.70% | 101% | 104% |
| Class A2 | -0.77% | -0.68% | -0.87% | 101% | 102% |
| Class B | -0.67% | -0.50% | -0.63% | 101% | 100% |
| Class C | -0.58% | -0.39% | -0.42% | 102% | 102% |
| Class E |  |  |  |  |  |
| Overall | -0.69% | -0.50% | -0.64% | 101% | 102% |
| Class D | -0.70% | -0.65% | -0.87% | 102% | 100% |
| Class F (optional) | -0.19% | -0.15% | -0.23% | 101% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.11% | 0.16% | -0.20% | 103% | 103% |
| Class C | -0.14% | -0.03% | 0.12% | 103% | 101% |
| Class E | -0.18% | -0.07% | -0.22% | 102% | 98% |
| Overall | -0.14% | 0.04% | -0.10% | 102% | 101% |
| Class D | -0.08% | 0.74% | 0.09% | 103% | 99% |
| Class F (optional) | -0.01% | 0.74% | 0.66% | 103% | 100% |

Table 5. Results of replacing HEVC combined candidates with pairwise average candidates with merge list size equal to 6

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.49% | -0.38% | -0.42% | 100% | 102% |
| Class A2 | -0.39% | -0.24% | -0.44% | 100% | 102% |
| Class B | -0.38% | -0.16% | -0.34% | 100% | 100% |
| Class C | -0.28% | -0.12% | -0.08% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.38% | -0.21% | -0.31% | 100% | 101% |
| Class D | -0.32% | -0.33% | -0.35% | 100% | 100% |
| Class F (optional) | -0.11% | -0.06% | -0.07% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.26% | 0.20% | 101% | 104% |
| Class C | 0.08% | 0.17% | 0.32% | 100% | 101% |
| Class E | -0.19% | -0.09% | -0.23% | 100% | 100% |
| Overall | 0.01% | 0.14% | 0.13% | 101% | 102% |
| Class D | 0.15% | 0.49% | -0.31% | 101% | 100% |
| Class F (optional) | -0.07% | 0.46% | 0.22% | 101% | 101% |

Table 6. Results of replacing HEVC combined candidates with pairwise average candidates with merge list size equal to 8

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.64% | -0.36% | -0.51% | 100% | 101% |
| Class A2 | -0.62% | -0.51% | -0.59% | 101% | 102% |
| Class B | -0.52% | -0.36% | -0.35% | 100% | 99% |
| Class C | -0.44% | -0.28% | -0.32% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.54% | -0.37% | -0.42% | 101% | 100% |
| Class D | -0.50% | -0.45% | -0.54% | 100% | 100% |
| Class F (optional) | -0.17% | -0.16% | -0.19% | 101% | 102% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.06% | 0.22% | -0.02% | 102% | 105% |
| Class C | -0.03% | 0.08% | 0.23% | 101% | 101% |
| Class E | -0.33% | -0.15% | -0.24% | 101% | 99% |
| Overall | -0.07% | 0.08% | 0.01% | 101% | 102% |
| Class D | 0.08% | 0.10% | 0.13% | 102% | 100% |
| Class F (optional) | -0.04% | 0.54% | 0.21% | 102% | 101% |

Table 7. Results of replacing HEVC combined candidates with pairwise average candidates with merge list size equal to 10

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.73% | -0.37% | -0.62% | 101% | 102% |
| Class A2 | -0.69% | -0.55% | -0.65% | 101% | 101% |
| Class B | -0.58% | -0.33% | -0.49% | 101% | 100% |
| Class C | -0.49% | -0.32% | -0.44% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.61% | -0.38% | -0.54% | 101% | 101% |
| Class D | -0.61% | -0.60% | -0.61% | 101% | 99% |
| Class F (optional) | -0.16% | -0.13% | -0.14% | 101% | 100% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.02% | -0.05% | 0.03% | 102% | 103% |
| Class C | -0.06% | 0.01% | 0.21% | 102% | 103% |
| Class E | -0.53% | -0.42% | -0.91% | 101% | 100% |
| Overall | -0.16% | -0.12% | -0.15% | 102% | 102% |
| Class D | 0.01% | 0.38% | 0.02% | 103% | 100% |
| Class F (optional) | -0.22% | 0.16% | 0.79% | 102% | 101% |

# Conclusion

Pairwise average candidates are proposed in this contribution. Adding pairwise average candidates achieved -0.40% and 0.05% luma BD-rates for VTM2.0.1-RA and VTM2.0.1-LB, respectively, when merge list size is equal to 6; -0.61% and -0.11% luma BD-rates for VTM2.0.1-RA and VTM2.0.1-LB, respectively, when merge list size is equal to 8; -0.69% and -0.14% luma BD-rates for VTM2.0.1-RA and VTM2.0.1-LB, respectively, when merge list size is equal to 10. Replacing HEVC combined candidates with pairwise average candidates achieved -0.38% and 0.01% luma BD-rates for VTM2.0.1-RA and VTM2.0.1-LB, respectively, when merge list size is equal to 6; -0.54% and -0.07% luma BD-rates for VTM2.0.1-RA and VTM2.0.1-LB, respectively, when merge list size is equal to 8; -0.61% and -0.16% luma BD-rates for VTM2.0.1-RA and VTM2.0.1-LB, respectively, when merge list size is equal to 10. The encoding and decoding time increases are all less than 2%.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] C.-W. Hsu, C.-Y. Chen, T.-D. Chuang, H. Huang, S.-T. Hsiang, C.-C. Chen, M.-S. Chiang, C.-Y. Lai, C.-M. Tsai, Y.-C. Su, Z.-Y. Lin, Y.-L. Hsiao, J. Klopp, I.-H. Wang, Y.-W. Huang, S.-M. Lei, "Description of SDR video coding technology proposal by MediaTek," Document of Joint Video Experts Team, JVET-J0018.

[2] Y.-L. Hsiao, Z.-Y. Lin, T.-D. Chuang, C.-C. Chen, C.-Y. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, "CE4.2.8: Merge mode enhancement," Document of Joint Video Experts Team, JVET-K0245.

[3] H. Yang, S. Liu, K. Zhangi, "Description of Core Experiment 4 (CE4): Inter prediction and motion vector coding," Document of Joint Video Experts Team, JVET-K1024.

[4] J. Boyce, K. Suehring, X. Li, V. Seregin, "JVET common test conditions and software reference configurations," Document of Joint Video Experts Team, JVET-K1010.

# Derivation process for pairwise average merging candidates

Derivation process for pairwise average merging candidates

Inputs to this process are:

- a merging candidate list mergeCandList,

- the reference indices refIdxL0N and refIdxL1N of every candidate N in mergeCandList,

- the prediction list utilization flags predFlagL0N and predFlagL1N of every candidate N in mergeCandList,

- the motion vectors in 1/16 fractional-sample accuracy mvL0N and mvL1N of every candidate N in mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList.

Outputs of this process are:

- the merging candidate list mergeCandList,

- the number of elements numCurrMergeCand within mergeCandList,

- the reference indices refIdxL0avgCandk and refIdxL1avgCandk of every new candidate avgCandk added into mergeCandList during the invocation of this process,

- the prediction list utilization flags predFlagL0avgCandk and predFlagL1avgCandk of every new candidate avgCandk added into mergeCandList during the invocation of this process,

- the motion vectors in 1/16 fractional-sample accuracy mvL0avgCandk and mvL1avgCandk of every new candidate avgCandk added into mergeCandList during the invocation of this process.

The variables numInputMergeCand, numCutoff, avgIdx, avgStop, numRefLists are set as follows:

- numInputMergeCand = numCurrMergeCand

- numCutoff = min(numCurrMergeCand, 4)

- avgIdx = 0

- avgStop = (avgIdx >= ( numCutoff * ( numCutoff − 1 ) / 2 ) || numCurrMergeCand == MaxNumMergeCand)

- numRefLists = (slice_type = = B) ? 2: 1

The following ordered steps are repeated until avgStop is equal to TRUE:

- The variables p0CandIdx and p1CandIdx are derived using avgIdx as specified in Table.

- The following assignments are made, with p0Cand being the candidate at position p0CandIdx and p1Cand being the candidate at position p1CandIdx in the merging candidate list mergeCandList:

- p0Cand = mergeCandList[ p0CandIdx ]

- p1Cand = mergeCandList[ p1CandIdx ]

- The candidate avgCandk with k equal to ( numCurrMergeCand − numInputMergeCand ) is added at the end of mergeCandList, i.e., mergeCandList[ numCurrMergeCand ] is set equal to avgCandk, and the reference indices, the prediction list utilization flags and the motion vectors of avgCandk are derived as follows:

For each reference picture list LX with X ranging from 0 to ( numRefLists - 1 ):

- If all of the following conditions are true:

- predFlagLXp0Cand  = =  1

- predFlagLXp1Cand  = =  1

refIdxLXavgCandk = refIdxLXp0Cand

predFlagLXavgCandk = 1

mvLXavgCandk[ 0 ] = ( mvLXp0Cand[ 0 ] + mvLXp1Cand[ 0 ] ) / 2

mvLXavgCandk[ 1 ] = ( mvLXp0Cand[ 1 ] + mvLXp1Cand[ 1 ] ) / 2

- If all of the following conditions are true:

- predFlagLXp0Cand  = =  1

- predFlagLXp1Cand  = =  0

refIdxLXavgCandk = refIdxLXp0Cand

predFlagLXavgCandk = 1

mvLXavgCandk[ 0 ] = mvLXp0Cand[ 0 ]

mvLXavgCandk[ 1 ] = mvLXp0Cand[ 1 ]

- If all of the following conditions are true:

- predFlagLXp0Cand  = =  0

- predFlagLXp1Cand  = =  1

refIdxLXavgCandk = refIdxLXp1Cand

predFlagLXavgCandk = 1

mvLXavgCandk[ 0 ] = mvLXp1Cand[ 0 ]

mvLXavgCandk[ 1 ] = mvLXp1Cand[ 1 ]

- If all of the following conditions are true:

- predFlagLXp0Cand  = =  0

- predFlagLXp1Cand  = =  0

refIdxLXavgCandk = -1

predFlagLXavgCandk = 0

mvLXavgCandk[ 0 ] = 0

mvLXavgCandk[ 1 ] = 0

- The variable numCurrMergeCand and avgIdx are incremented by 1.

- When avgIdx is larger or equal to ( numCutoff * ( numCutoff − 1 ) / 2 ) or numCurrMergeCand is equal to MaxNumMergeCand, avgStop is set equal to TRUE.

Table – Specification of p0CandIdx and p1CandIdx

| avgIdx | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| p0CandIdx | 0 | 0 | 1 | 0 | 1 | 2 |
| p1CandIdx | 1 | 2 | 2 | 3 | 3 | 3 |