| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0091-v1 |
| --- | --- |

| Title: | CE4-related: Shared merge list |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chia Chen, Chih-Wei Hsu Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chunchia.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to share the same merge list for all leaf coding units (CUs) of one ancestor node in the CU split tree for enabling parallel processing of merge mode for small CUs. The ancestor node is named merge sharing node. The shared merge list is generated at the merge sharing node pretending the merge sharing node is a leaf CU. When the proposed method is applied to VTM2.0.1, luma BD-rates are 0.09% and 0.05% for RA and LB, respectively. It is claimed that the shared merge list can effectively reduce difficulty for real-time 8K hardware decoding.

# Introduction

In a high-end (e.g., 8K 120fps) video decoder, a parallel-design-friendly algorithm is preferred. However, merge list construction of one CU cannot be computed in parallel at the same time with that of the previous CU. Table 1 shows the clock cycle budgets for small CUs to support real-time decoding. In hardware designs, the clock cycle budgets are very tight to support real-time 8K 60fps or 120fps decoding for merge modes. There are five spatial merge candidates and one temporal merge candidate. Before one new candidate is inserted into the list, pruning (i.e., comparing the new candidate with the candidates in the list to avoid redundancy) is performed to decide whether to insert. In addition to the above traditional merge modes, alternative temporal motion vector prediction (ATMVP) is a new merge mode that is much more complex than the traditional merge modes. Moreover, in the future, merge mode is possible to become even more complicated (e.g. history-based merge candidates, non-adjacent merge candidates, average merge candidates) for achieving higher coding gain. Real-time hardware decoding is becoming more challenging.

Table 1. Clock cycle budgets for one CU

| Assuming Clock @ 1GHz | 8K 60fps decoder | 8K 120fps decoder |
| --- | --- | --- |
| 4x4 CU | 8 | 4 |
| 4x8 or 8x4 CU | 16 | 8 |
| 8x8 CU | 32 | 16 |
| Assuming Clock @ 500MHz | 8K 60fps decoder | 8K 120fps decoder |
| 4x4 CU | 4 | 2 |
| 4x8 or 8x4 CU | 8 | 4 |
| 8x8 CU | 16 | 8 |

# Proposed method

It is proposed to share the same merge list for all leaf CUs of one ancestor node in the CU split tree for enabling parallel processing of merge mode for small CUs. The ancestor node is named merge sharing node. The shared merge list is generated at the merge sharing node pretending the merge sharing node is a leaf CU. The merge sharing node is the largest ancestor node among all the ancestor nodes of the leaf CUs satisfying the following two criteria.
(1) The merge sharing node size is equal to or smaller than a predefined size threshold.
(2) No samples of the merge sharing node are outside the picture boundary.
Therefore, in cases that the merge sharing mode is fully inside the current picture, the parent node of the merge sharing node is larger than the predefined size threshold.

Figure 1 shows four examples of merge sharing nodes assuming they are fully inside the current picture. The shared merge list is generated for the dotted virtual CU (i.e., merge sharing node). The merge sharing node originates from a binary split or a ternary split, so the parent node size can be 128 or 256, which is larger than 64 (the predefined size threshold), and the merge sharing node is equal to or smaller than 64 (the predefined size threshold).

Figure 1. Examples of merge sharing nodes

In Figure 1, if the size threshold is equal to 128 instead of 64, the merge sharing node must originate from a ternary split in which the parent node size is 256 and larger than the size threshold 128. The merge sharing node cannot originate from a binary split because the parent node size would be 128 and not larger than the size threshold.

# Simulation results

The proposed method is integrated on VTM2.0.1. All tests were conducted under the common test condition [1]. The size threshold is set according to the picture size, as listed in Table 2. Table 3 shows the simulation results of the proposed method.

It is also possible to signal the size threshold in sequence parameter set (SPS), picture parameter set (PPS), or slice header and make the settings in Table 2 to become encoder-only. It is believed that the BD-rate impact of explicit signaling the size threshold is negligible.

# Conclusions

In this contribution, it is proposed to share the same merge list for all leaf CUs of one ancestor node in the CU split tree. It is shown that the proposed concept can facilitate parallel processing of merge mode for small CUs while suffering less than 0.1% BD-rate on average.

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-K1010.

Table 2. Size threshold according to picture size (in number of luma samples)

| Condition | Size Threshold |
| --- | --- |
| 0 < Picture Size < 217 | 32 |
| 217 <= Picture Size < 219 | 64 |
| 219 <= Picture Size < 222 | 128 |
| 222 <= Picture Size | 256 |

Table 3. Results of the proposed method