| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0095-v1 |
| --- | --- |

| Title: | CE7-related: Modified dequantization scaling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Shawmin Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In VCC WD 2, when the size of a transform block is not a power of 4, the values of the transform coefficient levels shall be further scaled by a normalization factor equal to 1/sqrt(2). The number of fractional bits is such further increased by 8 for the scaling process. In this contribution, the scaling process on transform coefficient levels by a dequantization scaling factor and a normalization scaling factor is replaced by one scaling operation using a 2x6 look-up table. The number of fractional bits for the scaling process by the proposed method is reportedly kept the same as that of HEVC when the 6 fractional bits are employed for representing the entry values of the proposed look-up table. The proposed dequantization scaling method reportedly leads to -0.03%, 0.00%, and -0.03% average luma BD-rates for the AI, RA, and LB, respectively, under the VTM2.0.1 common test conditions (CTCs).

# Introduction

In VCC WD 2 as specified in [1], each CTU can be partitioned into one or multiple smaller size coding units (CUs) by a quadtree with nested multi-type tree using binary and ternary splits. The resulting transform units can be in square or rectangular shapes. When the size of a transform block is not a power of 4, the values of the transform coefficients and the coded transform coefficient levels are scaled by a square root of 2, related to variable rectNorm in VTM2.0.1 [2], for normalization of transform coefficient values. The number of fractional bits resulted from cascaded scaling on the coefficient level TransCoeffLevel[ x ][ y ] by the dequantization scaling factor levelScale[ qP % 6 ] and the normalization scaling factor rectNorm is equal to 14 (6 fractional bits for levelScale[ ] and 8 fractional bits for rectNorm), further increased by 8 bits.

# Proposed method

In the proposed method, the cascaded scaling operations on the coefficient level TransCoeffLevel[ x ][ y ] by the dequantization scaling factor and the normalization scaling factor is replaced by one scaling operation with a normalized dequantization scaling factor equal to the multiplication of the dequantization scaling factor by a normalization scaling factor. The values of the normalized dequantization scaling factors corresponding to different combinations of the dequantization scaling factors and the normalization scaling factors can be pre-calculated and stored in a 2x6 look-up table levelScaleRectNorm[ rectNormFlag ][ qP % 6 ], where rectNormFlag equal to 0 and 1 corresponds to a normalization factor equal to 0 and 1/sqrt(2), respectively.

Tables 1 and 2 provide the values of array levelScaleRectNorm[ rectNormFlag ][ qP % 6 ] for the number of fractinal bits equal to 6 and 8, respectively. Tables 3 and 4 provide the values of the normalized quantization factors with 14 fractional bits, corresponding to Tables 1 & 2, respectively, for quantization of transform cofficients at a encoder.

Table 1: The values of normalized dequantization scaling factors using 6 fractinal bits

| qP % 6 | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| rectNormFlag=0 | 40 | 45 | 51 | 57 | 64 | 72 |
| rectNormFlag=1 | 29 | 32 | 36 | 40 | 45 | 51 |

Table 2: The values of normalized dequantization scaling factors using 8 fractinal bits

| qP % 6 | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| rectNormFlag=0 | 161 | 181 | 203 | 228 | 256 | 287 |
| rectNormFlag=1 | 114 | 128 | 144 | 161 | 181 | 203 |

Table 3: The values of normalized quantization scaling factors for encoding, corresponding to normalized dequantization scaling factors in Table 1

| qP % 6 | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| rectNormFlag=0 | 26214 | 23302 | 20560 | 18396 | 16384 | 14564 |
| rectNormFlag=1 | 36158 | 32768 | 29127 | 26214 | 23302 | 20560 |

Table 4: The values of normalized quantization scaling factors for encoding, corresponding to normalized dequantization scaling factors in Table 2

| qP % 6 | 0 | 1 | 2 | 3 | 4 | 5 |
| --- | --- | --- | --- | --- | --- | --- |
| rectNormFlag=0 | 26052 | 23173 | 20662 | 18396 | 16384 | 14614 |
| rectNormFlag=1 | 36792 | 32768 | 29127 | 26052 | 23173 | 20662 |

# Experimental results

The proposed method was integrated into the VTM2.0.1 reference software [2]. The coding results generated by the proposed method were evaluated against the anchor results generated by the unmodified VTM2.0.1 reference software under VTM2.0.1 CTCs [3].

Table 5 shows the results of the proposed method using Tables 1 and 3 with 6 fractional bits for representing the values of the normalized dequantization scaling factors. Table 6 shows the results of the proposed method using Tables 2 and 4 with 8 fractional bits for representing the values of the normalized dequantization scaling factors. It is shown that the proposed method leads to minor BD-rate differences compared with the anchor results while reducing the number of the fractional bits for the overall dequantization scaling operation by 8 bits using Table 1 or by 6 bits using Table 2. Please note the results summarized in Table 6 are generated using a non-uniform computing cluster, and the runtime results in Table 6 are not accurate. It is asserted that runtime changes are negligible if measured accurately.

Table 5: Summary of the BD-rates, EnCT, and DecT using Tables 1 and 3

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.03% | 0.24% | 0.27% | 100% | 100% |
| Class A2 | -0.05% | 0.34% | 0.28% | 100% | 101% |
| Class B | -0.02% | 0.31% | 0.36% | 100% | 100% |
| Class C | -0.05% | 0.17% | 0.38% | 100% | 99% |
| Class E | -0.03% | 0.15% | 0.25% | 100% | 99% |
| Overall | -0.03% | 0.25% | 0.32% | 100% | 100% |
| Class D | -0.03% | 0.34% | 0.05% | 100% | 100% |
| Class F (optional) | -0.03% | 0.08% | 0.08% | 100% | 100% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.09% | 0.10% | 99% | 106% |
| Class A2 | -0.01% | 0.16% | 0.01% | 100% | 107% |
| Class B | 0.00% | 0.15% | 0.22% | 99% | 102% |
| Class C | 0.02% | 0.24% | 0.14% | 100% | 101% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | 0.16% | 0.13% | 100% | 103% |
| Class D | 0.03% | -0.24% | 0.31% | 99% | 99% |
| Class F (optional) | 0.00% | 0.09% | -0.01% | 100% | 101% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.00% | 0.02% | 0.12% | 100% | 103% |
| Class C | -0.02% | 0.08% | -0.03% | 100% | 101% |
| Class E | -0.10% | 0.75% | -0.10% | 100% | 99% |
| Overall | -0.03% | 0.22% | 0.01% | 100% | 101% |
| Class D | 0.00% | 1.13% | 0.29% | 100% | 100% |
| Class F (optional) | -0.07% | 0.95% | -0.20% | 100% | 101% |

Table 6: Summary of the BD-rates using Tables 2 and 4, EncT & DecT are not accurate

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.01% | 0.05% | 97% | 100% |
| Class A2 | -0.04% | 0.21% | 0.08% | 97% | 97% |
| Class B | -0.01% | 0.10% | 0.15% | 101% | 106% |
| Class C | -0.02% | -0.01% | 0.15% | 99% | 103% |
| Class E | -0.03% | 0.03% | -0.06% | 101% | 106% |
| Overall | -0.02% | 0.07% | 0.09% | 99% | 103% |
| Class D | -0.03% | 0.21% | -0.13% | 99% | 101% |
| Class F (optional) | 0.01% | -0.03% | 0.20% | 99% | 102% |
|  | Random Access Main 10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.04% | -0.01% | 0.06% | 95% | 96% |
| Class A2 | -0.02% | -0.12% | -0.12% | 98% | 100% |
| Class B | 0.02% | -0.07% | 0.08% | 100% | 102% |
| Class C | 0.01% | 0.14% | -0.07% | 106% | 113% |
| Class E |  |  |  |  |  |
| Overall | 0.00% | -0.01% | 0.00% | 100% | 103% |
| Class D | 0.02% | -0.23% | 0.19% | 105% | 109% |
| Class F (optional) | -0.03% | -0.08% | 0.03% | 104% | 107% |
|  | Low delay B Main10 |  |  |  |  |
|  | Over VTM-2.0.1 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.05% | 0.35% | 0.08% | 110% | 116% |
| Class C | 0.03% | 0.06% | -0.21% | 105% | 111% |
| Class E | -0.25% | 0.65% | -0.71% | 104% | 109% |
| Overall | -0.03% | 0.33% | -0.22% | 107% | 113% |
| Class D | -0.05% | 0.63% | -0.10% | 103% | 108% |
| Class F (optional) | -0.08% | 0.07% | 0.18% | 108% | 111% |

# Conclusions

For transform blocks with non-power of 4 sizes, the scaling process on transform coefficient levels by a dequantization scaling factor and a normalization scaling factor is replaced by one scaling operation using a 2x6 look-up table. The number of fractional bits for the scaling process is kept the same as that of HEVC when the 6 fractional bits are employed for representing the entry values of the proposed look-up table. It is shown that the BD-rate impact is negligible by this proposal.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 2),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1001, 11th Meeting: Ljubljana, SI, 10–18 July 2018.

- The software VTM2.0.1, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-2.0.1

- F. Bossen, J. Boyce, X. Li, and V. Seregin, K. Suehring, “JVET common test conditions and software reference configurations,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-K1010, 11th Meeting: Ljubljana, SI, 10–18 July 2018.