| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0098-v1 |
| --- | --- |

| Title: | CE9-related: Simplified DMVR with reduced internal memory |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chun-Chia Chen, Chih-Wei Hsu Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {chunchia.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

Two methods to reduce the hardware internal memory requirement for decoder-side motion vector refinement (DMVR) are proposed. The current DMVR requires 48.3K bytes internal memory to store the search ranges of the two lists. The first method turns off DMVR for large coding units (CUs). The second method splits a large CU into several partitions and executes DMVR for each partition independently. When the first method is applied to BMS2.1, luma BD-rates are reportedly 0.11% for RA with 100% encoding time and 95% decoding time. By applying the first method, it is claimed that 69.2% of the internal memory requirement for DMVR is saved. When the second method is applied to BMS2.1 to split a large DMVR CU into 32x32 partitions, luma BD-rates are reportedly -0.07% for RA with 100% encoding time and 105% decoding time, and it is claimed that 86.3% of the internal memory requirement for DMVR is saved. When the second method is applied to BMS2.1 to split a large DMVR CU into 64x64 partitions, luma BD-rates are reportedly -0.02% for RA with 100% encoding time and 102% decoding time, and it is claimed that 69.2% of the internal memory requirement for DMVR is saved.

# Introduction

In hardware decoder designs, internal memory is frequently used to reduce external memory bandwidth, especially for high-end real-time decoding (e.g., 8K 60fps) that can easily use up all available external memory bandwidth. For DMVR, a hardware decoder needs to fetch list 0 (L0) search region and list 1 (L1) search region into internal memory to reduce external memory access. However, the internal memory requirements for the search regions are huge for large CUs. For one CU of width CUW and height CUH, the internal memory size for the two search regions is ((CUW+FilterN+SR*2)*(CUH+FilterN+SR*2))*2 in terms of luma samples, in which FilterN means the number of extra samples needed for interpolation for fractional samples, SR means the search range (e.g., SR=1 means searching [-1, +1] x [-1, +1] samples). For the current DMVR design, FilterN = 7, SR = 2. For the current VVC, the maximum CU size of 128x128. If each sample has 10 bits, in total 48.3K bytes internal memory is needed to store the search regions, which is quite expensive. Therefore, an internal memory reduction is needed.

# Proposed methods

Two methods are proposed to reduce the internal memory requirement of DMVR.

## Disable DMVR for large CUs

In this method, DMVR is disabled if the current CU size is equal to or larger than a predefined threshold. Table 1 shows the internal memory reduction results for this method. Note that, if the threshold = 128x64, the internal memory is reduced to 30.8% of the original DMVR, that is, 69.2% internal memory is saved.

Table 1. DMVR internal memory reduction by disabling DMVR for large CUs

|  | Internal Memory Requirement for Search Ranges of Two Lists | Size in K bytes (10 bits per sample) |
| --- | --- | --- |
| Original | 38642 samples | 48.3 |
| Threshold = 128x128 | 20850 samples | 26.1 |
| Threshold = 128x64 | 11954 samples | 14.9 |
| Threshold = 64x64 | 7506 samples | 9.4 |

## Split one large CU into partitions for DMVR

In this method, one large CU is split into several partitions. Then, DMVR is applied to each partition independently. By the CU partitioning, the internal memory requirement is reduced effectively.

Assume the partition size is N x N, and the CU size is CUW x CUH. If the (CUW x CUH) is equal to or smaller than (N x N), the CU will not be split at all, and the entire CU is treated as one DMVR block. Otherwise, if CUW < N, vertical split is applied; if CUH < N, horizontal split is applied. For example, if CU size is 128x128 and the partition size is 32x32, there are 16 32x32 partitions in total. If CU size is 128x16 and the partition size is 32x32, the CU will be split into four 32x16 partitions.

Table 2 shows the internal memory reduction results for this method. For partition size equal to 32x32, the internal memory is reduced to 13.7% of the original DMVR, that is, 86.3% of internal memory is saved.

Table 2. DMVR internal memory reduction by splitting one large CU into partitions for DMVR

|  | Internal Memory Requirement for Search Ranges of Two Lists | Size (Kbytes, assuming 10 bit per pixel) |
| --- | --- | --- |
| Original | 38642 samples | 48.3 |
| Partition Size = 64 x 64 | 11954 samples | 14.9 |
| Partition Size = 32 x 32 | 5282 samples | 6.6 |
| Partition Size = 16 x 16 | 2250 samples | 2.8 |

# Simulation results

The proposed methods are integrated on BMS2.1. All tests were conducted under the common test condition [1]. Table 3 shows the results of disabling DMVR for large CUs with the threshold equal to 128x64. Table 4 shows the results of splitting one large CU into partitions for DMVR with the partition size equal to 32x32. Table 5 shows the results of splitting one large CU into partitions for DMVR with the partition size equal to 64x64. Because DMVR is only activated in RA, the proposed modification for DMVR only causes BD-rate changes for RA and results in no changes for LB. Tables 3, 4, and 5 only show RA results and does not show LB results.

Table 3. Results of disabling DMVR for large CUs with the threshold equal to 128x64

Table 4. Results of splitting one large CU into partitions for DMVR with the partition size equal to 32x32

Table 5. Results of splitting one large CU into partitions for DMVR with the partition size equal to 64x64

# Conclusions

This contribution proposes two methods to reduce the hardware internal memory requirement of DMVR. The first method turns off DMVR for large CUs. The second method splits a large CU into several partitions and executes DMVR for each partition independently. The first method slightly increases BD-rate while 69.2% of internal memory is saved. The second method has nearly no impact on BD-rate while 69.2% - 86.3% of internal memory is saved.

# Patent rights declaration(s)

MediaTek may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” Document of Joint Video Experts Team, JVET-K1010.