| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 12th Meeting: Macao, CN, 3–12 Oct. 2018 | Document: JVET-L0101-v1 |
| --- | --- |

| Title: | CE10.2.1: OBMC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Chun-Chia Chen, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhi-yi.lin, chunchia.chen, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

This contribution presents proposed complexity reduction methods for overlapped block motion compensation (OBMC). To reduce computational complexity, lossless data reuse, coding unit (CU) boundary only OBMC, removal of OBMC syntax, and disabling OBMC for small and large CUs are proposed. To remove all the additional motion compensation (MC) bandwidth, replacing additional reference samples required by the JEM OBMC with padded reference samples is proposed. To save the line buffer requirement, decreasing the number of blended lines from 4 to 2 at CTU row boundaries is proposed. To allow parallel processing for top and left boundaries of a CU, a parallel blending method is proposed. It is reported that OBMC with all the proposed complexity reduction methods results in -0.82% and -1.17% luma BD-rates with 2% and 3% encoding time increases and 8% and 9% decoding time increases for VTM2.0.1-RA and VTM2.0.1-LB, respectively.

# Introduction

Based on JVET-K0258 [1] and JVET-K0259 [2], this contribution proposed several methods to reduce the OBMC complexity. More details are described in the following sections.

# Proposed methods

Seven aspects are proposed. To reduce computational complexity, lossless data reuse, CU boundary only OBMC, removal of OBMC syntax, and disabling small and large CUs for OBMC are proposed and presented in Sections 2.1, 2.2, 2.3, and 2.4, respectively. To remove all the additional MC bandwidth, replacing additional reference samples with padded reference samples is proposed and presented in Section 2.5. To save the line buffer requirement, decreasing the number of blended lines at CTU row boundaries is proposed and presented in Section 2.6. To allow parallel processing for top and left boundaries of a CU, parallel blending is proposed and presented in Section 2.7.

## Lossless data reuse

In this proposal, redundant MC operations in JEM OBMC are removed. Two data reuse methods are proposed and described in Sections 2.1.1 and 2.1.2. The first method can reduce encoding time and does not change encoder outputs. The second method can reduce both encoding time and decoding time and does not change encoder and decoder outputs.

### L-buffer OBMC block reuse

At encoder, we can use one L-buffer to store OBMC blocks of the first row and first column blocks in the current CU at top and left CU boundaries. The OBMC blocks stored in the L-buffer can be shared for different rate-distortion optimization (RDO) processes of CU mode candidates. The L-buffer has a top part to store top block boundary OBMC blocks (top-direction OBMC of top 4x4 row) and a left part to store left block boundary OBMC blocks (left-direction OBMC of left 4x4 column). In our implementation, in the current CU, for each 4x4 block at the first row/column, when this 4x4 block needs top/left block boundary OBMC block for the first time, it will do actual 4x4 block MC to obtain the OBMC block and store it into this 4x4 block buffer. For any other RDO processes of different CU mode candidates, no 4x4 block MC will be executed, and the stored OBMC block can be reused. Readers can refer to JVET-K0258 for better understanding.

### MC merging

When we generate top/left block boundary OBMC blocks at the top/left CU boundaries, if adjacent OBMC blocks have the same motion information (reference index and motion vector), they can be generated together to save encoding time and decoding time. That is, the MC operations of adjacent OBMC blocks of the same motion information can be merged into one MC operation of a larger OBMC block for saving redundant memory access. Readers can refer to JVET-K0258 for better understanding.

## CU boundary only OBMC

OBMC will only be applied at top and left CU boundaries of a current CU.

## Removal of OBMC syntax

The CU-level OBMC flag is removed. In this way, the encoder does not need OBMC on/off decision, and the decoder does not need to parse the OBMC flag. OBMC is always turned on for inter CUs if not specified otherwise.

## Disabling OBMC for small and large CUs

OBMC is disabled for CUs with sizes smaller than 64 luma samples or larger than 1024 luma samples.

## Replacing additional reference samples with padded reference samples

It is proposed to pad (3+W+4)*(3+H+4) reference samples to (3+W+4+w’)*(3+H+4+h’), where w’ and h’ are the width and height of OBMC region respectively, as shown in Figure 1. The w’ columns in orange region, which are the additional samples required to derive OBMC block R, are generated by replicating the samples in the right-most column of the blue region. The h’ rows in green region, which are the additional samples required to derive OBMC block B, are generated by replicating the samples in the bottom row of the blue region. Readers can refer to JVET-K0259 for better understanding.

Figure 1. Reference samples with padding for OBMC.

## Decreasing the number of blended lines at CTU row boundaries

To reduce the line buffer size, if the current block is at a CTU row boundary, then the number of blended lines for OBMC is reduced from 4 to 2.

## Parallel blending

The proposed parallel blending can be applied to CU boundary only OBMC and sub-block OBMC. The idea is shown in equations of Figure 2, where the F(x,y) is the final blending result, the C(x,y) is the current 4x4 block predictor, and the L(x,y) / R(x,y) / T(x,y) / B(x,y) is the left / right / top / bottom block OBMC block. If the left / right / top / bottom block boundary OBMC is skipped (when (1) unavailable boundary or (2) same motion as current sub-block or (3) internal boundaries when CU boundary only OBMC is applied), the L(x,y) / R(x,y) / T(x,y) / B(x,y) is replaced with C(x,y). The width / height is the current sub-block width / height, e.g, in luma, the width / height is 4. The w(i) is a weight table. The rem_w(x,y) is the “remaining weight” defined as (32 – w(x) – w(width-x) – w(y) – w(height-y)). For fixed width, height, and blending type (normal blending or simplified blending), the rem_w(x,y) is fixed for any sub-block and can be easily implemented by table look-up. Readers can refer to JVET-K0258 for better understanding.

Figure 2. Parallel blending

# Experiment results

The proposed modifications have been implemented on top of BMS2.0.1 [3]. The common test conditions [4] are used for evaluation. Note that the simulations in VTM2.0.1 test condition are performed by using VTM2.0.1 encoder configuration settings without MACRO changes in the software. The anchors for all of the tests are VTM2.0.1. The descriptions of each table are listed as below:

- Test1: lossless data reuse, CU boundary only OBMC, removal of OBMC syntax, disabling OBMC for small CUs

- Test2: Test1 + replacing additional reference samples with padded reference samples

- Test3: Test2 + decreasing the number of blended lines at CTU row boundaries

- Test4: Test3 + parallel blending

- Test5: Test4 + disabling OBMC for large CUs

Tables 1, 2, 3, 4, and 5 show the results of Tests 1, 2, 3, 4, and 5, respectively.

# Conclusions

In this proposal, several OBMC complexity reduction methods are proposed. The overall results of the proposed OBMC are 0.82% and 1.17% BD-rate savings with 2% and 3% encoding time increases and 8% and 9% decoding time increases for VTM2.0.1-RA and VTM2.0.1-LB, respectively.

# References

[1] C.-C. Chen, C.-W. Hsu, Y.-W. Huang, S.-M. Lei, “CE10-related: OBMC complexity reduction and parallel blending,” Document of Joint Video Experts Team, JVET-K0258.

[2] Z.-Y. Lin, T.-D. Chuang, C.-Y. Chen, Y.-W. Huang, S.-M. Lei, “CE10-related: OBMC bandwidth reduction and line buffer reduction,” Document of Joint Video Experts Team, JVET-K0259.

[3] BMS-2.0.1: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_BMS

[4] F. Bossen, J. Boyce, X. Li, V. Seregin, K. Sühring, “JVET common test conditions and software reference configurations for SDR video,” JVET-K1010, August 2018.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Table 1.  Results of Test1

Table 2.  Results of Test2

Table 3. Results of Test3

Table 4. Results of Test4

Table 5.  Results of Test5